3GPP TSG RAN WG1 #110	R1-2207286

Toulouse, France, August 22nd – 26th, 2022

Source:	Sharp

Title:	Views on low power high accuracy positioning

Agenda Item:	9.5.2.3

Document for:	Discussion and Decision

Introduction

In RAN1#109-e, technical discussions on a new study item on “expanded and improved NR positioning” was started. For low power high accuracy positioning (LPHAP), the following agreements were made [1].

Agreement Confirm that use case 6 defined in TS 22.104 is the single representative use case for the study of LPHAP. Agreement At least the relative power unit is adopted as the performance metric to evaluate the power consumption of the Rel-17 RRC_INACTIVE state positioning and potential enhancements. Agreement A reference device (e.g., a mobile phone) with reference traffic type, reference battery capability, and reference battery life is defined for the purpose of identification of the performance gap that achieved by the Rel-17 RRC_INACTIVE state positioning baseline and the target battery life of LPHAP use case 6. Agreement - Adopt the following parameters as the common evaluation parameters for the LPHAP evaluation: - Frequency range: FR1 (baseline); FR2 (optional) - SCS: 30kHz for FR1 (baseline); 120kHz for FR2 (optional) - BW of the DL PRS and UL SRS pos: 100MHz; - Single-sample measurement per position fix (baseline); 4-sample measurement per position fix (optional) - UE mobility: up to 3km/h - Note: It is up to each company to provide detailed power model and evaluation results on power consumption in FR2. Agreement In the LPHAP evaluation, the power consumption of 5GC data traffic is not modelled. Only the power consumption of the traffic type related to LPHAP positioning (e.g., obtaining/updating SRS configurations, DL PRS measurement reporting, etc.) is considered. - Note: This does not preclude the power consumption of paging monitoring in the baseline evaluation, but rather assumes that no power consumption of 5GC data traffic is considered during a power cycle. Agreement Adopt the following power consumption model common for the baseline evaluation of Rel-17 RRC_INACTIVE state positioning. Power State; Relative power. PDCCH-only (PPDCCH); 50Note. PDCCH + PDSCH (PPDCCH+PDSCH); 120. SSB proc. (PSSB); 50. UL; 250 (0 dBm) 700 (23 dBm). (Optional) PRACH; [210]. (Optional) BWP switching; [50]. (Optional) Intra-frequency RRM measurement (Pintra); [60] (synchronous case, N=8, measurement only; Pintra, meas-only) [80] (combined search and measurement; Pintra, search+meas). (Optional) Inter-frequency RRM measurement (Pinter); [60] (measurement only per freq. layer; Pinter, meas-only) [150] (neighbor cell search power per freq. layer; Pinter, search-only) Micro sleep power assumed for switch in/out a freq. layer. Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}.. Agreement Adopt the following power consumption model for UL SRS for positioning transmission. Power State; Relative power. SRS; 210 (baseline); 700 (optional). Agreement - In Rel-18 low power and high accuracy positioning, adopt the following requirement: - Horizontal positioning accuracy < 1 m for 90% of UEs - Positioning interval / duty cycle of 15-30 s - UE battery life of 6 months – 1 year - Note: Setting an exact value each from the set of positioning interval / duty cycle and UE battery life in the evaluation and identification of performance gap will be discussed separately when necessary. Conclusion - At least when the positioning accuracy is evaluated without jointly evaluating the associated power consumption, the target horizontal positioning accuracy requirement on LPHAP of <1m can be achieved by Rel-16/17 positioning techniques with a positioning bandwidth of at least 100MHz. - The main aspect of RAN1 evaluation is on power consumption. - Note: This does not preclude the case that the positioning accuracy can be revisited, if found necessary at later stage. Agreement - Study further at least the following models and parameter values of conversion between the relative power unit and the battery life to identify the performance gap: - Alt. 1: battery life is used as the metric to identify the gap - Example: T2= P1*T1X* C2C1* 1P2 GapBatLife= T2req-T2 - Alt. 2: relative power unit is adopted as the metric to identify the gap - Example: P2req= P1*T1X* C2C1* 1T2req GapPowUnit= P2req-P2 in which - C1 is the battery capacity of the reference device; - T1 is the battery life of the reference device; - P1 is the relative power unit obtained based on the reference traffic type; - X is the percentage of the power consumed by the reference traffic type; - C2 is the battery capacity of the LPHAP device; - P2 is the evaluated relative power unit of the LPHAP device; - P2_req is the target relative power unit of the LPHAP device; - T2_req is the target battery life of the LPHAP device - Examples of these parameters are provided as follows: C1; T1; X; reference traffic type; C2; T2req. [4500] mAh; [10] hours; [20] %; [FTP (model 3)]; [800] mAh; [12] months. Agreement Adopt the following periodicity of DL PRS / UL SRS for positioning in the baseline evaluation of Rel-17 RRC_INACTIVE positioning: - 1 DL PRS / UL SRS for positioning occasion per N I-DRX cycle(s); - Candidate values of N to evaluate is 1 and 8 for I-DRX cycle of 1.28s; - Note: Individual company may consider either one or both in the evaluation. - Candidate value of N to evaluate is 1 for I-DRX cycle of 10.24s. Agreement - The I-DRX configuration is included in the baseline evaluation of Rel-17 RRC_INACTVIE positioning. - Note: This does not preclude the case where no I-DRX cycle nor paging is considered in the evaluation of potential solutions to maximize the battery life. - Adopt the following I-DRX cycle to evaluate: - 1.28s (baseline); 10.24s (optional). Agreement - Adopt the power consumption model, additional transition energy and total transition time of the three sleep types (deep sleep, light sleep, and micro sleep) in TR38.840 as the evaluation baseline: - FFS: whether/how an additional new ultra-deep sleep mode can be considered in the evaluation of potential solutions to maximize the battery life, including the determination of the relative power, additional transition energy and total transition time, if necessary. Agreement - Adopt the following reference configuration and assumption for DL PRS to define the power consumption model for DL PRS measurement: - 1 Number of PFL; - 8 DL PRS resources per slot are measured; - DL PRS instance of smaller than or equal to 1 slot duration; - Adopt the following table as the power consumption model for DL PRS measurement (derived from Table 22 in TR38.840): N: Number of TRPs for DL PRS measurement; Synchronous case (baseline); Asynchronous case (optional). FR1 (baseline); FR2 (optional); FR1; FR2. N=4 (baseline); 120; 195; 140; 255. N=8 (optional); 150; 225; 170; 285. Agreement - For DL positioning, at least the following power components and parameter values are considered for the baseline evaluation of Rel-17 RRC_INACTIVE positioning: - For the UE-assisted DL positioning, - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%; - DL PRS measurement with 0.5 ms duration; - CG-SDT with 1ms duration and the periodicity of positioning interval; - RRCRelsease after the CG-SDT can be optionally included with [1] ms duration; - (Optional) BWP switching with [1] ms duration; - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration; - (Optional) RA-SDT (e.g., including CORSET0 + SIB1, PRACH, RAR, Msg 3/4/5) in case of CG-SDT is unavailable; - For the UE-based DL positioning, - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%; - DL PRS measurement with 0.5 ms duration; - (Optional) BWP switching with [1] ms duration; - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration; - Note: The power component and parameter values for UE-assisted DL positioning is also applicable to the DL part of UE-assisted DL+UL positioning method. - Note: Individual company may consider additional power components and different parameter values in bracket in the evaluation. - Note: Companies are encouraged to provide the assumption on the timeline between different power consumption events in the evaluation of potential enhancements to reduce the transition times between different power states and to extend the sleeping time as much as possible. Agreement - For UL positioning, at least the following power components and parameter values are considered for the baseline evaluation of Rel-17 RRC_INACTIVE positioning: - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%; - UL SRS for positioning transmission with 0.5 ms duration; - (Optional) BWP switching with [1] ms duration; - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration; - Note: The power component and parameter values for UL positioning is also applicable to the UL part of UE-assisted DL+UL positioning method. - Note: Individual company may consider additional power components and different parameter values in bracket in the evaluation. - Note: Companies are encouraged to provide the assumption on the timeline between different power consumption events in the evaluation of potential enhancements to reduce the transition times between different power states and to extend the sleeping time as much as possible.

In this contribution, we share the view on low power high accuracy positioning.

Discussions

Positioning in idle state

Considering the positioning with low power consumption, potential solution is the measurement in RRC_IDLE state. To reduce power consumption in UE side, the periodic transmission by UE side should be reduced as much as possible. In DL positioning, such periodic transmission is not required. Therefore, at least the DL positioning in RRC-IDLE state should be studied. The PRS can be configured in RRC_INACTIVE state with SDT but it cannot be used for RRC_IDLE state. Therefore, some enhancements for RRC_IDLE state should be considered.

Proposal: For LPHAP, the DL positioning in RRC_IDLE state should be studied.

For UL positioning, one of the potential solutions to support the positioning in RRC_IDLE state is to use PRACH for the measurement. From the network perspective, the operation in RRC_IDLE state is more effective than RRC_INACTIVE state in terms of use of logical resources (e.g., C-RNTI). On the other hand, from the UE perspective, the difference of power consumption due to the periodic UL transmission in RRC_IDLE state and in RRC_INACTIVE state is not so clear. Therefore, when studying PRACH-based UL positioning in RRC_IDLE state, the difference of power consumption between the positioning in RRC_INACTIVE state and the positioning in RRC_IDLE should be the metric.

Observation: When studying PRACH-based UL positioning in RRC_IDLE state, the difference of power consumption between the positioning in RRC_INACTIVE state and the positioning in RRC_IDLE should be the metric.

Conclusion

In this contribution, we have the following observation and proposal:

Proposal: For LPHAP, the DL positioning in RRC_IDLE state should be studied.

Observation: When studying PRACH-based UL positioning in RRC_IDLE state, the difference of power consumption between the positioning in RRC_INACTIVE state and the positioning in RRC_IDLE should be the metric.

References

”RAN1 Chairman’s Notes,” RAN1 #109-e, May 2022.