3GPP TSG RAN WG1 #110			R1-2207316

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	8.16.2

Source:                Apple Inc.

Title:                     On Rel-17 NR coverage enhancement UE Features

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our analysis on UE features for NR coverage enhancement. The discussion is based on  the upated RAN1 UE features list for Rel-17 NR in [1].

# Discussion

From last RAN1 meeting, one left issue for coverage enhancement UE feature is the report granularity of DMRS bundling sub-feature. According to RAN4 incoming LS on DMRS bundling [2], RAN4 agreed the following cases for CA/DC with DMRS bundling.

- FR1+FR2 UL CA, FR1+FR2 DC, and EN-DC with NR on FR2. DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time.

- FR1 inter-band DL CA with a “single” uplink band configured, meaning no switching to transmit SRS on another carrier.

In addition, RAN4 asked any RAN1 specification impacts if the below cases are to be supported.

- FR1 inter-band UL CA with DMRS bundling

- SUL with DMR bundling

- DL CA with SRS carrier switching

RAN4 only agreed CA/DC inter-operation with DMRS bundling in only one band. With this agreement, per band reporting the capability is enough.

On supporting UL CA with DMRS bundling in FR1, no conclusion was reached in RAN4. If a UE is configured with inter-band UL CA, normally one band is lower frequency band for coverage and another band is higher frequency band for capacity. If UE already has the coverage issue in lower band, then the quality of another link in higher band is poor. From this point, the first interpretation makes sense. And there is no motivation to configured two bands with DMRS bundling, and no motivation to configure UL CA for a coverage limited UE.

Proposal 1: The report type of FGs 30-4a/b/c/d/e/f/g/h is per band.

Regarding DMRS bundling feature differentiation, there is no clear difference between FR1 and FR2, TDD and FDD from implementation perspective. Thus, no differentiation is needed.

Proposal2: No need to differentiate FDD/TDD and FR1/FR2 for FGs30-4a/b/c/d/e/f/g/h.

# Summary

In this contribution, we discuss the UL coverage enhancement related UE features and give the following proposals.

Proposal 1: The report type of FGs 30-4a/b/c/d/e/f/g/h is per band.

Proposal2: No need to differentiate FDD/TDD and FR1/FR2 for FGs30-4a/b/c/d/e/f/g/h.

# References

- R1-2205607, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #109-e Week2”, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2205715, “Reply LS to RAN1/RAN2 on DMRS bundling”, RAN4