3GPP TSG-RAN WG1 #110							            R1-2207370

Toulouse, France, August 22nd-26th, 2022

Agenda Item:	9.2.2.2

Source:	AT&T

Title:	Sub-use cases for AI/ML feedback enhancements

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. One of the identified use cases include:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

For each of the use cases, one of the objectives is to

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In RAN#109e [2], the following agreements were reached.

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

  - Note: Study of other sub use cases is not precluded.

  - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

Conclusion

- Further discuss temporal-spatial-frequency domain CSI compression using two-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

- Further discuss improving the CSI accuracy based on traditional codebook design using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

- Further discuss CSI prediction using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss CSI-RS configuration and overhead reduction as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss resource allocation and scheduling as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss joint CSI prediction and compression as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

This contribution discusses and proposes CSI enhancement sub-use cases that focus on CSI prediction, and resource allocation and scheduling.

# AI/ML based CSI prediction and Scheduling Optimization

Cellular networks optimization is an increasingly complex operation. The aggregate number of parameters available for optimization at various layers e.g. MIMO beamforming, power levels, coding and modulation rates, resource allocation to users, etc.  is increasingly large, and the choice of these parameters depends on the channel states of the users. Centralized optimization becomes even more non-tractable, and decentralized approaches need accurate channel state information for each transmission. This can be especially important in the context of multi-user MIMO (MU-MIMO) user selection/pairing and resource allocation.

When scheduling users for MU-MIMO operation, a channel state dependent optimization problem needs to be solved at each time slot, where a subset of users need to be selected, and transmission rates and power levels need to be jointly determined for each of these users from a set of constrained allowable parameters. The optimization problem at the network usually relies on SU CSI feedback from the scheduled users.  Furthermore, this limited channel state information can be potentially out-of-date when used at the DU for optimizing resource allocation.

Observation 1: An AI/ML based CSI prediction module is necessary to predict and improve the CSI feedback.

Another aspect to consider is that this optimization problem is dependent on the deployment scenario (indoor, UMi, UMa, etc.), and can vary significantly for different loading/use case scenarios; despite this scenario dependency, hand-tuned policies are often chosen at the network for good average performance across many scenarios.

This motivates a decentralized online AI/ML approach to improve the user selection and resource allocation decisions. The AI/ML approach is a decentralized, auto-tuning online learning at the network, that exploits the channel-state feedback to reduce the dimensionality of the optimization problem, resulting in a user or channel cluster dependent rate regions that can be used for user and rate selection, consequently reducing the complexity of user selection and scheduling, while improving the probability of successful transmissions, and average throughput of MU-MIMO transmissions.

Observation 2: An auto-tuning AI/ML scheduling algorithm can use deployment-specific CSI feedback adjustments to optimize user selection and scheduling based on the local environment.

Furthermore, based on the feedback from AI/ML online learner, referred to as CSI policy, the UE can adjust the periodicity and/or content of its CSI feedback. This can result in CSI overhead reduction while improving the overall performance of MU-MIMO scheduling.

Observation 3: An AI/ML CSI enhancement algorithm can be used to provide CSI policy feedback to UE to reduce CSI overhead.

Based on the above observations, we propose a CSI enhancements sub-use case for resource allocation and scheduling, where the CSI enhancements are used by the machine learning training and inference at the network, to considerably improve the average throughput and probability of success of MU-MIMO transmissions.  The proposed sub-use case is a one-sided model, whereas both the training and inference are done at the network.

Proposal 1: Include resource allocation and scheduling as a sub-use case for CSI feedback enhancement.

The proposed AI/ML online learning at the gNB comprises, in addition to the AI/ML resource allocation and scheduling block, a CSI prediction block, to further improve the channel state information at the gNB, for the MU-MIMO transmissions. This CSI prediction block can be combined with the resource allocation and scheduling block to improve the throughput and probability of success of MU-MIMO.

Proposal 2: Include CSI prediction as a sub-use case for CSI feedback enhancements in combination with resource allocation and scheduling to further improve MU-MIMO scheduling optimization.

Figure 1 shows an example of a CSI feedback enhancements sub-use case where the training and inference are both done at the network side and the policies inferred, and actions requested are sent to the UE. The UEs sends corresponding channel state information to the network, to be used in training and inference, for CSI prediction and resource allocation and scheduling.

gNB

CSI

UE

AI/ML CSI enhancement

MCS

AI/ML scheduler

Resource allocation

CSI predictor

CSI policy

updates

CSI

UE

MCS

Figure 1 Sub use case: CSI feedback enhancements for improved performance of MU-MIMO transmissions.

Figure 2: Average throughput performance comparison for a multi cell scenario with 4 users

Figure 3: Probability of successful transmission performance comparison for a multi cell scenario with 4 users

To illustrate the gains that can be achieved by an auto-tuning AI/ML scheduling algorithm for MU-MIMO scheduling, we provide preliminary results based on a multi cell scenario with 4 users served with full buffer traffic. Figures 2 and 3 show, respectively, the average throughput and the probability of successful transmission achieved by the AI/ML scheduling, as compared to the hand-tuning baseline scheduler. Figure 2 shows that over 150% gains in system throughput can potentially be achieved using an AI/ML online learning algorithm. The probability of successful transmissions in this case are on average increased by 10% (to the required value of above 90%.)

# Conclusion

In this contribution, we discussed a sub-use case on CSI feedback enhancements related to MU-MIMO scheduling optimization. We made the following observations and proposals.

Observation 1: An AI/ML based CSI prediction module is necessary to predict and improve the CSI feedback.

Observation 2: An auto-tuning AI/ML scheduling algorithm can use deployment-specific CSI feedback adjustments to optimize user selection and scheduling based on the local environment.

Observation 3: An AI/ML CSI enhancement algorithm is required to provide CSI policy feedback to UE to reduce CSI overhead.

Proposal 1: Include resource allocation and scheduling as a sub-use case for CSI feedback enhancement.

Proposal 2: Include CSI prediction as a sub-use case for CSI feedback enhancements in combination with resource allocation and scheduling to further improve MU-MIMO scheduling optimization.

# Reference

[1] RP-213599 Study on AI/ML for Air-Interface, RAN#94e.

[2] RAN1#109-e Chair’s Notes, e-meeting May 9-20th, 2022