3GPP TSG RAN WG1 #110			R1-2207403

Toulouse, France, August 22nd – 26th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on evaluation on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in terms of the evaluation on use cases as following.

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution, we discuss the evaluation on AI/ML for beam management.

- Discussion on the evaluation on AI/ML for beam management

At the RAN1#109-e meeting, the agreement supporting spatial-domain beam prediction and temporal beam prediction for characterization and baseline performance evaluations was made as following [2].

Agreement

For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

- FFS: details of BM-Case1 and BM-Case2

- FFS: other sub use cases

Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

Agreement

For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

- FFS: details of BM-Case1 and BM-Case2

- FFS: other sub use cases

Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

For both spatial-domain and temporal-domain beam prediction, the agreement was achieved regarding the general simulation approach for dataset construction and performance evaluation, as well as the evaluation metric such as the complexity of AI/ML model and the performance comparison between AI-based method and baseline. The simulation results in this contribution were conducted based on the agreed assumptions. In the subsequent sections, we discuss the evaluation methodology and simulation results of spatial-domain DL beam prediction and temporal DL beam prediction. It could be observed that AI/ML-based beam prediction improves the performance of beam management in terms of overhead, latency, and accuracy.

  - Spatial-domain beam prediction

Conclusion

For the sub use case BM-Case1, consider the following alternatives for further study:

- Alt.1: Set B is a subset of Set A

  - FFS: the number of beams in Set A and B

  - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

  - FFS: the number of beams in Set A and B

  - FFS: QCL relation between beams in Set A and beams in Set B

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The narrow and wide beam terminology is for SI discussion only and have no specification impact

- Note3: The codebook constructions of Set A and Set B can be clarified by the companies.

Conclusion

For the sub use case BM-Case1, consider the following alternatives for further study:

- Alt.1: Set B is a subset of Set A

  - FFS: the number of beams in Set A and B

  - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

  - FFS: the number of beams in Set A and B

  - FFS: QCL relation between beams in Set A and beams in Set B

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The narrow and wide beam terminology is for SI discussion only and have no specification impact

- Note3: The codebook constructions of Set A and Set B can be clarified by the companies.

Spatial domain beam prediction can be categorized into two scenarios, (Alt.1) fine beam prediction based on beam measurements with sampled beams and (Alt.2) narrow beam prediction based on beam measurements with wide beams as the above conclusion [2]. In this contribution, we focus on the Alt.1 for the implementation simplicity, because it does not require UE or gNB to support beamforming with different beam widths.

    - Evaluation methodology

At the RAN1#109-e meeting, the following agreement related to performance KPI for AI/ML in beam management was made [2].

Agreement

- To evaluate the performance of AI/ML in beam management, further study the following KPI options:

  - Beam prediction accuracy related KPIs, may include the following options:

    - Average L1-RSRP difference of Top-1 predicted beam

    - Beam prediction accuracy (%) for Top-1 and/or Top-K beams, FFS the definition:

      - Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

      - Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

    - CDF of L1-RSRP difference for Top-1 predicted beam

    - Beam prediction accuracy (%) with 1dB margin for Top-1 beam

      - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”

    - the definition of L1-RSRP difference of Top-1 predicted beam:

      - the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

    - Other beam prediction accuracy related KPIs are not precluded and can be reported by companies.

  - System performance related KPIs, may include the following options:

    - UE throughput: CDF of UE throughput, avg. and 5%ile UE throughput

    - RS overhead reduction at least for spatial-domain beam prediction at least for top-1 beam:

      - 1-N/M,

        - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement

        - where (FFS) M is the total number of beams

        - Note: Non-AI/ML approach based on the measurement of these M beams may be used as a baseline

      - FFS on whether to define a proper value for M for evaluation.

    - Other System performance related KPIs are not precluded and can be reported by companies.

  - Other KPIs are not precluded and can be reported by companies, for example:

    - Reporting overhead reduction: (FFS) The number of UCI report and UCI payload size, for temporal /spatial prediction

    - Latency reduction:

      - (FFS) (1 – [Total transmission time of N beams] / [Total transmission time of M beams])

        - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) in the input beam set required for measurement

        - where M is the total number of beams

    - Power consumption reduction: FFS on details

Agreement

- To evaluate the performance of AI/ML in beam management, further study the following KPI options:

  - Beam prediction accuracy related KPIs, may include the following options:

    - Average L1-RSRP difference of Top-1 predicted beam

    - Beam prediction accuracy (%) for Top-1 and/or Top-K beams, FFS the definition:

      - Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

      - Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

    - CDF of L1-RSRP difference for Top-1 predicted beam

    - Beam prediction accuracy (%) with 1dB margin for Top-1 beam

      - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”

    - the definition of L1-RSRP difference of Top-1 predicted beam:

      - the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

    - Other beam prediction accuracy related KPIs are not precluded and can be reported by companies.

  - System performance related KPIs, may include the following options:

    - UE throughput: CDF of UE throughput, avg. and 5%ile UE throughput

    - RS overhead reduction at least for spatial-domain beam prediction at least for top-1 beam:

      - 1-N/M,

        - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement

        - where (FFS) M is the total number of beams

        - Note: Non-AI/ML approach based on the measurement of these M beams may be used as a baseline

      - FFS on whether to define a proper value for M for evaluation.

    - Other System performance related KPIs are not precluded and can be reported by companies.

  - Other KPIs are not precluded and can be reported by companies, for example:

    - Reporting overhead reduction: (FFS) The number of UCI report and UCI payload size, for temporal /spatial prediction

    - Latency reduction:

      - (FFS) (1 – [Total transmission time of N beams] / [Total transmission time of M beams])

        - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) in the input beam set required for measurement

        - where M is the total number of beams

    - Power consumption reduction: FFS on details

RS overhead reduction was agreed as one of system performance related KPIs, Obviously, when the number of beams in Set B (N) is lower than Set A (M), the RS overhead could be reduced since UE is not required to measure all the RS resources in Set A. In case of top-K beam prediction, top-K beams in Set A could be inferred from measurement on Set B. In this optimal case, the RS overhead would be 1-N/M as captured in the above agreement.

However, if the top-1/K predicted beam(s) from Set A are not included in Set B, UE might be required to measure the beam quality with top-1/K predicted beam(s) from Set A. Fig.1 illustrates the additional beam measurement with top-1 predicted beam based on beam measurements of Set B. As shown in Fig.1, UE might have to perform the beam measurement with the predicted top-1/K beam(s) so that UE is ready to receive PDCCH/PDSCH with the corresponding top-1/K beam(s), when top-1/K predicted beam(s) from Set A is not measured in Set B. One of potential reasons is the known condition of TCI-state. In the current specification, L1-RSRP reporting is necessary before activating TCI-state with the corresponding QCL source RS for PDSCH/PDCCH. In AI/ML-based beam measurements, only the predicted RSRP(s) of top-1/K predicted beam(s) are available instead of actual RSRP(s) when the predicted top-1/K beam(s) are not included in Set B. If the predicted L1-RSRP(s) are sufficient to satisfy the known condition of TCI-state, the additional beam measurements can be ignored. Otherwise, it is necessary to consider the additional beam measurements in AI/ML-based beam management. In that case, the additional beam measurements should be taken into consideration even for performance KPI discussion to avoid the over-estimation for the performance of AI/ML-based beam managements. For example, the following equation can be considered as KPI for RS overhead reduction.

OH%=1-(# of beams in Set B)+(# of TopK beams not covered by Set B)(# of beams in Set A)

Observation 1: Additional beam measurements might be necessary for PDSCH/PDCCH reception with top1/K predicted beam(s), when the top-1/K predicted beam(s) are not included in beams measured for the beam prediction.

Proposal 1: Discuss whether additional RS overhead should be considered, when the top-1/K predicted beam(s) are not included in beams measured for the beam prediction.

Figure 1. Additional beam measurements with the top-1 predicted beam for reception with the beam, when the top-1 predicted beam is not included in beam measurements for the beam prediction.

Based on the discussion above, the simulation assumptions for spatial-domain beam prediction are as follows:

- EVM for dataset generation:

  - SLS approach is adopted with detailed parameters and assumptions listed in Table 1.

  - Set A consists of fine beams and Set B consists of sparse beams (subset of Set A) as shown in Figure 2.

    - Set A: M beam pairs assumed to be mapped to fine Tx beams×UE Rx beams.

    - Set B: N beam pairs measured based on uniformly distributed sparse Tx beams×UE Rx beams within Set A.

      - Fixed N beam pairs are assumed during the simulation.

- KPI (Evaluation metric):

  - Performance gain of beam prediction accuracy:

    - Average L1-RSRP difference of top-1 predicted beam.

    - Beam prediction accuracy (%) for top-1 beam:

    - Beam prediction accuracy (%) for top-5 beams:

      - Option 1: The beam prediction accuracy (%) is the percentage of “the top-1 predicted beam is one of the top-5 genie-aided beams”.

      - Option 2: The beam prediction accuracy (%) is the percentage of “the top-1 genie-aided beam is one of the top-5 predicted beams”.

    - Beam prediction accuracy (%) with 1dB margin for Top-1 beam

      - The beam prediction accuracy (%) with 1dB margin is the percentage of the top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the top-1 genie-aided beam”

    - RS overhead reduction: The equation described as above.

  - Complexity of AI/ML model:

    - FLOPs: floating point operations per second.

    - Model size: the amount of occupied RAM when AI/ML model is loaded for inference with batch size = 1.

    - Parameter size: the number of parameters of AI/ML model.

- Baseline: Both options proposed at the last meeting were adopted

  - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping).

  - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams.

    - The Top-K beam(s) within Set B is assumed to be the top-K beam(s) within Set A.

Table 1. Simulation parameters for dataset generation

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel mode | Uma with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | 3km/h |
| UE distribution | 80% indoor ,20% outdoor as in TR 38.901 |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline TxRU=2, 64 beams are assumed(H(16)*V(4)) TxRU mapping rule: TR 36.897 section 5.2.2 |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Panel structure: (M,N,P) = (1,4,2), 2 panels (left, right) with (Mg, Ng) = (1, 2) TxRU=2, 4 beams are assumed per panel (H(4)) according to ITU evaluation assumption; TxRU mapping rule: TR 36.897 section 5.2.2 |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | No beam correspondence is assumed (Only DL transmission is simulated.) |
| BS Tx Power | 40 dBm |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

Figure 2. Set B and Set A for spatial-domain beam prediction

In order to align the simulation assumptions for better calibration, we also provide some details of AI/ML model in Table 2. Although the detailed design of AI/ML model is outside the scope of 3GPP, the information of AI/ML model could provide some reference information to better understand and compare the simulation results.

Table 2. Information of AI/ML model

| Parameter | Value |
| --- | --- |
| Model | FNN |
| Input | L1-RSRP of Set A beam-pairs |
| Output | L1-RSRP of Set B beam-pairs |
| Training | 320K samples |
| Testing | 80K samples |

    - Performance evaluation results

In this section, the simulation results of the spatial domain beam prediction are provided. In order to observe the benefit of AI/ML model well, we adopted two sets of configurations on the value of N and M.

As shown in Table 3, it is observed that AI-based beam prediction could reduce the average L1-RSRP difference of Top-1 predicted beam from ~4dB to ~1dB when the RS overhead reduction is 0.74. Even if the RS overhead reduction is increased to 0.97, the average L1-RSRP difference of Top-1 predicted beam could still be largely reduced from ~11dB to ~3dB. In addition, all the percentage values of beam prediction accuracy are improved notably by AI/ML model.

Table 3. Performance of beam prediction accuracy

| KPI |  | Baseline O1 | N=6×4, M=12×8 |  | N=8×2, M=64×8 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Baseline O2 | AI model | Baseline O2 | AI model |
| Average L1-RSRP difference of Top-1 predicted beam(dB) |  | 0 | 3.93 | 0.92 | 10.89 | 3.26 |
| Beam prediction accuracy (%) for Top-1 beams |  | 100 | 23.7 | 60.4 | 1.3 | 24.9 |
| Beam prediction accuracy (%) for Top-5 beams | Option 1 | 100 | 79.9 | 97.7 | 7.43 | 62.4 |
|  | Option 2 | 100 | 23.7 | 93.9 | 1.3 | 61.6 |
| Beam prediction accuracy (%) with 1dB margin for Top-1 beam |  | 100 | 31.7 | 72.3 | 3.09 | 38.5 |
| RS overhead reduction |  | 0 | 0.74(Top-1) 0.7(Top-5) |  | 0.97(Top-1) 0.96(Top-5) |  |

Observation 2: Spatial domain DL beam prediction could notably improve the beam prediction accuracy.

In the simulation, fixed beam pairs in Set B are assumed for all the samples in the dataset. In practice, it is hard to fix the Tx beams and Rx beams considering different UE capabilities and configurations. Random beam pairs selection would be a solution, but it may sacrifice the performance gain. Therefore, it would be better to investigate the appropriate way of mapping the input into AI/ML model with enough flexibility while without losing too much performance gain.

Proposal 2: The mapping method of wide/sparse beam as input into AI/ML model should be investigated.

The complexity performance of AI/ML model is also provided in Table 4.

Table 4. AI/ML model complexity

| FLOPs | Memory size | Param. size |
| --- | --- | --- |
| 270K | 700MB | 271K |

  - Temporal beam prediction

Conclusion

For the sub use case BM-Case2, further study the following alternatives with potential down-selection:

- Alt.1: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

  - FFS: QCL relation between beams in Set A and beams in Set B

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

  - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.3: Set A and Set B are the same

- Note1: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B.

- Note2: It is up to companies to provide other alternative(s)

- Note3: The narrow and wide beam terminology is for SI discussion only and have no specification impact

Conclusion

For the sub use case BM-Case2, further study the following alternatives with potential down-selection:

- Alt.1: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

  - FFS: QCL relation between beams in Set A and beams in Set B

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

  - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.3: Set A and Set B are the same

- Note1: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B.

- Note2: It is up to companies to provide other alternative(s)

- Note3: The narrow and wide beam terminology is for SI discussion only and have no specification impact

Temporal beam prediction can be categorized into three scenarios as the above conclusion, (Alt.1) a set of measured beams is a subset of predicted beams, (Alt.2) a set of measured beams and predicted beam are different, and (Alt.3) a set of measured beam and predicted beam are the same. In Alt.1 and Alt.2, the further gain can be expected by exploiting the AI/ML spatial domain beam prediction performance. However, we analyse the performance under Alt.3 in this contribution, so that the performance gain solely brought by temporal beam prediction can be observed well.

    - Evaluation methodology

Reporting overhead reduction is an important KPI to evaluate the performance of temporal beam prediction with NW side model. As the AI model can predict the channel quality based on historical beam measurements, a certain beam tracking ability can be achieved with low frequent beam measurements. As shown in Fig.3, temporal beam prediction with NW side model can compensate for the smaller number of beam measurements than conventional beam management without beam prediction. Likewise, the reporting overhead, such as the number of uplink transmissions for CSI reports and UCI overhead, can be reduced by temporal beam prediction with NW side model. To analyse the performance gain of temporal beam prediction, companies should be encouraged to provide the reporting overhead for the temporal beam prediction with NW side model.

Proposal 3: Encourage companies to provide the reporting overhead, e.g., the number of uplink transmissions for CSI reports and UCI overhead, for the temporal beam prediction with NW side model.

Figure 3. Additional beam measurements with the top-1 predicted beam for reception with the beam, when the top-1 predicted beam is not included in beam measurements for the beam prediction.

Based on the discussion above, the simulation assumptions for temporal beam prediction are as follows:

- EVM for dataset generation:

  - Additional SLS simulation parameters and assumptions on top of those for spatial domain beam prediction are listed in Table 5.

  - Beams for future and historical measurements are shown in Figure 4:

    - Set A is assumed to be mapped to M future time instances within T2 with shorter time intervals (10ms).

    - Set B is assumed to be mapped to N historical time instances within T1 with long time intervals (960ms).

    - Set A and Set B are the same beam sets which consist of 12 Tx beams and 8Rx beams.

- KPI (Evaluation metric): the same KPIs as spatial domain beam prediction are adopted.

- Baseline: both options proposed in the last meeting were adopted

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2.

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1.

    - The best beam of the last time instant within T1 is selected as the best beams within T2.

    - Set A and Set B are the same.

Table 5. Additional simulation parameters for dataset generation

| Parameters | Values |
| --- | --- |
| UE speed | 30Km/h, 60Km/h, 90Km/h |
| UE distribution | 100% outdoor |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory | Random direction straight-line trajectories (Option 4) |
| UE orientation | Randomly per-UE chosen for UE orientation initially and is fixed |
| UE rotation speed | 0 |

Figure 4. Future beams and historical beams for temporal beam prediction

Due to the same reason discussed in Section 2.1.1, some details of AI/ML model are provided in Table 5.

Table 5. Information of AI/ML model

| Parameter | Value |
| --- | --- |
| Model | LSTM |
| Input | L1-RSRP of Set A beam-pairs during T1 |
| Output | L1-RSRP of Set B beam-pairs during T2 |
| Training | 180K samples |
| Testing | 50K samples |

    - Performance evaluation results

In this section, the performance of temporal beam prediction is provided. In order to show the benefit of AI-based method well, three UE speed values, i.e. 30km/h, 60km/h, 90km/h were assumed in the simulation.

Firstly, it is observed that the results of baseline Option 2 under 30km/h are already nice enough even if the measurement report periodicity is almost 1s. The reason could be the best beam of the last time instance within T1 almost does not change during T2 when the UE speed is low. Therefore, the high performance can be achieved even in the conventional method by using the identical best beam as the last time instance in T1.

Observation 3: For the UE speed 30km/h, baseline performance is nice enough even if 960ms reporting periodicity is assumed.

However, when the UE speed increases to 60km/h or 90km/h, the performance of baseline Option 2 deteriorates. The average L1-RSRP difference of top-1 predicted beam increases from ~1dB to ~3dB or ~5dB. However, if AI-based beam prediction is used, the similar performance to low UE speed, i.e. ~1dB, could be achieved even if the UE speed increases. The same tendency could also be observed from the percentage values of beam prediction accuracy.

Observation 4: For the UE speed 60km/h and higher, the AI/ML model could provide notable improvement in prediction accuracy.

Proposal 4: UE speed 60km/h or higher should be considered as the baseline assumption for performance evaluation.

Table 6. Performance of beam prediction accuracy

| KPI |  | Baseline O1a | 30Km/h | 60Km/h |  | 90Km/h |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Baseline O2 | Baseline O2 | AI/ML | Baseline O2 | AI/ML |
| Average L1-RSRP difference of Top-1 predicted beam(dB) |  | 0 | 1.10 | 2.72 | 0.77 | 5.16 | 0.90 |
| Beam prediction accuracy (%) for Top-1 beams |  | 100 | 80.7 | 67.4 | 79.0 | 54.0 | 77.7 |
| Beam prediction accuracy (%) for Top-5 beams | Option 1 | 100 | 96.0 | 89.9 | 97.7 | 79.3 | 96.5 |
|  | Option 2 | 100 | 95.9 | 89.4 | 96.6 | 77.2 | 96.3 |
| Beam prediction accuracy (%) with 1dB margin for Top-1 beam |  | 100 | 85.4 | 71.5 | 84.0 | 57.6 | 82.5 |
| Reporting overhead (# of uplink transmission during T1) |  | 96*6 | 6 |  |  |  |  |

The complexity performance of AI/ML model is also provided in Table 7.

Table 7. AI/ML model complexity

| FLOPs | Memory size | Param. size |
| --- | --- | --- |
| 90K | 700MB | 6.8K |

- Conclusion

In this contribution, we discussed evaluation on AI/ML for beam management. Based on the discussion we made the following observations and proposals.

Observation 1: Additional beam measurements might be necessary for PDSCH/PDCCH reception with top1/K predicted beams, when the top-1/K predicted beams are not included in beams measured for the beam prediction.

Observation 2: Spatial domain DL beam prediction could notably improve the beam prediction accuracy.

Observation 3: For the UE speed 30km/h, baseline performance is nice enough even if 960ms reporting periodicity is assumed.

Observation 4: For the UE speed 60km/h and higher, the AI/ML model could provide notable improvement in prediction accuracy.

Proposal 1: Discuss whether additional RS overhead should be considered, when the top-1/K predicted beams are not included in beams measured for the beam prediction.

Proposal 2: The mapping between wide/sparse beam as input into AI/ML model should be investigated.

Proposal 3: Encourage companies to provide the reporting overhead, e.g., the number of uplink transmissions for CSI reports and UCI overhead, for the temporal beam prediction with NW side model.

Proposal 4: UE speed 60km/h or higher should be considered as the baseline for performance evaluation.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- Moderator (OPPO), R1-2205454, “Discussion summary#4 for other aspects on AI/ML for beam management”, May. 2022.

- Moderator (Samsung), R1-2205641, “Feature lead summary #4 evaluation of AI/ML for beam management”, May. 2022.