3GPP TSG RAN WG1 Meeting #110 	R1-2207637

Toulouse, France, August 22 – 26, 2022

Agenda Item:	8.17

Source:	Huawei, HiSilicon

Title:	Remaining issues on paging monitoring and measurement during SDT procedure

Document for:	Discussion and Decision

# Introduction

This paper discusses remaining issues on paging monitoring and measurement during SDT procedure.

# Discussion

## 2.1 Paging/SI change indication monitoring

In last RAN1 meeting, it has been agreed that SDT resources can be configured in the separate initial BWP for RedCap UEs in the SDT topic.

RAN1#108-e Agreement for SDT[1] RAN1 confirms that the separate BWP in case of RedCap may still be considered as the initial BWP and SDT resources(both CG-SDT and RA-SDT) can hence be configured on this BWP for RedCap UEs. - Note: details can be further studied to ensure proper functionality of RedCap UE performing SDT.

According to the former agreements in the SDT topic, UEs should monitor SI change and PWS notification during SDT procedure, which means UEs have to monitor paging during SDT procedure.

RAN2#116b-e Agreement for SDT[2] 9. During the SDT procedure (i.e. while SDT timer is running), UE monitors SI change indication in any paging occasion at least once per modification period (i.e. same as legacy RRC_CONNECTED). 10. During the SDT procedure (i.e. while SDT timer is running), ETWS or CMAS capable Ues monitors PWS notification in any paging occasion at least once every defaultPagingCycle (i.e. same as legacy RRC_CONNECTED).

RAN2#116b-e Agreement for RedCap[2] A RedCap UE in idle/inactive mode monitors paging only in an initial BWP (default or RedCap specific) associated with CD-SSB and performs cell (re-)selection and measurements on the CD-SSB RAN1#108-e Agreement for RedCap[1] - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective

There are some conflict between agreements made in RedCap and SDT. Based on agreements in RedCap, in inactive mode, RedCap UEs do not monitor paging in the separate initial BWP which does not contain CD-SSB, as paging monitoring is only in the BWP associated with CD-SSB. While based on agreements in SDT, if gNB configure SDT resources for RedCap UEs in the separate initial BWP, the UEs should monitor paging in the separate initial DL BWP.

In order to resolve this, following options can be considered:

- Not monitoring Paging/SI update indication in this case

As SDT transmission contains not only initial transmission, but also subsequent data transmission, the procedure may last for several seconds. Then RedCap UEs may miss the important public safety message if Paging/SI update indication is not monitored.

Observation 1: A UE should attempt to monitor paging/SI update indication during SDT procedure, including RedCap UE in separate initial BWP.

- Up to network scheduling

It is agreed that UE can monitor paging/SI update indication in any paging occasion at least once per modification period. During subsequent data transmission, the UE can be scheduled by PDCCH in a CSS/USS. Thus, to avoid frequency resource waste and transmission failure in this approach, the gNB has to ensure the data transmissions in the separate initial DL BWP and every paging occasion in CORESET#0 are not overlapped in time domain, as shown in the figure 1, where the data transmissions include PDCCH monitor occasion, the scheduled PDSCH/PUSCH and CG transmissions for CG-SDT.

Furthermore, if no SSB is contained in the separate initial DL BWP, the UE may have to do measurement for synchronization before monitoring paging, and the measurement may be based on several SSB (with 20ms typical periodicity) for large coverage case, which turns out to be another issue for gNB scheduling.

Moreover, the forth and back retuning/switching time should also be considered.

Consequently, the total possible interruption time include potential SSB used for measurement, several paging monitoring occasions since RAN2 consider the UE can monitor any Paging Occasion, RF retuning time forth and back as well as the gap between SSB and paging messages. The interruption time therefore could last for tens of slots since gNB cannot assume a UE will quickly retune back to the separate initial DL BWP right after SSB measurement prior to paging, nor assume a UE will retune back right after monitoring paging given there could be SI update after the paging in the initial BWP. Significantly and unfortunately, it is unknown to gNB since measurement operation and PO selection is up to UE.

Figure 1: configuration/scheduling constrain for paging in time domain

Observation 2: There will be significant scheduling constrain if paging/SI update indication monitoring is up to UE when separate initial DL BWP is configured for SDT procedure.

## 2.2 CG resource validation

The above describes the issue for paging/SI update, where SSB is considered for paging purpose. Additionally, SSB measurement itself still has a similar issue even without considering paging, given RAN2 agreement below.

RAN2#117-e Agreement for SDT[3] 5. Confirm earlier agreement: During subsequent CG transmission phase (i.e. after the UE has received response from NW), if there is no available SSB above the configured RSRP CG-SDT threshold, the HARQ entity doesn’t use the CG-SDT resource, and the UE triggers SR when there is no valid UL grant (UE falls back to legacy RA for SR) (19/20).

Before transmission on a CG resource, SSB based measurement is applied to verify the resource. As there may be no SSB contained in the separate initial DL BWP, a UE has to retune/switching to CD-SSB for the measurement. So before every CG resource, interruption occurs which could overlap in time with PDCCH monitor occasions or PDSCH/PUSCH transmissions. The interruption time should at least includes retuning/switching time and SSB transmission time. The SSB transmission time occupies 2 or 4 slots for FR1, 32 slots for FR2 according to different deployments.

Figure 2: configuration/scheduling constrain for resource validation in time domain

Observation 3: There will be significant scheduling constrain for SSB measurement in the initial DL BWP for verifying the CG-SDT resources in the separate BWP.

Overall, the issue qualifies a need to specify a clear interruption time for SDT in order to have proper gNB and UE operations. The simplest approach seems to be to allow gNB configure an interruption time for the aforementioned use cases, and the exact time can be up to RAN4.

Proposal 1: If separate initial BWP does not include CD-SSB but configured for SDT, interruption time can be configured for paging/SI update indication monitoring or SSB measurement for CG-SDT resource validation.

# Conclusions

In this contribution, we presented our views on the remaining issues besides bandwidth reduction for RedCap UEs:

Observation 1: A UE should attempt to monitor paging/SI update indication during SDT procedure, including RedCap UE in separate initial BWP.

Observation 2: There will be significant scheduling constrain if paging/SI update indication monitoring is up to UE when separate initial DL BWP is configured for SDT procedure.

Observation 3: There will be significant scheduling constrain for SSB measurement in the initial DL BWP for verifying the CG-SDT resources in the separate BWP.

Proposal 1: If separate initial BWP does not include CD-SSB but configured for SDT, interruption time can be configured for paging/SI update indication monitoring or SSB measurement for CG-SDT resource validation.

# References

- RAN1 Chairman’s Notes, RAN1#108-e, e-Meeting, 21th February–3th March, 2022.

- RAN2 Chairman’s Notes, RAN2#116b-e, e-Meeting, 17th January–25th January, 2022.

- RAN2 Chairman’s Notes, RAN2#117-e, e-Meeting, 21th February–3th March, 2022.