3GPP TSG- RAN WG1 Meeting #110	R1-2207641

# Toulouse, France, August 22-26 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | XXX | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on description for SL CG type 2 PSSCH transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the discussion of [109-e-R16-V2X-01] and relevant CR agreed in R1-2205299, the PUCCH resource for reporting the SL HARQ corresponding to the PSSCH transmission(s) of SL CG Type 2 activated by DCI format 3_0 should be same as that for SL CG PSSCH without corresponding DCI configured by sl-N1PUCCH-AN-Type2. However, when a value of the PUCCH resource indicator field and a value of PSFCH-to-HARQ feedback timing indicator field (if present) is zero, the spec “a PUCCH resource is not provided” is misleading, since the PUCCH resource may be already configured by the RRC parameter. Thus, a more appropriate description is that such a (pre-) configured PUCCH resource is deactived when the values of related fields are set to zero, which implies the PUCCH resource is provided but not used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the wording “a PUCCH resource is not provided” into “a PUCCH resource is deactived”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mislead the understanding that the PUCCH resource for sidelink HARQ-ACK reporting is not provided or not used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

# 16.5	UE procedure for reporting HARQ-ACK on uplink

For a PSSCH transmission by a UE that is scheduled by a DCI format, the DCI format indicates to the UE that a PUCCH resource is not provided when a value of the PUCCH resource indicator field is zero and a value of PSFCH-to-HARQ feedback timing indicator field, if present, is zero. For a SL configured grant Type 2 PSSCH transmission without a corresponding PDCCH, or for a SL configured grant Type 2 PSSCH transmission activated by a DCI format, the DCI format activating the SL configured grant Type 2 indicates to the UE that a PUCCH resource is deactived when a value of the PUCCH resource indicator field is zero and a value of PSFCH-to-HARQ feedback timing indicator field, if present, is zero. For a SL configured grant Type 1 PSSCH transmission, a PUCCH resource can be provided by sl-N1PUCCH-AN and sl-PSFCH-ToPUCCH-CG-Type1. For transmission of HARQ-ACK information corresponding only to a SL configured grant Type 2 PSSCH transmission, including the PSSCH transmission(s) associated with the corresponding activation DCI format 3_0, a UE can be provided a PUCCH resource by sl-N1PUCCH-AN-Type2. If a PUCCH resource is not provided, the UE does not transmit a PUCCH with generated HARQ-ACK information from PSFCH reception occasions.

<Unchanged parts omitted>