3GPP TSG-RAN WG1 Meeting #110	R1-2207664

Toulouse, France, August 22 – 26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | xxx | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the conditions for channel sensing in FR2-2 in TS37.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current conditions for performing the channel access procedures in FR2-2, “When…condition A… or when…condition B…”, implies that satisfying either condition A (gNB/UE(s) is required by regulations to sense a channel(s)) or condition B (a gNB provides UE(s) with higher layer parameters channelAccessMode2-r17) is sufficient for performing the channel access procedures described in that clause. This is however in contrast to the common understanding in the group and to the following descriptions of the higher layer parameter channelAccessMode2-r17 in the latest version of TS 38.331 (v17.1.0). channelAccessMode2 If present ('enabled'), this field indicates that the UE shall apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the first sentence “When a gNB/UE(s) is required by regulations to sense a channel(s) for availability for performing transmission(s) on the channel(s) or” in Clause 4.4 of TS 37.213 v17.2.0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous procedures in Clause 4.4 of TS 37.213 and inconsistency of the specifications with the corresponding descriptions of the higher layer parameter channelAccessMode2-r17 in TS 38.331 v17.1.0. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.4	Channel access procedures for frequency range 2-2

When a gNB provides UE(s) with higher layer parameters channelAccessMode2-r17 by SIB1 or dedicated configuration indicating that the channel access procedures would be performed to sense a channel(s) for availability before transmission(s) on the channel(s), the channel access procedures described in this clause for accessing the channel(s) on which the transmission(s) are performed by the gNB/UE(s), are applied.

*** < Unchanged parts are ommitted> ***