3GPP TSG-RAN WG1 Meeting #110, 	R1-2207722

Toulouse, FR, August 22 – 28, 2022

,

Agenda item:    8.2

Source:              Qualcomm Incorporated

Title:                FL summary for Maintenance of Channel Access Mechanisms for NR in 52.6 to 71GHz band, ver01

Document for:  Discussion and Decision

# Introduction

The document summarizes the discussion papers and draft CRs related to Channel Access for FR2-2.  The following is the list of topics addressed.

- UL Contention Exempt Short Control Signaling

- Channel Access Indication in Fallback DCI and RAR

- LBT upgrade in COT Sharing

- ED Threshold and Bandwidth

- Multi-Beam Channel Access: Independent Per Beam LBT

- Multi-Beam Channel Access: ED Threshold

- COT Resumption after a gap: Rules for channel Access

- Miscellaneous

# UL Contention Exempt Short Control Signaling: Duty Cycle

The issue of enforcing the Duty Cycle Constrained was discussed in Section 5-2 of [1].

Proposal 5-2-1 For Contention Exempt Short Control Signaling based transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS, the 10% over any 100ms intervals restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell. - This 10% allowance is separated from the 10% allowance for gNB

From the CRs submitted to this meeting, we have the following positions

- The 10% over any 100ms interval to support UL SCS is based on all available msg1/msgA resource configured

  - ZTE [R1-2206082]

  - Qualcomm Incorporated [R1-2207181]

- The 10% over any 100ms interval to support UL SCSt is based on the msg1/msgA resource used by a UE

  - OPPO,

  - Vivo  [R1-2206733] (UE implementation)

  - Nokia

## Discussion paper views:

The key proposals on duty cycle constraint for short control signaling from discussion papers are captured below.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| ZTE Sanechips | Proposal 4: Adopt Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell. |
| ZTE Sanechips | Proposal 5: For the case of the transmission of DL/UL channels/signals considered as Short Control Signalling is in a COT initiated by gNB or UE and LBT is performed before Short Control Signalling transmission, it is suggested that such transmission should not be counted into 10ms limitation within the 100ms observation period. |
| vivo | Proposal 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective. |
| vivo | Proposal 3: It is up to UE implementation to transmit msg1 or msgA based on short control signalling or with LBT. |
| Nokia Nokia Shanghai Bell | Observation 1: EN 302 567, v2.2.0 allows for Short Control Signalling transmissions for up to 10% of time within an observation period of 100 ms. |
| Nokia Nokia Shanghai Bell | Proposal 4: There is a separate 10% allowance for the gNB, and another one common for all the UEs in the cell. |
| Nokia Nokia Shanghai Bell | Proposal 8: UEs may assume that if short control signalling is in use in a cell, the network shall not configure more than 10% of all time resources for msg1/msgA. |
| LGE | Proposal #3: When Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and msgA PRACH for the 2-step RACH, the 10% over any 100ms interval restriction is applicable to all available msg1/msgA PRACH resources configured (not limited to the resources actually used) in a cell. |
| LGE | Proposal #4: Whether a short control signaling rule is applicable or not to the configured msg1/msgA PRACH resources can be explicitly indicated by the gNB or implicitly determined by UE by checking duty cycle for the configured ROs (or ROs and POs) resources within the observation period. |

## Draft CRs:

Following is the overview of draft CRs received on this aspect from the companies.

| Company | Reason/Summary | Description |
| --- | --- | --- |
| Vivo [R1-2206733] | Reason for change: | The contraint of short control signalling is not fully included yet. |
|  | Summary of change: | Adding the 10% constraint for short control signaling as the actually transmitted signaling. |

# UL Contention Exempt Short Control Signaling: Enable/disable

The issue of Signaling for Enabling CET for msg1/msgA together was discussed in Section 5-3 of [1].

Proposed conclusion 5-3-2 (2nd round): gNB will not provide an RRC configuration to indicate if msg1 or msgA transmission with Contention Exempt Short Control Signaling based transmission is allowed. - It is left for UE implementation to decide if msg1 or msgA transmission with Contention Exempt Short Control Signaling based transmission is used - It is UE responsibility to comply with local regulation on the usage of Contention Exempt Short Control Signaling for msg1 or msgA transmission Proposal 5-3-3 [1]: - Introduce 1 bit of RRC signaling (SIB1), where: - 0 indicates that LBT is required for UL transmissions (e.g. to cover the Japan case) - For dynamic scheduled UL transmissions, UEs will follow the LBT indicator in the scheduled DCI, i.e., LBT indicator in the scheduled DCI will override the configuration of this SIB 1 bit (e.g., to cover other regions, where LBT is used for msg1/msgA, but no LBT could be used for UL transmissions in shared COT) - 1 indicates that msg1/msgA can be transmitted without LBT - FFS: whether/ in which cases LBT is needed within a COT (without additional RRC impact), including e.g.: - whether this bit could also be used as indication for using Type1/Type 2 (depending on UE capability) or Type 3 LBT for UL transmissions in shared COT - whether this bit could also be used as indication for using Type1/Type 2 (depending on UE capability) or Type 3 LBT when resuming a COT after a gap

From the CRs and discussion papers  submitted to this meeting, we have the following positions

- 1 bit SIB1 indication for LBT for Msg1 and MsgA

  - OPPO [R1-2206294

  - Intel Corporation [R1-2206541]

  - Qualcomm Incorporated [R1-2207181]

  - LGE

  - Nokia Nokia Shanghai Bell [R1-2207018]

- Unified Bit: Region Indication bit to decide contention exemption AND type 2 LBT in a gap:

  - Vivo [R1-2206734]

## Discussion paper views:

The key proposals on signaling to enable contention exempt transmission from discussion papers are captured below.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| OPPO | Observation 1: Using UE implementation to determine the SCS for MsgA and Msg1 may cause UE behavior non-aligned, which is not desired for unlicensed spectrum operation. Proposal 1: RAN1 to agree the draft CR in [1] |
| ZTE Sanechips | Proposal 6: It is proposed to introduce a new RRC parameter to indicate if msg1 or msgA transmission with Contention Exempt Short Control Signaling based transmission is allowed. |
| Intel Corporation | Proposal 1: The gNB indicates through a 1 bits RRC parameter in SIB1 whether the short signal exemption (SSE) should be applied or not. Endorse the draft CR for TS37.213 in R1-2206541. |
| vivo | Proposal 1: Introduce 1 bit of RRC signaling to indicate that whether SCS is allowed and whether Type 2 LBT is mandatory. |
| Nokia Nokia Shanghai Bell | Proposal 5: Agree the proposal 5-3-3B from RAN1#109-e: Introduce 1 bit of RRC signaling (SIB1), where: • 0 indicates that msg1/msgA cannot be transmitted without LBT and LBT (Type 1 or Type 2 LBT depending on UE capability) should be used in shared COT or resuming COT after a gap (e.g., to cover the Japan case) • 1 indicates that msg1/msgA can be transmitted without LBT and Type 3 LBT can be used in shared COT or resuming COT after a gap. |
| LGE | Proposal #5: Introduce separate 2 bits RRC signalling to indicate the SCS applicability of msg1/msgA and which LBT to perform within COT. In other words - 1 bit for msg1/msgA SCS applicability configuration - 0 indicates that msg1/msgA cannot be transmitted without LBT - 1 indicates that msg1/msgA can be transmitted without LBT - 1 bit to indicate Type 2 or Type 3 LBT for COT resuming in UE-initiated COT or LBT upgrade through DCI 2_0 detection in gNB’s COT - 0 indicates that LBT is required before each transmission - Type 2 LBT can be applicable for a gap depending on capability - 1 indicates that LBT is not required before each transmission - Type 3 LBT is used regardless of gap |
| Qualcomm Incorporated | Proposal 2: Adopt the CR in [7] incorporating the RRC parameter for indication of contention exemption for short control signaling, as well as a cell wise duty cycle constraint on the configured resources for first message of RACH procedure. |
| Ericsson | - Proposal 1 RAN1 to introduce a single bit in SIB1 to indicate the use of LBT for all contention-exempt short control signalling and configured UL transmissions. Modified Alt 1A: Introduce one bit in SIB1 indicates whether LBT is required for all contention exempt SCST and CG UL transmissions - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false - UE as initiating device can resume transmission within maximum COT without a type 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device, if the bit is set to false. If the bit is set to true, UE as initiating device can resume transmission after a type 2 LBT if device is capable. Note: In regions where LBT is required before every transmission, when a UE is scheduled by a DCI to transmit a UL transmission(s), the UE does not expect the scheduling DCI to indicate Type 3 channel access, and the bit is set to true. |

## Draft CRs:

Following is the overview of draft CRs received on this aspect from the companies.

| Company | Reason/Summary | Description |
| --- | --- | --- |
| OPPO [R1-2206294] | Reason for change: | Currently it is unclear from the specification text, whether the short control signaling exemption should apply per transmitting device or per cell, and also how to disable this exemption when this is not supported based on regional regulatory requirements. In order to solve this issue, the following CR is proposed. |
|  | Summary of change: | To specify that the short control signaling exemption applies per UE and to indicate to the UE when this exemption should be applied. |
| Intel Corporation [R1-2206541] | Reason for change: | Currently the specification text does not provide any procedure on how to signal to a UE whether the short control signalling exemption may be allowed to be used, given that there are regions where this exemption may not be applicable. Furthremore, it is important to note that a UE may not be able to retrieve this information implicitly by implementation, and such information has to be provided explicitly by the network. |
|  | Summary of change: | Clarify that short control signalling exemption is applied when configured by higher layers. |
| Xiaomi [R1-2206615] | Reason for change: | No LBT was introduced in R17 for FR 2-2. Based on the description of TS 38.331, when parameter channelAccessMode2-r17 is absent, UE shall not apply any channel access procedure.UE will transmit UL channels in no-LBT mode on shared spectrum, and none of the Type 1/2/3 LBT defined for FR 2-2 will be used. Based on the description of TS 38.212, for a cell operate on shared spectrum, the bit length of ChannelAccess-CPext-CAPC field in DCI 0-1/1-1 is bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1/1-1, and I>=1. The current TS 38.212 assumes as long as the cell operate on shared spectrum, ul-AccessConfigListDCI-0-1/1-1 should be configured at least one entry.. However, for FR2-2,even on shared spectrum, when channelAccessMode2-r17 is absent, UE shall not apply any channel access procedure. And ul-AccessConfigListDCI-0-1/1-1 should not be configured and the bit length of ChannelAccess-CPext-CAPC field in DCI 0-1/1-1 should be 0. |
|  | Summary of change: | Add in TS 38.212 the restriction that when channelAccessMode2-r17 is absent, ul-AccessConfigListDCI-0-1/1-1 should not be configured and the bit length of ChannelAccess-CPext-CAPC field in DCI 0-1/1-1 is 0. |
| Vivo [R1-2206734] | Reason for change: | It is not clear for UE that when to apply short control signalling, or when to apply Type 2 or Type 3 channel access for the transmissions within a shared COT. |
|  | Summary of change: | Adding indication of the regional information. |
| Nokia Nokia Shanghai Bell [R1-2207018] | Reason for change: | It is presently unclear how the UE can know when transmissions may occur without channel sensing as short control signalling. |
|  | Summary of change: | Add the description of how the applicability of short control signaling is determined for DL and UL transmissions. |
| Qualcomm Incorporated [R1-2207181] | Reason for change: | To specify criterion for sensing exempted transmission of first message of the random access procedure by the UE. |
|  | Summary of change: | Revise specification in section 4.4.5 to (1) Specify a 1 bit RRC parameter ShortControlSignaling-r17 that indicates exempted transmissions (2) further describe a cell wide constraint on the duty cycle of the first message of the random access procedure |
| ZTE Sanechips[R1-2206082] | Reason for change: | Clarify “Contention Exempt Short Control Signalling rules” for UL |
|  | Summary of change: | Change “UE” mentioned in the last paragraph of clause 4.4.5 in TS 37.213 to “all UEs in a Cell”. |

# Channel Access Indication within Fall-Back DCI and RAR UL Grant

This discussion was addressed in Section 5-9 of [1].

Most companies agree that for FR2-2, the ChannelAccess-Cpext field in the fall-back DCI should be 2 bit, with explicit signaling for Type 1, Type 2 or Type 3 signaling. Simlarly the RAR UL grant for FR2-2 may use 2 bit ChannelAccess-Cpext field. The modification involves introducing a new table for interpretation of the field, specific to FR2-2, in TS 38.212 and appropriate pointers to the table in 38.212 and 38.213, for fallback DCI and RAR UL grant respectively.

From the CRs submitted to this meeting, and discussion papers we have the following positions

- 2 Bit ChannelAccess-Cpext

  - Qualcomm Incorporated [R1-2207179],[R1-2207180]

  - Intel Corporation  [R1-2206538],[ R1-2206539]

  - LG Electronics [R1-2207031],[R1-2207030]

  - NTT DOCOMO INC. [R1-2207380]

- 1 Bit ChannelAccess-Cpext

  - Nokia Nokia Shanghai Bell [R1-2206977], [R1-2206978] (Type 1 vs Type 3)

  - Ericsson [R1-2207468]

- 0 Bit ( Conditioned on absence of RRC parameter channelAccessMode2-r17 ),

  - Xiaomi [R1-2206615]

## Discussion paper views:

The key proposals  from discussion papers are captured below.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| ZTE Sanechips | Proposal 9: From the impact on RRC signalling point of view , it is recommended that Type 2 channel access procedures or Type 3 channel access procedures will be used based on DCI indication for the case where the UE later finds out the transmission is in a gNB COT. |
| ZTE Sanechips | Proposal 10: For fallback DCI format 0_0 and 1_0 and RAR UL grant, it is proposed to use 2 bits for the ChannelAccess-Cpext field to indicate LBT type. |
| Nokia Nokia Shanghai Bell | Proposal 2: Fallback DCIs 0_0 and 1_0 support indication of Type 1 or Type 3 channel access, using 1 bit. The related Draft CRs are in [10] (38.212) and [11] (38.213). |
| LGE | Observation #1: Since the channel access type indication for the fallback DCI format applies also to the RAR UL grant during the RACH procedure, the specification impact will be large if bit length for ChannelAccess-CPext field is changed from 2 bits to 1 bit in fallback DCI. |
| LGE | Proposal #6: All three channel access types should be able to be indicated through 2-bit ChannelAccess-CPext field in fallback DCI formats and RAR grant. |
| LGE | Proposal #7: Adopt the following TPs for TS 38.212 and TS 38.213. |
| Qualcomm Incorporated | Proposal 7: Adopt the CR in [5] and [6] covering 2 bit indication of ChannelAccess-Cpext, field for fallback DCI and RAR UL grant. |
| Apple | Proposal: 1-bit CCA indication in fall back DCI indicating type 1 or type 3 channel access. |
| Ericsson | Proposal: For Table 8.2-1 Random Access Response Grant Content field sizefield in TS 38.21, the ChannelAccessCpext field indicates 1 bit for FR2-2 operation. Adopt the following TP #2 shown below. |
| Ericsson | Proposal: For fallback DCI formats 0_0 and 1_0 for FR2-2 operation, the ChannelAccessCpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, and “Type 3 channel access in 4.4.3 of 37.213”. Adopt the following TP #1 in the accompanying draft CR which is copied below. |

## Draft CRs:

Following is the overview of draft CRs received on this aspect from the companies.

| Company | Reason/Summary | Description |
| --- | --- | --- |
| CATT[R1-2206364] | Reason for change: | For shared spectrum operation in FR2_2 band, it is agreed that UE/gNB can transmit on a channel directly without LBT procedure if the NO-LBT mode is indicated (e.g channelAccessMode2 is not configured by high layer), and this agreement also applies to Msg2 transmission/reception. |
|  | Summary of change: | Clarify that size of ChannelAccess-CPext in RAR grant is 0 if channelAccessMode2 is not configured by high layer |
| Intel Corporation [R1-2206538] | Reason for change: | Currently no indication related to which channel access type should be used are included in the fall-back DCIs for FR2-2, which means this DCIs cannot be used for unlicensed operation in this band, given that a UE is unaware of this information and whether it may operate within or outside of gNB’s COT, and more importantly compliance with the regional regulatory requirements cannot be always guaranteed. In order to solve this issue, the following CR is proposed. |
|  | Summary of change: | Clarify that ChannelAccess-CPext field in DCI format 0-0 and 1-0 is present when operations with shared spectrum channel access and LBT-mode is indicated to perform channel access for FR2-2 and provide specific mapping with the channel access type that a UE should be using. |
| Intel Corporation [R1-2206539] | Reason for change: | Currently no indication related to which channel access type should be used are included in the RAR UL grant for FR2-2, which means that compliance with the regional regulatory requirements cannot be always guaranteed, and COT sharing during random access procedure cannot be performed. In order to solve this issue, the following CR is proposed. |
|  | Summary of change: | Clarify that ChannelAccess-CPext field in UL RAR DCI is present when operations with shared spectrum channel access and LBT-mode is indicated to perform channel access for FR2-2. |
| Nokia Nokia Shanghai Bell [R1-2206977] | Reason for change: | For operation in FR 2-2 it is yet undecided what is the number of bits in the ChannelAccess-Cpext field in fallback DCIs 0_0 and 1_0, and how the field should be interpreted. |
|  | Summary of change: | Clarify that the ChannelAccess-Cpext field is one bit and indicates whether to apply Type 1 or Type 3 channel access. |
| Nokia Nokia Shanghai Bell [R1-2206978] | Reason for change: | For operation in FR 2-2. it is yet undecided what is the number of bits in the ChannelAccess-Cpext field in random access repsponse DCI, and how the field should be interpreted. |
|  | Summary of change: | Clarify that the length of the ChannelAccess-Cpext field is one bit and it indicates whether to apply Type 1 or Type 3 channel access. |
| LG Electronics [R1-2207030] | Reason for change: | Details of channel access type indication via RAR UL grant have not been specified yet for FR2-2. |
|  | Summary of change: | Refer to Table 7.3.1.1.1-4B in TS 38.212 for Channel Access-CPext field in the RAR UL grant for FR2-2. |
| LG Electronics [R1-2207031] | Reason for change: | Details of channel access type indication via fallback DCI have not been specified yet for FR2-2. |
|  | Summary of change: | 1) Create Table 7.3.1.1.1-4B for channel acces type indication in FR2-2 |
|  |  | 2) Refer to Table 7.3.1.1.1-4B in TS 38.212 for Channel Access-CPext field in the DCI format 0_0 for FR2-2. |
|  |  | 3) Refer to Table 7.3.1.1.1-4B in TS 38.212 for Channel Access-CPext field in the DCI format 1_0 for FR2-2. |
| Qualcomm Incorporated [R1-2207179] | Reason for change: | When ChannelAccess-Cpext field is present in fallback DCI formats 0_0 and 1_0 for FR 2-2, the length is fixed to two bits. |
|  | Summary of change: | 1. Revise the spec text related to ChannelAccess-Cpext field in fallback DCI, specifically addition of a new table for FR2-2 operation |
| Qualcomm Incorporated [R1-2207180] | Reason for change: | The ChannelAccess-Cpext field is present in RAR UL grant for FR 2-2, the length is fixed to two bits. |
|  | Summary of change: | 1. Random access reponse procedure sections 8.2 and 8.2A, also point to a new table in 38.212 to interpre the ChannelAccess-CPext field in the DCI, namely Table 7.3.1.1.1-4B in TS 38.212 for FR2-2 |
| NTT DOCOMO INC. [R1-2207380] | Reason for change: | Correction on channel access type indication for DCI format 0_0 and 1_0 for FR2-2. |
|  | Summary of change: | · Clarify the number of bits for channel access type indication in DCI format 0_0 and 1_0 for FR2-2, and association between each entry of the field and the indicated channel access type for FR2-2 |
| Ericsson [R1-2207468] | Reason for change: | The specification does not include the requirement to enable LBT types in RAR UL grant as indicated in the Fallback DCIs |
|  | Summary of change: | In RAN1 107, it was agreed to indicate channel access types for non-fallback DCIs. However, specification text for indication of LBT in non-fallback DCIs and the RAR UL grant is not agreed yet. |
| Ericsson [R1-2207470] | Reason for change: | The specification does not include this requirement to enable LBT types in fallback DCIs |
|  | Summary of change: | In RAN1 107, it was agreed to indicate channel access types for non-fallback DCI formats with new tables for FR2-2 operation. However, specification text for indication of LBT in the fallback DCIs is not yet agreed. |

# LBT Upgrade in COT Sharing: Rule for Channel Access Type Change for UE from Type 1 to Type 2 or Type 3 LBT

This discussion was part of Section 5-7 of [1].

This discussion was captured in Proposal 5-7-2 of [1], quoted below.

Proposal 5-7-2 (2nd round): [1] For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to either Type 2 channel access or Type 3 channel access - If Type 2 Channel Access is configured as one of the entries in the configuration for DCI 0_1 (or DCI 1_1), the UE can use Type 2 channel access. - If Type 2 Channel Access is not configured as one of the entries in the configuration for DCI 0_1 (or DCI 1_1), the UE can use Type 3 channel access. FFS part of 5-3-3: - FFS: whether/ in which cases LBT is needed within a COT (without additional RRC impact), including e.g.: - whether this bit could also be used as indication for using Type1/Type 2 (depending on UE capability) or Type 3 LBT for UL transmissions in shared COT - whether this bit could also be used as indication for using Type1/Type 2 (depending on UE capability) or Type 3 LBT when resuming a COT after a gap

From the CRs and discussion papers submitted to this meeting, we have the following positions

- The decision to switch to Type 2 channel access is based on DCI 0_1/1_1 configuration:

  - Qualcomm Incorporated [R1-2207186]

  - ZTE

  - [Prior support from Oppo , HW,IDCC, Transsion]

- The decision to switch to Type 2  channel access is based a unified bit in the SIB:

  - Ericsson:

- The decision to switch to Type 2  channel access is based a 1 bit RRC parameter

  - LGE:

- No switch to Type 2 channel access  PUSCH later than DCI_2_0 :

  - CATT[R1-2206362]

## Discussion paper views:

The key proposals  from discussion papers are captured below.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Qualcomm Incorporated | Proposal 5: Adopt the draft CR in [12], that avoids an RRC change. |

## Draft CRs:

Following is the overview of draft CRs received on this aspect from the companies.

| Company | Reason/Summary | Description |
| --- | --- | --- |
| CATT[R1-2206362] | Reason for change: | For shared spectrum operation on FR1, the UE may switch from Type 1 channel access procedure to type 2A if the scheduled PUSCH duration is within COT duration indicated by format 2_0, and similar procedure shall be introduced for shared spectrum operation on FR2_2. |
|  |  | For operation on FR2_2, the COT duration indicated by format 2_0 may include a sub-set beams that are multiplexed in time domain since gNB may perform sensing of different beams separately and only a sub-set beams are sensed as idle channel. Because there is no beam information indicated in format 2_0, the UE shall decide the LBT type according to the following: |
|  |  | - LBT type can be changed to type 3 or type 2 if PUSCH scheduling reception is no later than the reception of DCI format 2_0, assuming the COT duration indicated by format 2_0 is meant for the previous scheduled beam. |
|  |  | - LBT can be performed as PUSCH scheduling indicated and LBT type will not be changed if PUSCH scheduling reception is later than the reception of DCI format 2_0. |
|  | Summary of change: | 1) Add section 4.4.8 in TS 37.213 to desrbile UE behaviour after receiving DCI format 2_0. |
| Qualcomm Incorporated [R1-2207186] | Reason for change: | 1. If agreed, the change will specify the rule for channel access type upgrade at the UE, when a UL Transmission with Type 1 Channel access is determined to be part of a gNB COT |
|  | Summary of change: | 1. In a new Section 4.4.4.1, the rule for switching channel access type at the UE is specified, that depends on whethter Type 2 channel access is configured as part of DCI 0_1 or DCI 1_1 configuration |

# Multi-Beam Channel Access: Independent per beam sensing and LBT Procedure for UE Initiated COT:  COT on a Subset of Beams

The discussions related to this were carried out as part of Section 5-5 of [1].

A consensus view is that on the gNB as well as UE side the Multi-Beam Channel access procedure closely mimic the Multi-Channel LBT procedure treating beams as though they are separate channels.

The rule for performing independent per beam sensing are described as . UE side independent per-beam LBT sensing eases the channel access by permitting transmission on a subset of beams where LBT is successful and therefore is desirable. On the other hand, there are situations where the network expects a multi-beam transmission on all the beams or no beams. For example, the key case is sDCI (single DCI) UL mTRP transmission, which must occur on all the transmission beams.   If the network needs to restrict the behavior of UEs to either ‘all or nothing’ channel occupancy, it should have the flexibility to do so.

The proposal 5-5-5 from [1] was close to convergence as quoted below.

Proposal 5-5-5 [1] When independent per-beam LBT sensing is performed at UE, a transmission is allowed to occur on a beam if the corresponding LBT procedure for all the beams the transmission is intended for has been successful before the channel occupancy start time - For one PUCCH or PUSCH transmission over multiple transmission beams, LBT for all the beams the PUCCH/PUSCH transmissions are intended shall be passed - TP in 5-5-4-B (same as in Proposal 5-5-4) Moderator note: The sub-bullet is modified to cover not only sDCI UL mTRP transmission, but also sTRP transmission over multiple transmission beams.

From the CRs and discussion papers submitted to this meeting, we have the following positions

When independent per-beam LBT sensing is performed at UE

- a transmission on a beam is allowed to occur if the corresponding LBT procedure for the beam is successful

  - Samsung [R1-2206792]

- a transmission on a beam is allowed to occur if the corresponding LBT procedure for all the beams the transmission (PUCCH or PUSCH) is intended for has been successful

  - LGE [R1-2207029]

  - Qualcomm [R1-2207183]

## Discussion paper views:

The key proposals from discussion papers are captured below.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| FUTUREWEI | Observation 1: When a node performs independent per-beam LBT sensing, it does not know in advance which LBT will be successful and which one will fail. |
| FUTUREWEI | Observation 5: Clarification is necessary whether the transmission across multiple beams is performed only if the channel access is successful on every beam. |
| FUTUREWEI | Proposal 1: Adopt the changes proposed in TP1. |
| ZTE Sanechips | Proposal 7: If the device has the capability to simultaneously sense in different beams, Alt B that“The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams” can be supported for the transmission with multiple beams. |
| ZTE Sanechips | Proposal 8: If the node has no the capability to simultaneously sense in different beams, Alt A-3 that “The node performs eCCA of the different beams simultaneous, round robin between different beams” can be considered for the transmission with multiple beams. |
| Intel Corporation | Proposal 1: For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during the COT. · Endorse the draft CR for TS37.213 in R1-2206543 |
| Nokia Nokia Shanghai Bell | Proposal 10: Agree the proposal 5-5-5 from RAN1#109-e: When independent per-beam LBT sensing is performed at UE, a transmission is allowed to occur on a beam if the corresponding LBT procedure for all the beams the transmission is intended for has been successful before the channel occupancy start time • For one PUCCH or PUSCH transmission over multiple transmission beams, LBT for all the beams the PUCCH/PUSCH transmissions are intended shall be passed |
| LGE | Proposal #1: For UE-initiated COT, if a UE intends to transmit UL transmission(s) across multiple transmission beams and performs independent per-beam sensing, the partial beam transmission can be allowed only when the channel access procedures on the sensing beam(s) corresponding to the same UL transmission occasion have all succeeded. |
| LGE | Proposal #2: Adopt the following TP for TS 37.213. |
| Qualcomm Incorporated | Proposal 3: Adopt the CR in [9]. |

## Draft CRs:

Following is the overview of draft CRs received on this aspect from the companies.

| Company | Reason/Summary | Description |
| --- | --- | --- |
| Samsung [R1-2206792] | Reason for change: | UE behavior for determining the beams for transmission is not specified, in multi-beam scenario. |
|  | Summary of change: | Add UE behavior for determining the beams for transmission is not specified, in multi-beam scenario. |
| LG Electronics [R1-2207029] | Reason for change: | In order to allow a transmission on a beam if the corresponding LBT procedure for all or part of the beams the transmission is intended for has been successful when independent per-beam LBT sensing is performed at UE. |
|  | Summary of change: | Describe in Clause 4.4 that the partial beam transmission is only allowed when the channel access procedures on the sensing beam(s) corresponding to the same UL transmission occasion have all succeeded when independent per-beam sensing is performed for UE-initiated COT. |
| Qualcomm Incorporated [R1-2207183] | Reason for change: | If agreed support UL transmission over LBT passing sensing beams only, except sDCI UL mTRP case, where all composition beams need to pass LBT |
|  | Summary of change: | Add description on what scenario the UL transmission in a subset of sensing beams is allowed |

# ED Threshold and LBT Bandwidth

The discussions related to this were carried out as part of Section 5-1 of [1].

From the CRs and discussion papers submitted to this meeting, we have the following positions

- To avoid uncontrolled high EDT, EDT should be not higher than a specific threshold:

  - ZTE

  - Qualcomm [R1-2207182]

- No further changes needed to the computation

  - Nokia

- Text to clarify that at least the actual bandwidth will be used for EDT computation:

  - Qualcomm [R1-2207182]

## Discussion paper views:

The key proposals from discussion papers are captured below.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| ZTE Sanechips | Proposal 1: EDT is determined based on the actual bandwidth that LBT is performed when LBT is performed over a wider bandwidth than the active BWP bandwidth. |
| ZTE Sanechips | Proposal 2: To avoid uncontrolled high EDT, it is proposed that EDT should be not higher than a specific threshold. |
| Nokia Nokia Shanghai Bell | Proposal 3: The LBT EDT for UL is determined by the actual LBT bandwidth used by the UE. There is no need for a further CR. |
| Qualcomm Incorporated | Proposal 1: Adopt the CR in [8] clarifying the use of BW for sensing and setting upper limit of -47 dBm on the ED Threshold. |

## Draft CRs:

Following is the overview of draft CRs received on this aspect from the companies.

| Company | Reason/Summary | Description |
| --- | --- | --- |
| Qualcomm Incorporated [R1-2207182] | Reason for change: | If agreed, the change would limit the ED threshold value to a maximum of -47 dBm to retain fairness with WiFi specs for 60GHz. Also the Spec is not complete in the sense of the BW in EDT determination is not well defined. |
|  | Summary of change: | Revise the spec text related Section 4.4.7 : Add definition for BW. Also add a cap for maximum EDT |

# Multi-Beam Channel Access: ED Threshold for independent per beam sensing

This issue was addressed in Section 5-6 of [1].

The proposal 5-6-2 in [1], quoted below, connects the EDT threshold computation for independent per beam LBT with the sensing beam used for performing the LBT operation.  This position has support from a majority of companies.

Proposal 5-6-2: (2nd round) [1] For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, for Pout in EDT determination for a sensing beam, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT - Note: By implementation, the gNB/UE can always use the maximum EIRP of all intended transmissions over all beams for EDT determination - Adopt TP 5-6-2-A

From the CRs and discussion papers submitted to this meeting, we have the following positions

- EDT should be computed separately per sensing beam:

  - FUTUREWEI

  - Intel [R1-2206543]

  - Qualcomm [R1-2207184]

## Discussion paper views:

The key proposals from discussion papers are captured below.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| FUTUREWEI | Observation 2: Clarifications of the EDT value(s) selection for independent per beam channel sensing are necessary. |
| FUTUREWEI | Observation 3: Always using a common Pout (common EDT) for multiple sensing beams can limit spatial reuse, which may increase the latency and the number of transmissions. |
| ZTE Sanechips | Proposal 3: For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, for Pout in EDT determination for a sensing beam, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT. |
| Qualcomm Incorporated | Proposal 4: Adopt the CR in [10] regarding LBT threshold for per beam sensing. |

## Draft CRs:

Following is the overview of draft CRs received on this aspect from the companies.

| Company | Reason/Summary | Description |
| --- | --- | --- |
| Intel Corporation [R1-2206543] | Reason for change: | Currently the specification lack of clarity on how to calculate the EDT when a node performs independent per-beam LBT sensing |
|  | Summary of change: | Clarify that energy detection threshold used for each sensing beam is determined independently per beam according to Clause 4.4.7 with all the transmission covered by the sensing beam. |
| Qualcomm Incorporated [R1-2207184] | Reason for change: | 1. Specify the EDT determination rule for a COT with MU-MIMO (SDM) transmission or TDM transmission with beam switching, when per beam LBT is performed at the start of the COT. |
|  | Summary of change: | 1. In Section 4.4, it is clarified to use section 4.4.7 to determine EDT for a sensing beam |

# COT resumption after a gap: Rule for Channel Access Type for resuming a UE initiated COT after a gap

This issue was discussed at length in Section 5-8 of [1].

The issue arises in the case where UE as the initiating device resume transmission after a gap within a COT. Two potential behaviors are possible.

- Behavior 1: Type 3 channel access (no LBT) no matter how long the gaps is from the previous transmission from initiating device or responding device

- Behavior 2: Type 2 channel access (Cat 2 LBT) if there is gap longer than Y us from the previous transmission from initiating device or responding device

If we support this functionality, we also need a mechanism to choose between the two behaviors. The following alternatives were discussed as part of RAN1-108e discussions.

Following the same principle as the previous section on Channel Access Type change, we can achieve a configurable support of this feature without RRC modification by tying the channel access type after a gap to the availability of Type 2 Channel Access Type for the RRC Configuration of DCI 0_1.  The idea is that if the network configures a UE for Type 2 Channel Access for DCI 0_1, for example, to satisfy regional requirements on sensing, the UE can use Type 2 channel access while resuming the COT after a gap. Otherwise, the UE will use Type 3 channel access.

The Alt 4 of the proposal 5-8-2 in [1] captures this choice that avoids further RRC modification.

Proposal 5-8-2: (2nd round) [1] - For a UE initiated COT, UE resumes transmission within MCOT after a gap of Y us from previous transmission from either gNB or UE is supported. - The UE can use either Type 2 channel access or Type 3 channel access before resuming transmission. Down select the following alternatives - Alt 1: The channel access type to use to resume COT after a gap is left to UE implementation. - It is UE’s responsibility to comply with local regulation. - If the UE is not aware of local regulation, the UE may resume transmission with Type 2 channel access if the UE is capable of Type 2 channel access. - If the UE is not aware of local regulation, and the UE is not capable of Type 2 channel access, the UE should not resume transmission - FFS: Spec impact - Alt 4: - If Type 2 Channel Access is configured as one of the entries in the configuration for DCI 0_1 (or DCI 1_1), the UE can use Type 2 channel access to resume the COT - If Type 2 Channel Access is not configured as one of the entries in the configuration for DCI 0_1, the UE can use Type 3 channel access to resume the COT - TP 5-8-2-A

From the CRs submitted to this meeting, we have the following positions

- Select Alt 1,  UE’s responsibility to comply with Regulation. Choice of Type 2 LBT is left to UE implementation:

  - Nokia Nokia Shanghai Bell [R1-2207098]

- Select Alt 4, i.e. decision to use Type 2 LBT is based on DCI Configuration of Type 2 LBT:

  - Qualcomm Incorporated [R1-2207185]

## Discussion paper views:

- The key proposals from discussion papers are captured below.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Nokia Nokia Shanghai Bell | Proposal 11: Agree the proposal 5-8-2 with Alt. 1 from RAN1#109-e: - For a UE initiated COT, UE resumes transmission within MCOT after a gap of Y us from previous transmission from either gNB or UE is supported. - The UE can use either Type 2 channel access or Type 3 channel access before resuming transmission. The channel access type to use to resume COT after a gap is left to UE implementation. - It is UE’s responsibility to comply with local regulation. - If the UE is not aware of local regulation, the UE may resume transmission with Type 2 channel access if the UE is capable of Type 2 channel access. - If the UE is not aware of local regulation, and the UE is not capable of Type 2 channel access, the UE should not resume transmission |
| Qualcomm Incorporated | Proposal 6: Adopt the CR in [11] that avoids RRC change. |

## Draft CRs:

Following is the overview of draft CRs received on this aspect from the companies.

| Company | Reason/Summary | Description |
| --- | --- | --- |
| Nokia Nokia Shanghai Bell [R1-2207098] | Reason for change: | For UE operation in FR 2-2 it is yet undecided whether a UE can resume an UE initiated COT after a transmission gap and what channel access type is to be used in that case. |
|  | Summary of change: | Add spec text defining UE resumption of UE initiated COT after a transmission gap |
| Qualcomm Incorporated [R1-2207185] | Reason for change: | 1. The change will specify the rule for resuming a transmission after a gap within an MCOT to ensure compliance with local regulation |
|  | Summary of change: | 1. Section 4.4.4 is modified to reflect the rule to follow for resumption of transmission after a gap. The rule determines the use of Type 2 channel access procedure based on whether Type 2 channel access is configured in DCI 0-1 or DCI 1-1. |

# Other issues in draft CRs and Discussion Papers

In this section, we collect remaining CRs on topics not extensively discussed before

- CP Extension: NR-U like CP extensions are not introduced for CG-PUSCH in FR 2-2

  - Nokia

- Beam Correspondence and Sensing beam for PUCCH or SRS:

  - NTT

- Editorial Clarification of language to allow Type 2 LBT at responding node: To replace “acquire channel occupancy” with “access the channel” to allow the text to be applicable to Type 2 access at the responding node,

  - Huawei HiSilicon [R1-2207663]

- CSI-RS Validation:  Clarify that validation is not needed  in  the case where CSI-RS is contention exempt

  - AsusTek [R1-2207495]

- Clarifying deletion for condition to apply channel access procedure: 37.213 text is in aparrent conflict with the RRC parameter channelAccessMode2-r17 understanding.

  - Huawei HiSilicon [R1-2207664]

- RRC Parameter to indicate if SSBs are subject to LBT: If depending on subcarrier spacing and periodicity a part of the SSBs may use short control signaling contention exemption. Knowing which SSBs are sent without LBT assists UE.

  - Nokia Nokia Shanghai Bell [R1-2207018]

- Clarification of QCL information assumed for intra-frequency and interfrequency RSSI measurements

  - Ericsson

- Channel Access mechanism after failure of Type 2 channel access procedure

  - Wilus

## Discussion paper views:

The key proposals from discussion papers are captured below.

Topic: CP Extension

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| FUTUREWEI | Observation 4: Clarification is necessary whether the CP extension is performed on a beam prior to the transmission across all beams. |
| ZTE Sanechips | Proposal 11: If directional LBT is used, it is recommended that per-beam LBT failure indication is supported in FR2-2 to better align the directional beam transmission characteristics and be compatible with the existing mechanisms. |
| Nokia Nokia Shanghai Bell | Proposal 1: NR-U like CP extensions are not introduced for CG-PUSCH in FR 2-2. |

Topic: L3-RSSI Measurements

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Ericsson | For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state in the reference serving cell and the reference BWP provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier on which the RMTC configuration is provided if that carrier is in FR2-2, otherwise in the active BWP of the FR2-2 serving cell with the lowest ID. |
| Ericsson | Proposal 2 In LS R1-2205582 from RAN1 on TCI assumption for RSSI measurement for FR2-2, for inter-frequency RSSI measurement, further clarifies the reference serving cell for TCI assumption as follows: |
| Ericsson | For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state in the reference serving cell and the reference BWP provided in the RMTC configuration. If the configured RSSI measurement resources are not confined within the bandwidth of any serving cell, UE can assume that the measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state of the active BWP of the carrier on which the RMTC configuration is provided in the reference serving cell and the reference BWP provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier on which the RMTC configuration is provided if that carrier is in FR2-2, otherwise in the active BWP of the FR2-2 serving cell with the lowest ID. |

Topic: Failure of Type 2 sensing

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| WILUS Inc. | Proposal 1: It should be discussed whether or not to specify the channel access mechanism after failure of Type 2 channel access procedure for UL/DL transmission including channel access for multiple beam operation in time domain on the 60GHz unlicensed band of FR2-2. |
| WILUS Inc. | Proposal 2: Similar with NR-U and LTE-LAA, we propose to perform Type 1 channel access procedure after failure of Type 2 channel access (Cat-2 LBT) both for DL/UL transmission followed by a UL/DL transmission(s) within the maximum Channel Occupancy Time in a shared channel occupancy on FR2-2 and for the case that a channel occupancy includes transmissions in different beams that are multiplexed in time domain. |

## Draft CRs:

Following is the overview of draft CRs received on this aspect from the companies.

Topic: Sensing Beam for PUCCH or SRS

| Company | Reason/Summary | Description |
| --- | --- | --- |
| NTT DOCOMO INC [R1-2207381]. | Reason for change: | Correction on determination of sensing beam to be used prior to PUCCH or SRS in FR2-2. |
|  | Summary of change: | · Clarify UE behavior on how to determine sensing beam for channel access procedure to be performed prior to PUCCH/SRS transmission in FR2-2 |

Topic: CSIRS validation when CSI-RS is contention exempt

| Company | Reason/Summary | Description |
| --- | --- | --- |
| AsusTek [R1-2207495] | Reason for change: | Currently, CSI-RS validation under FR2-2 only concerns the case that the UE is under LBT mode and CSI-RS verification is not performed for UE under No-LBT mode. However, CSI-RS could be part of discivery bust which could be exempted from LBT as short control signalling according to TS 37.213. When CSI-RS is exempted from LBT, gNB could transmit the CSI-RS without channel occupancy and perform CSI-RS validation for this situation would lead to errorneuos dropping of CSI-RS. |
|  | Summary of change: | Exclude the case of CSI-RS where short control signaling exemption is applicable from CSI-RS validation. |

Topic: Clarification of language to allow Type 2 LBT at responding node

| Company | Reason/Summary | Description |
| --- | --- | --- |
| Huawei HiSilicon [R1-2207663] | Reason for change: | The EDT determination mechanism can be used by a gNB/UE as an initiating node or as a responding node, i.e., if Type 2 is used during the COT. However, the current wording “to acquire a channel occupancy” restricts the use of the EDT to an initiating gNB/UE only. |
|  | Summary of change: | In Clause 4.4.7 in TS 37.213, clarify the definition of Pout such that it is unambiguously applicable as well to Type 2 channel access procedures within a COT. |

Topic Clarifying deletion for condition to apply channel access procedure

| Company | Reason/Summary | Description |
| --- | --- | --- |
| Huawei HiSilicon [R1-2207664] | Reason for change: | The current conditions for performing the channel access procedures in FR2-2, “When…condition A… or when…condition B…”, implies that satisfying either condition A (gNB/UE(s) is required by regulations to sense a channel(s)) or condition B (a gNB provides UE(s) with higher layer parameters channelAccessMode2-r17) is sufficient for performing the channel access procedures described in that clause. This is however in contrast to the common understanding in the group and to the following descriptions of the higher layer parameter channelAccessMode2-r17 in the latest version of TS 38.331 (v17.1.0). channelAccessMode2 If present ('enabled'), this field indicates that the UE shall apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure. |
|  | Summary of change: | Delete the first sentence “When a gNB/UE(s) is required by regulations to sense a channel(s) for availability for performing transmission(s) on the channel(s) or” in Clause 4.4 of TS 37.213 v17.2.0 |

Topic RRC Parameter to indicate if SSBs are subject to LBT

| Company | Reason/Summary | Description |
| --- | --- | --- |
| Nokia Nokia Shanghai Bell [R1-2207018] | Reason for change: | It is presently unclear how the UE can know when transmissions may occur without channel sensing as short control signalling. |
|  | Summary of change: | Add the description of how the applicability of short control signaling is determined for DL and UL transmissions. |

| Company | - Proposal |
| --- | --- |
| Nokia Nokia Shanghai Bell | Proposal 6: Introduce SIB-1 signaling (1-bit) to indicate whether LBT is performed prior to SSBs. Observation 2: Depending on SSB sub-carrier spacings and SSB periodicity, only a sub-set of all SSBs can be covered by short control signalling exemption. Proposal 7: It is possible to apply SCS exemption to a part of actually transmitted SSBs and LBT procedure for other/rest of the SSBs. Proposal 9: Use of short control signal contention exemption and use of LBT for different SSBs is predefined: as many lowest indexed SSBs as possible are transmitted without LBT, and the SSBs exceeding the 10% maximum are transmitted subject to LBT. The related Draft CR for 37.213 is in [12]. |

# References

- R1-2205378 FL summary on issues for channel access of 52.6GHz to 71GHz band [109-e-NR-52-71GHz] ver02, Moderator (Qualcomm),

- RP-221601 Rel-17 maintenance for extending NR operation to 71 GHz

- R1-2205652 draft CR 37.213 Corrections of the features extending NR operation to 71 GHz, Ericsson

- Chairperson’s notes: RAN1-109-e, Agenda 8.2.4

- 3GPP TS 37.213, Rel 17,  v 17.2.0

- 3GPP TS 38.212, Rel 17,  v 17.2.0

- R1-2205743, Remaining details for Beyond 52.6 channel access, FUTUREWEI

- R1-2205768, Remaining issue of initial access signals and channels for 52-71GHz spectrum, Huawei, HiSilicon

- R1-2205769, Corrections on HARQ codebook generation for 52-71GHz spectrum, Huawei, HiSilicon

- R1-2205770, Discussion on timeline of CSI request for 52-71GHz spectrum, Huawei, HiSilicon

- R1-2206080, Discussion on remaining issues of channel access for 52.6 to 71GHz, ZTE, Sanechips

- R1-2206081, Draft CR on multi-slot PDCCH monitoring for TS 38.213, ZTE, Sanechips

- R1-2206082, Clarification on Contention Exempt Short Control Signalling rules for UL in TS 37.213, ZTE, Sanechips

- R1-2206083, Correction on the subcarrier offset k_SSB in TS 38.211, ZTE, Sanechips

- R1-2206084, Correction on the tables for determining PDCCH monitoring occasions in TS 38.213, ZTE, Sanechips

- R1-2206085, Correction on a reference SCS configuration for co-DurationList in TS 38.213, ZTE, Sanechips

- R1-2206086, Correction on UE PDSCH processing procedure time for operation with shared spectrum channel access in FR2-2 in TS 38.214, ZTE, Sanechips

- R1-2206087, Correction on CD-SSB frequency indication using NCD-SSB in TS 38.213, ZTE, Sanechips

- R1-2206088, Discussion on CD-SSB frequency indication using NCD-SSB, ZTE, Sanechips

- R1-2206160, Correction on Type-1 HARQ-ACK codebook determination in TS 38.213, Fujitsu

- R1-2206180, Remaining issues for NR 52.6 GHz to 71 GHz, InterDigital, Inc.

- R1-2206293, Discussion on remaining issue short control signaling, OPPO

- R1-2206294, Draft CR on resolving issue for short control signaling, OPPO

- R1-2206362, Correction on channel access procedures upon detection of a common DCI for frequency range 2-2, CATT

- R1-2206363, Corrections for PDCCH monitoring occasion for DCI format 2_1 for the features extending NR operation to 71 GHz, CATT

- R1-2206364, Corrections on Random Access Response Grant Content field for frequency range 2-2, CATT

- R1-2206365, Corrections on the value of slot configuration period for the features extending NR operation to 71 GHz, CATT

- R1-2206533, Discussion on remaining issues on TCI states for the scheduled PDSCHs by a DCI, Intel Corporation

- R1-2206534, [draft CR] Correction on the activated TCI states for the scheduled PDSCHs by a DCI, Intel Corporation

- R1-2206535, Discussion on Type-2 HARQ-ACK CB generation when both of spatial bundling and time bundling are configured, Intel Corporation

- R1-2206536, [draft CR] Correction on Type-2 HARQ-ACK CB generation when both of spatial bundling and time bundling are configured, Intel Corporation

- R1-2206537, Discussion on Channel Access Indication within Fall-back and RAR UL Grant DCIs, Intel Corporation

- R1-2206538, [draft CR] correction on support of channel access indication within the fall-back DCIs, Intel Corporation

- R1-2206539, [draft CR] correction on support of channel access indication within the UL RAR grant, Intel Corporation

- R1-2206540, Discussion on Applicability of the Short Control Signalling Exemption, Intel Corporation

- R1-2206541, [draft CR] Correction on short control signaling LBT exemption applicability, Intel Corporation

- R1-2206542, Discussion on Pout and EDT Threshould for Independent per-beam LBT Operation, Intel Corporation

- R1-2206543, [draft CR] Correction on EDT threshold calculation for per-beam LBT operation, Intel Corporation

- R1-2206615, Correction on the bit length of ChannelAccess-CPext-CAPC field in DCI 0-1 and DCI 1-1 for FR 2-2, Xiaomi

- R1-2206730, Correction on indication of cell defined SSB from non-cell defined SSB, vivo

- R1-2206731, Remaining issues on CD-SSB frequency indication in initial access, vivo

- R1-2206732, Correction on multi-slot PDCCH monitoring in CA scenario with mixed capability types, vivo

- R1-2206733, Correction on short control signaling constrain, vivo

- R1-2206734, Correction on the indication of channel access Types, vivo

- R1-2206735, Remaining issues on channel access mechanism for NR operation from 52.6GHz to 71 GHz, vivo

- R1-2206736, Correction on division of TBGs for Type-2 codebook, vivo

- R1-2206737, Correction on time domain bundling with spatial bundling for Type-2 codebook, vivo

- R1-2206738, Remaining issues on Type-2 codebook for multi-PDSCH scheduling, vivo

- R1-2206789, Discussion for cell-defining SSB indication using non-cell-defining SSB in FR2-2, Samsung

- R1-2206790, Draft CR for cell-defining SSB indication using non-cell-defining SSB in FR2-2, Samsung

- R1-2206791, Draft CR for multi-slot PDCCH monitoring in FR2-2, Samsung

- R1-2206792, Draft CR for multi-beam channel access procedure in FR2-2, Samsung

- R1-2206793, Draft CR for HARQ-ACK timing parameters for FR2-2, Samsung

- R1-2206794, Draft CR for aperiodic CSI triggering offset for FR2-2, Samsung

- R1-2206976, Remaining issues on channel access mechanism, Nokia, Nokia Shanghai Bell

- R1-2206977, Correction on ChannelAccess-Cpext field in fallback DCIs 0_0 and 1_0 , Nokia, Nokia Shanghai Bell

- R1-2206978, Correction on ChannelAccess-Cpext field in random access response, Nokia, Nokia Shanghai Bell

- R1-2207018, Corrections on Short Control Signaling, Nokia, Nokia Shanghai Bell

- R1-2207021, On UE capability parameters for CA with per-slot group monitoring, Nokia, Nokia Shanghai Bell

- R1-2207023, Discussion on default value of duration-r17 in FR2-2, LG Electronics

- R1-2207024, Draft CR for SSSG switching with multiple cells in FR2-2, LG Electronics

- R1-2207025, Discussion on SSSG switching with multiple cells in FR2-2, LG Electronics

- R1-2207026, Draft CR for aperiodic CSI triggering offset in FR2-2, LG Electronics

- R1-2207027, Draft CR for type-1 HARQ-ACK codebook for multi-PDSCH scheduling, LG Electronics

- R1-2207028, Remaining issues of channel access mechanism to support NR above 52.6 GHz, LG Electronics

- R1-2207029, Draft CR for independent per-beam sensing and LBT procedure for UE in FR2-2, LG Electronics

- R1-2207030, Draft CR on channel access indication for RAR grant in FR2-2, LG Electronics

- R1-2207031, Draft CR for channel access indication within fallback-DCI in FR2-2, LG Electronics

- R1-2207082, Initial access aspects, Nokia, Nokia Shanghai Bell

- R1-2207098, Correction on UE resuming a UE initiated COT, Nokia, Nokia Shanghai Bell

- R1-2207179, Draft CR on ChannelAccess-Cpext in Fallback DCI, Qualcomm Incorporated

- R1-2207180, Draft CR on ChannelAccess-Cpext in RAR UL Grant, Qualcomm Incorporated

- R1-2207181, Draft CR on sensing exempted transmission of first message of RACH, Qualcomm Incorporated

- R1-2207182, Draft CR on BW parameter in EDT determination and ED Value cap, Qualcomm Incorporated

- R1-2207183, Draft CR on UL transmission with LBT per sensing beam, Qualcomm Incorporated

- R1-2207184, Draft CR on EDT determination rule for COT with SDM or TDM transmission with per beam LBT, Qualcomm Incorporated

- R1-2207185, Draft CR on rule for resuming a transmission after a gap within MCOT, Qualcomm Incorporated

- R1-2207186, Draft CR on rule for channel access type upgrade, Qualcomm Incorporated

- R1-2207187, Discussion paper on Maintenance for NR from 52.6GHz to 71 GHz, Qualcomm Incorporated

- R1-2207269, Draft CR for spatial HARQ-ACK bundling for type-2 codebook with multi-PDSCH scheduling, Nokia, Nokia Shanghai Bell

- R1-2207309, Maintenance on Supporting NR from 52.6GHz to 71GHz, Apple

- R1-2207380, Draft CR on channel access type indication by fallback DCI in FR2-2, NTT DOCOMO, INC.

- R1-2207381, Draft CR on spatial domain filter for sensing in FR2-2, NTT DOCOMO, INC.

- R1-2207382, Discussion on remaining issues for NR in FR2-2, NTT DOCOMO, INC.

- R1-2207464, Draft CR on UE capability name alignment, Ericsson R1-2207465,

- Discussion on UE capability name alignment, Ericsson

- R1-2207466, Draft CR on Group 2 search space configuration, Ericsson

- R1-2207467, Discussion on Group 2 search space configuration, Ericsson

- R1-2207468, Draft CR on LBT-type indication in RAR UL grant, Ericsson

- R1-2207469, Discussion on LBT-type indication in RAR UL grant, Ericsson

- R1-2207470, Draft CR on LBT-type indication in Fallback DCIs, Ericsson

- R1-2207471, Discussion on LBT-type indication in Fallback DCIs, Ericsson

- R1-2207472, Discussion on L3-RSSI measurements and LS to RAN4, Ericsson

- R1-2207473, Discussion on configuration to enable LBT for all transmissions, Ericsson

- R1-2207495, Correction on CSI-RS validation, ASUSTeK

- R1-2207519, Corrections on PDCCH monitoring enhancement for 52-71GHz spectrum, Huawei, HiSilicon

- R1-2207524, Corrections on the timeline of CSI request in TS38.214, Huawei, HiSilicon

- R1-2207595, Remaining issue on channel access for NR from 52.6GHz to 71GHz, WILUS Inc.

- R1-2207608, On spatial HARQ-ACK bundling for type-2 codebook with multi-PDSCH scheduling, Nokia, Nokia Shanghai Bell

- R1-2207642, Remaining issues of channel access mechanism for 60 GHz unlicensed operation, Huawei, HiSilicon

- R1-2207663, Corrections to ED threshold for use with Type 2 channel access procedure in FR2-2 in TS37.213, Huawei, HiSilicon

- R1-2207664, Corrections to the conditions for channel sensing in FR2-2 in TS37.213, Huawei, HiSilicon