3GPP TSG-RAN WG1 Meeting #110	R1-2207740

Toulouse, France, August 22 - 26

Source:	Intel Corporation

Title:	Evaluations on coverage impact for eRedCap UE

Agenda item:	9.6.2

Document for:	Discussion and Decision

# Introduction

In RAN1#109-e meeting, the following agreements were agreed on the evaluation of the coverage impact for eRedCap UE.

Agreement

         At least the option of RF+BB BW reduction to 5MHz is considered for coverage evaluation

  FFS whether/which other options are also considered

  FFS which DL/UL Channels of all the DL/UL channels are evaluated

Conclusion

- SLS evaluation for network capacity and spectral efficiency is not conducted in Rel-18 RedCap SI.

Agreement

- Evaluation methodology and assumption in Clause 6.3 in TR 38.875 is reused for coverage evaluation of reference UE and Rel-17 RedCap UE.

  - Note: It is up to each company whether to reuse the LLS results

Agreement

- Coverage for the following channels is evaluated for “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”

  - SIB1

  - PBCH

  - PDCCH CSS

  - [Msg4]

  - Following channels can be optionally evaluated

    - PUSCH

    - PUCCH 2bits

    - PUCCH 11bits

    - PUCCH 22bits

    - PRACH

    - PDSCH

    - PDCCH USS

    - Msg2

    - Msg3

- Evaluation methodology and assumption in Clause 6.3 in TR 38.875 is reused for coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels” by default, except for, UE bandwidth, cell edge data rate, and small form factor degradation

  - FFS which evaluation assumption should be updated for the above channels

Agreement

         Following evaluations are not conducted in Rel-18 RedCap SI

  Latency

  Throughput

  Power saving gain

Agreement

         Coverage of Msg4 can be optionally evaluated for “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”

Agreement

         For coverage evaluation of Rel-17 and Rel-18 RedCap UEs, only 1 Rx branch is assumed.

  Note: it does not mean that 2Rx is precluded for Rel-18 RedCap UE

Agreement

         For coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, following parameters are used.

| Parameters | FR1 values |
| --- | --- |
| UE bandwidth | Rural: 5 MHz (25 PRBs, 15 kHz SCS) Urban: 5 MHz (11 PRBs or 12 PRBs (optional), 30 kHz SCS) |

  Note: Rural scenario at 0.7 GHz, Urban scenario at 2.6 GHz, and Urban scenario at 4 GHz (optional) are considered.

Agreement

         For coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, target data rates are

  FR1 Rural: 250 kbps on DL and 25 kbps in UL

  FR1 Urban: 500 kbps on DL and 250 kbps in UL

  Note: The target data rates are the scaled value in the Rel-17 RedCap SI by a factor of 0.25

Agreement

         3dB antenna efficiency loss can be optionally assumed for coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”

Agreement

         For at least PDCCH USS coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, following revision are assumed

  For 15KHz SCS, CORESET size is 3 symbols and 24 PRBs, AL is 8.

  For 30KHz SCS,

  Opt1: CORESET size is 3 symbols and 6 PRBs, AL is 2 (baseline)

  Opt2: CORESET size is 3 symbols and 12 PRBs, AL is 4 (optional)

FFS：Use all CCEs of the CORESET

Agreement

         For coverage evaluation of Rel-18 RedCap UE, 1 Tx branch is assumed.

Conclusion

         Evaluation of PDCCH blocking probability is not conducted in Rel-18 RedCap SI

Agreement

         For SIB1 coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, followings are assumed

  Opt1: SIB1 BW is larger than 5MHz, e.g., 48PRB

  The UE can receive a part of SIB1 PDSCH at a time. Detail assumption of reception scheme (e.g., puncturing the bits transmitted outside UE BW) is reported by each company.

  Opt2: SIB1 BW is within 5MHz

  A TBS of 1256 bits(other size is not precluded)

Note: whether interleaving mapping is assumed depends on companies’ report

Agreement

         For PDCCH CSS coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, following revision are assumed

  Opt1: CORESET BW is larger than 5MHz

  The UE can receive a part of PDCCH at a time. Detail assumption of reception scheme (e.g., puncturing the bits transmitted outside UE BW) is reported by each company.

  For 15/30kHz SCS, CORESET size is 2 symbols and 48 PRBs, AL is 16.

  For 30kHz SCS, CORESET size is 2 symbols and 24 PRBs, AL is 8.  Other configuations are also not precluded

  Opt2: CORESET BW is within 5MHz

  For 15kHz SCS, CORESET size is 3 symbols and 24 PRBs, AL is 8.

  For 30kHz SCS,

  Opt2-1: CORESET size is 3 symbols and 6 PRBs, AL is 2.  Other configuations are also not precluded

  Opt2-2: CORESET size is 3 symbols and 12 PRBs, AL is 4

Agreement

         The LLS results of the option of “RF+BB BW reduction to 5MHz for all DL/UL channels” can be reused for the coverage evaluation of other BW reduction options, if applicable.

Agreement

         For coverage evaluation in Urban scenario at 4 GHz, DL PSD 33 dBm/MHz is baseline and DL PSD 24 dBm/MHz is optional.

Agreement

         Revise the agreement as follows:

Agreement

         For at least PDCCH USS coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, following revision are assumed

  For 15KHz SCS, CORESET size is 3 symbols and 24 PRBs, AL is 8.

  For 30KHz SCS,

  Opt1: CORESET size is 3 symbols and 6 PRBs, AL is 2 (baseline)

  Opt2: CORESET size is 3 symbols and 12 PRBs, AL is 4 (optional)

FFS: Use all CCEs of the CORESET Other configurations are also not precluded

Agreement

         For Msg4 coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, a TBS of 1040 bits is assumed

  a TBS smaller than 1040 bits can be optionally evaluated and reported by each company.

Agreement

         For PRACH coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, Format 0 is used for Rural scenario and Format B4 is used for Urban scenario

  Format C2 can be used optionally.

Agreement

For Msg2 coverage evaluation of reference UE, Rel-17 RedCap UE, and Rel-18 RedCap UE, A TBS of 72 bits is assumed.

At least the prioritized channels, i.e., PBCH, PDCCH CSS, SIB1 PDSCH should be simulated to check if there is a critical coverage problem. For the prioritized channels, gNB may transmit them using a large number of allocated PRBs which is better for exiting UEs. However, an eRedCap UE can only do partial reception which results in performance degradation. The similar observation may apply to Msg4 too. In this contribution, we present our evaluation results for the different channels.

# Partial reception for broadcast channels

In last RAN1 meeting, three options for BW reduction (BW1/2/3) and three options for peak data rate reduction (PR1/2/3) are identified for further study. BW1/2 has the strongest limitation on BB BW, i.e., an eRedCap UE can only receive in 5MHz bandwidth. It corresponds to 25 localized PRBs for SCS 15kHz, or 11 or 12 localized PRBs for SCS 30kHz. BW3 and PR3 are similarly with a difference on localized or distributed resource allocation [1]. For BW1/2/3 or PR3, if a broadcast channel, e.g., PBCH/PDCCH CSS/SIB1/Msg4 is allocated with large number of PRBs and is shared by eRedCap UE and legacy UEs, partial reception becomes inevitable. On the other hand, there is no limitation on the number of allocated PRBs in PR1/2. Therefore, PR1/PR2 doesn’t require partial reception for eRedCap UE.

Whether partial reception is necessary for a channel in an option depends on the SCS of the DL BWP and the configured number of PRBs for three prioritized channels.

- For PBCH, partial reception is necessary for BW1/2 if SCS 30kHz is deployed. Note: Certain NR bands defined in RAN4 can only support single SCS for SSB transmission of 30kHz. It needs further discussion whether the same limitation applies to BW3/PR3. On the other hand, there is no issue on PBCH reception for SCS 15kHz.

- For PDCCH CSS, partial reception is likely used for BW1/2 since gNB may allocate a large number of PRBs for CORESET#0 for better performance of legacy UEs. Except for one case of CORESET 0 with 24 PRBs and SCS 15kHz, partial reception is necessary for all other cases. There is no issue for BW3/PR3 since 20MHz BW for control channel is supported in BW3/PR3.

- Regarding SIB1 PDSCH, partial reception may be necessary for all options, BW1/2/3 and PR3, except for a case of CORESET 0 with 24 PRBs and SCS 15kHz. However, CORESET 0 of 24 PRBs may not be a good choice considering the deployment scenario since it can only support 12 CCEs, which means the largest CCE aggregation level 16 is inapplicable.

In the following section, we provide link level simulation results for the partial reception.

For PBCH with SCS 30kHz, an eRedCap UE may only receive the center 11 or 12 PRBs due to limitation of 5MHz BW. There is no issue for PSS/SSS detection. However, it means half number of REs for PBCH are punctured. Comparing with Rel-17 RedCap UE, the loss is about 4.5dB.

Figure 1: Performance of PBCH detection

For PDCCH CSS, two options are considered by RAN1 simulation assumption. One option assumes the CORESET has a BW > 5MHz and a UE can only do partial reception in 5MHz. Assuming the CORESET has N PRBs (>5MHz), we assume the lowest 25 or 11 or 12 PRBs are received by the eRedCap UE. A typical configuration is CORESET with 48 PRBs and 2 symbols. The total number of CCEs is 16. An eRedCap UE can only receive 12*12*2=288 REs of the PDCCH, which corresponds to 4 CCEs. As shown in Figure 2, all three options for PDCCH reception of eRedCap UE have much worse performance than Rel-17 RedCap UE. The punctured reception, i.e., only receiving 11 of 24 or 48 PRBs are 7-8 dB worse. On the other hand, relying on a CORESET with 6 PRBs is the worst since it can only support PDCCH with AL 2. From the results, the required SNR for PDCCH miss detection rate 0.01 is relatively high, which motivates necessary enhancement.

Figure 2: Performance of PDCCH detection

SIB1 PDSCH may have a large payload size which requires a large number of allocated PRBs for reliable transmission. Denote the number of allocated PRBs for SIB1 as N (>5MHz), we assume the lowest 25 or 11 or 12 PRBs are received by the eRedCap UE. For example, the agreed simulation assumption is 1256 bits for SIB1. If the UE only receive 11 allocated PRBs, the coding rate is about 0.4 which is too high for reliable SIB1 reception. Therefore, a typical value of 4 repetition of SIB1 in a 160ms cycle is considered in the simulation. We assume 48 PRBs are allocated for SIB1 transmission (for existing UEs). From Figure 3, the performance loss of partial reception is around 7.5dB compared to Rel-17 RedCap UE. The working point of residual BLER 0.01 for eRedCap is about 1 dB, which is bad for coverage. Therefore, it is expected more repetitions are necessary for SIB1 reception of eRedCap UE. From the simulation results, transmission of SIB1 in 11 or 12	 PRBs resource allocation is slightly better than partial reception.

Figure 3: Performance of SIB1 PDSCH reception

Based on the above simulation results, large performance loss is observed due to partial receptions. Comparing with Rel-17 RedCap UE with 1 Rx antenna,

- The loss is 4.5dB for PBCH, 7-8dB for PDCCH CSS, 7.5dB for SIB1 PDSCH.

- If CORESET is limited to 6 PRBs, i.e., PDCCH AL 2, the performance loss is about 12.3dB.

Observation 1: Large performance loss is observed due to partial receptions. Comparing with Rel-17 RedCap UE with 1 Rx antenna,

- The loss is 4.5dB for PBCH, 7-8dB for PDCCH, 7.5dB for SIB1 PDSCH.

- If CORESET is limited to 6 PRBs, i.e., PDCCH AL 2, the performance loss is about 12.3dB.

# Coverage evaluations

According to the agreement in RAN1#109-e to reuse the evaluation methodology of Rel-17 RedCap coverage recovery, the general procedure for the current evaluation should be done in following steps.

- Step 1: Obtain the link budget performance of the channel based on link budget evaluation

- Step 2: Obtain the target performance requirement for RedCap UEs within a deployment scenario

- Step 3: Find the coverage recovery value for the channel if the link budget performance is worse than the target performance requirement

The bottleneck channel is the channel with least MIL for Rel-15 reference UE, which is independent from Rel-17 or Rel-18 UE. Therefore, the bottleneck channel is still PUSCH for the interested scenarios as in Rel-17 RedCap study. The link level simulations are done for Rel-15 reference UE, Rel-17 RedCap UE and the eRedCap UE. Then, based on the results, the channel(s) with coverage problem for eRedCap UE can be identified. The exact simulation assumptions for the channels are provided in Annex.

One open issue is whether to apply 3dB loss due to form factor in the link budget analysis. We currently do not consider it in the following figures. However, the MILs for channels in the figures can be easily adjusted (-3dB) to take into account the form factor. As agreed in Rel-17, the antenna gain is modeled by four components. A correction factor is used to adjust the ideal gains of the antenna components. The same values as used in Rel-17 RedCap study can be reused, which is provided in Table 1. The antenna gain correction factors are obtained based on system level simulations. In the simulations, the antenna gain correction factor is determined based on the difference between theoretical antenna gain and 95th percentile antenna gain. For Rural 700MHz, the correction factors are 0 dB.

Table 1. Antenna gain correction factor for different scenarios

| Scenario | Theoretic antenna gain (dB) | 95th percentile antenna gain (dB) | Antenna gain correction factor (dB) |
| --- | --- | --- | --- |
| Urban at 4GHz: Broadcast channel | 16.81 | 12.10 | 4.71 |
| Urban at 4GHz: Unicast channel | 16.81 | 13.27 | 3.54 |
| Urban at 2.6GHz: Broadcast channel | 16.81 | 12.52 | 4.29 |
| Urban at 2.6GHz: Unicast channel | 16.81 | 13.78 | 3.03 |
| Indoor at 28GHz: Broadcast channel | 18.06 | 8.16 | 9.80 |
| Indoor at 28GHz: Unicast channel | 18.06 | 9.09 | 8.97 |

# Urban scenario, 2.6GHz

Figure 4 and 5 respectively illustrate MIL for various channels under Urban scenario at 2.6GHz with 11 or 12 PRBs bandwidth capability of eRedCap UE. The simulated options for each channel are ordered in the same way as the link budget template from moderator. All channels show better MIL performance than the bottleneck PUSCH. Even when -3dB form factor is further considered, no coverage problem is identified for all eRedCap channels. Note: Since low data rate for eRedCap UE is assumed in the agreed evaluation assumptions, i.e., 1/4 of that for DL/UL of Rel-17 RedCap UE, the observed MIL for PUSCH of eRedCap UE is even better than the bottleneck PUSCH.

Figure 4: MIL for Urban 2.6GHz, 11PRBs for eRedCap UE

Figure 5: MIL for Urban 2.6GHz, 12PRBs for eRedCap UE

Observation 2: For Urban scenario at 2.6GHz, all channels of eRedCap UE have better MIL than the bottleneck PUSCH, hence coverage recovery is not necessary.

# Rural scenario, 700MHz

Figure 6 illustrates MIL for various channels under Rural scenario at 700MHz. The simulated options for each channel are ordered in the same way as the link budget template from moderator. All channels show better MIL performance than the bottleneck PUSCH. Even when -3dB form factor is further considered, no coverage problem is identified for all eRedCap channels.

Figure 6: MIL for Rural 700MHz

Observation 3: For Rural scenario at 700MHz, all channels of eRedCap UE have better MIL than the bottleneck PUSCH, hence coverage recovery is not necessary.

# Urban scenario, 4GHz

For the Urban scenario at 4GHz, two options for DL PSD are simulated, i.e., 33 dBm versus 24 dBm (optional). For the option of high PSD 33dBm, similar observations on MIL of various channels compared to bottleneck PUSCH are made. The simulated options for each channel are ordered in the same way as the link budget template from moderator. From Figure 7, all channels show better MIL performance than the bottleneck PUSCH. Even when -3dB form factor is further considered, no coverage problem is identified for all eRedCap channels.

Figure 7: MIL for Urban 4GHz, PSD 33dBm

However, if low PSD 24dBm for gNB is considered, multiple DL channels will show worse MIL than the bottleneck PUSCH

- PDCCH CSS/USS and Msg4 need enhancements without applying 3dB loss of form factor

- If 3dB loss of form factor is considered, PDSCH, PDCCH CSS/USS, Msg2/4 need enhancements

Figure 8: MIL for Urban 4GHz, PSD 33dBm

Observation 4: For Urban scenario at 4GHz,

- If high PSD is 33dBm for gNB, all channels of eRedCap UE have better MIL than the bottleneck PUSCH, hence coverage recovery is not necessary.

- If low PSD is 24dBm for gNB,

  - PDCCH CSS/USS and Msg4 need enhancements before applying 3dB loss of form factor

  - If 3dB loss of form factor is considered, PDSCH, PDCCH CSS/USS, Msg2/4 need enhancements

# Conclusions

In this contribution, we presented our evaluation results for partial reception of PBCH, PDCCH CSS and SIB1 PDSCH. We make the following observations.

Observation 1: Large performance loss is observed due to partial receptions. Comparing with Rel-17 RedCap UE with 1 Rx antenna,

- The loss is 4.5dB for PBCH, 7-8dB for PDCCH, 7.5dB for SIB1 PDSCH.

- If CORESET is limited to 6 PRBs, i.e., PDCCH AL 2, the performance loss is about 12.3dB.

Observation 2: For Urban scenario at 2.6GHz, all channels of eRedCap UE have better MIL than the bottleneck PUSCH, hence coverage recovery is not necessary.

Observation 3: For Rural scenario at 700MHz, all channels of eRedCap UE have better MIL than the bottleneck PUSCH, hence coverage recovery is not necessary.

Observation 4: For Urban scenario at 4GHz,

- If high PSD is 33dBm for gNB, all channels of eRedCap UE have better MIL than the bottleneck PUSCH, hence coverage recovery is not necessary.

- If low PSD is 24dBm for gNB,

  - PDCCH CSS/USS and Msg4 need enhancements before applying 3dB loss of form factor

  - If 3dB loss of form factor is considered, PDSCH, PDCCH CSS/USS, Msg2/4 need enhancements

# References

- R1-2206593, “On solutions to further reduce UE complexity”, Intel Corp.

- R1-2206594, “Evaluations on coverage impact for eRedCap UE”, Intel Corp.

# ANNEX Simulation assumptions

Table A1: Common evaluation assumptions

| Parameters | FR1, Urban | FR1, Urban | FR1, Rural |
| --- | --- | --- | --- |
| Carrier Frequency | 2.6 GHz (TDD) | 4 GHz (TDD) (Low priority) | 0.7 GHz (FDD) |
| SCS | 30 kHz | 30 kHz | 15 kHz |
| Frame structure for TDD | DDDDDDDSUU (S: 6D:4G:4U) | DDDSUDDSUU (S: 10D:2G:2U) | N/A |
| # of gNB TX chains | 4 | 4 | 2 |
| # of gNB RX chains | 4 | 4 | 2 |
| Channel Model | TDL-C, NLOS | TDL-C, NLOS | TDL-C, NLOS |
| UE antenna correlation | low | low | low |
| delay spread | 300 ns | 300 ns | 300 ns |
| UE velocity | 3 km/h | 3 km/h | 3 km/h |
| Rel-15 UE |  |  |  |
| BW | 100MHz (273 PRBs) | 100MHz (273 PRBs) | 20MHz (106 PRBs) |
| # of UE TX chains | 1 | 1 | 1 |
| # of UE RX chains | 4 | 4 | 2 |
| Rel-17 RedCap UE |  |  |  |
| BW | 20MHz (51 PRBs) | 20MHz (51 PRBs | 20MHz (106 PRBs) |
| # of UE TX chains | 1 | 1 | 1 |
| # of UE RX chains | 1 | 1 | 1 |
| eRedCap UE |  |  |  |
| BW | 5MHz (11 or 12 PRBs) | 5MHz (11 or 12 PRBs) | 5MHz (25 PRBs) |
| # of UE TX chains | 1 | 1 | 1 |
| # of UE RX chains | 1 | 1 | 1 |

Table A2: Evaluation assumptions for PBCH

| Parameters | Values |
| --- | --- |
| waveform | CP-OFDM |
| performance target | 1% rBLER |
| T/F resources | Per RAN1 spec |
| Payload | 32 bits payload, 24 bits CRC |
| Number of transmissions | 4 |
| Rx combining | MRC |
| Diversity scheme | Precoder cycling |
| Periodicity | 20 ms |

Table A3: Evaluation assumptions for PDCCH

| Parameters | Values |
| --- | --- |
| waveform | CP-OFDM |
| performance target | 1% BLER |
| CORESET | For reference UE - 2 symbols, 48 PRBs, PDCCH AL 16 For eRedCap UE - 2 symbols, 48 PRBs, AL 16, 11 or PRBs partial reception - 2 symbols, 24 PRBs, AL 8, 11 or 12 PRB partial reception - 3 symbols, 6 PRBs, AL 2 - 3 symbols, 12 PRBs, AL 4 |
| Frequency distributed | Yes |
| Payload | 40 bits |
| Number of transmissions | 1 |
| Rx combining | MRC |
| Diversity scheme | PDCCH USS: Precoder cycling every 6 REGs PDCCH CSS: wideband RS |
| Aggregation level | 16, 8, 4, 2 |

Table A4: Evaluation assumptions for SIB1 PDSCH

| Parameters | Values |
| --- | --- |
| TBS | 1256 bits |
| Resource allocation | 48 PRBs |
| Performance target | 1% rBLER |
| TDRA | 12 OFDM symbols |
| DMRS | Type I, 1 DMRS symbol, no multiplexing with data. |
| Number of transmissions | 4 |

Table A5: Evaluation assumptions for Msg2

| Parameters | Values |
| --- | --- |
| waveform | CP-OFDM |
| performance target | 10% iBLER |
| Rx combining | MRC |
| Diversity scheme | Precoder cycling |
| TB scaling | 1, 1/4 |
| PRBs/TBS/MCS | MCS = 0, TBS = 72, PRB = 3 (TB scaling = 1) and 11 (TB scaling = 0.25) |
| PDSCH duration | 12 OS |
| DMRS configuration | Type I, 3 DMRS symbol, no multiplexing with data |
| HARQ configuration | No retransmission |

Table A6: Evaluation assumptions for Msg4

| Parameters | Values |
| --- | --- |
| waveform | CP-OFDM |
| performance target | 10% rBLER |
| Rx combining | MRC |
| Diversity scheme | Precoder cycling |
| PRBs/TBS/MCS | 1040 bits, TBS=1064 MCS is fixed to zero. N_PRB=37 |
| PDSCH duration | 12 OS |
| DMRS configuration | Type I, 2 DMRS symbol, no multiplexing with data |
| HARQ configuration | No retransmission |

Table A7: Evaluation assumptions for PUSCH

| Parameters |  | Values |  |  |
| --- | --- | --- | --- | --- |
|  |  | Rel15 UE | Rel-17 RedCap UE | eRedCap UE |
| waveform |  | DFT-s-OFDM |  |  |
| Target data rate |  | 1 Mbps (Urban), 100 kbps (Rural) |  | 250kbps (Urban), 25kbps (Rual) |
| Performance target |  | 10% iBLER |  |  |
| TDRA |  | 14 OFDM symbols |  |  |
| DMRS |  | Type I, 1+1 DMRS symbol, no multiplexing with data. |  |  |
| TBS | 4 GHz: | 1864 | 1864 | 504 |
|  | 2.6 GHz: | 2792 | 2792 | 704 |
|  | 700 MHz: | 128 | 128 | 32 |
| MCS | 4 GHz: | 1 | 1 | 2 |
|  | 2.6 GHz: | 2 | 2 | 3 |
|  | 700 MHz: | 0 | 0 | 0 |
| FDRA | 4 GHz: | 41 | 41 | 9 |
|  | 2.6 GHz: | 51 | 51 | 10 |
|  | 700 MHz: | 4 | 4 | 1 |
| Number of transmissions |  | 1 (no HARQ) |  |  |
| Rx combining |  | MRC |  |  |

Table A8: Evaluation assumptions for Msg3

| Parameters | Values |
| --- | --- |
| waveform | DFT-S-OFDM |
| performance target | 10% rBLER |
| Rx combining | MRC |
| PRBs/TBS/MCS | TBS 56bits, MCS is fixed to zero. N_PRB=2 |
| PDSCH duration | 14 OS |
| DMRS configuration | Type I, 3 DMRS symbol, no multiplexing with data. |
| HARQ configuration | No retransmission |