3GPP TSG RAN WG1 #110			                                    		   R1-2207744

Toulouse, France, August 22th – 26th, 2022

Agenda Item:	9.5.2.1

Source:	Moderator (InterDigital, Inc.)

Title:	FL summary #1 on integrity of RAT dependent positioning techniques

Document for:	Discussion

# Introduction

## RAN1 Task

As indicated in SID RP-213588, RAN1 is tasked to identify error sources for determining integrity for RAT dependent positioning.

... - Improved accuracy, integrity, and power efficiency: - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible. ...

## Contact information

To facilitate discussions, please provide your contact information below.

| Company | Point of contact | Email address |
| --- | --- | --- |
| InterDigital Inc. | Fumihiro Hasegawa | Fumihiro.hasegawa@InterDigital.com |
| vivo | Yuanyuan Wang | yuanyuan.wang.txyj@vivo.com |
| ZTE | Chuangxin Jiang | jiang.chuangxin1@zte.com.cn |
| Samsung | Pengru Li | pengru5.li@samsung.com |

### Priority indication in each section/subsection

In this document, [HIGH], [MED] and [LOW] are used to indicate priority of each discussion topic.

In addition, [CLOSED] is used to indicate that the issue will not be discussed further.

In the following, contributions from companies are summarized and proposals from the FL (feature lead) and template for collecting company inputs are listed.

# Background information

A list of agreements made in RA1#109e is shown below.

Agreement - Study sources of error for timing-based positioning and angle-based positioning methods, focusing on the following aspects - Origin of the error source - e.g., At UE and/or network side - e.g., From assistance information, and/or measurements - Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Criteria to become an error source (e.g., whether it is quantifiable, how much influence an error source has on determination on integrity) - It is encouraged to provide evaluation assumptions (e.g., requirements in TS 38.101, TS 38.104, TS 38.133, evaluation assumptions in TR 38.857) if evaluation is used to determine a distribution, mean and standard deviation or range of values of an error source - UE-based/assisted DL positioning methods, UL and DL&UL positioning methods are considered in the study Agreement - At least the following error sources for timing-based positioning methods are studied - TRP/UE measurements errors (e.g., ToA, Rx-Tx timing difference) - FFS: Effect of multipath/NLoS channels on TRP/UE measurement errors - Error in assistance data (e.g., TRP location, Inter-TRP synchronization errors (e.g., RTD)) - TRP/UE Timing error - FFS: Further study identification of error sources resulting from the multipath/NLoS channel/radio propagation environment, including multipath/NLoS channel itself as an error source - Other error sources are not precluded - FFS: details of each error source, e.g., mean/standard deviation/range associated with each error Agreement - At least the following error sources for angle -based positioning methods are studied - TRP/UE measurements errors (e.g., AoA, RSRP, RSRPP) - FFS: Effect of multipath/NLoS channels on TRP/UE measurement errors - Error in assistance data (e.g TRP location, TRP beam antenna information) - FFS: Further study identification of error sources resulting from the multipath/NLoS channel/radio propagation environment, including multipath/NLoS channel itself as an error source - Other error sources are not precluded - FFS: details of each error source, e.g., mean/standard deviation/range associated with each error Agreement For the purpose of discussion of error sources, reuse the definitions for RAT-dependent integrity and update the references to GNSS in Section 8.1.1a in TS38.305 to also include RAT-dependent methods. - Note: The intention of the proposal is not to make text proposals for TS 38.305 - FFS: whether to modify and/or how to modify, for the purpose of discussion in RAN1, terms in 8.1.1a in TS 38.305 (e.g., definitions for “Error”, “Bound”, “Time-to-Alert (TTA)”, “DNU”, “Residual Risk”, “irMinimum, irMaximum”) for RAT dependent positioning methods Agreement In addition to the agreed aspects for the study, study the following aspects for error sources for timing/angle based positioning methods - Mapping between an error source and a positioning method (e.g., DL, UL, DL&UL positioning method) - e.g., error in TRP location can be an error source for UE-based DL-AoD Other aspects are not precluded

In TS 38.305, according to the principle of integrity operation, the network will ensure the following :

For integrity operation, the network will ensure that: P(Error > Bound for longer than TTA ¦ NOT DNU) <= Residual Risk + IRallocation for all values of Irallocation in the range irMinimum <= Irallocation <= irMaximum Bound for a particular error is computed according to the following formula: Bound = mean + K * stdDev K = normInv(IRallocation / 2) irMinimum <= IRallocation <= irMaximum where: mean: mean value for this specific error, as per Table 8.1.2.1b-1 stdDev: standard deviation for this specific error, as per Table 8.1.2.1b-1

To following the principle above, an error source and associated parameters must be identified. More details about the principle of integrity from TS 38.305 are shown in Appendix A. Examples of GNSS related error sources described in TR 38.305 are shown in Appendix B. Integrity parameters specified in TS 37.355 are summarized in Appendix C. List of error sources discussed in R1-2205344 (FL summary from RAN1#109e) is shown in Appendix D.

# Suggested proposals for approval and discussion

## For online session Aug. 22, 2022

Proposals for approval

FL Proposal 1a-2

- For UE-based positioning integrity mode, error sources in assistance data are at least the following :

  - For DL-TDOA

    - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355)

    - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355)

  - For DL-AoD

    - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355)

    - FFS : whether boresight direction of DL-PRS (e.g., NR-DL-PRS-BeamInfo in TS 37.355) is an error source

    - FFS : whether beam information of DL-PRS (e.g., NR-TRP-BeamAntennaInfo in TS 37.355) is an error source

- FFS : Inter-TRP synchronization as an error source for UL-TDOA, for LMF-based positioning integrity mode

- FFS : ARP location (e.g., ARPLocationInformation in TS 38.455) for UL-AoA for LMF-based positioning integrity mode

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- Other error sources are not precluded

- Note : Definition of “UE-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

FL Proposal 4-2

- For LMF-based positioning integrity mode, at least the followings are error sources time of arrival is an error source for timing related measurements (e.g., RSTD, RTOA, Rx-Tx time difference) for DL, UL and DL&UL timing-based positioning methods.

  - Time of arrival (for UL-TDOA)

  - Time difference of arrival (for DL-TDOA)

  - Rx-Tx time difference (for Multi-RTT)

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

FL Proposal 5-1

- For LMF-based positioning integrity mode, angle of arrival is an error source in angle related measurements for UL-AoA

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- FFS: The error can be expressed as the error of the AoA/ZoA in LCS or GCS or the error of a defined function of AoA/ZoA in LCS.

- Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

FL Proposal 9-2

- Studies on error sources related to Rel. 18 positioning methods are deferred until more clarity is achieved on the Rel-18 positioning methods.

Proposals for discussion

FL Proposal 2-1

- Further study the following error sources and whether the error sources affect derivation of positioning integrity

  - PRS configuration (e.g., NR-DL-PRS-AssistanceData including ExpectedAoD, ExpectedAoA, ExpectedRSTD)

  - LOS/NLOS indicator (e.g., NR-DL-PRS-ExpectedLOS-NLOS-Assistance)

  - TEG configuration (e.g., NR-DL-PRS-TRP-TEG-Info)

  - Other error sources are not precluded

FL Proposal 1b-2

- For LMF-based positioning integrity mode, error sources in assistance data are at least the following:

  - For UL-TDOA

    - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355)

  - For UL-AoA

    - ARP location (e.g., ARPLocationInformation in TS 38.455)

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- Other error sources are not precluded

- Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

FL Proposal 3-2

- Error sources related to TX/RX timing are at least the following:

  - TRP TX timing error for UE-based positioning integrity mode for DL-TDOA and Multi-RTT

  - TRP RX timing error for LMF-based positioning integrity mode for UL-TDOA and Multi-RTT

  - UE TX timing error for LMF-based positioning integrity mode for UL-TDOA and Multi-RTT

  - UE RX timing error for LMF-based positioning integrity mode for DL-TDOA and Multi-RTT

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- FFS : whether TxRx timing error is an error source

- Note : Definitions of “UE-based positioning integrity mode” and “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

FL Proposal 7-1

- Further study whether multipath/NLOS channel is an independent error source

# Issues for discussion

## Organization, scheduled topics and materials for discussion

The planning for future meetings is shown below.

Table 1 Tentative schedule

| Meeting # | Main discussion points |
| --- | --- |
| RAN1#110 | - Agree on error sources and corresponding mapping to positioning methods - Identify error sources that require further studies - Discuss details of error sources (e.g., distribution, whether paired overbounding is applicable) |
| RAN1#110b-e | - Agree on the details of error sources (e.g., mean/range/standard deviation, distribution, applicability of paired overbounding) |
| RAN1#111 | - Agree on the remaining details of error sources |

The proposals related to potential error sources from the contributions [1]-[17] are summarized in Section 4. In addition, tables summarizing potential error sources for each positioning method can be found in [1, 3, 6, 7, 10, 11]. In [1], potential specification impacts have been identified for UE-based and LMF-based positioning integrity mode. In general, proposals focus on error sources in the following categories

- Error sources related to assistance data

  - Proposals in Section 4.1 (errors in assistance data), Section 4.2 (misconfiguration)

- UE/TRP timing error

  - Proposals in Section 4.3 (TX/RX timing error)

- Measurement error

  - Proposals in Section 4.4 (ToA), Section 4.5 (AoA), Section 4.6 (RSRP/RSRPP)

- Channel error

  - Proposal in 4.7 (multipath)

Definitions for “UE-based positioning integrity mode” and “LMF-based positioning integrity mode” are based on the definitions used in Table 9.4.1.1.1 in TR 38.857. The definitions are reproduced below and used in the FL summary:

- UE-based positioning integrity mode : Positioning integrity result is derived by the UE

- LMF-based positioning integrity mode : Positioning integrity result is derived by the LMF

## [HIGH] Issue #1 : Error sources in assistance data

### Summary

The issue #1 deals with uncertainties in assistance data provided by the LMF. Uncertainties may arise from inaccurate positioning of TRPs/ARPs or quality of synchronization. Uncertainties related to some of the aforementioned error sources can be found in specifications [13,14,17].

It should be noted that the proposals based on integrity determination methods, i.e., UE-based/LMF-based integrity determination are made in [1]. UE-based integrity mode and UE-based positioning share the same error sources since, in principle, integrity or position is determined at the UE based on assistance information provided by the network and measurements. In addition, whether errors qualify as error sources or not may depend on determination mechanism of integrity (i.e., UE-based, LMF-based). For example, for LMF-based positioning integrity mode, there is no need for the network to send error information (required for integrity determination) to the UE. If the UE sends measurements to the LMF, LMF-based positioning integrity mode is applicable since integrity can be computed based on the measurements and/or associated error information. Similarly, LMF-based positioning integrity mode is applicable for UL-based positioning methods.

For clarity, relationship between UE-based/assisted positioning methods and UE/LMF based positioning integrity mode is summarized in the table below.

Table 2 Relationship between positioning integrity mode for RAT dependent positioning methods and positioning methods

|  | UE based positioning integrity mode | LMF based positioning integrity mode |
| --- | --- | --- |
| UE-based DL-TDOA | Applicable | Not applicable |
| UE-assisted DL-TDOA | Not applicable | Applicable |
| UE-based DL-AoD | Applicable | Not applicable |
| UE-assisted DL-AoD | Not applicable | Applicable |
| Multi-RTT | Not applicable | Applicable |
| UL-TDOA | Not applicable | Applicable |
| UL-AoA | Not applicable | Applicable |

It has been discussed in contributions that assistance information such as BeamAntennaInfo (used by the location server to provide beam antenna information of the TRP) [6, 7,14] and BeamAntennaInfo (is used by the location server to provide spatial direction information of the DL-PRS Resources) [1, 3, 10, 14] can contain errors due to calibration errors or granularities in configuration. The following error sources are proposed by companies.

DL-TDOA

- TRP location error [1, 2, 3, 7 (ARP location), 10, 12 (FFS), 13, 14, 15 ,17]

- Inter-TRP synchronization error [1, 2, 3, 6, 7, 13, 14 ,15, 17 (sync error)]

DL-AoD

- TRP location error [1, 2, 3, 7 (ARP location), 10, 12 (FFS), 13, 14, 15, 17]

- Error in boresight direction of DL-PRS [1, 3, 10, 14]

- Beam information of DL-PRS  [6, 7(phase center offset), 14]

UL-AoA

- ARP location error [2, 3, 7, 13, 17]

### Proposal for the 1st round of discussion

It has also been discussed in [1] that inter-TRP synchronization is an error source applicable to UL-TDOA. However, since gNB processes measurements, inter-TRP synchronization may be resolved via implementation at gNB [1]. Therefore, it is not clear if inter-TRP synchronization is an error source for UL-TDOA. The FL proposes to further study inter-TRP synchronization as an error source for UL-TDOA.

Additionally, for UL-AoA, only LMF-based integrity determination is applicable. In that case, whether ARP location (e.g., ARPLocationInformation in TS 38.455) is an error source for UL-AoA positioning method is not clear since any potential errors related to ARP location can be processed internally in the LMF.

The details of additional assistance information related to error sources (e.g., mean/standard deviation of the error source) may have an impact on specification. Thus, it is important to clarify relationship between error sources and LMF/UE-based positioning integrity mode in the proposals such that we can clarify scope of the study for each category.

Based on companies’ views, the following proposal is made. Companies are invited to provide views on whether the proposal can be supported, suggest modifications, or present additional aspects for the proposal.

FL Proposal 1-1

- For UE-based positioning integrity mode, error sources in assistance data are at least the following :

  - For DL-TDOA

    - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355)

    - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355)

  - For DL-AoD

    - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355)

    - Boresight direction of DL-PRS (e.g., NR-DL-PRS-BeamInfo in TS 37.355)

    - Beam information of DL-PRS (e.g., NR-TRP-BeamAntennaInfo in TS 37.355)

- FFS : Inter-TRP synchronization as an error source for UL-TDOA, for LMF-based positioning integrity mode

- FFS : ARP location (e.g., ARPLocationInformation in TS 38.455) for UL-AoA for LMF-based positioning integrity mode

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- Other error sources are not precluded

- Note : Definition of “UE-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

Companies views:

| Company Name | Support Proposal 1-1 (Yes/No) | Comments |
| --- | --- | --- |
| CATT | Yes | For the two FFS, both “Inter-TRP synchronization” and “ARP location” should be the error sources in our view. Whether TRP can provide the information. Whether an error can be eliminated during the location calculation by UE or LMF should be a separate issue. For any error source, if the UE or LMF can eliminate it during the positioning calculation it may exclude it when it determines the PL. But, the source of the error (UE or TRP) should provide the information related to the error to LMF for LMF-based, or to UE for UE-based positioning in our vew. |
| Vivo | Yes with comment | For us, it is weird that the first bullet is about UE-based integrity and the first two FFS is for LMF-based integrity. We prefer to discuss this separately. |
| FL |  | To CATT and vivo : Thank you very much for your comments. Based on the comments, FL’s suggestion is to break FL Proposal 1-1 into two pieces, first (FL Proposal 1a-1) focusing on UE-based integrity and second (FL Propsosal 1b-1) focusing on LMF based integrity as follows. To all companies, please review the proposals below. FL Proposal 1a-1 - For UE-based positioning integrity mode, error sources in assistance data are at least the following : - For DL-TDOA - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355) - For DL-AoD - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) - Boresight direction of DL-PRS (e.g., NR-DL-PRS-BeamInfo in TS 37.355) - Beam information of DL-PRS (e.g., NR-TRP-BeamAntennaInfo in TS 37.355) - FFS : Inter-TRP synchronization as an error source for UL-TDOA, for LMF-based positioning integrity mode - FFS : ARP location (e.g., ARPLocationInformation in TS 38.455) for UL-AoA for LMF-based positioning integrity mode - FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Other error sources are not precluded - Note : Definition of “UE-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 FL Proposal 1b-1 - For LMF-based positioning integrity mode, error sources in assistance data are at least the following: - For UL-TDOA - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355) - For UL-AoA - ARP location (e.g., ARPLocationInformation in TS 38.455) - FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Other error sources are not precluded - Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 |
| ZTE | Partially Yes | We don’t think boresight direction and beam information should be an independent error source for DL-AoD. gNB would have no idea on the error of this kind of information, so it is unclear how to model them and how to configure/report them. At UE side, it is also impossible to get the error model for this configuration. |
| FL2 |  | To ZTE Thank you very much for the comment. The FL’s understanding is that there could be an error related to boresight information and beam information in the AD given by the LMF. Thus, if needed, the LMF may give additional error related ultipath data (e.g., range of error) related to the aforementioned AD to the UE. The FL wold like to hear companies’ views. |
| Huawei, HiSilicon | FL Proposal 1a-1 Yes | We do not think any assistance data enhancement is needed for LMF-based integrity mode; in fact the information transferred between gNB and LMF is not considered as assistance data, and there is no RTD for UL-TDOA. |
| Qualcomm | Yes to 1a-1 | Agree with FL2 response to ZTE. Note that errors in the boresight direction and beam information impact DL AoD positioning in a manner quite analogous to the way in which errors in the NR-RTD-Info impact the DL-TDOA positioning. Thus, they should both be error sources, for the same reasons. Modeling of the error sources is a separate topic that can be discussed separately, and applies to both these error sources. Regarding 1b-1 and Huawei’s comment above, we agree there is no RTD for UL-TDOA, the analogous quantity would instead be the SFN initialization time reported by gNB to LMF. |
| Ericsson | Partially OK | For the 1a proposal, we agree with ZTE that for beam information and boresight information, the error will not be known to the gNB. From the FL2 comment, is the intention to capture error statistics derived by the LMF without additional signalling from the gNB? For proposal 1b, we are ok with the AOA part but agree with other that for ULTDOA the RTD info is not used. |
| Samsung | Yes to 1a-1 | For the purpose of PRS resource(s) measurement and reporting, boresight direction information can be optionally sent by the LMF in the assistance data for each PRS resource, and TRP beam information also can be optionally provided by the LMF to the UE for UE-based DL-AoD with an order of priority of PRS resources. If boresight direction information and beam information are sent by LMF to enhance the signaling to the UE, it should be the error sources in our view. |
| FL3 |  | To Ericsson : FL’s understanding was that through implementation, error information can be exchanged between gNB and LMF and LMF can dliver error information to the UE. It seems like more discussions are needed so the FL would like to propose the following modification for Proposal 1a-1 by adding FFS to boresight and beam information. Proposal 1b-1 needs more discussions and we can try to discuss the modified proposal online. FL Proposal 1a-2 - For UE-based positioning integrity mode, error sources in assistance data are at least the following : - For DL-TDOA - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355) - For DL-AoD - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) - FFS : whether boresight direction of DL-PRS (e.g., NR-DL-PRS-BeamInfo in TS 37.355) is an error source - FFS : whether beam information of DL-PRS (e.g., NR-TRP-BeamAntennaInfo in TS 37.355) is an error source - FFS : Inter-TRP synchronization as an error source for UL-TDOA, for LMF-based positioning integrity mode - FFS : ARP location (e.g., ARPLocationInformation in TS 38.455) for UL-AoA for LMF-based positioning integrity mode - FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Other error sources are not precluded - Note : Definition of “UE-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 FL Proposal 1b-2 - For LMF-based positioning integrity mode, error sources in assistance data are at least the following: - For UL-TDOA - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355) - For UL-AoA - ARP location (e.g., ARPLocationInformation in TS 38.455) - FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Other error sources are not precluded - Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 |

## [HIGH] Issue #2 : Misconfiguration in assistance data provided by the LMF

### Summary

Likelihood of occurrence of the error (e.g., misconfiguration) should be discussed. Arguments presented in [12, 14] are related to misconfiguration of PRS parameters or inaccurate information related to LOS/NLOS indicators which lead to loss of accuracy in the estimated location information. Furthermore, it is discussed in [14] that discrete parameters such as PRS configuration or TEG may not be quantifiable for integrity determination. However, DNU flag may be used for error sources which are not quantifiable to prevent deterioration of positioning accuracy.

On the other hand, it is argued in [8] that the LMF should be able to provide expected values of RSTD/AoA/AoD with sufficient uncertainty that the UE/gNB should be able to use the information for measurement of PRS/SRS. If the LMF is not certain about the assistance information, it is argued in [8] that it will not provide such measurement. Some companies expressed their views for Issue #3 and Issue #4 in R1-2205344 on why expectedAoD/AoA or ExpectedRSTD should not be an error source.  Views from companies can be summarized as follows.

- Consider ExpectedAoD, ExpectedAoA, ExpectedRSTD as error source(s) [12(FFS), 14]

- Do not consider consider ExpectedAoD, ExpectedAoA, ExpectedRSTD as error source(s) [8]

- Further study ExpectedAoD, ExpectedAoA, ExpectedRSTD as error sources [9]

It is also noted that the LOS/NLOS indicator can be considered as an error source in [11]. On the other hand, it is discussed in [17] that the LOS/NLOS indicator should be reliable enough to reduce uncertainty in the presence of LOS/NLOS channel.

- LOS/NLOS indicator is an error source [11]

- LOS/NLOS indicator is not an error source [17]

### Proposal for the 1st round of discussion

The discussion point is whether misconfiguration can be considered as an error source for determination of positioning integrity. Companies are invited to provide views on whether the following proposal can be supported, suggest modifications, or present additional aspects for the study.

FL Proposal 2-1

- Further study the following error sources and whether the error sources affect derivation of positioning integrity

  - PRS configuration (e.g., NR-DL-PRS-AssistanceData including ExpectedAoD, ExpectedAoA, ExpectedRSTD)

  - LOS/NLOS indicator (e.g., NR-DL-PRS-ExpectedLOS-NLOS-Assistance)

  - TEG configuration (e.g., NR-DL-PRS-TRP-TEG-Info)

  - Other error sources are not precluded

Companies views:

| Company Name | Comments |
| --- | --- |
| CATT | In our view any of the LMF Misconfiguration in assistance data could potentially cause a problem in positioning. For example, the erroneous information of ExpectedAoA could potentially result in the error in TRP to make an error in the determination of the UL-AoA. This type of errors could be modelled as an random variable representing the probability of erroneous information, e.g., the probability of providing the erroneous information of ExpectedAoA is 10^(-X) with a variation of 10^(-Y). However, it would be difficult to include such information during the determination of the PL, since some gNB’s implementations may be robust to the errors of ExpectedAoA, why others may not. It would then be more effective to TRP to consider the potential impact of ExpectedAoA in AOA measurement errors. |
| Vivo | For those sub-bullet, no distribution can be provided for us, if the majority think it can be seen as an error source, only a flag (such as DNU)can be associated. |
| FL | The FL would like to collect more views for this proposal. |
| ZTE | We don’t think such PRS related configuration is an error source. For ExpectedAoD, ExpectedAoA, ExpectedRSTD, there is uncertainty range informed to UE, it will help for PRS measurement, no positioning accuracy issue is caused. For LOS/NLOS indicator, it is an optional feature. If network/UE cannot ensure the LOS/NLOS condition, the corresponding report/configuration can be skipped. So it should not be an error source. Further, the error of LOS/NLOS indication will be reflected to measurement errors. So extra error source for this is not needed. |
| Huawei, HiSilicon | We also think those can be replaced by DNU flag, and further discussion is not needed. |
| Qualcomm | Any parameter that can potentially impact the positioning calculation is potentially an error source. A parameter being optional doesn’t exclude it from being an error source. We don’t agree with the argument that an optional parameter will only be provided if it is known with sufficient accuracy and thus doesn’t need to be an error source. Note that these optional parameters may still be used to compute position, even if the accuracy is insufficient for the purpose of computing integrity of the position. Thus, we should provide a way to indicate even optional parameters, with their accompanying integrity-related parameters We generally agree with vivo’s comment that a distribution may not be applicable to all the parameters. If this is making it difficult for some companies to accept certain parameters as error sources, this could be clarified in the proposal as a sub-bullet against the corresponding parameter. E.g., ‘for at least some parts of PRS configuration (including time-frequency allocation and scrambling seeds), no bound formula applies, but a DNU flag may apply, and FFS if any other integrity related parameter applies’. Along these lines, since PRS configuration is a broad category, it may be required to break it down into its sub-Ies; it seems that we may have easier agreement on some sub-Ies than on others. |
| Ericsson | We would be ok to discuss the use of DNU flags, but think discussing distributions is not needed. Additionally, we don’t support adding further signalling NLOS/LOS indicators, which can be used as is for the purpose of integrity. |
| Samsung | All the sub-bullet mentioned above could be regarded as a separate error source from other error sources if its statistical error modeling is feasible; Otherwise, they should be covered by measurement errors or assistance data errors. |
| FL | It seems like some companies cannot accept the listed AD as error sources. If the discussion is about association of parameter(s) and corresponding DNU flag(s), we can discuss this issue separately. |

## [HIGH] Issue #3 : Timing error at TRP/UE

### Summary

It is discussed in contributions that timing error can be an error source for determination of positioning integrity for RAT dependent positioning. For example, according to contributions [3,8], characteristics of timing error can be related to TEG. Timing error at TRP or UE can be caused by clock errors such as timing offset or timing drift [7]. It should also be noted that timing error cam be absorbed into measurement error as described in [4]. Views related to timing error are summarized below

- TRP timing error [1, 3, 4 (absorbed into RSTD measurement error), 7, 8, 13,15,17], namely

  - TRP TX timing error

  - TRP RX timing error

- UE timing error [1, 3, 7, 8 (Rx timing error), 12 (UE clock drifting),13, 15, 17], namely

  - UE TX timing error

  - UE RX timing error

It should also be noted that whether we define the above sources as error depend on whether UE/LMF-based positioning integrity mode is supported. For example, if LMF-based positioning integrity mode is not supported, TRP RX, UE TX or UE RX timing error may not be considered as an error source. For example,  in UE-based positioning integrity mode, UE RX timing error can be processed internally at the UE and no additional integrity-related assistance information from the network is needed.

### Proposal for the 1st round of discussion

Based on the companies’ views, the following proposal is made. TxRx timing error is listed as FFS since TxRx timing error source can potentially be isolated to Tx and Rx timing error. Companies are invited to provide views on whether the proposal can be supported, suggest modifications, or present additional aspects for the study.

FL Proposal 3-1

- Error sources related to TX/RX timing are at least the following:

  - TRP TX timing error for UE-based positioning integrity mode for DL-TDOA and Multi-RTT

  - TRP RX timing error for LMF-based positioning integrity mode for UL-TDOA and Multi-RTT

  - UE TX timing error for LMF-based positioning integrity mode for UL-TDOA and Multi-RTT

  - UE RX timing error for LMF-based positioning integrity mode for DL-TDOA and Multi-RTT

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- FFS : whether TxRx timing error is an error source

- Note : Definitions of “UE-based positioning integrity mode” and “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

Companies views:

| Company Name | Support Proposal 3-1 (Yes/No) | Comments |
| --- | --- | --- |
| CATT | Yes | We like to ask the groups’ view how to consider the relation of TRP Tx timing errors and the inter-TRP time synchronization errors. For DL-TDOA, what it matters is obviously the timing synchronization at the Tx antenna reference point. Assume Tx time for the TRPs are perfectly synchronized at all ARPs, do we agree it implies there is no TRP TX timing error for all DL PRS resources (or all Tx TEGs), or it means all Tx TEG error margins is zero? |
| Vivo | Yes |  |
| InterDigital |  | To CATT : From our point of view, TX timing error (e.g., hardware related error) and inter-TRP time synchronization (e.g., inter-TRP coordination related error) error can happen simultaneously as they are independent error sources. In the example presented by CATT, TRPs can have differet TX hardware which may introduce independent Tx timing error. |
| ZTE | No | We think the TEG related timing error will be reflected to measurement results, so no extra error source for this is needed. Alternatively, the TEG margin value will be known at UE or LMF side, how to consider it for integrity calculation will be an implementation issue. |
| FL |  | To ZTE : The understanding of FL is that we are trying to identify error sources at this point. Whether additional information is needed (e.g,. assistance data or additional report from the UE/gNB) can be discussed separately. In this case it may be possible that the TEG margin can be used by the UE/LMF to determine range of the error source (timing error). |
| Huawei, HiSilicon | No | It is not clear on the motivation. Is this proposal to discuss the spec impact on the error, or discuss the error ultipath of spec impact? If it is about spec impact, how do we expect to capture those error in the spec for different positioning methods? If it is not about spec impact, why does not TRP Tx timing error for DL-TDOA and Multi-RTT not listed for LMF-based integrity mode? |
| Qualcomm | Yes | To ZTE: The TEG margin being known doesn’t exclude the timing error from being an error source for integrity. Two transmission or reception timings may have been indicated to be within a certain TEG margin, but in reality may be separated by more than that margin, which would impact the positioning accuracy |
| Ericsson | OK |  |
| Samsung | Yes |  |
| FL |  | The FL’s understanding is that we would like to discuss potential specification impacts for all error sources that have been identified. There was a typo in the proposal, and the FL would like to make a correction (multi-RTT should not be associated with UE-based positioning integrity). FL Proposal 3-2 - Error sources related to TX/RX timing are at least the following: - TRP TX timing error for UE-based positioning integrity mode for DL-TDOA and Multi-RTT - TRP RX timing error for LMF-based positioning integrity mode for UL-TDOA and Multi-RTT - UE TX timing error for LMF-based positioning integrity mode for UL-TDOA and Multi-RTT - UE RX timing error for LMF-based positioning integrity mode for DL-TDOA and Multi-RTT - FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - FFS : whether TxRx timing error is an error source - Note : Definitions of “UE-based positioning integrity mode” and “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 |

## [HIGH] Issue #4 : Timing measurements at the gNB/UE

### Summary

Time of arrival (ToA) is a measurement that is used to derive RSTD, RTOA or Rx-Tx time difference. Thus, any uncertainties in ToA measurements affect accuracy of DL-TDOA/UL-TDOA/Multi-RTT positioning methods. Alternatively, each measurement can be modeled differently depending on LOS/NLOS conditions as explained in [6]. The following summarizes the companies’ views.

- ToA measurement [1, 2 11 (Table 2-1), 12, 13, 15]

- RSTD/Rx-Tx time difference measurements [3, 4, 6, 16]

Additional details related to measurements, such as distribution or statistical characteristics, can be discussed in the study since the aforementioned parameters can depend on implementation at the UE/gNB to obtain the measurements. Thus additional error related information can be reported to the LMF from UE/gNB  along with the measurements.

It is also noted in [8] that ToA measurement error may be affected by multiple factors such as noise, interference signal, timing error at Tx/Rx, ultipath.

### Proposal for the 1st round of discussion

The majority of companies seem to agree that ToA is the main source of error in timing related measurements. ToA measurements affect both UE and LMF-based integrity. However, for UE-based integrity, any statistical information of the measurements can be processed internally within the UE. For LMF-based integrity, the UE/gNB may need to send error information related to the measurements to the LMF. Thus the following proposal is made. Companies are invited to provide views on whether the proposal can be supported, suggest modifications, or present additional aspects for the study.

FL Proposal 4-1

- For LMF-based positioning integrity mode, time of arrival is an error source for timing related measurements (e.g., RSTD, RTOA, Rx-Tx time difference) for DL, UL and DL&UL timing-based positioning methods.

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

Companies views:

| Company Name | Support Proposal 4-1 (Yes/No) | Comments |
| --- | --- | --- |
| CATT | Yes | For RSTD, it may be more precise to say “time difference of arrival” is an error source. |
| Vivo | Yes | Maybe “time of arrival” can be changed to timing measurement information |
| FL |  | From comments presented by CATT and vivo, the proposal is modified as follows. To all companies, please check the proposal below. FL Proposal 4-2 - For LMF-based positioning integrity mode, at least the followings are error sources time of arrival is an error source for timing related measurements (e.g., RSTD, RTOA, Rx-Tx time difference) for DL, UL and DL&UL timing-based positioning methods. - Time of arrival (for UL-TDOA) - Time difference of arrival (for DL-TDOA) - Rx-Tx time difference (for Multi-RTT) - FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 |
| ZTE | Yes | Agree with the update from FL |
| Huawei, HiSilicon | FL Proposal 4-2 Yes |  |
| Ericsson | OK | OK with the 4-2 update from FL |
| Samsung | Yes | Agree to the updated proposal. |

## [HIGH] Issue #5 : Angle measurements at the gNB

### Summary

In UL-AoA, gNB estimates AoA based on received SRS/SRS for positioning. Thus, AoA measurement becomes an error source for UL-AoA as discussed in [1, 2, 3, 4, 5, 7, 13, 15, 16 ,17], which leads to the following proposal.

### Proposal for the 1st round of discussion

AoA measurements affect LMF-based integrity. The gNB may need to send error information related to AoA measurements to the LMF since statistical characteristics of AoA measurement error may depend on gNB implementation to obtain AoA.  Based on the companies’ views, the following proposals are made. Companies are invited to provide views on whether the proposal can be supported, suggest modifications, or present additional aspects for the study.

FL Proposal 5-1

- For LMF-based positioning integrity mode, angle of arrival is an error source in angle related measurements for UL-AoA

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

Companies views:

| Company Name | Support Proposal 5-1 (Yes/No) | Comments |
| --- | --- | --- |
| CATT | Yes |  |
| vivo | Yes | Maybe “angle of arrival” can be changed to angle measurement information |
| ZTE | Yes |  |
| Huawei, HiSilicon | Yes, but | We prefer to add another bullet FL Proposal 5-1 - For LMF-based positioning integrity mode, angle of arrival is an error source in angle related measurements for UL-AoA - FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - FFS: The error can be expressed as the error of the AoA/ZoA in LCS or GCS or the error of a defined function of AoA/ZoA in LCS. - Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 |
| Ericsson | OK |  |
| Samsung | Yes |  |
| FL |  | The proposed change from Huawei, HiSilicon seems to be ok for clarification purpose. The proposal is updated as follows. FL Proposal 5-2 - For LMF-based positioning integrity mode, angle of arrival is an error source in angle related measurements for UL-AoA - FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - FFS: The error can be expressed as the error of the AoA/ZoA in LCS or GCS or the error of a defined function of AoA/ZoA in LCS. - Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 |

## [HIGH] Issue #6 : RSRP measurements at the UE/gNB

### Summary

In UL-AoA or DL-AoD, the gNB or UE makes RSRP measurements on SRS/SRSp or PRS, respectively. Companies have presented the following views on RSRP measurements as error sources.

- DL PRS RSRP [2, 3, 6, 7, 11, 15, 16, 17]

- UL SRS RSRP/RSRPP [3, 7, 11, 15, 16, 17]

### Proposal for the 1st round of discussion

RSRP/RSRPP measurements affect both UE and LMF-based integrity. However, for UE-based integrity, any statistical information of the measurements can be processed internally within the UE. For LMF-based integrity, the UE/gNB may need to send integrity information related to the measurements to the LMF. Based on the companies’ views, it seems reasonable to consider DL/UL RSRP/RSRPP measurements as error sources. It should also be noted that RSRP/RSRPP measurements are assisting measurements (e.g., the measurements are used to select PRS/SRS resource) and impact of the RSRP/RSRPP measurement error on computation of integrity results should be studied. Companies are invited to provide views on whether the proposal can be supported, suggest modifications, or present additional aspects for the study.

FL Proposal 6-1

- For LMF-based positioning integrity mode, DL PRS RSRP/RSRPP and UL SRS RSRP/RSRPP are error source in angle related measurements for DL-AoD and UL-AoA, respectively

- FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

- FFS : Impact of RSRP/RSRPP measurement error on derivation of positioning integrity

- Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

Companies views:

| Company Name | Support Proposal 6-1 (Yes/No) | Comments |
| --- | --- | --- |
| CATT |  | We assume UL SRS RSRP/RSRPP could be useful for UL-AoA, then could be considered as the error source. However, to obtain high-accuracy UL-AoA, the UL-AoA is usually not calculated based on SRS RSRP/RSRPP, but based on the processing of the CIR/CFR received from different TRP antennas and/or antenna elements. |
| Vivo |  | We still have some concerns about RSRP measurement being an error source, especially considering the model of RSRP measurement. It is known that RSRP varies with time and channel, and the impact of pathloss with distance on RSRP is relatively small. Quantifying the impact of RSRP measurement on the positioning is difficult for us. |
| FL |  | The FL would like to collect more views for this proposal. |
| ZTE | Partially Yes | For AoD, the proposal is fine. For UL-AOA, how RSRP/RSRPP affects the positioning accuracy is not clear. Further, for TDOA and Multi-RTT, RSRP/RSRPP is also supported, should we make it error source as well? |
| Huawei, HiSilicon | No | We do not support to model RSRP/RSRPP error. |
| Ericsson | OK |  |
| Samsung |  | If DL PRS RSRP/RSRPP and UL SRS RSRP/RSRPP have an important impact on the measurement results and will arouse some measurement errors, they should be identified as error sources. As CATT proposed, UL-AoA is usually not calculated based on SRS RSRP/RSRPP. Therefore, SRS RSRP/RSRPP may not be determined as an error source. For time-based method, we don’t think that RSRP/RSRPP are error sources. |

## [HIGH] Issue #7 : Multipath channel

### Summary

It has been discussed in RAN1#109e whether multipath/NLOS channels should be considered as an independent error source. As discussed in [6, 7, 8, 11], LOS/NLOS path or multipath causes spread in measurements which could cause error in determination in positioning. On the other hand, the contributions in [1, 3, 6, 12. 13] argue that the effect of multipath/NLOS can be absorbed into measurements. It should also be noted that in GNSS applications, the effect of multipath/NLOS is assumed to be minimized by the UE implementation. Thus, no specific error source related assistance information related to multipath channel is specified in the Rel. 17 integrity framework. The views from companies are divided as summarized below, and the feature lead encourages further discussion on this issue.

- Multipath/NLOS channel is an independent error source [6 (for timing based methods?), 7, 8, 11]

- Multipath/NLOS channel is not an independent error source (it can be absorbed into measurement error/modeling of the measurement error) [1, 3, 6, 12. 13]

### Proposal for the 1st round of discussion

Based on the companies’ views, the following proposal is made. Companies are invited to provide views on whether the proposal can be supported, suggest modifications, or present additional aspects for the study.

FL Proposal 7-1

- Further study whether multipath/NLOS channel is an independent error source

Companies views:

| Company Name | Comments |
| --- | --- |
| CATT | In our view, considering multipath/NLOS channel as an independent error source may make the implementation easier. For a UE/TRP, the equipment manufacture may have pretty good knowledge on the measurement’s errors when multipath/NLOS is not considered. Thus, it might be difficult to determine the impact of multipath/NLOS channel, which depends more on the RF environment. |
| Vivo | We prefer the multipath can be covered by measurement errors, and distinguish LoS by LoS/NloS indicator. |
| ZTE | We don’t think this should be an independent error source as the impact of NLOS channel will be reflected timing/angle measurement. |
| Huawei, HiSilicon | Same view as vivo |
| Qualcomm | Can we clarify if the proposal generally about LOS/multipath, or specifically about the LOS/NLOS indicator agreed in Rel-17? The LOS/NLOS indicator is part of assistance data, and thus it can be set badly, which could impact the positioning accuracy. In this sense it is an error source, but since it already represents a probability (the probability of LOS/NLOS), a bound formula would require bounding the error in the indicated probability, which can be quite challenging. So it may be associated with a DNU flag but not a bound formula |
| Ericsson | Same view as Qualcomm. |
| Samsung | Multipath/NLOS error could be regarded as a separate error source other than measurement errors in our opinion. For example, in the per path measurement like the RSRPP, the path has already identified in the receiving side, and the measurement error could consider only the error happened after path identification. However, the multipath error could consider the error in path identification, e.g., wrongly identify multiple paths to one path. But the statistical error modeling should be further discussed. |
| FL | To Qualcomm : For clarity, we are discussing about the multipath/NLOS channel as an error source here, not about the LOS/LOS indicator. |

## [MED] Issue #8 : Characteristics of error sources and paired over-bounding

### Summary

Companies’ views on distributions error sources are summarized below.

- ToA as Gaussian distribution [1, 2, 17]

- AoA as Gaussian distribution [1. 2, 4 (LOS), 6(LOS, large variance for NLOS) , 17]

- RSTD as Gaussian distribution [4 (LOS/NLOS), 6 (LOS)]

- RSTD as log-normal distribution [6(NLOS)]

- Rx-Tx as Gaussian distribution [6 (LOS) , 17]

- Rx-Tx as log-normal distribution [6 (NLOS)]

- Inter-TRP sync error as uniform distribution [6]

- Inter-TRP sync error as Gaussian distribution [17]

- Timing error as truncated Gaussian distribution [17]

- TRP location information as uniform distribution [17]

- RSRP/RSRPP measurement error as Gaussian distribution [6, 17]

- Boresight information as Gaussian distribution [6]

It has also been discussed in [17] that standard deviation of error sources related to measurements can depend on measurement quality. In contributions, the following contributions discuss how uncertainties in specifications can be used as reference for determination of distribution

- Error margin for TEG [7, 13]

- Uncertainty specified for TRP location used as the range [17]

Applicability of over-bounding Gaussian formula on error sources are discussed in [1, 2, 3 11, 14]. For example, in [14] it is explained that paired Gaussian overbounding can be applicable for TRP location, RTD info, ExpectedAoD, ExpectedAoA, ExpectedRSTD, NR-DL-PRS-BeamInfo,  NR-TRP-BeamAntennaInfo, as discussed in [14]. In [1, 2], it is explained that paired over-bounding Gaussian formula is applicable to ToA and AoA measurements.

### FL’s suggestion for the 1st round of discussion

The FL suggests to prioritize discussions on identification of error sources (i.e., Proposal 1-1 through 6-1) and encourages companies to discuss further on determination of distribution and corresponding characteristics for each error source. Once agreements on error sources are made, the FL would like to open the discussion with a set of new proposals. Companies are invited to provide views.

Companies views:

| Company Name | Comments |
| --- | --- |
| CATT | Our preference is to use Gaussian distribution to model most if not all of the errors for simplify and the feasibility in the implementation. |
| ZTE | We also think all error sources can be modeled as the Gaussian distribution, and the GNSS liked over-bounding Gaussian formula can be reused |
| Samsung | Gaussian distribution can model most of the error sources, and the paired over-bounding Gaussian formula can be reused in our opinion. |

## [MED] Issue #9 : Error sources for Rel. 18 positioning techniques

### Summary

It is mentioned in [7] whether there is a need to investigate Rel. 18 error sources. As suggested in [7], the FL would like to wait for the frameworks of Rel. 18 technologies (e.g., SL positioning, phase based positioning, RedCap, LPHAP) to solidify. Error sources related to Rel. 18 technologies can be studied in a future release.

### FL’s suggestion for the 1st round of discussion

Based on the current situation, the following proposal is made. Companies are invited to provide views on whether the proposal can be supported, suggest modifications, or present additional aspects for the study.

FL Proposal 9-1

- Studies on error sources related to Rel. 18 positioning methods are deprioritized

Companies views:

| Company Name | Comments |
| --- | --- |
| CATT | Support. The error sources related to Rel. 18 positioning methods can be considered in future release if needed. |
| ZTE | OK |
| Huawei, HiSilicon | OK |
| Qualcomm | We prefer to replace ‘deprioritized’ by ‘deferred until more clarity is achieved on the Rel-18 positioning methods’. We think it is too early to rule out altogether than this cannot be part of Rel-18; depending on the outcome of the relevant Rel-18 methods discussions |
| Ericsson | OK. |
| Samsung | Support. |
| FL | Qualcomm : Thank you very much for the suggestion. The FL would like to incorporate the suggested change for clarificaiotn. FL Proposal 9-2 - Studies on error sources related to Rel. 18 positioning methods are deferred until more clarity is achieved on the Rel-18 positioning methods. |

## [LOW] Issue #10 : DNU flag

Configuration of DNU (Do Not Use) flags are discussed in contributions [2, 14]. As discussed in contribution [2], DNU can be used to indicate useable/non-usable PRS configurations, i.e., discrete set of parameters. For example, in GNSS applications, the UE does not use satellites with DNU flag ON, since these satellites may not provide reliable signals for navigation purpose.

It is argued in [14] that statistical information or soft information (e.g., LOS indicator) can also be associated with DNU. For example, if the aforementioned information is deemed unreliable by the LMF, DNU can be turned on for the corresponding information.

The FL’s recommendation is to further study aspects related to DNU. In addition, the FL recommends to wait for discussion on DNU to progress in RAN2, i.e., clarify relationship between DNU and error sources.

## [LOW] Issue #11 : Error sources for E-CID

It is discussed in [9] that error sources for ECID should be considered. In Clause 4.3.10 in TS 38.305, E-CID positioning method is described as follows.

Although NR E-CID positioning may utilise some of the same measurements as the measurement control system in the RRC protocol, the UE generally is not expected to make additional measurements for the sole purpose of positioning; i.e., the positioning procedures do not supply a measurement configuration or measurement control message, and the UE reports the measurements that it has available rather than being required to take additional measurement actions.

For the E-CID positioning method, UE reports available measurements to the LMF. In addition, procedure for the E-CID positioning method should remain transparent. Thus, in principle , the UE or network may not provide any additional information related to error sources to the network or UE, respectively, related to E-CID positioning. The following proposal is made by the FL. Companies are invited to provide views on whether the proposal can be supported, suggest modifications, or present additional aspects for the study.

FL Proposal 11-1

- From RAN1’s point of view, no specification impact is expected for derivation of integrity results for E-CID positioning.

Companies views:

| Company Name | Comments |
| --- | --- |
| CATT | Support |
| ZTE | Support |
| Huawei, HiSilicon | We think that for Timing advance based E-CID, gNB could provide the measurement error for the timing advance measurement, which also applies to AoA measurement. So we have the following proposal modification. FL Proposal 11-1 - From RAN1’s point of view, no additional specification impact apart from the gNB measurement errors of timing advance and UL AoA is expected for derivation of integrity results for E-CID positioning. - Note: the framework of reporting the error of timing advance and UL AoA is reused from gNB Rx – Tx time difference in Multi-RTT positioning and UL AOA in UL-AoA positioning. |

## [LOW] Issue #12 : Terminologies

In [11, 13], definitions of terminologies used for integrity principle of operation are discussed. As error sources are not agreed yet, the FL proposes to postpone the discussion to future meetings. In addition, it may be productive to discuss the definitions jointly with RAN2. The FL would like to encourage companies to discuss and provide views on this issue.

Companies views:

| Company Name | Comments |
| --- | --- |

## Other issues

If there are other issues that should be discussed, please indicate them below.

Companies views:

| Company Name | Comments |
| --- | --- |

# Summary of proposals from contributions for RAN1#110

## [1] R1-2205869 (Huawei, HiSilicon)

Proposal 1: If UE-based integrity is supported for RAT-dependent positioning methods, the error sources that have impact are those in the LPP assistance data, including

- TRP location error

- Inter-TRP synchronization error

- TRP timing error

- TRP beam antenna information error

Proposal 2: If LMF-based integrity is supported for RAT-dependent positioning methods, the error sources that have impact are those in the LPP/NRPPa measurement report, and RRC UE assistance information, including

- TRP measurement error

- UE measurement error

- UE timing error

Proposal 3: Model the ToA error as the normal distribution, and report to the LMF the error bound associated with the allocated integrity risk for the ToA measurement via the paired over-bounding Gaussian formula.

- This should apply to all DL RSTD, UE Rx – Tx time difference measurement, UL RTOA, and gNB Rx – Tx time difference.

- The reference timing for DL RSTD should also have its reported bound.

Proposal 4: The AoA error is represented by the error of the following two quantities

- cosθ'

- sinθ'sinϕ'

- Where θ' and ϕ' are the reported ZOA and AOA in the local coordinate system

Proposal 5: Model the AoA error quantities cosθ'and sinθ'sinϕ'  as the normal distribution, and report to the LMF the error bound associated with the allocated integrity risk for the AoA measurement via the paired over-bounding Gaussian formula.

## [2] R1-2205902 (ZTE)

Proposal 1: Support the following mapping between an error source and each positioning method for RAT-dependent positioning:

- For Timing-based positioning methods:

  - Measurements errors:

    - For DL-TDOA: DL PRS measurement errors

    - For Muti-RTT: DL PRS, SRS measurement errors

    - For UL-TDOA: SRS measurement errors

  - Assistance data errors:

    - For DL-TDOA: TRP location, TRP synchronization error

    - For Muti-RTT: TRP location

    - For UL-TDOA: TRP location, TRP synchronization error

- Angle-based positioning methods:

  - Measurements errors:

    - For DL-AOD: DL PRS RSRP measurement

    - For UL-AOA: SRS measurement error

  - Assistance data errors:

    - For DL-AOD: TRP location

    - For UL-AOA: TRP location

Proposal 2: Timing positioning errors are modeled as Gaussian distribution, and the GNSS liked over-bounding Gaussian formula can be reused.

Proposal 3: Angle positioning errors are modeled as the Gaussian distribution, and the GNSS liked over-bounding Gaussian formula can be reused.

Proposal 4: Support to reuse the definition of error, bound, TTA,irMinimum, irMaximum and DNU flags.

Proposal 5: LMF should configure DNU flags per TRP/per frequency layer.

## [3] R1-2205995 (Spreadtrum Communications)

Proposal 1: The following error sources for timing-based positioning methods may be considered for the Integrity of RAT-dependent positioning.

- TRP/UE measurements error sources:

  - TRP measurement errors (RTOA/ UL-RSRP/UL-RSRPP/gNB Rx-Tx time difference measurements)

  - UE measurement errors (RSTD/ DL-RSRP/DL-RSRPP/UE Rx-Tx time measurements)

- Error in assistance data:

  - Synchronization errors (between TRPs)

  - TRP location

  - TRP antenna calibration error

- TRP/UE Timing error

  - Rx/Tx/RxTx TEG

  - Difference of Rx/Tx/RxTx TEG of the same UE

  - TRP Rx/Tx/RxTx TEG

  - Differences of Rx/Tx/RxTx TEG of the same TRP

- UE/TRP clock drifting

Proposal 2: The following error sources for angle-based positioning methods may be considered for the Integrity of RAT-dependent positioning.

- TRP/UE measurements error sources:

  - TRP measurement errors (UL-AOA/UL-RSRP/UL-RSRPP)

  - UE measurement errors (DL-RSRP/DL-RSRPP)

- Error in assistance data:

  - TRP location

  - TRP antenna reference point (ARP) errors (or antenna phase center errors)

  - TRP beam antenna information

Proposal 3: In order to standardization, the criteria to become an error source should be that error source can be quantified and modeled.

Proposal 4: Multipath errors can be covered by measurement errors.

Proposal 5: To identify the mean and deviation for the overbounding model，the following approaches are presented:

- Use information from assistance data

- Use performance requirements defined in RAN4

## [4] R1-2206047 (vivo)

Proposal 1:

- For RSTD measurement with LoS indication, the measurement error follows the Gaussian distribution with μ=0, and the σ=12 NR-TimingQuality or σ=13 NR-TimingQuality.

Proposal 2:

- For RSTD measurement with NLoS indication, two options are provided for integrity for TDOA positioning.

- Option 1: The measurement error follows the Gaussian distribution with μ=0, and the σ=12 NR-TimingQuality or σ=13 NR-TimingQuality.

  - Option 2: The measurement error follows the Gaussian distribution with the σ=12 NR-TimingQuality or σ=13 NR-TimingQuality, and μ =delta t

    - μ can be indicated by the network,  or

    - μ can be calculated by estimated UE location and measured RSTD.

Proposal 3:

- If more than one RSTD measurement error can be reported for a TRP,  more than one set of mean/standard can be associated and reported for a TRP

  - For example, for a TRP, up to 4(RSTD)* 8( path) *8 (TEG) sets of mean/standard can be associated.

Proposal 4:

- For UE-based timing-based positioning, introducing the minimum TRP number for positioning integrity,  and the measurement can be seen as available if the available TRP number in the position solver equation is larger than the minimum TRP number.

Proposal 5:

- Reuse the confidence parameter for angle measurement (e.g Azmuth quality or zenith quality )to estimate the statistical distributions of LoS angle measurement errors of AoA and ZoA.

Proposal 6:

- For AoA measurement with LoS indication, the angle measurement error can be converted to distance error for integrity for AoA positioning

  - The distance error of AoA measurement follows the Gaussian distribution with μ=0, and the σ=12 r∙sina or σ=13 r∙sina. where

    - a can be achieved by confidence parameter for angle measurement (e.g Azmuth quality or zenith quality )

    - r can be achieved by timing-based measurement, or calculated by estimated UE location and TRP location.

## [5] R1-2206125 (Sony)

Proposal 1: In the context of positioning integrity, support the error sources reporting at UE and gNB, particularly the origin error sources that may affect the positioning measurement at the receiver.

Proposal 2: Consider the origin error sources related to hardware imperfections, and the conditions when performing positioning measurements.

Proposal 3: Consider multiple positioning measurements from UE/gNB to support positioning integrity.

Proposal 4: Consider a timing window where  multiple positioning measurements are performed.

Proposal 5: Consider RAT-independent integrity KPIs/results be available for adapting the RAT-dependent positioning methods and integrity reporting.

## [6] R1-2206273 (OPPO)

Proposal 1: The RSTD measurement error can be modeled as a normal distribution.

Proposal 2: For the error in UL RTOA measurement:

- Without NLOS, the measurement error in RTOA can be modeled as normal distribution.

- The measurement error of NLOS can be modeled as lognormal distribution.

Proposal 3: For the error in UE Rx-Tx time difference and gNB Rx-Tx time difference:

- Without NLOS, the measurement error can be modeled as normal distribution.

- The measurement error caused by NLOS can be modeled as lognormal distribution.

Proposal 4: TRP synchronization error for DL TDOA and UL DTOA method can be modeled as uniform distribution and the TRP/UE timing error for DL TDOA, UL TDOA and multi-RTT can be modeled as uniform distribution.

Proposal 5: DL-AoD contains the following error sources:

- Measurement error in PRS RSRP or RSRPP: it can be modeled as normal distribution.

- Error in DL PRS Tx beam configuration: it can be the error in boresight direction angle and it can be modeled as normal distribution in both azimuth angle and zenith angle.

Proposal 6:  UL-AoA method contains the following error sources:

- Measurement errors in azimuth angle and zenith angle: they can be modeled as a normal distribution.

- Error caused by NLOS path: the error in azimuth angle and zenith angle can be approximated as a normal distribution with a larger variance.

## [7] R1-2206406 (CATT)

Proposal 1: RAN1 should discuss whether the AL needs to be provided for the calculation of PL, as defined in TS 38.305, or the PL can be determined without considering AL for RAT-dependent integrity.

Proposal 2: RAN1 should discuss whether all of the error sources for NR RAT-dependent positioning can be modelled statistically as Gaussian distribution. If not, RAN1 may need to further discuss to determine the bounds for the error sources that are not modelled statistically as Gaussian distribution.

Proposal 3: It may need to clarify whether the study of the error sources for the integrity of RAT-dependent positioning considers Rel-18 RAT-dependent positioning methods.

Proposal 4: Comparing UE-based DL positioning and UE-assisted DL positioning, the same source information is expected to be used for obtaining the measurements and the location calculation. Thus, the error sources of integrity for UE can be considered to be the same if we exclude the potential errors in data communication.

•Note: Although the error sources for UE-based DL positioning and UE-assisted DL positioning are the same, some error sources may only be included in UE-based DL positioning in the specification, but may not be included in UE-assisted DL positioning, or in different forms. For example, TRP synchronization errors for UE-based positioning may be related to the TRP timing difference in DL PRS assistance data, while for UE-assisted positioning, the bounds of the TRP synchronization errors may be provided by the TRPs.

Proposal 5: For the integrity of RAT-dependent positioning, the error sources related to the generation, propagation and measurements of DL/UL positioning reference signals can be modelled as Gaussian distribution with bounded bias and deviation.

Proposal 6: The default values of the error bounds can be defined with the consideration of the minimum performance requirements defined by RAN4 (e.g., TS 38.101 and TS 38.104 for the generation of the DL/UL positioning reference signals, and TS 38.133 for the positioning measurement errors). The additional values of the error bounds can be provided by UE, TRP and LMF based on the implementation.

Proposal 7: The following error sources can be considered for timing-based positioning methods:

- gNB/TRP error sources related to timing-based positioning

  - time synchronization errors between TRPs

  - clock errors (e.g., timing offset, timing drift)

  - TRP Rx/Tx/RxTx TEG timing errors (e.g., error margins)

  - Timing measurement errors (e.g., RTOA, gNB Rx-Tx time difference measurements)

  - TRP antenna reference point (ARP) coordinate errors

  - TRP antenna phase center offset (PCO)

- UE error sources related to timing-based positioning

  - clock errors (e.g., timing offset, timing drift)

  - UE Rx/Tx/RxTx TEG timing errors (e.g., error margins)

  - Timing measurement errors (e.g., RSTD, UE Rx-Tx time difference measurements)

  - UE antenna reference point (ARP) coordinate errors

  - UE antenna phase center offset (PCO)

- Signal propagation error sources related to timing-based positioning

  - Multipath/NLOS

- PRU error sources related to timing-based positioning

  - If PRU is a gNB/TRP, all of the above gNB/TRP error sources related to timing-based positioning

  - If PRU is a UE, all of the above UE error sources related to timing-based positioning

Proposal 8 The following error sources can be considered for angle-based positioning methods:

- gNB/TRP error sources related to angle-based positioning

  - Tx beam angle errors (related to main Tx beam direction, beam width, beam pattern)

  - Rx beam angle errors (related to main Rx beam direction, beam width, beam pattern)

  - AoA measurement errors

  - TRP antenna reference point (ARP) coordinate errors

  - TRP antenna phase center offset (PCO) errors

- UE error sources related to timing-based positioning

  - Tx beam angle errors (related to main Tx beam direction, beam width, beam pattern)

  - Rx beam angle errors (related to main Rx beam direction, beam width, beam pattern)

  - DL RSRP, RSRPP measurement errors

  - UE antenna reference point (ARP) coordinate errors

  - UE antenna phase center offset (PCO) errors

- Signal propagation error sources related to timing-based positioning

  - Multipath/NLOS

Proposal 9: The following table can be used for the mapping between the error sources and the RAT-dependent positioning methods.

Table 1: Mapping of error sources and RAT-dependent positioning methods

| Error sources |  | RAT-dependent positioning methods |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL-TDOA | UL-TDOA | Multi-RTT | DL-AOD | UL-AOA | E-CID |
| gNB/ TRP | Time synchronization errors |  |  |  |  |  |  |
|  | gNB Clock drift |  |  |  |  |  |  |
|  | TRP Rx timing errors |  |  |  |  |  |  |
|  | TRP Tx timing errors |  |  |  |  |  |  |
|  | TRP RxTx timing errors |  |  |  |  |  |  |
|  | Antenna phase center offset (PCO) |  |  |  |  |  |  |
|  | RTOA measurement errors |  |  |  |  |  |  |
|  | gNB Rx-Tx time difference measurement errors |  |  |  |  |  |  |
|  | UL RSRP/RSRPP measurement errors |  |  |  |  |  |  |
| UE | UE Clock drift |  |  |  |  |  |  |
|  | UE Rx timing errors |  |  |  |  |  |  |
|  | UE Tx timing errors |  |  |  |  |  |  |
|  | UE RxTx timing errors |  |  |  |  |  |  |
|  | UE Antenna phase center offset (PCO) |  |  |  |  |  |  |
|  | RSTD measurement errors |  |  |  |  |  |  |
|  | UE Rx-Tx time difference measurement errors |  |  |  |  |  |  |
|  | DL RSRP/RSRPP measurement errors |  |  |  |  |  |  |
| Signal propagation error | Multipath/NLOS |  |  |  |  |  |  |
| Position calculation errors | TRP/UE antenna reference point (ARP) coordinate errors |  |  |  |  |  |  |

Notes: For the determination of PL for multi-RTT, there may not be the need to include the impact of Rx/Tx/RxTx timing errors. For example, if RxTx TEG IDs are reported with UE/gNB Rx-Tx time difference measurements, the RxTx time error margin can be considered, while if {Rx TEG ID, Tx TEG ID} pair are reported, the Rx time error margin and Tx time error margin can be considered.

## [8] R1-2206490 (Nokia, Nokia Shanghai Bell)

Proposal 1: RAN1 first needs to treat separately the following error sources of timing measurement (e.g., ToA, RSTD, Rx-Tx time difference).

- noise and interference signals

- Tx timing error of Tx TEG

- Rx timing error of Rx TEG

- Multipath/NLoS

Proposal 2: RAN1 first needs to treat separately the following error sources of angle measurement.

- noise and interference signals

- Multipath/NLoS

Proposal 3: RAN1 to exclude the expected RSTD and the expected AoA/AoD from error sources, as their error does not always lead to measurement error by the implementation.

Propsoal 4: RAN1 to study the above list of fault cases in addition to the previously agreed error sources.

Proposal 5: RAN1 to include in its scope the detection all measures that allow to detect the presence of interference and spoofing, and generate integrity events accordingly.

## [9] R1-2206499 (Lenovo)

Proposal 1: RAN1 to further study confirm whether NR E-CID is considered in the error source evaluation, when combined or used as input for other positioning methods.

Proposal 2: Measurement errors should be defined with respect to the errors resulting from the actual measurement procedure. RAN1 to further consider mapping the supported positioning measurements with an associated FFS on the type of measurement error, (e.g., timing-based, angle-based).

Proposal 3: Further study assistance data errors in terms of the dependency on affecting critical positioning procedures such as performing measurements and positioning calculation.

Proposal 4: Further study timing errors in terms of UE-specific and gNB-specific errors and whether it should be considered as part of measurement errors.

Proposal 5: Further study the error sources arising from the radio propagation environment including the effect of LOS/NLOS, multipath, interference, radio link/beam failures, handover, sparse network coverage.

Proposal 6: RAN1 to further discuss the relationship between feared/exceptional events with respect to the error sources.

## [10] R1-2206650 (Xiaomi)

Proposal 1: It is better to make the criteria more clear.

Proposal 2: The model of UE/TRP measurement errors might be obtained via simulation without considering the impact of other error sources.

Proposal 3: For simplicity, Gaussian distribution can be assumed for these errors that are difficult to model.

Proposal 4: Whether these error sources can be assumed to be independent needs to be discussed.

## [11] R1-2206832 (Samsung)

Proposal 1: For multipath/NLoS channel, the error of LoS/NLoS indicator can be considered as a candidate of error source.

Proposal 2: For multipath/NLoS channel, multiple path error could be regarded as a separate error source from other error sources if its statistical error modeling is feasible. Study further about its feasibility.

Proposal 3: The paired over-bounding Gaussian formula can be used to model most of the error sources for RAT dependent positioning techniques, FFS how to calculate the mean and standard deviation of each error source.

Proposal 4: Whether the statistical distribution of the residual errors can be modeled shall be used as one criteria to select an error source.

Proposal 5:For the purpose of RAN1 discussion,  the definition itself for “Error”, “Bound”, “Time-to-Alert (TTA)”, “DNU”, “Residual Risk”, “irMinimum, irMaximum” and “Correlation Times” in GNSS can be reused for RAT dependent positioning.

- The wording can be revised when it is described for RAT dependent positioning and can be up to RAN2.

## [12] R1-2206918 (CMCC)

Proposal 1: Further study the following error sources of RAT-dependent positioning:

- Errors in assistance data

  - Expected RSTD

  - Expected AoA/AoD

- Errors in gNB/UE

  - TRP antenna referent point errors

  - UE clock drifting

Proposal 2: Model the measurement errors as normal distribution.

Observation 1: Errors caused by multipath channels can be covered by measurement errors, and errors caused by NLOS scenarios can be reflected by the LOS/NLOS indicator.

Conclusion 1: No need to further identify the error sources resulting from the multipath/NLoS channel.

## [13] R1-2207088 (InterDigital)

Proposal 1: From RAN1 perspective ,study quantifiable error sources where a quantifiable error source can be bounded numerically

Proposal 2: The potential error sources for timing and angle based positioning methods are the following :

For timing-based positioning

- DL based positioning

  - TRP location (NW side)

  - Inter-TRP synchronization (NW side)

  - measurement (time of arrival)

  - TX timing (NW side)

  - RX timing (UE side)

- UL based positioning

  - ARP location

  - Measurement (timing of arrival)

  - RX timing (NW side)

  - TX timing (UE side)

- DL & UL positioning method

  - ARP location

  - Measurement (timing of arrival)

  - Tx timing (NW side)

  - Rx timing (NW side)

  - Tx timing (UE side)

  - Rx timing (UE side)

For angle-based positioning

- DL based positioning

  - TRP location

  - Measurement error (RSRP and/or RSRPP)

- UL based positioning

  - ARP location

  - Measurement error (AoA)

Proposal 3: Agree on the following definition for the error

Error: Error is the difference between the true value of a parameter (e.g. TRP location, AoA, ToA etc.) and its estimated/measured value

## [14] R1-2207239 (Qualcomm)

Proposal 1: The following Assistance Data elements are considered as error sources for integrity computation, and paired Gaussian overbounding is used to bound the errors (i.e., deviations from their true values) in these elements

5) TRP LocationInfo

6) NR-RTD-Info

7) ExpectedRSTD, ExpectedAoA, ExpectedAoD

8) NR-DL-PRS-BeamInfo and NR-TRP-BeamAntennaInfo

Proposal 2: Assistance data elements that represent discrete quantities can also be part of integrity calculations, with their own DNU flags and risk allocations, but without a bound formula for their error.

- PRS configuration parameters and NR-DL-PRS-TRP-TEG-Info are considered as error sources of this kind.

Proposal 3: Statistical parameters within assistance data, such as variances, uncertainties, and even probabilities, may be associated with DNU flags, even if they are not associated with risk allocations or bound formulas.

## [15] R1-2207284 (Sharp)

Proposal:

- For timing based positioning methods, at least the following error sources are considered for determination of integrity

  - Inter-TRP synchronization (e.g., RTD)

  - TRP location

  - TRP/UE Timing

  - TRP/UE Measurement (e.g., ToA, Rx-Tx timing difference)

    - FFS : Effect of multipath/NloS channels on TRP/UE measurement errors

- For angle-based positioning methods, at least the following error sources are considered for determination of integrity

  - TRP location

  - TRP/UE Measurement (e.g., AoA, RSRP, RSRPP)

## [16] R1-2207412 (NTT DOCOMO, INC.)

Proposal 1:

- RSTD results, DL-PRS RSRP results and DL-PRS RSRPP results can be considered as error sources for UE-assisted DL-TDOA in addition to other errors in assisted data (e.g., Inter-TRP synchronization errors).

- RSTD results, DL-PRS RSRP results, DL-PRS RSRPP results, RTD information between a reference TRP and neighbor TRPs in assistance data can be considered as error sources for UE-based DL-TDOA.

Proposal 2:

- RTOA results, UL-SRS RSRP results and UL-SRS RSRPP results can be considered as error sources for UL-TDOA in addition to other errors in assisted data (e.g., Inter-TRP synchronization errors).

Proposal 3:

- UE Rx-Tx Time Difference results, DL-PRS RSRP results and DL-PRS RSRPP results, gNB Rx-Tx Time Difference results, UL-SRS RSRP results and UL-SRS RSRPP results can be considered as error sources for Multi-RTT.

Proposal 4:

- DL-PRS RSRP results and DL-PRS RSRPP results can be considered as error sources for UE-assisted/UE-based DL-AoD.

Proposal 5:

- Azimuth/zenith angle of arrival results, UL-SRS RSRP results and UL-SRS RSRPP results can be considered as error sources for UL-AoA.

## [17] R1-2207621 (Ericsson)

Proposal 1 For the purpose of integrity computation, the UE and/or TRP measurement error for a given time-based measurement is modelled as a Gaussian distribution with zero mean.

- The standard deviation is for TRP or UE measurement error is based on the reported measurement quality.

- FFS: whether a scaling of the reported measurement quality is necessary

Proposal 2 for the purpose of integrity computations, TRP location information error is assumed to be uniformly distributed between 0 and the maximum location uncertainty for both longitude and latitude.

- The range for maximum value of location uncertainty follows the format for  reported location uncertainty in LPP/NRPPa.

Proposal 3 For the purpose of integrity computation, the sync error distribution is assumed to be Gaussian.

Proposal 4 Discuss whether a scaling of the available timing quality is necessary to obtain the standard deviation of the synch error distribution.

Proposal 5 Do not introduce further signalling support for synch quality for the purpose of integrity computation

Proposal 6 For the purpose of integrity computation, the UE/gNB RX and TX timing error, in FR1/FR2, is modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1:

- T1 is provided by the gNB or UE for integrity computation, separately for Rx and Tx timing error.

- For multiple panels, T1 is assumed to be the same for all panels

- RX and TX timing errors are generated per panel independently

Proposal 7	For the purpose of integrity computation, the UE and/or TRP measurement error for a given angle-based method measurement is modelled as a Gaussian distribution with zero mean.

- The standard deviation is for TRP or UE measurement error is based on the reported measurement quality.

- FFS: whether a scaling of the reported measurement quality is necessary

Proposal 8	Do not support NLOS as an error source in the integrity framework.

# Appendix A : Integrity Principle of Operation from TS 38.305

### 8.1.1a	Integrity Principle of Operation

For integrity operation, the network will ensure that:

P(Error > Bound for longer than TTA | NOT DNU) <= Residual Risk + IRallocation               (Equation 8.1.1a-1)

for all values of IRallocation in the range irMinimum <= IRallocation <= irMaximum

for all the errors in Table 8.1.2.1b-1, which have corresponding integrity assistance data available and where the corresponding DNU flag(s) are set to false.

The integrity risk probability is decomposed into a constant Residual Risk component provided in the assistance data as well as a variable IRallocation component that corresponds to the contribution from the Bound according to the Bound formula in Equation 8.1.1a-2. IRallocation may be chosen freely by the client based on the desired Bound, therefore the network should ensure that Equation 8.1.1a-1 holds for all possible choices of IRallocation. The Residual Risk and IRallocation components may be mapped to fault and fault-free cases respectively, but the implementation is free to choose any other decomposition of the integrity risk probability into these two components.

The validity time of the integrity bounds is set as equal to twice the SSR Update Interval for the given SSR Assistance Data message, i.e. the time period between the SSR Epoch Time and the SSR Epoch Time plus twice the SSR Update Interval in the GPS time scale.

Equation 8.1.1a-1 holds for all assistance data that has been issued that is still within its validity period. If this condition cannot be met then the corresponding DNU flag must be set.

Equation 8.1.1a-1 holds at any epochs for which Assistance Data is provided. Providing Assistance Data without the Integrity Service Alert IE or Real Time Integrity IEs is interpreted as a DNU=FALSE condition. For any bound that is still valid (within its validity time), the network ensures that the Integrity Service Alert and/or Real Time Integrity IEs are also included in the provided Assistance Data if needed to satisfy the condition in Equation 8.1.1a-1. It is up to the implementation how to handle epochs for which integrity results are desired but there are no DNU flag(s) available, e.g. the Time To Alert (TTA) may be set such that there is a "grace period" to receive the next set of DNU flags.

Only those satellites for which the GNSS integrity assistance data are provided are monitored by the network and can be used for integrity related applications.

Where:

Error: Error is the difference between the true value of a GNSS parameter (e.g. ionosphere, troposphere etc.), and its value as estimated and provided in the corresponding assistance data as per Table 8.1.2.1b-1

Bound: Integrity Bounds provide the statistical distribution of the residual errors associated with the GNSS positioning corrections (e.g. RTK, SSR etc). Integrity bounds are used to statistically bound the residual errors after the positioning corrections have been applied. The bound is computed according to the Bound formula defined in Equation 8.1.1a-2. The bound formula describes a bounding model including a mean and standard deviation (e.g. paired over-bounding Gaussian). The bound may be scaled by multiplying the standard deviation by a K factor corresponding to an IRallocation, for any desired IRallocation within the permitted range.

Bound for a particular error is computed according to the following formula:

Bound = mean + K * stdDev																	(Equation 8.1.1a-2)

K = normInv(IRallocation / 2)

irMinimum <= IRallocation <= irMaximum

where:	mean: mean value for this specific error, as per Table 8.1.2.1b-1

stdDev: standard deviation for this specific error, as per Table 8.1.2.1b-1

Time-to-Alert (TTA): The maximum allowable elapsed time from when the Error exceeds the Bound until a DNU flag must be issued.

DNU: The DNU flag(s) corresponding to a particular error as per Table 8.1.2.1b-1. Where multiple DNU flags are specified, the DNU condition in Equation 8.1.1a-1 is present when any of the flags are true (logical OR of the flags).

Residual Risk: The residual risk is the component of the integrity risk provided in the assistance data as per Table 8.1.2.1b-1. This may correspond to the fault case risk but the implementation is permitted to allocate this component in any way that satisfies Equation 8.1.1a-1.

The Residual Risk is the Probability of Onset which is defined per unit of time and represents the probability that the feared event begins. Each Residual Risk is accompanied by a Mean Duration which represents the expected mean duration of the corresponding feared event and is used to convert the Probability of Onset to a probability that the feared event is present at any given time, i.e.

P(Feared Event is Present) = Mean Duration * Probability of Onset of Feared Event		(Equation 8.1.1a-3)

irMinimum, irMaximum: Minimum and maximum allowable values of IRallocation that may be chosen by the client. Provided as service parameters from the Network according to Integrity Service Parameters.

Correlation Times: The minimum time interval beyond which two sets of GNSS assistance data parameters for a given error can be considered to be independent from one another.

Table 8.1.2.1b-1: Mapping of Integrity Parameters

| Error | GNSS Assistance Data | Integrity Fields |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Integrity Alerts | Integrity Bounds (Mean) | Integrity Bounds (StdDev) | Residual Risks | Integrity Correlation Times |
| Orbit | SSR Orbit Corrections | Real-Time Integrity (see Clause 8.1.2.1.8) | Calculated according to Equation 8.1.1a-3 | Calculated according to Equation 8.1.1a-3 | Probability of Onset of Constellation Fault Probability of Onset of Satellite Fault Mean Constellation Fault Duration Mean Satellite Fault Duration | Orbit Range Error Correlation Time Orbit Range Rate Error Correlation Time |
| Clock | SSR Clock Corrections |  | Mean Clock Residual Error Vector | Standard Deviation Clock Error |  | Clock Range Error Correlation Time Clock Range Rate Error Correlation Time |
| Code Bias | SSR Code Bias |  | Mean Code Bias Error Mean Code Bias Rate Error | Standard Deviation Code Bias Error Standard Deviation Code Bias Rate Error |  |  |
| Phase Bias | SSR Phase Bias |  | Mean Phase Bias Error Mean Phase Bias Rate Error | Standard Deviation Phase Bias Error Standard Deviation Phase Bias Rate Error |  |  |
| Ionosphere | SSR STEC Correction | Ionosphere DNU | Mean Ionospherre Error Mean Ionospherre Rate Error | Standard Deviation Ionosphere Error Standard Deviation Ionosphere Rate Error | Probability of Onset of Ionosphere Fault Mean Ionosphere Fault Duration | Ionosphere Range Error Correlation Time Ionosphere Range Rate Error Correlation Time |
| Troposphere Vertical Hydro Static Delay | SSR Gridded Corrections | Troposphere DNU | Mean Troposphere Vertical Hydro Static Delay Error Mean Troposphere Vertical Hydro Static Delay Rate Error | Standard Deviation Troposphere Vertical Hydro Static Delay Error Standard Deviation Troposphere Vertical Hydro Static Delay Rate Error | Probability of Onset of Troposphere Fault Mean Troposphere Fault Duration | Troposphere Range Error Correlation Time Troposphere Range Rate Error Correlation Time |
| TroposphereVertical WetDelay |  |  | Mean Troposphere Vertical Wet Delay Error Mean Troposphere Vertical Wet Delay Rate Error | Standard Deviation Troposphere Vertical Wet Delay Error Standard Deviation Troposphere Vertical Wet Delay Rate Error |  |  |

# Appendix B : List of feared events from TR 38.857

#### 9.4.1.1		A-GNSS Positioning Integrity Methods

The 3GPP specifications can be extended to support the determination of positioning integrity, by defining information elements and signalling procedures to transport assistance information to mitigate feared events. A summary of the feared events studied in Section 9.3 is provided in Table 9.4.1.1 below, including examples of the types of assistance information to be considered for inclusion in LPP

Table 9.4.1.1: Summary of A-GNSS feared events and integrity assistance information considerations (FFS).

NOTE: The positioning integrity assistance information IEs are FFS as part of the WI.

*NOTE: The UE or LMF are responsible for mitigating these feared events locally, outside the scope of the specifications.

| Feared Event Category | Feared Event | Examples of positioning integrity assistance information (FFS) |
| --- | --- | --- |
| 1. Feared events in the GNSS Assistance Data | Incorrect computation of the GNSS Assistance Data, e.g. software bug, corrupt or lost data | Validity or quality flags for existing assistance information |
|  | External feared event impacting the GNSS Assistance Data, e.g. satellite, atmospheric or local environment feared events (Category 3) impacting the GNSS reference stations in the GNSS correction provider’s network. |  |
| 2. Feared events during positioning data transmission | Data integrity faults | Data corruption check, e.g. CRC |
|  |  | Data Authentication / Signature |
| 3. GNSS feared events | Satellite feared events e.g. bad signal-in-space or bad broadcast navigation data | Satellite health or quality flags |
|  | Atmospheric feared events | Ionospheric indicator |
|  |  | Tropospheric indicator |
|  | Local Environment feared events, e.g. Multipath, Spoofing, Interference | Assistance information: Trustable time reference, Data Authentication / Signature, Regionalized indicator of multipath, interference, jamming, spoofing, etc |
| 4. UE feared events | GNSS receiver measurement error | e.g., GNSS-MeasurementList |
|  | Hardware faults | * |
|  | Software faults | * |
| 5. LMF feared events | Hardware faults | * |

# Appendix C : Integrity parameters from TS 37.355

integrityInfo This field provides the integrity result for the locationEstimate. - horizontalProtectionLevel provides the Horizontal Protection Level (HPL) for the locationEstimate along the semi-major axis of the error ellipse. Scale factor 0.01 metre; range 0 – 500 metres. - verticalProtectionLevel provides the Vertical Protection Level (VPL) for the locationEstimate. Scale factor 0.01 metre; range 0 – 500 metres. - achievableTargetIntegrityRisk indicates the achievable Target Integrity Risk (TIR) for which the HPL and VPL are provided. The achievable TIR is given by P=10-0.1n [hour-1] where n is the value of achievableTargetIntegrityRisk and the range is 10-1 to 10-9 per hour. If this field is absent, the achievable TIR is the same as the targetIntegrityRisk in IntegrityInformationRequest.

NOTE: 	The Protection Level (PL) is a statistical upper-bound of the Positioning Error (PE) that ensures that, the probability per unit of time of the true error being greater than the AL and the PL being less than or equal to the AL, for longer than the TTA, is less than the required TIR, i.e., the PL satisfies the following inequality: 
Prob per unit of time [((PE>AL) & (PL<=AL)) for longer than TTA] < required TIR
When the PL bounds the positioning error in the horizontal plane or on the vertical axis then it is called Horizontal Protection Level (HPL) or Vertical Protection Level (VPL) respectively.
A specific equation for the PL is not specified as this is implementation-defined. For the PL to be considered valid, it must simply satisfy the inequality above.

# Appendix D : List of error sources from R1-2205344

The following is a list of error sources shown in R1-2205344.

| Timing based positioning methods Error sources | Angle based positioning methods Error sources |
| --- | --- |
| Inter-TRP synchronization (e.g., RTD) |  |
| TRP location | TRP location |
| Expected RSTD, uncertainty in RSTD | ExpectedAoD/AoA, uncertainty in RSTD |
|  | Beam information |
| Relative position of TRPs (GDOP) | Relative position of TRPs (GDOP) |
| Inherent issues with UE capability | Inherent issues with UE capability |
| TEG margins/difference in TEG margins |  |
| Tx timing delay at UE/TRP |  |
| Rx timing delay at UE/TRP |  |
| Clock drift at UE/TRP |  |
| Antenna calibration/ARP errors | Antenna calibration/ARP errors |
| RS (e.g., low power, low bandwidth) | RS (e.g., low power, low bandwidth) |
|  | Phase error between antennas |
| Interference | Interference |
| Multipath | Multipath |
| Noise | Noise |
| UE velocity/mobility | UE velocity/mobility |
| Timing measurements at UE/TRP | Angle/RSRP measurements at UE/TRP |
| LOS indicator | LOS indicator |
| Frequency of feedback from the UE | Frequency of feedback from the UE |
| Link/handover failure | Link/handover failure |
| Power outages, failure of regular software updates to the operating system, server configuration issues, hardware failure | Power outages, failure of regular software updates to the operating system, server configuration issues, hardware failure |
| Spoofing/jamming | Spoofing/jamming |
| RS configuration | RS configuration |
| Location estimate computation | Location estimate computation |