3GPP TSG-RAN WG1 Meeting #110	R1-2207909

Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for the SRS with available slot(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (ZTE), OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - One or more avaiable slots are configured via availableSlotOffsetList, rather than via availableSlotOffset. - RRC parameter “SlotOffset” should be “slotOffset” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change “availableSlotOffset” to “availableSlotOffsetList” - Change “SlotOffset” to “slotOffset” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Ambiguity on the understanding of t when when there are multiple available slots configured for the SRS resource set(s). - Misalignment between TS 38.214 and TS 38.331 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.1	UE sounding procedure

< Unchanged parts are omitted >

The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource.

-	srs-ResourceId or SRS-PosResourceId determines SRS resource configuration identity.

-	Number of SRS ports, as defined by the higher layer parameter nrofSRS-Ports and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1.

-	Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in clause 6.4.1.4 of [4, TS 38.211].

-	Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of zero up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is slotOffset is defined by the higher layer parameter availableSlotOffsetList. The parameter availableSlotOffsetList can be configured up to 4 different values. For an SRS-PosResourceSet configured with higher layer parameter resourceType set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset for each SRS resource.

< Unchanged parts are omitted >

If the UE receives the DCI triggering aperiodic SRS in slot n and at least one resource set is configured with parameter availableSlotOffset across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource,

-	If ca-SlotOffset is configured, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot ,

-	otherwise the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot n⋅2μSRS2μPDCCH+k, where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command, respectively;

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, SRSCA and μoffset,SRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. From the first symbol carrying the SRS request DCI to the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of available slot.

-	t is configured via higher layer parameter availableSlotOffsetList with up to four different values of AvailableSlotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission. When one or more SRS resource sets across all configured BWPs in a component carrier are configured, and at least one resource set is configured with  availableSlotOffsetList parameter of more than one values, the indicated value of  AvailableSlotOffset is indicated by SOI field in DCI scheduling PUSCH/PDSCH and DCI 0_1/0_2 without data and without CSI request described in [5, TS 38.212]. The UE shall apply indicated value of  AvailableSlotOffset specifically for those sets with configured availableSlotOffsetList parameter. When one or more SRS resource sets across all configured BWPs in a component carrier are configured and at least one resource set is configured with availableSlotOffsetList parameter, and the availableSlotOffsetList parameter for each SRS resource set has only one value, the UE shall apply the configured value of availableSlotOffset specifically for those sets with configured availableSlotOffsetList parameter. For SRS resource set configured with availableSlotOffsetList parameter, each of resource set is configured with K values of AvailableSlotOffset. For SRS resource set configured without availableSlotOffsetList parameter, t = 0 is applied for each of resource set.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and none of the resource sets is configured with parameter availableSlotOffsetList across all configured BWPs in a component carrier, and if the UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, except when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , otherwise, the UE transmits aperiodic SRS in each of the triggered resource set(s) in slot Ks=n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

< Unchanged parts are omitted >