3GPP TSG-RAN WG1 Meeting #110		R1-2207952

Toulouse, France, August 22nd – 26th, 2022

Source:	vivo

Title:	Moderator summary of Email Discussion on reply LS in R1-2205735

Agenda Item:	8.16.3

Document for:	Discussion and Decision

# Introduction

This document is to summarize the discussion of LS response to the RAN2 LS in R1-2205735.

# Discussion

RAN2 has sent the following LS [1] regarding the UE behavior when the higher layer parameter sl-AllowedResourceSelectionConfig is absent:

RAN2 discussed the intended UE behaviour in a resource pool for which sl-AllowedResourceSelectionConfig is absent. RAN2’s understanding is as follows: - When the sl-AllowedResourceSelectionConfig is absent for a mode-2 transmission resource pool indicated by sl-TxPoolSelectedNormal (which can be included within sl-BWP-PoolConfig-r16/sl-BWP-PoolConfigCommon-r16 and within sl-BWP-PoolConfigPS-r17/sl-BWP-PoolConfigCommonPS-r17, as specified in TS 38.331), the UE is only allowed to use full sensing based resource selection scheme for transmission in this resource pool.

  - Round 1 discussion

Based on the contributions from the companies [2 - 12], it seems that RAN2’s understanding is aligned with RAN1’s common understanding, i.e., the UE is only allowed to perform full sensing for mode-2 resource selection in the resource pool when the sl-AllowedResourceSelectionConfig is absent.

Moderator Proposal 1: RAN2’s understanding is aligned with RAN1’s understanding, i.e., a UE is only allowed to perform full sensing for mode-2 resource selection in the resource pool in which sl-AllowedResourceSelectionConfig is absent.

Q1: Do you agree with the Moderator Proposal 1 above?

| Company | Yes or no | Comment |
| --- | --- | --- |
| DCM | YES |  |
| Intel | Yes |  |
| Ericsson | Yes |  |
| Xiaomi | Yes |  |
| Samsung | Yes |  |
| Spreadtrum | Yes |  |
| Qualcomm | Yes |  |
| CATT, GHIGH | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ZTE,Sanechips | Yes |  |
| NEC | Yes |  |

The draft LS reply is provided below assuming the answer to the Q1 is yes. Companies can provide the views and comments.

RAN1 thanks RAN2 for the LS asking intended UE behaviour in a resource pool for which sl-AllowedResourceSelectionConfig is absent. Regarding the following RAN2’s understanding: - When the sl-AllowedResourceSelectionConfig is absent for a mode-2 transmission resource pool indicated by sl-TxPoolSelectedNormal (which can be included within sl-BWP-PoolConfig-r16/sl-BWP-PoolConfigCommon-r16 and within sl-BWP-PoolConfigPS-r17/sl-BWP-PoolConfigCommonPS-r17, as specified in TS 38.331), the UE is only allowed to use full sensing based resource selection scheme for transmission in this resource pool. RAN1 confirms that it is aligned with RAN1’s understanding.

Q2: Please provide your views if you have any comments to the draft LS reply.

| Company | Comment |
| --- | --- |
| Ericsson | OK |

# Conclusion

Based on the inputs, it seems the below Moderator Proposal 1 is acceptable to all, and the corresponding draft LS reply would be provided in R1-2207953.

Moderator Proposal 1: RAN2’s understanding is aligned with RAN1’s understanding, i.e., a UE is only allowed to perform full sensing for mode-2 resource selection in the resource pool in which sl-AllowedResourceSelectionConfig is absent.

# Reference

- R1-2205735	LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	RAN2, vivo

- R1-2205847	Discussion on LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	LG Electronics

- R1-2206093	Draft reply LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	ZTE, Sanechips

- R1-2206281	Discussion on the LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	OPPO

- R1-2206282	Draft reply LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	OPPO

- R1-2206344	Draft Reply LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	CATT, GOHIGH

- R1-2206709	Draft reply LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	vivo

- R1-2207168	Reply to RAN2 LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	Qualcomm Incorporated

- R1-2207518	Discussion on LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	Huawei, HiSilicon

- R1-2207559	[Draft] Reply LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	Ericsson

- R1-2207560	Discussion on LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	Ericsson

- R1-2207580	Discussion on LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig	Xiaomi