3GPP TSG RAN WG1 #110	R1- 2207977

August 22nd – 26th, 2022

Agenda item:	7.2.5

Source: 	Moderator (Qualcomm)

Title: 	Summary #2 of Corrections for PUSCH TDRA

Document for:	Discussion and Decision

# 1	Introduction

This document summarizes companies views on the corrections proposed in [1] regarding the selection of the PUSCH TDRA table.

According to [2], pusch-TimeDomainAllocationList is not configured simultaneously with pusch-TimeDomainAllocationListDCI-0-2-r16 or pusch-TimeDomainAllocationListDCI-0-1-r16:

pusch-TimeDomainAllocationList List of time domain allocations for timing of UL assignment to UL data (see TS 38.214 [19], table 6.1.2.1.1-1). The field pusch-TimeDomainAllocationList applies to DCI formats 0_0 or DCI format 0_1 when the field pusch-TimeDomainAllocationListDCI-0-1 is not configured (see TS 38.214 [19], table 6.1.2.1.1-1 and table 6.1.2.1.1-1A). The network does not configure the pusch-TimeDomainAllocationList (without suffix) simultaneously with the pusch-TimeDomainAllocationListDCI-0-2-r16 or pusch-TimeDomainAllocationListDCI-0-1-r16 or pusch-TimeDomainAllocationListForMultiPUSCH-r16.

However, in the TDRA tables provided in Section 6.1.2.1 of [3], as copied below, these constraints are not reflected:

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-1 | pusch-Config includes pusch-TimeDomainAllocationList-ForMultiPUSCH or pusch-Config includes pusch-TimeDomainAllocationList-ForMultiPUSCH-17 | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No/Yes | Yes | - | pusch-TimeDomainAllocationListDCI-0-1 provided in pusch-Config |
| No/Yes | No/Yes | - | Yes | pusch-TimeDomainAllocationList-ForMultiPUSCH or pusch-TimeDomainAllocationList-ForMultiPUSCH -17 provided in pusch-Config |

Table 6.1.2.1.1-1B: Applicable PUSCH time domain resource allocation for DCI format 0_2 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-2 | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- |
| No | No | No | Default A |
| Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No/Yes | Yes | pusch-TimeDomainAllocationListDCI-0-2 provided in pusch-Config |

# 2         Proposals and Summary of Views

Question #1: To align the RAN1 specification with the constraint set in [2], do you agree with the following modifications:

TP to TS38.214, Section 6.1.2.1 (Rel-16):

Reasons for change: Disparity between the TDRA configuration constraints between TS 38.331 and TS 38.214.

Summary of change: Apply the configuration limitation of TS 38.331.

Consequence if not adopted: UE behavior is unclear

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI pusch-ConfigCommon includes pusch-TimeDomainAllocationList; pusch-Config includes pusch-TimeDomainAllocationList; pusch-Config includes pusch-TimeDomainAllocationListDCI-0-1; pusch-Config includes pusch-TimeDomainAllocationList-ForMultiPUSCH or pusch-Config includes pusch-TimeDomainAllocationList-ForMultiPUSCH-17; PUSCH time domain resource allocation to apply. No; No; No; No; Default A. Yes; No; No; No; pusch-TimeDomainAllocationList provided in pusch-ConfigCommon. No/Yes; Yes; No; No; pusch-TimeDomainAllocationList provided in pusch-Config. No/Yes; No/Yes; Yes; -; pusch-TimeDomainAllocationListDCI-0-1 provided in pusch-Config. No/Yes; No/Yes; -; Yes; pusch-TimeDomainAllocationList-ForMultiPUSCH or pusch-TimeDomainAllocationList-ForMultiPUSCH -17 provided in pusch-Config. Table 6.1.2.1.1-1B: Applicable PUSCH time domain resource allocation for DCI format 0_2 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI pusch-ConfigCommon includes pusch-TimeDomainAllocationList; pusch-Config includes pusch-TimeDomainAllocationList; pusch-Config includes pusch-TimeDomainAllocationListDCI-0-2; PUSCH time domain resource allocation to apply. No; No; No; Default A. Yes; No; No; pusch-TimeDomainAllocationList provided in pusch-ConfigCommon. No/Yes; Yes; No; pusch-TimeDomainAllocationList provided in pusch-Config. No/Yes; No/Yes; Yes; pusch-TimeDomainAllocationListDCI-0-2 provided in pusch-Config.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia | Yes for 0_1 No for 0_2 | It is unclear to us, why we actually are not able to operate DCI format 0_2 with a smaller set of TDRA entries (configured specifically for 0_2) and not operate 0_1 with the TDRA table from Rel-15 in pusch-config i.e. we don’t see why the 38.331 does not allow configuring pusch-TimeDomainAllocationList if configuring pusch-TimeDomainAllocationListDCI-0-2 So we are questioning if this unnecessary restriction should not be removed from 38.331 – i.e. The network does not configure the pusch-TimeDomainAllocationList (without suffix) simultaneously with the pusch-TimeDomainAllocationListDCI-0-2-r16 or pusch-TimeDomainAllocationListDCI-0-1-r16 or pusch-TimeDomainAllocationListForMultiPUSCH-r16. |
| Samsung | No | In 38.212, there are rules to determine TDRA field size based on RRC configuration as follows. So, there is no issue even if both RRC parameters are configured for 0_1/0_2 according to 214 specification. Therefore, current restriction specified in 38.331 is not necessary. 1) DCI format 0_1 - Time domain resource assignment – 0, 1, 2, 3, 4, 5, or 6 bits - If the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 is not configured and if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is not configured and if the higher layer parameter pusch-TimeDomainAllocationList is configured, 0, 1, 2, 3, or 4 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationList; - If the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 is configured or if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured, 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as ⌈log2(𝐼)⌉ bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 or pusch- TimeDomainAllocationListForMultiPUSCH; - otherwise the bitwidth for this field is determined as ⌈log2(𝐼)⌉ bits, where I is the number of entries in the default table. 2) DCI format 0_2 - Time domain resource assignment – 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as ⌈log2(𝐼)⌉ bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationListDCI-0-2 if the higher layer parameter is configured, or I is the number of entries in the higher layer parameter PUSCH-TimeDomainResourceAllocationList if the higher layer parameter PUSCH-TimeDomainResourceAllocationList is configured and the higher layer parameter pusch-TimeDomainAllocationListDCI-0-2 is not configured; otherwise I is the number of entries in the default table.. |
| ZTE | No | The two tables (Table 6.1.2.1.1-1A/B) are actually to define which TDRA table to be used when multiple TDRA tables are available. So, it seems the current restriction in 38.331 is unnecessary. Even if we follow the configuration restriction in 38.331, should we also need to delete the ‘Yes’ in the first column of the concerned row? |
| Spreadtrum |  | Agree with Nokia. |
| vivo | No | Similar view as Samsung and ZTE |
| Huawei, HiSilicon |  | The main issue here is that there is inconsistence between 38.331 and 38.214, thus either changing 331 or 38.214 is fine. From RAN1 specification perspective, as long as pusch-TimeDomainAllocationList with suffix is configured, then the one without suffix will not be used anyway, therefore it looks like that the restriction in 331 makes sense, so we slightly prefer to make the change for TS 38.214. |
| DOCOMO | No | Similar view with Samsung. |
| Qualcomm | Yes | Agree with HW/HiSi that once the parameters with suffix are configured, there is no use for pusch-TimeDomainAllocationList. Even in the tables above, when both are configured, the one with suffix is used. |

# 3        Summary

- 8 Comanies have shared their view:

- 5 companies prefer to remove the constraint in 38.331.

- 1 company prefers to remove one constraint in 38.214 and the other one in 38.331.

- 2 companies prefer to make the change in 38.214.

# 4        References

[1] R1-2207172, “Maintenance for R16 eURLLC,” Qualcomm

[2] TS 38.331, “Radio Resource Control (RRC) protocol specification”

[3] TS 38.214, “Physical layer procedures for data”