3GPP TSG-RAN WG1 Meeting #110	R1-2207983

Toulouse, France, 22nd – 26th August 2022

Agenda Item:	9.6.1

Title:	FL summary #6 on potential solutions to further reduce RedCap UE complexity

Source:	Moderator (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This feature lead (FL) summary (FLS) concerns the Rel-18 study item (SI) on further NR RedCap UE complexity reduction [1, 2, 3, 4]. This study item was preceded by a Rel-17 study item [5, 6] and a Rel-17 work item [7, 8]. The last FLS from the previous meeting can be found in [9].

This document summarizes the UE complexity reduction evaluation results provided by companies in [43] and the contributions [11] – [39] submitted to agenda item 9.6.1 and captures this email discussion:

[110-R18-RedCap] To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, Tdoc number of the moderator summary for online session, etc – Johan (Ericsson)

The section numbering in this document follows the draft TR structure in [4]. The issues in this document are tagged and color coded with High Priority or Medium Priority. The previous FLSs can be found in [40, 41, 42, 44, 45].

The following proposals (tagged FL12 in this document) are candidates for treatment online or via email discussion:

High Priority Proposal 9-1a: Adopt the following TP as a baseline text for TR 38.865 clause 9: Further NR RedCap UE complexity reduction techniques have been analyzed individually in clauses 7.2 through 7.4 as well as in different combinations in clause 7.5. The coverage impacts of the complexity reduction techniques have been analyzed in clause 8. The main observations from the coverage impact evaluations are summarized in clause 8.2.4. Based on the analysis of the studied UE complexity reduction options, RAN1 recommends option BW3 or PR3 for a Rel-18 RedCap UE. Down-selection between these two options is to be done during the WI phase or at RAN plenary. - Option BW3: - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Option PR3: - Restriction of maximum number of PRBs for PDSCH and PUSCH. - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 11 or 12. - The restricted number of PRBs in Option PR3 is a hardcoded limit. Whether or not to also introduce support for option PT1 and/or PT2 for a Rel-18 RedCap UE can be decided at RAN plenary. - Option PT1: - Relaxation of UE processing time for PDSCH/PUSCH in terms of N1 and N2 (as defined in TS 38.214) compared to those of UE processing time capability 1 - The relaxation factor for N1 and N2 is assumed to be 2 in the study. - Option PT2: - Relaxation of UE processing time for CSI in terms of Z and Z’ compared to the values defined in TS 38.214 clause 5.4 - The relaxation factor for Z and Z’ is assumed to be 2 in the study.. High Priority Proposal 7.2.5-1e: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.5: BW1 and BW2 can have significant specification impacts, considering the impacts on initial access, random access, and SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication. For BW1 and BW2, if another initial BWP need to be introduced, huge spec impacts and network impacts are expected. For BW1, the specification impacts also include the SSB presence requirements. BW3 may have smaller spec impacts compared to BW1 and BW2.. High Priority Proposal 6.1-1d: Use the following TP on evaluation methodology as baseline text for TR 38.865 clause 6.1. For cost/complexity evaluation of UE cost/complexity reduction techniques, the methodology used in TR 38.875 was used as a starting point. The reference NR devices were defined for FR1 FDD and FR1 TDD in clause 6.1 in TR 38.875. Table 6.1-1: Detailed cost breakdown for the FR1 reference NR devices Functional block; FR1 FDD (2Rx); FR1 TDD (4Rx). RF. Antenna array for FR2. Power amplifier; ~25%; ~25%. Filters; ~10%; ~15%. RF transceiver (including LNAs, mixer, and local oscillator); ~45%; ~55%. Duplexer / Switch; ~20%; ~5%. Baseband. ADC / DAC; ~10%; ~9%. FFT/IFFT; ~4%; ~4%. Post-FFT data buffering; ~10%; ~10%. Receiver processing block; ~24%; ~29%. LDPC decoding; ~10%; ~9%. HARQ buffer; ~14%; ~12%. DL control processing & decoder; ~5%; ~4%. Synchronization / cell search block; ~9%; ~9%. UL processing block; ~5%; ~5%. MIMO specific processing blocks; ~9%; ~9%. NOTE: This study assesses, from a 3GPP standpoint, the technical feasibility of further reducing the complexity of RedCap devices. Given that factors outside 3GPP responsibility influence the cost of a modem/device, this study item (and this study report) cannot guarantee, or be used as a guarantee, that such modem/device will be low-cost in the market..

FL11 Question 1-1a: Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| Spreadtrum | Sicong Zhao | sicong.zhao@unisoc.com |
| Nokia, NSB | Rapeepat Ratasuk | rapeepat.ratasuk@nokia-bell-labs.com |
| Lenovo | Yuantao Zhang | zhangyt18@lenovo.com |
| CATT | Yongqiang FEI | feiyongqiang@catt.cn |
| FUTUREWEI | Vip Desai | vipul.desai@futurewei.com |
| Xiaomi | Xuemei Qiao | qiaoxuemei@xiaomi.com |
| ZTE, Sanechips | Youjun HU | hu.youjun1@zte.com.cn |
| Samsung | Feifei Sun Seunghoon Choi | feifei.sun@samsung.com seunghoon.choi@samsung.com |
| Qualcomm | Yongjun Kwak | yongkwak@qti.qualcomm.com |
| CMCC | Lijie Hu | hulijie@chinamobile.com |
| MediaTek | Chiou-Wei Tsai | cw.tsai@mediatek.com |
| Huawei, HiSilicon | Frank LONG | frank.longyi@huawei.com |
| Sierra Wireless | Serkan Dost | sdost@sierrawireless.com |
| OPPO | Zhisong Zuo | zuozhisong@oppo.com |
| Ericsson | Sandeep Narayanan Kadan Veedu | Sandeep.narayanan.kadan.veedu@ericsson.com |
| vivo | Lihui Wang | wanglihui@vivo.com |
| NTT DOCOMO | Mayuko Okano | mayuko.okano.ca@nttdocomo.com |
| Panasonic | Shotaro Maki | maki.shotaro@jp.panasonic.com |
| Nordic | Karol Schober | karol.schober@nordicsemi.no |
| LGE | Jay KIM | jaehyung.kim@lge.com |
| Transsion | Sha.wang | Sha.wang@Transsion.com |

# 6	Evaluation methodology

## 6.1	Evaluation methodology for UE complexity reduction

RAN1#109e made the following agreements regarding the evaluation methodology for UE complexity reduction:

Agreement: For cost reduction estimation, the detailed cost breakdown for the Rel-15 reference NR devices (as provided in Table 6.1-1 in TR 38.875) is reused. Agreement: For comparison with a Rel-17 baseline when evaluating the potential Rel-18 UE complexity reduction features, - The Rel-17 RedCap UE supports 20 MHz, 1 Rx, 1 layer, DL 64QAM, UL 64QAM, FDD or TDD. - In addition, optional results for the following comparisons can also be reported: - Results for HD-FDD UEs - Results for UEs with 2 Rx - In all comparisons, the UEs being compared have the same number of antenna branches, the same number of layers, the same maximum supported modulation order, and the same duplex mode (among HD-FDD, FD-FDD, and TDD). Agreement: - The impact on memory size/cost/complexity (external to the RF and BB parts) from the studied UE complexity reduction features can be considered in the study. - This potential impact will not be included in the quantitative UE complexity reduction estimates. - L2 buffer size assumptions can be based on TS 38.306 clause 4.1.4 (“Total layer 2 buffer size for DL/UL”). - FFS whether/how to capture in the TR Agreement: For each potential Rel-18 further UE complexity reduction feature, at least the following aspects will be studied: - UE complexity reduction - Performance impacts [details FFS] - Network deployment and coexistence impacts [details FFS] - Specification impacts

According to the above agreements, the cost/complexity evaluation of further UE complexity reduction techniques should be based on the methodology used for Rel-17 RedCap in TR 38.875. For cost reduction estimation, the detailed cost breakdown for the Rel-15 reference NR devices (as provided in Table 6.1-1 in TR 38.875) is reused. In FR1, the reference Rel-15 NR UE has the following features:

- FDD: 100 MHz, 2 Rx, 2 layers, DL 256QAM, FD-FDD

- TDD: 100 MHz, 4 Rx, 4 layers, DL 256QAM

The detailed cost breakdown for the reference NR devices is provided in Table 1 (Table 6.1-1 in TR.38875). The RF-to-baseband cost ratio was assumed to be 40:60 for an FR1 UE.

Table 1: Detailed cost breakdown for the reference Rel-15 NR devices (FR1).

| Functional block | FR1 FDD (2Rx) | FR1 TDD (4Rx) |
| --- | --- | --- |
| RF |  |  |
| Power amplifier | ~25% | ~25% |
| Filters | ~10% | ~15% |
| RF transceiver (including LNAs, mixer, and local oscillator) | ~45% | ~55% |
| Duplexer / Switch | ~20% | ~5% |
| Baseband |  |  |
| ADC / DAC | ~10% | ~9% |
| FFT/IFFT | ~4% | ~4% |
| Post-FFT data buffering | ~10% | ~10% |
| Receiver processing block | ~24% | ~29% |
| LDPC decoding | ~10% | ~9% |
| HARQ buffer | ~14% | ~12% |
| DL control processing & decoder | ~5% | ~4% |
| Synchronization / cell search block | ~9% | ~9% |
| UL processing block | ~5% | ~5% |
| MIMO specific processing blocks | ~9% | ~9% |

Also, for comparison with a Rel-17 baseline when evaluating the potential Rel-18 UE complexity reduction features, the Rel-17 RedCap UE supports 20 MHz, 1 Rx, 1 layer, DL 64QAM, UL 64QAM, FDD or TDD. In all comparisons, the UEs being compared have the same number of antenna branches, the same number of layers, the same maximum supported modulation order, and the same duplex mode (among HD-FDD, FD-FDD, and TDD).

The following text proposal for the TR can be considered for TR 38.865 clause 7.1:

For cost/complexity reduction estimation, the detailed cost breakdown for the Rel-15 reference NR devices (as provided in Table 6.1-1 in TR 38.875) is reused, where the RF-to-baseband cost ratio was assumed to be 40:60 for an FR1 UE. For comparison with a Rel-17 baseline when evaluating the potential Rel-18 UE complexity reduction features, the Rel-17 RedCap UE supports 20 MHz, 1 Rx, 1 layer, DL 64QAM, UL 64QAM, FDD or TDD. In addition, cases with 2 Rx and HD-FDD features are optionally evaluated. In all comparisons, the UEs being compared have the same number of antenna branches, the same number of layers, the same maximum supported modulation order, and the same duplex mode (among HD-FDD, FD-FDD, and TDD).

FL1 High Priority Question 6.1-1a: Regarding the evaluation methodology, can the above TP be used as a baseline text for TR 38.865? If no, please suggest any necessary modification in the Comments field.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| Nordic | Y |  |
| Lenovo | Y |  |
| CATT | Y |  |
| SONY | Y |  |
| FUTUREWEI | Y |  |
| Xiaomi | Y | Small comments for the optionally evaluated cases: In addition, cases with 2 Rx , 2 MIMO layers and HD-FDD features are optionally evaluated |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Samsung | Y |  |
| Qualcomm | Y |  |
| CMCC | Y |  |
| MediaTek | Y (with clarification) | Since Reference Rel-15 UEs has different antenna/MIMO complexity and supported modulation order, the following clarification is suggested: “In all comparisons, the Rel-17 RedCap UEs and the potential Rel-18 UEs being compared have the same number of antenna branches, the same number of layers, the same maximum supported modulation order, and the same duplex mode (among HD-FDD, FD-FDD, and TDD).” |
| Huawei, Hisilicon | Y |  |
| FL3 FL4 | Based on received responses, the following proposal can be considered. The proposal has been treated in an offline session on Tuesday 23rd August. High Priority Proposal 6.1-1b: Use the following TP on evaluation methodology as baseline text for TR 38.865. For cost/complexity reduction estimation, the detailed cost breakdown for the Rel-15 reference NR devices (as provided in Table 6.1-1 in TR 38.875) is reused, where the RF-to-baseband cost ratio was assumed to be 40:60 for an FR1 UE. For comparison with a Rel-17 baseline when evaluating the potential Rel-18 UE complexity reduction features, the Rel-17 RedCap UE supports 20 MHz, 1 Rx, 1 layer, DL 64QAM, UL 64QAM, FDD or TDD. In addition, cases with 2 Rx, 2 MIMO layers, and HD-FDD features are optionally evaluated. In all comparisons, the Rel-17 RedCap UEs and the potential Rel-18 UEs being compared have the same number of antenna branches, the same number of layers, the same maximum supported modulation order, and the same duplex mode (among HD-FDD, FD-FDD, and TDD).. |
| FL5 | RAN1 made the following agreement on Tuesday 23rd August: Agreement: Use the following TP on evaluation methodology as baseline text for TR 38.865. For cost/complexity reduction estimation, the detailed cost breakdown for the Rel-15 reference NR devices (as provided in Table 6.1-1 in TR 38.875) is reused, where the RF-to-baseband cost ratio was assumed to be 40:60 for an FR1 UE. For comparison with a Rel-17 baseline when evaluating the potential Rel-18 UE complexity reduction features, the Rel-17 RedCap UE supports 20 MHz, 1 Rx, 1 layer, DL 64QAM, UL 64QAM, FDD or TDD. In addition, cases with 2 Rx, 2 MIMO layers, and HD-FDD features are optionally evaluated. In all comparisons, the Rel-17 RedCap UEs and the potential Rel-18 UEs being compared have the same number of antenna branches, the same number of layers, the same maximum supported modulation order, and the same duplex mode (among HD-FDD, FD-FDD, and TDD).. |
| FL10 | In addition to the already agreed TP above, the following TP can be considered. It is inspired by the corresponding section in TR 38.875. High Priority Proposal 6.1-1c: Use the following TP on evaluation methodology as baseline text for TR 38.865 clause 6.1. For cost/complexity evaluation of UE cost/complexity reduction techniques, the methodology used in TR 38.875 was used as a starting point. The reference NR devices were defined for FR1 FDD and FR1 TDD in clause 6.1 in TR 38.875. Table 6.1-1: Detailed cost breakdown for the FR1 reference NR devices Functional block; FR1 FDD (2Rx); FR1 TDD (4Rx). RF. Antenna array for FR2. Power amplifier; ~25%; ~25%. Filters; ~10%; ~15%. RF transceiver (including LNAs, mixer, and local oscillator); ~45%; ~55%. Duplexer / Switch; ~20%; ~5%. Baseband. ADC / DAC; ~10%; ~9%. FFT/IFFT; ~4%; ~4%. Post-FFT data buffering; ~10%; ~10%. Receiver processing block; ~24%; ~29%. LDPC decoding; ~10%; ~9%. HARQ buffer; ~14%; ~12%. DL control processing & decoder; ~5%; ~4%. Synchronization / cell search block; ~9%; ~9%. UL processing block; ~5%; ~5%. MIMO specific processing blocks; ~9%; ~9%. NOTE: This study assesses, from a 3GPP standpoint, the technical feasibility of further reducing the complexity of RedCap devices. Given that factors outside 3GPP responsibility influence the cost of a modem/device, this study item (and this study report) cannot guarantee, or be used as a guarantee, that such modem/device will be low-cost in the market.. |
| Company | Y/N | Comments |
| vivo | Y |  |
| CATT | Y |  |
| OPPO | Q | Why just cite 38.875? |
| ZTE, Sanechips | Y |  |
| Spreadtrum | Y |  |
| Intel | Y |  |
| FUTUREWEI | Y | In the first paragraph, replace “cost/complexity” with “complexity”. In table caption, replace “cost” with “complexity” |
| CMCC | Y |  |
| FL11 FL12 | Based on received responses, the following updated proposal can be considered. High Priority Proposal 6.1-1d: Use the following TP on evaluation methodology as baseline text for TR 38.865 clause 6.1. For cost/complexity evaluation of UE cost/complexity reduction techniques, the methodology used in TR 38.875 was used as a starting point. The reference NR devices were defined for FR1 FDD and FR1 TDD in clause 6.1 in TR 38.875. Table 6.1-1: Detailed cost breakdown for the FR1 reference NR devices Functional block; FR1 FDD (2Rx); FR1 TDD (4Rx). RF. Antenna array for FR2. Power amplifier; ~25%; ~25%. Filters; ~10%; ~15%. RF transceiver (including LNAs, mixer, and local oscillator); ~45%; ~55%. Duplexer / Switch; ~20%; ~5%. Baseband. ADC / DAC; ~10%; ~9%. FFT/IFFT; ~4%; ~4%. Post-FFT data buffering; ~10%; ~10%. Receiver processing block; ~24%; ~29%. LDPC decoding; ~10%; ~9%. HARQ buffer; ~14%; ~12%. DL control processing & decoder; ~5%; ~4%. Synchronization / cell search block; ~9%; ~9%. UL processing block; ~5%; ~5%. MIMO specific processing blocks; ~9%; ~9%. NOTE: This study assesses, from a 3GPP standpoint, the technical feasibility of further reducing the complexity of RedCap devices. Given that factors outside 3GPP responsibility influence the cost of a modem/device, this study item (and this study report) cannot guarantee, or be used as a guarantee, that such modem/device will be low-cost in the market.. |
| Transsion | Y |  |
| Intel | Y |  |
| Ericsson | Y | “Cost” can be changed to “Complexity” in the table text. |
| CATT | Y |  |
| Nokia, NSB | Y |  |

In the Rel-17 RedCap SI, the cost estimates in TR 38.875 [5] were averaged over the results provided by the sourcing companies [6]. Evaluation results for TR 38.865 [4] for the present Rel-18 RedCap SI have been provided in [43].

FL1 High Priority Question 6.1-2a: Should cost reduction estimates be averaged over the results provided by the sourcing companies? If no, please elaborate in the Comments field.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y | Generally fine. Further, the differences between solutions can be averaged over the results provided by the sourcing companies |
| Nokia, NSB | Y | We support to use the average as in the Rel-17 analysis. |
| Nordic | N | We believe that results to include in averaging should be approved by RAN1. |
| Lenovo | Y |  |
| CATT | Y |  |
| SONY | Y | The averaged results from the XLSX file look reasonable. We have not see R1-2207730 yet, but assume that it will reflect the XLSX file. |
| FUTUREWEI | Y |  |
| Xiaomi | Y | Can be handled in the same way in Rel-17 |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Samsung | Y | We can further discuss whether results for “option” solutions/combinations will be capture or not. The number of companies are not the same, it may not be very fair to directly capture it. |
| CMCC | Y |  |
| MediaTek | Y (with extreme values removed) | To avoid bias effect to the averaged results, we suggest to remove extreme values (maximum and minimum) for the final averaged value. Taking Medium value is another alternative to consider. |
| FL2 | Based on received responses, the following proposal can be considered. High Priority Proposal 6.1-2b: Cost reduction estimates are averaged over the results provided by the sourcing companies. |  |
| Huawei, Hisilicon | Y |  |
| FL3 | RAN1 made the following conclusion in the online session on Monday 22nd August: Conclusion: Complexity reduction estimates are averaged as R17 over the results provided by the sourcing companies |  |

# 7	UE complexity reduction features

## 7.1	Introduction to UE complexity reduction features

The following text proposal for the TR can be considered for TR 38.865 clause 7.1:

The following UE complexity reduction techniques have been studied: - Further UE bandwidth reduction - Further UE peak rate reduction - Relaxed UE processing timeline The evaluation results for each one of the studied individual UE complexity reduction techniques are captured in clauses 7.2 through 7.4, respectively. The properties of combinations of different individual UE complexity reduction techniques are described in clause 7.5. The evaluation of the potential coverage impacts is described in clause 8. Recommendations based on the evaluations are captured in clause 9.

FL1 High Priority Question 7.1-1a: Can the above TP listing the studied techniques be used as a baseline text for TR 38.865?

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| Lenovo | Y |  |
| CATT | Y in general | But remind that NO individual result for ‘relaxed UE processing timeline’ has been evaluated. So precisely, for relaxed UE processing time, study and analysis can be provided in clause 7.4, but cost evaluation related to PT options only exists in combination case (clause 7.5). |
| SONY | Y |  |
| FUTUREWEI | Y | Similar observation as CATT – PT options have only been examined in combination. |
| Xiaomi | N | We prefer to keep similar statement in the SID as stated below - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI According to the SID, relaxed UE processing timeline should be combined with other UE complexity reduction scheme |
| Intel | Y |  |
| Samsung | Y |  |
| Qualcomm | Y but | Agree with CATT |
| CMCC | Y |  |
| FL3 | Based on received responses, the following proposal can be considered. High Priority Proposal 7.1-1b: Use the following TP as baseline text for TR 38.865. The following UE complexity reduction techniques have been studied: - Further UE bandwidth reduction - Further UE peak rate reduction - Relaxed UE processing timeline The evaluation results for each one of the studied individual UE complexity reduction techniques are captured in clauses 7.2 through 7.4, respectively. For relaxed UE processing timeline, analysis is provided in clause 7.4, while cost evaluations are provided only in combination with other techniques in clause 7.5 where t The properties of combinations of different individual UE complexity reduction techniques are described in clause 7.5. The evaluation of the potential coverage impacts is described in clause 8. Recommendations based on the evaluations are captured in clause 9.. |
| FL4 | Based on offline discussion Tuesday 23rd August, the following proposal can be considered. High Priority Proposal 7.1-1c: Use the following TP as baseline text for TR 38.865. The following UE complexity reduction techniques have been studied: - Further UE bandwidth reduction - Further UE peak rate reduction - Relaxed UE processing timeline The evaluation results for each one of the studied individual UE complexity reduction techniques are captured in clauses 7.2 through 7.4, respectively. For relaxed UE processing timeline, analysis is provided in clause 7.4, while complexity evaluations are provided only in combination with other techniques in clause 7.5 where the properties of combinations of different individual UE complexity reduction techniques are described in clause 7.5. The evaluation of the potential coverage impacts is described in clause 8. Recommendations based on the evaluations are captured in clause 9.. |
| FL5 | RAN1 made the following agreement on Tuesday 23rd August: Agreement: Use the following TP as baseline text for TR 38.865. The following UE complexity reduction techniques have been studied: - Further UE bandwidth reduction - Further UE peak rate reduction - Relaxed UE processing timeline The evaluation results for each one of the studied individual UE complexity reduction techniques are captured in clauses 7.2 through 7.4, respectively. For relaxed UE processing timeline, analysis is provided in clause 7.4, while complexity evaluations are provided only in combination with other techniques in clause 7.5 where the properties of combinations of different individual UE complexity reduction techniques are described in clause 7.5. The evaluation of the potential coverage impacts is described in clause 8. Recommendations based on the evaluations are captured in clause 9.. |

RAN1#109e made the following agreements regarding the evaluation of impact on memory size/cost/complexity:

Agreement: - The impact on memory size/cost/complexity (external to the RF and BB parts) from the studied UE complexity reduction features can be considered in the study. - This potential impact will not be included in the quantitative UE complexity reduction estimates. - L2 buffer size assumptions can be based on TS 38.306 clause 4.1.4 (“Total layer 2 buffer size for DL/UL”). - FFS whether/how to capture in the TR

A few contributions [11, 14, 23] provided their views on impact of complexity reduction techniques on memory:

- The required L2 buffer size at the UE scales linearly with the UE peak data rate and with the UE bandwidth [11].

- In addition to RF and BB, memory cost will be reduced along with BW reduction [14].

- It may be difficult to have a quantitative assessment for the memory [14].

- All BW and PR reduction options that reduce max TBS size to correspond to target max 10/5 Mbps data rate, resulting in reduction of L2 buffer memory and thus size of module/chipset [23].

FL1 High Priority Question 7.1-2a: Considering the above observations on memory impacts, which ones can be considered as a baseline for the TP drafting for the TR (e.g., section 7.1)?

| Company | Impacts (1-4) | Comments |
| --- | --- | --- |
| Spreadtrum | 1，2，3，4 | For impact 2, we have the following to add(marked in red): - In addition to RF and BB, memory cost will be reduced along with BW reduction and peak data rate reduction, as the communication data storage requirement in the memory is highly related to the supported peak rate. In general, the decrease rate of storage requirement is consistent with that of peak rate reduction. For impact 4, we have the following to add (marked in red): - It may be difficult to have a quantitative assessment for the memory. The ratio values of different storage parts in the memory are highly depend on the UE implementation and the application scenarios of the chipset. For a particular case, reducing the peak rate from 80Mbps to 10Mbps can bring more than 50% cost reduction in memory. For the relationship between L2 buffer and the memory, we have the following undemanding from our perspective: L2 buffer is inside the memory. How to partition L2 buffers in Memory is up to UE implementation. For us, L2 buffer is included in the communication data storage part. The communication data storage is used to store all the cellular communication related data (L1/L2 and so on). To our knowledge, L2 buffer can be a separate storage part rather than inside the communication data storage. |
| Nokia, NSB | 1, 4 | Number 2 is similar to 1 and therefore not needed. Also, since we agreed not to provide quantitative analysis there is no need for 3. |
| SONY | 1, 4 | Agree with the Nokia view. Our view is that the reduction of L2 buffer size makes a significant difference to complexity and we should at least mention that L2 buffer size reduces complexity, even if don’t provide a quantitative assessment. |
| FUTUREWEI | 4 |  |
| Xiaomi | 1 | For 1, we are OK For 2, 3, we are open For 4, the reduction in size of module/chipset may not valid considering different implementation scheme. It needs further discussion. And the impact of reduction in the size of chipset/module should be further clarified. |
| Intel | 1,4 |  |
| Samsung | 1,3,4 |  |
| CMCC |  | We are open to discuss the impact of BW and PR reduction options on memory. |
| FL3 | Most received responses are generally fine with impacts 1 and 4. The following proposal can be considered as a baseline for the TP drafting for the TR: High Priority Proposal 7.1-2b: Use the following TP as baseline text for TR 38.865. The required L2 buffer size at the UE scales linearly with the UE peak data rate and with the UE bandwidth. All BW and PR reduction options that reduce max TBS size to correspond to target max 10 Mbps data rate result in reduction of L2 buffer memory.. |
| FL4 | Based on offline discussion on Tuesday 23rd August, the following proposal can be considered. High Priority Proposal 7.1-2c: Use the following TP as baseline text for TR 38.865. All BW and PR reduction options that reduce max TBS size to correspond to target max 10 Mbps data rate result in reduction of L2 buffer memory size. The required L2 buffer size at the UE scales linearly with the UE peak data rate and with the UE bandwidth.. |
| FL5 | RAN1 made the following agreement on Tuesday 23rd August: Agreement: Use the following TP as baseline text for TR 38.865. All BW and PR reduction options that reduce max TBS size to correspond to target max 10 Mbps data rate result in reduction of L2 buffer memory size. The required L2 buffer size at the UE scales linearly with the UE peak data rate and with the UE bandwidth.. |

## 7.2	Further UE bandwidth reduction

### 7.2.1	Description of feature

RAN1#109e made the following agreements regarding the study of further UE bandwidth reduction:

Agreement: - The following options for further UE bandwidth reduction can be studied: - Option BW1: Both RF and BB bandwidths are 5 MHz for UL and DL. - Option BW3: 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - In addition, optional results for the following option can also be reported: - Option BW2: 5 MHz BB bandwidth for all signals and channels with 20 MHz RF bandwidth for UL and DL. - At least the following cases are studied: - The resource allocation spans a bandwidth of maximum 5 MHz (Maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - It is FFS whether to study other cases. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only. Agreement: - For Options BW1, - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz. - Larger number of RBs that fit within 5 MHz can optionally be studied. - For Options BW2, - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz. - Larger number of RBs that fit within 5 MHz can optionally be studied. - For Options BW3, - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz. - Larger number of RBs that fit within 5 MHz can optionally be studied. - Relevant assumptions (e.g., regarding potential scheduling restrictions) should be reported.

Based on the above agreements, the following text proposal for the TR can be considered.

In the study, the main UE bandwidth reduction options considered for FR1 are as follows: - Option BW1: Both RF and BB bandwidths are 5 MHz for UL and DL. - Option BW2: 5 MHz BB bandwidth for all signals and channels with 20 MHz RF bandwidth for UL and DL. - Option BW3: 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. For the above bandwidth reduction options, the following aspects are considered: - The resource allocation spans a bandwidth of maximum 5 MHz (Maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only. - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz.

FL1 High Priority Question 7.2.1-1a: Can the above TP on the UE bandwidth reduction options be used as a baseline text for TR 38.865?

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| Nordic | N | We should capture also optionality of 12RB for 30kHz SCS And 20MHz RF does not make any sense to support, 8MHz RF BW is sufficient. |
| Lenovo | Y |  |
| CATT | Y in general | Suggest also capturing that 12 contiguous PRBs are also considered and evaluated (as optional one) for 30 kHz SCS case. |
| SONY | Y |  |
| FUTUREWEI | Y |  |
| Xiaomi | N | The TP can’t reflect the following agreement Agreement - In addition, optional results for the following option can also be reported: - Option BW2: 5 MHz BB bandwidth for all signals and channels with 20 MHz RF bandwidth for UL and DL. |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Samsung | BW2 needs further clarification | We’d like to further clarify the BW2, i.e., the assumption on whether UE knows which 5MHz to buffer after FFT, and whether it require full FFT size. Based on the cost break down analysis, company has very different understanding. For example, some companies think FFT/Post FFT buffer can be directly cut into 5MHz. while, some companies think FFT/Post FFT buffer require 20Mhz. |
| Qualcomm | Y | Suggest to capture optional 12 contiguous PRBs |
| CMCC | Y |  |
| MediaTek | Y | Current statement looks valid for 5MHz, while there still require RAN4 check on whether 12 RBs can be fit within the 5MHz BW. In this regard, maybe one additional note may help “Note: For 30 kHz SCS, 12 contiguous RBs are optionally studied” |
| Huawei, Hisilicon | N | For 30 kHz SCS, both 11 and 12 contiguous RBs have been considered in coverage evaluation, thus, we propose to modify the TP as following: - For 30 kHz SCS, both 11 and12 contiguous RBs are assumed to fit within the 5 MHz. |
| FL3 FL4 | Several received responses propose to add a note about optional evaluation of 12 PRBs for 30 kHz SCS. Regarding BW2, it is clarified that it is optionally considered. Also, the impact of different BW reduction options on the complexity of UE functional blocks are separately discussed in Section 7.2.2. The following proposal can be considered as a baseline for the TP drafting for the TR. The proposal was treated in offline discussion on Tuesday 23rd August. High Priority Proposal 7.2.1-1b: Use the following TP as baseline text for TR 38.865. In the study, the main UE bandwidth reduction options considered for FR1 are as follows: - Option BW1: Both RF and BB bandwidths are 5 MHz for UL and DL. - Option BW2 (optionally considered for evaluations): 5 MHz BB bandwidth for all signals and channels with 20 MHz RF bandwidth for UL and DL. - Option BW3: 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. For the above bandwidth reduction options, the following aspects are considered: - The resource allocation spans a bandwidth of maximum 5 MHz (Maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only. - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz. - Note: For 30 kHz SCS, 12 contiguous RBs are also optionally studied.. |
| FL5 | RAN1 made the following agreement on Tuesday 23rd August: Agreement: Use the following TP as baseline text for TR 38.865. In the study, the main UE bandwidth reduction options considered for FR1 are as follows: - Option BW1: Both RF and BB bandwidths are 5 MHz for UL and DL. - Option BW2 (optionally considered for evaluations): 5 MHz BB bandwidth for all signals and channels with 20 MHz RF bandwidth for UL and DL. - Option BW3: 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. For the above bandwidth reduction options, the following aspects are considered: - The resource allocation spans a bandwidth of maximum 5 MHz (Maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only. - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz. - Note: For 30 kHz SCS, 12 contiguous RBs are also optionally studied.. |

### 7.2.2	Analysis of UE complexity reduction

Many contributions discuss potential UE complexity reduction from different UE bandwidth reduction options. Depending on the bandwidth reduction option, the complexity of different UE functional blocks (listed in Table 1) can be reduced.

Based on the majority of provided cost estimates, the following observations can be made for different bandwidth reduction options.

For BW1, the main contributors of the cost reduction are the following functional blocks: - Baseband: ADC/DAC - Baseband: FFT/IFFT - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block For BW2, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block For BW3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block

Meanwhile, a few contributions mention the potential complexity reduction of ADC/DAC and FFT/IFFT blocks for BW2 [12], DL control processing & decoder for BW1 [12, 23], Filters for BW1 [23], and Power Amplifier for BW1 [12].

FL1 High Priority Question 7.2.2-1a: Can the observations listed in the text box above regarding the BW options be captured in the TR? If no, please elaborate on what modifications are needed in the Comments field.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y | Generally fine. In addition, BW2 and BW3 may have different degrees of impacts on post-FFT data buffering, we think it is necessary to point out. |
| Nokia, NSB | Y |  |
| Nordic | N | BW1: Filters reduction should be captured as well BW3: We do not see possibility for any cost reduction in Post-FFT data buffering in option BW3 |
| CATT | Y in general | For BW3, regarding post-FFT data buffering, assuming the processing time is not relaxed, the UE needs to buffer all 20 MHz data, until the DCI is decoded and the 5 MHz data can be picked out. So, the cost reduction may not be achieved, or at least not that far as BW1. |
| SONY | Y | While BW3 doesn’t allow as much reduction in post-FFT data buffering as BW2, we don’t need to state that in the context of the text proposal. The text proposal is just discussing which are the main contributors to cost reduction and is not trying to quantify or rank those contributors. |
| FUTUREWEI | Y | BW3 can allow a reduction in post-FFT data buffering especially is cross-slot scheduling is used |
| Xiaomi | Y |  |
| ZTE, Sanechips | Y with adding note | Suggest to add a note for BW2, some companies assume that ADC/DAC and FFT/IFFT also would be reduced. For BW3, FFS whether cost reduction in Post-FFT data buffering in option BW3 should be considerd. |
| Intel |  | Clarification on the complexity analysis is needed for BW2, especially, whether ADC/DAC and FFT/IFFT can be reduced for BW2. Our understanding is at least FFT/IFFT can correspond to 5MHz, otherwise, BW2 can behave same as BW3. |
| Samsung | N | This cannot explain the reason why BW 2 can have more cost reduction than BW 3. We think the description of BW 2 needs some clarification first. |
| CMCC | Y |  |
| MediaTek | Y | The characterization of cost reduction is reasonable @Nordic: Since Post-FFT data buffering is dominated by PDSCH (most symbols), BW3 constraint helps UE to reduce needed RB number for PDSCH, thereby reducing the buffer requirement compared with PR3 (where UE cannot know exact RB locations before control channel is decoded). |
| Huawei, Hisilicon | N | BW1: MIMO specific processing blocks should be captured as well, because the CSI reporting component in MIMO specific processing block will be of less complexity by reduced number of PRB and sub-bands. BW3: we do not think Post-FFT data buffering should be captured. A UE has to receive 20MHz bandwidth until the scheduling PDCCH is decoded, unless additional scheduling constraint for timeline between the PDCCH and the scheduled PDSCH is applied. |
| FL3 | Regarding post-FFT data buffering, the received responses express that there be different degrees of impacts with different BW reduction options. Therefore, a clarification note is added to capture this aspect. Additional notes are added for BW2 and BW3 considering views from a few companies. Note that the presented lists for different BW options show the main contributors of the cost reduction which have been identified by most of the companies. The following proposal can be considered as a baseline for the TP drafting for the TR. High Priority Proposal 7.2.2-1b: Use the following TP as baseline text for TR 38.865. For BW1, the main contributors of the cost reduction are the following functional blocks: - Baseband: ADC/DAC - Baseband: FFT/IFFT - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block For BW2, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block For BW3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block Note 1: BW1, BW2, and BW3 may have different degrees of impacts on the post-FFT data buffering. Note 2: for BW2, some sources assume that the complexity ADC/DAC and FFT/IFFT bocks can also be reduced. Note 3: for BW1, one source shows cost reduction for RF filters and once source shows cost reduction for MIMO specific processing block.. |
| FL4 | Based on offline discussion on Tuesday 23rd August, the following proposal can be considered. High Priority Proposal 7.2.2-1c: Use the following TP as baseline text for TR 38.865. For BW1, the main contributors of the cost reduction are the following functional blocks: - Baseband: ADC/DAC - Baseband: FFT/IFFT - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW2, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block Note 1: BW1, BW2, and BW3 may have different degrees of impacts on the post-FFT data buffering depending on the scheduling aspects (cross-slot scheduling, RF retuning, etc.). [Note 2: For BW2, some sources have assumed that the frequency locations of control and data channels are known in advance and observe that the complexity of the ADC/DAC and FFT/IFFT blocks can also be reduced.] Note 3: For BW1, one source shows complexity reduction for RF filters and one source shows complexity reduction for MIMO specific processing block.. |
| FL5 FL6 FL7 | High Priority Question 7.2.2-1d: Companies are invited to comment on how the square brackets in the following agreement (made on Tuesday 23rd August) can be resolved. Agreement: Use the following TP as baseline text for TR 38.865. For BW1, the main contributors of the cost reduction are the following functional blocks: - Baseband: ADC/DAC - Baseband: FFT/IFFT - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW2, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block Note 1: BW3 and [BW2] may have different degrees of impacts on the post-FFT data buffering depending on the scheduling aspects (cross-slot scheduling, RF retuning, etc.). [Note 2: For BW2, some sources have assumed that the frequency locations of control and data channels are known in advance and observe that the complexity of the ADC/DAC and FFT/IFFT blocks can also be reduced.] Note 3: For BW1, one source shows complexity reduction for RF filters and one source shows complexity reduction for MIMO specific processing block. Note 4: BW1 and BW2 may have different degrees of impacts on the downlink control processing and decoder depending on the CCE and BD complexity reduction.. |
| CATT | For [BW2] in Note 1, maybe we can try to remove the bracket directly. Anyway, for both BW2 and BW3, the post-FFT data buffering indeed depends on whether the frequency location of 5 MHz can be known before decoding DCI for FDRA field. This can be realized by, e.g. cross-slot scheduling, or have some predefine rule for the frequency location of 5 MHz for data transmission. For Note 2, we think at least the complexity of ADC/DAC cannot be reduced since RF BW=20MHz need to be handled (e.g. sampling at a frequency that supporting RF BW=20MHz). Otherwise it has no difference with BW1. For FFT/IFFT, may be OK… with the newly added assumption. Anyway, for Note 2, we can consider changing ‘can also be reduced’ into ‘might also be reduced’ to reflect the contradictory to some degree, if this can help progress. |  |
| Samsung | First of all, we think even without cross-slot scheduling, BW 3 can have some cost saving on post-FFT data buffering. Both in eMTC study and UE power saving study. In eMTC study, only few OFDM symbols length duration is assumed for PDCCH decoding. Therefore, UE doesn’t have to buffer full 20MHz for all 14 symbols for PDSCH decoding. In R16 UE power saving study, mic sleep is assumed for normal UE after decoding PDCCH, while cross slot scheduling can allow more sleep right after buffer PDCCH. Secondly, for Note 1, we think BW 2 shall be kept. It is still unclear on the assumption of BW 2, for example, the band might be pre-known to the UE, or it might be dynamically indicated by the PDCCH. this requires further clarification. And same assumption should be used for spec/performance analysis. Finally, for Note 2, Note 2A: For BW2, some sources have assumed that the frequency locations of control and data channels are known in advance Note 2B: For BW2, some sources assume that the complexity of the ADC/DAC and FFT/IFFT blocks can also be reduced. To align with this Note 2B, in the average, the cost saving on ADC/DAC and FFT/IFFT shall not be averaged. |  |
| FUTUREWEI | For note 1, we have a similar understanding as CATT. We can try to remove the square bracket. For BW2, we considered its operation as analogous to MTC where there was a 6 RB active band within 20 MHz. For MTC, an implementation could have used an ADC operating for 1.4 MHz BW or a 20 MHz BW. Based on this analogy, we should remove the brackets. We can be ok with “might also be reduced” |  |
| Xiaomi2 | For note 1, we have same view as CATT. For note 2, we think it can be kept for the collected result clarification. But, even if the location for control and data channel is known in advance, how to design the FFT/IFFT and/or ADC/DAC block when the control channel and data channel are located with two end point of 20MHZ? |  |
| OPPO | Note1, we prefer to include BW2. The pos-FFT saving is really depending on some restriction. For us, most likely the tuning can not be done in one slot. You have DL control decoding and also retuning RF time. Note2. We think this is reasonable to assume no ADC and post-FFT saving, except you have specially processing. We don’t understand how the 20 to 8 MHz adaptation is calculated. Even that is possible it comes with lots of other constraints. So, we prefer to remove the square bracket of Note2. |  |
| Nordic | Note 1 We agree with CATT, we should capture “For BW3 and BW2 the degree of post-FFT data buffering depends on assumption whether PDSCH location is known beforehand PDSCH reception or not”. Note 2 Our assumption was that 20MHz RF UE may reduce RF to 8MHz to cover SSB and 5MHz BWP, thus we reduced ADC and FFT this resulting in reduction of FFT/ACK. @OPPO: what constraints? Yes, hopping is not possible, but it is still valid BW2 implementation. |  |
| vivo | We share CATT’s views to keep BW2 for Note1. For Note2, our understanding is also same as CATT that ADC/DAC cannot be reduced since RF BW=20MHz need to be handled (e.g. sampling at a frequency that supporting RF BW=20MHz). UE knows the frequency locations of control and data channels in advance is to help the Post-FFT data buffering which is already captured in Note1. About the usage for BW2, it provides the possibility to support fast RF retuning to improve the scheduling flexibility and diversity gain. |  |
| Intel | Note1: same view as CATT Note2: Due to 20MHz RF, ADC/DAC should be for 20MHz hence no reduction. On the other hand, it would be possible to reduce complexity of FFT/IFFT. We think the bracket can be removed with a change ‘ADC/DAC and/or FFT/IFFT blocks’ |  |
| SONY | We are not so keen on Note 1. Even without cross-slot scheduling, post-FFT data buffering can be reduced since the UE only has to buffer FFT samples until the PDCCH is decoded (similar comment to Samsung). Maybe we could change to something like: “For BW3 and BW2 the degree of post-FFT data buffering depends on assumption the time for which 20MHz bandwidth needs to be buffered before PDSCH location is known”. |  |
| ZTE, Sanechips | For Note 1, BW3 needs to buffer 20MHz BW PDSCH since UE does not know the frequency location of 5MHz BW PDSCH. Hence, the complexity on post-FFT data buffering of BW3 is different from BW1 and BW2. So [BW2] could be modified to BW1/BW2. For Note 2, baseband has the bandwidth 5MHz and can only process 5MHz data. In idle mode, PDCCH is indicated by PBCH and data channel is indicated by PDCCH within CORESET#0 bandwidth. In connected mode, UE-specific BWP can be configured for Rel-18 RedCap UE. Therefore, the frequency locations of control and data channels can be known by Rel-18 RedCap UE, which is similar to BW1. So the ADC/DAC and FFT/IFFT blocks can also be reduced. |  |
| CMCC | For Note 1, in our understanding, the active BWP will always be strict to smaller than 5MHz, UE have to know where to monitor PDCCH before the monitoring slot. So, the location of active BWP will be known by dynamic or semi-persistent indication. Then the post-FFT data buffering will be always restrict within 5MHz. So BW2 can be removed. For Note 2, similar view as CATT. Note 1: BW3 and [BW2] may have different degrees of impacts on the post-FFT data buffering depending on the scheduling aspects (cross-slot scheduling, RF retuning, etc.). [Note 2: For BW2, some sources have assumed that the frequency locations of control and data channels are known in advance and observe that the complexity of the ADC/DAC and FFT/IFFT blocks can also be reduced.] |  |
| FL8 | Based on offline discussion on Wednesday 24th August and received responses, the following proposal can be considered. High Priority Proposal 7.2.2-1e: Adopt the following TP as a baseline text for TR 38.865. For BW1, the main contributors of the cost reduction are the following functional blocks: - Baseband: ADC/DAC - Baseband: FFT/IFFT - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW2, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block Note 1: BW3 and [BW2] may have different degrees of impacts on the post-FFT data buffering depending on the scheduling aspects (cross-slot scheduling, RF retuning, etc.). [Note 2: For BW2, some sources have assumed that the frequency locations of control and data channels are known in advance and observe that the complexity of the ADC/DAC and FFT/IFFT blocks can might also be reduced.] Note 3: For BW1, one source shows complexity reduction for RF filters and one source shows complexity reduction for MIMO specific processing block. Note 4: BW1 and BW2 may have different degrees of impacts on the downlink control processing and decoder depending on the CCE and BD complexity reduction.. |
| FL10 | Based on received responses and on discussion in the online session on Thursday 25th August, the following updated proposal can be considered. High Priority Proposal 7.2.2-1g: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.2. For BW1, the main contributors of the cost reduction are the following functional blocks: - Baseband: ADC/DAC - Baseband: FFT/IFFT - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW2, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block Note 1: BW3 and [BW2] may have different degrees of impacts on the post-FFT data buffering depending on the scheduling aspects (cross-slot scheduling, RF retuning, etc.). [Note 2: For BW2, some sources have assumed that the frequency locations of control and data channels are known in advance and observe that the complexity of the ADC/DAC and FFT/IFFT blocks can might also be reduced.] Note 3: For BW1, one source shows complexity reduction for RF filters and one source shows complexity reduction for MIMO specific processing block. Note 4: BW1 and BW2 may have different degrees of impacts on the downlink control processing and decoder depending on the CCE and BD complexity reduction.. |
| Company | Y/N | Comments |
| LGE |  | For Note 1, we think the UE has to at least know where to monitor the PDCCH in advance, as stated by CATT. And unlike BW3 option, the UE only has to buffer the post-FFT data within the DL BWP which is no larger than 5 MHz for BW2. Some companies seem to have different assumptions on BW2, then we propose to separate the Note1 into two parts. For example, Note 1a: BW3 and [BW2] may have different degrees of impacts on the post-FFT data buffering depending on the scheduling aspects (cross-slot scheduling, RF retuning, etc.). Note 1b: BW2 may have different degrees of impacts on the post-FFT data buffering … Proponents can explain how the BW2 may have different degrees of impacts on the post-FF data buffering. We believe the clarification in Note 1b for BW2 would help make progress. For Note 2, similar to CATT, it is unclear to us how to reduce the complexity of the ADC/DAC blocks and FFT/IFFT blocks with the 20 MHz RF bandwidth intact. But, can live with the latest update on Note 2. |
| Nokia, NSB | Y |  |
| vivo | Y |  |
| CATT |  | As mentioned before, for Note 2, we think at least the complexity of ADC/DAC cannot be reduced since RF BW=20MHz need to be handled (e.g. sampling at a frequency that supporting RF BW=20MHz). If, by implementation or some method, the UE use other RF BW (e.g. RF BW = 8 MHz), it is actually a new BW option (like BW1.5?), which is between BW1 and BW2, not original definition of BW2…. Anyway, we are not keen on it. |
| OPPO | Y |  |
| ZTE, Sanechips |  | For Note 1, we can remove [BW2] for simplicity and add the clarification for BW2 in Note2. For Note 2, we think ADC/DAC, FFT/IFFT, Post-FFT data buffering can be reduced. For example, the sampling rate for the baseband ADC/DAC part is designed based on the maximum bandwidth of a physical channel. For BW2, if the maximum bandwidth of a physical channel is 5MHz, then the sampling rate for the baseband ADC/DAC part can be reduced. Alternatively, based on some filters, the UE can only receive the 5MHz data after the RF, and the ADC/DAC part can be based on 5MHz also. Therefore, it is possible to reduce the ADC/DAC part complexity. The RF bandwidth only defines the upper limit of processing capability for a component. It does not mean the UE should receive and process 20MHz data. So, we suggest the following updates: Note 1: BW3 and [BW2] may have different degrees of impacts on the post-FFT data buffering depending on the scheduling aspects (cross-slot scheduling, RF retuning, etc.). [Note 2: For BW2, some sources have assumed that the frequency locations of control and data channels are known in advance and observe that the complexity of the ADC/DAC, and FFT/IFFT blocks and Post-FFT data buffering can might also be reduced, which depend on the UE implementation.] |
| Intel | Y | For Note 1, prefer to keep BW2 (removing the bracket). Our assumption is the location of 5MHz is known to UE so BW2 can have better complexity reduction than BW3 in post-FFT data buffering For Note 2, agree to remove bracket. Since it just says ‘might also be reduced’, it is fine to mention ADC/DAC and FFT/IFFT |
| FUTUREWEI | Y |  |
| CMCC |  | We share similar view as LGE for Note1. The active BWP of both PDCCH and PDSCH is smaller than 5MHz, and will be known before each slot for monitoring PDCCH. The case that UE does not know where to buffer PDSCH only happens when PDCCH has changes the active BWP of PDSCH scheduled in the same slot. But what is the motivation of doing this? If for frequency selective, why not change the BWP of PDCCH together with PDSCH before the slot? So we also think the post-FFT data buffering will be always reduced for BW2, so it can be removed in Note1. |
| FL11 | The online session on Friday 26th August made the following agreement: Agreement: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.2. For BW1, the main contributors of the cost reduction are the following functional blocks: - Baseband: ADC/DAC - Baseband: FFT/IFFT - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW2, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: DL control processing & decoder - Baseband: UL processing block For BW3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block Note 1: BW3 and [BW2] may have different degrees of impacts on the post-FFT data buffering depending on the scheduling aspects (cross-slot scheduling, RF retuning, etc.). [Note 2: For BW2, some sources have assumed that the frequency locations of control and data channels are known in advance and observe that the complexity of the ADC/DAC and FFT/IFFT blocks can might also be reduced.] Note 3: For BW1, one source shows complexity reduction for RF filters and one source shows complexity reduction for MIMO specific processing block. Note 4: BW1 and BW2 may have different degrees of impacts on the downlink control processing and decoder depending on the CCE and BD complexity reduction.. |

Overall, companies generally agree that there is no significant further cost reduction in the RF part from further UE bandwidth reduction and that the main cost reduction is in the BB part.

FL1 High Priority Question 7.2.2-2a: Can the following observation regarding UE complexity reduction be captured in the TR?

- For the UE bandwidth reduction options BW1, BW2, and BW3, the cost reduction is mainly in the BB part, and there is no significant cost reduction in the RF part.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| Nordic | N | In BW1 there is possibility to reduce cost for BW1 in RF |
| Lenovo | Y |  |
| CATT | Y |  |
| SONY | Y |  |
| FUTUREWEI | Y, but | For BW1, there may be a possibility to reduce RF complexity |
| Xiaomi | Y |  |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Samsung | Y |  |
| Qualcomm | Y |  |
| CMCC | Y |  |
| MediaTek | Y | For RF bandwidth below 20 MHz, we do not see the cost reduction benefit (at least for DL part of RF). |
| Huawei, Hisilicon | Y |  |
| FL3 FL4 | Two received responses indicate that there may be a possibility to reduce RF complexity. The following proposal can be considered as a baseline for the TP drafting for the TR. High Priority Proposal 7.2.2-2b: Use the following TP as baseline text for TR 38.865. For the UE bandwidth reduction options BW1, BW2, and BW3, the cost reduction is mainly in the BB part, and although there may be a possibility to reduce RF complexity, there is no significant cost reduction in the RF part.. |
| FL5 | RAN1 made the following agreement on Tuesday 23rd August: Agreement: Use the following TP as baseline text for TR 38.865. For the UE bandwidth reduction options BW1, BW2, and BW3, the cost reduction is mainly in the BB part, and although there may be a possibility to reduce RF complexity, there is no significant cost reduction in the RF part.. |

Another general observation is that Rel-17 RedCap can already provide significant cost saving compared to Rel-15 reference UEs, and the additional cost saving by further UE bandwidth reduction in Rel-18 is limited.

FL1 High Priority Question 7.2.2-3a: Can the following observation regarding UE complexity reduction be captured in the TR?

- Rel-17 RedCap can already provide significant cost saving compared to Rel-15 reference UEs, and the additional cost saving by further UE bandwidth reduction in Rel-18 is limited.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB |  | We think it’s sufficient to just have the tables. After all, this observation is not there for the PR and PT option, even though the BW options provide the largest complexity reduction. |
| Nordic | N | We prefer to discuss relative cost reduction compared to R17. |
| Lenovo |  | We have same view with Nokia. |
| CATT | Y |  |
| SONY | N | Our view is that there is significant complexity saving relative to Rel-17. In the analysis, there is essentially a floor in the amount of possible complexity saving that is possible due to the assumed complexity of blocks like SSB and MIMO functions. If more realistic assumptions on the SSB and MIMO function complexity had been made in the first place, the complexity reduction benefits of eRedCap relative to Rel-17 would shine through. |
| FUTUREWEI | N | We should avoid using vague terms “significant” and “limited” when numerical results are available. There should be similar observations for PR, PT, and the combinations. |
| Xiaomi | N | This statement is a little bit subjective. In our view, in the TR, we just need to reflect how much cost saving is achieved in objective way. |
| ZTE, Sanechips |  | We can further visit it after we have all the complexity reduction tables. |
| Intel |  | Instead of saying the cost saving is limited, we prefer to directly capture the average cost reduction or a range of cost reduction for each option or combination of options |
| Samsung | Y |  |
| Qualcomm | N | We do not need this observation as the cost saving gain diverges for different companies. We can simply have tables summarizing companies’ inputs. |
| CMCC | Y |  |
| MediaTek | Y (with clarification) | We observe the relative cost reduction w.r.t. R17 is also limited (up to ~10%) and are supportive of the observation. But, since combined complexity reduction schemes can improve the overall saving, it is more precise to include “only” to the observation: Rel-17 RedCap can already provide significant cost saving compared to Rel-15 reference UEs, and the additional cost saving by only further UE bandwidth reduction in Rel-18 is limited. |
| Huawei, Hisilicon | N | This SI targets at low-end devices with peak data rate 10Mbps only. For such low-end IoT devices, in our view, the additional cost saving by further UE bandwidth reduction is sufficient and valuable. Therefore, we feel the proposed text seems misleading by stating “limited”. |
| FL3 | While some companies support this observation, some others prefer to directly capture the average cost reduction or a range of cost reduction. Therefore, we can further visit this observation after we have all the complexity reduction tables. |  |

The tables below show the average value of the initial cost evaluation results captured in [43]. Specifically, results are provided for Rel-17 RedCap and Rel-18 eRedCap cases with FD-FDD 1Rx, TDD 1Rx, HD-FDD 1Rx, FD-FDD 2Rx, TDD 2Rx, and HD-FDD 2Rx.

Note that the reference Rel-15 NR UE has the following features:

- FDD: 100 MHz, 2 Rx, 2 layers, DL 256QAM, FD-FDD

- TDD: 100 MHz, 4 Rx, 4 layers, DL 256QAM

FD-FDD 1Rx

| Reduced UE bandwidth | Rel-15 reference | Rel-17 RedCap | BW1 | BW2 | BW3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.12% | ## 24.02% | ## 24.64% | ## 24.09% |
| ## RF: Filters | ## 10% | ## 5.06% | ## 4.72% | ## 5.12% | ## 5.06% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 45% | ## 23.76% | ## 23.52% | ## 24.30% | ## 23.76% |
| ## RF: Duplexer / Switch | ## 20% | ## 19.52% | ## 19.52% | ## 19.57% | ## 19.52% |
| # RF: Total | # 100% | # 72.46% | # 71.78% | # 73.63% | # 72.43% |
| ## BB: ADC / DAC | ## 10% | ## 1.30% | ## 0.50% | ## 0.99% | ## 1.27% |
| ## BB: FFT/IFFT | ## 4% | ## 0.67% | ## 0.21% | ## 0.45% | ## 0.65% |
| ## BB: Post-FFT data buffering | ## 10% | ## 1.05% | ## 0.36% | ## 0.46% | ## 0.67% |
| ## BB: Receiver processing block | ## 24% | ## 4.42% | ## 2.00% | ## 1.86% | ## 2.07% |
| ## BB: LDPC decoding | ## 10% | ## 1.29% | ## 0.51% | ## 0.50% | ## 0.51% |
| ## BB: HARQ buffer | ## 14% | ## 1.46% | ## 0.45% | ## 0.45% | ## 0.45% |
| ## BB: DL control processing & decoder | ## 5% | ## 4.73% | ## 3.98% | ## 3.95% | ## 4.52% |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.61% | ## 4.55% | ## 4.59% | ## 4.58% |
| ## BB: UL processing block | ## 5% | ## 2.69% | ## 1.55% | ## 1.54% | ## 1.69% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 4.04% | ## 3.77% | ## 3.85% | ## 3.91% |
| # BB: Total | # 100% | # 26.26% | # 17.88% | # 18.65% | # 20.31% |
| RF+BB: Total | 100% | 44.74% | 39.44% | 40.65% | 41.15% |

TDD 1Rx

| Reduced UE bandwidth | Rel-15 reference | Rel-17 RedCap | BW1 | BW2 | BW3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.08% | ## 23.97% | ## 24.56% | ## 24.05% |
| ## RF: Filters | ## 15% | ## 3.94% | ## 3.86% | ## 4.08% | ## 3.94% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 55% | ## 19.05% | ## 18.78% | ## 19.98% | ## 19.05% |
| ## RF: Duplexer / Switch | ## 5% | ## 4.97% | ## 4.97% | ## 4.98% | ## 4.97% |
| # RF: Total | # 100% | # 52.04% | # 51.58% | # 53.59% | # 52.01% |
| ## BB: ADC / DAC | ## 9% | ## 0.76% | ## 0.30% | ## 0.57% | ## 0.73% |
| ## BB: FFT/IFFT | ## 4% | ## 0.40% | ## 0.14% | ## 0.27% | ## 0.38% |
| ## BB: Post-FFT data buffering | ## 10% | ## 0.59% | ## 0.22% | ## 0.28% | ## 0.36% |
| ## BB: Receiver processing block | ## 29% | ## 3.21% | ## 1.54% | ## 1.48% | ## 1.56% |
| ## BB: LDPC decoding | ## 9% | ## 0.79% | ## 0.33% | ## 0.34% | ## 0.33% |
| ## BB: HARQ buffer | ## 12% | ## 0.79% | ## 0.40% | ## 0.23% | ## 0.40% |
| ## BB: DL control processing & decoder | ## 4% | ## 3.68% | ## 3.15% | ## 3.19% | ## 3.55% |
| ## BB: Synchronization / cell search block | ## 9% | ## 2.49% | ## 2.44% | ## 2.45% | ## 2.45% |
| ## BB: UL processing block | ## 5% | ## 2.69% | ## 1.54% | ## 1.50% | ## 1.70% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 2.27% | ## 2.04% | ## 2.07% | ## 2.21% |
| # BB: Total | # 100% | # 17.66% | # 12.08% | # 12.40% | # 13.68% |
| RF+BB: Total | 100% | 31.41% | 27.88% | 28.88% | 29.01% |

HD-FDD 1Rx

| Reduced UE bandwidth | Rel-15 reference | Rel-17 RedCap | BW1 | BW2 | BW3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.50% | ## 24.50% | ## 24.50% | ## 24.50% |
| ## RF: Filters | ## 10% | ## 5.17% | ## 4.64% | ## 5.17% | ## 5.17% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 45% | ## 24.46% | ## 24.06% | ## 24.46% | ## 24.46% |
| ## RF: Duplexer / Switch | ## 20% | ## 3.68% | ## 3.68% | ## 3.68% | ## 3.68% |
| # RF: Total | # 100% | # 57.81% | # 56.87% | # 57.81% | # 57.81% |
| ## BB: ADC / DAC | ## 10% | ## 1.25% | ## 0.54% | ## 1.03% | ## 1.26% |
| ## BB: FFT/IFFT | ## 4% | ## 0.71% | ## 0.25% | ## 0.44% | ## 0.72% |
| ## BB: Post-FFT data buffering | ## 10% | ## 1.10% | ## 0.42% | ## 0.50% | ## 0.66% |
| ## BB: Receiver processing block | ## 24% | ## 4.22% | ## 1.92% | ## 1.92% | ## 2.04% |
| ## BB: LDPC decoding | ## 10% | ## 1.41% | ## 0.58% | ## 0.58% | ## 0.58% |
| ## BB: HARQ buffer | ## 14% | ## 1.61% | ## 0.52% | ## 0.52% | ## 0.52% |
| ## BB: DL control processing & decoder | ## 5% | ## 4.70% | ## 3.84% | ## 3.84% | ## 4.49% |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.68% | ## 4.63% | ## 4.63% | ## 4.68% |
| ## BB: UL processing block | ## 5% | ## 3.10% | ## 1.79% | ## 1.78% | ## 1.91% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 3.81% | ## 3.56% | ## 3.58% | ## 3.81% |
| # BB: Total | # 100% | # 26.58% | # 18.05% | # 18.82% | # 20.70% |
| RF+BB: Total | 100% | 39.07% | 33.58% | 34.41% | 35.60% |

FD-FDD 2Rx

| Reduced UE bandwidth | Rel-15 reference | Rel-17 RedCap | BW1 | BW2 | BW3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.87% | ## 24.87% | ## 25.00% | ## 24.87% |
| ## RF: Filters | ## 10% | ## 10.00% | ## 9.02% | ## 9.49% | ## 9.56% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 45% | ## 42.47% | ## 41.51% | ## 42.63% | ## 42.47% |
| ## RF: Duplexer / Switch | ## 20% | ## 20.00% | ## 20.00% | ## 20.00% | ## 20.00% |
| # RF: Total | # 100% | # 97.33% | # 95.40% | # 97.12% | # 96.90% |
| ## BB: ADC / DAC | ## 10% | ## 2.12% | ## 1.03% | ## 1.52% | ## 2.13% |
| ## BB: FFT/IFFT | ## 4% | ## 1.25% | ## 0.47% | ## 0.86% | ## 1.28% |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.17% | ## 0.84% | ## 1.11% | ## 1.52% |
| ## BB: Receiver processing block | ## 24% | ## 8.97% | ## 4.47% | ## 4.78% | ## 4.60% |
| ## BB: LDPC decoding | ## 10% | ## 2.71% | ## 1.18% | ## 1.19% | ## 1.18% |
| ## BB: HARQ buffer | ## 14% | ## 2.53% | ## 0.97% | ## 0.91% | ## 0.97% |
| ## BB: DL control processing & decoder | ## 5% | ## 4.93% | ## 4.13% | ## 4.25% | ## 4.93% |
| ## BB: Synchronization / cell search block | ## 9% | ## 8.61% | ## 8.47% | ## 8.38% | ## 8.61% |
| ## BB: UL processing block | ## 5% | ## 3.20% | ## 1.99% | ## 2.04% | ## 2.08% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 7.33% | ## 6.01% | ## 6.48% | ## 7.33% |
| # BB: Total | # 100% | # 43.84% | # 29.57% | # 31.52% | # 34.65% |
| RF+BB: Total | 100% | 65.24% | 55.90% | 57.76% | 59.55% |

TDD 2Rx

| Reduced UE bandwidth | Rel-15 reference | Rel-17 RedCap | BW1 | BW2 | BW3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.44% | ## 24.44% | ## 25.00% | ## 24.44% |
| ## RF: Filters | ## 15% | ## 7.69% | ## 7.11% | ## 7.75% | ## 7.69% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 55% | ## 33.61% | ## 31.83% | ## 33.90% | ## 33.14% |
| ## RF: Duplexer / Switch | ## 5% | ## 4.99% | ## 4.99% | ## 5.00% | ## 4.99% |
| # RF: Total | # 100% | # 70.72% | # 68.36% | # 71.65% | # 70.25% |
| ## BB: ADC / DAC | ## 9% | ## 1.13% | ## 0.56% | ## 0.89% | ## 1.13% |
| ## BB: FFT/IFFT | ## 4% | ## 0.66% | ## 0.25% | ## 0.52% | ## 0.70% |
| ## BB: Post-FFT data buffering | ## 10% | ## 1.17% | ## 0.47% | ## 0.52% | ## 0.83% |
| ## BB: Receiver processing block | ## 29% | ## 5.82% | ## 3.08% | ## 3.34% | ## 3.15% |
| ## BB: LDPC decoding | ## 9% | ## 1.33% | ## 0.64% | ## 0.67% | ## 0.64% |
| ## BB: HARQ buffer | ## 12% | ## 1.16% | ## 0.44% | ## 0.43% | ## 0.44% |
| ## BB: DL control processing & decoder | ## 4% | ## 3.75% | ## 3.10% | ## 3.13% | ## 3.75% |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.50% | ## 4.56% | ## 4.50% | ## 4.63% |
| ## BB: UL processing block | ## 5% | ## 2.83% | ## 1.76% | ## 1.82% | ## 1.83% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 3.82% | ## 3.05% | ## 3.26% | ## 3.75% |
| # BB: Total | # 100% | # 26.17% | # 17.90% | # 19.09% | # 20.85% |
| RF+BB: Total | 100% | 43.99% | 38.08% | 40.12% | 40.61% |

HD-FDD 2Rx

| Reduced UE bandwidth | Rel-15 reference | Rel-17 RedCap | BW1 | BW2 | BW3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 25.00% | ## 25.00% | ## 25.00% | ## 25.00% |
| ## RF: Filters | ## 10% | ## 10.00% | ## 9.03% | ## 10.00% | ## 10.00% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 45% | ## 43.04% | ## 41.51% | ## 43.04% | ## 43.04% |
| ## RF: Duplexer / Switch | ## 20% | ## 4.33% | ## 4.33% | ## 4.33% | ## 4.33% |
| # RF: Total | # 100% | # 82.38% | # 79.88% | # 82.38% | # 82.38% |
| ## BB: ADC / DAC | ## 10% | ## 2.06% | ## 1.09% | ## 1.52% | ## 2.07% |
| ## BB: FFT/IFFT | ## 4% | ## 1.15% | ## 0.49% | ## 0.73% | ## 1.18% |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.14% | ## 0.90% | ## 1.10% | ## 1.40% |
| ## BB: Receiver processing block | ## 24% | ## 9.01% | ## 4.78% | ## 4.78% | ## 4.78% |
| ## BB: LDPC decoding | ## 10% | ## 2.69% | ## 1.19% | ## 1.19% | ## 1.19% |
| ## BB: HARQ buffer | ## 14% | ## 2.41% | ## 0.91% | ## 0.91% | ## 0.91% |
| ## BB: DL control processing & decoder | ## 5% | ## 5.00% | ## 4.25% | ## 4.25% | ## 5.00% |
| ## BB: Synchronization / cell search block | ## 9% | ## 8.55% | ## 8.38% | ## 8.38% | ## 8.55% |
| ## BB: UL processing block | ## 5% | ## 3.17% | ## 2.04% | ## 2.04% | ## 2.04% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 7.23% | ## 6.48% | ## 6.48% | ## 7.23% |
| # BB: Total | # 100% | # 43.39% | # 30.52% | # 31.39% | # 34.36% |
| RF+BB: Total | 100% | 58.99% | 50.26% | 51.79% | 53.57% |

For comparison, the average cost reduction estimates from [43] for different BW reduction options compared to the corresponding Rel-17 RedCap baselines are presented in the table below.

Table 2: Average cost reduction achieved by BW reduction options compared to corresponding Rel-17 baselines

| Option | FD-FDD 1Rx | TDD 1Rx | HD-FDD 1Rx | FD-FDD 2Rx | TDD 2Rx | HD-FDD 2Rx |
| --- | --- | --- | --- | --- | --- | --- |
| BW1 | 11.85% | 11.25% | 14.06% | 14.31% | 13.42% | 14.79% |
| BW2 | 9.15% | 8.08% | 11.92% | 11.46% | 8.81% | 12.21% |
| BW3 | 8.02% | 7.66% | 8.90% | 8.72% | 7.68% | 9.19% |

FL1 High Priority Question 7.2.2-4a: Can the table formats used in the above 7 tables with cost reduction estimates for the BW options be used as a baseline text for TR 38.865? (Regarding the averaging method, please see Question 6.1-2a.)

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y | We suggested in 6.1-2a, the differences between solutions should be listed, the above 7 tables are fine. |
| Nokia, NSB | Y |  |
| Nordic | Y |  |
| Lenovo | Y |  |
| CATT | Y |  |
| SONY | Y |  |
| FUTUREWEI | Y |  |
| Xiaomi |  | We prefer to reflect which comparison is optional |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Samsung | N | We have strong concern on the number of ADC and FFT size for BW 2. This needs to be clarified first. |
| Qualcomm | Y |  |
| CMCC | Y |  |
| MediaTek | Y | We are supportive of the table structure. If the averaging method is not agreed before this proposal, we suggest to put square brackets on the values |
| FL2 | Based on received responses, the following proposal can be considered. High Priority Proposal 7.2.2-4b: The table formats used in the 7 tables with cost reduction estimates for the BW options listed in the end of Section 7.2.2 in R1-2207731 are used as a baseline text for TR 38.865. |  |
| Huawei, Hisilicon | Y |  |
| FL3 | RAN1 made the following conclusion in the online session on Monday 22nd August: Conclusion: The table formats used in the 7 tables with complexity reduction estimates for the BW options listed in the end of Section 7.2.2 in R1-2207731 are used as a baseline text for TR 38.865. Note: values in the table may be updated. |  |
| FL5 FL6 | The 7 tables above this feedback form show the average values of the initial cost evaluation results captured in [43]. High Priority Proposal 7.2.2-4c: The values in the 7 tables with cost reduction estimates for the BW options listed in the end of Section 7.2.2 in R1-2207733 are used as a baseline text for TR 38.865. |  |
| CATT | Y |  |
| Sierra Wireless | Y |  |
| Sharp | Y |  |
| OPPO | Y |  |
| Xiaomi2 | Y |  |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| FL7 | The online session on Wednesday 24th August made the following conclusion: Conclusion: The values in the 7 tables with cost reduction estimates for the BW options listed in the end of Section 7.2.2 in R1-2207733, with FD-FDD 1Rx corrected average, are used as a baseline text for TR 38.865. (The formulation “with FD-FDD 1Rx corrected average” refers to correction of the averaging across companies on the ‘FD-FDD 1Rx’ tab in [43] compared to the earlier version in [10]. In [10], the averages on this tab did not include the evaluation results from three companies, corresponding to columns C, D and E on that tab, but this has been fixed in [43].) |  |

### 7.2.3	Analysis of performance impacts

Many contributions analyze the potential performance impacts of bandwidth reduction options for Rel-18 eRedCap. The findings are summarized below.

Peak data rate:

- P1: Further UE bandwidth reduction results in peak data rate reduction [11, 20, 33]. (13)

- P2: Reducing the UE bandwidth leads to peak data rate reduction, but the reduced peak data rate can still fulfill the targeted data rate in Rel-18 [13, 20, 35]. (15)

- P3: In terms of the instantaneous peak data rate (i.e., sum of UL and DL), all BW reduction options fulfil the 10 Mbps peak data rate requirement [11]. (10)

- P4: In TDD, with 5 MHz UE bandwidth (BW1, BW2, BW3), the achievable peak data rate for UL or DL can be less than 10 Mbps depending on the TDD pattern [11]. (10)

- P5: BW1/2/3 can fulfil the data rate requirements of low-end RedCap use cases [14]. (9)

Coverage:

- C1: For BW1 and BW2, link performance degradation for PDCCH, and PBCH (30 kHz SCS), and SIB1-PDSCH [11, 14, 15, 20, 22, 28, 29, 31, 32, 33, 34, 35, 38] (15)

- C2: For BW1, BW2 and BW3, performance loss for Msg4 and PDSCH [33] (9)

- C3: For BW3, coverage impact for SIB1-PDSCH if the BW allocation for SIB1 PDSCH exceeds 5 MHz [11, 15, 19, 22, 33, 38] (13)

- C4: Based on link-budget analysis, there is no impact to coverage for BW1/BW2/BW3 [22] (1)

- C5: Coverage would be degraded if a Rel-18 eRedCap UE punctures the bits outside its BB bandwidth or Rel-18 eRedCap dedicated resource(s) which does not exceed the Rel-18 eRedCap UE BB bandwidth is specified/configured. No significant impact is expected if a Rel-18 eRedCap UE perform RF-retuning to receive the channels/signals which are periodically transmitted [37]. (4)

- C6: In case of SCS 30 kHz, the PBCH coverage loss can be compensated by UE itself with RF retuning [13]. (3)

Latency:

- L1: Impact of further UE bandwidth reduction on the latency is insignificant [11, 20]. (11)

- L2: 5 MHz UE bandwidth (BW1, BW2, BW3) can sufficiently fulfil relaxed latency requirements of Rel-18 eRedCap use cases [11, 15]. (15)

- L3: There will be additional latency introduced for UE bandwidth reduction Option BW1 if it supports RF retuning and for UE bandwidth reduction Option BW2 and Option BW3 if it uses cross-slot scheduling, compared to Rel-17 RedCap [15]. (3)

- L4: Latency would increase if a Rel-18 eRedCap UE perform RF-retuning to receive the channels/signals which are periodically transmitted. No significant impact is expected if a Rel-18 eRedCap UE punctures the bits outside its BB bandwidth or Rel-18 eRedCap dedicated resource(s) which does not exceed the Rel-18 eRedCap UE BB bandwidth is specified/configured [37]. (3)

- L5: For BW1, BW2 and BW3, the access latency of eRedCap UEs may increase due to common PDCCH or broadcast PDSCH performance reduction [33]. (2)

- L6: For Options BW1/2, there would be an impact to the latency of co-existing non-5MHz UEs if the SCS of CD-SSB and CORESET #0 is always configured to be 15 kHz [19]. (0)

PDCCH blocking:

- B1: For BW1 and BW2, significant impact on PDCCH blocking probability [11, 14, 20, 29] (8)

FL1/FL3/FL4/FL5/FL7 High Priority Question 7.2.3-1a: Considering the above observations on the UE bandwidth reduction, what performance impacts for BW1, BW2, BW3 should be considered as a baseline for the TP drafting for TR section 7.2.3?

| Company | Impacts (P1-P5, C1-C6, L1-L6, B1) | Comments or suggested revisions |
| --- | --- | --- |
| FL | If there are impacts among the listed potential impacts that should NOT be listed among the impacts in the TR section, please mention them in the Comments field and elaborate. |  |
| CATT | All of P1-P5; C1,C2,C3; L1, L2; | C4 is not accurate since coverage is impacted for sure (whether coverage recovery is needed is another story). C5, C6 use RF retuning to combine e.g. PBCH or SIB1. However, if compared to the case that ‘allow legacy UE to also combine PBCH/SIB1’, we believe coverage loss exists, under this fair comparison. Open to consider L3-L6, but we think they are not as essential as L1, L2. |
| Sierra Wireless | P1, C1, C2, C3 L1, L2 |  |
| Sharp | P1-P5, C1,C3 L1-L4 B1 | For C5,we can agree the first sentence. The periodically transmitted signal may have different contents and cannot be combined in different occasions For L5, it may have no latency impact due to the common PDCCH with option BW3 |
| Samsung | P1, P4, P3 (for FDD only) B1 | Coverage should be discussed in the other section. The definition of latency should be clarified. |
| FUTUREWEI | P1, P2, P3, P4, P5 C1, C3 L1, L2 | P2 is a good candidate among P1-P5. For C4, the summary of the simulation results should provide information. L5: for BW3, common PDCCH should not increase latency For the latency proposals, we should distinguish between idle / connected states as needed. For example with L3, it is unclear whether BW1 is referring to receiving SIB1 and BW2/BW3 is for other PDSCH. |
| OPPO | P2-P5 C1,C2,C3,C5,C6 L2 | For the C5 and C6, they can be merged. |
| Nordic |  | Regarding coverage statements, we should wait for observations from 9.6.2. before making any statements in 9.6.1 B1 : PDCCH blocking is not in scope of studying Regarding latency of retuning – statements should not be applicable, because RF does not change. In general we support statement L1 |
| Xiaomi2 | P1-P4 L1, L2, L3 B1 | - For P5, data rate of low-end RedCap is not clear enough. Typical value of the data rate of low-end RedCap should be present to avoid ambiguity For L4， if bits outside the BB is punctured, more NACK is expected. The latency would increase due to more retransmission For coverage, we share the same view as Samsung that it should be discussed in the other thread. |
| Spreadtrum | P2 or P1+P5 C1+C2+C3 L1+L2 B1 | Similar observations can be combined. |
| Nokia, NSB | P1, C1, L2 | For additional observations on coverage impact, we should wait for 9.6.2. |
| Ericsson | P1, P3 (or P2 or P5), P4, C1, C2, C3, L1, L2, B1 | It would be good to group the performance impacts into BWx-specific impacts and common impacts. Perhaps “coverage” can be replaced with “link performance”. For “coverage” C3 also applies to BW1 and BW2 (in addition to BW3) |
| vivo | P2 L1+L2 | For coverage, we share Nordic’s views. The observations should be aligned and based on the updated results. |
| Intel | P2, P3, P4 C1, C3 L1, L2 B1 |  |
| DOCOMO | P1 C1, C2, C3, C5 | For peak data rate, we need further discussion whether the target peak data rate can be achieved and reduced sufficiently. For coverage, corresponding reception scheme and/or configuration of resources should be described together. For example, whether there is coverage impacts or not depends on whether the resource allocation for Rel-18 RedCap does not exceed the max. BW of Rel-18 RedCap or UE puncture the bits outside the UE BW. |
| Huawei, Hisilicon | P1-P3,P5; C1-C3,C5-C6; L2 | P4: From UE capability perspective, a UE has to support all possible TDD configurations. Here the focus is reduced UE capability and does not depend on one certain TDD configuration. So P4 is not needed. |
| SONY | P2, P4 C1, C3 L1, L2 |  |
| Qualcomm | P2, C1, L2 |  |
| ZTE, Sanechips | P2, P5; C1,C2,C3, C6 Partially C5; L2, L4, L5 B1f | We think we can start with the following paragraph. If we check one by one, this would quite be complicated and waste massive time. Based on our option selection, we have some modifications as follows: For peak data rate reduction, - Reducing the UE bandwidth leads to peak data rate reduction, but the reduced peak data rate can still fulfill the targeted data rate in Rel-18 - In TDD, with 5 MHz UE bandwidth (BW1, BW2, BW3), the achievable peak data rate for UL or DL can be less than 10 Mbps depending on the TDD pattern For coverage, - For BW1 and BW2, link performance degradation are observed for PDCCH, and PBCH (30 kHz SCS), and SIB1-PDSCH [11, 14, 15, 20, 22, 28, 29, 31, 32, 33, 34, 35, 38] - For BW1, BW2 and BW3, performance loss for Msg4 and PDSCH are observed[33] - For BW3, coverage impact is expected for SIB1-PDSCH if the BW allocation for SIB1 PDSCH exceeds 5 MHz [11, 15, 19, 22, 33, 38] - In case of SCS 30 kHz, the PBCH coverage loss can be compensated by UE itself with RF retuning [13]. For latency - L4: Latency would increase if a Rel-18 eRedCap UE perform RF-retuning to receive the channels/signals which are periodically transmitted. No significant impact is expected if a Rel-18 eRedCap UE punctures the bits outside its BB bandwidth or Rel-18 eRedCap dedicated resource(s) which does not exceed the Rel-18 eRedCap UE BB bandwidth is specified/configured [37]. - L5: For BW1, BW2 and BW3, the access latency of eRedCap UEs may increase due to common PDCCH or broadcast PDSCH performance reduction [33]. - L2: 5 MHz UE bandwidth (BW1, BW2, BW3) can sufficiently fulfil relaxed latency requirements of Rel-18 eRedCap use cases [11, 15]. For PDCCH blocking: - B1: For BW1 and BW2, significant impact on PDCCH blocking probability [11, 14, 20, 29] Companies can modify the paragraph, then we can converge soon. |
| Panasonic | P1, P2, C1, C2, C3, L3 | L6 may be a coexisting impact |
| CMCC | P1-P5 C5 L4, L5 B1 | Although PDCCH blocking issue will not be handled, the impact still can be captured. |
| FL8 | The number of received responses that support capturing a specific impact is indicated with red text in the bullet list above this feedback form. High Priority Proposal 7.2.3-1b: Capture the highlighted impacts in R1-2207734 section 7.2.3 in the TR. |  |
| FL9 | The online session on Thursday 25th August made the following agreement: Agreement: The highlighted impacts in R1-2207734 are adopted as starting point for TR input. |  |

Based on the above agreement, the following proposal can be considered.

FL9 High Priority Proposal 7.2.3-1c: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.3:

| Peak data rate: - Further UE bandwidth reduction results in peak data rate reduction. - Reducing the UE bandwidth leads to peak data rate reduction, but the reduced peak data rate can still fulfill the targeted data rate in Rel-18. - In terms of the instantaneous peak data rate (i.e., sum of UL and DL), all BW reduction options fulfil the 10 Mbps peak data rate requirement. - In TDD, with 5 MHz UE bandwidth (BW1, BW2, BW3), the achievable peak data rate for UL or DL can be less than 10 Mbps depending on the TDD pattern. Coverage: - For BW1 and BW2, link performance degradation for PDCCH, and PBCH (30 kHz SCS), and SIB1-PDSCH. - For BW3, coverage impact for SIB1-PDSCH if the BW allocation for SIB1 PDSCH exceeds 5 MHz. Latency: - Impact of further UE bandwidth reduction on the latency is insignificant. - 5 MHz UE bandwidth (BW1, BW2, BW3) can sufficiently fulfil relaxed latency requirements of Rel-18 eRedCap use cases. |  |  |
| --- | --- | --- |
| Company | Y/N | Comments |
| FL10 | The following is the latest draft TP from the offline session on Thursday 25th August: Peak data rate: - Further UE bandwidth reduction results in peak data rate reduction. - Reducing the UE bandwidth leads to peak data rate reduction, but the reduced peak data rate can still fulfill the targeted data rate in Rel-18. - In terms of the instantaneous peak data rate (i.e., sum of UL and DL), all BW reduction options fulfil the 10 Mbps peak data rate requirement. - In TDD, with 5 MHz UE bandwidth (BW1, BW2, BW3), the achievable peak data rate for UL or DL can be less than 10 Mbps depending on the TDD pattern. Coverage: - [For BW1 and BW2, there is link performance degradation for PDCCH, and PBCH (30 kHz SCS), and SIB1-PDSCH.] - [For BW1/BW2/BW3, there is coverage impact for SIB1-PDSCH if the BW allocation for SIB1 PDSCH exceeds 5 MHz.] Latency: - Impact of further UE bandwidth reduction on the latency is insignificant. - 5 MHz UE bandwidth (BW1, BW2, BW3) can sufficiently fulfil relaxed latency requirements of Rel-18 eRedCap use cases.. Based on the outcome from the offline session, the following updated proposal can be considered. High Priority Proposal 7.2.3-1d: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.3: Peak data rate: Reducing the UE bandwidth leads to peak data rate reduction, but the reduced peak data rate can still fulfill the targeted data rate in Rel-18. In TDD, with 5 MHz UE bandwidth (for all BW options), the achievable peak data rate for UL or DL can be less than 10 Mbps depending on the TDD pattern. Coverage: [For all BW options, there is coverage impact for SIB1-PDSCH if the bandwidth allocation for SIB1 PDSCH exceeds 5 MHz. Furthermore, for BW1/BW2, there is link performance degradation for PDCCH, and PBCH (30 kHz SCS).] Latency: The impact of further UE bandwidth reduction on the latency is insignificant, and 5 MHz UE bandwidth (for all BW options) can sufficiently fulfil relaxed latency requirements of RedCap use cases.. |
| Nordic | OK |  |
| DOCOMO | Y |  |
| LGE |  | Rather than saying it is insignificant, we think the following observation suffices. “With further UE bandwidth reduction, 5 MHz UE bandwidth (for all BW options) can sufficiently fulfil relaxed latency requirements of RedCap use cases.” |
| Nokia, NSB | Y |  |
| vivo | Y |  |
| CATT | Y in general | We go through the R18 SID again, and sure Rel-18 RedCap UE is not targeting at fulfilling any ‘relaxed latency requirement’. To make the description more general and to avoid confusion, suggest deleting ‘relaxed’: The impact of further UE bandwidth reduction on the latency is insignificant, and 5 MHz UE bandwidth (for all BW options) can sufficiently fulfil relaxed latency requirements of RedCap use cases. |
| OPPO | Y | We think the updated version is better. Should take similar approach of merging for other options. |
| Qualcomm | Y |  |
| ZTE, Sanechips | Y with updates | OK with CATT’ suggestion to delete relaxed. Additionally, we think there would be latency impacts due to common PDCCH or broadcast PDSCH performance reduction, and make the following updates. Latency: - Impact of further UE bandwidth reduction on the latency is insignificant, even the access latency of eRedCap UEs may increase due to common PDCCH or broadcast PDSCH performance reduction - 5 MHz UE bandwidth (BW1, BW2, BW3) can sufficiently fulfil relaxed latency requirements of Rel-18 eRedCap use cases. |
| Spreadtrum | Y |  |
| Intel | Y | Consider PDCCH CSS with 24PRB and SCS 15kHz, a condition could be added for clarification for PDCCH. Coverage: [For all BW options, there is coverage impact for SIB1-PDSCH if the bandwidth allocation for SIB1 PDSCH exceeds 5 MHz. Furthermore, for BW1/BW2, there is link performance degradation for PDCCH if PDCCH exceeds 5MHz, and PBCH (30 kHz SCS).] |
| FUTUREWEI | Y |  |
| CMCC | Y |  |
| FL11 | The online session on Friday 26th August made the following agreement: Agreement: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.3: Peak data rate: Reducing the UE bandwidth leads to peak data rate reduction, but the reduced peak data rate can still fulfill the targeted data rate in Rel-18. In TDD, with 5 MHz UE bandwidth (for all BW options), the achievable peak data rate for UL or DL can be less than 10 Mbps depending on the TDD pattern. Coverage: [For all BW options, there is coverage impact for SIB1-PDSCH if the bandwidth allocation for SIB1 PDSCH exceeds 5 MHz. Furthermore, for BW1/BW2, there is link performance degradation for PDCCH, and PBCH (30 kHz SCS).] Latency: The impact of further UE bandwidth reduction on the latency is insignificant, and 5 MHz UE bandwidth (for all BW options) can sufficiently fulfil relaxed latency requirements of RedCap use cases.. |

### 7.2.4	Analysis of network deployment and coexistence impacts

Many contributions discuss the potential network deployment and coexistence impacts of bandwidth reduction options for Rel-18 eRedCap. The findings are summarized below.

- Bandwidth reduction options BW1 and BW2 can have coexistence impacts in terms of support of SSB/CORESET #0 configurations (especially 30 kHz SCS [11, 14, 15, 19, 29, 33, 35, 36]. (19)

- For all the BW reduction options BW1, BW2 and BW3, there is no coexistence issue with legacy UEs if the common channels such as SIB1, OSI, RAR, MSG3 etc. are scheduled within 5MHz; Otherwise, there is some coexistence issue with legacy UEs [15]. (13)

- For BW1 and BW2, impact on SSB transmissions (e.g., NCD-SSB overhead) and BWP operation [11, 12, 19, 20, 35] (13)

- For BW1 and BW2, there are limitations of RACH configurations and PRACH sharing procedure [11, 14, 19, 20, 21, 35] (15)

- For BW1 and BW2, there are impacts if new initial BWPs are introduced [14] (8)

- The potential coexistence impact for BW3 (BB-only bandwidth reduction for data channels) is small [11]. (15)

- RedCap UE supporting BW1/BW2/BW3 can coexist with legacy UEs [22]. (3)

- Bandwidth reduction option BW3 achieves better coexistence performance than option BW1 in aspects of reusing legacy SSB, CORESET0, SIB1, paging, RACH, and avoiding additional SSB overhead [29]. (13)

- Early indication might be needed for BW1, BW2, and BW3 [11, 12, 22, 29, 35, 36, 37] (15)

- For Option BW2, significant coexistence impacts are observed in terms of CORESET#0 configuration, paging, SIB sharing, separate initial BWP sharing and RACH resource sharing [20]. (8)

- For Option BW1, significant coexistence impacts are observed in terms of CD-SSB acquisition, CORESET#0 configuration, paging, SIB sharing, separate initial BWP sharing, RACH resource sharing and NCD-SSB burden [20]. (9)

- The NW overhead would be increased if Rel-18 eRedCap dedicated resource(s) which does not exceed the Rel-18 eRedCap UE BB bandwidth is specified/configured separately from those for legacy UEs [37]. (7)

- No significant impact is expected for coexistence with legacy UEs if a Rel-18 eRedCap UE perform RF-retuning to receive the channels/signals which are periodically transmitted or a Rel-18 eRedCap UE punctures the bits outside its BB bandwidth [37]. (4)

- For BW1 and BW2, impact on network deployment and configuration flexibility [Ericsson] (9)

- PUSCH resource fragmentation for BW1 [12, 32, 35] and BW2/BW3 [12] (8)

- Handling PUCCH resources for BW1 [21, 32] (5)

- DL common message might be limited to a small TBS to support BW 1 and BW 3 [28]. (5)

- BW1/BW2 seems to have the largest coexistence issues among evaluated options [23]. (10)

- For BW1/BW2 SCS of 60kHz is not applicable (6)

- For BW1 and BW2, impact on the support of PUCCH format 2 and 3 bandwidth (up to 16 PRBs) [33, 37] (6)

- For BW1 and BW2, impact on the support of TRS BW (52 or 48 PRBs) [37] (5)

FL1/FL3/FL4/FL5/FL7 High Priority Question 7.2.4-1a: Considering the above observations on the UE bandwidth reduction, what network deployment and coexistence impacts for BW1, BW2, BW3 should be considered as a baseline for the TP drafting for TR section 7.2.4?

| Company | Impacts (1-21) | Comments or suggested revisions |
| --- | --- | --- |
| FL | If there are impacts among the listed potential impacts that should NOT be listed among the impacts in the TR section, please mention them in the Comments field and elaborate. |  |
| CATT | 1-6, 8-11, 14-21 |  |
| Sierra Wireless | 1-4, 6, 8-11, 18 |  |
| Sharp | All except 13 | For 13, periodically transmitted signals may have different contents/resource allocation in occasions. The bit punctures may have significant impact on performance |
| Samsung | 1, 2 (for 15kHz SCS only), 6,8, 10 and 11 for both BW1 and BW2 16 | We suggest to decouple “solutions/spec impacts to resolve some potential issue” from “NW deployment impact”. |
| FUTUREWEI | 1-6, 8-12, 14, 15, 18-20 | The statement in 17 also applies to BW2. For 19, there is no 5 MHz channel with 60 kHz SCS defined in RAN4 in Rel-17. |
| OPPO | 1,2,13 |  |
| Nordic | not OK with 3) | In our opinion SSB reception has no issue in BW2, since SSB is within UEs RF. Note that such operation is strongly considered also to apply to legacy UEs as part of FG 6-1a. CORESET#0 however, is not possible to receive in BW2. In general we prefer to capture list of channels and signals, structure that - (a) have to be redesigned - (b) there is configuration flexibility limitation For example, RACH resources can be shared if gNB configures RACH format that fits both legacy and BW1 UEs? So this belongs to (b) |
| Xiaomi2 | 1-6, 8-9 ,10-11, 14,18 |  |
| Spreadtrum | 1, 4, 5, 6, 8, 11, 12, 14, 18 | Similar observations can be combined. |
| Nokia, NSB | 1, 6, 7, 9 |  |
| Ericsson | 1, 3, 4, 5, 6, 8, 9, 14 |  |
| vivo | 1, 2, 4, 9 |  |
| Intel | 1, 3, 4, 6, |  |
| DOCOMO | 1 (with clarification), 2-5, 9, 12, 13 | First of all, we would like to clarify whether it is common understanding or not that some impacts listed above can be solved by UE implementation, e.g., puncturing of bits and/or RF-retuning. For 1, it was noted in SID objective that current SSB design is reused thus we think there is no impact on SSB itself from NW deployment perspective, however, we understood that SSB/CORESET#0 multiplexing configuration can be affected since some CORESET#0 configuration in the current specification exceeds the max UE BB BW for BW1 and BW2. For 20 and 21, we can move them to the part of analysis of specification impact. |
| Huawei, Hisilicon | 1-4,6,8-13,15,17-21 |  |
| SONY | 1,2,6,8,9,14,18 |  |
| Qualcomm | 1, 3, 4, 9, 15 |  |
| ZTE, Sanechips | 1,2,3,4,6,8,9,12,14~21 | Similar, maybe we can start with the following paragraph with some modifications based on our option selection, - Bandwidth reduction options BW1 and BW2 can have coexistence impacts in terms of CORESET#0 configuration, paging, SIB sharing, separate initial BWP sharing and RACH resource sharing - For all the BW reduction options BW1, BW2 and BW3, there is no significant coexistence issue with legacy UEs if the common channels such as SIB1, OSI, RAR, MSG3 etc. are limited within 5MHz. - For BW1 and BW2, impact on SSB transmissions (e.g., NCD-SSB overhead) and BWP operation [11, 12, 19, 20, 35] - For BW1 and BW2, there are limitations of RACH configurations and PRACH sharing procedure [11, 14, 19, 20, 21, 35] - The potential coexistence impact for BW3 (BB-only bandwidth reduction for data channels) is small may except for shared common PDSCH/PUSCH [11]. - Bandwidth reduction option BW3 achieves better coexistence performance than option BW1 in aspects of reusing legacy SSB, CORESET0, SIB1, paging, RACH, and may avoiding additional SSB overhead [29]. - Early indication might be needed for BW1, BW2, and BW3 [11, 12, 22, 29, 35, 36, 37] - The NW overhead would be increased if Rel-18 eRedCap dedicated resource(s) which does not exceed the Rel-18 eRedCap UE BB bandwidth is specified/configured separately from those for legacy UEs [37]. - For BW1 and BW2, impact on network deployment and configuration flexibility [Ericsson] - PUSCH resource fragmentation for BW1 [12, 32, 35] and BW2/BW3 [12] - Handling PUCCH resources for BW1 [21, 32] - DL common message might be limited to a small TBS to support BW 1 and BW 3 [28]. - For BW1/BW2 SCS of 60kHz is not applicable - For BW1 and BW2, impact on the support of PUCCH format 2 and 3 bandwidth (up to 16 PRBs) [33, 37] - For BW1 and BW2, impact on the support of TRS BW (52 or 48 PRBs) [37] |
| Panasonic | 1, 3, 4, 6, 8, 9, 15 |  |
| CMCC | All except 7 | The following 7 does not reflect the detail impact when coexist with legacy UEs. - RedCap UE supporting BW1/BW2/BW3 can coexist with legacy UEs [22]. |
| FL8 | The number of received responses that support capturing a specific impact is indicated with red text in the bullet list above this feedback form. High Priority Proposal 7.2.4-1b: Capture the highlighted impacts in R1-2207734 section 7.2.4 the TR. |  |
| FL9 | The online session on Thursday 25th August made the following agreement: Agreement: The highlighted impacts in R1-2207734 are adopted as starting point for TR input. |  |

Based on the above agreement, the following proposal can be considered.

FL9 High Priority Proposal 7.2.4-1c: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.4:

| Network deployment and coexistence impacts: - Bandwidth reduction options BW1 and BW2 can have coexistence impacts in terms of support of SSB/CORESET #0 configurations (especially 30 kHz SCS). - For all the BW reduction options BW1, BW2 and BW3, there is no coexistence issue with legacy UEs if the common channels such as SIB1, OSI, RAR, MSG3 etc. are scheduled within 5MHz; Otherwise, there is some coexistence issue with legacy UEs. - For BW1 and BW2, impact on SSB transmissions (e.g., NCD-SSB overhead) and BWP operation. - For BW1 and BW2, there are limitations of RACH configurations and PRACH sharing procedure. - The potential coexistence impact for BW3 (BB-only bandwidth reduction for data channels) is small. - Bandwidth reduction option BW3 achieves better coexistence performance than option BW1 in aspects of reusing legacy SSB, CORESET0, SIB1, paging, RACH, and avoiding additional SSB overhead. - Early indication might be needed for BW1, BW2, and BW3. - BW1/BW2 seems to have the largest coexistence issues among evaluated options |  |  |
| --- | --- | --- |
| Company | Y/N | Comments |
| FL10 | The following is the latest draft TP from the offline session on Thursday 25th August: Network deployment and coexistence impacts: - Bandwidth reduction options BW1 and BW2 can have coexistence impacts in terms of support of SSB/CORESET #0 configurations (especially 30 kHz SCS). - For all the BW reduction options BW1, BW2 and BW3, there is no coexistence issue with legacy UEs if the common channels such as SIB1, OSI, RAR, MSG3 etc. are scheduled within 5MHz; oOtherwise, there is some coexistence issue with legacy UEs. - For BW1 and BW2, there is impact on SSB transmissions (e.g., NCD-SSB overhead) and BWP operation. - For BW1 and BW2, there are limitations of RACH configurations and PRACH sharing procedure. - The potential coexistence impact for BW3 (BB-only bandwidth reduction for data channels) is small. - Bandwidth reduction option BW3 achieves better coexistence performance than option BW1 in aspects of reusing legacy SSB, CORESET0, SIB1, paging, RACH, and avoiding additional SSB overhead. - Early indication might be needed for BW1, BW2, and BW3. - BW1/BW2 seems are expected to have the largest coexistence issues impacts among the evaluated options, whereas the expected coexistence impacts for BW3 are smaller.. Based on the outcome from the offline session, the following updated proposal can be considered. High Priority Proposal 7.2.4-1d: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.4: If the common channels such as SIB1, OSI, RAR, MSG3 etc. are scheduled within 5MHz, then none of the UE bandwidth reduction options (BW1, BW2, BW3) have coexistence issues with legacy UEs, but otherwise there are some coexistence issues with legacy UEs. BW1 and BW2 are expected to have the largest coexistence impacts among the evaluated options, whereas the expected coexistence impacts for BW3 are smaller. BW1 and BW2 can have coexistence impacts in terms of support of SSB/CORESET #0 configurations (especially 30 kHz SCS) and limitations of RACH configurations and PRACH sharing procedure. Furthermore, BW1 has impact on SSB transmissions (e.g., NCD-SSB overhead) and BWP operation. Early RedCap UE indication (through Msg1/MsgA) might be needed for all BW options.. |
| Nordic | OK |  |
| DOCOMO | Y |  |
| LGE | Y |  |
| Nokia, NSB | Y |  |
| vivo | Y |  |
| CATT | Y in general | Strictly speaking, Msg3 is not a common channel, so need to delete it in the first paragraph. |
| OPPO |  | Some wording change: “BW1 and BW2 are expected to have larger coexistence impacts among the evaluated options, whereas the expected coexistence impacts for BW3 are smaller.” -we are not selecting one of BW1/2 become the largest one. |
| Qualcomm | Y |  |
| ZTE, Sanechips | Y |  |
| Spreadtrum | Y |  |
| Intel | Y |  |
| FUTUREWEI | Y |  |
| CMCC | Y |  |
| FL11 | The online session on Friday 26th August made the following agreement: Agreement: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.4: If the common channels such as SIB1, OSI, RAR, MSG3 etc. are scheduled within 5MHz, then none of the UE bandwidth reduction options (BW1, BW2, BW3) have coexistence issues with legacy UEs, but otherwise there are some coexistence issues with legacy UEs. BW1 and BW2 are expected to have the largest coexistence impacts among the evaluated options, whereas the expected coexistence impacts for BW3 are smaller. BW1 and BW2 can have coexistence impacts in terms of support of SSB/CORESET #0 configurations (especially 30 kHz SCS) and limitations of RACH configurations and PRACH sharing procedure. Furthermore, BW1 has impact on SSB transmissions (e.g., NCD-SSB overhead) and BWP operation. Early RedCap UE indication (through Msg1/MsgA) might be needed for all BW options.. |

### 7.2.5	Analysis of specification impacts

Many contributions discuss the potential specification impacts of bandwidth reduction options for Rel-18 eRedCap. The findings are summarized below.

- BW1 and BW2 can have significant specification impacts, considering the impacts on initial access, random access, SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication [11, 15, 20, 22]. (18)

- For BW1, there is large specification impact in aspects of coverage enhancement, CORESET0/SIB1/paging/RACH design and additional SSB transmission in connected mode [29]. (9)

- For BW1/2, if another initial BWPs need to be introduced, huge spec impacts and network impacts were expected [14]. (11)

- For Option BW3, specification impact is foreseen on FDRA, RACH procedure, paging procedure and SIB transmission. (7)

- BWP operation and SSB transmission for BW1 and BW2 [11, 15] (9)

- Operation with TDD center frequency alignment for BW1 [12, 15] (3)

- BW3 should be considered for the WID for its relatively small deployment and specification impact [12] (5)

- No significant specification impact is expected if a Rel-18 eRedCap UE perform RF-retuning to receive the channels/signals which are periodically transmitted or a Rel-18 eRedCap UE punctures the bits outside its maximum bandwidth [37]. (0)

- For BW1 and BW2, enhancements needed on the user multiplexing capacity of common PUCCH resources for both PUCCH format 0 and 1 especially when FH for the common PUCCH resources is disabled [32]. (2)

- For BW3, consider source allocation enhancement as the potential specification impact [28]. (4)

- For BW1, consider use of separate 5-MHz allocation as a potential specification impact [12]. (2)

In addition, a few contributions propose to support 12 PBRs within 5 MHz UE bandwidth for 30 kHz SCS with the potential benefits for supporting PRACH and CORESET configurations, and DFT-s-OFDM [20, 23, 26, 31, 32]. In this case, there will be specification impacts which also requires RAN4 confirmation. One contribution [23] proposes to send LS to RAN4, ask whether 12 PRBs could be supported in 5MHz carrier of 30 kHz SCS at least for (i) BW2 and for (ii) BW1 if gNB carrier is larger than 5 MHz.

FL1/FL3/FL4/FL5/FL7 High Priority Question 7.2.5-1a: Considering the above observations on the UE bandwidth reduction, what specification impacts for BW1, BW2, BW3 should be considered as a baseline for the TP drafting for TR section 7.2.5?

| Company | Impacts (1-10) | Comments or suggested revisions |
| --- | --- | --- |
| FL | If there are impacts among the listed potential impacts that should NOT be listed among the impacts in the TR section, please mention them in the Comments field and elaborate. |  |
| CATT | 1-5, 10 |  |
| Sierra Wireless | 1-5 |  |
| Sharp | All except 8,11 |  |
| Samsung | 1 (may have), | 1, it may have impact to maintain the same performance. 7. support it as final recommendation. |
| FUTUREWEI | 1, 2, 5-7, 11 (with correction) | 2: should “BW” be “BW1”? 11: revision “as a potential” to “to mitigate” |
| OPPO | 1, 5 | We cannot conclude if there are large or medium or small specification impact in the SI stage. That kind of conclusions should be avoided. May be we can just identify the few major impacts. |
| Nordic | Not OK with 1 and 5 | We disagree with statement that SSB has to be redesigned for BW2 |
| Xiaomi2 | 1,2,3,5 |  |
| Spreadtrum | 1, 2, 3 |  |
| Nokia, NSB | 1 |  |
| Ericsson | 1, 3, 5 |  |
| vivo | 1, 4 |  |
| Intel | 1, |  |
| DOCOMO |  | Similar comment as on Question 7.2.4-1a. Some of them listed above can be solved by UE implementation, for example puncturing the bits outside the max. UE BW. Therefore, we suggest to describe the corresponding assumption, e.g., spec impact can be observed if the dedicated resource of common channel for Rel-18 RedCap is configured/specified. For 1, we think some impacts on CORESET#0 can be expected even for BW3. More specifically, if SIB1 is configured separately for Rel-18 RedCap, and then it needs to be discussed how to schedule/configure such SIB1 resource. |
| Huawei, Hisilicon | 1-5,7 |  |
| SONY | 1, 3, 7 |  |
| Qualcomm | 1 |  |
| ZTE, Sanechips | 1,3,4,10 | Similar, maybe we can start with the following paragraph with some modifications based on our option selection, - BW1 and BW2 can have significant specification impacts, considering the impacts on initial access, random access, SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication [11, 15, 20, 22]. - For BW1/2, if another initial BWPs need to be introduced, significant spec impacts and network impacts were expected [14]. - For Option BW3, specification impact is foreseen on RACH procedure, paging procedure,and SIB transmission. - For BW3, consider resource allocation enhancement as the potential specification impact [28]. |
| Panasonic | 1, 2, 3 |  |
| CMCC | 1-6, 9-11 | We have added the missing number after BW for 2. 2.For BW 1, there is large specification impact in aspects of coverage enhancement, CORESET0/SIB1/paging/RACH design and additional SSB transmission in connected mode [29]. |
| FL8 | The number of received responses that support capturing a specific impact is indicated with red text in the bullet list above this feedback form. High Priority Proposal 7.2.5-1b: Capture the highlighted impacts in R1-2207734 section 7.2.5 in the TR. |  |
| FL9 | The online session on Thursday 25th August made the following agreement: Agreement: The highlighted impacts in R1-2207734 are adopted as starting point for TR input. |  |

Based on the above agreement, the following proposal can be considered.

FL9 High Priority Proposal 7.2.5-1c: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.5:

| Specification impacts: - BW1 and BW2 can have significant specification impacts, considering the impacts on initial access, random access, SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication. - For BW1/2, if another initial BWPs need to be introduced, huge spec impacts and network impacts were expected. |  |  |
| --- | --- | --- |
| Company | Y/N | Comments |
| FL10 | The following updated proposal can be considered. High Priority Proposal 7.2.5-1d: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.5: BW1 and BW2 can have significant specification impacts, considering the impacts on initial access, random access, SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication. For BW1 and BW2, if another initial BWP need to be introduced, huge spec impacts and network impacts are expected.. |
| Nordic | Not OK | Not fully sure that new initial BWP would be needed, it would be enough to introduce new CORESET#0 and UE would receive SI only within new CORESET#0. But this is already capture in the first bullet. |
| DOCOMO |  | In the network deployment and coexistence impact section, early indication might be needed not only for BW1 and BW2 but also for BW3. But in this TP, it can be read that early indication is required only for BW1 and BW2, and we think it should be aligned. |
| LGE | No | We don’t agree to say the spec impacts are significant or huge for BW1 and BW2 in the baseline text. We can still strive for minimal spec impact with the adoption of BW1/BW2. It would be agreeable to say “the spec impact may be larger for BW1/BW2 compared to BW3”. |
| Nokia, NSB |  | We also don’t think another initial BWP would be needed, so we don’t need this part. We are OK with the first sentence. |
| vivo |  | We share views expressed by Nordic and DOCOMO. In addition, we are not sure what the impacts on “BWP configuration” means. So, we suggest following: BW1 and BW2 can have significant specification impacts, considering the impacts on initial access, random access, SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication. For BW1 and BW2, if another initial BWP need to be introduced, huge spec impacts and network impacts are expected. |
| CATT | Y | If Separate initial DL BWP and legacy initial DL BWP is 20 MHz, another initial BWP is needed, no matter a configured one or predefined one. |
| OPPO | N | We think this list is not unavoidable. Would be better to justify them. E.g. Earlier identification would be possible for BW3 as earlier agreed. |
| Qualcomm |  | Second sentence is redundant, so it is suggested to remove it. |
| ZTE, Sanechips | Y |  |
| Spreadtrum | Y |  |
| Intel |  | Agree with DoCoMo and vivo that ‘BWP configuration, and early indication’ can be removed. Regarding ‘another initial BWP’, it is necessary for BW1/BW2 since BW1/BW2 only has 5MHz capability. Separate initial DL BWP was already specified in Rel-17 and can be reused. Therefore, we don’t think the spec change is large. The impact to network may be true since the carrier may become more fragmented. BW1 and BW2 can have significant specification impacts, considering the impacts on initial access, random access, SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication. For BW1 and BW2, if another initial BWP need to be introduced, huge spec impacts and large network impacts are expected. |
| FUTUREWEI |  | Ok to remove second sentence. Since Early indication was mentioned in previous section, there is no need to repeat. |
| CMCC |  | For the spec impact, BW1 will have impact on SSB presence issue for active BWP. Since the SSB overhead issue will be more serious for smaller active BWP, we think whether to mandate the FG28-1a will be brought up for BW1. So we suggest to add the related impact for BW1. BW1 and BW2 can have significant specification impacts, considering the impacts on initial access, random access, SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication. For BW1 and BW2, if another initial BWP need to be introduced, huge spec impacts and network impacts are expected. For BW1, the specification impacts also include the SSB presence requirements.. |
| FL11 FL12 | Based on the received responses and the discussion in the online session on Friday 26th August, the following updated proposal can be considered. High Priority Proposal 7.2.5-1e: Adopt the following TP as a baseline text for TR 38.865 clause 7.2.5: BW1 and BW2 can have significant specification impacts, considering the impacts on initial access, random access, and SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication. For BW1 and BW2, if another initial BWP need to be introduced, huge spec impacts and network impacts are expected. For BW1, the specification impacts also include the SSB presence requirements. BW3 may have smaller spec impacts compared to BW1 and BW2.. |
| Transsion |  | We can combine the last sentence with the first sentence. BW1 and BW2 can have larger significant specification impacts compared to BW3, considering the impacts on initial access, random access, and SSB/CORESET #0 configurations (especially 30 kHz SCS), BWP configuration, and early indication. For BW1 and BW2, if another initial BWP need to be introduced, huge spec impacts and network impacts are expected. For BW1, the specification impacts also include the SSB presence requirements. BW3 may have smaller spec impacts compared to BW1 and BW2.. |
| Intel | Y | Our understanding on the intention of the first sentence is to emphasize large specification impact of BW1/2 but is not necessarily relative to BW3. Therefore, ‘larger’ in the first sentence could be changed into ‘large’. If so, we don’t need to merge the first and the third sentence. |
| Ericsson | Y |  |
| CATT | Y | Similar understanding with Intel. Prefer FL’s version. |
| Nokia, NSB | Y |  |
| OPPO |  | Genearlly OK, we would like to say BW1/2 have “higher” specification impact. Since the later bullet use “Smalller” for BW3. This can also address the online comments. |

## 7.3	Further UE peak rate reduction

### 7.3.1	Description of feature

RAN1#109e made the following agreements regarding the study of further UE peak rate reduction:

Agreement: - The following options for further UE peak rate reduction can be studied: - Option PR1: Relaxation of the constraint (vLayersj⋅Qmj⋅fj≥4) for peak data rate reduction. - Option PR2: Restriction of maximum TBS for PDSCH and PUSCH. - Option PR3: Restriction of maximum number of PRBs for PDSCH and PUSCH. - At least the following cases are studied: - The studied peak rate reduction applies to both UE-specific (unicast) and common (broadcast) channels. - The resource allocation spans a bandwidth of maximum 20 MHz (maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - It is FFS whether to study other cases. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only. Agreement: - The restricted number of PRBs in Option PR3 is a hardcoded limit. Agreement: - For Option PR1, - The relaxed constraint is 1 (instead of 4). - Other values for the relaxed constraint that meet the 10-Mbps peak rate target can optionally be studied. - The parameters (, , ) [38.306] can be as in Rel-17 RedCap. - For Option PR2, - For 15 kHz SCS, the maximum TBS is 10000 bits per TB and per slot. - For 30 kHz SCS, the maximum TBS is 5000 bits per TB and per slot. - For Option PR3, - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 11. - Other number of RBs that meet the 10-Mbps peak rate target can optionally be studied. - Note: It is not precluded to report results also for other values. - Relevant assumptions (e.g., regarding potential limitations of the TBS sum in case of more than one simultaneous TB) should be reported.

Based on the above agreements, the following text proposal for the TR can be considered.

In the study, the main UE peak rate reduction options considered for FR1 are as follows: - Option PR1: Relaxation of the constraint (vLayersj⋅Qmj⋅fj≥4) for peak data rate reduction. - The relaxed constraint is 1 (instead of 4). - The parameters (, , ) [38.306] can be as in Rel-17 RedCap. - Option PR2: Restriction of maximum TBS for PDSCH and PUSCH. - For 15 kHz SCS, the maximum TBS is 10000 bits per TB and per slot. - For 30 kHz SCS, the maximum TBS is 5000 bits per TB and per slot. - Option PR3: Restriction of maximum number of PRBs for PDSCH and PUSCH. - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 11. - The restricted number of PRBs in Option PR3 is a hardcoded limit. For the above peak rate reduction options, the following aspects are considered: - The studied peak rate reduction applies to both UE-specific (unicast) and common (broadcast) channels. - The resource allocation spans a bandwidth of maximum 20 MHz (maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only.

FL1 High Priority Question 7.3.1-1a: Can the above description on the UE peak rate reduction options be used as a baseline text for TR 38.865?

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nordic | Y |  |
| Lenovo | Y |  |
| CATT | Y |  |
| SONY | Y |  |
| FUTUREWEI | Y |  |
| Xiaomi | Y |  |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Samsung | Y |  |
| Qualcomm | Y |  |
| CMCC | Y |  |
| MediaTek | Y (with clarification) | For PR3, we would like to include additional description for clarifying the difference from BW3 (of the same RB limit as PR3): For the above peak rate reduction options, the following aspects are considered: - … - The resource allocation spans a bandwidth of maximum 20 MHz (maximum UE channel bandwidth). PR3 assumes post-FFT buffering of 20MHz span of PDSCH since precise RB location is not possibly known before DCI information is successfully extracted. |
| Huawei, Hisilicon | Y |  |
| FL3 FL4 FL5 | The received responses support the description of the PR options. One response suggests a clarification for PR3. The following proposal can be considered as a baseline for the TP drafting for the TR. High Priority Proposal 7.3.1-1b: Use the following TP as baseline text for TR 38.865. In the study, the main UE peak rate reduction options considered for FR1 are as follows: - Option PR1: Relaxation of the constraint (vLayersj⋅Qmj⋅fj≥4) for peak data rate reduction. - The relaxed constraint is 1 (instead of 4). - The parameters (, , ) [38.306] can be as in Rel-17 RedCap. - Option PR2: Restriction of maximum TBS for PDSCH and PUSCH. - For 15 kHz SCS, the maximum TBS is 10000 bits per TB and per slot. - For 30 kHz SCS, the maximum TBS is 5000 bits per TB and per slot. - Option PR3: Restriction of maximum number of PRBs for PDSCH and PUSCH. - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 11. - The restricted number of PRBs in Option PR3 is a hardcoded limit. For the above peak rate reduction options, the following aspects are considered: - The studied peak rate reduction applies to both UE-specific (unicast) and common (broadcast) channels. - The resource allocation spans a bandwidth of maximum 20 MHz (maximum UE channel bandwidth). - Note: Different from BW3, PR3 assumes post-FFT buffering of 20 MHz span of PDSCH since precise RB location is not possibly known before DCI information is successfully extracted. - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only.. |
| CATT | Y in general | If cross-slot scheduling is applied, PR3 may still reduce post FFT buffering. For example, after decoding DCI and abstracting FDRA, the UE directly pick out the allocated PRBs (although distributed) from the output of FFT/IFFT, and deliver them into post FFT/IFFT buffering. However, when cross-slot scheduling is not allowed, PR3 cannot reduce post FFT buffering, but BW3 may still do (e.g. the 5 MHz is fixed/predefined within 20MHz). This makes PR3 and BW3 different. So in general, BW3 has higher potential in post FFT reduction than PR3. |
| Sharp | Y |  |
| Samsung | Y | Some clarification on the difference between PR3 and BW 3 needs to be captured in the TR. We think thePR3 and BW3 can share same note for Post FFT buffering. |
| FUTUREWEI | Y | The note in Red should be moved, perhaps 7.3.2. In addition, we have similar observation as CATT that “BW3 has higher potential in post FFT reduction than PR3” |
| OPPO | Y | We agreed to add the Red note, with following one more sentence in the note. “The cost reduction would be largely shared between BW3 and PR3, especially if the cross-slot scheduling is not mandated.” |
| Nordic | N | The added text is technically incorrect, because also in BW3 UE does not know in which part of 20MHz BWP the PDSCH is scheduled until it decodes scheduling PDCCH |
| Xiaomi2 | Y in general | PR3 has the same cost saving as BW3. After DCI is decoded, the exact frequency domain location can be identified. Thus, only storing the allocated PRBs is sufficient. Of course, it may effect the post-FFT buffer matrix design and the time-frequency domain location index should be additional recorded. |
| Spreadtrum |  | As discussed by some companies in the offline, we also think the red part should be removed. - Note: Different from BW3, PR3 assumes post-FFT buffering of 20 MHz span of PDSCH since precise RB location is not possibly known before DCI information is successfully extracted. |
| Nokia, NSB |  | Note is not needed. Differences between BW3 and PR3 with respect to post-FFT buffering can be captured in the analysis section if necessary. |
| FL6 | Based on the discussion in an offline session on Wednesday 24th August, the following proposal can be considered. High Priority Proposal 7.3.1-1c: Use the following TP as baseline text for TR 38.865. In the study, the main UE peak rate reduction options considered for FR1 are as follows: - Option PR1: Relaxation of the constraint (vLayersj⋅Qmj⋅fj≥4) for peak data rate reduction. - The relaxed constraint is 1 (instead of 4). - The parameters (, , ) [38.306] can be as in Rel-17 RedCap. - Option PR2: Restriction of maximum TBS for PDSCH and PUSCH. - For 15 kHz SCS, the maximum TBS is 10000 bits per TB and per slot. - For 30 kHz SCS, the maximum TBS is 5000 bits per TB and per slot. - Option PR3: Restriction of maximum number of PRBs for PDSCH and PUSCH. - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 11. - The restricted number of PRBs in Option PR3 is a hardcoded limit. For the above peak rate reduction options, the following aspects are considered: - The studied peak rate reduction applies to both UE-specific (unicast) and common (broadcast) channels. - The resource allocation spans a bandwidth of maximum 20 MHz (maximum UE channel bandwidth). - Note: Different from BW3, PR3 assumes post-FFT buffering of 20 MHz span of PDSCH since precise RB location is not possibly known before DCI information is successfully extracted. - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only.. |
| FL7 | The online session on Wednesday 24th August made the following agreement: Agreement: Use the following TP as baseline text for TR 38.865. In the study, the main UE peak rate reduction options considered for FR1 are as follows: - Option PR1: Relaxation of the constraint (vLayersj⋅Qmj⋅fj≥4) for peak data rate reduction. - The relaxed constraint is 1 (instead of 4). - The parameters (, , ) [38.306] can be as in Rel-17 RedCap. - Option PR2: Restriction of maximum TBS for PDSCH and PUSCH. - For 15 kHz SCS, the maximum TBS is 10000 bits per TB and per slot. - For 30 kHz SCS, the maximum TBS is 5000 bits per TB and per slot. - Option PR3: Restriction of maximum number of PRBs for PDSCH and PUSCH. - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 11. - The restricted number of PRBs in Option PR3 is a hardcoded limit. For the above peak rate reduction options, the following aspects are considered: - The studied peak rate reduction applies to both UE-specific (unicast) and common (broadcast) channels. - The resource allocation spans a bandwidth of maximum 20 MHz (maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only.. |

### 7.3.2	Analysis of UE complexity reduction

Many contributions discuss potential UE complexity reduction from different UE peak rate reduction options. Depending on the peak reduction option, the complexity of different UE functional blocks (listed in Table 1) can be reduced.

Based on the majority of provided cost estimates, the following observations can be made for different peak rate reduction options.

For PR1 and PR2, the main contributors of the cost reduction are the following functional blocks: - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block For PR3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block

FL1 High Priority Question 7.3.2-1a: Can the observations listed in the text box above regarding the PR options be captured in the TR? If no, please elaborate on what modifications are needed in the Comments field.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum |  | From our perspective, Receiver processing block is another main contributor of the cost reduction for PR1/PR2. We prefer to add it under PR1/PR2. |
| Nokia, NSB | Y |  |
| Nordic | N | No UL processing cost saving is possible for PR1/2 |
| CATT | Y |  |
| SONY | N | For PR3, post-FFT data buffering also contributes to complexity saving. Once PDCCH has been decoded, the UE only needs to buffer the PRBs of interest. This seems similar to BW3. |
| FUTUREWEI | Y |  |
| Xiaomi | N | Since the UE BW is not reduced, channel UL processing cost saving is marginal for PR3, which is not the main contributors of the cost reduction. |
| ZTE, Sanechips | Y |  |
| Intel | N | Agree with Sony. For PR3, the post-FFT data buffering can also be reduced since the number of PRBs is limited to 25 or 11 instead of 106 or 51. |
| Samsung |  | Further clarify on assumption of post-FFT buffer is preferred |
| CMCC | Y |  |
| MediaTek | N | Agree with Nordic, there is little saving in UL processing for PR1/2. |
| Huawei, Hisilicon | Y |  |
| FL3 FL4 FL5 FL7 | One received responses indicates that the Receiver processing block should be included in the list for PR1/PR2. However, based on the provided results by other companies, no reduction is observed for Receiver processing block for PR1/2/3. Three responses express that no/small UL processing cost saving is possible for PR1/2/3. Regarding post-FFT buffer for PR3, there is a clarification note in Proposal 7.3.1-1b which can be discussed: - Note: Different from BW3, PR3 assumes post-FFT buffering of 20 MHz span of PDSCH since precise RB location is not possibly known before DCI information is successfully extracted. Based on the above views, the following question can be considered: High Priority Question 7.3.2-1b: Can the complexity of UL processing block be reduced with PR1/PR2/PR3? |  |
| CATT | Y |  |
| Sierra Wireless | Y |  |
| Sharp | Y |  |
| Samsung |  | OK for PR3. Not for PR1. We’d like to hear more view for PR 2 |
| FUTUREWEI | See comment | In our understanding, the complexity of TB encoding can be reduced for P1/P2/P3. But encoding is fairly small in complexity. For PR3, it has further reduction in FFT since 5 MHz is maximum bandwidth (assuming UL is using single carrier) |
| OPPO | Yes |  |
| Spreadtrum | Y | From our perspective, the complexity of UL processing block be reduced with PR1/PR2/PR3. |
| Nokia, NSB | Y | We think UL processing block e.g. encoding complexity can be reduced. |
| Ericsson | Y | Assuming PUSCH preparation is included in the UL processing block, the complexity UL processing block can be reduced with PR1/PR2/PR3. |
| vivo | Y |  |
| Intel | Y |  |
| Huawei, Hisilicon | Y |  |
| SONY | Y |  |
| ZTE, Sanechips | Y | The UL TBS reduction can bring less encoding complexity and encoding time. |
| CMCC | Y |  |
| FL8 | Based on offline discussion on Wednesday 24th August and received responses, the following proposal can be considered. High Priority Proposal 7.3.2-1c: Adopt the following TP as a baseline text for TR 38.865. For PR1 and PR2, the main contributors of the cost reduction are the following functional blocks: - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block For PR3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block. |
| FL9 | The online session on Thursday 25th August made the following agreement: Agreement: Adopt the following TP as a baseline text for TR 38.865, clause 7.3.2: For PR1 and PR2, the main contributors of the cost reduction are the following functional blocks: - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block For PR3, the main contributors of the cost reduction are the following functional blocks: - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block. |

The tables below show the average value of the initial cost evaluation results captured in [43]. Specifically, results are provided for Rel-17 RedCap and Rel-18 eRedCap cases with FD-FDD 1Rx, TDD 1Rx, HD-FDD 1Rx, FD-FDD 2Rx, TDD 2Rx, and HD-FDD 2Rx.

FD-FDD 1Rx

| Reduced UE peak rate | Rel-15 reference | Rel-17 RedCap | PR1 | PR2 | PR3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.12% | ## 24.12% | ## 24.12% | ## 24.12% |
| ## RF: Filters | ## 10% | ## 5.06% | ## 5.06% | ## 5.06% | ## 5.06% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 45% | ## 23.76% | ## 23.76% | ## 23.76% | ## 23.76% |
| ## RF: Duplexer / Switch | ## 20% | ## 19.52% | ## 19.52% | ## 19.52% | ## 19.52% |
| # RF: Total | # 100% | # 72.46% | # 72.46% | # 72.46% | # 72.46% |
| ## BB: ADC / DAC | ## 10% | ## 1.30% | ## 1.26% | ## 1.26% | ## 1.26% |
| ## BB: FFT/IFFT | ## 4% | ## 0.67% | ## 0.65% | ## 0.65% | ## 0.65% |
| ## BB: Post-FFT data buffering | ## 10% | ## 1.05% | ## 1.07% | ## 1.07% | ## 1.01% |
| ## BB: Receiver processing block | ## 24% | ## 4.42% | ## 4.21% | ## 4.21% | ## 2.46% |
| ## BB: LDPC decoding | ## 10% | ## 1.29% | ## 0.45% | ## 0.44% | ## 0.49% |
| ## BB: HARQ buffer | ## 14% | ## 1.46% | ## 0.43% | ## 0.41% | ## 0.43% |
| ## BB: DL control processing & decoder | ## 5% | ## 4.73% | ## 4.67% | ## 4.67% | ## 4.55% |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.61% | ## 4.58% | ## 4.58% | ## 4.58% |
| ## BB: UL processing block | ## 5% | ## 2.69% | ## 1.94% | ## 1.89% | ## 1.66% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 4.04% | ## 3.91% | ## 3.91% | ## 3.91% |
| # BB: Total | # 100% | # 26.26% | # 23.18% | # 23.09% | # 21.00% |
| RF+BB: Total | 100% | 44.74% | 42.89% | 42.84% | 41.58% |

TDD 1Rx

| Reduced UE peak rate | Rel-15 reference | Rel-17 RedCap | PR1 | PR2 | PR3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.08% | ## 24.08% | ## 24.08% | ## 24.08% |
| ## RF: Filters | ## 15% | ## 3.94% | ## 3.94% | ## 3.94% | ## 3.94% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 55% | ## 19.05% | ## 19.05% | ## 19.05% | ## 19.05% |
| ## RF: Duplexer / Switch | ## 5% | ## 4.97% | ## 4.97% | ## 4.97% | ## 4.97% |
| # RF: Total | # 100% | # 52.04% | # 52.04% | # 52.04% | # 52.04% |
| ## BB: ADC / DAC | ## 9% | ## 0.76% | ## 0.73% | ## 0.73% | ## 0.73% |
| ## BB: FFT/IFFT | ## 4% | ## 0.40% | ## 0.39% | ## 0.39% | ## 0.39% |
| ## BB: Post-FFT data buffering | ## 10% | ## 0.59% | ## 0.57% | ## 0.57% | ## 0.56% |
| ## BB: Receiver processing block | ## 29% | ## 3.21% | ## 3.03% | ## 3.03% | ## 1.84% |
| ## BB: LDPC decoding | ## 9% | ## 0.79% | ## 0.30% | ## 0.29% | ## 0.32% |
| ## BB: HARQ buffer | ## 12% | ## 0.79% | ## 0.39% | ## 0.38% | ## 0.39% |
| ## BB: DL control processing & decoder | ## 4% | ## 3.68% | ## 3.58% | ## 3.58% | ## 3.58% |
| ## BB: Synchronization / cell search block | ## 9% | ## 2.49% | ## 2.45% | ## 2.45% | ## 2.45% |
| ## BB: UL processing block | ## 5% | ## 2.69% | ## 1.91% | ## 1.85% | ## 1.66% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 2.27% | ## 2.21% | ## 2.21% | ## 2.22% |
| # BB: Total | # 100% | # 17.66% | # 15.56% | # 15.49% | # 14.14% |
| RF+BB: Total | 100% | 31.41% | 30.15% | 30.11% | 29.30% |

HD-FDD 1Rx

| Reduced UE peak rate | Rel-15 reference | Rel-17 RedCap | PR1 | PR2 | PR3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.50% | ## 24.50% | ## 24.50% | ## 24.50% |
| ## RF: Filters | ## 10% | ## 5.17% | ## 5.17% | ## 5.17% | ## 5.17% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 45% | ## 24.46% | ## 24.46% | ## 24.46% | ## 24.46% |
| ## RF: Duplexer / Switch | ## 20% | ## 3.68% | ## 3.68% | ## 3.68% | ## 3.68% |
| # RF: Total | # 100% | # 57.81% | # 57.81% | # 57.81% | # 57.81% |
| ## BB: ADC / DAC | ## 10% | ## 1.25% | ## 1.25% | ## 1.25% | ## 1.25% |
| ## BB: FFT/IFFT | ## 4% | ## 0.71% | ## 0.71% | ## 0.71% | ## 0.71% |
| ## BB: Post-FFT data buffering | ## 10% | ## 1.10% | ## 1.10% | ## 1.10% | ## 1.04% |
| ## BB: Receiver processing block | ## 24% | ## 4.22% | ## 4.05% | ## 4.04% | ## 2.37% |
| ## BB: LDPC decoding | ## 10% | ## 1.41% | ## 0.50% | ## 0.49% | ## 0.56% |
| ## BB: HARQ buffer | ## 14% | ## 1.61% | ## 0.48% | ## 0.46% | ## 0.49% |
| ## BB: DL control processing & decoder | ## 5% | ## 4.70% | ## 4.70% | ## 4.70% | ## 4.49% |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.68% | ## 4.68% | ## 4.68% | ## 4.68% |
| ## BB: UL processing block | ## 5% | ## 3.10% | ## 2.07% | ## 2.00% | ## 1.77% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 3.81% | ## 3.81% | ## 3.81% | ## 3.81% |
| # BB: Total | # 100% | # 26.58% | # 23.33% | # 23.24% | # 21.20% |
| RF+BB: Total | 100% | 39.07% | 37.12% | 37.06% | 35.90% |

FD-FDD 2Rx

| Reduced UE peak rate | Rel-15 reference | Rel-17 RedCap | PR1 | PR2 | PR3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.87% | ## 24.87% | ## 24.87% | ## 24.87% |
| ## RF: Filters | ## 10% | ## 10.00% | ## 9.56% | ## 9.29% | ## 9.29% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 45% | ## 42.47% | ## 42.47% | ## 40.32% | ## 40.32% |
| ## RF: Duplexer / Switch | ## 20% | ## 20.00% | ## 20.00% | ## 20.00% | ## 20.00% |
| # RF: Total | # 100% | # 97.33% | # 96.90% | # 94.48% | # 94.48% |
| ## BB: ADC / DAC | ## 10% | ## 2.12% | ## 2.05% | ## 2.05% | ## 2.05% |
| ## BB: FFT/IFFT | ## 4% | ## 1.25% | ## 1.20% | ## 1.20% | ## 1.20% |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.17% | ## 2.11% | ## 2.11% | ## 2.08% |
| ## BB: Receiver processing block | ## 24% | ## 8.97% | ## 8.68% | ## 8.68% | ## 5.31% |
| ## BB: LDPC decoding | ## 10% | ## 2.71% | ## 0.93% | ## 0.90% | ## 1.08% |
| ## BB: HARQ buffer | ## 14% | ## 2.53% | ## 0.84% | ## 0.80% | ## 0.88% |
| ## BB: DL control processing & decoder | ## 5% | ## 4.93% | ## 4.93% | ## 4.93% | ## 4.93% |
| ## BB: Synchronization / cell search block | ## 9% | ## 8.61% | ## 8.36% | ## 8.36% | ## 8.36% |
| ## BB: UL processing block | ## 5% | ## 3.20% | ## 2.12% | ## 2.12% | ## 2.11% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 7.33% | ## 7.08% | ## 7.08% | ## 7.08% |
| # BB: Total | # 100% | # 43.84% | # 38.30% | # 38.23% | # 35.07% |
| RF+BB: Total | 100% | 65.24% | 61.74% | 60.73% | 58.83% |

TDD 2Rx

| Reduced UE peak rate | Rel-15 reference | Rel-17 RedCap | PR1 | PR2 | PR3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 24.44% | ## 24.44% | ## 24.44% | ## 24.44% |
| ## RF: Filters | ## 15% | ## 7.69% | ## 7.69% | ## 7.69% | ## 7.69% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 55% | ## 33.61% | ## 33.14% | ## 33.14% | ## 33.14% |
| ## RF: Duplexer / Switch | ## 5% | ## 4.99% | ## 4.99% | ## 4.99% | ## 4.99% |
| # RF: Total | # 100% | # 70.72% | # 70.25% | # 70.25% | # 70.25% |
| ## BB: ADC / DAC | ## 9% | ## 1.13% | ## 1.13% | ## 1.13% | ## 1.13% |
| ## BB: FFT/IFFT | ## 4% | ## 0.66% | ## 0.68% | ## 0.68% | ## 0.68% |
| ## BB: Post-FFT data buffering | ## 10% | ## 1.17% | ## 1.21% | ## 1.19% | ## 1.20% |
| ## BB: Receiver processing block | ## 29% | ## 5.82% | ## 5.71% | ## 5.71% | ## 3.55% |
| ## BB: LDPC decoding | ## 9% | ## 1.33% | ## 0.54% | ## 0.53% | ## 0.61% |
| ## BB: HARQ buffer | ## 12% | ## 1.16% | ## 0.41% | ## 0.40% | ## 0.43% |
| ## BB: DL control processing & decoder | ## 4% | ## 3.75% | ## 3.75% | ## 3.75% | ## 3.75% |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.50% | ## 4.63% | ## 4.63% | ## 4.63% |
| ## BB: UL processing block | ## 5% | ## 2.83% | ## 1.93% | ## 1.90% | ## 1.91% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 3.82% | ## 3.76% | ## 3.76% | ## 3.76% |
| # BB: Total | # 100% | # 26.17% | # 23.75% | # 23.68% | # 21.65% |
| RF+BB: Total | 100% | 43.99% | 42.35% | 42.31% | 41.09% |

HD-FDD 2Rx

| Reduced UE peak rate | Rel-15 reference | Rel-17 RedCap | PR1 | PR2 | PR3 |
| --- | --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 25.00% | ## 25.00% | ## 25.00% | ## 25.00% |
| ## RF: Filters | ## 10% | ## 10.00% | ## 10.00% | ## 10.00% | ## 10.00% |
| ## RF: Transceiver (incl. LNAs, mixer, and local oscillator) | ## 45% | ## 43.04% | ## 43.04% | ## 43.04% | ## 43.04% |
| ## RF: Duplexer / Switch | ## 20% | ## 4.33% | ## 4.33% | ## 4.33% | ## 4.33% |
| # RF: Total | # 100% | # 82.38% | # 82.38% | # 82.38% | # 82.38% |
| ## BB: ADC / DAC | ## 10% | ## 2.06% | ## 2.04% | ## 2.04% | ## 2.04% |
| ## BB: FFT/IFFT | ## 4% | ## 1.15% | ## 1.15% | ## 1.15% | ## 1.15% |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.14% | ## 2.14% | ## 2.14% | ## 2.09% |
| ## BB: Receiver processing block | ## 24% | ## 9.01% | ## 8.77% | ## 8.77% | ## 5.43% |
| ## BB: LDPC decoding | ## 10% | ## 2.69% | ## 0.94% | ## 0.90% | ## 1.11% |
| ## BB: HARQ buffer | ## 14% | ## 2.41% | ## 0.82% | ## 0.78% | ## 0.87% |
| ## BB: DL control processing & decoder | ## 5% | ## 5.00% | ## 5.00% | ## 5.00% | ## 5.00% |
| ## BB: Synchronization / cell search block | ## 9% | ## 8.55% | ## 8.55% | ## 8.55% | ## 8.55% |
| ## BB: UL processing block | ## 5% | ## 3.17% | ## 2.09% | ## 2.09% | ## 2.07% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 7.23% | ## 7.23% | ## 7.23% | ## 7.23% |
| # BB: Total | # 100% | # 43.39% | # 38.74% | # 38.65% | # 35.55% |
| RF+BB: Total | 100% | 58.99% | 56.19% | 56.14% | 54.28% |

For comparison, the average cost reduction estimates from [43] for different peak rate reduction options compared to the corresponding Rel-17 RedCap baselines are presented in the table below.

Table 3: Average cost reduction achieved by peak rate reduction options compared to corresponding Rel-17 baselines

| Option | FD-FDD 1Rx | TDD 1Rx | HD-FDD 1Rx | FD-FDD 2Rx | TDD 2Rx | HD-FDD 2Rx |
| --- | --- | --- | --- | --- | --- | --- |
| PR1 | 4.13% | 4.02% | 4.99% | 5.36% | 3.73% | 4.74% |
| PR2 | 4.26% | 4.16% | 5.14% | 6.91% | 3.82% | 4.82% |
| PR3 | 7.06% | 6.74% | 8.12% | 9.81% | 6.59% | 7.98% |

FL1 High Priority Question 7.3.2-2a: Can the table formats used in the above 7 tables with cost reduction estimates for the PR options be used as a baseline text for TR 38.865? (Regarding the averaging method, please see Question 6.1-2a.)

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| Nordic | Y |  |
| Lenovo | Y |  |
| CATT | Y |  |
| SONY | Y |  |
| FUTUREWEI | Y |  |
| Xiaomi |  | Suggest to reflect optional evaluation cases in these tables, such as HD-FDD with 1RX, TDD with 2 RX, etc. |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Qualcomm | Y |  |
| CMCC | Y |  |
| MediaTek | Y (with square brackets) | Due to the dependency with averaging method, if this proposal is to be agreed prior to the decision on averaging method, we suggest to include square brackets on the values. |
| FL2 | Based on received responses, the following proposal can be considered. High Priority Proposal 7.3.2-2b: The table formats used in the 7 tables with cost reduction estimates for the PR options listed in the end of Section 7.3.2 in R1-2207731 are used as a baseline text for TR 38.865. |  |
| FL3 | RAN1 made the following conclusion in the online session on Monday 22nd August: Conclusion: The table formats used in the 7 tables with complexity reduction estimates for the PR options listed in the end of Section 7.3.2 in R1-2207731 are used as a baseline text for TR 38.865. Note: values in the table may be updated. |  |
| FL5 FL6 | The 7 tables above this feedback form show the average values of the initial cost evaluation results captured in [43]. High Priority Proposal 7.3.2-2c: The values in the 7 tables with cost reduction estimates for the PR options listed in the end of Section 7.3.2 in R1-2207733 are used as a baseline text for TR 38.865. |  |
| CATT | Y |  |
| Sierra Wireless | Y |  |
| Sharp | Y |  |
| OPPO | Y |  |
| Nordic | N | Only with PR3 there is reduction on Receiver processing, this because demodulation, channel estimation, rate-matching is made on all 106/51 RBs for PR1 and PR2. Regarding post data buffering, BW3 does not different from PR3, both must buffer 20MHz until PDCCH is decoded. But we agree that compared to R17, post-FFT buffering can be slightly reduced for PR3/BW3. |
| Xiaomi2 | Y |  |
| Nokia, NSB | Y |  |
| FL7 | The online session on Wednesday 24th August made the following conclusion: Conclusion: The values in the 7 tables with cost reduction estimates for the PR options listed in the end of Section 7.3.2 in R1-2207733, with FD-FDD 1Rx corrected average, are used as a baseline text for TR 38.865. (The formulation “with FD-FDD 1Rx corrected average” refers to correction of the averaging across companies on the ‘FD-FDD 1Rx’ tab in [43] compared to the earlier version in [10]. In [10], the averages on this tab did not include the evaluation results from three companies, corresponding to columns C, D and E on that tab, but this has been fixed in [43].) |  |

### 7.3.3	Analysis of performance impacts

Many contributions analyze the potential performance impacts of peak rate reduction options for Rel-18 eRedCap. The findings are summarized below.

- There is no impact to coverage, network deployment, and coexistence for PR1/PR2/PR3. There are small impacts to cell spectral efficiency, power consumption, and latency [11]. (6)

- PR1/PR2/PR3 can fulfil the data rate requirements of in Rel-18 [11, 14, 20, 31, 35]. (17)

- For PR1, for high modulation order or 2 layers, the existing values of cannot satisfy the requirement of peak data rate reduction in Rel-18 [33]. (1)

- For PR1/PR2/PR3, no coverage loss was expected [11, 14, 20]. (14)

- For Option PR3, there would be coverage loss on PDSCH/PUSCH compared with legacy UEs and Options PR1/2 [19]. (1)

FL1/FL3/FL4/FL5/FL7 High Priority Question 7.3.3-1a: Considering the above observations on the UE peak rate reduction, what performance impacts for PR1, PR2, PR3 should be considered as a baseline for the TP drafting for TR section 7.3.3?

| Company | Impacts (1-5) | Comments or suggested revisions |
| --- | --- | --- |
| Xiaomi | 2, 4 | - For PR3, one issue needs to be clarified: Whether the number of RBs allocated for PDSCH and PUSCH is always less than 25RBs@15KHz or 11RBs@30KHZ? If so, both SIBx, paging and RACH messages in initial access phase should always be scheduled within 5MHZ and early indication may be needed in order to avoid the scheduling flexibility of legacy UEs. Otherwise, no coexistence issue exists, and the actually number of RBs reception or transmission by eRedCap UE can be less the allocated ones. 3. The assumption of high modulation order is not appropriate. In any case, the eRedcap UE can report a lower modulation order than the actually scheduled one to calculate the peak data rate. |
| CMCC | 4 |  |
| FL | If there are impacts among the listed potential impacts that should NOT be listed among the impacts in the TR section, please mention them in the Comments field and elaborate. |  |
| CATT | 2, 4 | Agree with Xiaomi. |
| Sharp | 2,4 | For 1, the coexistence issue for PR3 can be observed if the gNB schedule a SIB with PRBs wider than 25/11 |
| Samsung | 4 | For 2, suggest follow the same as for BW, i.e., separated for TDD and FDD. |
| FUTUREWEI | 2, 4 | For 4, “was” -> “is” |
| OPPO | 1,2,4 |  |
| Nordic |  | Should we wait for outcome of 9.6.2. to conclude whether there is coverage issue or not? Also 3 is not technically correct in our opinion. This because, every UE can indicate single layer and BPSK if it likes, so every UE may satisfy the limit. |
| Spreadtrum | 1,2,4 |  |
| Nokia, NSB | 2, 4 |  |
| Ericsson | 1, 2, 4 |  |
| vivo | 2 |  |
| Intel | 2, |  |
| DOCOMO | 2, 3, 4 | For peak data rate reduction, given that it was not agreed to restrict the modulation order/number of MIMO layer which can be supported by UE, the peak data rate can be a bit larger than the target peak data rate of Rel-18 RedCap (i.e., 10Mbps). For example, peak data rate of PR3 can be 20 Mbps with 64QAM, 1 MIMO layer, 25 RB for 15 kHz SCS and scaling factor of 1. We discussed that memory size would be laniary reduced depending on the peak data rate. In that sense, if the achievable peak data rate is restricted no to be much larger than 10 Mbps (e.g., restrict the scaling factor that Rel-18 RedCap UE does not support scaling factor of 1. or introducing new smaller value of scaling factor, etc.), we can ensure the further UE complexity reduction. Therefore, to discuss whether such restriction is necessary or not in the later stage, we think it also should be captured that peak data rate would not be reduced sufficiently with some parameters. |
| Huawei, Hisilicon | 2,4 |  |
| SONY | 1, 2, 4 |  |
| Qualcomm | 2, 4 |  |
| ZTE, Sanechips | 1, 2 | Similar, maybe we can start with the following paragraph with some modifications based on our option selection, - There is no impact to coverage, network deployment, and coexistence for PR1/PR2/PR3. There are some impacts to cell spectral efficiency, power consumption, and latency [11].] - PR1/PR2/PR3 can fulfil the data rate requirements of in Rel-18 [11, 14, 20, 31, 35]. |
| Panasonic | 2, [4,] 5 | 5: For PR3, compared with PR1/2, there would be a potential loss of link performance because the number of RB is limited. The limited number of RBs causes the reduced Rx power as the amount of power boosting of gNB is limited. The limited number of RBs also cause the higher coding rate of PDSCH/PUSCH when the TB size is not changed compared to the case of no limitation. 4: As commented by Nordic, it may be better to wait the 9.6.2 outcome. |
| CMCC | 1,2,4 |  |
| FL8 | The number of received responses that support capturing a specific impact is indicated with red text in the bullet list above this feedback form. High Priority Proposal 7.3.3-1b: Capture the highlighted impacts in R1-2207734 section 7.3.3 in the TR. |  |
| FL9 | The online session on Thursday 25th August made the following agreement: Agreement: The highlighted impacts in R1-2207734 are adopted as starting point for TR input. |  |

Based on the above agreement, the following proposal can be considered.

FL9 High Priority Proposal 7.3.3-1c: Adopt the following TP as a baseline text for TR 38.865 clause 7.3.3:

| Peak data rate: - PR1/PR2/PR3 can fulfil the data rate requirements of in Rel-18. Coverage: - For PR1/PR2/PR3, no coverage loss was expected. |  |  |
| --- | --- | --- |
| Company | Y/N | Comments |
| FL10 | The following updated proposal can be considered. High Priority Proposal 7.3.3-1d: Adopt the following TP as a baseline text for TR 38.865 clause 7.3.3: Peak data rate: - The UE peak rate reduction options (PR1/PR2/PR3) can all fulfil the data rate requirements. Coverage: - For the UE peak rate reduction options (PR1/PR2/PR3), no coverage loss is expected.. |
| Nordic | Not OK | PR3 should have similar coverage impact as BW3, due to truncation of SIB1 |
| Nokia, NSB |  | OK with peak data rate part. Agree that PR3 would have similar impact to BW3 on SIB1, and we can use similar observation for BW3 from 9.6.2 here. |
| vivo |  | We share Nordic’s views that there may be coverage loss in case the frequency domain resource allocation for SIB1/OSI/RAR/MSG3 is beyond 5MHz. |
| CATT | Y in general | Agree with Nordic. Maybe we can apply similar description for PR3 as BW3: For PR3, there is coverage impact for SIB1-PDSCH if the bandwidth allocation for SIB1 PDSCH exceeds 5 MHz. |
| OPPO | Y |  |
| Qualcomm |  | Share the same view with Nordic and others regarding coverage impact of PR3. |
| ZTE, Sanechips |  | We agree PR3 can have similar coverage impacts as BW3. Additionally, the following also should be captured. “For peak data rate reduction options, there are some impacts to cell spectral efficiency. ” |
| Spreadtrum | Y |  |
| Intel |  | Same view as Nordic. Suggest to separate PR1/PR2 and PR3 for coverage aspect |
| FUTUREWEI |  | PR3 has similar impact as BW3 |
| CMCC |  | Agree with Nordic, may be the following can be added. For PR3, there is coverage impact for SIB1-PDSCH if the BW allocation for SIB1 PDSCH exceeds the maximum number of RBs for PR3. |
| FL11 | The online session on Friday 26th August made the following agreement: Agreement: Adopt the following TP as a baseline text for TR 38.865 clause 7.3.3: Peak data rate: - The UE peak rate reduction options (PR1/PR2/PR3) can all fulfil the data rate requirements. Coverage: - For the UE peak rate reduction options (PR1/PR2[/PR3]), no coverage loss is expected.. |

### 7.3.4	Analysis of network deployment and coexistence impacts

Many contributions discuss the potential network deployment and coexistence impacts of peak rate options for Rel-18 eRedCap. The findings are summarized below.

- For PR1 and PR2, there is no or small coexistence issue [12, 15]. (15)

- For PR3, similar as BW3, it requires SIB1, OSI, RAR, MSG4 are scheduled within 5MHz which may have some impact on legacy UEs [15]. (8)

- For PR3, a scheduling limitation for SIB was observed [14]. (6)

- For option PR3, configurable resource would be restricted for SIB1 if it is shared between legacy and Rel-18 eRedCap UEs [37]. (7)

- PR3 will introduce scheduling restrictions for shared SIB1 transmission and for Msg4 without early identification [29]. (5)

- For Option PR3, coexistence impacts are observed in terms RACH resource sharing, paging sharing and SIB sharing [20]. (7)

- For PR2, the use of MCS is restricted, which is dependent on the size of transmission resource of PDSCH or PUSCH [33]. (4)

- PR1, PR2, & PR3 bring scheduling restriction for the transmission of system information due to peak rate restriction [35]. (6)

- For PR1, PR2, & PR3, early indication may be required for indicating peak rate restriction for random access procedure [35]. (5)

FL1/FL3/FL4/FL5/FL7 High Priority Question 7.3.4-1a: Considering the above observations on the UE peak rate reduction, what network deployment and coexistence impacts for PR1, PR2, PR3 should be considered as a baseline for the TP drafting for TR section 7.3.4?

| Company | Impacts (1-9) | Comments or suggested revisions |
| --- | --- | --- |
| Xiaomi | 1 | - “Paging” should be added. 2, 3, 4, 5, 6, 9. As we mentioned in Question 7.3.3-1a, further clarification is needed. 7. We can’t see the need to restrict the use of MCS. For TBS restriction, it can be implemented by gNB scheduling with appropriate time and frequency domain resource allocation, MCS and MIMO layer indication. 8. All three option PRs can satisfy the peak data rate requirement of eRedCap UE. And, according to DL peak data rate 10Mbps, the maximum TBS is about 10000bits and 5000bits in TDD and FDD band, separately, which is much larger than the TBS of SIBx. So, it is hard to say the scheduling of system information is limited by the reduced peak data rate. |
| FL | If there are impacts among the listed potential impacts that should NOT be listed among the impacts in the TR section, please mention them in the Comments field and elaborate. |  |
| CATT | 1-6, 8-9 | 1 is the essential one. For others, in fact most of them concerns about common channel sharing (e.g. SIB1, OSI, Msg2, paging …) with legacy/R17 RedCap UEs. We are fine if FL would like to pick out/combine some of them. |
| Sharp | all |  |
| Samsung |  | Most of the observation on BW 3 can be applied to PR3 |
| FUTUREWEI | 1-7 | Selecting 4 assuming it should be Rel-17, not Rel-18. Would consider 9 if it were only PR3 |
| OPPO |  | We also think BW3 can have similar observation as PR3. May be we can have that kind of conclusion before adding something specific for PR3. |
| Nordic |  | 7, and 9 should be still clarified. |
| Spreadtrum | At least 1,2 |  |
| Nokia, NSB | 1 |  |
| Ericsson | 1 | The max TBS for SIB, as per 38.331, is only 2976 bits. So, we do not expect PR1/PR2/PR3 to impose scheduling restrictions on SIB transmission. |
| vivo | 1, 2, 6, 8, 9 |  |
| Intel | 1, |  |
| DOCOMO | 1, 4 |  |
| Huawei, Hisilicon | 1-4,6-8 | For 5, even with early identification, restrictions for shared SIB1 transmission is still needed |
| SONY | 1 | 1: We also think that PR3 has no or small coexistence issues |
| Qualcomm | 8, 9 |  |
| ZTE, Sanechips | 1-6, 9 | For 8, the current 38.214 specifies a TBS for SI does not exceed 2976 bits. Hence, the peak data rate PR1, PR2 and PR3 do not impact the scheduling of system information. For 7, it should be moved to specification impacts. |
| Panasonic | 1 |  |
| CMCC | 1-8 |  |
| FL8 | The number of received responses that support capturing a specific impact is indicated with red text in the bullet list above this feedback form. High Priority Proposal 7.3.4-1b: Capture the highlighted impacts in in R1-2207734 section 7.3.4 the TR. |  |
| FL9 | The online session on Thursday 25th August made the following agreement: Agreement: The highlighted impacts in R1-2207734 are adopted as starting point for TR input. |  |

Based on the above agreement, the following proposal can be considered.

FL9 High Priority Proposal 7.3.4-1c: Adopt the following TP as a baseline text for TR 38.865 clause 7.3.4:

| Network deployment and coexistence impacts: - For PR1 and PR2, there is no or small coexistence issue. |  |  |
| --- | --- | --- |
| Company | Y/N | Comments |
| FL10 | The following updated proposal can be considered, where the impact “For PR3, similar as BW3, it requires SIB1, OSI, RAR, MSG4 are scheduled within 5MHz which may have some impact on legacy UEs” has been included, so that the section says something about all PR options, not just PR1 and PR2. High Priority Proposal 7.3.4-1d: Adopt the following TP as a baseline text for TR 38.865 clause 7.3.4: For UE peak rate reduction options PR1 and PR2, there is no or small coexistence issue. For UE peak rate reduction option PR3 (in the same way as for UE bandwidth reduction option BW3 described in clause 7.2), SIB1, OSI, RAR and MSG4 need to be scheduled within 5 MHz, otherwise there may be coexistence impacts on legacy UEs.. |
| Nordic | OK |  |
| DOCOMO | Y |  |
| Nokia, NSB | Y |  |
| vivo | Y |  |
| CATT | Y in general | Msg4 is UE-specific and will not be shared with legacy UE. Considering Rel-17 RedCap UE will always be identified no later than Msg3, Rel-18 RedCap UE may have similar way, and hence the gNB shall know it before Msg4. So Msg4 has no coexistence impact and should be deleted. |
| OPPO | OK |  |
| Qualcomm | Y |  |
| ZTE, Sanechips | Y |  |
| Spreadtrum | Y |  |
| Intel | Y | Agree with CATT. Maybe to add a clarification that - Note: whether RAR and MSG4 need to be scheduled within 5 MHz depends on the support of early identification |
| FUTUREWEI |  | For consistency, a statement similar to clause 7.2.4 should be added “Early RedCap UE indication (through Msg1/MsgA) might be needed for PR3 option.” |
| CMCC |  | For the impact of PR3, “otherwise there may be coexistence impacts on legacy UEs” is better changed to “which may have some impact on legacy UEs when those channels are shared ” |
| FL11 | The online session on Friday 26th August made the following agreement: Agreement: Adopt the following TP as a baseline text for TR 38.865 clause 7.3.4: For UE peak rate reduction options PR1 and PR2, there is no or small coexistence issue. For UE peak rate reduction option PR3 (in the same way as for UE bandwidth reduction option BW3 described in clause 7.2), SIB1, OSI, RAR and MSG4 need to be scheduled within 5 MHz, otherwise there may be coexistence impacts on legacy UEs. Early RedCap UE indication (through Msg1/MsgA) might be needed for PR3.. |

### 7.3.5	Analysis of specification impacts

Many contributions discuss the potential specification impacts of peak rate reduction options for Rel-18 eRedCap. The findings are summarized below.

- PR1/PR2/PR3 has minimal specification impact [11, 12, 14, 22]. (12)

- For Option PR1 and PR2, minor specification impact is foreseen for TBS calculation and/or data rate restriction [11, 20,29]. (13)

- For Option PR3, specification impact is foreseen on FDRA, RACH procedure, paging procedure and SIB transmission [20]. (9)

- For option PR1, no significant impact is expected for specification if the modulation order and/or number of MIMO layers supported by Rel-18 eRedCap is not restricted/reduced [37]. (1)

- For option PR2 and PR3, Rel-18 eRedCap early indication would be necessary [37]. (1)

- For option PR3, distributed resource allocation is not applied for uplink [35]. (1)

FL1/FL3/FL4/FL5/FL7 High Priority Question 7.3.5-1a: Considering the above observations on the UE peak rate reduction, what specification impacts for PR1, PR2, PR3 should be considered as a baseline for the TP drafting for TR section 7.3.5?

| Company | Impacts (1-5) | Comments or suggested revisions |
| --- | --- | --- |
| Xiaomi | 2 | 3, 5. Further clarification is needed. Besides, for sentence 5, we can’t see the necessity to introduce early indication mechanism for option PR2. 4. The restriction/reduction of modulation order and/or number of MIMO layers is out of scope, which should be removed. |
| Qualcomm | 1, 2, 3, 6 | 6 is newly added as distributed resource allocation is not supported for PUSCH in current spec. (38.214) So PR3 is only applied for DL in principle. |
| CMCC | 2 |  |
| FL | If there are impacts among the listed potential impacts that should NOT be listed among the impacts in the TR section, please mention them in the Comments field and elaborate. |  |
| CATT | 1-3 | In fact distributed resource allocation (FDRA type 0) can be used in PUSCH, when CP-OFDM waveform is configured. But anyway we think Point 6 is not as essential as Point 1,2 and 3, so no need to include it. |
| Sierra Wireless | 1-3 |  |
| Sharp | 1,2,3 |  |
| FUTUREWEI | 1-3 |  |
| Nordic |  | 5 is contradicting to “9. For PR1, PR2, & PR3, early indication may be required for indicating peak rate restriction for random access procedure [35]” -> Some discussion is needed. Otherwise, we tend to agree with the statements. |
| Spreadtrum | 1,2 |  |
| Nokia, NSB | 1 |  |
| Ericsson | 1, 2, 4 | Do NOT list: 5, 6 |
| Vivo | 2, 3 |  |
| Intel | 1, 2 |  |
| DOCOMO | 2, 5 | For 5, it can be applied only for PR3. |
| Huawei, Hisilicon | 1-3 |  |
| SONY | 1 |  |
| ZTE, Sanechips | 2,3 | Similar, maybe we can start with the following paragraph with some modifications based on our option selection, - For Option PR1 and PR2, minor some specification impact is foreseen for TBS calculation and/or data rate restriction [11, 20]. - For Option PR3, specification impact is foreseen on FDRA if potential enhancement is considered, RACH procedure, paging procedure and SIB transmission [20]. |
| Panasonic | 1 |  |
| CMCC2 | 2,3 | Also we add [29] in 2. |
| FL8 | The number of received responses that support capturing a specific impact is indicated with red text in the bullet list above this feedback form. High Priority Proposal 7.3.5-1b: Capture the highlighted impacts in R1-2207734 section 7.3.5 in the TR. |  |
| FL9 | The online session on Thursday 25th August made the following agreement: Agreement: The highlighted impacts in R1-2207734 are adopted as starting point for TR input. |  |

Based on the above agreement, the following proposal can be considered.

FL9 High Priority Proposal 7.3.5-1c: Adopt the following TP as a baseline text for TR 38.865 clause 7.3.5:

| Specification impacts: - PR1/PR2/PR3 has minimal specification impact. - For Option PR1 and PR2, minor specification impact is foreseen for TBS calculation and/or data rate restriction. |  |  |
| --- | --- | --- |
| Company | Y/N | Comments |
| FL10 | The following updated proposal can be considered, where the impact “For PR3, similar as BW3, it requires SIB1, OSI, RAR, MSG4 are scheduled within 5MHz which may have some impact on legacy UEs” has been included, so that the section says something about all PR options, not just PR1 and PR2. High Priority Proposal 7.3.5-1d: Adopt the following TP as a baseline text for TR 38.865 clause 7.3.5: The UE peak rate reduction options (PR1/PR2/PR3) all have minimal specification impact. For PR1 and PR2, minor specification impact is foreseen for TBS calculation and/or data rate restriction.. |
| Nordic | OK |  |
| DOCOMO | Y |  |
| Nokia, NSB | Y |  |
| vivo | Y |  |
| CATT | Y |  |
| OPPO | OK |  |
| Qualcomm |  | Current wording looks like that PR1/PR2 have bigger spec impacts than PR3 but it is not the case. We can either remove the 2nd sentence, or we add one additional sentence for PR3 spec impact, e.g., “For PR3, specification impact is foreseen for FDRA and/or resource restriction”. |
| ZTE, Sanechips | Y | Maybe for PR3, additional sentence can be : For Option PR3, potential specification impact is foreseen on RACH, paging and SIB transmission or resource allocation. |
| Spreadtrum | Y | Share similar views as ZTE. |
| Intel | Y | Agree with QC, ZTE and Spreadtrum |
| FUTUREWEI | Y | For consistency, a statement similar to clause 7.2.4 should be added “Early RedCap UE indication (through Msg1/MsgA) might be needed for PR3 option.” |
| CMCC | Y |  |
| FL11 | The online session on Friday 26th August made the following agreement: Agreement: Adopt the following TP as a baseline text for TR 38.865 clause 7.3.5: The UE peak rate reduction options (PR1/PR2/PR3) all have minimal specification impact.. |

## 7.4	Relaxed UE processing timeline

### 7.4.1	Description of feature

Relaxed UE processing timeline was part of the study in Rel-17 RedCap study item where the following description was captured in TR 38.875 [5].

In the [Rel-17] RedCap study item, relaxed UE processing time is considered in terms of more relaxed N1 and N2 values (as defined in TS 38.214) compared to those of UE processing time capability 1. In the study, for the purpose of evaluation, the relaxed UE processing time in terms of N1 and N2 are assumed to be doubled compared to those of capability 1, i.e., - N1 = 16, 20, 34, and 40 symbols for 15, 30, 60, and 120 kHz SCS (assuming only front-loaded DMRS) - N2 = 20, 24, 46, and 72 symbols for 15, 30, 60, and 120 kHz SCS In the study, for the purpose of evaluation, relaxed CSI computation time was also considered, assuming doubled Z and Z’ compared to the values defined in TS 38.214 clause 5.4.

RAN1#109e made the following agreements regarding the study of relaxed UE processing timeline in the Rel-18 study item:

Agreement: - The following options for relaxed UE processing timeline will be studied: - Option PT1: Relaxation of UE processing time for PDSCH/PUSCH in terms of N1 and N2 - Option PT2: Relaxation of UE processing time for CSI in terms of Z and Z’ - UE complexity reduction estimates for relaxed UE processing timeline are only reported for combinations with UE bandwidth reduction or UE peak rate reduction. Agreement: - In Option PT1, the relaxation factor for N1 and N2 is 2. - In Option PT2, the relaxation factor for Z and Z’ is 2. - The combination of Options PT1 and PT2 is also studied.

The agreements are in line with the description captured in TR 38.875 with an additional aspect where the combination of relaxed N1/N2 and Z/Z’ is also studied. In addition to the agreements, some contributions provide other possible alternatives. One contribution [31] consider relaxation factor for N1 and N2 other than 2, e.g., half slot or more than 1 slot. One contribution [24] mentioned simplifications (reductions) to the number of CSI processing units (CPUs) and the number of ports in a CSI-RS resource as possible alternatives for relaxed CSI computation.

Based on the above, the following text proposal for the TR can be considered:

In the study, relaxed UE processing timeline is considered for FR1. The main options for the study are as follows: - Option PT1: Relaxation of UE processing time for PDSCH/PUSCH in terms of N1 and N2 (as defined in TS 38.214) compared to those of UE processing time capability 1 - The relaxation factor for N1 and N2 is assumed to be 2 in the study. - Option PT2: Relaxation of UE processing time for CSI in terms of Z and Z’ compared to the values defined in TS 38.214 clause 5.4 - The relaxation factor for Z and Z’ is assumed to be 2 in the study. For the above relaxed UE processing timeline options, the following aspects are considered: - The combination of Options PT1 and PT2 is also studied. - UE complexity reduction estimates for relaxed UE processing timeline are only reported for combinations with UE bandwidth reduction or UE peak rate reduction.

FL1 High Priority Question 7.4.1-1a: Can the above description on the relaxed UE processing timeline options be used as a baseline text for TR 38.865?

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| Nordic | Y |  |
| Lenovo | Y |  |
| CATT | Y |  |
| SONY | Y |  |
| FUTUREWEI | Y |  |
| Xiaomi | Y |  |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Samsung | Y |  |
| Qualcomm | Y |  |
| CMCC | Y |  |
| MediaTek | Y |  |
| Huawei, Hisilicon | Y |  |
| FL3 FL4 | Based on received responses, the following proposal can be considered as a baseline for the TP drafting for the TR. High Priority Proposal 7.4.1-1b: Use the following TP as baseline text for TR 38.865. In the study, relaxed UE processing timeline is considered for FR1. The main options for the study are as follows: - Option PT1: Relaxation of UE processing time for PDSCH/PUSCH in terms of N1 and N2 (as defined in TS 38.214) compared to those of UE processing time capability 1 - The relaxation factor for N1 and N2 is assumed to be 2 in the study. - Option PT2: Relaxation of UE processing time for CSI in terms of Z and Z’ compared to the values defined in TS 38.214 clause 5.4 - The relaxation factor for Z and Z’ is assumed to be 2 in the study. For the above relaxed UE processing timeline options, the following aspects are considered: - The combination of Options PT1 and PT2 is also studied. - UE complexity reduction estimates for relaxed UE processing timeline are only reported for combinations with UE bandwidth reduction or UE peak rate reduction.. |
| FL5 | RAN1 made the following agreement on Tuesday 23rd August: Agreement: Use the following TP as baseline text for TR 38.865. In the study, relaxed UE processing timeline is considered for FR1. The main options for the study are as follows: - Option PT1: Relaxation of UE processing time for PDSCH/PUSCH in terms of N1 and N2 (as defined in TS 38.214) compared to those of UE processing time capability 1 - The relaxation factor for N1 and N2 is assumed to be 2 in the study. - Option PT2: Relaxation of UE processing time for CSI in terms of Z and Z’ compared to the values defined in TS 38.214 clause 5.4 - The relaxation factor for Z and Z’ is assumed to be 2 in the study. For the above relaxed UE processing timeline options, the following aspects are considered: - The combination of Options PT1 and PT2 is also studied. - UE complexity reduction estimates for relaxed UE processing timeline are only reported for combinations with UE bandwidth reduction or UE peak rate reduction.. |

### 7.4.2	Analysis of UE complexity reduction

Relaxed UE processing time in terms of N1 and N2:

Many contributions [11, 12, 13, 14, 15, 16, 20, 23, 25, 28, 31, 33] described potential UE complexity reduction from relaxed UE processing time similarly to those captured in TR 38.875. That is, relaxed UE processing time in terms of N1 and N2 potentially reduces UE complexity by allowing a longer time for the processing of PDCCH and PDSCH and preparing PUSCH and PUCCH. These potentially allow for possible slower processor with reduced clock frequency, distribution of the computation load over time, and reduced the number of parallel processing and chip area. It is noted though that complexity reduction from relaxed UE processing timeline option PT1 is highly dependent on implementation.

From several contributions, it can be observed that the cost of the following functional blocks can be reduced:

- Baseband: Receiver processing block

- Baseband: LDPC decoding

- Baseband: DL control processing & decoder

- Baseband: UL processing block

One contribution [23] argued that potential complexity reduction of “DL control processing block” may be limited since relaxation of PDCCH processing would increase cost of post-FFT buffer.

Relaxed UE processing time in terms of Z and Z’:

Several contributions [11, 12, 13, 14, 15, 16, 20, 23, 25, 28, 33] provided some analysis on how relaxed Z and Z’ may help to reduce UE complexity and cost. For example, with relaxed Z and Z’, the UE has additional time to generate the report and thus can compute the CSI-based statistics at a slower rate. It is noted though that complexity reduction from relaxed UE processing timeline option PT2 is highly dependent on implementation.

From several contributions, it can be observed that the cost of the following functional blocks can be reduced:

- Baseband: DL control processing & decoder

- Baseband: UL processing block

- Baseband: MIMO specific processing blocks

A few contributions [11, 20] consider that receiver processing block is also affected by PT2.

According to the agreement, a combination of PT1 and PT2 is also studied. Most contributions provided analysis on the complexity reduction of the combination. A few contributions [11, 20] mentioned that complexity reduction from PT1 and PT2 are not linear with respect to the level of relaxation. They also mention that PT1 and PT2 share some common processing blocks, e.g., DL control processing & decoder and UL processing block, and that the complexity/cost reduction benefits from a combination should not be over-estimated. A few contributions [13, 24] mentioned that PT1 and PT2 should be considered together for further complexity reduction benefits.

It was also agreed that UE complexity reduction estimates for relaxed UE processing timeline are only reported for combinations with UE bandwidth reduction or UE peak rate reduction. The summary of the estimates can be founded in Section 7.5.2.

FL1 High Priority Question 7.4.2-1a: Can the following observations regarding UE complexity reduction from relaxed UE processing timeline option PT1 be captured in the TR?

- For the relaxed UE processing timeline option PT1, the cost reduction is mainly in the BB part, especially on receiver processing block, LDPC decoding, DL control processing & decoder, and UL processing block.

- Level of complexity reduction from PT1 is dependent on implementation.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| Nordic | Y,but | we should capture also that up to 2fold cost reduction has been reported for the corresponding blocks |
| CATT | Y |  |
| SONY | Y |  |
| FUTUREWEI | Y |  |
| Xiaomi | Y |  |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Samsung | N | We have concerns to capture “DL control processing & decoder”. Based on the cost breakdown, most(half) of companies didn’t consider PDCCH relaxation. It is better to have clarification and common understanding on whether it shall be considered or not. It may have different spec impact. |
| CMCC | Y |  |
| MediaTek | Y |  |
| Huawei, Hisilicon | Y |  |
| FL3 FL4 FL5 FL7 | Most of the received responses express a common view about Question 7.4.2-1a. However, one response indicates that further clarification is needed regarding “DL control processing & decoder” block. Based on the above views, the following question can be considered: High Priority Question 7.4.2-1b: Can the complexity of DL control processing & decoder block be reduced with PT1? |  |
| CATT | Y | But small. |
| Sharp | Y |  |
| Samsung |  | We suggest the following change - For the relaxed UE processing timeline option PT1, the cost reduction is mainly in the BB part, especially on receiver processing block, LDPC decoding, DL control processing & decoder, and UL processing block. Some companies think DL control processing & decoder block may have additional cost reduction. - Level of complexity reduction from PT1 is dependent on implementation. We think the cost is mainly from PDSCH decoding and PUSCH preparing. Moreover, we think relaxing on DL control may have negative impact for BW 2.3.PR3 on post FFT buffering. |
| FUTUREWEI | Y | We based our numbers from TR38.875 which listed “Baseband: DL control processing & decoder” having 25% complexity reduction. |
| Nordic | Yes, but | If processing is relaxed for PDCCH decoding, more post-FFT buffering is needed for PR3 and BW3. so that should be taken into account. |
| Xiaomi2 | Y, but | Share the same view as CATT that the cost of DL control processing & decoder block is small. Besides, relaxed processing timeline may have negative impact on the cost of post-FFT buffer, considering that more-slots data should be buffering post FFT due to the relaxed processing timeline when consecutive slots is scheduled. |
| Spreadtrum | Y | Our answer is Yes. We are fine with Samsung’s suggestion or maybe we can reuse the descriptions in TR38.875: TR 38.875 7.5.2 Analysis of UE complexity reduction: …it can be observed that the cost of the following functional blocks can be reduced: - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: DL control processing & decoder - Baseband: UL processing block Whether the relaxed UE processing time in terms of N1 and N2 may reduce the cost/complexity in the 'DL control processing & decoder' block depends on the UE implementation.. |
| Nokia, NSB | Y | In our analysis we did not consider complexity reduction for the PDCCH processing. But we feel that it is feasible to do so. |
| Ericsson | Y, marginally |  |
| OPPO | N | In first look the processing seems can be slow in one slot. However, processing amount of all slot is not reduced. We believe the capability 2 do it in burst manner, that is higher complexity comes out. |
| vivo | Y | We support Spreadtrum’s modifications. |
| Intel | Y | Same view as Nordic |
| Huawei, HiSilicon | Y |  |
| ZTE, Sanechips | Y |  |
| SONY | Y |  |
| CMCC | Y |  |
| FL8 | Based on offline discussion on Wednesday 24th August and received responses, the following proposal can be considered. High Priority Proposal 7.4.2-1c: Capture the following observations regarding UE complexity reduction from relaxed UE processing timeline option PT1 in TR 38.865. - For the relaxed UE processing timeline option PT1, the cost reduction is mainly in the BB part, especially on receiver processing block, LDPC decoding, DL control processing & decoder, and UL processing block. - Whether the relaxed UE processing time in terms of N1 and N2 may reduce the cost/complexity in the 'DL control processing & decoder' block depends on the UE implementation. |  |
| FL9 | The online session on Thursday 25th August made the following agreement: Agreement: Capture the following observations regarding UE complexity reduction from relaxed UE processing timeline option PT1 in TR 38.865, clause 7.4.2. - For the relaxed UE processing timeline option PT1, the cost reduction is mainly in the BB part, especially on receiver processing block, LDPC decoding, DL control processing & decoder, and UL processing block. - Whether the relaxed UE processing time in terms of N1 and N2 may reduce the cost/complexity in the 'DL control processing & decoder' block depends on the UE implementation. |  |

FL1 High Priority Question 7.4.2-2a: Can the following observations regarding UE complexity reduction from relaxed UE processing timeline option PT2 be captured in the TR?

- For the relaxed UE processing timeline option PT2, the cost reduction is mainly in the BB part, especially on DL control processing & decoder, UL processing block, and MIMO specific processing blocks.

- Level of complexity reduction from PT2 is dependent on implementation.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| Nordic | Y, but | we should capture also that up to 2fold cost reduction has been reported for the corresponding blocks |
| CATT | Y | Receiver processing block might also be impacted, but we agree that the difference will be quite marginal. So fine with FL’s observation. |
| SONY | Y |  |
| FUTUREWEI | Y | To FL: Our contribution did not mention “LDPC decoding block affected by PT2”. Can the statement be corrected to: “One contribution [12] consider LDPC decoding block affected by PT2 and expect some discussions to converge on complexity estimates for PT2.” |
| Xiaomi | Y |  |
| ZTE, Sanechips | Y |  |
| Intel | Y |  |
| Samsung | N | We have concerns to capture “DL control processing & decoder”. Based on the cost breakdown, most(half) of companies didn’t consider PDCCH relaxation. It is better to have clarification and common understanding on whether it shall be considered or not. It may have different spec impact. |
| CMCC | Y |  |
| MediaTek | Y |  |
| Huawei, Hisilicon | Y |  |
| FL3 FL4 FL5 FL7 | Most of the received responses express a common view about Question 7.4.2-2a. However, one response indicates that further clarification is needed regarding “DL control processing & decoder” block. Based on the above views, the following question can be considered: High Priority Question 7.4.2-2b: Can the complexity of DL control processing & decoder block be reduced with PT2? |  |
| CATT | Y | But small. |
| Sharp | Y |  |
| Samsung |  | We suggest the following change - For the relaxed UE processing timeline option PT2, the cost reduction is mainly in the BB part, especially on DL control processing & decoder, UL processing block, and MIMO specific processing blocks. Some companies think DL control processing & decoder block may have additional cost reduction. - Level of complexity reduction from PT2 is dependent on implementation. |
| FUTUREWEI |  | It might be small |
| Nordic | No | If PT1 is omitted, we do not see any room for complexity reduction in this block |
| Spreadtrum |  | Similar comments as question 7.4.2-1b. |
| Ericsson | Y, marginally |  |
| OPPO | Yes | We see some relaxation, due to the overall relation of PT2. |
| vivo |  | Not sure PT2 can help reduce DL control processing & decoder block cost. But we are fine with the modification to say Whether the relaxed UE processing time in terms of Z and Z’ may reduce the cost/complexity in the 'DL control processing & decoder' block depends on the UE implementation. |
| Intel | Y | But small. |
| Huawei, HiSilicon | Y |  |
| ZTE, Sanechips | Y | DL control processing can be relaxed due to timeline relaxing, which brings some kind of complexity reduction. |
| SONY | Y | The wording from vivo or Samsung seems good. |
| CMCC | Y |  |
| FL8 | Based on offline discussion on Wednesday 24th August and received responses, the following proposal can be considered. High Priority Proposal 7.4.2-2c: Capture the following observations regarding UE complexity reduction from relaxed UE processing timeline option PT2 in TR 38.865. - For the relaxed UE processing timeline option PT2, the cost reduction is mainly in the BB part, especially on DL control processing & decoder, UL processing block, and MIMO specific processing blocks. - Whether the relaxed UE processing time in terms of Z and Z’ may reduce the cost/complexity in the 'DL control processing & decoder' block depends on the UE implementation. |  |
| FL9 | The online session on Thursday 25th August made the following agreement: Agreement: Capture the following observations regarding UE complexity reduction from relaxed UE processing timeline option PT2 in TR 38.865, clause 7.4.2. - For the relaxed UE processing timeline option PT2, the cost reduction is mainly in the BB part, especially on DL control processing & decoder when PT1 is supported, UL processing block, and MIMO specific processing blocks. - Whether the relaxed UE processing time in terms of Z and Z’ may reduce the cost/complexity in the 'DL control processing & decoder' block depends on the UE implementation. |  |

### 7.4.3	Analysis of performance impacts

Several contributions [11, 13, 14, 15, 16, 20, 22, 24, 29, 33, 35] provided analysis of performance impact of PT1 and most of them are similar to those captured in TR 38.875 which are summarized below:

- No impacts on coverage

- No impact on instantaneous data rate but throughput might be reduced due to longer HARQ round trip time

- Increase on transmission latency

Several contributions [11, 13, 14, 15, 16, 20, 22, 24, 29, 33, 35] provided analysis of performance impact of PT2 which is similar to that of PT1. The findings are summarized below:

- No impacts on coverage

- No impact on instantaneous data rate but throughput might be reduced due to delayed PUSCH scheduling

- Increase on transmission latency

A few companies [14, 15] mention that power consumption can be reduced by PT1 and PT2 but [25] argue that the power consumption also depends on other factors which is not easy to give a quantitative evaluation.

Based on the above, the following text proposal for the TR can be considered:

Coverage: No coverage impact is expected from PT1 and PT2. Data rate: No impact on instantaneous peak data rate is expected from PT1 and PT2. However, the UE throughput may be reduced if the HARQ round trip time is extended due to PT1 and delayed PUSCH scheduling due to PT2. The throughput requirements identified for the RedCap use cases are still expected to be fulfilled. Latency: Both PT1 and PT2 have impact on latency. For downlink transmission, relaxed N1 value in PT1 impacts how fast HARQ-ACK feedback can be sent after the reception of PDSCH. For uplink transmission, relaxed N2 value in PT1 impacts how fast PUSCH can be scheduled with respect to the UL grant and relaxed Z/Z’ in PT2 impacts the scheduling of the following data since no other PUSCH can be scheduled before the A-CSI is transmitted. How significant the impact on latency depends on use cases and scheduled number of retransmissions. Power consumption: PT1 and PT2 may allow for processing with lower clock frequency and lower voltage which may help reducing the UE power consumption. The impact on power consumption of relaxed UE processing time depends on implementation and traffic characteristics.

FL1/FL3/FL4/FL5/FL7 High Priority Question 7.4.3-1a: Can the above TP on performance impacts from relaxed UE processing timeline options be used as a baseline text for TR 38.865?

| Company |  | Y/N | Comments |
| --- | --- | --- | --- |
| Spreadtrum |  | Y |  |
| Xiaomi |  |  | Data rate: It should be discussed whether throughput really needs to be identified for eRedCap use cases. Power consumption: Since the baseband works longer than the UE processing capability 1, so it can’t directly conduct a positive impact on power consumption for relaxed processing time. |
| Intel |  | Y |  |
| CMCC |  | Y |  |
| FL |  | If there are impacts among the listed potential impacts that should NOT be listed among the impacts in the TR section, please mention them in the Comments field and elaborate. |  |
| CATT |  | Y in general | To address Xiaomi’s concern on power consumption, will it help by adding ‘On the other hand, PT1 and PT2 may lead to longer baseband working time and subsequently increased power consumption’ in the middle of the paragraph? |
| Sharp |  | Y |  |
| Samsung |  |  | Power consumption shall not be double counted. This is unclear. Because on the other hand, UE can finish data transmission/reception within a short time. |
| FUTUREWEI |  | Y in general | We suggest an edit because voltage is an implementation. “PT1 and PT2 may allow for processing with lower clock frequency and lower voltage which may help reducing the UE power consumption. The impact on power consumption of relaxed UE processing time depends on implementation and traffic characteristics.” |
| Nordic |  |  | We are OK with all the statements. |
| Xiaomi2 |  | N | Can’t see the impact on DL control processing & decoder block with CSI computation time relaxation and it may be clearer for us if further clarification is made. |
| Nokia, NSB |  | Y |  |
| Ericsson |  | Y, but | We would prefer to remove the paragraph about power consumption, since it has not been thoroughly studied. |
| OPPO |  | Y |  |
| vivo |  | Y |  |
| Intel |  | Y |  |
| Huawei, HiSilicon |  | N | For Data rate As specified in TS 38.306, calculation of peak data rate has no dependency of RTT. So we suggest: No impact on instantaneous peak data rate is expected from PT1 and PT2. However, the UE throughput may be reduced if the HARQ round trip time is extended due to PT1 and delayed PUSCH scheduling due to PT2. The throughput requirements identified for the RedCap use cases are still expected to be fulfilled. For Latency It is not true that “no other PUSCH can be scheduled before the A-CSI is transmitted”. We suggest the following clarification: “relaxed Z/Z’ in PT2 impacts the scheduling of a PUSCH traffic that arrives after the DCI triggering A-CSI is sent the following data since such PUSCH TB cannot be scheduled to be transmitted before the A-CSI is transmitted.” |
| Qualcomm |  | Y | Share the view with Ericsson. |
| ZTE, Sanechips |  | Y |  |
| SONY |  |  | Data rate: we are not convinced that there is an impact on throughput. Latency: there is only an impact on latency when there are retransmissions. However, RAN1 has traditionally defined latency in terms of RTT assuming retransmissions. We think that performance impacts from PT1 and PT2 are not significant. Our overall comment is that we should write something that is consistent with 38.875. |
| CMCC |  | Y |  |
| FL9 FL10 |  | Based on received responses, the following proposal can be considered. High Priority Proposal 7.4.3-1b: Adopt the following TP on performance impacts from relaxed UE processing timeline options as a baseline text for TR 38.865 clause 7.4.3. Coverage: No coverage impact is expected from PT1 and PT2. Data rate: No impact on instantaneous peak data rate is expected from PT1 and PT2. However, the UE throughput may be reduced if the HARQ round trip time is extended due to PT1 and delayed PUSCH scheduling due to PT2. The throughput requirements identified for the RedCap use cases are still expected to be fulfilled. Latency: Both PT1 and PT2 have impact on latency. For downlink transmission, relaxed N1 value in PT1 impacts how fast HARQ-ACK feedback can be sent after the reception of PDSCH. For uplink transmission, relaxed N2 value in PT1 impacts how fast PUSCH can be scheduled with respect to the UL grant and relaxed Z/Z’ in PT2 impacts the scheduling of a PUSCH traffic that arrives after the DCI triggering A-CSI is sent the following data since such PUSCH TB cannot be scheduled to be transmitted before the A-CSI is transmitted. How significant the impact on latency depends on use cases and scheduled number of retransmissions. Power consumption: PT1 and PT2 may allow for processing with lower clock frequency and lower voltage which may help reducing the UE power consumption. The impact on power consumption of relaxed UE processing time depends on implementation and traffic characteristics.. |
| Nordic |  | OK |  |
| DOCOMO |  | Y |  |
| Nokia, NSB |  | Y |  |
| vivo |  | N | We do not support remove the text for power consumption. It has already been captured in TR38.875 section 7.5.3 . The text just provides qualitative analysis, and that is why ‘may’ is used in the description. |
| CATT |  | Y | Fine if companies would like to have a short version for performance of PT1 and PT2, although many of the deleted sentences are similar to that in TR 38.875. |
| OPPO |  | Y |  |
| Qualcomm |  | Y |  |
| ZTE, Sanechips |  | Y | We think the following red part can be added back, since it does has the impact on data rate(throughput). Data rate: No impact on instantaneous peak data rate is expected from PT1 and PT2. However, the UE throughput may be reduced if the HARQ round trip time is extended due to PT1 and delayed PUSCH scheduling due to PT2. |
| Spreadtrum |  |  | We also prefer to keep the power consumption part |
| Intel |  |  | Agree with ZTE. It is for UE perceived throughput. |
| CMCC |  | Y |  |
| FL11 | The online session on Friday 26th August made the following agreement: Agreement: Adopt the following TP on performance impacts from relaxed UE processing timeline options as a baseline text for TR 38.865 clause 7.4.3. Coverage: No coverage impact is expected from PT1 and PT2. Data rate: No impact on instantaneous peak data rate is expected from PT1 and PT2. However, the UE throughput may be reduced if the HARQ round trip time is extended due to PT1 and delayed PUSCH scheduling due to PT2. The throughput requirements identified for the RedCap use cases are still expected to be fulfilled. Latency: Both PT1 and PT2 have impact on latency. For downlink transmission, relaxed N1 value in PT1 impacts how fast HARQ-ACK feedback can be sent after the reception of PDSCH. For uplink transmission, relaxed N2 value in PT1 impacts how fast PUSCH can be scheduled with respect to the UL grant and relaxed Z/Z’ in PT2 impacts the scheduling of a PUSCH traffic that arrives after the DCI triggering A-CSI is sent the following data since such PUSCH TB cannot be scheduled to be transmitted before the A-CSI is transmitted. How significant the impact on latency depends on use cases and scheduled number of retransmissions. Power consumption: PT1 and PT2 may allow for processing with lower clock frequency and lower voltage which may help reducing the UE power consumption. The impact on power consumption of relaxed UE processing time depends on implementation and traffic characteristics.. Note: Note for power consumption and throughput is to be added by editor. |

### 7.4.4	Analysis of network deployment and coexistence impacts

Relaxed UE processing time in terms of N1 and N2:

Several contributions [11, 13, 14, 15, 16, 20, 21, 22, 24, 28, 29, 33, 35] provided some analysis of network deployment and coexistence impacts of PT1. Most of the analysis are similar to those captured in TR 38.875 and are summarized below:

- In scenarios where Rel-18 RedCap UEs coexist with legacy UEs, relaxed UE processing time capability in terms of N1 and N2 may increase the complexity for the scheduling as there will be up to three different UE processing time capabilities for the scheduler to handle which may have impacts on current scheduling mechanisms. This may also hinder the software update of gNB to support Rel-18 eRedCap UE.

- Relaxed UE processing time in terms of N1/N2 can impact scheduling flexibility, e.g., same-slot scheduling may not be possible. The impact is not only limited to operation in connected mode since N1/N2 are also related to other timing requirements such as the time from Msg2 to Msg3 or PUCCH timing of Msg4 in random access procedure.

- If relaxed UE processing time in terms of N1 and N2 is applicable during the initial/random access, it may cause potential coexistence issues with legacy UEs if early identification of Rel-18 RedCap UEs prior to Msg2 scheduling (e.g., in Msg1) is not supported or conservative scheduling is not possible. If gNB schedules all UEs according to relaxed timing relationships for Rel-18 RedCap UEs, legacy UEs may experience an increase in control plane latency.

One contribution [15] mentioned a possibility that Rel-18 eRedCap UE may operate using capability #1 during initial access and UE can be configured to operate with relaxed processing timeline only in connected mode to avoid coexistence issues with legacy UEs.

Relaxed UE processing time in terms of Z and Z’:

In addition, some contributions [11, 13, 14, 16] provided analysis of network deployment and coexistence impacts of PT2. A summary is provided below:

- Due to out-of-order scheduling restriction, if Z and Z’ are relaxed, any PUSCH scheduled after A-CSI triggering may need to be delayed resulting in less scheduling flexibility and potentially more complexed scheduler.

- Relaxed UE CSI computation time Z/Z’ may impact the scheduler’s ability to track the channel when making scheduling decisions, especially in a fast-varying channel condition.

- Relaxed Z and Z’ do not cause any coexistence impacts.

Based on the above summary, the following text proposal for the TR can be considered:

Relaxed UE processing time in terms of N1 and N2: - In scenarios where Rel-18 RedCap UEs coexist with legacy UEs, PT1 may increase the complexity for the scheduling. - PT1 may have an impact on scheduling flexibility as several timing requirements are related to N1/N2 values. - If PT1 is applicable during the initial/random access, it may cause potential coexistence issues with legacy UEs if early identification of Rel-18 RedCap UEs prior to Msg2 scheduling is not supported, or conservative scheduling is not possible. If gNB schedules all UEs according to relaxed timing relationships for Rel-18 RedCap UEs, legacy UEs may experience an increase in control plane latency. Relaxed UE processing time in terms of Z and Z’: - PT2 may have impacts on scheduling flexibility and potentially make scheduler more complexed. - PT2 may impact the scheduler’s ability to track the channel when making scheduling decisions, especially in a fast-varying channel condition. - No coexistence impact is expected from PT2.

FL1/FL3/FL4/FL5/FL7 High Priority Question 7.4.4-1a: Can the above TP on network deployment and coexistence impacts from relaxed UE processing timeline options be used as a baseline text for TR 38.865?

| Company |  | Y/N | Comments |
| --- | --- | --- | --- |
| Xiaomi |  | Y |  |
| CMCC |  | Y |  |
| FL |  | If there are impacts among the listed potential impacts that should NOT be listed among the impacts in the TR section, please mention them in the Comments field and elaborate. |  |
| CATT |  | Y in general | In fact, due to PT1, sharing of common channels in RRC_IDLE or RRC_INACTIVE mode (all before RACH) may be problematic. For example, the network may not be able to page a PT1 UE and a legacy UE by the same DCI/PDSCH. We are also not confident that it is always OK to share SIB1 in all configurations of CORESET#0&Type0CSS. |
| Sharp |  | Y |  |
| Samsung |  |  | As we commented before, if PDCCH block is not changed, it may not have serious issue for initial access/random access. |
| Nordic |  | Y | At the same time, early identification may be needed anyway for all options except PR1/2. So depending on the choice selected, early indication would be there for other reasons. |
| Nokia, NSB |  | Y |  |
| Ericsson |  | Y |  |
| OPPO |  | Y |  |
| vivo |  | Y in general | We share the similar views with CATT. |
| Intel |  | Y | But small. |
| DOCOMO |  | Y |  |
| Qualcomm |  | Y |  |
| ZTE, Sanechips |  | Y |  |
| SONY |  | Y | We should correct the typo “complexed” -> “complex” in the text below: Relaxed UE processing time in terms of Z and Z’: - PT2 may have impacts on scheduling flexibility and potentially make scheduler more complexed. - PT2 may impact the scheduler’s ability to track the channel when making scheduling decisions, especially in a fast-varying channel condition. - No coexistence impact is expected from PT2. |
| FL9 FL10 |  | Based on received responses, the following proposal can be considered. High Priority Proposal 7.4.4-1b: Adopt the following TP on network deployment and coexistence impacts from relaxed UE processing timeline options as a baseline text for TR 38.865 clause 7.4.4. Relaxed UE processing time in terms of N1 and N2: - In scenarios where Rel-18 RedCap UEs coexist with legacy UEs, PT1 may increase the complexity for the scheduling. - PT1 may have an impact on scheduling flexibility as several timing requirements are related to N1/N2 values. - If PT1 is applicable during the initial/random access, it may cause potential coexistence issues with legacy UEs if early identification of Rel-18 RedCap UEs prior to Msg2 scheduling is not supported, or conservative scheduling is not possible. If gNB schedules all UEs according to relaxed timing relationships for Rel-18 RedCap UEs, legacy UEs may experience an increase in control plane latency. Relaxed UE processing time in terms of Z and Z’: - PT2 may have impacts on scheduling flexibility and potentially make scheduler more complexed. - PT2 may impact the scheduler’s ability to track the channel when making scheduling decisions, especially in a fast-varying channel condition. - No coexistence impact is expected from PT2.. |
| Nordic |  | Y |  |
| DOCOMO |  | Y |  |
| Nokia, NSB |  | Y |  |
| vivo |  | Y |  |
| CATT |  |  | With PT1, we still doubt that Rel-18 eRedCap UE has some problems in: - SIB1 reception, as same-slot scheduling is often used in FR1. Should a UE with PT1 still buffer same-slot data? Or say, a PT1 UE can support all current SIB1 scheduling (including same-slot scheduling) in FR1? - Paging PDSCH reception. Can a PT1 UE and a Rel-17 RedCap UE (or a Rel-15 UE) be paged together within a paging DCI and PDSCH, using a same default TDRA table and same-slot scheduling? If the answers are NO, we think this should be captured in the network deployment and co-existence impact. |
| OPPO |  | Y |  |
| Qualcomm |  | Y |  |
| ZTE, Sanechips |  | Y | Maybe the following can capture CATT’s concern - PT1 may have an impact on scheduling flexibility as several timing requirements are related to N1/N2 values, e.g., SIB1 scheduling, paging PDSCH scheduling. |
| Spreadtrum |  | Y |  |
| Intel |  | Y |  |
| FUTUREWEI |  | Y |  |
| CMCC |  | Y |  |
| FL11 | The online session on Friday 26th August made the following agreement: Agreement: Adopt the following TP on network deployment and coexistence impacts from relaxed UE processing timeline options as a baseline text for TR 38.865 clause 7.4.4. Relaxed UE processing time in terms of N1 and N2: - In scenarios where Rel-18 RedCap UEs coexist with legacy UEs, PT1 may increase the complexity for the scheduling. - PT1 may have an impact on scheduling flexibility as several timing requirements are related to N1/N2 values. - If PT1 is applicable during the initial/random access, it may cause potential coexistence issues with legacy UEs if early identification of Rel-18 RedCap UEs prior to Msg2 scheduling is not supported, or conservative scheduling is not possible. If gNB schedules all UEs according to relaxed timing relationships for Rel-18 RedCap UEs, legacy UEs may experience an increase in control plane latency. Relaxed UE processing time in terms of Z and Z’: - PT2 may have impacts on scheduling flexibility and potentially make the scheduler more complex. - PT2 may impact the scheduler’s ability to track the channel when making scheduling decisions, especially in a fast-varying channel condition. - No coexistence impact is expected from PT2.. |

### 7.4.5	Analysis of specification impacts

Several contributions [11, 13, 14, 15, 16, 20, 21, 22, 24, 28, 29, 33, 35] provided potential specification impacts of PT1 and PT2, which are summarized below:

- A new UE processing time capability needs to be defined if relaxed UE processing time in terms of N1 and N2 is introduced.

- Depending on the degree of relaxation of the N1 and N2 values, specification details on scheduling timing related to the default TDRA table for PUSCH, and HARQ-ACK timing range may also need to be updated.

- One or more new CSI computation delay requirements need to be defined if relaxed UE processing time in terms of Z and Z’ is introduced

One company [15] mention that other timelines requirements related to N1 and N2 like UCI multiplexing timeline, dynamic SFI cancelling the UL transmission etc. should be studied as well.

Based on the above, the following text proposal for the TR can be considered:

A new UE processing time capability needs to be defined if relaxed UE processing time in terms of N1 and N2 is introduced. New values of N1 and N2, as well as how the PDSCH processing time and PUSCH preparation time are determined by N1 and N2, need to be defined. Depending on the degree of relaxation of the N1 and N2 values, specification details on scheduling timing related to the default TDRA tables and HARQ-ACK timing range may also need to be updated. New CSI computation delay requirements need to be defined if relaxed UE processing time in terms of Z and Z’ is introduced. New values of Z and Z’, as well as how the CSI computation time is determined by Z and Z’, need to be defined.

FL1/FL3/FL4/FL5/FL7 High Priority Question 7.4.5-1a: Can the above TP on specification impacts from relaxed UE processing timeline options be used as a baseline text for TR 38.865?

| Company |  | Y/N | Comments |
| --- | --- | --- | --- |
| Spreadtrum |  | Y |  |
| Xiaomi |  |  | All the scheduling timing requirements in TS 38.213 and/or TS 38.214 should be revisited, such as, the timeline requirements between PDCCH ordered PRACH and the scheduling PDCCH, the definition of TA effective time, and etc. Besides, UE capability signaling design and enabling/disabling mechanism for UE processing capability should also be reconsidered. |
| CMCC |  | Y |  |
| FL |  | If there are impacts among the listed potential impacts that should NOT be listed among the impacts in the TR section, please mention them in the Comments field and elaborate. |  |
| CATT |  |  | As explained in 7.4.4-1a, PT1 will also introduce need for early indication in Msg1. PT1 will also bring need of separate/new common channels (e.g. paging, SIB1) that cannot be shared with legacy UEs. The part of PT2 seems OK. |
| Sharp |  | Y |  |
| Nordic |  | Y |  |
| Xiaomi2 |  |  | Agree with CATT that early indication may be needed. Besides, suggest to modify FL‘s TP as following: Depending on the degree of relaxation of the N1 and N2 values, specification details on scheduling timing should be updated, such as related to the default TDRA tables, and HARQ-ACK timing range, the time gap between RAR and Msg1 retransmission, TA effective time, the time gap beteween PDCCH ordering PRACH and scheduled PDCCH, may also need to be updated and etc. Furthermore, adding the following sentence in the section of spec impact analysis: UE capability signaling and enabling/disabling mechanism design should be re-considered with introducing a new UE processing capability. |
| Nokia, NSB |  | Y |  |
| Ericsson |  | Y |  |
| vivo |  |  | Share CATT’s views. |
| Intel |  | Y | But small. |
| DOCOMO |  | Y |  |
| Qualcomm |  | Y |  |
| ZTE, Sanechips |  |  | Agree with CATT. Suggest to add the following for PT1 A new UE processing time capability needs to be defined if relaxed UE processing time in terms of N1 and N2 is introduced. New values of N1 and N2, as well as how the PDSCH processing time and PUSCH preparation time are determined by N1 and N2, need to be defined. Depending on the degree of relaxation of the N1 and N2 values, specification details on scheduling timing related to the default TDRA tables and HARQ-ACK timing range may also need to be updated. Moreover, PT1 may introduce need for early indication in Msg1. And PT1 may also bring need of separate/new common channels (e.g. paging, SIB1) that cannot be shared with legacy UEs. New CSI computation delay requirements need to be defined if relaxed UE processing time in terms of Z and Z’ is introduced. New values of Z and Z’, as well as how the CSI computation time is determined by Z and Z’, need to be defined. |
| SONY |  | Y |  |
| CMCC2 |  |  | Agree with CATT |
| FL9 FL10 |  | Based on received responses, the following proposal can be considered. High Priority Proposal 7.4.5-1b: Adopt the following TP on specification impacts from relaxed UE processing timeline options as a baseline text for TR 38.865 clause 7.4.5. A new UE processing time capability needs to be defined if relaxed UE processing time in terms of N1 and N2 is introduced. New values of N1 and N2, as well as how the PDSCH processing time and PUSCH preparation time are determined by N1 and N2, need to be defined. Depending on the degree of relaxation of the N1 and N2 values, specification details on scheduling timing should be updated, such as related to the default TDRA tables, and HARQ-ACK timing range, the time gap between RAR and Msg1 retransmission, TA effective time, the time gap between PDCCH ordering PRACH and scheduled PDCCH, etc. Moreover, PT1 may introduce need for early indication in Msg1. And PT1 may also bring need of separate/new common channels (e.g., paging, SIB1) that cannot be shared with legacy UEs. UE capability signaling and enabling/disabling mechanism design should be re-considered with introducing a new UE processing capability. New CSI computation delay requirements need to be defined if relaxed UE processing time in terms of Z and Z’ is introduced. New values of Z and Z’, as well as how the CSI computation time is determined by Z and Z’, need to be defined.. |
| Nordic |  | Not OK | Since gaps are typically specified as function of N1 and N2, then specification is insignificant. E.g. we do not agree that PDCCH to PRACH timing need any redesign. UE capability does not need to be redesigned. Also disagree with “And PT1 may also bring need of separate/new common channels (e.g., paging, SIB1) that cannot be shared with legacy UEs.” Since SI does not have HARQ-ACK, so not clear why new common channels are needed. |
| Nokia, NSB |  |  | Not clear whether a separate/new common channels would be needed with PT1. Agree with the text on impact to scheduling timing and other timing delay if the UE processing times are relaxed too much. |
| vivo |  | N | We do not understand what the impact caused by relaxed processing timeline for “TA effective time”. In addition, we do not agree following And PT1 may also bring need of separate/new common channels (e.g., paging, SIB1) that cannot be shared with legacy UEs. |
| CATT |  | Y | If the answer to the following two questions are YES, we are fine to delete the impact of paging and SIB1: - SIB1 reception, as same-slot scheduling is often used in FR1. Should a UE with PT1 still buffer same-slot data? Or say, a PT1 UE can support all current SIB1 scheduling (including same-slot scheduling) in FR1? - Paging PDSCH reception. Can a PT1 UE and a Rel-17 RedCap UE (or a Rel-15 UE) be paged together within a paging DCI and PDSCH, using a same default TDRA table and same-slot scheduling? If the answer is NO, they should be reflected. |
| OPPO |  | Y | We think the issues is those will have impact wider than RedCap. The Z values are highly related to MIMO topics and the N values are related to both control and data decoding. |
| ZTE, Sanechips |  | Y | We think It would be true that the separate new SIB 1 may be needed if the SIB1 can not be shared due to timeline relaxing. Additionally, we may add the ‘potential’ for the following the potential time gap between RAR and Msg1 retransmission, potential TA effective time, the potential time gap between PDCCH ordering PRACH and scheduled PDCCH |
| Intel |  | Y | We think the following sentence is needed. If adopting PT1/PT2 means low processing with increased post-FFT data buffer, the complexity reduction gain may not be justified. On the other hand, assuming post-FFT data buffering is not impacted, UE may not have the capability to buffer SIB1/paging for long decoding which means same-slot scheduling for SIB1/paging is questionable. And PT1 may also bring need of separate/new common channels (e.g., paging, SIB1) that cannot be shared with legacy UEs if buffer. |
| FUTUREWEI |  |  | Revise “specification details on scheduling timing should may be updated,” It seems most of the red text has objections. Perhaps better to remove |
| FL11 | The online session on Friday 26th August made the following agreement: Agreement: Adopt the following TP on specification impacts from relaxed UE processing timeline options as a baseline text for TR 38.865 clause 7.4.5. A new UE processing time capability needs to be defined if relaxed UE processing time in terms of N1 and N2 is introduced. New values of N1 and N2, as well as how the PDSCH processing time and PUSCH preparation time are determined by N1 and N2, need to be defined. Depending on the degree of relaxation of the N1 and N2 values, specification details on scheduling timing may be updated, such as related to the default TDRA tables, and HARQ-ACK timing range, the time gap between RAR and Msg1 retransmission, TA effective time, the time gap between PDCCH ordering PRACH and scheduled PDCCH, etc. Moreover, PT1 may introduce need for early indication in Msg1. And PT1 does not need to define new default TDRA table for downlink. New CSI computation delay requirements need to be defined if relaxed UE processing time in terms of Z and Z’ is introduced. New values of Z and Z’, as well as how the CSI computation time is determined by Z and Z’, need to be defined.. |

## 7.5	Combinations of UE complexity reduction features

### 7.5.1	Description of feature combinations

RAN1#109e made the following agreements regarding the study of combinations of UE complexity reduction features:

Agreement: - UE complexity reduction is studied for the following combinations: - Reference case (Rel-17 RedCap UE) - BW1 + PT1 + PT2 - BW3 + PT1 + PT2 - PR1 + PT1 + PT2 - PR3 + PT1 + PT2 - In addition, optional results for the following combinations can also be reported: - BW1 + PT1 - BW3 + PT1 - PR1 + PT1 - PR3 + PT1 - BW2 + PT1 + PT2 - PR2 + PT1 + PT2

### 7.5.2	Analysis of UE complexity reduction

The tables below show the average value of the UE complexity reduction evaluation results for different combinations of UE complexity reduction features captured in [43].

Table 4: Average cost reduction achieved by combinations of UE complexity reduction features compared to corresponding Rel-17 baselines

| Option | FD-FDD 1Rx | TDD 1Rx | HD-FDD 1Rx | FD-FDD 2Rx | TDD 2Rx | HD-FDD 2Rx |
| --- | --- | --- | --- | --- | --- | --- |
| BW1 | 11.85% | 11.25% | 14.06% | 14.31% | 13.42% | 14.79% |
| BW1 + PT1 | 12.44% | 11.64% | 14.30% | 17.65% | 14.58% | 16.38% |
| BW1 + PT1 + PT2 | 14.75% | 14.73% | 17.51% | 19.10% | 15.80% | 17.89% |
| BW2 | 9.15% | 8.08% | 11.92% | 11.46% | 8.81% | 12.21% |
| BW2 + PT1 + PT2 | 11.54% | 10.91% | 15.27% | 16.70% | 10.99% | 15.18% |
| BW3 | 8.02% | 7.66% | 8.90% | 8.72% | 7.68% | 9.19% |
| BW3+PT1 | 8.70% | 7.84% | 10.15% | 12.48% | 8.98% | 10.77% |
| BW3 + PT1 + PT2 | 11.55% | 11.50% | 12.92% | 14.59% | 10.82% | 12.76% |
| PR1 | 4.13% | 4.02% | 4.99% | 5.36% | 3.73% | 4.74% |
| PR1 + PT1 | 5.40% | 4.85% | 6.58% | 8.80% | 5.49% | 6.54% |
| PR1 + PT1 + PT2 | 7.88% | 8.67% | 9.33% | 10.99% | 6.76% | 8.63% |
| PR2 | 4.26% | 4.16% | 5.14% | 6.91% | 3.82% | 4.82% |
| PR2 + PT1 + PT2 | 7.17% | 6.23% | 9.48% | 10.89% | 6.81% | 8.70% |
| PR3 | 7.06% | 6.74% | 8.12% | 9.81% | 6.59% | 7.98% |
| PR3 + PT1 | 7.69% | 7.23% | 9.32% | 11.49% | 8.11% | 9.67% |
| PR3 + PT1 + PT2 | 10.22% | 10.70% | 12.07% | 13.55% | 9.88% | 11.60% |

FL1 High Priority Question 7.5.2-1a: Can the above table format with cost reduction achieved by combinations of UE complexity reduction features be used as a baseline text for TR 38.865? (Regarding the averaging method, please see Question 6.1-2a.)

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum | Y |  |
| Nokia, NSB | Y |  |
| Nordic | Y |  |
| Lenovo | Y |  |
| CATT | Y |  |
| SONY | Y |  |
| Xiaomi | Y, but | The optional evaluation cases should be reflected. |
| ZTE, Sanechips | Y | Other cases, e.g., PT2, BW3/PR3+PT2, for the complexity reduction are not precluded based on the agreement. They also need to be captured in the TR. |
| Intel | Y |  |
| Samsung | FFS | Until clarification on the cost breakdown mentioned in above questions. |
| Qualcomm | Y |  |
| CMCC | Y |  |
| MediaTek | Y | Due to the dependency with averaging method, if this proposal is to be agreed prior to the decision on averaging method, we suggest to include square brackets on the values. |
| FL2 | Based on received responses, the following proposal can be considered. High Priority Proposal 7.5.2-1b: The table format used in the table with cost reduction estimates achieved by combinations of UE complexity reduction features listed in the end of Section 7.5.2 in R1-2207731 is used as a baseline text for TR 38.865. |  |
| Huawei, Hisilicon | Y |  |
| FL3 | RAN1 made the following conclusion in the online session on Monday 22nd August: Conclusion: The table format used in the table with complexity reduction estimates achieved by combinations of UE complexity reduction features listed in the end of Section 7.5.2 in R1-2207731 is used as a baseline text for TR 38.865. Note: values in the table may be updated. |  |
| FL5 FL6 | The 7 tables above this feedback form show the average values of the initial cost evaluation results captured in [43]. High Priority Proposal 7.5.2-1c: The values in the table with cost reduction estimates achieved by combinations of UE complexity reduction features in Section 7.5.2 in R1-2207733 are used as a baseline text for TR 38.865. |  |
| CATT | Y |  |
| Sierra Wireless | Y |  |
| Samsung |  | Suggest to not counting AD/DA and FFT size for BW 2. |
| Nokia, NSB | Y |  |
| FL7 | The online session on Wednesday 24th August made the following conclusion: Conclusion: The values in the table with cost reduction estimates achieved by combinations of UE complexity reduction features in Section 7.5.2 in R1-2207733, with FD-FDD 1Rx corrected average, are used as a baseline text for TR 38.865. (The formulation “with FD-FDD 1Rx corrected average” refers to correction of the averaging across companies on the ‘FD-FDD 1Rx’ tab in [43] compared to the earlier version in [10]. In [10], the averages on this tab did not include the evaluation results from three companies, corresponding to columns C, D and E on that tab, but this has been fixed in [43].) |  |

### 7.5.3	Analysis of performance impacts

The potential performance impacts of different complexity reduction techniques were discussed in Sections 7.2.3, 7.3.3, and 7.4.3.

### 7.5.4	Analysis of network deployment and coexistence impacts

The potential network deployment and coexistence impacts of different complexity reduction techniques were discussed in Sections 7.2.4, 7.3.4, and 7.4.4.

### 7.5.5	Analysis of specification impacts

The potential specification impacts of different complexity reduction techniques were discussed in Sections 7.2.5, 7.3.5, and 7.4.5.

# 9	Conclusions and recommendations

Based on the submitted contributions and discussion during the meeting, the following proposal can be considered.

FL12 High Priority Proposal 9-1a: Adopt the following TP as a baseline text for TR 38.865 clause 9:

| Further NR RedCap UE complexity reduction techniques have been analyzed individually in clauses 7.2 through 7.4 as well as in different combinations in clause 7.5. The coverage impacts of the complexity reduction techniques have been analyzed in clause 8. The main observations from the coverage impact evaluations are summarized in clause 8.2.4. Based on the analysis of the studied UE complexity reduction options, RAN1 recommends option BW3 or PR3 for a Rel-18 RedCap UE. Down-selection between these two options is to be done during the WI phase or at RAN plenary. - Option BW3: - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Option PR3: - Restriction of maximum number of PRBs for PDSCH and PUSCH. - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 11 or 12. - The restricted number of PRBs in Option PR3 is a hardcoded limit. Whether or not to also introduce support for option PT1 and/or PT2 for a Rel-18 RedCap UE can be decided at RAN plenary. - Option PT1: - Relaxation of UE processing time for PDSCH/PUSCH in terms of N1 and N2 (as defined in TS 38.214) compared to those of UE processing time capability 1 - The relaxation factor for N1 and N2 is assumed to be 2 in the study. - Option PT2: - Relaxation of UE processing time for CSI in terms of Z and Z’ compared to the values defined in TS 38.214 clause 5.4 - The relaxation factor for Z and Z’ is assumed to be 2 in the study. |  |  |
| --- | --- | --- |
| Company | Y/N | Comments |
| Transsion | Y |  |
| Nordic | N | We would prefer to include also BW1. Based on the analysis of the studied UE complexity reduction options, RAN1 recommends option BW1, BW3 or PR3 for further down-selection of a Rel-18 RedCap UE at plenary. RAN1 observed that BW1 may have the largest specification impacts, coexistence challenges. On the other hand, no significant coverage challenges has been observed, and BW1 also allows for the largest complexity reduction among evaluated options. |
| Intel |  | Agree to propose BW3 and PR3. Further, our evaluation shows large complexity reduction by introducing PT1/PT2. Therefore, we prefer to recommend the adoption of PT1/PT2 in addition to BW3/PR3. However, considering the limited time for discussion, we are fine to decide it in RAN plenary. |
| DOCOMO |  | We support Nordic that we also would prefer to include BW1 at this point. In general, complexity reduction gain and impacts (network deployment and/or spec) are tradeoff and it should be investigated further which UE complexity reduction option should be the target feature of Rel-18 RedCap. |
| Ericsson | Y | We are fine with recommending BW3 or PR3. We would also have been fine with PR1 or PR2. We do not want to recommend any of BW1, BW2, PT1 or PT2 due to the expected large impacts compared to the expected cost benefits. |
| CATT | Y | We are fine to recommend BW3/PR3. PR1/PR2 would also OK to us. Based on the trade-off, we think PT1/PT2 should not be recommended. Due to lack of time and consensus, we are fine to bring PT1/PT2 to RANP. |
| Nokia, NSB | Y | We are fine to recommend BW3 or PR3 and would prefer down selection at the WI phase. We would be OK not to include BW1 given the spec impact in order to narrow down the options as much as possible.. We support to decide on PT1/PT2 at the RAN plenary. |
| Nordic |  | BW1 can bring more than 5% more complexity reduction compared to BW3 for HD-FDD, and 3x more than PR1/PR2 for HD-FDD. Whether specification effort for BW1 is worth of gain, should be for plenary to discuss. |
| OPPO |  | We think it is more clear with this proposal that the BW3 and PR3 can be replace each other in large sense. Then, better to conclusion to selection one of them. We suggest PR3 with least impact to achieve the most gain. In addition, Nodic’s proposal of BW1 is worthy to be included since more cost saving could be reached. We can justify if that be finally supported. |

# References

| [1] | RP-221161 | Revised SID on Study on further NR RedCap (reduced capability) UE complexity reduction | Ericsson |
| --- | --- | --- | --- |
| [2] | R1-2204058 | Work plan for Study on further NR RedCap UE complexity reduction | Rapporteur (Ericsson) |
| [3] | RP-221160 | Status report for Study on further NR RedCap (reduced capability) UE complexity reduction | RAN1 |
| [4] | TR 38.865 V0.0.1 | Study on further NR RedCap UE complexity reduction (Release 18) | RAN1 |
| [5] | TR 38.875 V17.0.0 | Study on support of reduced capability NR devices (Release 17) | 3GPP |
| [6] | R1-2009293 | FL summary on [Rel-17] RedCap evaluation results | Moderator (Ericsson, Apple, Qualcomm) |
| [7] | R1-221163 | Summary of [Rel-17] WI on support of reduced capability (RedCap) NR devices | Ericsson |
| [8] | R1-2205427 | RAN1 agreements for Rel-17 NR RedCap | Rapporteur (Ericsson) |
| [9] | R1-2205435 | FL summary #4 on potential solutions to further reduce RedCap UE complexity | Moderator (Ericsson) |
| [10] | R1-2207730 | FL summary for collection of evaluation results for Rel-18 RedCap SI | Moderator (Ericsson, MediaTek) |
| [11] | R1-2205739 | Potential solutions for further RedCap UE complexity reduction | Ericsson |
| [12] | R1-2205741 | Analysis of complexity reduction techniques for RedCap UEs in Rel-18 | FUTUREWEI |
| [13] | R1-2205873 | Discussion on potential solutions to further reduce UE complexity | Huawei, HiSilicon |
| [14] | R1-2205997 | Discussion on potential solutions to further reduce UE complexity | Spreadtrum Communications |
| [15] | R1-2206051 | Techniques to further reduce RedCap UE complexity | vivo, Guangdong Genius |
| [16] | R1-2206127 | Complexity reduction for RedCap UEs | Sony |
| [17] | R1-2206201 | Discussion on further RedCap UE complexity reduction | DENSO CORPORATION |
| [18] | R1-2206303 | Solution study on further reduced UE complexity | OPPO |
| [19] | R1-2206333 | Potential solutions to further reduce UE complexity for eRedCap | Panasonic |
| [20] | R1-2206409 | Discussion on solutions to further reduce UE complexity in Rel-18 | CATT |
| [21] | R1-2206417 | Discussion on potential solutions to further reduce UE complexity | NEC |
| [22] | R1-2206443 | Further RedCap UE Complexity Reduction | Nokia, Nokia Shanghai Bell |
| [23] | R1-2206494 | On further complexity reduction of NR UE | Nordic Semiconductor ASA |
| [24] | R1-2206593 | On solutions to further reduce UE complexity | Intel Corporation |
| [25] | R1-2206653 | Potential solutions to further reduce UE complexity | Xiaomi |
| [26] | R1-2206672 | Discussion on potential solutions to further reduce UE complexity | Transsion Holdings |
| [27] | R1-2206695 | Discussion on potential solutions to further reduce UE complexity | China Telecom |
| [28] | R1-2206836 | Further UE complexity reduction for eRedCap | Samsung |
| [29] | R1-2206923 | Discussion on further reduced UE complexity | CMCC |
| [30] | R1-2206937 | Discussion on solutions to further reduce UE complexity | Sharp |
| [31] | R1-2207002 | Potential solutions to further reduce UE complexity | MediaTek Inc. |
| [32] | R1-2207036 | Discussion on potential solutions for further UE complexity reduction | LG Electronics |
| [33] | R1-2207057 | Discussion on eRedCap UE complexity reduction | ZTE, Sanechips |
| [34] | R1-2207149 | On complexity reduction for RedCap UE | InterDigital, Inc. |
| [35] | R1-2207243 | Further complexity reduction for eRedCap device | Qualcomm Incorporated |
| [36] | R1-2207260 | Considerations for further UE complexity reduction | Sierra Wireless. S.A. |
| [37] | R1-2207416 | Discussion on potential solutions for further UE complexity reduction for eRedCap | NTT DOCOMO, INC. |
| [38] | R1-2207477 | Potential solutions to further reduce UE complexity | Lenovo |
| [39] | R1-2207684 | Discussion on potential solutions for further UE complexity reduction for eRedCap (update of R1-2207416) | NTT DOCOMO, INC. |
| [40] | R1-2207731 | FL summary #1 on potential solutions to further reduce RedCap UE complexity | Moderator (Ericsson) |
| [41] | R1-2207732 | FL summary #2 on potential solutions to further reduce RedCap UE complexity | Moderator (Ericsson) |
| [42] | R1-2207733 | FL summary #3 on potential solutions to further reduce RedCap UE complexity | Moderator (Ericsson) |
| [43] | R1-2207981 | FL summary #2 for collection of evaluation results for Rel-18 RedCap SIs | Moderator (Ericsson, MediaTek) |
| [44] | R1-2207734 | FL summary #4 on potential solutions to further reduce RedCap UE complexity | Moderator (Ericsson) |
| [45] | R1-2207982 | FL summary #5 on potential solutions to further reduce RedCap UE complexity | Moderator (Ericsson) |