3GPP TSG-RAN WG1 Meeting #110	R1-2208010

# Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0300 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for aperiodic CSI triggering offset for FR2-2 in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (vivo), LG Electronics, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#109-e meeting, it was agreed that the range of aperiodicTriggeringOffset-r17 for 480kHz and 960kHz was scaled by 4 of the corresponding range of aperiodicTriggeringOffset-r16 as {0,…,31}*4. These values are not specified as the CSI-RS triggering offset value in the 38.214 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add aperiodicTriggeringOffset-r17 and clarify the applicable value range of the aperiodic CSI triggering offset for 480kHz and 960kHz SCS. The CSI-RS triggering offset has the values of {0, 4, 8, 12, …, 60, 64, 96} slots for 480/960 kHz if µPDCCH >= µCSIRS The CSI-RS triggering offset has the values of {0, 4, 8, …, 124} slots for 480/960 kHz if µPDCCH < µCSIRS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Aperiodic CSI triggering is incomplete for 480kHz and 960kHz SCS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.5.1, 5.2.1.5.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

*** Unchanged text is omitted ***

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 or aperiodicTriggeringOffset-r17. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots for μCSIRS≤3  or {0, 4, 8, 12, …, 60, 64, 96} slots for μCSIRS=5 and μCSIRS=6, where μCSIRS is the subcarrier spacing configurations for CSI-RS. If the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP and minimumSchedulingOffsetK2 for any UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states, the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement. The aperiodic CSI-RS is transmitted in a slot , Ks=n+X+Nslot,offset,PDCCHCA2μoffset,PDCCH-Nslot,offset,CSIRSCA2μoffset,CSIRS∙2μCSIRS, if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and in slot , otherwise, and where

-	n is the slot containing the triggering DCI, X is the CSI-RS triggering offset according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 or aperiodicTriggeringOffset-r17,

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the  which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, CSIRSCA and μoffset,CSIRS are the Nslot, offsetCA and the  which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5.

*** Unchanged text is omitted ***

##### 5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

*** Unchanged text is omitted ***

Aperiodic CSI-RS timing:

-	When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 or aperiodicTriggeringOffset-r17, including the case that the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or minimumSchedulingOffsetK2 for any UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1, …, 31} slots for μCSIRS≤3 or {0, 4, 8, …, 124} slots for  μCSIRS=5 and μCSIRS=6 when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} for  μCSIRS≤3 or {0, 4, 8, 12, …, 60, 64, 96} slots for μCSIRS=5 and μCSIRS=6 when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where

-	n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 or aperiodicTriggeringOffset-r17,

-	μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

-	Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and  μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5

-	If the µPDCCH < µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than the first symbol of the CSI-RS carrier's slot that starts at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

-	If the µPDCCH > µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

*** Unchanged text is omitted ***