3GPP TSG RAN WG1 Meeting #110	R1-2208015

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.1.3.2

Source:	Moderator (Futurewei)

Title:	FL Summary #4 on SRS enhancements

Document for:	Discussion and decision

# Introduction

In RAN#94-e, a new Work Item for Rel-18 on “MIMO Evolution for Downlink and Uplink” was approved, and the motivations, scopes, and objectives were agreed in [1]. Among the objectives, the underlined in the following are related to SRS enhancements, mainly in the aspects of SRS for TDD Coherent Joint Transmission (CJT or C-JT) and 8 Tx operation:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

25 contributions [2-26] have been submitted to Agenda Item 9.1.3.2 of RAN1#110 on SRS Enhancements targeting TDD CJT and 8 Tx operations. Main views and further discussion points based on these contributions are collected in this document. Any additional inputs from any company can also be provided in this document.

# SRS enhancement specification work

Many companies have expressed interest and support for Rel-18 SRS enhancements for TDD CJT and 8 Tx, as partially demonstrated by the large number of potential enhancement proposals from the last and present RAN1 meetings. A variety of LLS and SLS have been provided [2, 4, 5, 10, 16, 20, 24], which shows the benefits with enhanced SRS on top of existing Rel-17 standards. These are generally aligned with the collective view expressed by operators on the promising gain from CJT field test [27] and the common understanding of the usefulness for 8 Tx SRS. While the discussions on specific enhancements are ongoing and may take multiple meetings, it seems that there is a sufficient amount of support from companies to justify moving forward with the specification work at the high level. Therefore, we suggest confirming a high-level agreement on the specification work for Rel-18 SRS enhancements based on the scope of the WID, though whether to specify a particular enhancement will be further discussed.

Proposal 2.1: For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

SRS enhancements to enable 8 Tx UL operation.

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Google | Support |
| LGE | We think that there is no information in this proposal. It would be better to agree on detailed proposals in section 3, 4, if there is consensus. |
| OPPO | We are fine with the second bullet which is necessary to support uplink 8 ports transmission. For the first bullet, RAN1 agreed to study 11 candidate options for the enhancement in the last meeting. We are not sure whether there could be any solution with sufficient proponents. If we agree on the first bullet, does it mean that RAN1 should agree on at least one solution out of the candidate options? |
| Huawei | Support. Based on the company's analysis and evaluations, it is reasonable to believe that SRS enhancement (randomization/ capacity enhancement) can further improve the performance of TDD CJT. Currently, most companies are interested in this topic and many candidate solutions are proposed. It is recommended that a general agreement can be reached firstly to specify the SRS enhancement. |
| Apple | We support the second bullet For the first bullet, there are 11 options from the last RAN1 meeting which makes the scope of the discussion difficult. It is more important to discuss possible down selection at this stage. |
| DOCOMO | Support |
| Samsung | Support the proposal. Based on this, we can discuss/specify detailed proposals. |
| Spreadtrum | Support |
| CMCC | Support |
| InterDigital | Support second bullet. For the first bullet, it depends on the evaluation of the different schemes highlighted in Section 3. |
| Intel | Fine |
| ZTE | Support In addition, for the first bullet, we suggest to down select some methods with higher priority and we support to prioritize the capacity enhancement over interference enhancement. Because each TRP has to accurately receive SRS from more UE. Each TRP allocates the orthogonal SRS to the serving UE and CJT UE in neighboring cell is an efficient method. In addition, the CJT TRP is with ideal backhaul. |
| Xiaomi | Support. For the first bullet, we suggest to add some constraints, i.e., 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without introducing new SRS root sequences. |
| MediaTek | Support the second bullet. In our opinion first bullet is rather vague and we need more clarification on the overall constraint of the possible solutions and down select approaches and the first. |
| Lenovo | Support. We have similar view with Huawei that SRS enhancement (randomization/capacity enhancement) can further improve the performance of TDD CJT. After agreeing on this proposal, we can go on to discuss and down select schemes for the 11 options below. |
| Nokia/NSB | Support the proposal. Share view with Apple, need further down selection for 11 options. |
| CATT | Support the second bullet. Regarding the first bullet, we have 11 candidate solutions. If the intention is to agree on the support of this feature before down-selection, the number of solutions to be specified shall be limited. For example, one solution for interference randomization and/or one solution for capacity enhancement. |
| Sharp | Support |

## FL update

Most inputs so far support the 8 Tx part, and some companies would like to clarify the TDD CJT SRS part.

I do agree that down-selection / prioritization will be inevitable for the 11 schemes, but any down-selection / prioritization should be based on good technical understanding of each scheme. My thinking is to give each proponent enough time to describe/explain their proposals, and give any other company enough time to ask for clarification questions, weigh the pros and cons, or even evaluate by themselves. This will be slow at the initial stage, but should facilitate convergence later and ensure good enhancement(s) to be selected. While that is taking some extra time, we can make a small step with this proposal based on the agreed WID objectives.

@LGE, OPPO, Apple, InterDigital: As you can see, discussion of each scheme is ongoing, and necessary down-selection will happen. No scheme is guaranteed to be supported and no scheme is ruled out, at this stage. That does not prevent us from confirming to work towards the high-level goal as a small step to achieve the WID objectives.

@Xiaomi: The sentence ‘the agreed WID scopes apply’ should already be sufficient.

Based on the inputs, please consider the updated proposal, with a sub-bullet to address some companies’ concerns:

Proposal 2.1-1: For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

Whether a particular enhancement is to be supported or not will be decided in next meetings

SRS enhancements to enable 8 Tx UL operation.

| Company | View |
| --- | --- |
| Nokia/NSB | We are generally fine. Also, agree the we need to narrow down candidate schemes but not necessary to make final decision to select one scheme. |
| ZTE | support |
| Huawei, HiSilicon | Support |
| Lenovo | Support. We have similar view with Nokia that it is not necessary to select only one scheme. On account of 11 candidate schemes, it will give some guidance for further discussion/down selection if the maximum number of schemes can be agreed to select based on standardization overload. |
| Ericsson | We are supportive of both the first bullet and second bullet. For the first bullet, we believe some down-selection of candidates can be done already in this meeting. The second bullet needs some revision to add SRS enhancements to cover DL operation as well (i.e., to support 8 Tx antenna switching, which is considered, e.g., in Proposal 4-1). Suggest the following modification to the 2nd bullet: SRS enhancements to enable 8 Tx UL operation and 8Tx SRS for DL operation. |
| NEC | Support |
| CATT | Support the second bullet. Regarding the first bullet, we have 11 candidate solutions. If the intention is to agree on the support of this feature before down-selection, the number of solutions to be specified shall be limited. For example, one solution for interference randomization and/or one solution for capacity enhancement. |
| vivo | Support |

## FL update 2

Most companies support this proposal.

@Ericsson: Agree with you on the 8 Tx SRS for DL operation. In the last meeting, we had agreement on “Study the potential enhancements for SRS of 8T8R with usage antennaSwitching.” in parallel with “Study the potential enhancements for SRS for 8 Tx operation … CB …  NCB”. In addition, RAN1 never had a SRS design of n Tx supporting only CB/NCB but not AS. To be consistent with the last agreement, I think “8Tx SRS” can be changed to “8T8R SRS”. BTW, also agree with you that the down-selection of candidates may be done starting from this meeting. The proposal is revised accordingly.

@CATT: I do agree with you that realistically, only a very limited number of candidate solutions may be specified eventually. For now, however, it is difficult to agree on a specific number. I think we can leave that for future decision.

The proposal is further revised below, mainly based on Ericsson’s suggestions. Any further comments are welcome.

Proposal 2.1-2: For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

Whether a particular candidate solution is to be supported or not will be decided

SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

| Company | View |
| --- | --- |

## Online Agreement:

Proposal 2.1-2:

For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

RAN1 should strive to minimize the number of schemes supported in Rel-18

SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

Target usage includes antenna switching, codebook/non-codebook based SRS

# SRS enhancements to manage inter-TRP cross-SRS interference targeting TDD CJT

In the previous meeting, 11 categories of potential SRS enhancements to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization were agreed for further study. Some of the categories also include sub-categories of different schemes. Prioritization or down-selection of the categories, if any, should be based on good technical understanding of the potential enhancements. Therefore, at least for the initial round(s), the group can focus on technical aspects of the enhancements, especially on clarifying the key ideas of the enhancements, analyze pros (e.g., performance benefit, necessity, the problems that they can solve) and cons (e.g., limitations, issues to addressed), etc. Based on these discussions, the group can achieve common understanding of the technical aspects. The initial rounds are not intended to collect any views on priorities/down-selection, but priorities/down-selection will be discussed after the initial rounds.

Please provide your inputs on the following questions for the enhancements in the subsections:

Q1: Description/clarification of the enhancement

- If you feel the key idea of the enhancement is not clear enough, please ask for clarification from the proponents.

- The proponents can answer the questions and can provide any additional descriptions to make the enhancement clearer to the group.

Q2: Pros and cons of the enhancement

- Please analyze pros (e.g., performance benefit, necessity, the problems that they can solve, use cases, potential source of gains, etc.) and cons (e.g., limitations, issues to addressed, etc.). Possible alternative solutions can also be suggested.

Other key points you’d like to bring up, if any

A list of proposed schemes under each category is provided, together with a list of proponents. Please feel free to suggest if something is missing or inaccurate.

Examples of answers to Q1 and Q2 are provided below, based on the contribution [2].

## Randomized frequency-domain resource mapping for SRS transmission

At least the following potential enhancements have been discussed:

A1: Further enhancements to frequency hopping (e.g., Lenovo, Xiaomi, Samsung, Futurewei)

A1.1: Different bandwidths in FH (e.g., Samsung, Futurewei)

A1.2: Additional parameters for FH (e.g., Lenovo,)

A1.3: More SRS bandwidth configurations for FH (e.g., Futurewei)

A2: Comb hopping (e.g., Spreadtrum, Lenovo, Intel, Qualcomm, Ericsson, Futurewei, CMCC, ZTE)

A2.1: Comb offset hopping (e.g., Spreadtrum, Lenovo, Intel, Qualcomm, Ericsson, Futurewei)

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Futurewei [2] | Q2: 1) Some potential enhancements (except for probably A1.3) may lead to non-uniform SRS sample pattern in time/frequency domain, which may impact the SRS-based channel estimation design / complexity / performance. 2) With the higher randomness introduced, previously orthogonal (FDMed) SRSs may become non-orthogonal, unless some additional design or network coordination is used. For example, FDMed SRSs may need to be configured with the same hopping / randomization patterns. |
| Google | We are ok to discuss the potential enhancement but we think randomized FD resource mapping can be deprioritized compared to randomized CD resource mapping. In our view, all the candidate schemes in 3.1/3.2/3.3 can achieve interference randomization, but we have to prioritize one or two schemes. It is hard to specify all of them. |
| QC | We prefer to not introduce new schemes (we already have a lot of them to choose from). The intention for this part was “randomization” and it is not clear how A1 can result in randomization. Regarding A2.2, we appreciate some more details as to how UEs can be multiplexed on different comb offsets if the comb spacing itself is randomized (comb 2/4/8). With A2.1, SRS pattern is obviously still uniform, and Ues with the same seed (e.g., based on configuration) can hop consistently to maintain orthogonal property within the same group (seed). |
| LGE | Regarding enhancement on SRS interference randomization, we think that it is not well fit into CJT transmission case since ideal backhaul between TRPs with tight coordination of SRS resource allocation would be assumed for CJT scenario. In ideal backhaul cases, inter-TRP inference can usually be handled by NW scheduling coordination, with proper SRS capacity. |
| OPPO | The current frequency hopping can already support frequency-domain resource randomization via hopping pattern per UE. The enhancement on frequency hopping and comb hopping would destroy the orthogonality with legacy UE and restrict the gNB scheduling flexibility. We propose to deprioritize this option. |
| Huawei | We are fine for A2.1. @LGE, considering the limited SRS resources and implementation complexity, it is impossible to always perform joint orthogonal SRS resources allocation for multi-TRPs. |
| Apple | In our view, among 11 candidate solutions, we should first remove some of the solutions before going into details. Otherwise, the discussion can be very long. We prefer Alt 2, we do not prefer Alt 1 which changes the FH |
| DOCOMO | For A2.1, is it not applicable to the case of only 1 SRS symbol? If so, we think its benefit might be a bit smaller than e.g., code-domain randomization because of the narrower use case? |
| Samsung | We support A1. In our view, A1 can randomize interference depending on how to determine FH patterns. Different BWs in FH can differentiate/randomize cross-SRS interference. Hence, we suggest to down-select and study A1. |
| Spreadtrum | We are fine for either A2.1 or A2.2. From our perspective, any of them can achieve the same performance of interference randomization. |
| CMCC | We prefer A2.1. Maybe more details are needed to understand how to achieve interference randomization. For A2.2, it seems comb hopping will result in the non-uniform SRS pattern, which might increase UE or gNB complexity. |
| Intel | Ok with A2.1. Changing frequency hopping would be complicated and is not preferred. |
| ZTE | We prefer A2.1 For A2.2. the number of the co-scheduled SRS is different and the channel estimation accurate at different transmission is different. One power parameter can not be suitable to different combs. The UE needs to generate more than one sequence for one SRS resource. |
| Xiaomi | For Alt1.2, Our proposal is that the start RB index of SRS transmission for partial frequency sounding is randomized, such that different SRS can be randomly transmitted in different frequency domain location. This proposal is similar with I2 of partial frequency sounding extensions in subsection 3.9. Thus, it can remove our positioning in Alt1.2. We are also fine for Alt2.1. |
| Futurewei | @Qualcomm: Thank you for the questions. For A1, our understanding is that with more FH behaviors supported, overall SRS interference will appear to be more random. For A2.2, one example is to multiplex SRSs on the first transmission occasion with comb 2, and then transmit the same SRSs on a later transmission occasion with comb 4, which are still orthogonal. There is not much support along this direction, and we can remove this option so that the group can be more focused. For A2.1, we think what you described can work if the SRSs are also configured with the same transmission occasions, which requires proper network coordination. So there is indeed a tradeoff as mentioned by OPPO. @LGE: What you suggested is true if the SRS capacity is high and the coordinated UEs are few. With limited SRS capacity and a large number of UEs, orthogonalization of all SRSs may be extremely challenging, if not infeasible. @CMCC: Agree with you on A2.2. We can remove the option for now. |
| FL | @all: Please try to understand/clarify the schemes as much as possible. Proponents, please try to address the technical questions by others as much as possible. We hope there won’t be any outstanding questions left after a couple of rounds. |
| MediaTek | We support Alt 2.1. We prefer not to introduce new frequency hopping behaviors. |
| Lenovo | We are fine with A1.2 and A2.1. For A1.2, the initial frequency position can be made hopping (Not use existing fixed RRC configured value) based on an existing hopping mechanism and further improve randomization. It is simple for realization and have no impact on legacy hopping pattern. |
| Nokia/NSB | We agree with OPPO that both A1 and A2 may cause problems with coexistence of legacy and Rel-18 UEs introducing scheduling restrictions. Furthermore, if non-uniform patterns in frequency/time are applied, channel estimation performance at gNB can be degraded. Therefore, we prefer to depriorize these both Alternatives (A1 and A2). |
| Ericsson | We support Alt 2.1; the existing frequency hopping and partial sounding schemes are already highly configurable. As pointed out by some companies, the enhancements listed under A1 may result in non-uniform time/frequency domain SRS samples which will impact the channel estimator at the gNB. Hence, we do not prefer to introduce further enhanced frequency hopping. Since comb offset hopping does not have this problem, we can further study comb offset hopping. Co-existence with legacy UEs can be supported with Alt 2.1 by restricting the comb-offset hopping to a subset of the possible comb offsets. |
| NEC | We share similar view with OPPO and Nokia, and we also prefer to deprioritize the two alternatives, which will cause issue at least for legacy UEs. |
| Sharp | We share similar view with OPPO. |
| vivo | We also have concern on the compatibility with legacy UE. These potential schemes would affect the SRS estimation of legacy UE. |

### FL update

Hopefully by now, all companies are familiar with the key ideas, pros, and cons of the candidate solutions. Props and cons as well as supporting companies are quickly summarized below based on the inputs and my understanding.

A1: Further enhancements to frequency hopping (e.g., Lenovo, Xiaomi, Samsung, Futurewei, Huawei, HiSilicon, Spreadtrum)

A1.1: Different bandwidths in FH (e.g., Samsung, Futurewei, CMCC)

Pros: Interference randomization, SRS capacity enhancement

Cons: Non-uniform SRS sample pattern in time/frequency domain; additional NW coordination needed for multiplexing orthogonality

A1.2: Additional parameters for FH (e.g., Lenovo)

Pros: Interference randomization, SRS capacity enhancement

Cons: Non-uniform SRS sample pattern in time/frequency domain; additional NW coordination needed for multiplexing orthogonality

A1.3: More SRS bandwidth configurations for FH (e.g., Futurewei)

Pros: Interference randomization, SRS capacity enhancement

A2: Comb hopping (e.g., Spreadtrum, Lenovo, Intel, Qualcomm, Ericsson, Futurewei, CMCC, ZTE, Apple, Xiaomi, MediaTek)

A2.1: Comb offset hopping (e.g., Spreadtrum, Lenovo, Intel, Qualcomm, Ericsson, Futurewei)

Pros: Interference randomization

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |

## Randomized code-domain resource mapping for SRS transmission

At least the following potential enhancements have been discussed:

B1: Cyclic shift hopping / randomization (e.g., Huawei, HiSilicon, LG Uplus, Spreadtrum, Google, Lenovo, OPPO, Intel, Samsung, CMCC, Sharp, Ericsson, Futurewei)

B2: Sequence hopping / randomization (e.g., ZTE, Futurewei)

B3: Per-hop sequence from a long SRS sequence (e.g., Huawei, HiSilicon, LG Uplus, Futurewei)

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Futurewei [2] | Q2: 1) For B1 and B2, with the higher randomness introduced, previously orthogonal (CDMed) SRSs may become non-orthogonal, unless some additional design or network coordination is used. For example, CDMed SRSs may need to be configured with the same hopping / randomization patterns. 2) B3 leads to higher PAPR. |
| Google | We think B1 could be a good candidate for interference randomization. |
| QC | B1: The benefit over sequence hopping over a larger set of sequences is not clear. For example, if both sequence hopping and group hopping are enabled (simple enhancement using two legacy mechanisms) hopping can be across 60 sequences. Then, probability of collision reduces. B2: We think sequence hopping + group hopping is a simple enhancement that can provide benefit. B3: Not only PAPR is increased but joint processing is required at the receiver over multiple hops. We do not support this. |
| LGE | Same comment as section 3.1. |
| OPPO | Cyclic shift hopping has been studied/specified since LTE. In Rel-18, we can further evaluate whether it is helpful for interference randomization for coherent JT. For B3, we have the following observations: - Sequence hopping based on index of slot and OFDM symbol is difficult to be adopted for each hop. - SRS with different bandwidths and the same RBs in one hop cannot be multiplexed together, which reduce the scheduling flexibility. - PAPR may be increased with a truncated sequence. |
| Huawei | B1: CS hopping is a promising solution for interference randomization. Obvious gain can achieved compared with interference randomization captured in current Spec [4]. B3: PAPR may be increased slightly. The performance gain can still be obtained considering power backoff (caused by PAPR), especially under the interference-dominated scenario. |
| Apple | We can consider B1. However, we already have group and sequence hopping already in the code domain. |
| DOCOMO | We prefer B1 and/or B2 as updated above. We are not sure if configuring both sequence hopping and sequence group hopping is sufficient considering the number of sequence candidates to be increased. As the target scenario is M-TRP CJT, further randomization based on the number of TRPs could be considered. |
| Samsung | We can focus on supporting B1. |
| Spreadtrum | We think B1 can be studied first, and low priority for B2/B3. |
| CMCC | We think B1 can be considered for interference randomization. For B2, maybe the current sequence hopping and sequence group hopping is sufficient. |
| Intel | Fine with B1. |
| ZTE | We prefer B2. For B1, we don’t see any gain for CS hopping without group hopping. We suggest to consider both group hopping/sequence hopping and CS hopping being enable simultaneously, the CS hopping is to ensure different CS is used for multiple SRS resources with same group according to current group hopping. |
| Xiaomi | Both sequence hopping and sequence group hopping has been support in current spec. Hence, B2 can be considered due to less specification impact. |
| Futurewei | @Qualcomm: For B3, joint processing over multiple hops is needed with or without this design. |
| MediaTek | We prefer B2. |
| QC | @Futurewei: Thanks for the follow-up. For B3, can you please elaborate a bit more why joint processing over multiple hops is needed for the purpose of SRS channel estimation without this proposal (when SRS sequence is based on # of REs per hop)? |
| Lenovo | We are fine with B1. For B3, the PAPR issue needs to be considered. |
| Nokia/NSB | Similar view as in section 3.1. We don’t support the schemes. |
| Huawei | @ZTE The gain provided in our Tdoc is obtained by the CS hopping ONLY (i.e., the baseline is group hopping, the proposed scheme is CS hopping). May be further alignment about the detail (e.g., hopping granularity) can be performed. Although the analysis from our side shows that CS hopping can already bring adequate benefit in terms of sequence-domain randomization, we are relatively open about enabling CS hopping and group hopping together. Further simulation will follow to show whether there exists extra benefit. |
| Ericsson | We are positive to further considering B1 (cyclic shift hopping). There are some early results available in submitted contributions this meeting that show gains of CS hopping over sequence hopping and group hopping. Hence, we think cyclic shift hopping should be further evaluated. With respect to B2, we think it can be low priority. We are not supportive of B3 as it will increase PAPR. |
| Sharp | We prefer B1. |
| vivo | Regarding B1, it seems CS hopping would also affect the SRS estimation of legacy UE. Is there any mechanism to avoid it? Regarding B2, sequence hopping + group hopping is an easier solution. Regarding B3, the sequence per FH would be different and PAPR would be increased. |
| Futurewei | @QC: Thanks for the follow-up comment. We thought the processing with or without B3 would be the same as the time/frequency/CS resource allocations are the same. Maybe we can further discuss offline. |

### FL update

Props and cons as well as supporting companies are quickly summarized below based on the inputs and my understanding.

B1: Cyclic shift hopping / randomization (e.g., Huawei, HiSilicon, LG Uplus, Spreadtrum, Google, Lenovo, OPPO, Intel, Samsung, CMCC, Sharp, Ericsson, Futurewei, Apple, DOCOMO, CMCC, Sharp)

Pros: Interference randomization; one source shows throughput performance benefit

Cons: potential multiplexing issue with legacy UE; restriction on scheduling flexibility; potential reduction of SRS capacity

B2: Sequence hopping / randomization (e.g., ZTE, Futurewei, QC, DOCOMO, Xiaomi, MediaTek, vivo)

Pros: Interference randomization

B3: Per-hop sequence from a long SRS sequence (e.g., Huawei, HiSilicon, LG Uplus, Futurewei)

Pros: SRS multiplexing; one source shows cross-correlation performance benefit

Cons: PAPR increased

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |
| OPPO | We think B1 can be further evaluated and studied in Rel-18 as commented in previous round. It doesn’t mean that we propose to support it now. Hence we delete OPPO from the proponents for now. |

## Randomized transmission of SRS

At least the following potential enhancements have been discussed:

C: Randomized transmission of SRS, e.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS (e.g., vivo, Qualcomm, Futurewei)

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Futurewei [2] | Q2: C may lead to non-uniform SRS sample pattern in time domain, which may impact the SRS-based channel estimation design / complexity / performance. |
| Google | This functionality can be achieved by aperiodic SRS. The benefit is limited compared to schemes in 3.1/3.2. We think this should be deprioritized. |
| QC | This scheme can provide good gains as shown in our contribution. Obviously, aperiodic SRS requires one DCI for each instance of transmission, but wasting resources to send DCIs are not needed for the purpose of randomization. Also, the scheme can be designed to ensure that regularity/rate of transmissions by controlling the possible patterns (e.g., to grantee that number of consecutive muting is small). Please see “Approach 2” in our Tdoc and corresponding evaluations. |
| LGE | Similar view as Google. |
| OPPO | We think a longer periodicity for P/SP SRS or AP SRS can achieve similar functionality. It would also make the channel estimation algorithm at gNB more complicated due to partial muting of SRS. |
| Huawei | Pseudo-random muting of SRS transmission may increase the equivalent measurement period, which decreases the SRS channel estimation accuracy, especially in mobility scenarios. |
| Apple | We do not prefer this solution |
| DOCOMO | Agree with Google. Also, generally muting causes a trade-off between sounding opportunity and interference. So we prefer to deprioritize this issue. |
| Samsung | Agree with FL and Google. We suggest to focus on other proposals, and deprioritize this direction. |
| Spreadtrum | The performance of muting SRS could be unpredictable in practice. We don’t support this solution. |
| Intel | Same view as Google and other companies. This could be deprioritized. |
| ZTE | Same view as Intel. |
| Xiaomi | We have concerns the estimation accuracy due to randomized transmission. The scheme can be studied with low priority. |
| MediaTek | We prefer to deprioritize this solution |
| QC | @OPPO, Huawei: Longer periodicity does not achieve interference randomization. Static allocation with longer periodicity is actually what is simulated as baseline in our evaluation. @Docommo: The proper comparison is as follows: Assuming the same number of SRS transmissions in an interval, is it better to transmit SRS with a fixed patter or a randomized patter. Hence, both baseline and pseudo-random muting in our evaluations have the same sounding opportunity. @ Spreadtrum, Xiaomi: The performance can be made predictable as in approach 2 in our contribution. As a simple example, if you consider a bundle of 2 SRS transmission opportunity, UE can randomly select {10 or 01} for one bundle. Then, there is exactly one transmission in a bundle. As discussed in our contribution, the set of binary sequences can be configured by network, which can control various properties such as max run length (consecutive muting / transmission), ratio of transmissions, etc. |
| Lenovo | We think it has impact on channel estimation accuracy. A similar functionality can be carried out by semi-persistent or aperiodic SRS to some extent. |
| Nokia/NSB | We share view with Google, Samsung and other companies. We prefer to deprioritize this approach. |
| Ericsson | Lower priority. We agree with other companies that Aperiodic SRS can achieve similar functionality. It should be noted that with Aperiodic SRS, the gNB can trigger the SRS when it needs to estimate the channel on the uplink. If the muting pattern is RRC configured to the UE, then the gNB has to wait for the next SRS transmission opportunity to estimate the channel in uplink, which seems inflexible. So, we don’t see a clear advantage of pseudo-random muting over aperiodic SRS. For periodic and semi-persistent SRS, configuring larger periodicity instead of pseudo-random muting will lead to less interference and fixed measurement delay. |
| Sharp | We don’t prefer this solution |
| vivo | Support C. Compared with A and B, C can be regarded as a randomized time-domain resource mapping for SRS transmission. It can handle the collision of SRS resources based pseudo-random sequence or muting pattern with bitmap, which would cause limited spec effect. Besides, some answers are given to the questions from companies have concern on C. Channel estimation issue: Some companies mention that C may decrease the channel estimation accuracy. We would like to notice that all of the potential enhancements to handle the collision/interference of two SRS resources would have impact on the channel estimation accuracy, compared with legacy orthogonal SRS allocation. Similar with CS/comb hopping randomized in code/frequency-domain to randomize the interference, C is based on randomized SRS occasion muting/hopping in time-domain to handle the interference with estimation gain. Signaling overhead issue: C is quite different with AP-SRS based on DCI. The randomized muting can be configured by RRC or determined by default rules. The overhead is small. Longer periodicity issue: Some companies also noticed that C is equivalent to P/SP SRS with longer periodicity. However, C is an enhancement based on randomization, which is different essentially. The periodicity of SRS in C would be shorter than legacy orthogonal SRS allocation. |

### FL update

Props and cons as well as supporting companies are quickly summarized below based on the inputs and my understanding.

C: Randomized transmission of SRS, e.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS (e.g., vivo, Qualcomm, Futurewei)

Pros: Interference randomization; one source shows channel estimation performance benefit

Cons: [Channel estimation accuracy; potential alternatives exist (long-periodicity P/SP SRS + AP SRS)]

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |
| QC | [For presentation during offline] - Approach 1: UE decides whether to transmit an SRS transmission opportunity according to a formula that is determined based on the pseudo-random sequence c(i) as a function of time (e.g., slot number, symbol number). - For a given SRS transmission opportunity, the formula generates 0 or 1 corresponding to muting or transmission, respectively. - Approach 2: UE selects a binary sequence of length L corresponding to L SRS transmission opportunities according to a formula that is based on the pseudo-random sequence c(i) as a function of time (e.g., slot number or symbol number of the first SRS transmission opportunity). - For a given L SRS transmission opportunities, the formula generates an index pointing to a binary sequence of length L out of a set of binary sequences. Approach 2 can grantee the performance based on the possible set of binary sequences with respect to the following aspects: - The min / max number transmissions / muting’s in the L transmission opportunities is based min / max number of 1’s / 0’s of any binary sequence in the set of binary sequences; - We can ensure that there are not too many consecutive muting’s or too many consecutive transmissions for a given UE; - The number of collisions between UEs that select different binary sequences from the set can be controlled. For example, if L=2 and the possible set of binary sequences is {01, 10}: - There is exactly one transmission and one muting in the L=2 transmission opportunities; - The max number of consecutive muting’s / transmissions is two; - There is no collision between UEs that select different binary sequences in the same L=2 transmission opportunities. Consider the case where each cluster consists of 4 TRPs, and hence, there are 8 UEs in each cluster in each drop. In this case, each UE transmits SRS in 100 occasions out of 800 candidate occasions. Similar as before, for both cases of static allocation and pseudo-random muting, it is ensured that 2 UEs in the same cluster do not transmit on the same SRS candidate occasion. In the Fig, the user CDF of UL SINR is plotted wrt both the strongest TRP (solid line) as well as the second strongest TRP (dashed line) subject to the condition that PL delta compared to the strongest TRP is not larger than 6dB (otherwise, we do not consider the second TRP as an intended receiver for the SRS transmission). As it can be seen, the benefit of pseudo-random muting is even more when cluster size is more than one TRP as there is less SRS “pollution” in the system and interference randomization has larger impact in terms of median UL SINR per UE for the tail UE performance. |

## Per-TRP power control and/or power control of one SRS towards to multiple TRPs

At least the following potential enhancements have been discussed:

D: Per-TRP power control and/or power control of one SRS towards to multiple TRPs (e.g., ZTE, Google, Lenovo, Intel, Samsung, Apple, Ericsson, Nokia, Nokia Shanghai Bell, Futurewei)

D1: Per-TRP power control

D2: Power control based on more than 1 DL pathloss RS (e.g., ZTE, Nokia, Nokia Shanghai Bell)

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Futurewei [2] | Q2: gNB can control SRS power control open-loop and closed-loop (TPC command) parameters based on existing mechanisms in implementation. Are the existing mechanisms sufficient already? |
| Google | Support to study this enhancement. |
| QC | D1 is not clear. D2 can be studied with lower priority as existing power control mechanisms are quite flexible as mentioned by Futurewei. |
| LGE | Question on D2, how multiple PL-RSs are mapped to each transmission instance of SRS? |
| OPPO | On one hand, we think per TRP power control is not related to “SRS capacity enhancement and/or interference randomization”. We doubt whether it is within the scope of the WID. For D1, per TRP power control via multiple SRS resources has been supported by NR. We don’t think one SRS resource can support per TRP power control since only single transmit power would be applied. For D2, only one pathloss value would be applied to power control finally. Then the benefit of multiple RS based pathloss over single RS based pathloss is unclear considering the pathloss RS and other power control parameters are controlled by gNB. |
| Huawei | The benefit of power control enhancement is not clear. |
| Apple | We are fine to discuss this solution especially D2 |
| DOCOMO | We also think the benefit of power control enhancement is still not clear. |
| Spreadtrum | D2 causes power control issue at UE-side. How UE use these multiple PLRS to get one single value? We think D1 is a more convincible solution. |
| CMCC | We tend to agree with OPPO, the per-TRP power control is not in the scope (capacity enhancement or interference randomization). |
| Intel | Support to study the power control issue. |
| ZTE | For D1, if the UE transmits multiple TRP-specific SRS resources with different transmits powers, how the gNB know the power offset among the multiple TRP-specific SRS resources to determine the downlink CJT precoding. In addition, using different powers causes different phase errors at the UE side, then it also impacts the gNB to determine CJT precoding. In a word, using different powers for different TRP-specific SRS impact the gNB to determine downlink CJT precoding. For D2, first the TRP common SRS should be supported for CJT considering different powers for different TRP-specific SRS resources does not work as discussed in D1. In addition, TRP-common SRS resource can reduce UE power and interference compared with TRP specific SRS. For TRP common SRS, PL should be determined according to multiple CSI-RS resources each of which corresponds to one TRP. For example, the PL is based on the maximal or the mean of PLs of multiple TRPs to ensure each TRP can accurately receive the TRP-common SRS @Futurewei, QC, OPPO, Huawei, the gNB can’t control the power by configuring power parameter due to the gNB don’t know the PL offset between the TRPs and the PL offsets between different TRPs changes while the UE moves. In addition, using power control adapt signaling leads unnecessary signaling overhead, more coordinate between TRPs and lower performance because the transmitting power can not match the channel in real time. @LGE, Spreadtrum, the final PL is the maximal or the mean of PLs of the multiple PL-RSs @Huawei, TRP common SRS is important for CJT and the power enhancement is important to ensure each TRP can accurately receive the TRP common SRS resource. |
| Xiaomi | we agree with OPPO. We do not think power control bring SRS capacity enhancement and/or interference randomization. |
| FL | D1 proponents, please reply to QC and others to describe the main idea. D2 proponents, please reply to LGE/Spreadtrum. |
| MediaTek | Agree with Huawei, we don’t see the benefit of such power control enhancements for SRS capacity improvement or interference randomization. |
| Lenovo | We are fine to study D2 in case of large pathloss difference since the power control mechanism can be used for mitigating interference. |
| Nokia/NSB | We are also fine to discuss more about UL PC enhancements (e.g. D2). |
| ZTE(2) | Thanks Moderator’s suggestion. @LGE, Spreadtrum, the final PL is the maximal or the mean of PLs of the multiple PL-RSs to ensure every CJT TRP of the UE can accurately receive the TRP-common SRS. |
| Ericsson | We are in favor of studying D1. As we explain in our contribution, there is a near-far issue if a same SRS resource is received simultaneously at multiple TRPs. For example, if the power control is optimized for the TRP with the weakest link, the SRS will be received at unnecessarily high power at another TRP, resulting in additional interference for other SRS ports on the same comb offset. If the power control is optimized for the TRP with the strongest link, the channel-estimation quality may be poor at another TRP. With per-TRP power control, it is possible to alternate between a higher and lower power setting and, hence, change/randomize how much interference that will be received by co-scheduled UEs. |
| Sharp | We prefer to deprioritize this solution. |
| KDDI | We support D1. There is a near-far problem when SRS is received at multiple TRPs. The pathloss between a UE and multiple TRP’s will be significantly different. Therefore, per-TRP power control is needed. |
| vivo | We also think the benefit of enhancements on power control for SRS capacity improvement or interference randomization is not clear. Besides, compared with other potential schemes, this scheme requires the network to frequently adjust the power control if UE moves. |

### FL update

Props and cons as well as supporting companies are quickly summarized below based on the inputs and my understanding.

D: Per-TRP power control and/or power control of one SRS towards to multiple TRPs (e.g., ZTE, Google, Lenovo, Intel, Samsung, Apple, Ericsson, Nokia, Nokia Shanghai Bell, Futurewei)

D1: Per-TRP power control (Spreadtrum, Ericsson, KDDI)

Pros: No gNB receive power imbalance issue due to near-far problem

Cons: Higher SRS overhead

D2: Power control based on more than 1 DL pathloss RS (e.g., ZTE, Nokia, Nokia Shanghai Bell, Apple, Lenovo)

Pros: Low SRS overhead; one source shows throughput/channel estimation performance benefit

Cons: gNB receive power imbalance issue due to near-far problem

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |

## SRS TD OCC

At least the following potential enhancements have been discussed:

E: TD OCC (e.g., ZTE, Spreadtrum, Lenovo, OPPO, NEC, Intel, CMCC, Ericsson)

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Futurewei [2] | Q1: Not too clear if TD OCC and cyclic shift based CDM can be combined or not. If yes, please describe how. If no, then it seems it reduces SRS capacity. Please clarify. |
| Google | TD-OCC requires additional SRS symbols. We would like to see how much benefit it can provide. |
| QC | With TD-OCC, the performance is impacted due to loss of orthogonality if one SRS symbol is dropped (as in current spec, SRS dropping is at the symbol level). Additionally, TD-OCC may be more relevant to coverage enhancements (discussed in Rel-17 but not agreed) as opposed to capacity enhancements. |
| LGE | Capacity gain should be more studied, as Futurewei, Google mentioned. |
| OPPO | TD-OCC could be beneficial for SRS capacity. However, it can only be applied to UE configured with SRS repetition, e.g. cell edge UEs. The scenarios would be a little restricted. |
| Huawei | The maximum capacity is not really increased and the multiplexing between different repetition factors should be considered. |
| Apple | We are open to discuss, however, this is more for capacity enhanmcent. |
| DOCOMO | Agree with Google. The cost of 1 OFDM symbol might make the benefit much smaller in our view. |
| Samsung | TD-OCC was extensively discussed in Rel-17, but not agreed. We prefer to deprioritize it and focus on discussing other proposals. |
| Spreadtrum | Similar view as OPPO. The benefit is clear in certain cases. |
| CMCC | We are open to consider this for capacity enhancement. |
| Intel | Support to introduce TD-OCC for capacity enhancement. |
| ZTE | First, in large delay spread case, the cyclic shift based CDM is not suitable and TD-OCC has lower overhead compared with FDM with different combs in the case with repetition. Second, the SRS enhancement is for CJT, there are many CJT UEs. For example , the ratio of CJT UE is 60% in our SLS simulation. CJT UE needs to transmits SRS with repetition to ensure each TRP can accurately receive the SRS. In a word, at least 60% UE transmits SRS with repetition. In this case, the capacity of SRS can be increase to times of TD-OCC length. Because for one same CS in one comb, the orthogonal SRS resources is TD-OCC length compared with 1 using legacy method. The number of orthogonal SRS resources for TD-OCC is instead of without TD-OCC. Third, if TD-OCC is used, two SRS resources with partial overlapping frequency resources can occupy same comb, otherwise they have to occupy different combs. @Futurewei, Yes, they can be combined. The gNB first estimate the SRS on each OFDM symbol using legacy method, the SRS resources with different CS can be distinguished. Then the gNB determines channel with different TD-OCC indexes for one same CS. For example, as shown in Table1. In the first step, for CS 0, the gNB gets H121 = H1+H2 in the first OFDM symbol and H122 = H1-H2 in the second OFDM symbol, the gNB gets H1, H2 respectively in the second step, that is H1=(H121+H122)/2 and H2= (H121-H122)/2. Similarly, for CS 6, the gNB gets H341 = H3+H4 in the first OFDM symbol and H342 = H3-H4 in the second OFDM symbol, the gNB gets H3, H4respectively in the second step. Hi, i=1,2,3,4 is channel of port i. Table 1 SRS port index; comb; CS; TD-OCC. 1; 0; 0; [1,1]. 2; 0; 0; [1,-1]. 3; 0; 6; [1,1]. 4; 0; 6; [1,-1]. @ Google, LG, OPPO, Huawei, DOCOMO, QC, it is clear that the SRS capacity can be increased from to as discussed above. The key reason is that the orthogonal SRS increase to TD-OCC length times for each same CS in each comb. @ QC, if partial OFDM symbol drops, the TD-OCC length can be changed or the SRS resource with some new TD-OCC index can be dropped. Regardless which scheme is adopt, the SRS capacity is higher than legacy. @ OPPO, the CJT with repetition UE can be 60% in CJT case. @ Huawei, for SRS with different repetition, if their repetition is multiple of TD-OCC length, then they also can be multiplexed using TD-OCC, otherwise they can be with different CS or combs as legacy and SRS resources with same repetition can be with same CS in same comb and different TD-OCC. |
| Xiaomi | The scheme had been discussed and was not identified in Rel-17 SRS enhancement. The reason is that it needs more restriction conditions. e.g., more SRS symbol repetition, multiple SRS transmission of multiple users. Once a condition is not satisfied, TD-OCC cannot work. |
| FL | Proponents, please reply to the technical questions/issues from Google, QC, OPPO, Huawei, etc. The baseline for comparison with TD OCC may need clarification, e.g., the baseline uses less OFDM symbol or not. |
| MediaTek | We don’t see the benefit in introducing TD-OCC. As mentioned by multiple companies this require extra SRS symbols and ultimately may not help with capacity improvements. |
| QC | @ ZTE: Just to clarify the issue wrt SRS symbol dropping, assume that 2 UEs have TD-OCC [1 1] and [1 -1], and the first SRS symbol of the first UE drops (e.g. based on overlap with DL/other UL). Then does it mean that TD-OCC of UE2 needs to change (e.g. not apply TD-OCC) or does it mean one symbol of UE2 (second symbol) also needs to be dropped to maintain orthogonality? Obviously [0 1] and [1 -1] are no longer orthogonal. So, either both symbols of UE1 or the second symbol of UE2 need to be dropped in this case. |
| Lenovo | We think it is only used for multiple symbol repetition transmission. So it can be considered as an auxiliary SRS enhancement scheme. |
| Nokia/NSB | We are fine to discuss this more. |
| ZTE(2) | Thanks for moderator’s great suggestion. The baseline for comparison with TD OCC is SRS resource with same repetition OFDM symbols as legacy. @QC, Thanks for your further discussion. For the drop case you mentioned, we split it in such following cases Case 1, if the drop is based on UL which has same frequency structure as the dropped SRS, such as the UL is an aperiodical SRS or DMRS of type I, the gNB can get H1 of UE1 based on the UL and H2 of the SRS 2 based on SRS 2 in the first OFDM symbol if the UL is with different CS or different combs from SRS 2, otherwise if the UL is with same CS in same comb as the dropped SRS, the [1,1] also can be performed on [UL, SRS] in first OFDM symbol at UE 1 side, the SRS 1 and SRS 2 is orthogonal at UE side. In a word, the SRS 1 and SRS 2 is orthogonal in this case. Case 2, if the drop is based on UL which has different frequency structure from the dropped SRS, the UL and an SRS 2 of UE2 in first OFDM symbol is not orthogonal. It is a same case between UL and a SRS 3 of legacy UE 3. The gNB should avoid to schedule this case as much as possible. Case3, if the drop is based on DL, we believe the drop will happen at both UE1 and UE2 instead of just at UE1, otherwise there is strong interference between DL and UL at gNB side, then we suggest to let UE1 corresponding to [1,1] transmits on second symbol and UE2 drop SRS on both symbols , or allow the SRS with TD-OCC has higher priority in this case. |
| Ericsson | We agree with ZTE that the typical use case for CJT is UEs near the cell edge which generally requires SRS repetition. What TD-OCC provides is additional capacity improvement for such UEs, hence we think the baseline for comparison should be the CJT UEs which are at the cell edge, and which use SRS repetition. Also, it should be noted that CJT scenario will typically have larger delay spreads due to delay difference between TRPs. Hence, in the cases with large delay spreads, TD-OCC will provide better performance than using different cyclic shifts. Also, TD-OCC provides some important gains over frequency hopping. With frequency hopping, we get shorter sequence length which results in additional inter-cell interference due to correlated sequences for small SRS bandwidths. This can be alleviated when TD-OCC is used instead. With regards to Futurewei’s question, yes, cyclic-based CDM and TD-OCC can be combined for SRS in the same way that FD-OCC and TD-OCC can be combined for DM-RS. With regards to the issue pointed out by Qualcomm, it can be further studied on how to handle SRS symbol dropping with TD-OCC. We don’t see this as a show-stopper for further considering TD-OCC. |
| Sharp | We are fine to discuss it. |
| vivo | Open to discuss it as a capacity enhancement, but the benefit of TD-OCC should be further evaluated in CJT. |

### FL update

Props and cons as well as supporting companies are quickly summarized below based on the inputs and my understanding.

E: TD OCC (e.g., ZTE, Spreadtrum, Lenovo, OPPO, NEC, Intel, CMCC, Ericsson)

Pros: Capacity enhancement (for SRS with repetition); two sources show spectrum efficiency / channel estimation performance benefit

Cons: High overhead (compared to SRS without repetition); FFS when SRS is dropped on one of the OFDM symbols

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |
| OPPO | Maybe our input was misunderstood. We only propose to further evaluate TD-OCC to see whether it can be beneficial. |

## Increasing the maximum number of cyclic shifts

At least the following potential enhancements have been discussed:

F: Increasing the maximum number of cyclic shifts (e.g., Huawei, HiSilicon, LG Uplus, Spreadtrum, vivo, Google, Xiaomi, Futurewei)

F1: Multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts (e.g., Huawei, HiSilicon, LG Uplus)

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Futurewei [2] | Q2: May need to study the impact on minimum SRS sequence length. |
| Google | Support this enhancement. |
| QC | We do not see the need to increase the maximum number of cyclic shifts since delay spread in case of CJT is not going to be smaller than the case of single-TRP. |
| OPPO | Current SRS framework already supports maximal 12 cyclic shifts. It is difficult for gNB to fully exploit the 12 cyclic shifts due to the potential interference among ports with different CSs in multi-path channel, especially when the sequence length is not so large. We think it is meaningless to further increase the maximal number of CSs. |
| Huawei | The performance of SRS channel estimation via increasing the maximum # CSs directly is unacceptable (more sensitive to channel delay spread). Multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts can be considered. |
| Apple | We do not prefer this solution |
| DOCOMO | For CJT, the benefit of extending max. number of CSs would not be very clear. If the benefit is clarified, we would be open to consider. |
| Samsung | Same view as QC. This direction has been discussed in Rel-17 as well. Hence, we suggest to deprioritize and focus on discussing other proposals. |
| CMCC | We don’t prefer to increase the maximum number of cyclic shifts since the SRS in CJT might still be sensitive to delay spread. |
| Intel | Same view as QC and Samsung. It’s not necessary for CJT scenario. |
| ZTE | Same view as QC. |
| Xiaomi | Support this scheme due to its SRS capacity enhancement and interference refraining. |
| MediaTek | Same view as QC. |
| Lenovo | We have similar view with QC on whether the maximum CS value can be increased since delay spread in case of CJT is expected to be larger than the case of single-TRP. |
| Nokia/NSB | Agree with QC and Samsung. We prefer to depriorize this approach. |
| Ericsson | Agree with QC. Increasing the number of cyclic shifts leads to capacity enhancements only for channels with small delay spread, which is not typical for CJT where delay differences between TRPs may restrict the number of cyclic shifts that can be used. |
| Sharp | Agree with QC. |
| vivo | Same view as QC. |

### FL update

Props and cons as well as supporting companies are quickly summarized below based on the inputs and my understanding.

F: Increasing the maximum number of cyclic shifts (e.g., Huawei, HiSilicon, LG Uplus, Spreadtrum, vivo, Google, Xiaomi, Futurewei)

Pros: Capacity enhancement

Cons: Applicable only for cases with short enough delay spreads

F1: Multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts (e.g., Huawei, HiSilicon, LG Uplus)

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |

## Precoded SRS for DL CSI acquisition

At least the following potential enhancements have been discussed:

G: Precoded SRS for DL CSI acquisition (e.g., Huawei, HiSilicon, LG Uplus, ZTE, Nokia, Nokia Shanghai Bell, Sony, Futurewei)

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Google | We are open to study it. But is it correct understanding that the benefit is only for >= 4Tx UE? If yes, we suggest we deprioritize this. |
| QC | This should be deprioritized. With this, gNB has no longer access to the actual channel, and hence scheduling flexibility and ability to schedule MU-MIMO is seriously impacted. Also, this does not work with antenna switching xTyR (e.g., 1T4R), and even in other cases (x=y) the max rank for DL has to be semi-statically configured to something lower than UE capability, which is against the motivation of CJT (capacity enhancements for PDSCH). Otherwise, number of SRS ports become variable (if the rank decision is up to the UE and not RRC configured). Furthermore, this results in more overhead as for 4 TRPs, 4 additional CSI-RS resources would be needed. |
| LGE | Open to discuss. |
| OPPO | Precoded/beamformed SRS requires channel reciprocity at UE to derive the precoder/beam for the SRS. Furthermore, UE Tx/Rx antennae need to be calibrated to ensure the consistency between transmission and reception, which introduces significant complexity/requirement to UE. On the other hand, the precoding of SRS should be based on the rank for DL transmission. But without non-precoded SRS, how can gNB derive the precoder for CSI-RS and then transmit precoded CSI-RS for rank feedback from UE? If non-precoded SRS is needed anyway, then the SRS capacity cannot be increased actually. Also, If UE reports the precoder/beam of SRS to the gNB, additional UCI overhead is needed especially when the precoder is changing. |
| Huawei | For precoded SRS, how to perform the SRS precoding and how to obtain the DL CSI are described in [4] (for 4T4R and antenna switching, e.g. 2T4R). Evaluation results are also provided to illustrate the benefit. The benefit can be achieved if the rank of DL transmission is smaller than the number of antennas for UE. Under the MU-MIMO scenario, which is the target scenario for CJT, the PDSCH layer number is mainly 1 or 2, so the required SRS port number is relatively stable. We can configure a SRS resource with a fixed number of ports (e.g. 2) semi-persistently. The DL channel measurement can be conducted based on the CSI-RS resources configured in practical TDD systems (e.g. used for CQI measurement). We think precoded SRS deserves further study and discussion. |
| Apple | We do not prefer this solution |
| DOCOMO | We think QC’s point is valid. Also, the overhead to control/report SRS precoder seems to be quite large. Thus, we do not see strong motivation to move this forward now. |
| Samsung | We have also similar concerns as QC raised. |
| Sony | Positive to further investigation |
| Intel | Same view as other companies to deprioritize it. If SRS is pre-coded, then there is no channel reciprocity between DL and UL. |
| ZTE | @QC, First the performance loss in MU-MIMO case is small and can be ignored as shown in our contribution. We believe there is no the performance loss in SU-MIMO case. But precoding SRS allow the UE focus power on target direction. Second, in the case of x<y, the L precoding beams can be repeatedly transmitted by y/x times. Each SRS ports correspond to L precoded beams such that each precoded beam is transmitted by Y SRS ports, and each of the Y SRS ports corresponds to L precoded beams. For example, the signal model used is: . si for i = 1, 2, 3, 4 corresponds to the i-th precoded beams, i.e i-th beamformed SRS port. is 8 Rx ports at UE side. x=2, y=8, then the L precoded beams are repeatedly transmitted by y/x times. Third, the number of precoded SRS ports L can be DL rank or more than DL rank and less than or number of receiving antennas of UE. The L can be configured by gNB. @ OPPO, it is same as non codebook SRS.The UE with the ability to support SRS for non codebook, it also can support precoded SRS for antenna switching |
| Futurewei | @Google, OPPO, DOCOMO: The precoded SRS for AS could be very similar to SRS for NCB. It seems that your questions can be answered in the same way as for NCB. @QC: Our previous SLS results showed that precoded SRS can have significant DL throughput benefits (with some advanced designs as provided in Rel-17 SRS tdocs), even if the gNBs have only CSI of the precoded channel with only some or all UE antennas. The reason is because the gNBs have a large number of Tx/Rx antennas, which makes MU pairing relatively easy. In our view, precoded SRS for AS can be a necessary step for many advanced enhancements for TDD/FDD massive MIMO. |
| MediaTek | We propose to deprioritize this solution, agree with QC’s concerns. |
| QC | @ ZTE, Futurewei: Regarding performance loss in MU-MIMO, we were wondering what is the precoding used in the evaluation (for baseline and for precoded SRS)? Our understanding is that ZF (or any precoder with interference nulling toward co-scheduled UEs) can no longer be used with precoded SRS. In order to do interference nulling, the full channel matrix to all UEs are needed, and also the rank can no longer be decided by the UE as it depends on other UEs’ channel. Also, what is the SRS periodicity, CSI-RS periodicity (to obtain precoding) and number of co-scheduled UEs? @ ZTE: For x=1 and y=4 (1T4R), can you please clarify how SRS overhead reduction is achieved? Let’s assume DL rank L=1 in this case. Similarly, for the example you mentioned above (2T8R), it seems that even when DL rank is 1, 4 SRS transmissions are still needed. More importantly, precoding cannot be across more than 2 SRS ports. Is that correct understanding? In other words, a “virtual SRS port” can only be from 2 actual antennas (as UE cannot transmit from more than 2 antennas). If that’s the case, what is the performance loss in this case as a result of not being able to precode over 8 Tx antennas? @ Futurewei: For NCB-based UL with associated CSI-RS, we do not think UE is mandated to calibrate Tx/Rx chains. UE may choose to do so, but the point is that it is not required. For example, UE may only do antennas selection or only do SRS port virtualization based on CSI-RS. Calibrating Tx/Rx chains is very difficult for UE. This is not an issue for NCB-based PUSCH as the SRS is measured by gNB based on which one or more SRS resources are indicated to the UE for PUSCH transmission. |
| Lenovo | We think multiple CSI-RS resources have to be configured although this scheme is for TDD system and this will increase CSI-RS overhead. Also, our preference is to defer the discussion on this point until the CSI-RS association per TRP for CJT is decided in CSI agenda 9.1.2 |
| Nokia/NSB | We have fine to discuss this further. We think that achievable gains (e.g. coverage and capacity) are not limited to >= 4Tx UE. Since resource configurations for precoded and non-coded UL SRS are different, network needs to anyhow indicate whether the UE applies precoded or non-precoded UL SRS transmission. In other words, the gNB can control whether it has access to actual channel “plain” or precoded channel state information. Hence, we do not see this as restriction for scheduling. We think that the use of precoded UL SRS is not limited to symmetrical antenna switching configurations, but can be also applied with non-symmetric ones. |
| ZTE(2) | @ QC, thanks for your further discussion. First for the MU-MIMO, the precoding of DL is based on SLNR for both non precoding SRS and precoding SRS. The precoded SRS just delete the U of the H, the gNB can get the majority component of D*V of the H based on the precoded SRS. So it has similar performance as non precoded SRS. Second for the case x=1, y=4, L=1, the overhead doesn’t reduce, but the power is boosted. The 1 layer is repeated transmitted 4 times. The gNB can add the channel of the 4 transmission occasion together because the four transmission occasion corresponds to the same channel of the 1 precoded beam. The power is boosted, then the repetition for each x is reduced, so we can also say the overhead is reduced. Third, in my opinion, the precoding is across more than 2 SRS ports. Each precoded beam is transmitted by the 8 Tx which is similar x=y. The only difference is that the y ports are transmitted in y/x time units respectively. When the gNB add the channel in y/x time units together for each precoded SRS port, it is similar as that the channel of each Precoded beam are from y ports. For example, if the y ports are transmitted simultaneously, then the received signal of receiving antenna j at gNB side is . If the y ports are transmitted in 4 times, then is the signal received by gNB at receiving antenna j and time unit t. If the gNB add the signal at the 4 times, then . In a word, the sum of received signal in the 4 time unit is same as the received signal in the case the y ports are transmitted simultaneously in one time unit. |
| Huawei, HiSilicon2 | Thanks for further discussion. @DOCOMO: If the gNB only needs to obtain several eigenvectors of the DL channel matrix, To further obtain full-channel information, the feedback is not required as shown in our contribution. @Intel: The precoded SRS actually utilizes the reciprocity to obtain the DL precoding matrix information. Hope the detailed formula in our contribution can solve your concern. @QC: ZF or eigenvector-based ZF can still be performed based on precoded SRS with negligible performance loss. As you mentioned, as long as SRS port virtualization based on CSI-RS is a candidate option for NCB, arguing NCB and precoded SRS need different calibration capability seems meaningless. |
| Ericsson | This scheme does not seem to work for UEs with 1 Tx. Also, how does it work for UEs with 2 Tx with 1 antenna per polarization? In these cases, precoding of SRS does not seem possible. Furthermore, precoded SRS does not give gNB full channel knowledge. Hence, we suggest deprioritizing this enhancement. |
| Sharp | We prefer to deprioritize this solution. |
| vivo | When to obtain the DL CSI, beamformer should be reported to the network, then additional uplink resource would be occupied. From this perspective, it seems this scheme is against the allocation of SRS. |

### FL update

Props and cons as well as supporting companies are quickly summarized below based on the inputs and my understanding.

G: Precoded SRS for DL CSI acquisition (e.g., Huawei, HiSilicon, LG Uplus, ZTE, Nokia, Nokia Shanghai Bell, Sony, Futurewei)

Pros: Capacity enhancement; one source shows throughput performance benefit, and one source shows power reduction benefit

Cons: NW does not have full MIMO channel information; UE Tx/Rx calibration complexity

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |

## Enhanced signaling for flexible SRS transmission

At least the following potential enhancements have been discussed:

H: Enhanced signaling for flexible SRS transmission, e.g., dynamic update of SRS parameters (e.g., Interdigital, Intel, Xiaomi, Samsung, CMCC, Qualcomm, NTT DOCOMO, Sharp, Ericsson, Futurewei)

E.g, frequency-domain parameter (e.g., BW change, comb change and/or hopping location change), power control parameter, activation by DCI for SP SRS, RPFS parameters, etc.

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Futurewei [2] | Q2: May require higher signaling overhead. |
| Google | We suggest we make a more specific proposal, e.g. what parameters to be updated. This proposal is too broad. |
| LGE | Open to discuss. |
| OPPO | It is unclear to us which SRS parameters need to be further dynamically updated. In NR, the SRS resource set with different parameters can be dynamically triggered by DCI. Dynamic scheduling offset, which was introduced in Rel-17, can also provide some flexibility. It is unclear whether further flexibility is needed and would be actually adopted at gNB. |
| Apple | We are open to discuss. But the scope needs to be clearer. |
| DOCOMO | As a proponent, we are open to a specific parameter to be changed while we think frequency-domain parameter (e.g., BW change, comb change and/or hopping location change) can be simpler and more beneficial slightly. We assume there might be difference on implementation impact on different parameters. Thus, we think it would be good if we can select some parameter(s) which do not cause so larger implementation impact. Re OPPO’s comment, we think it would be ok to narrow down the scope within P-SRS or SP-SRS only. If DCI overhead is allowed, using AP-SRS indeed makes some sense. Meanwhile, we think P-SRS/SP-SRS itself have good benefits due to less control overhead. So we think the enhancement of P-/SP-SRS parameter update is a good candidate in Rel-18. |
| Samsung | We are open to discuss, but the current proposal is too broad and needs to down-scope. |
| Spreadtrum | Open to discuss |
| CMCC | We are open to consider this. |
| InterDigital | We support this enhancement to avoid high SRS configuration overhead, and instead enable a more dynamic triggering/updating of P/SP/AP SRS resources with different parameters (e.g., for interference randomization, or power control). |
| Intel | Open to discuss. But the proposal is too broad. |
| ZTE | We are open to consider this. |
| FL | @NTT DOCOMO: Thank you for the detailed reply. I think what you described could be a good starting point. Based on contribution review, it seems there are other proposals of the parameters, e.g., power control parameters (Samsung), DCI for SP SRS activation (Qualcomm), RPFS parameters (Xiaomi), etc. Proponents please further describe. Updated the description accordingly. |
| MediaTek | We are open to discuss this further. |
| Lenovo | We are open for discussion. But the content for signalling indication needs to be clarified. Also, it is necessary to consider the impact of increased signalling overhead for PDCCH reliability for CJT UEs, which are likely to be cell edge UEs. |
| Nokia/NSB | For us this is further optimization. However, we are fine to discuss this further. |
| Ericsson | We are positive to considering this enhancement further. We suggest adding the following enhancement under this topic: “Signalling for adapting the number of UE antennas that are sounded for antenna switching”. SRS resources for xTyR antenna switching, where x<y, consumes significant UL resources. This enhancement would allow faster than RRC switching between different supported xTyR antenna switching configurations to, e.g., reduce UL overhead for high traffic loads. We are also supportive of considering dynamic update of frequency-domain parameters (e.g., SRS bandwidth configuration). |
| Sharp | We are open to discuss this further. |
| DOCOMO | For Ericsson’s additional suggestion, we are fine. |
| vivo | In Rel-16, uplink cancellation indication (CI) in DCI format 2_4 has been introduced for PUSCH and SRS cancellation/muting for URLLC. Therefore, for Rel-18 SRS enhancement, enhanced CI mechanism/signaling can be considered as a potential way to handle the across-SRS interference as shown in our tdoc. Therefore, as an enhanced signaling enhancements, we prefer to add the enhanced cancellation indication of SRS transmission in this issue as follows. At least the following potential enhancements have been discussed: H: Enhanced signaling for flexible SRS transmission, e.g., dynamic update of SRS parameters (e.g., Interdigital, Intel, Xiaomi, Samsung, CMCC, Qualcomm, NTT DOCOMO, Sharp, Ericsson, Futurewei), enhanced cancellation indication (CI) of SRS transmission (vivo) E.g, frequency-domain parameter (e.g., BW change, comb change and/or hopping location change), power control parameter, activation by DCI for SP SRS, RPFS parameters, cancellation indication in DCI format 2_4, etc. |

### FL update

@Ericsson: Your suggestion is added as an example.

@ vivo: Your suggestion is added as an example. (The main bullet is not changed; it is better to keep it as is.)

Props and cons as well as supporting companies are quickly summarized below based on the inputs and my understanding.

H: Enhanced signaling for flexible SRS transmission, e.g., dynamic update of SRS parameters (e.g., Interdigital, Intel, Xiaomi, Samsung, CMCC, Qualcomm, NTT DOCOMO, Sharp, Ericsson, Futurewei, vivo,)

E.g, frequency-domain parameter (e.g., BW change, comb change and/or hopping location change), power control parameter, activation by DCI for SP SRS, RPFS parameters, number of antennas in antenna switching, cancellation indication in DCI format 2_4, etc.

Pros: Flexible SRS for interference randomization

Cons: Higher signaling overhead

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |

## Partial frequency sounding extensions

At least the following potential enhancements have been discussed:

I: Partial frequency sounding extensions (e.g., ZTE, Spreadtrum, Intel, Xiaomi, NTT DOCOMO, Futurewei)

I1: Larger partial frequency sounding factor (e.g., ZTE, Spreadtrum, Xiaomi, NTT DOCOMO, Futurewei)

I2: Starting RB location hopping enhancements (e.g., Futurewei)

I3: Partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS (e.g., ZTE)

I4: Dynamic changing partial sounding parameters (e.g., Intel, ZTE)

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Futurewei [2] | Q2: 1) For these enhancements, with the higher randomness introduced, previously orthogonal FDMed SRSs may become non-orthogonal, unless some additional design or network coordination is used. For example, FDMed SRSs may need to be configured with the same hopping / randomization patterns. 2) For I1, may need to study the impact on minimum SRS sequence length. 3) The potential enhancements (except for probably I1) may lead to non-uniform SRS sample pattern in time/frequency domain, which may impact the SRS-based channel estimation design / complexity / performance. |
| Google | Partial frequency sounding was studied in R17. We suggest we deprioritize this proposal to avoid repeat discussion. |
| QC | Same view as Google. |
| LGE | Lager value of Pf and dynamic indication of RPFS parameters can be studied with less specification impact. |
| OPPO | Agree with google to deprioritize this option. The partial frequency hopping introduced in Rel-17 is sufficient for most target scenarios. |
| Huawei | Same view as Google. |
| Apple | We do not support this solution. In our view, RPFS was not a good design in Rel-17 and we should stop working on it any further. |
| DOCOMO | Not sure why de-prioritize because of Rel-17 discussion is needed. We view another alternative, especially in frequency domain, requires a certain impact on specification. Meanwhile, the needed specification impact by e.g., I1 is easily expected, which is very reasonable. |
| Samsung | Same view as Google. RPFS is extensively discussed in Rel-17, hence we suggest to focus on discussing other proposals. |
| Spreadtrum | Open to discuss. The specification efforts are limited for I1/I2 based on Rel-17 RPFS. |
| CMCC | Agree with Google, current RPFS design in R17 is enough. |
| Intel | Support I4. |
| ZTE | Support to discuss it. |
| Xiaomi | Support I1 and I2. I1 is not only used to improve SRS capacity, but also randomize SRS interference by using I2, i.e., starting RB location hopping enhancements. |
| FL | It would be appreciated if opponents can provide technical questions on what needs to be clarified/addressed by the proponents. As of now, it does not seem the opponents described any significant technical issue for RPFS. |
| MediaTek | We propose to deprioritize, as mentioned by Google this has been already looked at in R17. |
| Lenovo | Same view as Google, QC, Huawei. |
| Nokia/NSB | Share same view as QC and Google. |
| Ericsson | We propose to deprioritize this proposal. Due to a limited amount of valid SRS sequence lengths, it is not possible to decrease the per-hop SRS bandwidth any further. Hence, I1 and I3 does not provide capacity gains. Furthermore, changing the hopping pattern as in I2 makes it challenging to co-schedule legacy UEs. Finally, dynamic update of frequency-domain parameters (including partial sounding, I4) can be studied under Proposal 3.8. |
| Sharp | Same view as Google |
| vivo | Share the same view as Google. |

### FL update

There is not sufficient technical discussion. In my view, potentially there could be some motivations in terms of interference randomization and/or capacity enhancements as discussed in Rel-17, but how much further benefit can be achieved in Rel-18 is not clear yet. Further discussion is needed.

I: Partial frequency sounding extensions (e.g., ZTE, Spreadtrum, Intel, Xiaomi, NTT DOCOMO, Futurewei)

I1: Larger partial frequency sounding factor (e.g., ZTE, Spreadtrum, Xiaomi, NTT DOCOMO, Futurewei, LGE)

I2: Starting RB location hopping enhancements (e.g., Futurewei, Xiaomi)

I3: Partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS (e.g., ZTE)

I4: Dynamic changing partial sounding parameters (e.g., Intel, ZTE, LGE)

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |
| LGE | Incorrectly captured our view is revised. |

## Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

At least the following potential enhancements have been discussed:

J: Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment (e.g., Samsung, Qualcomm, Futurewei)

J1: Configuration of v (sequence index within a group) per SRS resource (e.g., Qualcomm)

J2: Configuration of cyclic shift per SRS port per SRS resource (e.g., Samsung, Qualcomm, Futurewei)

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Futurewei [2] | Q2: For J1, can it achieve more interference randomization than sequence hopping? |
| Google | We would like to understand the benefit for this proposal. If this is for interference randomization, we suggest to move them into 3.1/3.2/3.3. |
| QC | Both enhancements are beneficial and very simple. This is unrelated to randomization. For J1, the benefit is enhancing the reuse factor of SRS sequence (more SRS sequences can be configured by the network to ensure that 2 UEs with the same SRS sequence are far away and do not clear inter-cell / inter-cluster interference). For J2, the benefit is more optimized allocation of cyclic shifts considering intra-UE ports and inter-UE ports. Please see the evaluation results showing the benefit in our contribution. |
| OPPO | In current SRS framework, configuration of u per resource can already provide the functionality of interference randomization. Also, the cyclic shift is already generated per SRS port based on fixed offset, which is sufficient for port multiplexing. Backward compatibility issue needs to be considered, e.g. multiplexing with legacy Ues. |
| Apple | We do not prefer this solution |
| DOCOMO | We think the benefit of J1/J2 is small, and not beyond the control overhead. |
| Samsung | We support J2. In order to deal with different delay from UE to each TRP, equi-distant CS allocation based on the maximum CS for each port as in current specification is not beneficial. |
| CMCC | We think the benefit of this J1/J2 is limited. For J2, it might cause the cyclic shift b/w different ports is not uniform, which might increase the channel estimation complexity. |
| Intel | Same view as OPPO. This could be deprioritized. |
| ZTE | We don’t see the benefit of J For J1, Enable the sequence hopping can get the same gain as J1. For J2, we don’t understand why J2 can get the additional gain compared with legacy configuration. First J2 leads one SRS resource can not be suitable bot for SU-schedule case and MU-schedule case. Second , J2 can not bring additional gain considering CS is one form of FD-OCC and the multiple SRS resources from UEs in different TRP areas arrive each target TRP in CP rang. |
| Xiaomi | We can study it with low priority. |
| Futurewei | @Qualcomm: Thanks for replying to us on J1. It is now understood. So the reuse factor is increased from 30 to 60, which should be beneficial. |
| MediaTek | We propose to deprioritize. |
| QC | @OPPO: There is no backward compatibility issue as the proposal is to configure cyclic shift per SRS port. When legacy UEs are multiplexed, the occupied cyclic shifts are simply not configured for the Rel-18 UE. @Docomo: Can you please explain a bit more on “control overhead”? The proposal is related to RRC configuration of v (o or 1) or cyclic shift per port (a vector equal to # of SRS ports). @CMCC: In our understanding (as well as in our evaluation), gNB can still estimate different ports separately (same as legacy). The only knowledge needed is cyclic shift per port. Can you please explain a bit more on the complexity issue? @ZTE: On J1, sequence hopping has a different purpose (randomization). This enhancement is to increase the reuse factor of SRS sequence. On J2, why the same SRS resource cannot be used for both MU and SU scheduling? Even with SU scheduling, multiple UEs may send SRS in the same REs using different cyclic shifts. We also did not understand the second point regarding J2. I think the scenario you mentioned is exactly what is simulated in our Tdoc. |
| Lenovo | For J2, it may reduce CS spacing between SRS ports and the impact on SRS channel estimation performance needs to be considered especially for channels with larger delay spread. |
| ZTE(2) | @ QC, On J2, the best CS gap for SU is in my opinion. In my opinion, if the transmission delay gap of the multiple SRS resources from UEs in different TRP areas is in CP rang at each TRP, then the CS can be viewed as FD-OCC, the FD-OCC length is same for cyclic shift [0,3,6,9] regardless which cyclic shift allocation scheme is adopted. Please provide more clarification/explanation for the gain of J2. Thanks. |
| Ericsson | We think the benefit of J1/J2 is limited, but we are open to study further with low priority. |
| DOCOMO | Our point is similar to Ericsson, i.e., the benefit is limited which may not deserve DCI overhead. Studying J1/J2 is ok, but it should be lower priority in our view. |
| vivo | We think this could be deprioritized. |

### FL update

There is some discussions on the potential gains over existing mechanisms. Qualcomm and Samsung provided evaluation results to justify the gains for J2, but some companies still questioned the effectiveness. Further discussion is needed, such as more analysis of the evaluation results.

J: Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment (e.g., Samsung, Qualcomm, Futurewei)

J1: Configuration of v (sequence index within a group) per SRS resource (e.g., Qualcomm)

Pros: Improved reused factor

J2: Configuration of cyclic shift per SRS port per SRS resource (e.g., Samsung, Qualcomm, Futurewei)

Pros: Optimized CS allocation; two sources show channel estimation performance benefit

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |
|  | [For presentation during offline] The per-port cyclic shift assignment provides benefit when UEs have different delay spread (DS) or different Rx timing (different fixed delay, or different arrival time at a given intended receiver). |

## Resource mapping for SRS transmission based on network-provided parameters or system parameters

At least the following potential enhancements have been discussed:

K: Resource mapping for SRS transmission based on network-provided parameters or system parameters (e.g., ZTE, Spreadtrum, Lenovo, NTT DOCOMO, Futurewei)

K1: Based on network-provided parameters, e.g., network-provided ID (e.g., Spreadtrum, Lenovo, NTT DOCOMO, Futurewei)

E.g., based on network-provided ID (different from cell ID) at least for nIDSRS, which may also be related to TRP IDs of the CJT network

K2: Based on system parameters, e.g., time indexes (e.g., ZTE, Lenovo, NTT DOCOMO, Futurewei)

E.g., based on OFDM symbol index, slot index, radio frame index, etc.

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Google | We would like to understand the benefit for this proposal. If this is for interference randomization, we suggest to move them into 3.1/3.2/3.3. |
| QC | This proposal (and the benefit) is still very unclear to us (even after the discussions in the previous meeting and after reading the contributions for this meeting). Some examples in illustrating the proposal (and the benefit) would help a lot. |
| OPPO | The solution is still unclear to us. For example, how to apply the network-provided ID, for resource mapping or sequence generation? |
| Apple | We do not prefer this solution |
| DOCOMO | One thing we are considering would be to consider such identity for sequence determination. SRS sequence is determined considering quasi-cell ID to avoid ICI. Consuming the cell ID within a cell might make ICI mitigation more difficult. In this case, additional factor for sequence determination (e.g., number of coherent TRPs, TRP identify, etc) can be introduced. |
| Samsung | The statement of this proposal is unclear to us, and looks broad. We prefer to deprioritize this proposal. |
| Intel | The benefit is not clear. |
| ZTE | We support to discuss it. Specifically, there are following examples: Example1: the frame index can be included in the group hopping /sequence hopping to avoid collision pattern being same in each frame. Example2: the can be changed in different time unit to avoid collision pattern being same in each time unit. The time unit can be slot/frame. |
| Xiaomi | The details of the proposal should be provided more for clarifying the scheme. |
| FL | Updated the description based on DOCOMO/ZTE’s replies and some tdocs. Hope this is clearer. |
| MediaTek | We don’t support this proposal. |
| QC | @ ZTE: Thanks for the two examples. In example 1, the purpose is basically to reset initialization every multiple frames (rather than current spec, which is every frame). If that is the intention, we think the scheme / enhancement is clear, and we would be open to consider this if benefit is shown. For example 2, it is still not very clear. In current sequence / group hopping, is used for initialization of the pseudo-ransom sequence c(i) which is used for randomization. Is the proposal that on top of this, also randomize the initialization? Then, a second initialization is needed to randomize the first initialization. The benefit of this is not clear to us. |
| Lenovo | We prefer to discuss this issue together with interference randomization since some coordination on hopping parameters can achieve tradeoff between tight coordination for interference avoidance and interference randomization. |
| Nokia/NSB | The current form of the proposal remains unclear for us. Further clarification is needed. |
| ZTE(2) | @QC, Thanks for your comments. For example 2, the SRS 1 are configured with multiple and the SRS 2 are configured with multiple , the of each SRS can be selected from the multiple according to period index, then it avoids the collision pattern of SRS 1 and SRS 2 is same in each period which can be one or more frames. |
| Ericsson | The benefit is unclear, we prefer to deprioritize this proposal. |
| vivo | This enhancement is not very clear for us. Base on this, would Rel-18 UE still collide with each other? |

### FL update

For these candidate schemes, proponents provided more examples on how they may work, which was helpful, but some companies still think more clarifications are needed. Furthermore, some companies suggested to discuss this together with interference randomization candidate schemes, which seems reasonable. The group can have more discussions.

K: Resource mapping for SRS transmission based on network-provided parameters or system parameters (e.g., ZTE, Spreadtrum, Lenovo, NTT DOCOMO, Futurewei)

K1: Based on network-provided parameters, e.g., network-provided ID (e.g., Spreadtrum, Lenovo, NTT DOCOMO, Futurewei, ZTE)

E.g., based on network-provided ID (different from cell ID) at least for nIDSRS, which may also be related to TRP IDs of the CJT network

Pros: Interference randomization

K2: Based on system parameters, e.g., time indexes (e.g., ZTE, Lenovo, NTT DOCOMO, Futurewei)

E.g., based on OFDM symbol index, slot index, radio frame index, etc.

Pros: Interference randomization

Your further inputs are welcome, especially if you think there is something missing from my summary or anything you would like to further emphasize (such as some point not brought up above or unclear from other companies).

| Company | View |
| --- | --- |

## Others

Any other views, issues, potential enhancements, and clarifications, if any, can be provided in below table.

| Company | View |
| --- | --- |

# SRS enhancements targeting 8 Tx operation

It is well known that increasing UE Tx antenna ports can significantly improve various performance metrics for UL/DL transmissions. 8 Tx transmissions can be feasible for at least CPE/FWA/vehicle/industrial devices and hence can be beneficial.

## Whether to support 8 ports in one or multiple resources for 8 Tx SRS

The following has been discussed on whether to support 8 ports in one or multiple resources for 8 Tx SRS:

Codebook and antenna switching with 8T8R

Support 8 ports in 1 resource (e.g., ZTE, Spreadtrum, OPPO, Intel, Xiaomi, Samsung, CMCC, CEWiT, Qualcomm, Apple, Ericsson, Nokia, Nokia Shanghai Bell, Futurewei)

Support 8 ports in multiple resources (e.g., Huawei, HiSilicon, LG Uplus, vivo, Lenovo, CATT, KDDI, Samsung, NTT DOCOMO, Ericsson)

Non-codebook

Support 8 ports in 8 resources, each with 1 port (e.g., ZTE, Spreadtrum, Samsung, CMCC, CEWiT, Futurewei)

Support 8 ports in 4 resources, each with 2 ports (e.g., LGE)

It seems that most companies support 8 ports in 1 resource for CB/AS, and 8 ports in 8 resources (each with 1 port) for NCB. We suggest adopting this design, while other designs can still be discussed.

Proposal 4.1: For 8 Tx SRS, at least support

8 ports in 1 SRS resource for ‘codebook’ / ‘antennaSwitching’;

FFS 8 ports in multiple SRS resources for ‘codebook’ / ‘antennaSwitching’

8 ports in 8 SRS resources for ‘nonCodebook’ (each SRS resource with 1 port).

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Google | Support |
| OPPO | We are fine with proposal 4.1 in principle. For nonCodebook, it would be clearer if we can clarify that the 8 resources are within one set: 8 ports in 8 SRS resources in one SRS resource set for ‘nonCodebook’ (each SRS resource with 1 port). |
| Huawei | For ‘codebook’ / ‘antennaSwitching’, both “8 ports in 1 SRS resource” and “8 ports in multiple SRS resources” should be treated equivalently. |
| Apple | We support the proposal |
| DOCOMO | Support |
| Samsung | Support |
| Spreadtrum | Support |
| CMCC | Support second bullet for non-codebook. For codebook, how to realize 8 ports in 1 SRS resource or 8 ports in multiple SRS resources may be discussed firstly, based on the details we could further identify or compare these two mechanisms. |
| InterDigital | Support |
| Intel | Generally fine with FL proposal |
| ZTE | Support the proposal in principle except that for the second bullet. We support up to 8 SRS resources, not only 8 SRS resources. So we suggest following The updated Proposal 4.1: For 8 Tx SRS, at least support - 8 ports in 1 SRS resource for ‘codebook’ / ‘antennaSwitching’; - FFS 8 ports in multiple SRS resources for ‘codebook’ / ‘antennaSwitching’ - up to 8 ports in up to 8 SRS resources for ‘nonCodebook’ (each SRS resource with 1 port) and each SRS resource with one SRS port. |
| Xiaomi | Fine with the proposal |
| MediaTek | Fine with the proposal |
| Lenovo | Support |
| Nokia/NSB | Support |

### FL update

So far most companies are fine with this. OPPO’s suggestion and ZTE’s suggestion are good and the proposal is updated below.

@CMCC: Based on the contributions, how to realize 8 ports in 1 or multiple SRS resources has been provided and either way seems feasible / not difficult. Therefore, we can see if we can make a high-level agreement and then proceed to decide the details.

Please continue to discuss.

Proposal 4.1-1: For 8 Tx SRS, at least support

8 ports in 1 SRS resource for ‘codebook’ / ‘antennaSwitching’;

FFS 8 ports in multiple SRS resources for ‘codebook’ / ‘antennaSwitching’

Up to 8 ports in up to 8 SRS resources in one SRS resource set for ‘nonCodebook’ (each SRS resource with 1 port).

| Company | View |
| --- | --- |
| Nokia/NSB | The second bullet need more discussion. |
| Lenovo | As discussed in our contribution, in some case, all 8 resources cannot be transmitted within one slot. Then more than one SRS resource set may be required. We suggest the following change: Proposal 4.1-1: For 8 Tx SRS, at least support 8 ports in 1 SRS resource for ‘codebook’ / ‘antennaSwitching’; FFS 8 ports in multiple SRS resources for ‘codebook’ / ‘antennaSwitching’ Up to 8 ports in up to 8 SRS resources in one SRS resource set for ‘nonCodebook’ (each SRS resource with 1 port) can be configured for a UE in a BWP of a serving cell. |
| CEWiT | Support |
| Ericsson | Support the first bullet. The second bullet need more discussion. In AI 9.1.4.2 (FL proposal 3.2), it is being discussed if up to two SRS resource sets will be supported for ‘nonCodebook’. |
| NEC | Support the first bullet, and we also think second bullet needs further discussion, and Lenovo’s proposal seems fine. |
| CATT | Support the second bullet. For codebook/antennaSwitching, we support both “8 ports in 1 SRS resource” and “8 ports in multiple SRS resources”. |
| KDDI | In the first bullet, “8 ports in 1 SRS resource” and “8 ports in multiple SRS resources” should be treated equally. We think that both of them have spec impact and pros and cons. |
| vivo | Support the first bullet. In the second bullet, whether in one or multiple SRS sets needs more discussion. |

### FL update 2

Some companies do not think we can limit the number of SRS resource sets now for NCB, which is reasonable as we can make a separate decision on that. We should note that for a non-codebook based PUSCH configured in a BWP, up to 8 SRS resources with ‘nonCodebook’ can be configured. The proposal is updated as follows.

Proposal 4.1-2: For 8 Tx SRS, at least support

8 ports in 1 SRS resource for ‘codebook’ / ‘antennaSwitching’;

FFS 8 ports in multiple SRS resources for ‘codebook’ / ‘antennaSwitching’

Up to 8 SRS resources for ‘nonCodebook’ (each SRS resource with 1 port) can be configured for a non-codebook based PUSCH in a BWP of a serving cell.

| Company | View |
| --- | --- |
| CATT | For the first bullet, we prefer “8 ports in 1 SRS resource” and “8 ports in multiple SRS resources” be treated equally. There are several pros of supporting 8 ports in multiple SRS resources, e.g. current 2-port/4-port SRS resource can be reused; enabling 8 ports by FDM/TDM/CDM can be flexibly supported. |
| ZTE | For the second bullet, we support multiple sets can be configured for different time domain behaviors to improve gNB scheduling flexibility. So we suggest following updated Proposal 4.1-2 Updated Proposal 4.1-2: For 8 Tx SRS, at least support - 8 ports in 1 SRS resource for ‘codebook’ / ‘antennaSwitching’; - FFS 8 ports in multiple SRS resources for ‘codebook’ / ‘antennaSwitching’ - Up to 8 SRS resources for ‘nonCodebook’ (each SRS resource with 1 port) can be configured for a non-codebook based PUSCH in a BWP of a serving cell for each time domain behavior. |

### Online agreement

Proposal 4.1-2:

For 8 Tx SRS, at least support

8 ports in 1 SRS resource for ‘antennaSwitching’;

FFS 8 ports in one or multiple SRS resources for ‘codebook’

Above does not imply support for 8 ports in one or multiple OFDM symbols

## Whether to support 8 ports in one or multiple OFDM symbols for 8 Tx SRS

The following has been discussed on whether to support 8 ports in one or multiple OFDM symbols for 8 Tx SRS:

Support 8 ports in 1 OFDM symbol (e.g., Interdigital, OPPO, CATT, Intel, Samsung, LGE, CMCC, Qualcomm, NTT DOCOMO, Sharp, Futurewei)

Support 8 ports in multiple OFDM symbols (e.g., Interdigital, Huawei, HiSilicon, LG Uplus, ZTE, Spreadtrum, OPPO, CATT, Intel, Samsung, LGE, CMCC, CEWiT, Qualcomm, NTT DOCOMO, Sharp)

It seems that there is large support for 8 ports in both 1 and multiple OFDM symbols. We suggest adopting both designs. Further details on how to support 8 ports in 1 OFDM symbol and in multiple OFDM symbols will be discussed.

Proposal 4.2: For 8 Tx SRS, support

8 ports in one OFDM symbol;

8 ports in multiple OFDM symbols.

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Google | We think 8 ports in one symbol should be sufficient. To support both looks redundant. |
| LGE | Fine in principle, and prefer to revise as follows for clarity. Proposal 4.2: For 8 Tx SRS, support 8 ports in one OFDM symbol; 8 ports multiplexed across multiple OFDM symbols. |
| OPPO | Support. We think supporting 8 ports in multiple symbols can provide some benefits for power limited UE. |
| Huawei | Support. |
| Apple | We prefer to support 8 ports in one OFDM symbol. FFS 8 ports in multiple OFDM symbols. |
| DOCOMO | Agree with Google that supporting both seems to be redundant. We prefer to support 8-port SRS in 1 OFDM symbol only. We think 8Tx can be implemented mainly for a specific type of UEs, e.g., FWA. Thus we do not think power limitation is a significant issue. If power limitation for 8-port SRS is a big issue, then 8Tx UL itself is not very realistic. Such a scenario is not considered here. |
| Samsung | We support this proposal |
| Spreadtrum | Support |
| CMCC | Support 8 ports in multiple OFDM symbols, it is benefit for cell edge UEs, which can sound the 8 ports in multiple OFDM symbols, with guaranteed sounding quality. But this should be further clarified whether 1 SRS resource with 8 ports in multiple OFDM symbols or multiple SRS resources with 8 ports in multiple OFDM symbols. |
| InterDigital | Support |
| Intel | Prefer to support 8-ports in one OFDM symbols. Supporting 8-ports in multiple OFDM symbols requires more uplink resource. Considering uplink symbols are limited in TDD, it’s not preferred to support 8-ports in multiple symbols. |
| ZTE | Support Especially the second bullet should be supported considering UE power and flexibility of gNB scheduling. |
| Xiaomi | Support the proposal |
| MediaTek | Support |
| Lenovo | We want to clarify the second bullet: Does 8 ports in multiple OFDM symbols mean that 8 ports are sounded by multiple SRS resources or one SRS resource occupies multiple OFDM symbols? |
| Nokia/NSB | Share the same view as Apple. |

### FL update

So far most companies are fine with at least one of the bullets. From the inputs above and the tdocs, it seems both designs have large support and hence down-selection may not be possible. For now, let’s keep both.

LGE’s suggestion is good and the proposal is updated below.

@CMCC: Whether the 8 ports are in 1 or multiple resources can be separately decided in Sec. 4.1, which can help decouple the two design parameters here and facilitate the decision process. Anyway you are correct that these are related, and we can discuss them jointly if we identify the need to do so.

Please continue to discuss.

Proposal 4.2-1: For 8 Tx SRS, support

8 ports in one OFDM symbol;

8 ports multiplexed across multiple OFDM symbols.

| Company | View |
| --- | --- |
| Huawei, HiSilicon2 | Support |
| QC | We support the proposal in general. We just have some confusion why in the second bullet the wording “multiplexed” is used. To me, multiplexing sounds something different such as two channels of PUCCH/PUSCH multiplexed together. We suggest revise the wording “multiplexed” to “transmitted”, or something else, or revert back to original FL proposal. |
| Lenovo | Please check our comment on the original Proposal 4.2. We understand this proposal targets SRS for codebook. We suggest to add a FFS on the second sub-bullet with the following update: Proposal 4.2-1: For 8 Tx SRS for codebook, support 8 ports in one OFDM symbol; 8 ports multiplexed across multiple OFDM symbols. FFS: multiple OFDM symbols correspond to one or more SRS resources. |
| CEWiT | Support |
| Ericsson | Support the first bullet. We should strive for SRS to occupy as few UL resources (OFDM symbols) as possible. Existing coverage enhancement schemes (e.g., repetition and/or frequency hopping) can be used to improve cell-edge performance also for 8 Tx UEs. |
| NEC | We think the first bullet is sufficient. |
| CATT | Support the original version. For the updated version, we also think “multiplexed” is not clear. |
| KDDI | We support both. We think that changing the number of OFDM symbols used in SRS to match the UE situation will improve uplink coverage and allow for more efficient resource allocation. |
| vivo | One solution is enough, and we prefer 8 ports in one OFDM symbol. Regarding the second bullet, in our understanding, as 8Tx is targeted for CPE/FWA/vehicle/Industrial devices, power issue seems like an optional optimization. |

### FL update 2

Several companies prefer the original version. In essence, both versions are meant for the same idea. We can use the wording of the original version, and we can further improve the wording if needed.

A couple of companies prefer only one of the bullets. Given the large number of supporting companies for either bullet, I think for now we can see if the group can agree on both; otherwise we can make the one with more support to be the baseline and FFS the other one.

@Lenovo: This proposal is intended for CB/NCB/AS. In existing design of n Tx SRS, all the n SRS ports for CB/AS or the up to n SRS ports for NCB are in 1 OFDM symbol. That is similar to the case here, i.e., the proposal is applicable to CB/NCB/AS.

Proposal 4.2-2: For 8 Tx SRS , support

8 ports in one OFDM symbol;

8 ports in multiple OFDM symbols.

| Company | View |
| --- | --- |
| CATT | Support. For 8 ports in multiple OFDM symbols, compared to 8 ports in a single OFDM symbol, more transmission power can be used by each SRS port. It is benefit for SRS coverage. |

### Round 2

Version from Tuesday Online:

Proposal 4.2-2:

For 8 Tx SRS, support both

8 ports in one OFDM symbol

8 ports in multiple OFDM symbols where different ports are mapped to different symbols

For this version of the proposal, it was intended to apply to AS/CB/NCB. However, we can split them separately for the different usages. CB/NCB can be discussed in 9.1.4.2 and AS can be discussed here, though there can be some intrinsic relations between them. Nevertheless, the split proposals are listed below and comments can be made for all of them.

Proposal 4.2-2A:

For 8T8R SRS with usage antennaSwitching, support both

8 ports in one OFDM symbol

8 ports in multiple OFDM symbols where different ports are mapped to different symbols

Proposal 4.2-2B:

For 8 Tx SRS with usage codebook, support both

8 ports in one OFDM symbol

8 ports in multiple OFDM symbols where different ports are mapped to different symbols

Proposal 4.2-2C:

For 8 Tx SRS with usage nonCodebook, support both

8 ports in one OFDM symbol

8 ports in multiple OFDM symbols where different ports are mapped to different symbols

| Company | View |
| --- | --- |
| Sharp | We prefer to support only “8 ports in one OFDM symbol”. SRS with 8 ports in one OFDM symbol can be repeated over multiple symbols and SNR is improved. For this reason, “8 ports in multiple OFDM symbols” has no advantage of power gain. |
| KDDI | We prefer to support both. SRS with one/multiple OFDM symbols have advantages and disadvantages in terms of uplink resource consumption and coverage. So, the two schemes should be used in different situations. |
| LGE | We are fine to discuss separately usage by usage. However, we don’t think that both options should be supported for each usage of SRS. For antennaSwitching(for Proposal 4.2-2A), as we agreed in this meeting, 8 ports in 1 SRS resource is supported. For the consistency with legacy antenna switching behavior, we only support 8 ports in one OFDM symbol. Also, especially for nonCodebook(for Proposal 4.2-2C), 8 ports in one OFDM symbol has consistency with the legacy behavior, where the SRS resources (for nonCodebook) occupy the same RBs. So, we also only support 8 ports in one OFDM symbol for nonCodebook case. For codebook(for Proposal 4.2-2B), we are open to discuss. |
| Lenovo | We support both options for all three usages. |
| Ericsson | We are fine to discuss each usage separately. However, we think it needs to be made clear, in the above proposals, whether the 8 ports are spread over one or multiple SRS resources. Note that ‘usage’ is a parameter that is set on an SRS resource set level, whereas the number of ports is set on an SRS resource level. Hence, we need to further clarify in the above proposals whether the 8 ports, split over the one or multiple OFDM symbols, are in one or multiple SRS resources. Clearly, this will depend on the usage, as explained below. 8T8R antenna switching For 8T8R antenna switching, at the number of Tx ports is equal to the number of Rx ports and, hence, there is only one SRS resource in an SRS resource set with usage ‘antennaSwitching’. The number of ports in this SRS resource is 8. To minimize SRS overhead (e.g., the number of OFDM symbols occupied by SRS), it must be possible to place the 8 ports in one OFDM symbol. Note that legacy coverage-enhancement schemes (e.g., frequency hopping) can be used to increase SRS power and, hence, there is no gain with splitting the 8 ports over multiple OFDM symbols. With this in mind, we think the proposal could read as follows: Proposal 4.2-2A For an SRS resource set with usage ‘antennaSwitching’ support one 8-port SRS resource with 8 ports in one OFDM symbol. 8 Tx non-codebook For non-codebook operation, the number of ports per SRS resource is limited to 1. Hence, 8 1-port SRS resources are needed. The network, as in legacy NR, may decide to spread these ports over a single (to minimize SRS overhead) or multiple OFDM symbols (to increase the power per port). It is worth mentioning that UE may indicate support for 1—4 simultaneous SRS resources via UE-capability signaling (see, 38.306, 38.331). We see no reason to remove this flexibility for 8 Tx non-codebook. Thus, we think the proposal could read as follows: Proposal 4.2-2C For an SRS resource set with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols. - Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling. 8 Tx codebook According to Proposal 4.1-2, the number of SRS resources in an SRS resource with usage ‘codebook’ is FFS. Hence, we can postpone this discussion. Note that if an 8-port SRS resource in an SRS resource set with usage ‘codebook’ is agreed, it is possible to reuse the resource configuration for an 8-port SRS resource in an SRS resource set with usage ‘antennaSwitching’. |
| ZTE | We support all the options except that the restriction of different ports being mapped to different symbols. That means SRS ports are TDM. We think the TD-OCC is better than TDM because the TD-OCC can reduce the power of noise compared with TDM and have same power of signal. |
| OPPO | We think at least Proposal 4.2-2B is needed. Considering that the SRS resource for 8T8R can be reused for 8 Tx codebook based transmission, we prefer a unified design for these two usages. That is, Proposal 4.2-2A and 2B can be supported. For non-codebook based transmission, we intend to agree with Ericsson, and the number of SRS ports that can be simultaneously transmitted is indicated via UE capability. |
| LGE | Regarding the ‘antennaSwitching’ case, we are also fine with the below Proposal 4.2-2A provided by Ericsson with small editorial change. Proposal 4.2-2A (provided by Ericsson) For an SRS resource set with usage ‘antennaSwitching’ support one 8-port SRS resource with 8 ports in one OFDM symbol. |

### FL update 3

Thank you for the useful inputs. For today’s online session, we can try to go over the 3 proposals for the 3 usages, with the primary focus on AS.

@Ericsson: Thank you for the good suggestions, and I generally agree with you. For AS, since agreements on the resource an resource set are already made, we can clearly describe the proposal based on them. As there has been a large number of supports for multiple OFDM symbols, maybe we can keep the proposal a bit open for multiple OFDM symbols, For CB/NCB, resources / resource sets are to be decided, so here we intend to make the proposals decoupled from those aspects.

@ZTE: I believe TD OCC is possible with 8 ports on 1 OFDM symbol, since the next symbol may also have the same 8 ports, and the 2 symbols are based on TD OCC. Unless you plan to use at least 4 symbols for 8Tx, which is clearly not desirable.

Proposal 4.2-3A:

For the 8T8R SRS resource in a SRS resource set with usage antennaSwitching, the SRS resource is transmitted in one OFDM symbol.

FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols

Proposal 4.2-3B:

For SRS resource set(s) with usage codebook for 8 Tx operation, support

8 ports in one OFDM symbol

FFS: 8 ports in multiple OFDM symbols where different ports are mapped to different symbols

Proposal 4.2-3C:

For SRS resource set(s) with usage nonCodebook for 8 Tx operation, support both

8 ports in one OFDM symbol

8 ports in multiple OFDM symbols where different ports are mapped to different symbols

| Company | View |
| --- | --- |
| Ericsson | Fine with Proposal 4.2-3A and Proposal 4.2-3C. For Proposal 4.2-3B, it should be noted that this proposal depends on whether an 8-port SRS resource will be supported in an SRS resource set with usage codebook, as discussed in 9.1.4.2. A minor editorial comment on Proposal 4.2-3A, since the usage is a property of the SRS resource set and since one configures only the number of SRS ports (i.e., no Rx ports) per SRS resource, it would make it clearer to move replace “8T8R” with “8-port”, e.g., as follows: Proposal 4.2-3A: For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in one OFDM symbol. FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols. |

### Online agreement

Proposal 4.2-3A:

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

- FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Proposal 4.2-2C

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

## The maximum number of SRS resource sets for 8 Tx SRS

Regarding the maximum number of SRS resource sets for 8 Tx SRS, some companies proposed to keep the existing value of the maximum number of SRS resource sets (generally 2, except for some UEs supporting a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets]), whereas the proposals of some companies indicate to increase the maximum number of SRS resource sets. Note that some companies seem to be fine with both alternatives.

Proposal 4.3: For the maximum number of SRS resource sets for 8 Tx SRS:

Alt. 1: Keep the existing value of the maximum number of SRS resource sets;

Alt. 2: Increase the existing value of the maximum number of SRS resource sets;

FFS the increased maximum number

Alt. 3: Alt. 1 + Alt. 2.

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| Google | We failed to see the necessity to increase number of SRS resource sets. |
| LGE | It is ambiguous that the proposal is for antennaSwitching or codebook or nonCodebook. We can discuss separately for each cases. |
| OPPO | We think the current number of SRS resource sets is sufficient. |
| Huawei | Support Alt. 1. |
| Apple | We prefer Alt 1 |
| DOCOMO | We are wondering what use case requires more number of SRS resource sets. Could a proponent clarify? Otherwise we prefer Alt1. |
| Samsung | Support Alt. 1. |
| CMCC | Support Alt. 1. |
| InterDigital | Support Alt. 1. |
| Intel | In AI 9.1.4.2, there is also some discussion on the number of SRS resource sets, e.g., FL proposal 3.2 in AI 9.1.4.2. Our suggestion is to discuss the number of SRS resource sets for antenna switching in AI 9.1.3.2, and to discuss the number of SRS resource sets for CB/NCB in AI 9.1.4.2. Hope FLs can have some coordination and clarify. |
| ZTE | It depends on the usage of 8Tx. For antenna switching, Alt1/Alt2 can be adopted. For codebook and non codebook, Alt2 should be adopted. More than one sets for codebook or non codebook can be configured for different time domain behaviors. It is same as antenna switching to improve the flexibility of gNB scheduling SRS for codebook and non codebook. |
| Xiaomi | We also think this can be discussed separately for AS/CB/NCB, different sounding methods may relate to different number of SRS resource set(s). |
| FL | This proposal is intended for all of CB/NCB/AS. Please continue to discuss. |
| MediaTek | Support Alt. 1 |
| Lenovo | We agree with LGE that this issue should be separately discussed for different usages. At least for codebook, we can support Alt.1. |
| QC | Is the use case of increase number of SRS resource sets only for non-codebook based PUSCH? We agree with LGE that this issue should be separately discussed for different usages. |
| Ericsson | Agree with LGE that this issue needs to be discussed separately for different usages. We propose to discuss the number of SRS resource sets for usage ‘antennaSwitching’ in AI 9.1.3.2 and for usage ‘codebook’/’nonCodebook’ in AI 9.1.4.2. |
| CATT | Agree to discuss for different usages separately. At least for codebook and non-codebook, we support Alt 1. |
| vivo | Support Alt 1. |

### FL update

@Intel @Ericsson: Thanks for suggesting the separated discussions for CB/NCB and AS. After coordinating with AI 9.1.4.2, the suggestion is adopted.

Most companies support Alt 1. We can focus on Alt 1 for now. Note that in the legacy design for AS with nTnR, where n =1,2,4, the number of SRS resource sets is up to 2 for all P/SP/AP (except for some UEs, there can be up to 1 resource set of P SRS and 2 resource sets of SP SRS).

Proposal 4.3-1: For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets.

| Company | View |
| --- | --- |
| CATT | It can be discussed after there are conclusions on whether 8 ports in one OFDM symbol or multiple OFDM symbols, and whether 8 ports in one SRS resource or multiple SRS resources. |

### Online agreement

Proposal 4.3-1:

For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR)

## Other designs / design parameters for 8 Tx SRS

Some other designs / design parameters for 8 Tx SRS were also discussed:

Number of SRS resources per SRS resource set, for CB/NCB/AS

Cyclic shift allocation

Comb and comb offset allocation

TD OCC for 8 Tx SRS

Precoded SRS for AS

Full power mode

Antenna switching downgrade

PAPR issue

- These can be further discussed. Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| OPPO | We can discuss these issues when 4.1-4.3 are stable. |
| Apple | What about support 8T8R? SRS has 4 use cases, we only discussed 2 so far. |
| ZTE | TD-OCC can be adopted to increase SRS capacity for 8Tx as for CJT considering the SRS ports increases. |
| Nokia/NSB | support for 8T8R and potential other antenna switching configurations should be also discussed. |
| Ericsson | These can be discussed later. |

## Others

Any other views, issues, potential enhancements, and clarifications, if any, can be provided in below table.

| Company | View |
| --- | --- |

# Conclusions

## Online Agreement:

Agreement

For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

- SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

  - RAN1 should strive to minimize the number of schemes supported in Rel-18

- SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

  - Target usage includes antenna switching, codebook/non-codebook based SRS

Agreement

For 8 Tx SRS, at least support

- 8 ports in 1 SRS resource for ‘antennaSwitching’;

  - FFS 8 ports in one or multiple SRS resources for ‘codebook’

Above does not imply support for 8 ports in one or multiple OFDM symbols

Agreement

For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR)

Agreement

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

- FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Agreement

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

# References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), RAN#94-e.

R1-2205750, SRS enhancements for TDD CJT and 8TX operation, FUTUREWEI.

R1-2205820, On SRS Enhancements for CJT and 8TX UEs, InterDigital, Inc.

R1-2205883, SRS enhancement for TDD CJT and UL 8Tx operation in Rel-18, Huawei, HiSilicon, LG Uplus.

R1-2205922, SRS enhancement targeting TDD CJT and 8 TX operation, ZTE.

R1-2205985, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, Spreadtrum Communications.

R1-2206028, Discussion on SRS enhancement, vivo.

R1-2206191, On SRS Enhancement, Google.

R1-2206213, Discussion of SRS enhancement, Lenovo.

R1-2206267, SRS enhancement targeting TDD CJT and 8 TX operation, OPPO.

R1-2206379, On SRS enhancement for R18, CATT.

R1-2206419, Views on SRS configuration targeting TDD CJT and 8 TX operation, KDDI Corporation.

R1-2206461, Discussion on SRS enhancement, NEC.

R1-2206574, Discussion on SRS enhancement in Rel-18, Intel Corporation.

R1-2206624, Discussion on SRS enhancements, Xiaomi.

R1-2206816, Views on SRS enhancements, Samsung.

R1-2206870, SRS enhancement targeting TDD CJT and 8 TX operation, LG Electronics.

R1-2206898, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, CMCC.

R1-2207067, Discussion on SRS Enhancements for 8Tx Operation, CEWiT.

R1-2207219, SRS enhancement for TDD CJT and 8 Tx operation, Qualcomm Incorporated.

R1-2207324, Views on Rel-18 MIMO SRS enhancement, Apple.

R1-2207397, Discussion on SRS enhancement, NTT DOCOMO, INC.

R1-2207454, SRS enhancement targeting TDD CJT and 8 TX operation, Sharp.

R1-2207499, On SRS enhancements targeting TDD CJT and 8 TX operation, Ericsson.

R1-2207548, SRS enhancement for TDD CJT and 8Tx operation, Nokia, Nokia Shanghai Bell.

R1-2207604, Considerations on precoded SRS, Sony.

RP-212159, “Proposal for Coherent Joint Transmission (CJT) in Rel-18”, Huawei, HiSilicon, Samsung, LG Uplus, China Unicom, China Telecom, China Mobile, CAICT, Verizon, SKT, RAN#93-e.

# Appendix

## Appendix 1: Agreements from RAN1#109-e

Agreement

For SRS EVM, adopt combined relevant parts from Rel-17 SRS EVM and Rel-18 FDD CJT EVM as starting point

- Details are provided in Appendix 3 of R1-2205330 for system-level simulations

- Details are provided in Appendix 4 of R1-2205330 for link-level simulations.

Agreement

For 8 Tx SRS, a starting point of UE antenna configurations can be:

- (M, N, P; Mg,Ng; Mp, Np) = (2,2,2; 1,1; 2,2), (dH, dV) = (0.5, 0.5)λ, or

- (M, N, P; Mg,Ng; Mp, Np) = (1,4,2; 1,1; 1,4), (dH, dV) = (0.5, 0.5)λ.

- FFS other 8 Tx UE antenna configuration and alignment with outcomes from other agenda items.

Agreement

For SRS EVM, consider additional EVM as follows

- Realistic channel estimation based on sequence generation for SRS modelling, at least for TDD CJT SRS LLS and 8 Tx SRS LLS as baseline

- Evaluation metrics for 8 Tx SRS LLS can be MSE , BLER or throughput

- TDL-C for TDD CJT SRS LLS can be included as optional.

Agreement

Consider the scenario where there exists SRSs sent by a UE and utilized by multiple TRPs for channel estimation, and the pathlosses between the UE and the TRPs differ by at least x dB in Rel-18 SRS study

- x can be {3,6,10}, and other values can be used.

Agreement

Study the following for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS interference randomization and/or capacity enhancement

- Randomized frequency-domain resource mapping for SRS transmission

  - E.g., further enhancements to frequency hopping, comb hopping

- Randomized code-domain resource mapping for SRS transmission

  - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence

- Randomized transmission of SRS

    - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

- Per-TRP power control and/or power control of one SRS towards to multiple TRPs

- SRS TD OCC

- Increasing the maximum number of cyclic shifts

  - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts

- Precoded SRS for DL CSI acquisition

- Enhanced signaling for flexible SRS transmission

  - E.g., dynamic update of SRS parameters

- Partial frequency sounding extensions

  - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS

- Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

  - E.g., configuration of v (sequence index within a group) per SRS resource

  - E.g., configuration of cyclic shift per SRS port per SRS resource.

- Resource mapping for SRS transmission based on network-provided parameters or system parameters

  - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index)

Note: PAPR performance and maintaining DFT waveform property should be considered when deciding the enhancement for Rel-18.

Agreement

Study the potential enhancements for SRS of 8T8R with usage antennaSwitching.

Agreement

Study the potential enhancements for SRS for 8 Tx operation

- SRS resource(s) with 8 ports are configured for codebook-based PUSCH

- Up to 8 single-port SRS resources are configured for non-codebook-based PUSCH

Agreement

For SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices, study aspects include, for SRS for CB/NCB/AS,

- Design parameters, including the maximum number of SRS resource sets, number of SRS resource sets, number of SRS resources, number of ports per resource, number of OFDM symbols, the allowed configurations for comb / comb shifts / cyclic shifts, number of simultaneous ports / resources / resource sets per OFDM symbol

- For the next decision point, study

  - Whether to support 8 ports in one or multiple resources

  - Whether to support 8 ports in one or multiple OFDM symbols

  - The maximum number of SRS resource sets.

- Note: For SRS for NCB, number of ports per SRS resource is still 1 (same as R15)

| Rel-18 SLS Assumptions for TDD CJT SRS |  |  |  |
| --- | --- | --- | --- |
| Parameter |  |  | Value |
| Duplex, Waveform |  |  | TDD, OFDM |
| Multiple access |  |  | OFDMA |
| Scenario |  |  | Outdoor OptA Companies can simulate from the following 2 layouts. 1) Outdoor (typical 57-sector, or 21-sector, SLS): OptionA: 1 TRP per sector, 3 sectors per site. N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). The N_TRP TRPs can be selected either only from the same site (intra-site - limited to 3 TRPs), or also from other sites (inter-site) - company should describe what is assumed OptionB: N_TRP co-located (at BS) panels per sector - companies describe how the panels are (azimuthally) oriented - Dense Urban (macro only) 200m ISD or Urban Macro 500m ISD 2) Indoor Hotspot: model in TS 38.802 - N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). |
| Frequency Range |  |  | FR1 only, 3.5GHz |
| Inter-BS (site) distance |  |  | Outdoor: 200m or 500m Indoor Hotspot: per TS 38.802 |
| Channel generation model |  |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| Antenna setup and port layouts at gNB |  |  | - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32,64} |
| Antenna setup and port layouts at UE |  |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 |
| BS Tx power |  |  | Dense Urban or Urban Macro: - Per TRP: 44 dBm for 20MHz, 47dBm for 40MHz, 51dBm for 100MHz Indoor: per TRP 24dBm |
| BS antenna height |  |  | Depending on scenarios (cf. table A.2.1-1 of TS 38.802): DU (25m), UMa (25m), Indoor Hotspot (3m) |
| UE antenna height & gain |  |  | Follow TR36.873 |
| UE receiver noise figure |  |  | 9dB |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |  |
|  | SCS | 30kHz |  |
| Number of RBs |  | 52RB for 20MHz, 104RB for 40MHz, 272RB for 100MHz |  |
| Frame structure |  | DSUDD, or companies to state the used frame structure |  |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |  |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers |  |
| Overhead |  | Companies shall provide the downlink overhead assumption |  |
| Traffic model |  | FTP 1 or FTP 3 with 20%, 50% or 70% traffic load |  |
| UE distribution |  | According to TS 38.802 - DU and UMa: 80% indoor (3km/h), 20% outdoor (30km/h) - Indoor Hotspot: 100% indoor (3km/h) |  |
| UE receiver |  | MMSE-IRC as the baseline receiver |  |
| DL Channel estimation |  | Realistic |  |
| Evaluation Metric |  | DL throughput |  |
| Baseline for performance evaluation |  | R17 SRS design |  |
| SRS modeling for UL channel estimation |  | Companies to state the used SRS periodicity. Companies to state the SRS channel estimation modeling Number of ports = 2 or 4 Tx power = 23 dBm |  |

| Rel-18 LLS Assumptions for TDD CJT SRS |  |
| --- | --- |
| Parameter | Value |
| Scenario | N_TRP (#TRPs): 2, 3, 4 |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 40MHz, 100MHz |
| Channel model | CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| UE velocity | 3km/h |
| Antennas at UE | 1T4R, 2T4R, 4T4R |
| Antennas at gNB | 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Rank and MCS | Rank/MCS can be adaptive or fixed. |
| Evaluation metrics | MSE, BLER or throughput |
| Baseline | R17 SRS design |
| Precoding granularity | Fixed: 2, 4 or wideband for DL, wideband for UL. |
| SRS configurations | Companies to state the used SRS periodicity. Frequency hopping：Companies to state whether SRS frequency hopping is enabled and the hopping pattern if so. |
| DL SNR | Companies to state the used difference between DL SNR and UL SNR |