3GPP TSG-RAN WG1 Meeting #110	R1-2208045

Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | - | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on DCI size for Rel-17 NTN HARQ in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (ZTE),Huawei, HiSilicon,ASUSTek,Xiaomi, OPPO, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, for determining bitwidth of HARQ process number field for DCI format 0_2, TS 38.212 is written in a way that “harq-ProcessNumberSizeDCI-0-2-r17” could indciate an interger between 0 and 5. However, this parameter can only indicate a fixed interger 5 in existing TS 38.331: harq-ProcessNumberSizeDCI-0-2-v1700 INTEGER (5) OPTIONAL, -- Need R |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 0, 1, 2, 3,4, 5 bits to 5 bits if harq-ProcessNumberSizeDCI-0-2-r17 is configured for the field of “HARQ process number”. Correct the postfix of harq-ProcessNumberSizeDCI-0-2-r17 from “r17” to “v1700”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between TS 38.212 and TS 38.331 exists. According to the current texts, 0,1,2,3,4 bits cannot be used if harq-ProcessNumberSizeDCI-0-2-r17 is configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR can be mirrored in Rel-17 version. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged part omitted>>

7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

<<Unchanged part omitted>>

-	HARQ process number – number of bits determined by the following:

-	5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2-v1700 if configured;

-	otherwise 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2

<<Unchanged part omitted>>