3GPP TSG-RAN WG1 #110	Tdoc R1-2208161

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.2.4.1

Source:	Moderator (Ericsson)

Title:	Summary #5 of Evaluation on AI/ML for positioning accuracy enhancement (Final Summary)

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussions during RAN1#110 for the agenda item 9.2.4.1, Evaluation on AI/ML for positioning accuracy enhancement.

This discussion corresponds to the objectives related to the positioning use case described in RP-213599 (SID) below.

RP-213599 (SID): Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. … Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

# Deployment Scenarios

## Companies’ view from contribution

For the topic of deployment scenario(s) for the evaluation of AI/ML based positioning, companies’ views are listed below, based on the submitted contributions.

- Nokia (R1-2206972) Observation-1: Considering the significant number of aspects that needs to be considered as part of the study such as various simulation assumptions, methodology and performance aspects, it would be beneficial for RAN1 to limit the Rel-18 study on AI/ML for positioning accuracy enhancements to InF-DH sub-scenario. Proposal-1: For evaluation of AI/ML based positioning, only InF-DH sub-scenario is considered as part of the Rel-18 study item.

- Qualcomm (R1-2207228) Proposal 5: Consider outdoor wide-area scenario, e.g., Umi, as an additional baseline scenario for evaluation.

## 1st round discussion

Based on the above, most companies do not ask for further discussion on deployment scenario, while the two companies with proposals have different views.

Thus Moderator’s suggestion is: no further discussion on deployment scenario is expected during RAN1#110.

Please comment if you have concern on the suggestion above.

| Company | Comments |
| --- | --- |
| Fujitsu | Support |
| CATT | We agree Moderator’s suggestion. In the last meeting, we already agree that at least InF-DH sub-scenario is prioritized. That means current companies need to focus on InF-DH sub-scenario, if time allow, companies can study other scenario based on further discussion. |
| QC | Companies agreed on considering InF-DH as a baseline sub scenario. However, as noted in our contributions, AI/ML positioning can be applicable not only for indoor scenarios but also for outdoor scenarios. We note that AI/ML assisted positioning methods can also be very beneficial in the outdoor scenario and significant performance gains has been demonstrated in our contribution for this scenario. We believe it is important for the evaluations and ultimately the TR to show that both indoor and outdoor scenarios can benefit from one or more of direct AI/ML based or AI/ML assisted positioning. Thus, we propose to consider the Umi scenario as a baseline scenario as well. |
| ZTE | Support. |
| Apple | Support the FL’s proposal |
| HW/HiSi | We share the same view with Nokia, it would be beneficial for RAN1 to limit the Rel-18 study on AI/ML for positioning accuracy enhancements to InF-DH sub-scenario. |
| Nokia/NSB | We are fine with this suggestion, though we believe that we should agree to limit the scope of the study item in order to make progress. |

# Evaluation Methodology

For evaluation methodology, further discussion is needed.

## Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei (R1-2205894) Proposal 1 : AI/ML-based fingerprint positioning should be adopted for the evaluation of the direct AI/ML positioning accuracy enhancements under heavy NLOS conditions in Rel-18. Proposal 2: For evaluation on AI/ML-based fingerprint positioning, spatial consistency should be modeled at large scale parameters, small scale parameters and absolute time of arrival as the baseline, where - the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance = dclutter⁄2 for InF (Section 7.6.3.1 of TR 38.901). - the small scale parameters are according to Section 7.6.3.1 of TR 38.901 - the absolute time of arrival is according to Section 7.6.9 of TR 38.901 Proposal 3 : For comparison of evaluation on AI/ML-based fingerprint positioning evaluation results, support the channel impulse response (CIR) as the model inputs. Proposal 5 : For evaluation on AI/ML-based fingerprint positioning, support UL-TDoA positioning as a candidate baseline of the legacy non-AI/ML method. Proposal 8 : For AI/ML assisted positioning, AI/ML-based LOS/NLOS identification should be evaluated for positioning accuracy enhancements at least under slight/moderate NLOS scenarios in Rel-18. Proposal 9 : For comparison of evaluation on AI/ML-based LOS/NLOS identification, support the power delay profile (PDP) as the model inputs. Proposal 10 : For evaluation of the AI/ML-based LOS/NLOS identification, the achievable positioning accuracy should be compared with the performance of an existing traditional algorithm based on Rel-17 without using AI/ML.

- vivo (R1-2206036) Observation 6: Fine-tuning the model with small amounts of samples can achieve significantly positioning accuracy improvement when the pre-trained model is transferred to a new scenario. Observation 7: Semi-supervised learning can achieve a more accurate position estimation as compared to traditional supervised learning with small amount of labeled data. Observation 8: Positioning with mult-port data can achieve a more accurate position estimation as compared to single-port positioning. Proposal 2: Support time domain CIR as the model input for AI/ML based positioning. Proposal 4: Study further on the benefits of fine-tuning for AI/ML based positioning in terms of positioning accuracy and AI model generalization. Proposal 5: Study further on model training framworks with low labeled data dependence, including fine tuning, semi-supervised learning, multi-port positioning. Proposal 6: Study further on the benefits of semi-supervised learning for AI/ML based positioning in terms of positioning accuracy. Proposal 7: Study further on the benefits of multi-port positioning for AI/ML based positioning in terms of positioning accuracy.

- ZTE (R1-2206072) Proposal 1: For evaluations on AI for positioning, at least include following sub-use cases for direct AI/ML positioning and AI/ML assisted positioning: - Direct AI/ML positioning - Sub-use case 1-1: AI model input is path timings and RSRPPs from single port PRS - Sub-use case 1-2: AI model input is CIR (i.e., path timings, RSRPPs and path phases) from single port PRS - Sub-use case 1-3: AI model input is path timings and RSRPPs (or CIR) from multi-port PRS - AI/ML assisted positioning - Sub-use case 2-1: AI model output is DL PRS RSTD values - Sub-use case 2-2: AI model output is LOS/NLOS indicator Proposal 2: Study and evaluate the performances of direct AI/ML positioning under different number of path timings and RSRPPs for AI/ML model input. Proposal 3: Study and evaluate performances of direct AI/ML positioning when AI model input includes path phase information. Proposal 4: Study and evaluate performances of direct AI/ML positioning when AI model input includes measurement results from multi-port PRS. Proposal 6: Study and evaluate performances of AI/ML assisted positioning when AI model output includes confidence levels of LOS/NLOS identification.

- Rakuten Mobile (R1-2206252) Observation 1: AI/ML based positioning using CIR as input provides high accuracy. Proposal 1: AI/ML based positioning with CIR as input should be considered.

- CATT (R1-2206395) Proposal 1: For AI/ML-based positioning, the evaluation methodology is as follows: - For directly estimating UE’s position based on AI/ML model, consider evaluating the positioning accuracy as eventual evaluation. - For estimating timing and/or angle of measurement based on AI/ML model, consider evaluating the result of ToA/AoA/AoD estimation as intermediate evaluation, and evaluating the positioning accuracy as eventual evaluation. - For identifying LOS/NLOS identification based on AI/ML model, consider evaluating the LOS/NLOS identification as intermediate evaluation, and evaluating the positioning accuracy as eventual evaluation. Proposal 3: For AI/ML-based positioning, two-sided model is supported to be further studied.

- CAICT (R1-2206679) Proposal 1: With the agreed evaluation methodology, the effects of simplified measurement and incomplete coordinate labels need to be considered.

- Nokia (R1-2206972) Observation-2: Currently, there are no existing agreed assumptions for modelling issues introduced by implementation-specific issues such as UE clock drift. However it would be beneficial for RAN1 to study such issues further. Proposal-2: For evaluation of AI/ML based positioning, the dataset construction can optionally utilize the network synchronization and the UE/Gnb RX and TX timing error by reusing the assumptions in TR 38.857 Table 6-1. Proposal-3: RAN1 to study further issues introduced by implementation-specific issues such as UE clock drift, and develop an evaluation framework for AI/ML based positioning. Proposal-11: For evaluation of AI/ML based positioning, RAN1 should assess the need for standardizing the procedures for triggering and/or controlling the ML model adaptation and fine-tuning after model deployment.

- InterDigital (R1-2207094) Proposal 1: Adopt direct AI/ML positioning to achieve high positioning accuracy. Proposal 2: Study enhancements to improve positioning accuracy for the UEs dropped on the edge of the deployment area. Proposal 3: Study further whether AI/ML assisted positioning can be used to estimate observable measurements or unobservable measurements.

- Fraunhofer (R1-2207123) Figure 2: Principle of using merging two sets of channel data Proposal 1 Adopt the idea of the TR38.901 model to merge two sets of channel coefficients, and replace the Ray-tracer with a second channel builder. The configuration of the second channel builder can depend on the use case. Proposal 2: The second channel builder generates additional clusters resulting in a better representation of critical data in the training data set. For the evaluation of ToA measurement enhancements the second builder is configured to add “early cluster” (EC) representing reflections at objects close to the UE or TRP. Proposal 3: For the evaluation of AI/ML based technologies a second channel builder shall add randomly generated clusters to the test data set to cover changes in the environment between the training phase and the operational phase. The power ratio between randomly generated clusters and static clusters shall be configurable.

- Qualcomm (R1-2207228) Observation10: Direct AI/ML methods may be better suited for scenarios where model switching is possible, or for scenarios where devices operate only within a given premises (e.g., AGVs in a factory), while AI/ML-assisted methods may be better suited for scenarios where a common model is required for different scenarios. Proposal6: Study both direct AI/ML and AI/ML assisted positioning approaches.

- Lenovo (R1-2206514) Proposal 4: The positioning AI/ML model evaluation methodology should support scenarios evaluating a model's robustness and adaptability based on the dynamics and/or conditions of the environment via feedback. FFS definition of AI/ML model robustness and adaptability and associated metrics.

- Samsung (R1-2206824) Proposal 2: RAN1 to study the generalization ability for imperfect input/output data and how to model the imperfections.

- China Telecom (R1-2206689) Proposal 1: one-side model for AL/ML based positioning accuracy enhancement can be selected as baseline.

- PML (R1-2205915, R1-2206224) Observation 1: When the UE position is fixed, hardware impairments will cause offset of CIR amplitude and delay. Observation 2: The offset of CIR delay and amplitude caused by hardware impairments will lead to unstable network output. Proposal 1: The channel modeling containing synchronization and amplitude errors should be employed for the evaluation of AI/ML-based positioning enhancement. • The network synchronization parameters in TR 38.857 Table 6.1 can be adopted.

- Google (R1-2206199) Proposal 1: The study of AI/ML based AI/ML based positioning should be based on actual channel estimation. Proposal 2: The study of AI/ML based AI/ML based positioning should take random phase offset between cells into account.

## What to evaluate?

### 1st round discussion

Based on the proposals and the evaluation results submitted, it is clear that there are extensive support for both direct AI/ML positioning and AI/ML assisted positioning. Thus the following is proposed.

Proposal 3.2.1-1

For AI/ML-based positioning, both approaches below are studied by RAN1, and the evaluation results of both are expected to be captured in TR38.843:

- Direct AI/ML positioning

- AI/ML assisted positioning

|  | Company |
| --- | --- |
| Support | Fujitsu, InterDigital, CAICT, LG, CATT, QC, OPPO, Apple, Nokia/NSB, Samsung, |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | We support to evaluate both direct AI/ML and AI/ML assisted positioning. However, we have a clarification question on the wording “and the evaluation results of both are expected to be captured in TR38.843”. If this proposal were agreed, does this mean that capturing evaluation results of both will not subject to further discussion/agreement in RAN1? |
| CAICT | The details of AI/ML assisted positioning need FFS. |
| ZTE | We’re fine to study both cases. However, whether the evaluation results should be captured in TR depends on the performances of different sub-use cases, which should be discussed later. |
| HW/HiSi | Regarding to the question from vivo: “If this proposal were agreed, does this mean that capturing evaluation results of both will not subject to further discussion/agreement in RAN1” – our understanding is “yes”. |
| Nokia/NSB | We are fine with this proposal. |

Regarding one-sided vs two-sided model, only CATT provided evaluation results for two-sided model, and proposed to study it. Given that majority companies focus on one-sided model only, the following is proposed. Note that this does not affect discussion of two-sided model in other agenda items, e.g., 9.2.1, 9.2.4.2.

Proposal 3.2.1-2

For evaluation of AI/ML-based positioning, one-sided model is prioritized over two-sided model.

|  | Company |
| --- | --- |
| Support | Fujitsu, CAICT, LG, ZTE, OPPO, HW/HiSi, Samsung, |
| Not support | CATT |

| Company | Comments |
| --- | --- |
| InterDigital | Companies should have freedom to choose which model to evaluate. |
| CATT | This is the first meeting we discuss the evaluation results. Not all companies give the sufficient simulation. Some aspects can be further studied and encouraged companies to give more simulation results in the future meetings. Thus, we think this proposal is unnecessary. Maybe in this meeting, we can focus on the discussion of one-sided model. The two sided mode can be further studied. |
| QC | This is the second meeting, and we believe it is too early to decide on any prioritization. |
| Nokia/NSB | We agree with QC and would prefer not to prioritize one approach over the other at this stage. |

## Model update / refinement / tuning

Several companies raised the issue of model transfer, ML model adaptation, and fine-tuning after model deployment.  While the procedure, signalling, and potential specification impact are expected to be handled under AI 9.2.1 or AI 9.2.4.2, RAN1 can investigate the performance impact under AI 9.2.4.1.

### 1st round discussion

Based on companies’ input, the following is proposed.

Proposal 3.3.1-1

For AI/ML-based positioning, RAN1 study model transfer and fine-tuning after deployment, including: impact to training dataset (e.g., dataset size reduction), positioning accuracy, and AI model generalization.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | 1. Current definition of “model transfer” in the WA Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. It’s not clear to us what to evaluate the performance impact on positioning accuracy for model transfer (which is just a delivery). In our contribution, we’re showing the performance benefits of model updating/fine-tuning. 2. As moderator commented “RAN1 can investigate the performance impact under AI 9.2.4.1”, we suggest some refined wording to better reflect that: For AI/ML-based positioning, RAN1 evaluate the performance of model updating and fine-tuning after deployment, including: impact to training dataset (e.g., dataset size reduction), positioning accuracy, and AI model generalization. |
| Fujitsu | We do not want to preclude the “model transfer” operation in this positioning use case, but according to the definition given by 9.2.1, we have no idea why a pure model transfer (download or upload) deserves a discussion here. |
| InterDigital | We should agree on how impact of model transfer/fine-tuning on training dataset/accuracy/generalization can be evaluated. |
| CAICT | We prefer to handle model transfer for positioning in a later stage. |
| LG | We have a similar view with vivo that the related performance change can be considered not only for positioning accuracy but also resource efficiency and overhead issue on supporting model transfer. |
| CATT | We wonder what model transfer means after deployment. The definition of “model deployment” in the WA is: Delivery of a fully developed and tested model runtime image to a target UE/gNB where inference is to be performed. Thus, deployment already includes model transfer. After deployment, the model transfer is already down. |
| QC | It is too early to discuss such a proposal without the general framework in 9.2.1 discussion having a stable conclusion. Whether model transfer and fine-tuning can be supported still needs to be agreed upon in general framework. We believe that it is difficult to achieve in the first set of specifications for AI/ML study. |
| ZTE | According to definitions in AI 9.2.1, - Model deployment happens after model training (or model update) and model deployment. - Model update includes model retraining or model fine-tuning via online/offline training. We prefer the following modifications to the proposal on top of vivo’s revision: For AI/ML-based positioning, RAN1 evaluates the performance of model updating and fine-tuning after deployment model update, including: impact to training dataset (e.g., dataset size reduction), positioning accuracy, and AI model generalization. |
| OPPO | Agree with CAICT. Model transfer could lead to more challenges for spec design and practical deployment. We suggest to focus on the schemes without model transfer at an early stage. |
| Apple | Agree with Oppo and CAICT |
| HW/HiSi | Share the same view with vivo, model transfer and fine tuning are not part of the same discussion. For model updating/fine-tuning, we are supportive to study for positioning. And fine with vivo’s refinement. |
| Nokia/NSB | We are broadly supportive of studying model transfer and fine-tuning. We are fine with the suggestion from InterDigital to focus the proposal on the evaluation of positioning use case, and perhaps agree on some evaluation KPIs. |

### 2nd round discussion

Thanks to companies for input on terminology and clarifications. The updated proposal by vivo/ZTE is copied below for further discussion.

Note that in Monday session, the agreement contains the following explanation about “model update”. Thus it should be clear to use “model update” phrase in the discussion.

Agreement … - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

Proposal 3.3.2-1

For AI/ML-based positioning, RAN1 evaluate the performance of model update, including: impact to training dataset (e.g., dataset size reduction), positioning accuracy, and AI model generalization.

|  | Company |
| --- | --- |
| Support | Fujitsu, Nokia/NSB |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| Fujitsu | Seems OK. |
| OPPO | We are fine with the proposal. But the condition for applying model update is unclear for us. Maybe the proposal can be updated as: For AI/ML-based positioning, RAN1 evaluate the performance of model update, including: Condition of AI/ML model update, impact to training dataset (e.g., dataset size reduction), positioning accuracy, and AI model generalization. |
| Xiaomi | The terminology is not clear enough. According to the online discussion for the Model update, it seems companies have different understanding on the model re-training, fine-tuning and re-development. Thus, we prefer to discuss this proposal after we are aligned in what is model update. |
| Apple | Agree with Xiaomi. |
| ZTE | OK in general. However, we feel that this issue may be better to be discussed in model generalization (i.e., how to increase model generalization capability) |
| CATT | Generally OK. But we think this proposal can be discussed after companies have a clear view on some procedures and terminologies, e.g., LCM, mode update, model generalization. |
| Qualcomm | We suggest the following modified proposal: For AI/ML-based positioning, RAN1 evaluates whether generalization can be achieved by finetuning on a small dataset that is matched to the test dataset. |
| HW/HiSi | We think that these issues can be taken into account, but we don’t need an agreement for it (at this stage). This could be treated after we have progressed with the use cases, input to AI model, output. Then, we will have a clearer picture about the impact of this proposal. |
| Nokia/NSB | We are fine with this proposal. |

#### Offline discussion

Proposal:

For AI/ML-based positioning, RAN1 evaluates whether generalization can be achieved by model update on a small dataset that is not matched to the test dataset.

### 3rd round discussion

Regarding model monitoring and model update, preliminary results have been provided by companies (e.g., vivo (R1-2206036)). More results from more participating companies will help RAN1 to understand the issues better.

Accordingly, Proposal 3.3.2-1 is revised below. The goal is to encourage companies to study model update, and share evaluation results and observations with RAN1.

Proposal 3.3.3-1

For AI/ML-based positioning, RAN1 evaluate the performance of model update. Participating companies report at least the following:

- training dataset (e.g., training dataset size necessary for performing model update),

- horizontal positioning accuracy (in meters) before and after model update.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, Fujitsu |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We are fine with this proposal. |
| CMCC | According to the agreements, model update includes model fine-tuning, retraining, and re-development via online/offline training. But the condition of model re-training and re-development is not clear. We prefer to discuss more details about how to evaluate the performance of model update, or we can just focus on the performance of model fine-tuning. |
| Fujitsu | For the first bullet, we suggest removing the word “necessary” since as CMCC mentioned, the dataset quantity to perform model re-training or fine-tuning will be quite different, and the word “necessary” seems to be subjective. |
| Qualcomm | The term "evaluating the performance of model update" is not clear. Request the wording be updated or clarified. |

Related to model update, another issue is model monitoring. For example, what metric (e.g., positioning error) and what threshold (e.g., positioning error > X meters) to use, in order to determine that a model is out-of-date, and need an update. Companies are invited to share your view on how to study this.

Question 3.3.3-2

Do you support that RAN1 evaluate methods for performing model monitoring for AI/ML based positioning?

|  | Company |
| --- | --- |
| Yes | Nokia/NSB |
| No |  |

Please share your view on how to perform model monitoring for direct AI/ML positioning and AI/ML assisted positioning, and how to evaluate the effectiveness of the model monitoring methods.

| Company | Comments |
| --- | --- |
| Nokia/NSB | While we agree that model monitoring is important, the exact metric to evaluate its effectiveness requires further study. For e.g., the use of positioning error as the metric assumes that the ground truth during model deployment and inference stage is known, which might not be realistic. |
| CMCC | We prefer to evaluate model monitoring after the model monitoring procedure has been discussed in AI 9.2.4.2. |
| Fujitsu | We support to evaluate model monitoring because it is one crucial aspect of AI/ML model, and it is quite important for positioning use case due to the generalization issue. However, maybe the moderator could also notice that there were too many discussions on those terminologies and related procedures which should be defined in 9.2.1, so in order to avoid long trivial discussion, we prefer to wait for everything is clear enough in 9.2.1. Besides, how to measure the positioning error in the live network is still pending. |
| Qualcomm | We are in general ok with intent of the evaluation. However, it is not clear how evaluating model monitoring would work in the positioning context and how it would be distinguished from model generalization. We prefer companies to provide more inputs before further discussion. |

## Imperfections at UE and gNB

### 1st round discussion

The issue of implementation imperfections at UE and gNB is real and cannot be ignored in deployment. Several companies proposed to evaluate AI/ML under implementation imperfections, similar to the study carried out for Rel-17 positioning. Simulation results have been submitted by several companies as well.

Regarding simulation parameters to use, it seems sufficient to follow the agreement in RAN1#109e. That is, the simulation assumptions can reuse those of Rel-17 (i.e., Table 6-1 of TR 38.857). It does not appear necessary to have additional agreements.

Thus, the following is proposed to capture the intention.

Proposal 3.4.1-1

For AI/ML-based positioning, companies are encouraged to study impact from implementation imperfections, including:

- Network synchronization error

- UE/gNB RX and TX timing error

- UE clock drift

|  | Company |
| --- | --- |
| Support | Fujitsu, CAICT, QC, Nokia/NSB, Samsung, |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | We have a question and concern on the 2nd bullet. Rel-17 positioning WI already specified and support UE/gNB RX and TX timing error report. It’s not clear to us what could be the remaining error after Rel-17 REG/TEG report. Moderator said “the simulation assumptions can reuse those of Rel-17 (i.e., Table 6-1 of TR 38.857).” Our understanding is that those assumptions are for Rel-17 SI before the solution introduced in Rel-17 positioning WI. As such, the study should be compared against Rel-17 positioning with REG/TEG feature enabled. |
| InterDigital | As the FL explained we have the following agreement from RAN1#109e. Agreement For evaluation of AI/ML based positioning, reuse the common scenario parameters defined in Table 6-1 of TR 38.857. This agreement should be sufficient to model errors at Tx/Rx. Companies should disclose the details assumed in evaluation. |
| CATT | Share the same view as vivo. |
| ZTE | We have following agreement in last meeting: Agreement For evaluation of AI/ML based positioning, reuse the common scenario parameters defined in Table 6-1 of TR 38.857. The Table 6-1 already includes the simulation assumptions for Network synchronization and UE/gNB RX and TX timing error. There is no need to further discuss this issue and companies can provide their assumptions for evaluation. Regarding UE clock drift, it evaluates the UE timing error over time. Even in Rel-17, we don’t have clear model for the assumption, we prefer not to discuss this issue in Rel-18 AI positioning. It may be more appropriate for studying UE mobility over time, which should be deprioritized in Rel-18. |
| HW/HiSi | Since the first and the second bullet have already been agreed in simulation assumption in RAN1#109e, we think no separate proposal is required for this issue, but these aspects can be included in the generalization evaluation aspects. For the UE clock drift, we think it is too early to agree on, since there is currently no model in RAN1 for it and we are wondering how to describe it. Some discussion would be needed firstly we think. @vivo, enabling TEG feature could only solve the timing error within one node’s different Rx/Tx chains coarsely, so we don’t think TEG could solve the timing error randomly existing between training and inference datasets. In our paper(R1-2205894) we provide the evaluation results of baseline and Test scenario 12 and 13, when the added UE Tx/Rx timing error are randomly distributed with T_1= 10 ns, fingerprint positioning model provides poor generalization performance without special consideration of the non-ideal factors. |
| Nokia/NSB | We support this proposal. Regarding the earlier comments, our understanding of this proposal is to study the impact of the timing error on dataset generation, model training and deployment, rather than the reporting of the error. It is important to note that TEG is an error group, i.e. a report of what the expected error interval is, and not an instantaneous error and for this reason, the error cannot be compensated efficiently. It is also unclear how the LMF uses TEG, since such details are left to implementation. Studying the impact of these imperfections could help understand whether ML models could be robust to the timing errors, rather than relying on reporting error intervals. |

# KPI

A list of agreements was made in RAN1#109e in KPI.  In the following, the remaining issues on KPI are discussed.

## AI/ML Complexity

Regarding AI/Ml complexity, selected input from companies’ contribution are provided below. The complexity reported by companies for the evaluated models is also included.

### Companies’ views and reported AI/ML complexity from contributions

- Huawei (R1-2205894) Table 1 Model complexity of AI/ML-based fingerprint positioning. Model complexity. Model size (in terms of Number of Parameters); 34 K. Floating point Operations (FLOPs); 10 M. Table 2 Model complexity of LOS/NLOS Identification. Model complexity. Model size (in terms of Number of Parameters); 582. Floating point Operations (FLOPs); 192 K.

- PML (R1-2205915, R1-2206224) Table 3 Model complexity statistics of designed CNN Total memory; Total Madd; Total Flops; Total MemR+W. 0.24MB; 14.27MMAdd; 7.19Mflops; 8.46MB.

- Vivo (R1-2206036) Table 12 Cost evaluation of AI models AI models; CNN; Vision Transformer. Computational complexity; 1.45G FLOPs; 22.30M FLOPs. Number of Parameters; 44M; 1.65M.

- ZTE (R1-2206072) Model backbone; CNN( Modified ResNet [4]). Model complexity (number of parameters); 9.50 M. Model computational complexity (FLOPs); 158.47 M. Table.2 Model complexity of AI model for sub-use case 1-1

- Fujitsu (R1-2206168) Proposal 1 Floating point of operations is suggested to be used as the KPI for evaluating both the model complexity and computational complexity. Others such as the number of model parameters are encouraged to report optionally. Table A-2 Direct AI model parameters Dataset Size; 90000 (81000/9000 for Training/Testing). Cost Function; MSE. Training Epoch; <50. Optimizer; Adam. Learning Rate; 0.00005. FLOPs; 1.14M. Table A-3 Indirect AI model parameters Dataset Size; 90000*18 (81000*18/9000*18) for Training/Testing). Cost Function; MAE. Training Epoch; <50. Optimizer; Adam. Learning Rate; 0.0001. FLOPs; 0.296M.

- Ericsson (R1-2206248) Proposal 1 For evaluation of AI/ML based positioning, model complexity is reported via the AI/ML model size and number of AI/ML parameters. The model size includes the total number of layers, and the type of each layer. The number of AI/ML parameters include the total number of parameters used in model inference. Table 4 Key features of the ML method for LoS classification and time of arrival estimation. ML model input; Time domain CIR, obtained from SRS 256x2 complex array. ML model output; (1). LoS/NloS classification (2). ToA estimate. Model complexity:; Model size; 6 layers: 3 Conv1D layers, 3 Dense layers. Number of parameters in the ML model; 33,429 complex parameters. Computation complexity for model inference: number of FLOPs; 343,227 FLOPs. Number of ML models obtained from training; One. The same ML model is deployed at each TRP. Number of ML models deployed for inference; 18 One ML model per TRP. Function for position estimation of the target UE; Legacy method: UTDOA. Table 5 Key features of the direct ML positioning method ML model input; 18 ToA values for a target UE, where each value is an integer in time units of sampling duration (8.138ns). One ToA value is obtained from each TRP based on its SRS reception. ML model output; Horizontal position of the target UE. Model complexity:; Model size; 11 Dense layers; 600 neurons per layer. Number of parameters in the ML model; around 3 million. Number of ML models obtained from training; One. Number of ML models deployed for inference; One.

- OPPO (R1-2206319) Table 10: Complexity of AI modes Direct; Assisted. MFLOPs; ~ 5.32; ~ 2.96. No. of trainable parameters; ~ 2.66M; ~ 1.48M.

- Lenovo (R1-2206514) Observation 1: AI/ML models for positioning require a careful balance between performance and complexity depending on the type of positioning mode (UE-assisted or UE-based). Proposal 3: In addition to FLOP counts, the evaluation should also consider the hardware and software platforms used to evaluate the positioning AI/ML algorithms, type of data being used as input, training type, e.g., offline vs online, complexity type, e.g., worst-case/average-case.

- NVIDIA (R1-2206524) Observation 1: Increasing hardware performance can support successively more complex AI/ML models. For example, GPU inference performance has improved by 317x in 8 years (2012-2020), more than doubling each year. Proposal 3: AI/ML model complexity and computational complexity should not be regarded as a roadblock to the adoption of AI/ML based algorithms for positioning accuracy enhancements. Figure 1: GPU inference performance is more than doubling every year. (Source: Ref. [4]) Figure 2: Single GPU performance scaling. (Source: Ref. [4])

- Xiaomi (R1-2206639) Table 4 Computation in the AI model size and the computation complexity Solution; Input options; AI model size(Bytes); Computation complexity (FLOPs). Finger printing; Option 1; 52.81M; 5.76G. Option 2; 4.95M; 539.95M. AI-based ToA predication; Option 1; 52.81M; 5.76G. Option 2; 4.95M; 539.94M. Proposal 2: On the basis of satisfying the positioning accuracy requirement, study solution to reduce the model size, computation complexity and involved ignaling overhead

- China Telecom (R1-2206689) Proposal 2: For the evaluation of the AI/ML based positioning accuracy enhancement, floating point operations (FLOPs) can be used as part of the ‘Evaluation Metric’, and reported by companies.

- Samsung (R1-2206824) Table 1 – FLOP vs parameter budget Type; FLOP; Parameter budget. Dense layer; O(MN); O(MN). Convolutional Layer; O(KhKwCinCoHW); O(KhKwCinCo). Observation 2: When use different neural network layer (e.g., DenseLayer vs ConvLayer), a trade-off can be made between FLOPs and Parameter budget. Proposal 3: RAN1 supports that the parameter budget for storage consumption is used for evaluating an AI Model in Positioning.

- CMCC (R1-2206906) Table 3. Parameters for model training AI model; FLOPs(×106); Trainable Par(×106). CNN; 3.72; 3.71.

- Nokia (R1-2206972) Proposal-4: The model complexity metrics agreed as part of Agenda Item 9.2.1 could be reused for the positioning accuracy improvement use case. Table 2: Computational complexity, model complexity and training dataset size. AI models; FCNN training based on random data selection; FCNN training based on on-demand labeled data. FLOPs (floating point operations); 2.38M; 4.33M. Model size (parameter #); 0.24M; 0.31M. Training data requirements; 13k; 13k. Table 7: Computational complexity, model complexity and training dataset size. Two-Step Positioning; One-Step Positioning. Model complexity (parameter size); 3 K; 1.9M. #FLOPs; 200 K; 19.2M. Dataset size; 10k+; 10k+.

- InterDigital (R1-2207094) Table 6 FLOP and horizontal accuracy of Direct AI/ML positioning (m) Simulation Cases; Complexity (FLOP); 50% ile; 67% ile; 80% ile; 90 %ile. Case 1: DL-TDOA; N.A.; 2.2944; 4.5906; 7.9878; 10.9090. Case 2: mRTT; N.A.; 0.5963; 1.0681; 1.6487; 2.4705. Case 3: M1-fingerprinting(RSRP); 18.34 M; 1.6021; 2.0669; 2.6096; 3.2796. Case 4: M2-fingerprinting(RSTD); 9.38 M; 0.6599; 0.8839; 1.1878; 1.6511. Case 5: M3-fingerprinting(RSTD); 37.34 M; 0.6165; 0.8424; 1.1224; 1.5719. Case 6: M4-fingerprinting(RSRP+RSTD); 11.41 M; 0.9465; 1.2377; 1.5658; 2.0405. Table 5 Conventional positioning method using estimated measurements : FLOP, RSTD NMSE, horizontal accuracy Simulation cases; Complexity (FLOP); RSTD NMSE (1016 m); 50% ile; 67% ile; 80% ile; 90 %ile. Case 7: DL-TDOA with actual RSTD measurements; N.A.; N.A.; 2.0004; 4.3218; 8.2266; 11.8934. Case 8: DL-AoD; N.A.; N.A.; 9.6731; 12.8102; 16.4264; 37.1792. Case 9: DL-TDOA with estimated RSTD measurements(M5); 12.61 M; 0.0409; 5.7499; 8.7265; 12.1417; 16.0340. Case 10: DL-TDOA with estimated RSTD measurements(M6); 11.42 M; 0.0445; 5.4290; 7.6270; 12.3024; 15.6817.

### 1st round discussion

For the KPI of AI/ML complexity, it was agreed in RAN1#109e that the computational complexity can be reported via the metric of floating point operations (FLOPs). However, no detailed metric was agreed for the model complexity. Based on companies input to RAN1#110, three metrics were reported or proposed by companies:

- Number of model parameters (9): Huawei/HiSi, vivo, ZTE, Ericsson, OPPO, Samsung, CMCC, Nokia

- Total memory (2): PML, Xiaomi

- Model size (total number of layers, and the type of each layer) (1): Ericsson

Thus majority companies prefer to use “number of model parameters”. Moderator’s understanding is, this refers to the total number of trainable parameters, which are trained and then used in model inference.

While the metric of total memory has the advantage of providing memory cost directly in bytes, it is more difficult to agree exactly how to define the memory storage size, for example, number of bits used to quantize the model parameters.

Hence the following is proposed to reflect the majority view. This is also consistent with the agreement made in RAN1#109e for CSI topic.

Proposal 4.1.2-1

For evaluation of AI/ML based positioning, the model complexity is reported via the metric of “number of model parameters”.

|  | Company |
| --- | --- |
| Support | Fujitsu, CATT, QC, ZTE, OPPO, Apple, HW/HiSi, Nokia/NSB, Samsung, |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | We support the proposal, however we have noticed that in the 109e agreement, other metrics for evaluating computational complexity are not precluded, so we suggest the same for model complexity. Besides, even the FLOPs and number of parameters are not linearly proportional, we think FLOPs only can be used as the single metric to evaluate the overall complexity of the AI/ML models. |
| InterDigital | The number of model parameters is one of the factors that affect computational complexity. The number of layers in the AI/ML model affects the complexity as well. When companies report FLOPs, more detailed descriptions (e.g., how many parameters/layers assumed for the AIML model) should clarify. |
| CAICT | AI model complexity should be aligned between evaluations of different use cases. FLOPs should also be considered. |
| LG |  |
| Nokia/NSB | We are open to studying other relevant parameters as long as the number of model parameters are reported. Not sure if it makes sense to add an FFS for possible other relevant parameters. |
| Samsung | Considering that there is a trade-off on the preference on computation and storage consumption, RAN1 should not only look into the computation aspect, but also the storage consumption aspect, which is important for evaluating whether a model could be actually applied or implemented eventually. |

## Generalization of AI/ML Models

Generalization capability of AI/ML models is a very important KPI, and extensive investigation has been carried out by companies.

### Companies’ view from contribution

- Huawei (R1-2205894) Proposal 6 : Formulate generalization evaluation criteria based on the specific sub use case. The AI/ML based fingerprint positioning is considered as a starting point. Table 2 Generalization study of AI/ML-based fingerprint positioning Methodology for evaluating the model generalization. Dimension 1: Drops; Dimension 2: Clutter parameters; Dimension 3: Non-ideal assumptions. Trained and inferred at different drops; Trained and inferred at different clutter parameters; Trained w/o & inferred w/ network synchronization error. Trained and inferred at mixed drops; Trained and inferred at hybrid clutter parameters; Trained w/o & inferred w/ UE/gNB Rx and Tx timing error. Proposal 7 : For evaluation of AI/ML-based positioning, RAN1 should formulate the methodology for evaluating the model generalization, and the following dimensions are proposed to be considered as the baseline: • Dimension 1: various inference dataset from different drops under the same channel parameters; • Dimension 2: various inference dataset from different channel parameters, including at least {60%, 6m, 2m} and {40%, 2m, 2m} under InF-DH scenario; • Dimension 3: various inference dataset with non-ideal assumptions, including the network synchronization error and the UE Tx/Rx timing error.

- Vivo (R1-2206036) Proposal 1: The positioning accuracy performance of AI/ML based positioning should be evaluated under different settings/scenarios (i.e. the training and testing dataset are from different settings) - different drop in the same scenario; - different clutter parameter settings; - different InF scanrios. Proposal 3: Study further on the benefits of training dataset with mixed/different configurations for AI/ML based positioning in terms of positioning accuracy and AI model generalization. Proposal 4: Study further on the benefits of fine-tuning for AI/ML based positioning in terms of positioning accuracy and AI model generalization. Observation 4: Training AI/ML model with a mixed dataset is an effective way to improve model generalization performance. Observation 5: When the distribution of test dataset is different from that of training dataset, the positioning accuracy of AI/ML based positioning can be seriously degraded.

- ZTE (R1-2206072) Proposal 7: Discuss the simulation assumptions to evaluate the model generalization, at least consider following categorizations: - Cat.1: Training dataset and inference dataset are in the same simulation drop. - Cat.2: Training dataset and inference dataset are from different simulation drops but with the same random seed for large scale parameters. - Cat.3 : There is no spatial consistency between training dataset and inference dataset. Select at least one of the following options: - Cat.3-1: Training dataset and inference dataset are from different simulation drops but with different random seeds. - Cat.3-2: Different parameter settings for training dataset and inference dataset under the same scenario. (e.g., with different clutter settings for InF-DH scenario) - Cat.3-3: Training dataset and inference dataset are from different scenarios. (e.g., InF-DH scenario vs InF-SH scenario) Proposal 8: Study the metrics to evaluate the model generalization capability, at least consider: - Performance loss when the same AI model used for a training dataset and an inference dataset - Efforts to fine-tune an AI model trained on a training dataset so as to get similar performance in an inference dataset (e.g., the number of new data)

- Ericsson (R1-2206248) Proposal 2 For model generalization KPI, companies are expected to report the model generalization aspect in model training, model testing, and its impact to positioning accuracy.

- OPPO (R1-2206319) Proposal 1: For the evaluation of AI model generalization, at least the following configurations should be considered for the performance comparison: • Data sets of the same scenario with different settings are used for training and testing, respectively • Different drops of the same scenario with the same setting are used for training and testing, respectively • Mixed data sets of the same scenario with different settings are used for training, and the data set with one setting is used for testing • Mixed data sets of different scenarios are used for training, and the data set with one scenario is used for testing - E.g., InF-DH + InF-SH, different numbers of TRPs or different positioning for TRPs, … Proposal 2: For the evaluation of AI/ML-based positioning accuracy improvement, the data sets from companies should be generated with the same values of the following parameters to keep the results from different sources be comparable: - X drops - Y UEs per drop - FFS: values of X, Y

- CATT (R1-2206395) There are some typical ways to verify the generalization capability of AI model with different training and inference dataset for positioning: - Training and inference dataset from different or mixed drops with the same channel configuration; - Training and inference dataset from different or mixed channel configurations; - Non-ideal parameters, i.e., network synchronization and UE/gNB Rx and Tx timing error, are included in training and inference dataset. Proposal 2: Different drops, the variation of channel configurations and non-ideal parameters could be considered to verify AI model generalization capability.

- Lenovo (R1-2206514) Proposal 2: The evaluation methodology should be designed under a common generalizability framework, including how AI/ML models perform in different channel environments and configuration scenarios.

- Xiaomi (R1-2206639) Proposal 1: Evaluate the generalization capability from the aspect of different cluster parameters Observation 3: - For AI-model trained by dataset generated from one scenario without parameter change, inferior generalization capability is observed - Generating the data set with different cluster parameters could relax the problem of inferior generalization capability

- Samsung (R1-2206824) Proposal 1: RAN1 to study the generalization ability for different data sets at least from different drops and different scenario. Proposal 2: RAN1 to study the generalization ability for imperfect input/output data and how to model the imperfections.

- LG (R1-2206878) Proposal #1: As a metric for generalization of AI/ML model for positioning, consider the difference of the performance or its variance value by taking different deployment scenario and/or UE drop settings into account.

- Nokia (R1-2206972) Proposal-5: For the evaluation of model generalization aspects of AI/ML based positioning, the model performance in terms of horizontal positioning accuracy could be evaluated for a model trained using InF-DH scenario in a particular simulation drop with a certain set of small-scale and large-scale parameters and tested using a different simulation drop of the same scenario with either the same or different sets of small-scale and large-scale parameters. - Other InF scenarios could also be considered as part of generalization evaluations.

- Qualcomm (R1-2207228) - Type 1: Heterogeneous inter-site: Performance of AI/ML model on unseen deployment type (e.g., Umi vs. InF scenarios) - Type 2: Homogeneous inter-site: Performance of AI/ML model on unseen deployment of the same type (e.g., trained on drop 1 and tested on drop 2 of the same scenario) - Type 3: Intra-site: Performance of AI/ML model on unseen variations within the same site (e.g., moving objects, small environment variations over time) - Type 4: Cross-configuration: Performance of AI/ML models across TX/RX configurations (e.g., training and testing can have different beam or transmit powers). Proposal1: Prioritize Type 2 and Type 3 generalization for studying generalization and robustness of AI/ML assisted and direct AI/ML positioning methods. Some aspects of Type 1 generalization may also be further considered as additional evaluation. Proposal3: Study Type 3 generalization by applying slightly different changes in cluster generations between training and testing dataset.

### 1st round discussion

Based on companies’ input, there is a need to define a set of aligned cases for companies to evaluate the model generalization capability.

The following is the most common suggestion among companies.

Proposal 4.2.2-1

To investigate the model generalization capability, at least the following cases are evaluated, in addition to the baseline performance (i.e., training dataset and test dataset are from the same simulation drop):

- Training dataset from one drop, test dataset from a different drop of the same scenario.

- Training dataset from mixed drops, test dataset from mixed drops.

|  | Company |
| --- | --- |
| Support | Nokia/NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | On bullet (b), it’s not clear to us mixed drops will already lead to difference between training and test dataset. Both can be from mixed drop 1+2, but then training and test dataset are actually the same, which is not the intention to evaluate model generalization under different setting. 2. As many companies (including us) have shown, training and testing dataset from different clutter parameters in the same scenario and from different scenarios actually have much bigger performance impact than different drop from the same scenario. 3. We believe the intention of investigation on model generalization is to verify performance gain in wider set-up, not just some picked set-up. |
| Fujitsu | We need to clarify the configuration in bullet (b), does that mean we have two different mixed drops, A contains drop 1+2 and B contains drop 3+4 or both A and B contain drop 1+2? We have already evaluated the former one in our contribution. Besides, we agree with VIVO for not limiting the generalization evaluation to different drops, but we suggest not to have different scenarios such as InF-SH at this moment. |
| CAICT | We are fine in general. The details of different drop meaning should be FFS. |
| LG | Fine in general. Regarding (b), the meaning of mixed drops/dataset can be clarified further. |
| CATT | We have concern about bullet (b). Here is the mixed drop from different scenario or the same scenario? How to differentiate the training dataset and test dataset from mixed drops? |
| QC | Site-specific ML models are not expected to show good generalization to (a) and (b) above. This is something that has been reported by many companies at least for direct AI/ML aspects. Therefore, we think other options that model unseen variations in the same site need to be also included, e.g., Type 3 from our proposal. - Type 3: Intra-site: Performance of AI/ML model on unseen variations within the same site (e.g., moving objects, small environment variations over time) This can be studied, for example, by adding/removing clusters within a given drop. |
| ZTE | At least for direct AI/ML positioning, spatial consistency between training dataset and and test dataset is very important. Without spatial consistency, we cannot foresee the AI model can maintain its performance in test dataset. - For the case without spatial consistency, model retraining or fine-tuning is always necessary. - For the case with spatial consistency, training dataset and test dataset from the same simulation drop is the baseline. Then, we should discuss the variations applied to training dataset and test dataset. For example, training dataset and test dataset share the same large scale parameters but with different small scale parameters. In this case, the model retraining or fine-tuning may not be necessary. |
| OPPO | For bullet (b), some clarifications are needed: test dataset and training dataset are from same mixed drops of the same scenario? |
| Apple | Add mixed clutter parameters |
| HW/HiSi | We are supportive to (a). But considering for evaluating the model generalization capability, there should be one comprehensive proposal for all the dimensions of generalization. We would recommend that this proposal can be refined and treated after the group has provided input on Question 4.2.2-2. For (b) we would like to understand if training and test data sets are from the same mixed drops? Or if (b) means that for training we have the data from drop A+ drop B and for test we have the data from drop A or from drop B? |
| Nokia/NSB | We are fine with this proposal. |
| Samsung | We share the same question with vivo on bullet (b). Can FL explain the intention further? |

There are further dimensions or aspects that can be evaluated for model generalization capability. The group is invited to provide input on if and which of the following aspects should be included also.

Question 4.2.2-2

To investigate the model generalization capability, which aspect(s) is necessary for the evaluation? Which aspect(s) can be optional and up to each company?

- Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}, test dataset from a different clutter parameter {60%, 6m, 2m};

- InF scenarios,  e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH);

- Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error;

- UE/gNB RX and TX timing error, e.g., training dataset without UE/gNB RX and TX timing error, test dataset with UE/gNB RX and TX timing error;

- Other (please describe)

|  | Necessary or Optional? | Company |
| --- | --- | --- |
| - Clutter param | Necessary | Vivo, Fujitsu, CAICT, CATT, Apple, HW/HiSi, Nokia/NSB, Samsung, |
|  | Optional |  |
| (b) InF scenarios | Necessary | Vivo, CAICT, Nokia/NSB, Samsung, |
|  | Optional | Fujitsu, CATT, Apple |
| I Network sync | Necessary | Vivo, Fujitsu, CAICT, CATT, HW/HiSi, Nokia/NSB, Samsung, |
|  | Optional | Apple |
| (d) Timing error | Necessary | HW/HiSi, Nokia/NSB |
|  | Optional | Fujitsu, Apple |
| I Other |  |  |

| Company | Comments |
| --- | --- |
| vivo | 1.As we commented toward proposal 3.4.1-1, we have a concern on UE/gNB RX and TX timing error. First of all, Rel-17 positioning WI already specified and support UE/gNB RX and TX timing error report. It’s not clear to us what could be the remaining error if any after Rel-17 REG/TEG report. Moderator said for proposal 3.4.1-1 that “the simulation assumptions can reuse those of Rel-17 (i.e., Table 6-1 of TR 38.857).” Our understanding is that those assumptions are for Rel-17 SI before the solution introduced in Rel-17 positioning WI. As such, the study should be compared against Rel-17 positioning with REG/TEG feature enabled. 2. As many companies (including us) have shown, training and testing dataset from different clutter parameters in the same scenario and from different scenarios actually have much bigger performance impact than different drop from the same scenario. 3. We believe the intention of investigation on model generalization is to verify performance gain in wider set-up, not just some picked set-up. Although we have not evaluate network sync error, we think it’s necessary. |
| Fujitsu | We take (b) as optional since the scenarios will be basically constant for the indoor factory environments, according to Table 7.2-4 in TR38.901, the ceiling height, BS height and clutter type will be almost fixed for one factory and will not be changed frequently. We also take (d) as optional since more details are necessary for the Timing error. |
| CATT | We also prefer (b) as optional since the scenarios will be basically constant for the indoor factory environments. |
| QC | We propose to study: - Type 2: Homogeneous inter-site: Performance of AI/ML model on unseen deployment of the same type (e.g., trained on drop 1 and tested on drop 2 of the same scenario) - Type 3: Intra-site: Performance of AI/ML model on unseen variations within the same site (e.g., moving objects, small environment variations over time) - and (b) are optional. They are “Type1: Heterogeneous inter-site”. It is not essential for a model to generalize across heterogeneous sites, because one can always develop models specific to each deployment type. I is not needed. The network synchronization is either assumed (as in TDOA) or not assumed (as in RTT). We need to study each case separately. The model does not need to generalize across the two. (d) Timing error between UE/gNB is unavoidable. The model should be trained with timing errors such that it would be able to handle timing errors during inference. |
| Nokia/NSB | We think that network synchronization and timing errors between different UEs/TRPs/gNBs within the same scenario might be different. Thus, we can optionally consider aspects I and (d) within training and testing dataset with varying errors between TRPs/gNBs, especially to account for service that might have stringent positioning requirements. |
| Samsung | The imperfections on the training data should be studied, e.g., noisy/missing CIR, or incorrect location label etc. |

Proposal 4.2.2-3

To investigate the model generalization capability for AI/ML based positioning, impact of clutter parameters is evaluated, in addition to the baseline performance (i.e., training dataset and test dataset are from the same simulation drop):

- Training dataset from scenario with one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from scenario with a different clutter parameter (e.g., {60%, 6m, 2m}), while other simulation parameters are held constant.

### 2nd round discussion

In first round of discussion, companies raised questions about precise meaning of “mixed drop”. The following proposal attempts to clarify “mixed drop”. Based on FL understanding of companies’ input, mixed drop can be either of the following alternatives.

Proposal 4.2.3-1

For investigation of model generalization, mixed drops can be either of the following alternative. In the following, (A, B, C, D) are different drops of the same scenario, hence no spatial consistency among them.

- Training dataset from drop A+B, test dataset from drop C;

- Training dataset from drop A+B, test dataset from drop C+D;

|  | Company |
| --- | --- |
| Support | Fujitsu, Qualcomm (with modification), Nokia/NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | Basically, no need to distinguish Alt1 and Alt2, we just want to make sure the test dataset comprises different drops from the training dataset. The number of drops for both training and test dataset can be decided by companies according to their simulation configurations. |
| OPPO | Same view as Fujitsu. Companies can train and test on different mixed drop(s). The number of drops for training dataset and test dataset can be decided by companies. |
| Xiaomi | In our understanding, there are other alternatives - Alt.3: Training dataset from drop A+B, test dataset from drop A and drop B, respectively In addition, the discussion on the genelization is also under going in section 9.2.1. We think this proposal can be discussed in the AI framework section. |
| CATT | We think Alt.1 and Alt.2 have the same meaning. Also share the same view as Fujitsu and OPPO. The number of drops can be reported by companies. |
| Qualcomm | It is better to consider more than two drops for training, e.g., training dataset from drops {A1, A2,…, AN-1}, test dataset from unseen drops, e.g., drop AN or AN+ AN+1. The two alternatives above can produce different evaluation numbers, but they are more likely to give same conclusion about model generalization. There is probably no need to distinguish between them. |
| HW/HiSi | Agree with Fujitsu that Alt 1 and Alt2 in principle are the same: A drop that has been included training is not included in testing. But, if we go this definition for mixed drops, we also need a definition for that the situation that drops A+B have been used in training and drop A in testing. This scenario is also valuable to investigate for testing. |
| Nokia/NSB | We are supportive of evaluating model generalization with mixed drops within test and training datasets, however we also tend to agree with Fujitsu. We are not sure if we need to differentiate between the two alternatives. In our opinion, the following aspect from Proposal 4.2.3-2 covers these alternatives as well: “Different drops, i.e., Training dataset from one drop, test dataset from a different drop of the same scenario.” |

In first round of discussion, there was also discussion about the exact meaning of “Network synchronization”. FL understanding is, this was part of the agreement from RAN1#109, i.e., the simulation assumptions can reuse those of Rel-17 (i.e., Table 6-1 of TR 38.857), and the network synchronization row is copied below.

| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |
| --- | --- |

Regarding Rel-17 UE/gNB RX and TX timing error, there are divergent views at the moment. There is also dispute if Rel-17 REG/TEG report should be assumed. Thus this aspect is left out at the moment.

Regarding InF scenarios, there are divergent views also. Hence it’s also left out in the proposal below.

With the above clarification, the following is proposed, which includes all the aspects with extensive support.

Proposal 4.2.3-2

To investigate the model generalization capability, at least the following aspect(s) are considered for AI/ML based positioning:

- Different drops, i.e., Training dataset from one drop, test dataset from a different drop of the same scenario.

- Mixed drops.

- Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}, test dataset from a different clutter parameter {60%, 6m, 2m};

- Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error;

|  | Company |
| --- | --- |
| Support | OPPO, CATT, Nokia/NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | 1. In our understanding, (a) and (b) may be redundant in the sense that essentially they are examining the same setup where training and test dataset are from different drops of the same scenario. 2. As we commented before, we think different InF scenarios are important to be evaluated for AI/ML model generalization performance. So we suggest add a bullet to capture that. |
| Xiaomi | For the aspect of clutter parameter, besides the case of different clutter parameters i.e., item (c), we think the case of mixed clutter parameter should be considered as well. We suggest to add the following items Mixed clutter. The training data set are mixed from dataset with clutter parameter A and clutter parameter B. And the test data set are data set with clutter parameter A and data set with clutter parameter B, respectively. |
| ZTE | In our view, (a)(c) has no too much difference as the training dataset and test dataset has no any spatial consistency. We can foresee that the AI model cannot maintain its performance in test dataset as the relationship between channel and UE position has been changed completely. Either case in (a)(c) should always need fine-tuning. For (b), for example, two drops have dropped UEs in the same location. However, even for the same UE location, UE may see totally differently channels as two drops have no spatial consistency. So what’s the meaning for this? Do we mean all the objects/clutters/machinery have been changed/removed so UE may see total different channels even in the same location of the two drops? We think this is not a typical case. A more general scenario would be only some of the objects/clutters/machinery have been changed so that some new channel paths may be blocked or generated. In this case, the channels for a given UE position can still have partial spatial consistency. So we prefer to further study intra-site variation as mentioned in proposal 6.2-2. |
| Qualcomm | Please add an aspect that covers intra-site generalization. Some approaches, e.g., direct AI/ML positioning, are well-understood to not generalize to (a), (b), and (c) above. Therefore, it is good to include an aspect that covers generalization to site-specific changes, e.g., addition and removal of blockers and reflectors. Aspect (b) should be more realistic for consideration than aspect (a). If the ML model is expected to generalize across drops, it is better to be trained on mixture of drops. The same applies to aspect (c). It is better to consider a variant to aspect (c) that covers mixed clutters. For aspect (d), in addition to network synchronization error, other impairments such as UE clock drift as well as UE/gNB RX and TX errors can be evaluated. The modeling of impairments during training and testing should be further discussed. For example, training and testing datasets have similar or different impairments. |
| HW/HiSi | We also wondering about the different between (a) and (b). Could this be clarified? We could add a scenario: Training from drop A+B, Test from B |
| Nokia/NSB | We support this proposal and think that Proposal 4.2.3-1 is covered by this proposal. |

#### Offline discussion

Proposal 4.2.3-1

To investigate the model generalization capability, at least the following aspect(s) are considered for AI/ML based positioning:

- Different drops

    - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1.

- Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m});

- Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error;

- Other aspects are not excluded.

- It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time.

Discussion: Qualcomm: intra-site for generalization study

Based on offline discussion, there is no clear support to discuss how to define the channel model for intra-site.

### 4th round discussion

For investigation of the model generalization, another aspect to discuss is UE/gNB RX and TX timing error. During the earlier discussion, some companies asked whether Rel-17 enhancement features like TEG should be enabled or not.

As several companies commented, this error is unavoidable, thus valuable to investigate. The simulation assumptions can reuse those of Rel-17 (i.e., Table 6-1 of TR 38.857), see below.

| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: X ns for gNB and Y ns for UE - X and Y are up to sources - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |
| --- | --- |

Regarding Rel-17 enhancement features, the agreement below was made in RAN1#109e about Rel-16/Rel-17 positioning methods, i.e., Rel-17 enhancement features (e.g., UE Rx TEG, UE RxTx TEG) can be included as part of baseline performance.

Agreement For AI/ML-based positioning evaluation, the baseline performance to compare against is that of existing Rel-16/Rel-17 positioning methods. - As a starting point, each participating company report the specific existing positioning method (e.g., DL-TDOA, Multi-RTT) used as comparison.

Proposal 4.2.4-1

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(d) UE/gNB RX and TX timing error.

- The baseline non-AI/ML method may enable the Rel-17 enhancement features (e.g., UE Rx TEG, UE RxTx TEG).

Another imperfection that can be investigated is UE clock drift. In Qualcomm contribution (R1- 2207228), UE clock drift is modelled as a random variable within [-150, 150] nanoseconds.

Proposal 4.2.4-2

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(e) UE clock drift.

- UE clock drift is modelled as a random variable in the range of [-n, n] nanoseconds. Value n is up to the participating companies.

Offline Discussion

Since UE/gNB RX and TX timing error is already included in Proposal 4.2.4-1, offline discussion indicates that it is not necessary to additionally include UE clock drift.

Proposal 4.2.4-3

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(e) InF scenarios,  e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH)

Proposal 4.2.4-4

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(f) Intra-site

Offline Discussion

Offline discussion indicates that there is a lack of understanding of how to model intra-site variation at this meeting.

## Intermediate KPI(s)

For AI/ML assisted positioning, the quality of model output is not UE position, and various types of model output have been used by companies. It is pointed out that in addition to the eventual KPI (i.e., UE location), intermediate KPI should be captured as well, i.e., accuracy of model output.

- ZTE (R1-2206072) Proposal 9: For evaluation of AI/ML assisted positioning, companies are encouraged to report the intermediate KPI(s), which provides the accuracy of the AI/ML model output. - Accuracy rate of LOS/NLOS identification - Average value of estimation errors between AI predicted timing/angle measurementz and ideal timing/angle measurements - CDF of the estimation errors at some percentiles (e.g., {50%, 67%, 80%, 90%}) can be optionally presented.

- CATT (R1-2206395) Proposal 2: For AI/ML-based positioning, in addition to AI-specific KPIs, the following KPIs are considered: - Eventual KPI: Positioning accuracy, (e.g. 90% CDF percentiles of horizontal accuracy) - Intermediate KPI: The accuracy of intermediate measurement results (e.g. error of ToA/AoA/AoD) if estimating timing and/or angle of measurement based on AI/ML model is applied. - Intermediate KPI: The correct rate of LOS/NLOS identification if identifying LOS/NLOS identification based on AI/ML model is applied.

- Nokia (R1-2206972) Proposal-6: For evaluation of two-step or AI/ML assisted positioning, intermediate KPI(s) such as the accuracy of LOS/NLOS identification, accuracy of timing and/or angle of measurement, accuracy of the likelihood measurement, etc., should be reported together with the horizontal positioning accuracy.

### 1st round discussion

Reflecting companies’ input, the following is proposed. It is understood that the intermediate KPI is reported in addition to the generally applicable KPI, for example, 90% CDF percentiles of horizontal accuracy, model complexity, computational complexity.

Proposal 4.3.1-1

For evaluation of AI/ML assisted positioning, intermediate KPI(s) is reported, where the intermediate KPI is the accuracy of model output.

|  | Company |
| --- | --- |
| Support | Nokia/NSB |
| Not support | Fujitsu |

| Company | Comments |
| --- | --- |
| vivo | We have a question for clarification. Is the intention to only report intermediate KPI(s) or for AI/ML assisted positioning, both intermediate KPI(s) and final KPI (positioning error) are reported together? We have concern if only intermediate KPI(s) is reported as a high accurate intermediate result may not always lead to the better final positioning accuracy. |
| Fujitsu | We think the report of intermediate results are optional. It is not necessary to report the intermediate results, because the final purpose is to improve the positioning accuracy of the UEs so only the final KPIs are needed. |
| InterDigital | For AIML assisted positioning, both intermediate and final KPI should be evaluated. Intermediate KPI indicates quality of the output of the AIML model. For AIML assisted positioning, final KPI depends on the method that derives the estimated position based on the output of AIML. Thus analyzing the final KPI only may focus on the quality of the method not on AIML. |
| CAICT | Both intermediate KPI(s) and positioning accuracy should be reported. |
| LG | Based on the quality of intermediate KPI is related to that of final KPI and it can be utilized as an assistance information for estimating the position. |
| CATT | The intermediate KPI(s) can be used to AI/ML model performance monitoring in LCM. Thus, we support to report the intermediate KPI(s) and eventual KPI. Maybe we can have some sub-bullets to give some clarification on this proposal, e.g., For evaluation of AI/ML assisted positioning, intermediate KPI(s) is reported, where the intermediate KPI is the accuracy of model output. - For example, the accuracy of LOS/NLOS identification, accuracy of timing and/or angle of measurement, accuracy of the likelihood measurement, etc. - Note: Eventual KPI also need to be reported. |
| QC | We suggest adding a list of accuracy options for difference intermediate quantities. In principle, we are OK with reporting an intermediate KPI(s) for AI/ML assisted positioning. For example, in LOS/NLOS identification, a company can report the precision and recall of the classification accuracy. Similar reasoning can be also applied to other intermediate quantities (e.g., RSTD, RxTx turn-around time, etc.). If the output of the AI/ML assisted algorithm is a distribution (see Section 5 - Likelihood Fusion Section in [1]), then representing KPI can be further discussed. However, we want to note that for the final positioning only a subset of links can be used (e.g., after outlier rejection) and, thus, the intermediate KPIs may not fully reflect the actual positioning performance. [1] R1-2207229, “Other aspects on AI-ML for positioning accuracy enhancement,” Qualcomm Incorporated, 2022 |
| ZTE | Support to report both intermediate KPIs and final KPIs. The intermediate KPIs should be discussed per sub-use case. |
| OPPO | The report of intermediate results can be reported optionally with final KPI. |
| Apple | There needs to be definition of what the KPI is based on the intermediate output and the sub-usage case. TOA difference ? LOS % error ? |
| HW/HiSi | We are not sure of intermediate KPI are needed and would prefer to evaluate the final positioning accuracy instead. The accuracy can be misleading. For example for NLOS identification, we probably will have some misclassifications where a classified LOS in reality is NLOS or vice versa. And a true LOS that has wrongly been classified to be a NLOS, won’t impact the performance if there are still sufficient LOS available for further processing, which also happens in legacy systems. The key point is that the performance of the AI assisted method should be compared to the Rel-17 baseline. |
| Nokia/NSB | We are fine with this proposal, however it is important to note that accuracy as a metric is relevant only for classifier models. We would prefer to further study other relevant intermediate KPIs as well. Our understanding regarding the final positioning KPI (horizontal positioning accuracy) is that it will always be reported. |
| Samsung | The relationship between the intermediate KPI(s) and the horizontal/vertical positioning accuracy may need to be further discussed, and intermediate KPI(s) can only be reported if it helps to improve the positioning accuracy. |

### 4th round discussion

In AI 9.2.1, it has been agreed that intermediate KPI is an important performance metric.

Agreement The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Performance - Intermediate KPIs - Link and system level performance - Generalization performance …

For AI/ML assisted positioning, intermediate KPI is highly relevant. For example,

- If the model output is LOS/NLOS identification, the intermediate KPI is the accuracy of model predicted LOS/NLOS identification;

- If the model output is ToA, the intermediate KPI is the accuracy of model predicted time-of-arrival of the link.

The intermediate KPI is useful for checking the ML model performance, in addition to the agreed KPI of horizonal positioning accuracy. Thus the following is proposed for discussion.

Proposal 4.3.2-1

For evaluation of AI/ML assisted positioning, an intermediate performance metric of model output is reported.

- FFS: Detailed definition of the intermediate performance metric of the model output

## Other KPI

Several other KPIs are also suggested by a few companies, for example, latency, efficiency, overhead, input size.

- LG (R1-2206878) Observation #1: In addition to accuracy aspects, consider latency/efficiency as a major metric where it affects the amount of positioning accuracy in a trade-off manner.

- Google (R1-2206199) Proposal 3: Support UE feedback overhead as a KPI for AI/ML based positioning.

- PML (R1-2205915, R1-2206224) Proposal 7 : For DNN-based LOS/NLOS identification evaluation with transfer learning, adopt the variation scale of DNN parameters as one of KPIs. Proposal 8: For DNN-based LOS/NLOS identification evaluation, adopt the input data size as one of KPIs. Proposal 9: For DNN-based LOS/NLOS identification evaluation, adopt the input image size as one of KPIs.

- Nokia (R1-2206972) Proposal-7: Optional KPIs such as position estimation latency, radio resource efficiency and higher layer signaling overhead should be reported together with the horizontal positioning accuracy.

### 1st round discussion

Regarding the proposal of adding model input size as KPI, the input data size clearly have significant impact to AI/ML implementation, including feature engineering effort, performance, and complexity. In RAN1#109e, it was agreed that companies were encouraged to describe the input of AI/ML model inference, e.g., type and size of model input.

For RAN1#110, Xiaomi (R1-2206639) evaluated the impact from model input size. Otherwise, there has been no dedicated effort to study how to perform rigorous feature engineering to reduce input size. It seems that the following proposal is adequate for this meeting.

Proposal 4.4.1-1

For AI/ML based positioning, if an evaluated model is to be captured in TR38.843, the model input size is reported.

|  | Company |
| --- | --- |
| Support | CATT |
| Not support | QC |

| Company | Comments |
| --- | --- |
| CAICT | We are fine to capture some typical AI model from different companies in TR. The details of model description could FFS. The model input size anyway should be captured. |
| QC | We think companies need first to discuss and agree on what need to be included in the TR38.843 and whether the model structure is part of it before considering this proposal. |
| HW/HiSi | We are not sure if the proposal is needed, because the following agreement have been made for simulation assumptions in RAN1#109e already: Agreement For AI/ML-based positioning evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline. We think the prerequisite in the proposal “if an evaluated model is to be captured in TR38.843” does not exist. |
| Nokia/NSB | We are supportive of capturing details related to model inputs. However, we wonder if the input size alone is relevant if the type is not known. For e.g., a binary vector of size 100 is processed very differently than the same length vector with complex values. Thus, it would be beneficial to clarify more details related to the model input size that needs to be reported. |

For the other candidate KPIs, the group is invited to provide input which of the following is to be reported.

Question 4.4.1-2

Which of the following is considered a KPI for AI/ML based positioning, and expected to be reported?

- Latency

- Resource efficiency (e.g., amount of reference signal needed)

- UE feedback overhead

- Other

|  | Yes/No | Company |
| --- | --- | --- |
| (a) Latency |  | Nokia/NSB (Yes) |
| (b) Resource efficiency |  | Nokia/NSB (Yes) |
| (c) UE feedback overhead |  | CATT(Yes), Nokia/NSB (Yes) |
| (d) Other (please describe) |  |  |

| Company | Comments |
| --- | --- |
| Fujitsu | We want to deprioritize (a) and think (c) should be considered. |
| CATT | Due to the CIR as an input, the UE feedback overhead is an important issue that need to be considered. |
| QC | We do not see the need to report KPIs for latency and resource efficiency. For UE feedback overhead, reporting a KPI depends on the model output, inference location, and whether the model is one- or two-sided. Further clarification is needed. |
| HW/HiSi | Defer the discussion of common KPIs until more progress has been made on common KPIs discussion in agenda 9.2.1. |
| Nokia/NSB | We think positioning latency, (radio/spectral) resource efficiency and signaling overhead are relevant KPIs that could be reported and considered for model evaluation. |

# Dataset

## Companies’ view from contribution

- Huawei (R1-2205894) Proposal 4 : For the training dataset generation of AI/ML-based positioning, support uniform UE distribution.

- PML (R1-2205915, R1-2206224) R1-2205915 Observation 1 : The evaluation of AI/ML-based methods via simulations might not be consistent with that obtained by field test. Observation 2 : Some real channel characteristics that are very helpful for the AI/ML-based LOS/NLOS identification cannot be effectively simulated via the current NR channel model, such as the consistency of NLOS paths and the relative change between LOS path and NLOS paths. Proposal 1 : For AI/ML-based LOS/NLOS identification evaluation, adopt the real measurements as the training inputs. Proposal 2 : For AI/ML-based LOS/NLOS identification, adopt the transfer learning to reduce the data relabeling effort. Figure 1 AL/ML-based LOS/NLOS identification. Proposal 3 : For DNN-based LOS/NLOS identification, adopt the feature space transformation before the DNN. Proposal 4 : For DNN-based LOS/NLOS identification, adopt the channel calibration in the preprocessing. Proposal 5 : For DNN-based LOS/NLOS identification, adopt the DNN well-trained from the similar task for knowledge transfer. Proposal 6 : For DNN-based LOS/NLOS identification, adopt the reference labeled data for fine-tuning during the online training. Proposal 7 : For DNN-based LOS/NLOS identification evaluation with transfer learning, adopt the variation scale of DNN parameters as one of KPIs. Proposal 8: For DNN-based LOS/NLOS identification evaluation, adopt the input data size as one of KPIs. Observation 3: For DNN-based LOS/NLOS identification, adopting transfer learning can reduce the data relabeling effort. Proposal 9: For DNN-based LOS/NLOS identification evaluation, adopt the input image size as one of KPIs. R1-2206224 Proposal 2: When generating training and inference datasets, multiple CSI should be collected in a single UE drop. Figure 4 Flow of the pre-processing Observation 3: When hardware impairments are unavoidable, to improve the output performance of the network, it is of great significance to pre-process CIR to maintain its mode stability. Proposal 3: For AI/ML-based LOS/NLOS identification or fingerprint positioning evaluation, adopt the amplitude and TOF normalized CIR as the training inputs.

- Vivo (R1-2206036) Proposal 5: Study further on model training framworks with low labeled data dependence, including fine tuning, semi-supervised learning, multi-port positioning.

- CATT (R1-2206395) Proposal 1: With the agreed evaluation methodology, the effects of simplified measurement and incomplete coordinate labels need to be considered.

- Lenovo (R1-2206514) Proposal 1: RAN1 to further differentiate whether the evaluations employed data labelling, i.e., used unlabelled (for unsupervised learning) or labelled (supervised learning) simulation data. Alternatively, an indication on whether supervised or unsupervised learning approaches were employed could be beneficial for the performance study.

- NVIDIA (R1-2206524) Proposal 2: Companies are encouraged to contribute real data to evaluate AI/ML based algorithms for positioning accuracy enhancements.

- Samsung (R1-2206824) Proposal 2: RAN1 to study the generalization ability for imperfect input/output data and how to model the imperfections.

- Nokia (R1-2206972) Observation-3: The availability of good quality data with sufficient diversity of positioning ground truth labels with accurate information, for model training and testing/validation is one of the key challenges in AI/ML based positioning. Proposal-10: For evaluation of AI/ML based positioning, consider additional UE distribution options such as sparse or clustered deployment of UEs, while evaluating model performance.

- Qualcomm (R1-2207228) Proposal4: Study specifying a range of user area density in training datasets used for AI/ML positioning evaluation.

## 1st round discussion

For the evaluation, vivo and CAICT raised the issue of incomplete label, and vivo investigated methods with low labelled data dependency. While not all companies may choose to investigate this issue, it is reasonable that companies should at least describe data labelling when reporting the evaluation results. Thus the following is proposed.

Proposal 5.2-1

When providing evaluation results for AI/ML based positioning, participating companies are expected to describe data labelling details, including:

- Meaning of the label (e.g., UE coordinates; binary identifier of LOS/NLOS; ToA)

- Percentage of training data without label, if incomplete labeling is considered in the evaluation

- Imperfection of the ground truth labels, if any

|  | Company |
| --- | --- |
| Support | Fujitsu, CAICT, LG, CATT, QC, ZTE, Apple, Nokia/NSB, Samsung, |
| Not support |  |

| Company | Comments |
| --- | --- |
| OPPO | Generally supportive. On the second bullet, we think that the method to apply training data without label need to be clarified. |
| Nokia/NSB | We are fine with this proposal. Companies could optionally describe whether the labels are accurate or not, for e.g., using a parameter that indicates the trust/accuracy of the labels. |

For the investigation of inadequate ground truth labels, this is very useful exercise.  Thus interested companies are invited to provide more results to share with the group.

Proposal 5.2-2

For evaluation of AI/ML based positioning, companies are encouraged to study the impact from availability of the ground truth labels. The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is also reported.

|  | Company |
| --- | --- |
| Support | Fujitsu, CAICT, QC, Nokia/NSB, Samsung, |
| Not support |  |

| Company | Comments |
| --- | --- |
| CATT | We don’t think unsupervised learning is useful for AI/ML positioning. |
| ZTE | Prefer to study supervised learning as a starting point. |
| Nokia/NSB | We are fine with this proposal, especially related to studying aspects related to availability of ground truth. We are also fine with reporting the learning algorithm, though we wonder how the information is used by the RAN. |

The amount of training data has significant impact to the model performance, as well as practical life cycle management issues like data collection. Following Qualcomm (R1-2207228) suggestion, the following is proposed. For a given floor area, the user area density is equivalently reflected by the training dataset size. Thus the following is proposed.

Proposal 5.2-3

For evaluation of AI/ML based positioning, study the impact from the user area density in the training datasets. A range of user area density is to be evaluated. The user area density is reflected by training dataset size.

|  | Company |
| --- | --- |
| Support | QC, Nokia/NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | Actually, if only offline-training is considered then we may distribute large amount of UEs as many as we want in the simulation to improve the model accuracy so it seems no need to further evaluate this point. |
| QC | We are OK with the proposal in general giving the sampling for training is uniform. |
| ZTE | Prefer to reuse the grid based distribution as agreed in last meeting. No further discussion is needed. |
| Nokia/NSB | We are supportive of this proposal, and believe that sparse deployment of UEs and scenarios with limited dataset are important considerations. However, we think that training dataset size might not fully reflect this aspect. |
| Samsung | The fact is the positioning accuracy will be improved with larger size of training datasets. Higher user area density will lead to larger size of training datasets, and this is the fundamental reason for the performance gain. Considering there are many different method to collect training datasets, it seems unnecessary to study the impact from the user area density in our view. |

## 2nd round discussion

Proposal 5.2-1 has broad support. However, it was not given time during the first online session. Thus it’ll be recommended for second online session.

Proposal 5.2-2 is updated to Proposal 5.3-1 according to companies’ input. The phase “study the impact” is used according to Chairman guidance. For reporting the learning algorithm, this is for sharing information among companies, e.g., whether/how the problem can be resolved with proper learning algorithm. No impact to RAN1 specification is expected. The agreed terminology on the learning algorithms is copied below for reference.

| Supervised learning | A process of training a model from input and its corresponding labels. |
| --- | --- |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |

Proposal 5.3-1

For evaluation of AI/ML based positioning, study the impact from availability of the ground truth labels. The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies.

|  | Company |
| --- | --- |
| Support | NVIDIA, Fujitsu, Apple,ZTE, Qualcomm, Nokia/NSB |
| Not support | HW/HiSi |

| Company | Comments |
| --- | --- |
| OPPO | Generally okay with the proposal. We prefer to study supervised learning as a starting point. |
| Xiaomi | We are OK to study it. But we think low priority should be assigned. At current stage, we prefer to prioritize the evaluation and study by assuming full availability of the ground truth labels |
| ZTE | Agree with OPPO. |
| CATT | Agree with OPPO. |
| HW/HiSi | In our view the availability of ground-truth is an issue that could have impact on all use cases, and the definition what "availability of ground truth" really is supposed to mean is not clear to us. Therefore, it is better to first define a common definition of availability and how to evaluate it in the framework agenda item. Furthermore, we are not sure what is meant with „impact“ of the availability. Could the intention be more clarified, e.g. is it impact on performance if a limited set of ground truth labels is available? In this case, we think it would be better to progress firstly with the sub-use cases and metods, so that more details are known to be able to evaluate. The proposal 5.2-1 already has a bullet that says that companies can report the percentage of labeled data, if incomplete labelling is evaulated. In our understanding this seems to overlap with the proposal here. As otehr companies, we also prefer to start with supervised learning |
| Nokia/NSB | We support this proposal. From our perspective, the availability of ground truth labels imply that we are not entirely sure whether the ground truth labels within the datasets are noisy or contains other forms of error, which might affect the model training and performance evaluations. |

### Offline discussion

Proposal 5.3-1

For evaluation of AI/ML based positioning, study the impact from availability of the ground truth labels. The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies.

- Supervised learning is the starting point

## 4th round discussion

In the first round of discussion, proposal 5.2-3 based on Qualcomm (R1-2207228) suggestion was discussed. Based on companies’ feedback, it’s common understanding that training dataset size (or user area density) affects model performance, i.e., large training dataset leads to better performance. However, there is no quantified results to demonstrate this impact. It’s useful to understand how big the training dataset has to be in order to produce an adequate ML model.

Since the user area density is reflected by training dataset size, the proposal uses training dataset size directly. This avoids the issue of differentiating grid based distribution vs random distribution.

Thus the proposal is presented below for further discussion.

Proposal 5.4-1

For evaluation of AI/ML based positioning, study the impact from the labelled training dataset size, when model finetuning is not assumed. A range of labelled training dataset size is to be evaluated.

# Performed evaluation of direct AI/ML positioning

For RAN1#110, 16 companies submitted simulation results for direct AI/ML positioning. A high-level summary of the evaluation studies is provided in Table 6.1-1, based on companies’ input. For all the evaluation studies below, the ML output is the UE position, i.e., the 2-D coordinates of the target UE.

Table 6.1-1. Summary of AI/ML evaluation for direct AI/ML positioning

|  | UE side or network side or two-sided? | ML input | ML Model | Studied model generalization (Y/N) | Notes |
| --- | --- | --- | --- | --- | --- |
| CATT | One sided | CIR | ResNet | No | Also evaluated performance with network sync error |
|  | Two sided | CIR | ResNet | No |  |
| China Telecom | One sided | TOA, DL-TDOA, RSRP | multilayer perceptron | No |  |
| CMCC | One sided | CIR | CNN-based | Yes |  |
| Ericsson | Network side | ToA | ResNet | Yes |  |
| Fujitsu | UE side | CIR |  | No |  |
| Huawei | Network side | CIR | ResNet | Yes | Extensive study of model generalization in 3 dimensions |
| InterDigital | UE side | RSRP and/or RSTD measurements | ResNet | No |  |
| Nokia | One sided | CIR | CNN | Yes |  |
| NVIDIA | One sided | CIR | CNN | No |  |
| OPPO | UE side | Normalized CIR + RSRP | ResNet | Yes |  |
| Qualcomm | UE side | CIR | ? | Yes | Also evaluated many factors, e.g., spatial consistency, UE clock drift, TRP sync error. Also evaluated using OTA field data; raytracing-generated data |
| Rakuten | One sided | CIR | Conv2D +Dense | Yes |  |
| Samsung | UE side | CIR | ? | No |  |
| vivo | One sided | CIR | Vision Transformer model | Yes | Also evaluated many issues, e.g., various types of ML input. |
| Xiaomi | One sided | CIR | ? | Yes | Also evaluated input size reduction |
| ZTE | UE side | path timing, RSRPP, path phase (i.e., CIR) | CNN | No | Studied single-port vs multi-port PRS |

## Companies’ views and results from contribution

Selected results submitted by companies are copied below.

- Huawei (R1-2205894) Table 5 Performance of AI/ML-based fingerprint positioning under heavy NLOS conditions. BS receiving antennas; Positioning; Positioning Accuracy @90%. Clutter parameters: 60%, 6m, 2m. 32; UL-TDoA in Rel-17 without LOS detection; > 10 m. AI/ML-based fingerprint; 0.5m. 4; UL-TDoA in Rel-17 without LOS detection; > 10 m. AI/ML-based fingerprint; 0.492 m. Clutter parameters: 40%, 2m, 2m. 4; UL-TDoA in Rel-17 without LOS detection; > 10 m. AI/ML-based fingerprint; 0.606 m. Observation 1 : From the evaluation results, it is observed that AI/ML-based fingerprint positioning provides reliable positioning accuracy under a small number of receiving ports. Observation 2 : From the evaluation results, it is observed that AI/ML-based fingerprint positioning provides reliable positioning accuracy under both heavy and moderate NLOS conditions. Table 7 Dataset composition for generalization studies and evaluation results Generalization studies. Test scenario; Training dataset parameters and size; Inference dataset parameters and size; Positioning Accuracy @90%/m. Test scenario 1; Clutter paras:{60%, 6m, 2m} Drop1 25000 samples; Clutter paras:{60%, 6m, 2m} Drop2 5000 samples; >10. Test scenario 2; Clutter paras:{60%, 6m, 2m} 5 Drops 25000 (5000/drop) samples; Clutter paras:{60%, 6m, 2m} Drop 2 (outside of the trained Drops) 5000 samples; 8.04. Test scenario 3; Clutter paras:{60%, 6m, 2m} Drop 1 (inside the trained Drops) 5000 samples; 1.28. Test scenario 4; Clutter paras:{60%, 6m, 2m} Drop1 & 2 25000 (12500/drop) samples; Clutter paras:{60%, 6m, 2m} Drop 1 5000 samples; 0.69. 1sample：1UE * 18BSs * 4 receiving ports. Figure 5 CDF curves of positioning accuracy under Test scenarios 2 to 4 Observation 3 : From Test scenario 1, 3, and 4, when the inference dataset and the training dataset are from different drops, AI/ML-based fingerprint positioning model provides poor generalization performance. But when the mixed training dataset consists of samples from the same drop of the inference dataset, the generalization performance is improved. Observation 4 : From the evaluation results of Test scenario 1 and 2, enriching composition of training dataset can improve unknown drop’s positioning accuracy. Observation 5 : From the evaluation results of Test scenario 3 and 4, the positioning performance for a drop improves when the amount/ratio of data samples from that drop in the mixed training dataset increases. Table 9 Dataset composition for generalization studies and evaluation results Generalization studies. Test scenario; Training dataset parameters and size; Inference dataset parameters and size; Positioning Accuracy @90%/m. Test scenario 6; Clutter paras:{60%, 6m, 2m} Drop 1 12500 samples + Clutter paras:{40%, 2m, 2m} Drop 1 12500 samples; Clutter paras:{60%, 6m, 2m} Drop 1 5000 samples; 0.86. Test scenario 7; Clutter paras:{40%, 2m, 2m} Drop 1 5000 samples; 0.88. Test scenario 8; Clutter paras:{60%, 6m, 2m} Drop 2 5000 samples; >10. Test scenario 9; Clutter paras:{40%, 2m, 2m} Drop 2 5000 samples; >10. 1sample：1UE * 18BSs * 4 receiving ports. Figure 6 CDF curves of Positioning Accuracy under Test scenarios 6 to 9 Observation 7 : From the evaluation results of Test scenario 6 to 9, when the mixed training dataset consists of samples from the same drop and with the same channel parameters of the inference dataset, the positioning performance is improved and reaches the sub-meter level. But when inferred at a different drop, the generalization performance is still poor. Table 10 Dataset composition for generalization studies and evaluation results Generalization studies. Test scenario; Training dataset parameters and size; Inference dataset parameters and size; Positioning Accuracy @90%/m. Test scenario 10; The model trained in Test scenario 8 + Fine-tuned with Clutter paras:{60%, 6m, 2m} Drop 2 1000 samples; Clutter paras:{60%, 6m, 2m} Drop 2 5000 samples; 3.2. Test scenario 11; Clutter paras:{60%, 6m, 2m} Drop 2 1000 samples; Clutter paras:{60%, 6m, 2m} Drop 2 5000 samples; 5.26. 1sample：1UE * 18BSs * 4 receiving ports. Figure 7 CDF curves of Positioning Accuracy under Test scenarios 8, 10 and 11 Observation 8 : From the evaluation results of Test scenario 8 and 10, on top of the dataset with large amount of samples from a different drop, fine-tuned with a relatively small amount of samples from the same drop of the inference dataset will also be helpful to improve the generalization performance. Observation 9 : From the evaluation results of Test scenario 10 and 11, as compared to the training dataset only consisting of a relatively small amount of samples from the same drop of the inference dataset, the training dataset additionally including large amount of samples from a different drop is also beneficial to improve the generalization performance. Generalization studies. Test scenario; Training dataset parameters and size; Inference dataset parameters and size; Positioning Accuracy @90%/m. Test scenario 12; Clutter paras:{60%, 6m, 2m} Without timing error Drop 1 25000 samples; Clutter paras:{60%, 6m, 2m} With UE timing error@T1=10ns Drop 1 5000 samples; 3.12. Test scenario 13; Clutter paras:{60%, 6m, 2m} Without network synchronization error Drop 1 25000 samples; Clutter paras:{60%, 6m, 2m} With network synchronization error@T1=50ns Drop 1 5000 samples; 15.49. 1sample：1UE * 18BSs * 4 receiving ports. Figure 8 CDF curves of Positioning Accuracy under Test scenarios 12 and 13 Observation 10 : From the evaluation results of baseline and Test scenario 12 and 13, when the added UE Tx/Rx timing error are randomly distributed with T_1= 10 ns or the added network synchronization error randomly distributed with T_1= 50ns, AI/ML-based fingerprint positioning model provides poor generalization performance without special consideration of the non-ideal factors.

- vivo (R1-2206036) - Table 1 CDF of positioning accuracy (m) of different positioning methods Scenario; Positioning methods; 50%; 67%; 80%; 90%. InF-DH {0.6,6,2}; DL-TDOA; 8.38; 11.09; 15.95; 32.12. UL-TDOA; 8.60; 11.52; 16.33; 32.81. RTT; 8.32; 11.42; 15.72; 32.41. AOA; 8.13; 10.36; 14.09; 20.16. Machine learning; 0.35; 0.49; 0.70; 0.99. Table 2 CDF of positioning accuracy (m) of different model inputs Scenario; Model inputs; 50%; 67%; 80%; 90%. InF-DH {0.6,6,2}; CIR; 0.35; 0.49; 0.70; 0.99. Power + delay + angle of the first path; 0.51; 0.70; 0.90; 1.19. Power + delay of the first path; 0.52; 0.75; 1.00; 1.31. Delay + angle of the first path; 0.65; 0.84; 1.11; 1.43. Angle + power of the first path; 0.83; 1.14; 1.45; 1.79. Table 3 CDF of positioning accuracy when AI model is transferred to other drops Scenario; Training dataset; Test dataset; 50%; 67%; 80%; 90%. InF-DH {0.6,6,2}; Drop 1; Drop 1; 0.35; 0.49; 0.70; 0.99. Drop 1; Drop 2; 2.76; 3.68; 4.58; 6.00. Drop 1; Drop 3; 2.78; 3.70; 4.62; 5.81.

- ZTE (R1-2206072) Positioning error (m); 50%; 67%; 80%; 90%. 8 path timings + DL PRS RSRPPs; 0.876; 1.154; 1.446; 1.931. 16 path timings + DL PRS RSRPPs; 0.633; 0.851; 1.097; 1.400. 32 path timings + DL PRS RSRPPs; 0.520; 0.659; 0.822; 1.749. 64 path timings + DL PRS RSRPPs; 0.383; 0.512; 0.642; 0.825. 128 path timings + DL PRS RSRPPs; 0.346; 0.461; 0.576; 0.714. 256 path timings + DL PRS RSRPPs; 0.323; 0.420; 0.532; 0.688. Table.3 Positioning errors at some percentiles for sub-use case 1-1(Grid width is 1.0 m) Positioning error (m); 50%; 67%; 80%; 90%. 32 path timings + DL PRS RSRPPs; 0.298; 0.405; 0.509; 0.634. 32 path timings + DL PRS RSRPPs + path phases; 0.290; 0.390; 0.497; 0.645. 64 path timings + DL PRS RSRPPs; 0.243; 0.320; 0.405; 0.508. 64 path timings + DL PRS RSRPPs + path phases; 0.183; 0.245; 0.314; 0.421. 128 path timings + DL PRS RSRPPs; 0.206; 0.275; 0.349; 0.446. 128 path timings + DL PRS RSRPPs + path phases; 0.126; 0.164; 0.212; 0.278. Table.5 Positioning errors at some percentiles for sub-use case 1-2(Grid width is 0.5 m) Observation 1: The AI/ML based positioning method can have excellent performances even in heavy NLOS conditions. Observation 2: With the increase in number of path timings and RSRPPs, positioning performances are improved significantly. Observation 3: With path phase information included in AI model input, positioning performance is improved obviously when compared to AI model input without path phase information. Observation 4: With measurement results from multi-port PRS included in AI model input, increased positioning performance can be observed when compared to AI model input only includes measurement results from single port PRS.

- Fujitsu (R1-2206168) Table 1 Simulation results percentiles of the direct AI method and non-AI DL-TDOA 50%; 67%; 80%; 90%. Direct AI; 2.51; 3.25; 4.08; 5.42. Non-AI DL_TDOA; 8.97; 12.55; 17.32; 23.13. Observation 1 The SLS simulation results of InF-DH scenario by using direct AI method (CIR) is 5.42m in terms of 90% percentile of the positioning accuracy, a dramatic performance improvement can be witnessed compared to the 23.13m for the conventional DL_TDOA method. Statistically, the positioning accuracy can be improved by more than 75% by using direct AI method.

- Google (R1-2206199) Proposal 4: Compared to AI/ML assisted positioning, direct AI/ML positioning should be prioritized. Proposal 5: For direct AI/ML positioning, consider to use CIR and L1-SINR from each cell as the input.

- Ericsson (R1-2206248) Table 9 Percentiles of horizontal errors for ML vs conventional (UL-TDOA) positioning methods Figure 17 An example performance (cumulative-density-function of horizontal positioning errors) of applying ML inference for another scene while keeping the clutter parameter the same Observation 7 With Direct AI/ML positioning, the ML method outperforms the legacy method with a large performance improvement. Observation 8 Direst AI/ML positioning should carefully consider the model generalization issue before deploying the ML model.

- Rakuten Mobile (R1-2206252) Table 8 Positioning accuracy of 90% UE Dataset; Positioning accuracy of 90% UE; Positioning accuracy of 90% UE from [4]. Drop1; 0.67m; 0.46m. Drop80k; 5.55m; 5.23m. Observation 1: AI/ML based positioning using CIR as input provides high accuracy. Proposal 1: AI/ML based positioning with CIR as input should be considered.

- OPPO (R1-2206319) Table 1: Positioning accuracy achieved for direct AI/ML positioning Accuracy achieved @90% (m); w/ spatial consistency; w/o spatial consistency. 1 drop, 80,000 UEs per drop; 0.33; 0.40. 10 drop, 80,000 UEs per drop; 0.52; 0.86. 80,000 drop, 1 UE per drop; 4.35; 4.35. Observation 2: For InF-DH scenario with clutter setting {0.6, 6m, 2m}, no matter the data set is generated by modeling the spatial consistency of small scale parameters or not, the performance gaps are quite large for different alternatives of data set construction. - The number of drops has larger impact than the enabling/disabling of spatial consistency for small scale parameters on the AI/ML performance - When the number of drops increased, the achieved accuracy become worse Table 2: Generalization performance for direct AI/ML positioning (different clutter settings for training and testing) Accuracy achieved @90% (m); w/ spatial consistency; w/o spatial consistency. Training: {60%, 6, 2} Inference: {60%, 6, 2}; 0.33; 0.4. Training: {60%, 6, 2} Inference: {40%, 2, 2}; 13.2; 6.8. Training: {40%, 2, 2} Inference: {60%, 6, 2}; 8.47; 7.2. DL-TDOA: {60%, 6, 2}; 12.3. Observation 3: For the InF-DH scenario, if different clutter settings are used for AI model training and testing, there will be large performance degradation. - Compared to the case w/ spatial consistency of small scale parameters, there will be smaller performance loss in the case w/o spatial consistency of small scale parameters.

- CATT (R1-2206395) (a) Perfect network synchronization (b) Network synchronization with 50 ns synchronization error Figure 2: CDF of horizontal accuracy of one-sided model Observation 1: For directly estimating UE’s positioning based on one-sided AI/ML model, the horizontal accuracy is 0.42m@90% if perfect network synchronization is assumed, and the horizontal accuracy is 2.25m@90% if network synchronization with a truncated Gaussian distribution of 50 ns is assumed. Figure 4: CDF of horizontal accuracy of two-sided model Observation 2: For two-sided model, the horizontal accuracy is degraded to 0.78m@90%, but the resource overhead for transmitting compressed CIR is one third of the original CIR. Proposal 3: For AI/ML-based positioning, two-sided model is supported to be further studied.

- NVIDIA (R1-2206524) Table 9: Summary of CDF percentiles of horizontal positioning accuracy CDF percentile; 50%; 67%; 80%; 90%. ToA; 12.1 m; 17.2 m; 24.1 m; 34.9 m. AI/ML; 1.1 m; 1.5 m; 1.8 m; 2.3 m. Observation 2: AI/ML-based algorithms for direct AI/ML positioning can significantly improve positioning estimation accuracy when compared to classical approaches. Proposal 4: Capture the presented evaluation results in the TR to highlight that direct AI/ML positioning can significantly improve positioning estimation accuracy when compared to classical approaches.

- Xiaomi (R1-2206639) Table 10 Positioning error for typical percentiles for finger printing solution Training data set; Test data set; 50%; 67%; 80%; 90%. Case 1; {60%, 6m, 2m}; {60%, 6m, 2m}; 0.2277; 0.2925; 0.3622; 0.4462. Case 2; {40%, 2m, 2m }; 3.1079; 4.2086; 5.4704; 7.0914. Case 3; {40%, 2m, 2m}; {60%, 6m, 2m}; 0.6833; 0.9120; 1.1738; 1.5328. Case 4; {40%, 2m, 2m }; 0.3997; 0.5141; 0.6245; 0.7566. Case 5; mix of {60%, 6m, 2m} and {40%, 2m, 2m}; {60%, 6m, 2m}; 0.2765; 0.3535; 0.4404; 0.5419. Case 6; {40%, 2m, 2m }; 0.4018; 0.5134; 0.6279; 0.7684. Observation 1: - AI-based solution could greatly improve the positioning accuracy performance for both finger priniting and AI-based ToA prediction - The positioning error is less than 1m for both finger printing and AI-based ToA prediction Observation 2: - The finger printing outperforms the AI-based ToA predication solution sligntly Table 11 Computation in the AI model size and the computation complexity Solution; Input options; AI model size(Bytes); Computation complexity (FLOPs). Finger printing; Option 1; 52.81M; 5.76G. Option 2; 4.95M; 539.95M. AI-based ToA predication; Option 1; 52.81M; 5.76G. Option 2; 4.95M; 539.94M.

- China Telecom (R1-2206689) Table 2: Positioning accuracy for different schemes Methods; 50%; 67%; 80%; 90%. Traditional method; 11.89; 13.62; 14.78; 16.36. AI + TOA; 0.37; 0.46; 0.57; 0.69. AI + DL-TDOA; 0.38; 0.49; 0.59; 0.73. AI + RSRP; 0.27; 0.34; 0.41; 0.52. AI + RSRP +TOA; 0.22; 0.29; 0.36; 0.43. AI + RSRP + DL-TDOA; 0.19; 0.25; 0.31; 0.38. Observations 1: When there are multipath, non-line-of-sight (NLOS), indoor coverage, and non-ideal synchronization in the positioning scenarios, the positioning accuracy of traditional positioning algorithms will be affected. However, AI/ML-based positioning methods can improve positioning accuracy significantly. Observations 2: The positioning accuracy can be further improved when the measurement information is combined as the input of AI/ML model.

- Samsung (R1-2206824) Pos error(m); 50%; 67%; 80%; 90%. ResNet; 0.29; 0.38; 0.48; 0.67. DL-TDOA; 17; 20; 26; 32. Observation 3: At least for data set from the same large-scale and small-scale propagation parameters setting in InF-DH, the AI based positioning method could provide significant improvement comparing to DL TDOA.

- CMCC (R1-2206906) Observation 1: The positioning accuracy is sensitive to the generalization and spatial consistency of the small-scale parameters. Observation 2: For AI/ML based positioning schemes, how to improve the generalization capability over different drops should be studied.

- Nokia (R1-2206972) a: Benchmark two-step positioning b: ML two-step positioning c: ML one-step positioning Exemplary results, InF-DH with clutter parameters {40%, 2m, 2m}: Table 3: Horizontal positioning error in Scenario 1 for random selection of training and test data Positioning Error (m); ML one-step; ML two-step; Peak detection (benchmark scheme). Mean; 3.90; 9.39; 9.62. 80th%; 5.63; 9.95; 10.74. 90th%; 7.24; 12.94; 15.31. 95th%; 8.99; 16.97; 21.04.

- InterDigital (R1-2207094) Table 12 FLOP and horizontal accuracy of Direct AI/ML positioning (m) Simulation Cases; Complexity (FLOP); 50% ile; 67% ile; 80% ile; 90 %ile. Case 1: DL-TDOA; N.A.; 2.2944; 4.5906; 7.9878; 10.9090. Case 2: mRTT; N.A.; 0.5963; 1.0681; 1.6487; 2.4705. Case 3: M1-fingerprinting(RSRP); 18.34 M; 1.6021; 2.0669; 2.6096; 3.2796. Case 4: M2-fingerprinting(RSTD); 9.38 M; 0.6599; 0.8839; 1.1878; 1.6511. Case 5: M3-fingerprinting(RSTD); 37.34 M; 0.6165; 0.8424; 1.1224; 1.5719. Case 6: M4-fingerprinting(RSRP+RSTD); 11.41 M; 0.9465; 1.2377; 1.5658; 2.0405. Observation 1: Direct ML positioning technique achieves better positioning accuracy than conventional positioning techniques (Case 5 vs Case 1&2). Observation 2: Direct positioning technique yields significantly higher positioning accuracy approximately 9.3 m than DL-TDOA positioning at CDF=0.9. (Case 5 vs Case 1). Observation 3: Direct positioning technique results higher positioning accuracy approximately 0.9 m than mRTT at CDF=0.9. (Case 5 vs Case 2) Observation 4: Model complexity in order of ~10 M FLOP achieves better positioning accuracy than conventional positioning techniques. Proposal 1: Adopt direct AI/ML positioning to achieve high positioning accuracy. Proposal 2: Study enhancements to improve positioning accuracy for the UEs dropped on the edge of the deployment area.

- Fraunhofer (R1-2207123) (a); (b) (without clipping). (c) (large errors clipped to 1.2m). Figure 8 - ML based positioning in a real world scenario: (a) scenario TRPs in green, blockers/reflectors in Red and highlighted training areas A1, A2 and A3, (b) and (c) Performance in the whole positioning area

- Qualcomm (R1-2207228) Table 13 Horizontal positioning error (meters) of RFFP with Type 2 generalizations Train; Test; 50%; 67%; 80%; 90%tile. Drop A; Drop A; 1.41; 1.79; 2.19; 2.77. Drop A; Drop B; 5.98; 7.81; 9.88; 12.33. Classical; Drop A; 14.65; 23.75; 36.3; 64.57. Classical; Drop B; 13.88; 21.58; 31.79; 54.61. Observation2: RFFP method is site specific and can provide excellent performance when operated on the site being trained on. It should not be expected to generalize over unseen sites that have entirely different reflections and multipath realization. Observation3: RFFP shows good robustness to new unseen reflections and multipath components. Observation4: RFFP method can show different sensitivities to new unseen intra-site blocking effects depending on which paths get blocked. Observation5: RFFP method shows better robustness to unseen blocking effects when trained on mixture of intra-site changes.

## 1st round discussion

It has been observed by companies that direct AI/ML positioning has advantages in an environment with severe NLOS condition. Thus the following is proposed.

Proposal 6.2-1

For evaluation of direct AI/ML positioning, the deployment scenario includes at least InF-DH with clutter parameter {60%, 6m, 2m}.

|  | Company |
| --- | --- |
| Support | CATT |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | In last meeting, agreements on evaluation assumptions were already made. We don’t think this proposal is needed. |
| Fujitsu | Agree with VIVO. |
| CAICT | Agree with vivo. |
| LG | Same understanding with vivo |
| CATT | The previous agreement is just for InF-DH deployment scenario. This proposal is focus on direct AI/ML positioning, from our point of view. Maybe the proposal can be updated as: For evaluation of dDirect AI/ML positioning is supported and the deployment scenario includes at least InF-DH with clutter parameter {60%, 6m, 2m}. |
| ZTE | Similar view with vivo. |
| OPPO | Agree with vivo. |
| Apple | Same understanding as Vivo |
| Nokia/NSB | We agree with vivo and are not fully sure whether this proposal is required to be agreed. |
| Samsung | Agree with vivo. |

Based on companies’ input, direct AI/ML positioning is site specific, and sensitive to changes in the environment. Thus the following is proposed. For intra-site variations, RAN1 should further discuss exactly how to model it, e.g., if the modification to 38.901 by Qualcomm (R1-2207228) should be adopted.

Proposal 6.2-2

For evaluation of direct AI/ML positioning, the model generalization capability is investigated. The ML model is expected to be trained to handle at least intra-site variations.

- RAN1 discuss how to model intra-site variations.

|  | Company |
| --- | --- |
| Support | QC |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | It’s not clear what the intention of this proposal is. 1. Both direct AI/ML and AI/ML assisted positioning are proposed to be evaluated. Model generalization is also proposed to be investigated. 2. We don’t see agree with “The ML model is expected to be trained to handle at least intra-site variations.” This is speculation before the study. If the intention is to capture observation based on evaluation results, we think this could be discussed when all results are ready. |
| CATT | For generalization, we prefer to discuss details until getting more simulation results from companies. |
| QC | There are many different degrees of intra-site variations, and we cannot make a blanket statement that the ML model is expected to handle any “intra-site variations”. Rather, we propose to say to study ML model generalization for intra-site variations, as we proposed in - Type 3: Intra-site: Performance of AI/ML model on unseen variations within the same site (e.g., moving objects, small environment variations over time) |
| ZTE | This proposal should be discussed in section 4.2, which is related to the evaluation of model generalization. |
| HW/HiSi | In InF scenario, there is only one sector in one site, therefore we are not clear the difference between “intra-site” and “inter-site”. Could this please be elaborated? We think this proposal could inflict maybe unnecessary efforts that divert us from the goal to evaluate the capability of the AI model under various conditions, e.g. that we spend time onto discuss what intra site variations and inter site variation mean. For generalization, it could be straight forward instead to spell out what should be investigated, like discussed in Proposal 4.2.2-1 and Question 4.2.2-2. In our paper, e.g., we have proposed concrete test scenarios that we think would not cause subsequent discussion and could be easier to decide upon: Proposal 7: For evaluation of AI/ML-based positioning, RAN1 should formulate the methodology for evaluating the model generalization, and the following dimensions are proposed to be considered as the baseline: • Dimension 1: various inference dataset from different drops under the same channel parameters; • Dimension 2: various inference dataset from different channel parameters, including at least {60%, 6m, 2m} and {40%, 2m, 2m} under InF-DH scenario; • Dimension 3: various inference dataset with non-ideal assumptions, including the network synchronization error and the UE Tx/Rx timing error |
| Nokia/NSB | We are supportive of this proposal in terms of RAN1 discussing how to model intra-site variations. We propose the following change: Proposal 6.2-2 For evaluation of direct AI/ML positioning, the model generalization capability is investigated. The ML model is expected to be trained to handle at least intra-site variations. RAN1 to discuss how to model intra-site variations. |
| Samsung | Share the same question with vivo. |

Based on the evaluation results submitted to RAN1#110, 13 out of 16 companies used CIR as model input. Thus the following is proposed.

Proposal 6.2-3

For evaluation of direct AI/ML positioning, as a baseline, CIR is used as model input.

|  | Company |
| --- | --- |
| Support | Fujitsu, CAICT, CATT, OPPO, HW/HiSi, Samsung, |
| Not support | QC, ZTE, Nokia/NSB |

| Company | Comments |
| --- | --- |
| Fujitsu | We support to have CIR as the model input. |
| QC | There is no need to restrict the input to one option. |
| ZTE | In current specification, UE/TRP can only report path timings and path RSRPs, so the CIR is not supported. In our view, the baseline should be the measurements that are already supported in spec. We prefer to start from path timing(s) and path RSRP(s) as model input. |
| Nokia/NSB | We agree with ZTE regarding the availability of CIR dataset for model training. Perhaps, it needs to be clarified how the CIR information is acquired, for e.g., a simpler approach could be to use the received signal as a baseline, and then each company may decide what to do with the received signal samples. |

For evaluation of AI/ML assisted positioning, RAN1 should discuss and decide on a common template to capture evaluation results from different companies. This is useful for sharing results among companies, and also useful for TR description in the end.

Discussion 6.2-4

Please share your thoughts on how to define a common template for capturing evaluation results for direct AI/ML positioning. As a starting point, the following is provided for your consideration:

- Basic performance (i.e., training dataset and testing dataset are drawn from the same environment)

  - Model input: input type, input size

  - Model output: output type, output size

  - Model description (e.g., CNN, ResNet)

  - Which side the model is deployed, UE side or network side

  - Key simulation parameters, e.g., clutter parameter

  - Dataset size, including the size of training dataset and test dataset

  - Ground truth label

  - AI/ML complexity: Model complexity and computation complexity

  - Horizontal positioning accuracy at CDF=90%, including the comparison to a conventional non-ML method

- Model generalization investigation

  - Description of {Training dataset, test dataset} pair, e.g., training dataset from Drop 1, test dataset from Drop 2

  - Horizontal positioning accuracy at CDF=90%, as compared to the basic performance above

| Company | Comments |
| --- | --- |
| Fujitsu | We propose to have agreed limited options for each item for companies to choose rather than report freely, e.g., “UE side or network side”, some companies may consider both gNB and LMF as network side. And for “Model description (e.g., CNN, ResNet)”, actually ResNet structure also contains multiple CNNs so it may cause confusion. |

# Performed Evaluation of AI/ML-assisted positioning

For RAN1#110, 12 companies submitted simulation results for AI/ML-assisted positioning. A high level summary of the evaluation studies is provided in Table 7.1-1, based on companies’ input.

Table 7.1-1. Summary of AI/ML evaluation for AI/ML-assisted positioning

|  | UE side or network side or two-sided? | ML Input | ML Output | ML Model | Studied model generalization (Yes/No) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Network side | CIR | LoS/NLoS classification + ToA | CNN | Yes | Also studied how the conventional method use the ML output to achieve better UE position accuracy |
| Fujitsu | UE side | CIR | ToA (?) | multiple DNN | No |  |
| Huawei | Network side | Normalized PDP | LOS/NLOS probability | CNN | Yes |  |
| InterDigital | UE side | RSRP | RSTD value per TRP | ResNet | No |  |
| Nokia | One sided | CIR | LOS/NLOS identification | KNN or FCNN | No (?) |  |
|  | One sided | CIR | TOA | CNN | Yes |  |
| NVIDIA | One sided | CIR | LOS/NLOS probability | CNN | No |  |
| OPPO | UE side | CIR | TOA | ResNet | Yes |  |
| PML | Network side | Transformed channel H | LOS/NLOS probability | CNN | No | Used field test data (Underground parking lot) |
| Qualcomm | UE side | CER | Probability distribution of TDOA | ? | Yes | Also evaluated Umi |
| vivo | One sided | CIR, input size = 4096×1×18 | LOS/NLOS identification; ToA | CNN | Yes |  |
| Xiaomi | One sided | CIR | ToA | ? | Yes |  |
| ZTE | One sided | path timings and RSRPPs | DL PRS RSTD values; Confidence levels of LOS and NLOS | CNN | No |  |

## Companies’ views and results from contribution

Selected results submitted by companies are copied below.

- Huawei (R1-2205894) Table 14 Performance of LOS/NLOS Identification. BS receiving ports; LOS ID method; Positioning Accuracy @90%. Clutter parameters: 40%, 2m, 2m. 32; Baseline LOS ID; 0.484m. 4; Baseline LOS ID; 6.447m. 4; AI/ML LOS ID; 0.353m. Observation 11 : From the evaluation results, it is observed that for a small number of gNB antennas, AI/ML-based LOS/NLOS identification could significantly improve the positioning accuracy. Observation 12 : For AI/ML-based LOS/NLOS identification evaluation, the applied model only needs very small number of parameters and does not require tremendous FLOPs.

- Vivo (R1-2206036) Table 10 CDF of positioning accuracy of different positioning methods Scenario; Methods; 50%; 67%; 80%; 90%. InF-DH {0.6,6,2}; Direct AI/ML positioning (CIR-pos) same drop; 0.35; 0.49; 0.70; 0.99. AI/ML based TOA est. (CIR-TOA-pos) same drop; 0.19; 0.28; 0.40; 0.60. Direct AI/ML positioning (CIR-pos) different drops; 2.76; 3.68; 4.58; 6.00. AI/ML based TOA est. (CIR-TOA-pos) different drops; 0.93; 1.27; 1.73; 2.51. Observation 9: AI/ML based TOA estimation for positioning has great advantages in positioning performance, deployment flexibility, compatibility with existing positioning protocol framework, and generalization capability. Observation 10: AI/ML based LOS/NLOS identification for positioning has the advantages: - More accurate LOS/NLOS identification along with a confidence metric - Better compatibility with existing positioning protocol framework. - Great generalization capability. And disadvantages: - Positioning performance could suffer from severe degradation in heavy-NLOS scenarios. - Obtain LOS/NLOS labels is a challenging task for data collection. Proposal 8: Study further on the benefits of AI/ML assisted positioning for AI/ML based positioning in terms of positioning accuracy and AI model generalization.

- ZTE (R1-2206072) Positioning error (m); 50%; 67%; 80%; 90%. Sub-use case 1-2; 0.126; 0.164; 0.212; 0.278. Sub-use case 2-1; 0.117; 0.163; 0.209; 0.269. Table.7 Positioning errors at some percentiles for sub-use case 1-2 (Grid width is 0.5 m) Observation 5: The AI/ML based positioning method can have excellent accuracy on estimation of DL PRS RSTD values even in heavy NLOS conditions. Observation 6: AI/ML assisted positioning can achieve slightly better positioning performance than direct AI/ML positioning to some degree. Observation 7: The AI/ML based positioning method can have a good accuracy rate of LOS/NLOS identification.

- Fujitsu (R1-2206168) Table 2 Simulation results percentiles of the indirect AI method and non-AI DL-TDOA 50%; 67%; 80%; 90%. Indirect AI; 4.12; 5.61; 7.24; 10.05. Non-AI DL_TDOA; 8.97; 12.55; 17.32; 23.13. Observation 2 The SLS simulation results of InF-DH scenario by using indirect AI method is 10.05m in terms of 90% percentile of the positioning accuracy, a dramatic performance improvement can be witnessed compared to the 23.13m for the conventional DL_TDOA method. Statistically, the positioning accuracy can be improved by more than 50% by using indirect AI method.

- Ericsson (R1-2206248) Figure 8 Final positioning error distributions for using AI/ML outputs with conventional L1 error minimizing positioning solutions on the first test dataset. Table 5 UE positioning errors for the first test dataset at different percentiles. CDF Percentile (%); ML based, UE horizontal position error [m]; Baseline: Non-ML based, UE horizontal position error [m] (copied from Table 3). 50; 0.14; 0.13. 67; 0.19; 1.83. 80; 0.25; 4.82. 90; 0.37; 9.27. Table 7 UE positioning errors for the second test dataset at different percentiles. Performance with the first test dataset is included for comparison CDF Percentile (%); Dataset 2, UE horizontal position error [m]; Dataset 1, UE horizontal position error [m], copied from Table 5. 50; 0.14; 0.14. 67; 0.19; 0.19. 80; 0.25; 0.25. 90; 0.36; 0.37. Observation 1 A single simple ML model can be deployed to all TRPs to generate reliable LoS classification and ToA estimates in the InF-DH environment with {40%, 2m, 2m} clutter parameters. Observation 2 LoS classification false positives can cause significant degradation to conventional UL-TDOA positioning solutions using L2 loss functions minimization even with reliable ToA estimates. Performance can be improved by biasing the LoS logit decision boundary with domain knowledge. Alternatively, preferential weighting between the two classes of classification errors/entropy can be introduced into the training loss function. Observation 3 Conventional UL-TDOA positioning solutions using L1 loss function minimization can achieve high positioning accuracy even in the presence of false positive LoS classification reports. Observation 4 Reliable positioning performance can be achieved by deploying an identical simple AI/ML model to operate independently at different TRPs in the InF-DH environment with {40%, 2m, 2m} clutter parameters. Simple conventional UL-TDOA positioning solutions at the centralized node can be retained to process the reports generated by the TRPs. Observation 5 AI/ML-assisted positioning can substantially improve the UE positioning accuracy for the difficult cases where existing methods tend to fail. Observation 6 A single simple AI/ML model deployed to all TRPs for LoS classification and ToA estimation can generalize to different InF-DH {40%, 2m, 2m} environment realizations. Reliable positioning performance is achieved irrespective of environment change.

- OPPO (R1-2206319) Table 4: Positioning accuracy achieved for AI/ML assisted positioning Accuracy achieved @90% (m). 1 drop, 80000 Ues per drop, w/ spatial consistency; 0.52. 1 drop, 80000 Ues per drop, w/o spatial consistency; 0.72. 10 drops, 80000 Ues per drop, w/ spatial consistency; 1.03. 80000 drops, 1 Ues per drop, w/ spatial consistency; 5.78. Table 5: Generalization performance for AI/ML assisted positioning (different clutter settings for training and testing) Accuracy achieved @90% (m); w/ spatial consistency; w/o spatial consistency. Training: {60%, 6, 2} Inference: {60%, 6, 2}; 0.52; 0.72. Training: {60%, 6, 2} Inference: {40%, 2, 2}; 15.7; 6.65. Training: {40%, 2, 2} Inference: {60%, 6, 2}; 7.94; 7.88. DL-TDOA: {60%, 6, 2}; 12.3.

- NVIDIA (R1-2206524) Table 15: Confusion matrix of LOS/NLOS classification Target class: NLOS; Target class: LOS; Precision. Output class: NLOS; 70120; 45; 99.94%. Output class: LOS; 9; 1539; 99.42%. Recall; 99.99%; 97.16%; Overall accuracy: 99.92%. Observation 3: AI/ML assisted positioning can provide high-fidelity intermediate estimates such as LOS/NLOS classification, which in turn can be used to derive final position estimate. Proposal 5: Capture the presented evaluation results in the TR to highlight that AI/ML assisted positioning can provide high-fidelity intermediate estimates such as LOS/NLOS classification.

- Xiaomi (R1-2206639) Table 16 Positioning error for typical percentiles for AI-based ToA predication Training data set; Test data set; 50%; 67%; 80%; 90%. Case 1; {60%, 6m, 2m}; {60%, 6m, 2m}; 0.2914; 0.3991; 0.5228; 0.6778. Case 2; {40%, 2m, 2m }; 3.4631; 4.5923; 5.8269; 7.1173. Case 3; {40%, 2m, 2m}; {60%, 6m, 2m}; 0.7145; 0.9397; 1.1901; 1.5413. Case 4; {40%, 2m, 2m }; 0.4274; 0.5522; 0.6854; 0.8533. Case 5; mix of {60%, 6m, 2m} and {40%, 2m, 2m}; {60%, 6m, 2m}; 0.2875; 0.4092; 0.5382; 0.6867. Case 6; {40%, 2m, 2m }; 0.3420; 0.4906; 0.6357; 0.7974. Observation 1: - AI-based solution could greatly improve the positioning accuracy performance for both finger rinting and AI-based ToA prediction - The positioning error is less than 1m for both finger printing and AI-based ToA prediction Observation 2: - The finger printing outperforms the AI-based ToA predication solution sligntly Table 17 Computation in the AI model size and the computation complexity Solution; Input options; AI model size(Bytes); Computation complexity (FLOPs). Finger printing; Option 1; 52.81M; 5.76G. Option 2; 4.95M; 539.95M. AI-based ToA predication; Option 1; 52.81M; 5.76G. Option 2; 4.95M; 539.94M.

- Nokia (R1-2206972) a: Benchmark two-step positioning b: ML two-step positioning c: ML one-step positioning Exemplary results, InF-DH with clutter parameters {40%, 2m, 2m}: Table 3: Horizontal positioning error in Scenario 1 for random selection of training and test data Positioning Error (m); ML one-step; ML two-step; Peak detection (benchmark scheme). Mean; 3.90; 9.39; 9.62. 80th%; 5.63; 9.95; 10.74. 90th%; 7.24; 12.94; 15.31. 95th%; 8.99; 16.97; 21.04. Table 7: Computational complexity, model complexity and training dataset size. Two-Step Positioning; One-Step Positioning. Model complexity (parameter size); 3 K; 1.9M. #FLOPs; 200 K; 19.2M. Dataset size; 10k+; 10k+.

- InterDigital (R1-2207094) Table 5 Conventional positioning method using estimated measurements : FLOP, RSTD NMSE, horizontal accuracy Simulation cases; Complexity (FLOP); RSTD NMSE (1016 m); 50% ile; 67% ile; 80% ile; 90 %ile. Case 7: DL-TDOA with actual RSTD measurements; N.A.; N.A.; 2.0004; 4.3218; 8.2266; 11.8934. Case 8: DL-AoD; N.A.; N.A.; 9.6731; 12.8102; 16.4264; 37.1792. Case 9: DL-TDOA with estimated RSTD measurements(M5); 12.61 M; 0.0409; 5.7499; 8.7265; 12.1417; 16.0340. Case 10: DL-TDOA with estimated RSTD measurements(M6); 11.42 M; 0.0445; 5.4290; 7.6270; 12.3024; 15.6817. Observation 5: Positioning based on predicted RSTD measurements yields significantly higher positioning accuracy than DL-AoD positioning technique. Proposal 3: Study further whether AI/ML assisted positioning can be used to estimate observable measurements or unobservable measurements.

- Fraunhofer (R1-2207123) (a) KFEC,10ns = [0,20]; (b) KFEC,10ns = [-10,10]. I KFEC,10ns = [-100,-10]; The results show that especially for critical LOS conditions (“OLOS”) with low KFEC,10ns ML based ToA estimator provide a high gain.. Figure 6: ToA performance with ML-Magnitude, ML-IQ and a classical approach in an early cluster (a) light, (b) moderate, (c) heavy scenarios Observation 5: AI/ML assisted ToA measurement can provide a significant gain especially for critical LOS conditions. Even for very weak first arriving paths can be detected with a measurement error in the range of 1m. Observation 6: Using the complex valued CIR (I and Q value or magnitude and phase) as input to the AI/ML assisted ToA measurements outperforms the measurements using the magnitude only.

- Qualcomm (R1-2207228) Figure 15. CDF of horizontal positioning error for likelihood fusion and classical schemes (InF-DH layout). Observation7: The ML-assisted likelihood fusion method generalizes well across inter-site changes with homogeneous clutter settings. Observation8: Training on a mix of clutter settings achieves good accuracy in each setting without the overhead of model switching, while training a separate model for each setting provides better accuracy. - The 90th percentile error increases from 5.10 m to 7.34 m when testing on (60%, 6m, 2m) clutter, and - from 0.53 m to 0.91 m when testing on (40%, 2m, 2m) clutter Observation9: The ML-assisted likelihood fusion method generalizes well across intra-site changes.

## 1st round discussion

It has been observed by companies that AI/ML assisted positioning is more suitable to an environment with moderate NLOS condition. Thus the following is proposed.

Proposal 7.2-1

For evaluation of AI/ML assisted positioning, the deployment scenario includes at least InF-DH with clutter parameter {40%, 2m, 2m}.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Again, in last meeting, agreements on evaluation assumptions were already made. We don’t think this proposal is needed. Furthermore, it’s not clear to us why we need to pick some different set-up for different positioning methods. |
| Fujitsu | We do not see the point for limiting the clutter parameter to 422, the indirect positioning methods contains measurement enhancement and LOS/NLOS indication which need to be valid under heavy NLOS such as 662 as well. Besides, what is the concern to evaluate 662 for direct and 422 for indirect, both 422 and 662 datasets generated by simulation can be reused for different positioning methods. |
| CAICT | Same view as vivo. |
| CATT | We prefer to keep InF-DH deployment scenario with {60%, 6m, 2m}. Same view as direct AI/ML positioning. Maybe the proposal can be updated as: For evaluation of AI/ML assisted positioning is supported and the deployment scenario includes at least InF-DH with clutter parameter {40%, 2m, 2m}{60%, 6m, 2m}. |
| ZTE | Similar view with vivo. |
| OPPO | Share similar view with vivo. Agreements on evaluation assumptions were already made in last meeting. |
| Apple | Same understanding as Vivo |
| Nokia/NSB | We agree with vivo, and think that this proposal has already been agreed. |
| Samsung | Similar view with vivo. |

Based on evaluation results submitted, AI/ML assisted positioning is observed to have better robustness to environment change. Thus, the following is proposed.  For inter-site variations, RAN1 should further discuss exactly how to model it, e.g., changing random seeds without changing clutter parameter; or changing clutter parameter without changing other simulation assumptions.

Proposal 7.2-2

For evaluation of AI/ML assisted positioning, the model generalization capability is investigated. The ML model is expected to be trained to handle both intra-site variations (e.g., moving objects) and inter-site variations.

- RAN1 discuss how to model inter-site variations.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | It’s not clear what the intention of this proposal is. 1. Both direct AI/ML and AI/ML assisted positioning are proposed to be evaluated. Model generalization is also proposed to be investigated. 2. We don’t see agree with “The ML model is expected to be trained to handle both intra-site variations (e.g., moving objects) and inter-site variations.” This is speculation before the study. If the intention is to capture observation based on evaluation results, we think this could be discussed when all results are ready. |
| CATT | For generalization, we prefer to discuss details until getting more simulation results from companies. |
| QC | Similar comment as above. We should study Type2 and Type3 generalizations. |
| ZTE | This proposal should be discussed in section 4.2, which is related to the evaluation of model generalization. |
| HW/HiSi | In InF scenario, there is only one sector in one site, so we are wondering what “intra-site” means in this case and how it is distinguished from “inter-site”? We think the intention of this proposal is not clear and it can cause a lot of efforts that divert us from the real goal of the discussion to evaluate the capability of the AI model under various conditions. For generalization, it would be straight forward to spell out what should be investigated, like discussed in Proposal 4.2.2-1 and Question 4.2.2-2. In our paper we have also proposed concrete test scenarios that think this would not cause much subsequent discussion and could be easier to decide upon: Proposal 7: For evaluation of AI/ML-based positioning, RAN1 should formulate the methodology for evaluating the model generalization, and the following dimensions are proposed to be considered as the baseline: • Dimension 1: various inference dataset from different drops under the same channel parameters; • Dimension 2: various inference dataset from different channel parameters, including at least {60%, 6m, 2m} and {40%, 2m, 2m} under InF-DH scenario; • Dimension 3: various inference dataset with non-ideal assumptions, including the network synchronization error and the UE Tx/Rx timing error |
| Nokia/NSB | We are supportive of this proposal, please also refer to our proposed change in Proposal 6.2-2. Perhaps it makes sense to combine both proposals into a single proposal. |
| Samsung | Share the same question with vivo. |

Based on the evaluation results submitted to RAN1#110, 8 out of 12 companies used CIR as model input. Thus the following is proposed.

Proposal 7.2-3

For evaluation of AI/ML assisted positioning, as a baseline, CIR is used as model input.

|  | Company |
| --- | --- |
| Support | Fujitsu, CAICT, CATT, OPPO, Samsung, |
| Not support | QC, ZTE, Nokia/NSB |

| Company | Comments |
| --- | --- |
| QC | We do not see a need to restrict the input for baseline |
| ZTE | In current specification, UE/TRP can only report path timings and path RSRPs, so the CIR is not supported. In our view, the baseline should be the measurements that are already supported in spec. We prefer to start from path timing(s) and path RSRP(s) as model input. |
| Nokia/NSB | Similar views as mentioned in Sec. 6.2. |

Based on the evaluation results submitted to RAN1#110, 7 out of 12 companies studied model output of LOS/NLOS identification. Some used binary value output, others used soft value output (i.e., probabilities). Thus the following is proposed.

Proposal 7.2-4

For evaluation of AI/ML assisted positioning, one type of model output is LOS/NLOS identification.

|  | Company |
| --- | --- |
| Support | InterDigital, CAICT, LG, CATT, ZTE, HW/HiSi, Samsung, |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | It is OK to have one type of model output as LOS/NLOS identification, but it seems not necessary to be deliberately written in a proposal. The output of the model should be left for free. |
| QC | We do not see a strong need to discuss output types. We see this as an observation rather than a proposal. |
| Nokia/NSB | We think proposal 7.2-4 and 7.2-5 can be combined into one proposal. However, the important aspect is to report and compare the KPIs related to intermediate features/model outputs. |

Based on the evaluation results submitted to RAN1#110, 8 out of 12 companies studied model output of timing information that are useful for positioning, either ToA (time of arrival) or RSTD (i.e., time difference). Thus the following is proposed.

Proposal 7.2-5

For evaluation of AI/ML assisted positioning, one type of model output is timing information (e.g., ToA, RSTD).

|  | Company |
| --- | --- |
| Support | Fujitsu, InterDigital, CAICT, CATT, OPPO, Samsung, |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | It is OK to have one type of model output as timing information, but it seems not necessary to be deliberately written in a proposal. The output of the model should be left for free. |
| QC | We do not see a strong need to discuss output types. We see this as an observation rather than a proposal. |
| ZTE | Prefer to start from RSTD that is already supported in current specification and has a clear definition. |
| Nokia/NSB | As mentioned above, we think proposal 7.2-4 and 7.2-5 can be combined into one proposal. In our opinion, it is important to agree on relevant KPI to evaluate the performance of these model outputs and their impact on final horizontal positioning accuracy metric. |

For evaluation of AI/ML assisted positioning, RAN1 should discuss and decide on a common template to capture evaluation results from different companies. This is useful for sharing results among companies, and also useful for TR description in the end.

Discussion 7.2-6

Please share your thoughts on how to define a common template for capturing evaluation results for AI/ML assisted positioning. As a starting point, the following is provided for your consideration:

- Basic performance (i.e., training dataset and testing dataset are drawn from the same environment)

  - Model input: input type, input size

  - Model output: output type, output size

  - Model description (e.g., CNN, ResNet)

  - Which side the model is deployed, UE side or network side

  - Key simulation parameters, e.g., clutter parameter

  - Dataset size, including the size of training dataset and test dataset

  - Ground truth label

  - AI/ML complexity: Model complexity and computation complexity

  - How the AI/ML model output is used to obtain the UE’s location

  - Intermediate KPI on model output

  - Horizontal positioning accuracy at CDF=90%, including the comparison to a conventional non-ML method

- Model generalization investigation

  - Description of {Training dataset, test dataset} pair, e.g., training dataset from Drop 1, test dataset from Drop 2

  - Horizontal positioning accuracy at CDF=90%, as compared to the basic performance above

| Company | Comments |
| --- | --- |
| Fujitsu | We propose to have agreed limited options for each item for companies to choose rather than report freely, e.g., “UE side or network side”, some companies may consider both gNB and LMF as network side. And for “Model description (e.g., CNN, ResNet)”, actually ResNet structure also contains multiple CNNs so it may cause confusion. |

## 2nd round discussion

Proposal 7.2-4 and Proposal 7.2-5 are widely supported by companies. For better understanding, the two proposals are combined into one below. It is also clarified that the two types of model output can be used alone or together.

Proposal 7.3-1

For evaluation of AI/ML assisted positioning, one type of model output is LOS/NLOS identification, another type of model output is timing information (e.g., ToA, RSTD). A model may generate one of the two types of output; or generate both types of output.

|  | Company |
| --- | --- |
| Support | NVIDIA, Fujitsu, OPPO, Apple, CATT, HW/HiSi, Nokia/NSB |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| ZTE | Ok for progress. Note that we don’t have clear definition of TOA in current specification. |
| Qualcomm | It is not clear what the intention and benefit of this proposal. We do not see a need to restrict the type of outputs for AI/ML assisted positioning. There are other output types not included in this proposal such as multipath timing, RX-TX time difference, angle information, RSRP, RSRPP, likelihood of measurement (e.g., distribution of timing or angle measurements), etc. |
| Nokia/NSB | We support this proposal. In our opinion, it is important to agree on relevant KPI to evaluate the performance of these model outputs and their impact on final horizontal positioning accuracy metric. |

Regarding the template for reporting evaluation results for AI/ML assisted positioning, the following is proposed. Since the following is the same between direct AI/ML positioning and AI/ML assisted positioning, the same table can be used for both. Note that, the following is different from Discussion 7.2-6:

- “How the AI/ML model output is used to obtain the UE’s location” is left out, since it’s hard to fit into a table. Companies are expected to describe it in separate text.

- “Intermediate KPI on model output” is left out, since this is pending further discussion.

- “Model generalization investigation” is left out, since this is pending further discussion.

Proposal 7.3-2

For both direct AI/ML positioning and AI/ML assisted positioning, the following table is used by companies to report the evaluation results.

| Model input | Model output | Model description | Which side the model is deployed | Clutter param | Dataset size | Ground truth label | AI/ML complexity | Horizontal positioning accuracy at CDF=90% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Company |
| --- | --- |
| Support | Fujitsu, OPPO, Qualcomm, Nokia/NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | The basic structure is OK, if only fundamental evaluation (no model generalization) is required. |
| Xiaomi | Generally we are OK to report the information in the table. And we want to further clarify the following point - Is there any guidance for the model description. For example, companies are only required report the type of AI model or report the detailed model structure - What’s the “AI/ML complexity” referring to? Is it the computation complexity or the AI/ML model complexity or both. |
| ZTE | OK in general. We propose following changes: Model input (i.e., input type and size) Model output (i.e., output type and size) InF channel type (InF-DH or other InF channels) UE distribution type( uniform distribution or grid based distribution) Number of training data used for model training Baseline scheme(i.e., training dataset and test dataset are in the same drop) or model generalization scheme(if applied) |
| CATT | Generally OK. We suggest giving some examples for each column so that companies can align the context of reporting. |
| Qualcomm | We suggest adding an additional column for comments. |
| HW/HiSi | Fine in principle. But we think that Fujitsu has a point when they bring up generalization. In a second step, whether/how generalization has been taken into account could then also be included. Adding a line for comments as QC suggests is also useful. |
| Nokia/NSB | We support this proposal, and agree with Fujitsu that model generalization could be included at a later stage as part of evaluation result reporting. Perhaps, we first need to agree on the metrics/KPI for model generalization. |

### Offline discussion

Proposal 7.3.1-1

For evaluation of AI/ML assisted positioning, one type of model output is LOS/NLOS identification, another type of model output is timing information (e.g., ToA, RSTD). A model may generate one of the two types of output; or generate both types of output.

      - Other types of model output are not precluded.

Proposal 7.3.1-2

For both direct AI/ML positioning and AI/ML assisted positioning, the following table is used by companies to report the evaluation results.

| Model input | Model output | Model description | Which side the model is deployed | Clutter param | Dataset size | Ground truth label | AI/ML complexity | Horizontal positioning accuracy at CDF=90% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note (take into account ZTE input):

- Model input: input type and size

- Model output: output type and size

- Model description (e.g., CNN, ResNet)

- InF channel type: InF-DH or other InF channels

- UE distribution type: uniform distribution or grid based distribution

- Dataset size, including the size of training dataset and test dataset

- Baseline scheme (i.e., training dataset and test dataset are in the same drop) or model generalization scheme(if applied)

## 3rd round discussion

Proposals 7.3.1-1 and 7.3.1-2 have been recommended for 2nd online discussion, but no time was given. They are provided below for further checking and confirmation.

Regarding Proposal 7.3.2-1, it is noted that the sub-bullet is added to address QC concern that other types of model output can still be used by interested companies. The goal is not to restrict model output; the goal is to align evaluations among companies and have easier cross-check.

Proposal 7.3.2-1

For evaluation of AI/ML assisted positioning, one type of model output is LOS/NLOS identification, another type of model output is timing information (e.g., ToA, RSTD). A model may generate one type of output only; or generate both types of output.

      - Other types of model output are not precluded.

|  | Company |
| --- | --- |
| Support | Nokia/NSB,CMCC, Fujitsu |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We support this proposal. In our opinion, it is important to agree on relevant KPI to evaluate the performance of these model outputs and their impact on final horizontal positioning accuracy metric. This could be considered at a later stage as well. |
| Fujitsu | We are fine with this proposal. |
| Qualcomm | The Moderator indicated that the intention of this proposal was not to restrict model output but to help align evaluations among companies. We acknowledge many companies submitted results with LOS/NLOS identification or ToA/TDoA. However, one of the goals of the evaluation should be to explore all potential outputs that enable positioning enhancement and evaluate their relative gains. Hence, we do not support restricting the evaluation of AI/ML assisted positioning to a specific type of output. |

Regarding the table template for reporting evaluation results, the revised proposal is shown below. Changes are:

      - “Which side the model is deployed” is moved outside the table, to save space in the table.

      - “Model description” is moved outside the table, to save space in the table.

      - Considering the progress made on generalization, added “Model generalization investigation” as reported info.

      - Considering the progress made on ground truth label, “Label” column is added.

Proposal 7.3-2

For both direct AI/ML positioning and AI/ML assisted positioning, the following table is used by companies to report the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description]

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Training | test | Model complexity | Computation complexity | Non-AI/ML | AI/ML |

Companies are expected to report the following in table caption:

      - Which side the model is deployed

      - Model generalization investigation, if applied

      - Short model description: e.g., CNN

Further info for the columns:

- Model input: input type and size

- Model output: output type and size

- Label: meaning of ground truth label; percentage of training data set without label if data labeling issue is investigated (default = 0%)

- Clutter parameter: e.g., {60%, 6m, 2m}

- Dataset size, both the size of training dataset and the size of test dataset

- AI/ML complexity: both model complexity in terms of “number of model parameters”, and computational complexity in terms of FLOPs

- Horizontal positioning accuracy: include both accuracy (in meters) of the conventional non-AI/ML method, and the accuracy (in meters) of the AI/ML based method

|  | Company |
| --- | --- |
| Support | Nokia/NSB, Fujitsu |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We would propose a minor change of “Computation complexity” to “Computational complexity” |

## 4th round discussion

For model output of AI/ML assisted positioning, some companies are still interested in capturing it. Thus, the text is updated to formulate an observation.

Observation

For evaluation of AI/ML assisted positioning, majority evaluation results submitted to RAN1#110 used LOS/NLOS identification as one type of model output, and used timing information (e.g., ToA, RSTD) as another type of model output.

Offline Discussion

# Proposals for online discussion

## 1st round proposals for online discussion

Proposal 3.2.1-1

For AI/ML-based positioning, both approaches below are studied by RAN1, and the evaluation results of both are expected to be captured in TR38.843:

- Direct AI/ML positioning

- AI/ML assisted positioning

Proposal 3.4.1-1

For AI/ML-based positioning, companies are encouraged to study impact from implementation imperfections, including:

- Network synchronization error

- UE/gNB RX and TX timing error

- UE clock drift

Proposal 4.1.2-1

For evaluation of AI/ML based positioning, the model complexity is reported via the metric of “number of model parameters”.

Proposal 4.2.2-3

To investigate the model generalization capability for AI/ML based positioning, impact of clutter parameters is evaluated, in addition to the baseline performance (i.e., training dataset and test dataset are from the same simulation drop):

- Training dataset from scenario with one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from scenario with a different clutter parameter (e.g., {60%, 6m, 2m}).

  - Note: other simulation parameters are held constant.

Proposal 5.2-1

When providing evaluation results for AI/ML based positioning, participating companies are expected to describe data labelling details, including:

- Meaning of the label (e.g., UE coordinates; binary identifier of LOS/NLOS; ToA)

- Percentage of training data without label, if incomplete labeling is considered in the evaluation

- Imperfection of the ground truth labels, if any

Combined Proposal 7.2-4 and Proposal 7.2-5

For evaluation of AI/ML assisted positioning, one type of model output is LOS/NLOS identification, another type of model output is timing information (e.g., ToA, RSTD).

Proposal 7.2-4 For evaluation of AI/ML assisted positioning, one type of model output is LOS/NLOS identification. Proposal 7.2-5 For evaluation of AI/ML assisted positioning, one type of model output is timing information (e.g., ToA, RSTD).

## 2nd round proposals for online discussion

Proposal 4.2.3-1

To investigate the model generalization capability, at least the following aspect(s) are considered for AI/ML based positioning:

- Different drops

    - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1.

- Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m});

- Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error;

- Other aspects are not excluded.

- It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time.

Proposal 5.2-1

When providing evaluation results for AI/ML based positioning, participating companies are expected to describe data labelling details, including:

- Meaning of the label (e.g., UE coordinates; binary identifier of LOS/NLOS; ToA)

- Percentage of training data without label, if incomplete labeling is considered in the evaluation

- Imperfection of the ground truth labels, if any

Proposal 5.3-1

For evaluation of AI/ML based positioning, study the impact from availability of the ground truth labels (i.e., some training data may not have ground truth labels). The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies.

- Supervised learning is the starting point

Proposal 7.3.1-1

For evaluation of AI/ML assisted positioning, one type of model output is LOS/NLOS identification, another type of model output is timing information (e.g., ToA, RSTD). A model may generate one type of output only; or generate both types of output.

      - Other types of model output are not precluded.

Proposal 7.3.1-2

For both direct AI/ML positioning and AI/ML assisted positioning, the following table is used by companies to report the evaluation results.

| Model input | Model output | Model description | Which side the model is deployed | Clutter param | Dataset size | Ground truth label | AI/ML complexity | Horizontal positioning accuracy at CDF=90% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note:

- Model input: input type and size

- Model output: output type and size

- Model description (e.g., CNN)

- Clutter parameter (e.g., {60%, 6m, 2m})

- Dataset size, both the size of training dataset and the size of test dataset

- AI/ML complexity: both model complexity in terms of “number of model parameters”, and computational complexity in terms of FLOPs

- Horizontal positioning accuracy: include both accuracy (in meters) of the AI/ML based method, and the accuracy (in meters) of the conventional non-AI/ML method

## 3rd round proposals for online discussion

Proposal 3.3.3-1

For AI/ML-based positioning, RAN1 evaluate the performance of model update. Participating companies report at least the following:

- training dataset setting (e.g., training dataset size necessary for performing model update),

- horizontal positioning accuracy (in meters) before and after model update.

Proposal 7.3-2

For both direct AI/ML positioning and AI/ML assisted positioning, the following table is used by companies to report the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description]

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Training | test | Model complexity | Computational complexity | Non-AI/ML | AI/ML |

Companies are expected to report the following in table caption:

      - Which side the model is deployed

      - Model generalization investigation, if applied

      - Short model description: e.g., CNN

Further info for the columns:

- Model input: input type and size

- Model output: output type and size

- Label: meaning of ground truth label; percentage of training data set without label if data labeling issue is investigated (default = 0%)

- Clutter parameter: e.g., {60%, 6m, 2m}

- Dataset size, both the size of training dataset and the size of test dataset

- AI/ML complexity: both model complexity in terms of “number of model parameters”, and computational complexity in terms of FLOPs

- Horizontal positioning accuracy: include both accuracy (in meters) of the conventional non-AI/ML method, and the accuracy (in meters) of the AI/ML based method

Proposal 7.3.2-1

For evaluation of AI/ML assisted positioning, one type of model output is LOS/NLOS identification, another type of model output is timing information (e.g., ToA, RSTD). A model may generate one type of output only; or generate both types of output.

      - Other types of model output are not precluded.

## 4th round proposals for online discussion

Proposal 4.3.2-1

For evaluation of AI/ML assisted positioning, an intermediate performance metric of model output is reported.

- FFS: Detailed definition of the intermediate performance metric of the model output

Proposal 4.2.4-1

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(d) UE/gNB RX and TX timing error.

- The baseline non-AI/ML method may enable the Rel-17 enhancement features (e.g., UE Rx TEG, UE RxTx TEG).

Proposal 4.2.4-3

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH)

# Conclusion

The agreements reached in RAN1#110 agenda item 9.2.4.1 are copied below.

Agreement

For AI/ML-based positioning, both approaches below are studied and evaluated by RAN1:

- Direct AI/ML positioning

- AI/ML assisted positioning

Agreement

For AI/ML-based positioning, study impact from implementation imperfections.

Agreement

For evaluation of AI/ML based positioning, the model complexity is reported via the metric of “number of model parameters”.

Agreement

To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning:

- Different drops

    - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1.

- Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m});

- Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error;

- Other aspects are not excluded.

Note: It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time.

Agreement

When providing evaluation results for AI/ML based positioning, participating companies are expected to describe data labelling details, including:

- Meaning of the label (e.g., UE coordinates; binary identifier of LOS/NLOS; ToA)

- Percentage of training data without label, if incomplete labeling is considered in the evaluation

- Imperfection of the ground truth labels, if any

Agreement

For evaluation of AI/ML based positioning, study the performance impact from availability of the ground truth labels (i.e., some training data may not have ground truth labels). The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies.

Agreement

For AI/ML-based positioning, for evaluation of the potential performance benefits of model finetuning, report at least the following:

- training dataset setting (e.g., training dataset size necessary for performing model finetuning)

- horizontal positioning accuracy (in meters) before and after model finetuning.

Agreement

For both direct AI/ML positioning and AI/ML assisted positioning, the following table is adopted for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description]

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Training | test | Model complexity | Computational complexity | AI/ML |

To report the following in table caption:

      - Which side the model is deployed

      - Model generalization investigation, if applied

      - Short model description: e.g., CNN

Further info for the columns:

- Model input: input type and size

- Model output: output type and size

- Label: meaning of ground truth label; percentage of training data set without label if data labeling issue is investigated (default = 0%)

- Clutter parameter: e.g., {60%, 6m, 2m}

- Dataset size, both the size of training/validation dataset and the size of test dataset

- AI/ML complexity: both model complexity in terms of “number of model parameters”, and computational complexity in terms of FLOPs

- Horizontal positioning accuracy: the accuracy (in meters) of the AI/ML based method

Note: To report other simulation assumptions, if any.

Agreement

For evaluation of AI/ML assisted positioning, an intermediate performance metric of model output is reported.

- FFS: Detailed definition of the intermediate performance metric of the model output

Agreement

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(d) UE/gNB RX and TX timing error.

- The baseline non-AI/ML method may enable the Rel-17 enhancement features (e.g., UE Rx TEG, UE RxTx TEG).

# References

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP RAN#96, June 2022.

R1-2205894	Evaluation on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2205915	Evaluation on AI/ML for positioning accuracy enhancement	PML

R1-2206036	Evaluation on AI/ML for positioning accuracy enhancement	vivo

R1-2206072	Evaluation on AI for positioning enhancement	ZTE

R1-2206168	Preliminary evaluation results and discussions of AI positioning accuracy enhancement	Fujitsu

R1-2206199	On Evaluation of AI/ML based Positioning	Google

R1-2206224	Evaluation method on AI/ML for positioning accuracy enhancement	PML

R1-2206248	Evaluation of AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2206252	Evaluation on AI/ML for positioning accuracy enhancement	Rakuten Mobile, Inc

R1-2206319	Evaluation methodology and preliminary results on AI/ML for positioning accuracy enhancement	OPPO

R1-2206395	Evaluation on AI/ML for positioning	CATT

R1-2206514	Discussion on AI/ML Positioning Evaluations	Lenovo

R1-2206524	Evaluation of AI and ML for positioning enhancement	NVIDIA

R1-2206639	Evaluation on AI/ML for positioning accuracy enhancement	Xiaomi

R1-2206679	Some discussions on evaluation on AI-ML for positioning accuracy enhancement	CAICT

R1-2206689	Evaluation on AI/ML for positioning accuracy enhancement	China Telecom

R1-2206824	Evaluation on AI ML for Positioning	Samsung

R1-2206878	Evaluation on AI/ML for positioning accuracy enhancement	LG Electronics

R1-2206906	Discussion on evaluation on AI/ML for positioning accuracy enhancement	CMCC

R1-2206972	Evaluation of ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

R1-2207094	Evaluation on AI/ML for positioning accuracy enhancement	InterDigital, Inc.

R1-2207123	Evaluation on AI/ML for positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2207228	Evaluation on AI/ML for positioning accuracy enhancement	Qualcomm Incorporated

(Withdrawn) R1-2207332	Evaluation on AI/ML for positioning accuracy enhancement	Apple

# Appendix A. Selected Agreements from RAN1#109e

In RAN1#109e meeting for the AI PHY SI, the following agreements were made for AI 9.2.4.1 (Evaluation on AI/ML for positioning accuracy enhancement).

Agreement The IIoT indoor factory (InF) scenario is a prioritized scenario for evaluation of AI/ML based positioning. Agreement For evaluation of AI/ML based positioning, at least the InF-DH sub-scenario is prioritized in the InF deployment scenario for FR1 and FR2. Agreement For InF-DH channel, the prioritized clutter parameters {density, height, size} are: - {60%, 6m, 2m}; - {40%, 2m, 2m}. - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. Agreement For evaluation of AI/ML based positioning, reuse the common scenario parameters defined in Table 6-1 of TR 38.857. Agreement For evaluation of InF-DH scenario, the parameters are modified from TR 38.857 Table 6.1-1 as shown in the table below. - The parameters in the table are applicable to InF-DH at least. If another InF sub-scenario is prioritized in addition to InF-DH, some parameters in the table below may be updated. Agreement For AI/ML-based positioning evaluation, the baseline performance to compare against is that of existing Rel-16/Rel-17 positioning methods. - As a starting point, each participating company report the specific existing positioning method (e.g., DL-TDOA, Multi-RTT) used as comparison. Agreement For all scenarios and use cases, the main KPI is the CDF percentiles of horizonal accuracy. - Companies can optionally report vertical accuracy. Agreement The CDF percentiles to analyse are: {50%, 67%, 80%, 90%}. - 90% is the baseline. {50%, 67% 80%} are optional. Agreement Target positioning requirements for horizonal accuracy and vertical accuracy are not defined for AI/ML-based positioning evaluation. Agreement For evaluation of AI/ML based positioning, the KPI include the model complexity and computational complexity. - FFS: the details of model complexity and computational complexity Agreement Synthetic dataset generated according to the statistical channel models in TR38.901 is used for model training, validation, and testing. Agreement The dataset is generated by a system level simulator based on 3GPP simulation methodology. Agreement As a starting point, the training, validation and testing dataset are from the same large-scale and small-scale propagation parameters setting. Subsequent evaluation can study the performance when the training dataset and testing dataset are from different settings. Agreement For AI/ML-based positioning evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline. Agreement The entry “UE horizontal drop procedure” in the simulation parameter table for InF is updated to the following. UE horizontal drop procedure; Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - (baseline) the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. - (optional) the convex hull of the horizontal BS deployment, and the CDF values for positioning accuracy is obtained from the convex hull.. Agreement The entries “UE antenna height” and “gNB antenna height” in the simulation parameter table for InF is updated to the following. UE antenna height; Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH). …; …. gNB antenna height; Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}.. Agreement If spatial consistency is enabled for the evaluation, companies model at least one of: large scale parameters, small scale parameters and absolute time of arrival, where - the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance = for InF (Section 7.6.3.1 of TR 38.901) - the small scale parameters are according to Section 7.6.3.1 of TR 38.901 - the absolute time of arrival is according to Section 7.6.9 of TR 38.901 Agreement If spatial consistency is enabled for the evaluation of AI/ML based positioning, the baseline evaluation does not incorporate spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). - It is optional to implement spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). Agreement For evaluation of AI/ML based positioning, companies are encouraged to evaluate the model generalization. - FFS: the metrics for evaluating the model generalization (e.g., model performance based on agreed KPIs under different settings) Agreement Companies are encouraged to provide evaluation results for: - Direct AI/ML positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - AI/ML assisted positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - details of the output of the AI/ML model inference, how the AI/ML model output is used to obtain the UE’s location Agreement When reporting evaluation results with direct AI/ML positioning and/or AI/ML assisted positioning, proponent company is expected to describe if a one-sided model or a two-sided model is used. - If one-sided model (i.e., UE-side model or network-side model), the proponent company report which side the model inference is performed (e.g. UE, network), and any details specific to the side that performs the AI/ML model inference. - If two-sided model, the proponent company report which side (e.g., UE, network) performs the first part of interference, and which side (e.g., network, UE) performs the remaining part of the inference. Agreement For evaluation of AI/ML based positioning, the computational complexity can be reported via the metric of floating point operations (FLOPs). - Note: For AI/ML assisted methods, computational complexity for the AI/ML model is only one component of the overall complexity for estimating the UE’s location. - Note: Other metrics to measure the computational complexity are not precluded. Agreement For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable): - The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset; - The distribution of UE location for generating the training dataset may be one of the following: - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m. - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area.

The following agreements were made for AI 9.2.4.2 (Other aspects on AI/ML for positioning accuracy enhancement).

Agreement Study further on sub use cases and potential specification impact of AI/ML for positioning accuracy enhancement considering various identified collaboration levels. - Companies are encouraged to identify positioning specific aspects on collaboration levels if any in agenda 9.2.4.2. - Note1: terminology, notation and common framework of Network-UE collaboration levels are to be discussed in agenda 9.2.1 and expected to be applicable to AI/ML for positioning accuracy enhancement. - Note2: not every collaboration level may be applicable to an AI/ML approach for a sub use case Agreement For further study, at least the following aspects of AI/ML for positioning accuracy enhancement are considered. - Direct AI/ML positioning: the output of AI/ML model inference is UE location - E.g., fingerprinting based on channel observation as the input of AI/ML model - FFS the details of channel observation as the input of AI/ML model, e.g. CIR, RSRP and/or other types of channel observation - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement - E.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement - FFS the details of input and output for corresponding AI/ML model(s) - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - Companies are encouraged to clarify all details/aspects of their proposed AI/ML approaches/sub use case(s) of AI/ML for positioning accuracy enhancement Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection - AI/ML model indication/configuration - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection) - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output - UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring) - Other aspects are not precluded - Note: not all aspects may apply to an AI/ML approach in a sub use case - Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1