3GPP TSG RAN WG1 Meeting #110															R1-2208169

Toulouse, France, August 22 – 26, 2022

Source: 	Moderator (NTT DOCOMO, INC.)

Title:	Summary#3 on draft CRs on CAPC for DL

Agenda item:	7.2.2

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 7.2.2 regarding channel access priority class for downlinks.

- Summary of the submitted contributions

Below are provided in the contributions for the RAN1#110 meeting.

| Companies | Proposals |
| --- | --- |
| NTT DOCOMO, INC [1] | Propose the following CR: ### 4.1.1 Type 1 DL channel access procedures This clause describes channel access procedures to be performed by an eNB/gNB where the time duration spanned by the sensing slots that are sensed to be idle before a downlink transmission(s) is random. The clause is applicable to the following transmissions: - Transmission(s) initiated by an eNB including PDSCH/PDCCH/EPDCCH, or - Any transmission(s) initiated by a gNB. The eNB/gNB may transmit a transmission after first sensing the channel to be idle during the sensing slot durations of a defer duration Td and after the counter N is zero in step 4. The counter N is adjusted by sensing the channel for additional sensing slot duration(s) according to the steps below: 1) set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4; 2) if N>0 and the eNB/gNB chooses to decrement the counter, set N=N-1; 3) sense the channel for an additional sensing slot duration, and if the additional sensing slot duration is idle, go to step 4; else, go to step 5; 4) if N=0, stop; else, go to step 2. 5) sense the channel until either a busy sensing slot is detected within an additional defer duration Td or all the sensing slots of the additional defer duration Td are detected to be idle; 6) if the channel is sensed to be idle during all the sensing slot durations of the additional defer duration Td, go to step 4; else, go to step 5; If an eNB/gNB has not transmitted a transmission after step 4 in the procedure above, the eNB/gNB may transmit a transmission on the channel, if the channel is sensed to be idle at least in a sensing slot duration Tsl when the eNB/gNB is ready to transmit and if the channel has been sensed to be idle during all the sensing slot durations of a defer duration Td immediately before this transmission. If the channel has not been sensed to be idle in a sensing slot duration Tsl when the eNB/gNB first senses the channel after it is ready to transmit or if the channel has been sensed to be not idle during any of the sensing slot durations of a defer duration Td immediately before this intended transmission, the eNB/gNB proceeds to step 1 after sensing the channel to be idle during the sensing slot durations of a defer duration Td. The defer duration Td consists of duration Tf=16μs immediately followed by mp consecutive sensing slot durations Tsl, and Tf includes an idle sensing slot duration Tsl at start of Tf. CWmin,p≤CWp≤CWmax,pis the contention window. CWp adjustment is described in clause 4.1.4. CWmin,p and CWmax,p are chosen before step 1 of the procedure above. mp, CWmin,p, and CWmax,p are based on a channel access priority class p associated with the eNB/gNB transmission, as shown in Table 4.1.1-1. An eNB/gNB shall not transmit on a channel for a Channel Occupancy Time that exceeds Tmcot,p where the channel access procedures are performed based on a channel access priority class p associated with the eNB/gNB transmissions, as given in Table 4.1.1-1. If an eNB/gNB transmits discovery burst(s) as described in clause 4.1.2 when N>0 in the procedure above, the eNB/gNB shall not decrement N during the sensing slot duration(s) overlapping with discovery burst(s). A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including discovery burst(s) satisfying the conditions described in this clause. A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including neither discovery burst(s) satisfying the conditions described in this clause nor unicast PDSCH(s) with user plane data. A gNB shall use a channel access priority class applicable to the unicast user plane data multiplexed in PDSCH for performing the procedures above to transmit transmission(s) including unicast PDSCH with user plane data. For p=3 and p=4 , if the absence of any other technology sharing the channel can be guaranteed on a long term basis (e.g. by level of regulation), Tmcot,p=10ms, otherwise, Tmcot,p=8ms. Table 4.1.1-1: Channel Access Priority Class (CAPC) Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tmcot,p; allowed CWpsizes. 1; 1; 3; 7; 2 ms; {3,7}. 2; 1; 7; 15; 3 ms; {7,15}. 3; 3; 15; 63; 8 or 10 ms; {15,31,63}. 4; 7; 15; 1023; 8 or 10 ms; {15,31,63,127,255,511,1023}.. with the following information: Reason for change:; During NR-U maintenance discussed at RAN1#104bis-e, it was agreed that Type 1 channel access procedure is applicable to any DL transmission. Meanwhile, how to decide CAPC for type 1 channel access prior to DL transmission is currently described for DL including DB or unicast PDSCH only (i.e., no description on CAPC for type 1 channel access prior to other DL). Moreover, CAPC for DL includling neither DB nor unicast PDSCH has not been discussed so far.. Summary of change:; - Clarify how to decide CAPC for type 1 channel access procedure prior to DL transmission(s) not including DB or unicast PDSCH.. Consequences if not approved:; Incomplete support for NR-U. |
| WILUS Inc. [2] | - Proposal 1: We propose to have one of the following options on CAPC selection for transmission initiated by a gNB including PDCCH only transmission. - Alt-1: - We propose to have the highest priority (i.e., CAPC, p=1) for transmission initiated by a gNB including PDCCH only transmission with DCI format 2_x series for other purposes. - We propose to follow the CAPC of UL data scheduled by the UL grant for transmission including PDCCH only transmission with UL grant only. - The detail text proposal in TS 37.213 can be provided if this principle above is agreed. - Alt-2: If left undefined on selecting CAPC for that transmission, it needs to be captured in the Chairman's Note as follows: - It is up to a gNB’s implementation on CAPC selection for transmission initiated by a gNB including PDCCH only transmission with DCI format 2_x series for other purposes which is applicable to NR-U without user plane data or DCI format 0_x (i.e., UL grant) without user plane data. - Alt-3: Adopt the following text proposal in section 4.1.1. on TS 37.213 ### 4.1.1 Type 1 DL channel access procedures This clause describes channel access procedures to be performed by an eNB/gNB where the time duration spanned by the sensing slots that are sensed to be idle before a downlink transmission(s) is random. The clause is applicable to the following transmissions: - Transmission(s) initiated by an eNB including PDSCH/PDCCH/EPDCCH, or - Any transmission(s) initiated by a gNB. The eNB/gNB may transmit a transmission after first sensing the channel to be idle during the sensing slot durations of a defer duration Td and after the counter N is zero in step 4. The counter N is adjusted by sensing the channel for additional sensing slot duration(s) according to the steps below: 1) set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4; 2) if N>0 and the eNB/gNB chooses to decrement the counter, set N=N-1; 3) sense the channel for an additional sensing slot duration, and if the additional sensing slot duration is idle, go to step 4; else, go to step 5; 4) if N=0, stop; else, go to step 2. 5) sense the channel until either a busy sensing slot is detected within an additional defer duration Td or all the sensing slots of the additional defer duration Td are detected to be idle; 6) if the channel is sensed to be idle during all the sensing slot durations of the additional defer duration Td, go to step 4; else, go to step 5; If an eNB/gNB has not transmitted a transmission after step 4 in the procedure above, the eNB/gNB may transmit a transmission on the channel, if the channel is sensed to be idle at least in a sensing slot duration Tsl when the eNB/gNB is ready to transmit and if the channel has been sensed to be idle during all the sensing slot durations of a defer duration Td immediately before this transmission. If the channel has not been sensed to be idle in a sensing slot duration Tsl when the eNB/gNB first senses the channel after it is ready to transmit or if the channel has been sensed to be not idle during any of the sensing slot durations of a defer duration Td immediately before this intended transmission, the eNB/gNB proceeds to step 1 after sensing the channel to be idle during the sensing slot durations of a defer duration Td. The defer duration Td consists of duration Tf=16μs immediately followed by mp consecutive sensing slot durations Tsl, and Tf includes an idle sensing slot duration Tsl at start of Tf. CWmin,p≤CWp≤CWmax,pis the contention window. CWp adjustment is described in clause 4.1.4. CWmin,p and CWmax,p are chosen before step 1 of the procedure above. mp, CWmin,p, and CWmax,p are based on a channel access priority class p associated with the eNB/gNB transmission, as shown in Table 4.1.1-1. An eNB/gNB shall not transmit on a channel for a Channel Occupancy Time that exceeds Tmcot,p where the channel access procedures are performed based on a channel access priority class p associated with the eNB/gNB transmissions, as given in Table 4.1.1-1. If an eNB/gNB transmits discovery burst(s) as described in clause 4.1.2 when N>0 in the procedure above, the eNB/gNB shall not decrement N during the sensing slot duration(s) overlapping with discovery burst(s). A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including discovery burst(s) satisfying the conditions described in this clause. A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including PDCCH only transmission without user plane data. A gNB shall use a channel access priority class applicable to the unicast user plane data multiplexed in PDSCH for performing the procedures above to transmit transmission(s) including unicast PDSCH with user plane data. For p=3 and p=4 , if the absence of any other technology sharing the channel can be guaranteed on a long term basis (e.g. by level of regulation), Tmcot,p=10ms, otherwise, Tmcot,p=8ms. Table 4.1.1-1: Channel Access Priority Class (CAPC) Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tmcot,p; allowed CWpsizes. 1; 1; 3; 7; 2 ms; {3,7}. 2; 1; 7; 15; 3 ms; {7,15}. 3; 3; 15; 63; 8 or 10 ms; {15,31,63}. 4; 7; 15; 1023; 8 or 10 ms; {15,31,63,127,255,511,1023}.. |

In summary:

- Issue:

  - Both [1] and [2] argue similar issue, that is, channel access priority class (CAPC) to be used for type 1 channel access procedure prior to some DL transmission(s) is not clear in 37.213 [3].

- Target scenario:

  - [1] focuses on CAPC to be used for type 1 channel access procedure prior to transmission(s) including neither discovery burst(s) satisfying the conditions described in this clause nor unicast PDSCH(s) with user plane data.

  - [2] focuses on DL transmission(s) including PDCCH only transmission without user plane data

- Proposed resolutions:

  - [1] propose to clarify that any CAPC can be used for type 1 channel access procedure prior to the targeted DL transmission(s)

  - [2] provides the three alternatives below:

    - Alt-1: CAPC to be used for type 1 channel access procedure prior to the targeted DL transmission(s) is either

      - (1) always 1, or

      - (2) follow the same rule as for CAPC to be used for type 1 channel access procedure by UE prior to UL data scheduled by the UL grant

    - Alt-2: to conclude in Chair’s note as follows:

It is up to a gNB’s implementation on CAPC selection for transmission initiated by a gNB including PDCCH only transmission with DCI format 2_x series for other purposes which is applicable to NR-U without user plane data or DCI format 0_x (i.e., UL grant) without user plane data.

    - Alt-3: clarify that any CAPC can be used for type 1 channel access procedure prior to the targeted DL transmission(s)

      - Moderator would like to note that Alt-3 in [2] is almost equivalent to the text proposal in [1], although there is a difference on the targeted DL transmission(s)

- Discussion

1st Round Discussion:

Companies are invited to input their views for the following three questions, i.e., Q1-1, Q1-2 and Q1-3:

Q1-1: Does the argued issue exist (i.e., CAPC for some DL is not clear)?

| Company | Short answer (Yes/No) | Comments |
| --- | --- | --- |
| DOCOMO | Yes | The issue was identified during SI phase but left untouched so far. One reason could be, RAN1 didn’t consider type 1 channel access procedure itself for initiating DL transmission including neither unicast PDSCH with user plane data nor discovery burst at the end of WI, that can be seen in 37.213 V16.5.0: ### 4.1.1 Type 1 DL channel access procedures This clause describes channel access procedures to be performed by an eNB/gNB where the time duration spanned by the sensing slots that are sensed to be idle before a downlink transmission(s) is random. The clause is applicable to the following transmissions: - Transmission(s) initiated by an eNB including PDSCH/PDCCH/EPDCCH, or - Transmission(s) initiated by a gNB including unicast PDSCH with user plane data, or unicast PDSCH with user plane data and unicast PDCCH scheduling user plane data, or - Transmission(s) initiated by a gNB with only discovery burst or with discovery burst multiplexed with non-unicast information, where the transmission(s) duration is larger than 1ms or the transmission causes the discovery burst duty cycle to exceed 1/20.. Therefore, CAPC did not need to be considered either, at least at the time. It was modified in RAN1#104bis-e so that any DL transmission from gNB can be initiated by performing type 1 channel access procedure as in the latest 37.213. However, CAPC was not considered during the CR discussion in our understanding. |
| Intel | No | It is our understanding that similarly as LTE-LAA, it was indeed left up to implementation which CAPC a gNB should use, and therefore nothing was captured in the spec. Therefore, we believe no further action is needed. |
| ZTE, Sanechips | No | We share same view with Intel, that is, it’s up to the implementation for gNB side. |
| WILUS | Yes | For transmission initiated by a gNB including PDCCH only transmission with DCI format 2_x series for other purposes which is applicable to NR-U without user plane data or DCI format 0_x (i.e., UL grant) without user plane data in same channel, which CAPC is used to perform Type 1 channel access procedure should be defined. |
| Qualcomm | Yes | We believe the CAPC is left to gNB implementation. However, if it is common understanding that gNB implementation is used if not explicitly defined in the spec, it is fine for us no further clarification is needed. |
| Huawei, HiSilicon | Yes | The mapping of CAPC to channel generated from higher layer is defined in the section of TS38.300. The only missing parts are the channels and signals only generated in PHY, e.g. PDCCH only transmission. |
| Samsung | Yes | Agree with the assessment from DOCOMO and WILUS. |
| vivo | Yes |  |

Q1-2: If the answer to Q1 is ‘yes’, which DL transmission(s) is/are affected by the issue?

| Company | Comments |
| --- | --- |
| DOCOMO | We believe any DL transmission including neither DB nor unicast PDSCH with user plane data should be considered since all of them can be initiated by type 1 channel access procedure. |
| WILUS | We think that at least DL transmission initiated by a gNB including PDCCH only transmission with DCI format 2_x series should be considered. |
| Qualcomm | Share the same view as DOCOMO |
| Huawei, HiSilicon | the channels and signals only generated in PHY, e.g. PDCCH only transmission |
| Samsung | Agree with DOCOMO’s comment. The DL transmission can includs PDCCH only, or DL RS standalone transmission. |
| vivo | Agree with Docomo |

Q1-3: If the answer to Q1 is ‘yes’, which resolution should be taken for DL transmission(s) identified in Q2?

- Alt-1: no discussion/conclusion

- Alt-2: having conclusion in chair note

  - Alt-2a: concluding that “any CAPC can be used for type 1 channel access procedure prior to the DL transmission(s) identified in Q2

- Alt-3: agree a CR

  - Alt-3a: any CAPC can be used for type 1 channel access procedure prior to the DL transmission(s) identified in Q2

  - Alt-3b: the highest CAPC (i.e., p=1) is used for type 1 channel access procedure prior to the DL transmission(s) identified in Q2

  - Alt-3c: the CAPC used for type 1 channel access procedure prior to the DL transmission(s) identified in Q2 is determined by the same rule as for the one performed by UE prior to UL data transmission scheduled by the UL grant

| Company | Short answer (Alt#) | Comments |
| --- | --- | --- |
| DOCOMO | Alt-3a | Our best preference is Alt-3a. From operator’s perspective, it is preferred to have freedom on CAPC selection even for DL transmission including neither DB nor unicast PDSCH with user plane data. Alt-3b doesn’t achieve such configurability. We are not sure how Alt-3c works since CAPC for UL PUSCH is determined per the content of the PUSCH based on higher layer specification. Our understanding is that we now discuss on PHY signaling, to which the same rule is not applicable as it is. Alt-2A may be one way, but not our preference. As per another agreement in WI, 37.213 describes any CAPC can be selected by gNB for DL transmission(s) including DB satisfying the conditions specified in 4.1.1 of 37.213. This conclusion may result in some confusion at reader side because of different specification status for different DL transmissions, for which the CAPC determination is actually identical. Another approach to avoid such confusion could be to remove CAPC determination rule for CAPC, however, we are not sure if it is acceptable for the group since it was generated per an explicit agreement. |
| WILUS | Alt-3b (can accept Alt-3a) | Our 1st preference is Alt-3b for DL transmission including PDCCH only transmission with DCI format 2_x series. However, we can also accept Alt-3a. |
| Qualcomm | Alt-3a | We prefer Alt-3a if additional clarification is needed |
| Huawei, HiSilicon | Alt-3a |  |
| Samsung | Alt-31 | Alt-3a should be more aligned with current specification wording and spirit. |
| vivo | Alt-3a | We suggest merge the following text to make spec simple. A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including discovery burst(s) satisfying the conditions described in this clause. A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including neither discovery burst(s) satisfying the conditions described in this clause nor unicast PDSCH(s) with user plane data. e.g. A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) which doesn’t include unicast PDSCH(s) with user plane data. |

1st Round Summary:

- Q1-1: Does the argued issue exist (i.e., CAPC for some DL is not clear)?

- Q1-2: If the answer to Q1 is ‘yes’, which DL transmission(s) is/are affected by the issue?

- Q1-3: If the answer to Q1 is ‘yes’, which resolution should be taken for DL transmission(s) identified in Q2?

  - Alt-1: no discussion/conclusion

  - Alt-2: having conclusion in chair note

    - Alt-2a: concluding that “any CAPC can be used for type 1 channel access procedure prior to the DL transmission(s) identified in Q2

  - Alt-3: agree a CR

    - Alt-3a: any CAPC can be used for type 1 channel access procedure prior to the DL transmission(s) identified in Q2

    - Alt-3b: the highest CAPC (i.e., p=1) is used for type 1 channel access procedure prior to the DL transmission(s) identified in Q2

    - Alt-3c: the CAPC used for type 1 channel access procedure prior to the DL transmission(s) identified in Q2 is determined by the same rule as for the one performed by UE prior to UL data transmission scheduled by the UL grant

For Q1-1, companies have a common understanding on the exact behavior on how to choose CAPC for type 1 channel access procedure prior to the DL transmission(s) being considered. Regarding whether we need to change the spec or not, the following is the latest situation:

- Yes: DOCOMO, WILUS, Qualcomm (ok), Huawei, HiSilicon, Samsung, vivo

- No: Intel, ZTE, Sanechips

For Q1-2, 6 companies have their own inputs as follows:

- any DL transmission including neither DB nor unicast PDSCH with user plane data

  - DOCOMO, Qualcomm, Samsung, vivo

- PDCCH only transmission

  - WILUS, HW, Hisilicon

For Q1-3: 5 companies have their own inputs:

- Alt-3a: DOCOMO, WILUS (ok), Qualcomm, HW, Hisilicon, Samsung, vivo (with another suggestion)

- Alt-3b: WILUS (preferred)

Below is some comments from Moderator:

Moderator would like to point out that the applicability of type 1 channel access procedure itself is slightly different between LAA and NR-U as per the following agreement in RAN1#104bis-e:

TP#2 in R1-2102325, the two TPs in Annex A of R1-2102938, and alternative TP for option 1 in R1-2103212 (same as TP1’ in R1-2103554) are endorsed. R1-2104069 (TS37.213, CR0018, Rel-16) Correction on the conditions for DL channel access procedure Moderator (Nokia), Samsung, NTT DOCOMO, Ericsson

<Unchanged parts are omitted> ### 4.1.1 Type 1 DL channel access procedures This clause describes channel access procedures to be performed by an eNB/gNB where the time duration spanned by the sensing slots that are sensed to be idle before a downlink transmission(s) is random. The clause is applicable to the following transmissions: - Transmission(s) initiated by an eNB including PDSCH/PDCCH/EPDCCH, or - Any transmission(s) initiated by a gNB. 1ms1/20 . <Unchanged parts are omitted>

Therefore, Moderator is not very sure if LAA can be referred to on deciding whether the spec change is needed. Also note that any proposal doesn’t have any impact on LAA (or even on already specified NR-U).

Given the (small) majority, Moderator propose the following, while expecting some discussions in online.

Proposal:

- Adopt the CR based on R1-2207376

2nd Round Discussion:

Companies’ views for Q1-1/Q1-2/Q1-3 are summarized as follows (copied from the previous section):

For Q1-1, companies have a common understanding on the exact behavior on how to choose CAPC for type 1 channel access procedure prior to the DL transmission(s) being considered. Regarding whether we need to change the spec or not, the following is the latest situation:

- Yes: DOCOMO, WILUS, Qualcomm (ok), Huawei, HiSilicon, Samsung, vivo

- No: Intel, ZTE, Sanechips

For Q1-2, 6 companies have their own inputs as follows:

- any DL transmission including neither DB nor unicast PDSCH with user plane data

  - DOCOMO, Qualcomm, Samsung, vivo

- PDCCH only transmission

  - WILUS, HW, Hisilicon

For Q1-3: 5 companies have their own inputs:

- Alt-3a: DOCOMO, WILUS (ok), Qualcomm, HW, Hisilicon, Samsung, vivo (with another suggestion)

- Alt-3b: WILUS (preferred)

A

Moderator’s reading/assessment are the following

- It seems that the most controversial point is whether the specification should fix the issue or not (i.e., Q1-1). So far 6 companies are supportive, 1 company is ok if it is needed, and 3 companies do not support.

  - Moderator thinks it might be good to have some further discussion for this point.

- For Q1-2 (i.e., which DL transmission(s) should be considered when fixing the issue), 4 companies proposed any DL transmission including neither DB nor unicast PDSCH with user plane data. 3 companies (WILUS, HW, Hisilicon) proposed to focus on PDCCH only transmission.

  - It seems WILUS is open to consider another DL transmission. It also seems that HW/Hisilicon’s point is applicable to some other DL transmission. Given that, moderator thinks it could be ok to focus on any DL transmission including neither DB nor unicast PDSCH with user plane data.

- For Q1-3 (i.e., which resolution to be taken when fixing the issue), almost all companies are supportive/ok with Alt3-1a. Meanwhile, vivo proposed another alternative.

  - Moderator thinks vivo’s alternative deserves some further discussion. It can be simpler and cleaner, while it will impact on some existing specification text.

Given above, moderator would like to trigger some further discussion in the following table:

Q2-1: Regarding the need of fixing the issue, any further view from companies to convince the other camp?

| Company | Comments |
| --- | --- |
| DOCOMO | As clarified above, as per the previous CR agreed in RAN1#104bis-e, the situation on type 1 channel access procedure applicability is different between LAA and NR-U. Because of the CR, for NR-U, CAPC determination procedure is clearly described for a part of DL transmission only. This can be a source of confusion. Since this issue doesn’t exist for LAA DL, LAA has no relation here. |

Q2-1: Regarding the exact CR for this issue (if needed, of course), which of the following is better?

Original Alt3-1a:

### 4.1.1 Type 1 DL channel access procedures <Unchanged parts are omitted> A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including discovery burst(s) satisfying the conditions described in this clause. A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including neither discovery burst(s) satisfying the conditions described in this clause nor unicast PDSCH(s) with user plane data. A gNB shall use a channel access priority class applicable to the unicast user plane data multiplexed in PDSCH for performing the procedures above to transmit transmission(s) including unicast PDSCH with user plane data. <Unchanged parts are omitted>

vivo’s update on original Alt3-1a:

### 4.1.1 Type 1 DL channel access procedures <Unchanged parts are omitted> A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) which doesn’t include unicast PDSCH(s) with user plane data. A gNB shall use a channel access priority class applicable to the unicast user plane data multiplexed in PDSCH for performing the procedures above to transmit transmission(s) including unicast PDSCH with user plane data. <Unchanged parts are omitted>

| Company | Comments |
| --- | --- |
| DOCOMO | We think vivo’s update is indeed very nice in terms of simplicity. Since our motivation is to just to clarify the issue, we are also fine with vivo’s suggestion. The only issue, which may be quite small, is that vivo’s suggestion has some impacts on the existing text, which may not be very preferred by the group. If some companies do not prefer it, then we think it would be good to just go with original Alt-3a. |

2nd Round Summary:

There is no additional input other than NTT DOCOMO in this round. Based on the inputs in 1st/2nd round, Moderator would like to propose the following:

Proposal

Endorse in principle the draft CR to TS37.213 in R1-2207376 and prepare the final CR

3rd Round:

After some offline discussions with interested companies, Moderator has identified that there are some companies (e.g., Ericsson, Intel) that opposes the CRs strongly.

Given that, Moderator thinks the following is the only way forward in this meeting:

Proposed conclusion (updated by HW):

gNB may use any channel access priority class for performing type 1 channel access procedure to transmit DL transmission(s) if it is not stated in the specifications

Please check whether the proposed conclusion would work for you. If not, you can leave your comment below (although Moderator assumes the discussion will be just closed in that case).

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | As well as TS37.213, other specifications also include the mapping of CAPC, such as TS38.300. To be safe, I would suggest the a general description as following. “gNB may use any channel access priority class for performing type 1 channel access procedure to transmit DL transmission(s) if it is not stated otherwise in Clause 4.1.1 of TS 37.213 in the specifications.” ” |
| Moderator | I think HW’s suggestion is trying to avoid overlooking any relevant specification, which should be ok considering this is a conclusion. I have updated accordingly and will ask Mr. Chair to check whether it is acceptable in online session. |

- Conclusion

[TBD]

Reference

- R1-2207376, Draft CR on CAPC for DL, NTT DOCOMO, INC.

- R1-2207594, Correction on DL channel access procedure for NR-U	, WILUS Inc.

- TS 37.213 V16.9.0