3GPP TSG RAN WG1 Meeting # 110			R1-2208182

Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0319 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CSI-RS port restriction for mTRP CSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-8-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In R15/16, the maximum number of CSI-RS ports contained by each resource per CSI-RS resource set is 16 when Ks=2 and is 8 when 2<Ks≤8. For multi-TRP CSI feedback configured for given CSI-RS resource set, the UE complexity to process individual single-TRP resource, e.g. for Mode 2 which will determine multi-TRP or single-TRP CSI to be reported by UE itself, should not be increased beyond R15/16 restrictions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reuse Rel-15/16 CSI-RS port restrictions for each CMR group of CSI-RS resource set, depending on value of max{K1, K2} |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional UE complexity is needed if the restriction on CSI-RS port number in Rel-15/16 is not applied to CMR groups for Rel-17 CSI. UE may under-report the supported max number of ports per CSI-RS resource for NC-JT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated impact analysis: This CR has no isolated impact on network and UE hehavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | First version. |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

5.2.1.4.2	Report Quantity Configurations

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RI-LI-PMI-CQI', 'cri-SINR', or 'cri-SINR-Capability[Set]Index ', and Ks>1 resources are configured in the corresponding resource set for channel measurement, then the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet (if configured for CSI-ReportConfig with reportQuantity set to 'cri-SINR' or 'cri-SINR-Capability[Set]Index ') for interference measurement. If Ks=2 CSI-RS resources are configured, each resource shall contain at most 16 CSI-RS ports. If CSI-RS resources are configured, each resource shall contain at most 8 CSI-RS ports.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with  resources, two Resource Groups with  resources in Group 1,  resources in Group 2, , and  Resource Pairs:

-	each resource can contain, subject to UE capability, at most 32 CSI-RS ports. For two Resource Groups with Ki  resources (i=1,2), if  maxK1,K2=1, the resource in NZP-CSI-RS-ResourceSet shall contain at most 32 CSI-RS ports; if   maxK1,K2=2, each resource in NZP-CSI-RS-ResourceSet shall contain at most 16 CSI-RS ports; if  2<maxK1,K2<8, each resource in NZP-CSI-RS-ResourceSet shall contain at most 8 CSI-RS ports.

-	each of the  Resource Pairs is associated to a CRI value.

-	The CSI-ReportConfig may be configured with higher layer parameter sharedCMR.  and  are the numbers of resources associated to a CRI value, other than the N CRIs defined above, in Group 1 and Group 2, respectively, with , such that the total number of CRI values configured for the CSI-ReportConfig is M+N.

< Unchanged parts are omitted >