3GPP TSG-RAN WG1 Meeting #110bis-e	  R1-2208430

e-Meeting, October 10 – 19, 2022

Agenda Item:	9.2.2.2

Source:	Huawei, HiSilicon

Title:	Discussion on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

Introduction

In RAN1#109-e meeting [1] and RAN1#110 meeting [2], sub use cases and potential specification impact for CSI feedback enhancement have been discussed and several agreements have been achieved. This contribution will further discuss other sub use cases and the potential specification impact for CSI feedback enhancement.

Sub use cases

Spatial-frequency domain CSI compression involving temporal domain compression

In RAN1#109-e meeting [1], spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

  - Note: Study of other sub use cases is not precluded.

  - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

In the previous meetings, there have been discussions on whether/how to consider the temporal-spatial-frequency domain CSI compression, which also adopts a two-sided model with auto-encoder structure while the encoder and the decoder include an additional LSTM module for each to accumulate the temporal domain information for the compression purpose. The description of the AI/ML solution is provided in our companion contribution [3], according to which, the EVM including input/output information from the outside of the AI/ML model, the quantization/dequantization method, and the potential pre/post-processing, are the same as the spatial-frequency domain CSI compression without involving temporal domain compression. Similarly, the specification impact of the spatial-frequency domain CSI compression without involving temporal domain compression can be reused to the spatial-frequency domain CSI compression involving temporal domain compression. On the other hand, when involving temporal domain compression, a significant performance gain can be additionally achieved as per the evaluation results, which better justifies the benefits of the application of AI/ML on the CSI compression sub use case.

Therefore, we prefer to consider the spatial-frequency domain CSI compression involving temporal domain compression (i.e., temporal-spatial-frequency domain CSI compression) as a specific implementation of spatial-frequency domain CSI compression, other than a separate sub use case.

Proposal 1: Categorize spatial-frequency domain CSI compression involving temporal domain compression using two-sided model as a specific AI/ML solution under spatial-frequency domain CSI compression for sub use case study.

CSI prediction

As per our evaluations as shown in our companion contribution [3], AI/ML-based CSI prediction can achieve better performance than the case without CSI prediction (i.e., using the nearest history CSI). On the other hand, as the EVM for CSI compression may not be fully reused for CSI prediction, it will require relatively more efforts to discuss the specification impact and EVM. Therefore, whether to determine the CSI prediction as a representative sub use case can be considered after the EVM is clearer and more evaluation results from companies are raised to justify the gain.

Joint CSI prediction and compression

The benefits of joint CSI prediction and compression over two separate procedure including CSI prediction and CSI compression are not clear. This sub use case can be studied after separately performing the study of CSI compression and study of CSI prediction.

One-sided model based on traditional codebook

It seems this sub use case can be achieved by implementation and we don’t see strong necessity to study it in this SI unless more details are revealed.

Proposal 2: For AI/ML-based CSI feedback enhancement use case, joint CSI prediction and compression as well as one-sided model based on traditional codebook are NOT selected as representative sub-use cases.

Potential specification impact

In the last RAN1 meeting, potential specification impact for CSI feedback enhancement has been discussed from several aspects, and agreements have been reached for some aspects. In this section, we will further discuss the potential specification impact for CSI feedback enhancement.

In the last meeting, the life cycle management has been discussed in the FL summary [4]:

- Proposal 3-4-2(v2): In CSI compression using two-sided model use case, further study CSI compression specific potential specification for life-cycle management procedure, e.g., [model selection], [model configuration], model activation/de-activation, model switching across various configurations/scenarios.

In our view, some of the procedures in the life cycle management can be more or less common to all use cases, such as [model registration], [model configuration], model activation/deactivation, model selection, model switching. But as the definitions of model registration and model configuration are still unclear, they can be FFS. In this section, the specification impacts of the procedures in life cycle management specific to CSI feedback use case are discussed in individual sub sections in below, including data collection, training, updating, inference, monitoring, fallback, and UE capability.

Proposal 3: For the study of life cycle management for CSI feedback use case, discuss use case specific procedures in 9.2.2.2, including data collection, training, updating, inference, monitoring, fallback, and UE capability.

- FFS: [model registration], and [model configuration]

Data collection

In the last meeting, the following agreement has been achieved for data collection.

Agreement In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact, for data collection for AI/ML model training/inference/update/monitoring: - Assistance signaling for UE’s data collection - Assistance signaling for gNB’s data collection - Delivery of the datasets

In the following, the discussions on data collection for ground-truth CSI, assistance signaling and delivery of the dataset are provided.

Data collection for ground-truth CSI

In order to support AI/ML model training/updating/monitoring at the Network side, data collection for the ground-truth CSI at the Network side is required. Several options are analyzed in the following.

- Option 1: Use the ground-truth CSI from simulation platform or test field

- Option 2: Use the ground-truth CSI of realistic UL channels measured by Network

- Option 3: Use the ground-truth CSI of realistic DL channels measured by UE and reported to Network

From the evaluation results for generalization in our companion contribution, when an AI/ML model trained under Scenario#A dataset but applied to Scenario#B for inference, the performance may be degraded if the two scenarios have different channel characteristics. In general, the channel characteristics from simulation platform/test field and realistic network are not the same, thus Option 1 may not adapt well to the diverse and varying realistic scenarios.

Option 2 works if the UL channel and DL channel are reciprocal. However, as a high prioritized applicable case for CSI feedback, the FDD system generally cannot provide good enough reciprocity between UL and DL for the AI/ML model trained with UL but inference with DL. Even though the reciprocal requirement for training dataset is not strict, the difference between TX and RX settings may also bring some uncertainties for using dataset constructed by UL channels to train AI/ML models for CSI feedback. Moreover, the accuracy of model monitoring relies more heavily on the precision of the ground-truth CSI which should be the exact measured DL CSI instead of being acquired from UL measurement.

Option 3 can make use of the realistic data samples to much better adapt to the realistic scenarios due to the ground-truth CSI report from UE to Network. To take the role of labels, the ground-truth CSI should have higher resolution than the legacy PMI. On the other hand, the overhead of the ground-truth CSI can be studied so that the concerns on transmitting ground-truth CSI over the air-interface can be relieved. To analyze the overhead of the ground-truth CSI report, how to quantize the ground-truth CSI and feedback to the Network can be studied in advance. For offline training, data collection is not required to be real-time and thus there is no strong restriction on the time duration of the data collection procedure in the realistic network. In addition, considering the period of performing training/updating/monitoring can be relatively long due to slow varying channel characteristics in a cell, data collection is not required to be performed very frequently neither. Therefore, the average overhead for the report of ground-truth CSI would not be a critical issue. Moreover, some compression methods can be adopted to largely reduce the overhead of the ground-truth CSI.

An example of the overhead analysis for Option 3 is provided in Table 1, where two compression methods are provided, including 1) Float32 without compression, and 2) an enhanced Rel-16 Type II CB with new parameters to achieve higher resolution. The total overheads of datasets for training an AI/ML model from scratch with Transformer backbone as adopted in [3] are provided for both compression methods. The average overhead analysis in Table 1 has shown that the average overhead per hour is only 5.9 MB if data collection period (i.e., the time distance between two data collection procedures) is 1 week, and it can be reduced to 238 KB if training dataset is quantized by Rel-16 TypeII CB with new parameters. As analyzed in the table as well as shown by the evaluation results in our companion contribution [3], using training dataset quantized by Rel-16 TypeII CB with new parameters can provide only minor performance margin (<0.7%) but reduce 96% overhead of training dataset delivery.

Table 1 Overhead analysis for ground-truth CSI

| Compression method | Total overhead, 300k samples | Average overhead per hour |  |  |
| --- | --- | --- | --- | --- |
|  |  | data collection period = 1 month | data collection period = 1 week | data collection period = 1 day |
| Float32 | 992 MB | 1.4 MB | 5.9 MB | 41 MB |
| Rel-16 TypeII CB with new parameters: L=10, p=0.9, beta=0.31, amplitude: 4 bits, phase: 6 bits | 40 MB | 56 KB | 238 KB | 1.7 MB |

Observation 1: The overhead of data collection and report for ground-truth CSI may not be a big issue regarding the average overhead of data collection during the long period of model training/updating/ monitoring as well as dataset compression.

Proposal 4: Potential specification impact of reporting ground-truth CSI from UE to Network via air-interface should be studied for the model training/updating/monitoring purpose.

Assistance signaling

For the CSI compression sub use case, following discussions provides some further thinking on the assistant signaling for enabling the data collection for the UE and the Network, respectively.

For enabling the data collection at the UE side, firstly the signaling for triggering the data collection event or configuring the period of data collection may be needed. In addition, enhanced CSI-RS may be considered specifically for the data collection procedure to generate the dataset with accurate ground-truth CSI samples, as in principle the quality of the dataset can significantly contribute to the performance of the AI/ML model.

For enabling the data collection at the Network side, besides the signaling to enable the UE measurement to obtain the ground-truth CSI labels, the signaling/procedure for the UE report of the ground-truth CSI to Network is also needed. In addition, as the Network can also generate the dataset by taking into account the channels based on SRS measurement, the enhancement of SRS signal for data collection can also be studied.

Proposal 5: For the potential specification impact of data collection of the CSI compression sub use case, the potential assistance signaling for UE’s data collection includes at least:

- Enhanced CSI-RS for DL channel measurement

- Signaling for triggering/configuring the data collection procedure

Proposal 6: For the potential specification impact of data collection of the CSI compression sub use case, the potential assistance signaling for Network’s data collection includes at least

- Enhanced CSI-RS/SRS for channel measurement

- Signaling for triggering/configuring the data collection procedure

- Signaling/procedure for the UE report of the ground-truth CSI

Delivery of the dataset

As will be analyzed in Section 3.2, some training types such as Type 2/3 need the Network side and the UE side to train the CSI reconstruction part and the CSI generation part, respectively, based on aligned dataset, so that the signaling and the procedure for dataset delivery need to be studied as part of the training types. In order to align the understanding of the delivered dataset between the Network side and the UE side, the following aspects should be considered for dataset delivery:

- Size of the dataset, e.g., how many data samples are contained in the delivered dataset.

- Format of data sample, e.g., the dimension of the data samples.

- Type of data sample, e.g., whether the data samples are channel matrix or eigenvectors

Proposal 7: For the potential specification impact of dataset delivery of the CSI compression sub use case, study the signaling and procedure to deliver the dataset, including the size of the dataset, format of data sample, type(s) of the data sample, etc.

Training procedure for CSI compression

During the last RAN1 meeting, the following agreement had been achieved for training.

Agreement

In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

- Type 2: Joint training of the two-sided model at network side and UE side, repectively.

- Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

- Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes).

- Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW

- Other collaboration types are not excluded.

The specification impact of each training collaborations is analyzed in below.

Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided

For Type 1, the two-sided AI/ML model is trained at one side, which could be at Network side or at UE side. After model training is completed, Network delivers the trained CSI generation part to UE, as shown in Figure 1(a), or UE delivers the trained CSI reconstruction part to Network, as shown in Figure 1(b).

| (a) Joint training of the two-sided model at Network side | (b) Joint training of the two-sided model at UE side |
| --- | --- |

Figure 1 Joint training of the two-sided model at a single side/entity

Joint training at Network side

Joint training at the Network side has several advantages:

- First, it can achieve the optimal network performance since CSI generation part and CSI reconstruction part are designed and trained jointly at Network based on the dataset collected from the overall networks.

- Second, Network can dynamically update UE’s model when the wireless scenario changes, e.g., UE handover.

- Third, the Network can train and maintain a unified CSI reconstruction part over multiple UEs which avoids the multi-vendor combination issue (i.e., many pairs of models need to be maintained if models are trained per Network-UE vendor pair).

Joint training at the Network side also faces some issues:

- Compatibility Issue: The algorithm design of AI/ML model is coupled with the hardware (e.g., chipset) and the software platforms (e.g., runtime environment), which may result in that the AI/ML model trained at the Network side cannot be compiled successfully at the UE side. In addition, different computing efficiency at the Network side and UE side may result in low operating efficiency, long operating delay, or even failed to run at the UE side. That is to say, the UE may not deploy or perform inference for a totally unseen model structure delivered by Network. Whether there is any method to solve these kind of issues needs further study. For the model transfer/delivery of only the parameters, the compatibility issue may not impact, but the alignment of the model structure will again incur the proprietary issue as will be analysed later.

- MRF Issue: Model transfer/delivery faces the MRF issue, where UE may not interpret and compile the model transferred by Network due to different platforms for generating AI/ML models. A common MRF needs to be defined for transferring AI/ML model. Some formats, such as ONNX or NNEF, were raised for addressing this issue, however these formats are not designed for wireless communications and the feasibility is unclear at this stage. Defining 3GPP-specific MRF should be the right way to go, while this may bring non-trivial spec effort since it would potentially involve heavy workload across working groups, thus it is not clear whether it should be the first step to study for AI/ML for air interference. From long term perspective, it is a worthy aspect to study though.

- Proprietary Issue: The implementation of AI/ML models are usually proprietary. Whether or how to keep the proprietary of AI/ML models when Network side model is transferred/delivered to the UE needs to be further studied.

Joint training at UE side

The pros of joint training at UE side is list as below:

- UE can maintain a unified CSI generation part for multiple Network vendors which reduces the storage burden of UE.

However, the cons of joint training at UE side includes follows:

- First, the model trained by UE may not match the specific networks/scenario due to lack of matched dataset. Hence, the network performance may be bottlenecked by the models trained by UE.

- Second, the Network need to maintain or infer UE-specific models which increase the burden of computing and storage on the Network side.

- Third, joint training at UE side also faces the compatibility issue, MRF issue, and model proprietary issue as described under joint training at Network side of Type 1.

The potential specification impact for Type 1 may include AI/ML model transfer, e.g., the signaling and container for model structure and/or parameters.

Type 2: Joint training of the two-sided model at Network side and UE side, respectively

Type 2 can be defined as: a process to train the CSI generation part at UE side and CSI reconstruction at Network side in one forward propagation (FP) & backward propagation (BP) loop across the Network and the UE. In this type, both Network and UE are involved in model training while no AI/ML model is transferred over air-interface.

Figure 2 Joint training of the two-sided model at Network side and UE side, respectively

In particular, the model structure of CSI generation part and CSI reconstruction is designed separately by UE vendor and Network vendor, respectively, as shown in Figure 2, and the model of one vendor is unaware of by the vendor at the opposite side. By defining the BP and the FP interaction procedure and under a common dataset, the parameters of CSI generation part and CSI reconstruction can be trained jointly through iterative FP/BP loops, without disclosing the model/hardware/software to the other vendor. As an interaction approach, the FP information (e.g., the compressed CSI) and the BP information (e.g., the gradients information) during training process can be exchanged, e.g., in offline manner or over air-interface. This approach creates better independence between vendors and offers protection in model proprietary. However, this solution relies on complex design to support real-time interaction of FP/BP iterations between Network and UE which introduces challenges. As analyzed in our companion contribution [3], the overhead of FP and BP information is about 1.1G Bytes for a training progress.

The potential specification impact for Type 2 is FP/BP information exchange. If such exchange is via air-interface, e.g., the signaling to carry the gradients and the procedure to complete the iterations of FP/BP information exchange across Network and UE. In addition, the dataset of the target CSI labels for supporting the training has to be aligned between Network and UE, so how to share the dataset from one side to the other may also lead to specification impact.

Type 3: Separate training at Network side and UE side, where the UE side CSI generation part and the Network side CSI reconstruction part are trained by UE side and Network side, respectively

The procedure of how to proceed Type 3 needs further clarifications.

Sequential training starting with Network side training

The procedure of Type 3 for the sequential training starting with Network side training is illustrates in Figure 3, including following steps:

- Step 1, Network trains a two-sided AI/ML model, which includes a CSI generation part and a CSI reconstruction part with dataset#1 of original CSI, Vin. Note that the Network side CSI generation part is used only for training but will not be deployed for inference.

- Step 2, Network side shares the dataset#2 to UE side. The dataset#2 contains both input (Vin) and output (CSI feedback, Vq) of the Network side CSI generation part.

- Step 3, UE trains a UE-side CSI generation part using the dataset#2, with the training input as Vin and the loss function generated as MSEVq, Vq', where Vq' is the output of UE-side CSI generation part. The output of the Network side CSI generation part, Vq, is regarded as labels for the UE-side CSI generation part.

- Step 4, Network side CSI reconstruction part at Step 1 and UE-side CSI generation part in Step 3 can be separately deployed for joint inference.

Once the training at UE is finished, the output of the UE-side CSI generation part will be close to the output of the Network side CSI generation part used in Step 1 under the same input for them, so the Network side CSI reconstruction part can recognize the output of the UE-side CSI generation part and accurately recover the target CSI accordingly. For Type 3, the UE-side CSI generation part is designed and trained by the UE with UE-side FP/BP iterations, and the Network side CSI reconstruction part is designed and trained by Network with Network side FP/BP iterations. The design of the Network side CSI reconstruction part and the UE-side CSI generation part can therefore be kept proprietary.

Proposal 8: Further clarify the steps of performing training Type 3, e.g., for the sequential training starting with Network side training:

- Step1: Network trains the Network side CSI generation part (which is not used for inference) and the Network side CSI reconstruction part jointly

- Step2: Network shares UE side with the dataset including the input (original CSI) and output (CSI feedback) of the Network side CSI generation part

- Step3: UE side trains the UE side CSI generation part based on the dataset shared by Network

Figure 3 Procedure of separate training for CSI compression

Separate training facilitates the training of two-sided model by introducing only dataset sharing between Network side and UE side. The advantages of separating training with dataset sharing is as follows:

- It works with collaboration level y since model transfer is not required, and consequently avoids hardware/software compatibility issue and MRF issues. The Network can maintain a unified CSI reconstruction part over multiple UEs as the CSI reconstruction part at UE are trained based on an identical sharing dataset. Furthermore, model proprietary can be guaranteed as joint development between Network vendor and UE vendor is not needed.

The potential issue faced by separate training is list as below:

- The Type 3 may face the issue of suboptimal performance compared with joint training and the issue of overhead due to dataset transmission. In our companion contribution [3], the evaluation results have shown that there is only minor margin (<0.5%) between the performance of the separate training and the performance of the joint training even when the UE-side CSI generation part has a different structure with the Network side CSI generation part; in addition, the overhead of training dataset can be reduced significantly by using some quantization methods such as Rel-16 Type II-like codebook generation method with larger than legacy parameters to achieve higher resolution.

Sequential training starting with UE side training

There is an alternative mode for Type 3 which is symmetric to the above mode, i.e., the sequential training starting with UE side training: UE trains a two-sided AI/ML model and shares the dataset#2’ including the input and output of the UE-side CSI reconstruction part, then Network trains a Network side CSI reconstruction part based on dataset#2’. However, the dataset collected by UE side may not match the channel characteristics at the Network, regarding the Network vendor may want to perform cell/scenario specific model trainings while the dataset provided by UE vendors may not involve that categorization. In addition, Network has to maintain multiple different models based on multiple UE’s shared dataset, respectively, which would impose heavy burden on the Network storage. Moreover, the network performance relies on the quality of datasets sent from the UE vendor, which brings uncertainties to the Network vendor.

The potential specification impact for Type 3 is the training datasets delivery between Network and UE. E.g., for the sequential training starting with Network side training, the dataset sharing from gNB to UE may need to be specified, while for the sequential training starting with UE side training, the dataset sharing is the other way around.

Based on the above analysis, the pros and cons of aforementioned four training types are summarized in following Table 2.

Table 2 Brief comparison of the training types for two-sided model

| Training type |  | Pros |  | Cons |
| --- | --- | --- | --- | --- |
| Type 1 | NW-sided | - Optimal network performance - Dynamic model updating - Network can maintain a unified model over multiple UEs | - Compatibility issue on hardware/software at UE - AI/ML model representative format (MRF) needs more 3gpp efforts - How to protect model proprietary is not clear |  |
|  | UE-sided | - UE can maintain a unified model for multiple Network vendors | - Dataset for training at UE may not match the network channel characteristics - Compatibility issue on hardware/software at Network - Network may need to maintain/infer UE-specific models - AI/ML model MRF needs more 3gpp efforts - How to protect model proprietary is not clear |  |
| Type 2 |  | - Avoid hardware or software compatibility issue - Avoid MRF issue | - Complex design to support real-time interaction of FP/BP iterations between Network and UE - Dataset sharing to the opposite side is needed |  |
| Type 3 |  | - Avoid hardware or software compatibility issue - Avoid MRF issue - Model proprietary can be guaranteed - Avoid joint development between Multi-Network vendor and Multi-UE vendor | - Performance may be not optimal - Dataset sharing to the opposite side is needed |  |

Based on the above discussion, we make the following proposal on the potential specification impact.

Proposal 9: Study the potential specification impact for each of the following training types:

- Type 1 (Joint training at a single side/entity), including AI/ML model transfer, e.g., the model structure, model parameters, etc.

- Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), including FP/BP information exchange, training dataset delivery, etc.

- Type 3 (Separate training at Network side and UE side), including training dataset delivery.

Model inference for CSI compression

Input of CSI generation model

During the last RAN1 meeting, the following proposal related with model input had been discussed in the FL summary [4].

- Proposal 3-3-2(v2):

In CSI compression using two-sided model use case, further study potential specification impact on CSI generation model input, including at least

- Model input type/dimension/configuration and potential pre-processing, e.g.

- Model-based MIMO channel feature extraction methods (pre-processing) based on spatial- and frequency-domain DFT codebooks (using Type-II CSI Rel.16) and associated specification impacts (e.g., additional required signaling over the air interface).

- MIMO channel normalization methods, and associated specification impacts (e.g., additional required signaling over the air interface).

- Note: The CSI generation model input can be left for implementation if UE knows the input format through proprietary method.

For AI/ML-based CSI compression, the input of CSI generation part can be channel matrix or eigenvector. From the model inference perspective, the input of CSI generation part can be left for implementation if the CSI generation part is trained at the UE side. However, from the model monitoring perspective, the Network may need to be aware what kind of input is used for CSI generation part to better understand what is reported by UE and calculate the CSI accuracy correspondingly. Furthermore, for training Type 1 with joint training at Network side, training Type 2 and training Type 3, it is necessary for UE and Network to align the understanding of the CSI generation part input as well as the pre-processing of the input.

Proposal 10: Study the potential specification impact for the input of CSI generation part including input type/dimension/configuration and the potential pre-processing of the input.

CSI report

During the last RAN1 meeting, the following agreement had been achieved.

Agreement

In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least

- CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input.

- CQI determination

- RI determination

In the legacy CQI calculation, UE first derives the PMI for a CSI report and then calculate the CQI based on the PMI in the same CSI report, and such CQI is aligned with Network since the same PMI is used at the Network side to generate the precoder. However, for AI/ML-based CSI compression, since UE doesn’t know the output channel matrix/eigenvector recovered by the CSI reconstruction part at Network, a straightforward way is that the UE adopts the original channel matrix/eigenvectors (i.e., the input of the CSI generation part) for CQI calculation which is different from what will be recovered by Network. Such misalignment between the original channel and recovered channel will lead to misalignment of the CQI between Network and UE, and the CQI calculated by UE would be overestimated. To overcome this issue, some CQI compensation methods can be studied, e.g., CQI compensation based on some assistance of Network indication applied at UE on top of the CQI calculated by UE with the original channel matrix/eigenvectors.

For RI determination, it is our understanding that the UE can simply reuse the legacy approach to calculate RI (e.g., based on SINR and eigenvector). Though in some AI/ML solutions, the Network can reconstruct the original channel matrix, the RI determination is still better to be performed and reported by UE as the UE has more accurate measurement on SINR.

Proposal 11: For the study of the potential specification impact of CQI determination for AI/ML-based CSI compression, CQI compensation based on some assistance of Network indication can be considered as a candidate solution.

Proposal 12: For the CSI report of AI/ML-based CSI compression, legacy RI report procedure can be reused as a starting point.

Quantization

Similar as legacy CSI feedback, the quantization/dequantization method should be aligned between UE and Network to let Network understand the meaning of the CSI feedback. There are some different quantization methods for AI/ML-based CSI compression, such as scalar quantization and vector quantization, thus the alignment of the quantization/dequantization method is needed between Network and UE. For scalar quantization, the dictionary can be understood as a mapping from floating values to certain levels, and the quantized bit number can be configured. On the other hand, vector quantization is usually trained together with the AI/ML models and it is hard to specify a fixed dictionary, but still the Network can send/update the quantization dictionary for quantization to the UE. Thus, potential specification impact for delivery of vector quantization dictionary can be studied, e.g., the format of the vector quantization dictionary and the method to configure/update the dictionary.

Proposal 13: In CSI compression using two-sided model use case, further study potential specification impact on the quantization/dequantization method for the compressed CSI, e.g.,

- Alignment of the quantization/dequantization method between Network and UE

- Configuration/updating of the quantization dictionary

Assistance information for inference

Based on the evaluation results in our companion contribution [3], AI/ML-based CSI feedback can already achieve considerable and generalized performance gains without assistance information, while the benefit of assistance information to the performance is not clear yet. Therefore, the necessity of introducing the assistance information needs to be justified with critical evaluation gains. Moreover, some assistance information, such as the TxRU mapping information, beam angle/width information, etc., include the implementation related information which are proprietary of vendors and should not be disclosed to others. Therefore, it should be carefully studied and justified which specific assistance information are of great importance for performance improvement and are immune from proprietary disclosure before the normative work.

Observation 2: The provision of some assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

Model monitoring for AI/ML-based CSI feedback

During the last RAN1 meeting, the following proposal related with monitoring had been discussed in the FL summary [4].

- Proposal 3-4-1: In CSI compression using two-sided model use case, study potential specification impact for performance monitoring, considering at least the following aspects: - NW-based AI model performance monitoring and UE-based AI model performance monitoring - Performance monitoring KPIs - Assisted information and performance report - Co-existence and fall-back mechanisms between AI/ML mode and legacy non-AI/ML mode.

For AI/ML-based CSI feedback, AI/ML model monitoring can be performed at either the UE side or Network side. But in order to better ensure the network performance, it is preferred the Network takes responsibility for making decision on activate/deactivate/switch/update the AI/ML model. E.g., if UE side performs the monitoring, it can report the monitored performance KPIs to Network for making the final decision.

As discussed in [5], the monitoring metric can be intermediate results or the eventual KPI. For AI/ML-based CSI compression, an example of the intermediate result is the SGCS between the target CSI and the recovered CSI, and examples of the eventual KPI include throughput, RSRP, SINR, etc. If the SGCS is monitored at the UE side, it may need the recovered CSI information of the other side, and it may need to report the monitoring results to the Network. If SGCS is monitored at the Network side, it may need the UE to feedback the ground-truth CSI labels for calculating SGCS, which is similar to the data collection procedure. For the throughput, both Network and UE can calculate the throughput with legacy methods. The intermediate KPI of CSI accuracy and eventual KPI (e.g., throughput, RSRP, SINR, etc.) can both be considered for monitoring to provide information from different perspectives.

In addition, co-existence of the AI/ML mode and the legacy non-AI/ML mode is needed to compare the intermediate results/throughputs between the AI/ML mode and the legacy mode. To mitigate the fluctuation of other impact factors (e.g., varying channel status, varying scheduling/pairing mechanism, etc.) for the metrics of AI/ML mode and metrics of non-AI/ML mode, AI/ML mode and non-AI/ML mode could be indicated to operate in an interleaving manner. For example, a switching pattern of AI/ML-based mode and non-AI/ML-based mode could be configured with relatively short switching period. When the metric statistics in AI/ML mode is lower than legacy non-AI/ML mode statistics within the monitoring window, UE can be triggered to fallback to legacy non-AI/ML mode.

Proposal 14: For the monitoring of the AI/ML-based CSI feedback enhancement, consider the following modes:

- Network monitors the performance KPI for making decisions of model activation/deactivation/ updating/switching

- UE monitors the performance KPI and reports to Network for making decisions of model activation/ deactivation/updating/switching

Proposal 15: For the monitoring of the AI/ML-based CSI feedback enhancement, study the intermediate result of CSI accuracy (e.g., SGCS) as a performance KPI.

Proposal 16: Study the potential specification impact for the co-existence of AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

UE capability

AI/ML solution is a brand new feature for NR system and not all the UE may support AI/ML-based CSI feedback. Therefore, UE capability for supporting AI/ML-based CSI feedback should be studied. AI/ML-based CSI feedback involves many sub-features, such as data collection, dataset delivery, training type, model switching, model updating, model monitoring and CSI report timeline. Each of the above procedure may require a new UE capability. For example, different UEs may have different capabilities to support data collection/dataset delivery due to different storages; different UEs may or may not support the capacity for model training/updating/monitoring. For the inference, different UEs may have different inference latency which may result in different the CSI report timelines for AI/ML-based CSI feedback.

Proposal 17: Study the potential specification impact for UE capability, including the following as a starting point: data collection, dataset delivery, training, model switching, model updating, monitoring, and CSI report timeline.

Conclusions

In this contribution, we discussed the potential specification impact and the considerations of sub use cases for CSI feedback enhancement. Based on the discussions, we have the following observations and proposals.

Proposal 1: Categorize spatial-frequency domain CSI compression involving temporal domain compression using two-sided model as a specific AI/ML solution under spatial-frequency domain CSI compression for sub use case study.

Proposal 2: For AI/ML-based CSI feedback enhancement use case, joint CSI prediction and compression as well as one-sided model based on traditional codebook are NOT selected as representative sub-use cases.

Proposal 3: For the study of life cycle management for CSI feedback use case, discuss use case specific procedures in 9.2.2.2, including data collection, training, updating, inference, monitoring, fallback, and UE capability.

- FFS: [model registration], and [model configuration]

Proposal 4: Potential specification impact of reporting ground-truth CSI from UE to Network via air-interface should be studied for the model training/updating/monitoring purpose.

Proposal 5: For the potential specification impact of data collection of the CSI compression sub use case, the potential assistance signaling for UE’s data collection includes at least:

- Enhanced CSI-RS for DL channel measurement

- Signaling for triggering/configuring the data collection procedure

Proposal 6: For the potential specification impact of data collection of the CSI compression sub use case, the potential assistance signaling for Network’s data collection includes at least

- Enhanced CSI-RS/SRS for channel measurement

- Signaling for triggering/configuring the data collection procedure

- Signaling/procedure for the UE report of the ground-truth CSI

Proposal 7: For the potential specification impact of dataset delivery of the CSI compression sub use case, study the signaling and procedure to deliver the dataset, including the size of the dataset, format of data sample, type(s) of the data sample, etc.

Proposal 8: Further clarify the steps of performing training Type 3, e.g., for the sequential training starting with Network side training:

- Step1: Network trains the Network side CSI generation part (which is not used for inference) and the Network side CSI reconstruction part jointly

- Step2: Network shares UE side with the dataset including the input (original CSI) and output (CSI feedback) of the Network side CSI generation part

- Step3: UE side trains the UE side CSI generation part based on the dataset shared by Network

Proposal 9: Study the potential specification impact for each of the following training types:

- Type 1 (Joint training at a single side/entity), including AI/ML model transfer, e.g., the model structure, model parameters, etc.

- Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), including FP/BP information exchange, training dataset delivery, etc.

- Type 3 (Separate training at Network side and UE side), including training dataset delivery.

Proposal 10: Study the potential specification impact for the input of CSI generation part including input type/dimension/configuration and the potential pre-processing of the input.

Proposal 11: For the study of the potential specification impact of CQI determination for AI/ML-based CSI compression, CQI compensation based on some assistance of Network indication can be considered as a candidate solution.

Proposal 12: For the CSI report of AI/ML-based CSI compression, legacy RI report procedure can be reused as a starting point.

Proposal 13: In CSI compression using two-sided model use case, further study potential specification impact on the quantization/dequantization method for the compressed CSI, e.g.,

- Alignment of the quantization/dequantization method between Network and UE

- Configuration/updating of the quantization dictionary

Proposal 14: For the monitoring of the AI/ML-based CSI feedback enhancement, consider the following modes:

- Network monitors the performance KPI for making decisions of model activation/deactivation/ updating/switching

- UE monitors the performance KPI and reports to Network for making decisions of model activation/ deactivation/updating/switching

Proposal 15: For the monitoring of the AI/ML-based CSI feedback enhancement, study the intermediate result of CSI accuracy (e.g., SGCS) as a performance KPI.

Proposal 16: Study the potential specification impact for the co-existence of AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

Proposal 17: Study the potential specification impact for UE capability, including the following as a starting point: data collection, dataset delivery, training, model switching, model updating, monitoring, and CSI report timeline.

Observation 1: The overhead of data collection and report for ground-truth CSI may not be a big issue regarding the average overhead of data collection during the long period of model training/updating/ monitoring as well as dataset compression.

Observation 2: The provision of some assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- References

RAN1 Chairman’s notes, RAN1#109-e, May 9-20, 2022.

RAN1 Chairman’s notes, RAN1#110, August 22-26, 2022.

R1-2208429, “Evaluation on AI/ML for CSI feedback enhancement”, RAN1#110bis-e, Huawei, HiSilicon, October 10 - 19, 2022.

R1-2208077, “Summary #4 on other aspects of AI/ML for CSI enhancement”, RAN1#110, Moderator (Apple), August 2022.

R1-2208428, “Discussion on general aspects of AI/ML framework”, RAN1#110bis-e, Huawei, HiSilicon, October 10 - 19, 2022