3GPP TSG-RAN WG1 Meeting #110bis-e 	R1-2208452

e-Meeting, October 10 – 19, 2022

Agenda Item:	9.5.1.1

Source:	Huawei, HiSilicon

Title:	SL positioning evaluations

Document for:	Discussion and decision

# Introduction

In RAN1#109e, the evaluation methodology for SL positioning have been agreed [1]. In the contribution, we will present the evaluation results for each use case regarding the accuracy of absolute positioning, relative positioning and ranging with distance and angle.

# General evaluation assumptions

The general evaluation assumptions for each use case are shown in Table 1.

Table 1 Evaluation assumptions

| Parameter | V2X | Public safety | Commercial | IIOT |
| --- | --- | --- | --- | --- |
| Carrier frequency | Uu: 4GHz SL: 6GHz | Uu: 3.5GHz SL: 0.7GHz | SL: 3.5GHz | Uu: 3.5GHz SL: 3.5GHz |
| BS Tx power | 49dBm | 49dBm | / | 24dBm |
| UE Tx power | Vehicle UE or UE type RSU: 23dBm | UE or UE type RSU: 23dBm | UE or UE type RSU: 23dBm | UE or UE type RSU: 23dBm |
| BS receiver noise figure | 5dB | 5dB | 5dB | 5dB |
| UE receiver noise figure | 9dB | 9dB | 9dB | 9dB |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |
| RSU deployment | Highway: Option 1, 200m spacing on both sides of highway symmetrically; Option 2, staggered on both sides of the highway with distance of 200m Urban grid: in the middle of the crossroads | Distributed on a square lattice with spacing 200m (4×4, 16 in total) | Indoor-office(120m×50m×3m): Distributed on a square lattice with spacing 20m (6×2, 12 in total) | Distributed on a square lattice with spacing D near the wall (4×7, 28 in total) InF-SH: D=50m InF-DH: D=20m |
| Maximum UE-UE distance for relative positioning | Highway:50m and 150m Urban: 10m and 50m | 20m | 10m | 10m |

# Evaluations for V2X use cases

## General description

The scenarios to be presented in section 3 include

- Highway scenario

- Urban grid scenario

The UE dropping model follows the option A defined in [2]. The antenna model of vehicle UE follows the option 1 defined in section 6.1.4 of [2].

## Highway

### Absolute positioning

The evaluated cases are for FR1, and the results of SL only positioning and joint Uu/SL positioning are provided for Multi-RTT. The general description of all evaluated cases is shown in the Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 2.

Table 2  Positioning accuracy (meter)

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 1001, V2X, Highway, [RSU], 20MHz, RTT, Absolute | 0.570 | 0.828 | 1.165 | 1.631 |
| 1002, V2X, Highway, [RSU], 40MHz, RTT, Absolute | 0.383 | 0.586 | 0.861 | 1.231 |
| 1003, V2X, Highway, [RSU], 100MHz, RTT, Absolute | 0.120 | 0.183 | 0.275 | 0.432 |
| 1004, V2X, Highway, [BS,RSU], 20MHz, RTT, Absolute | 0.440 | 0.691 | 0.968 | 1.458 |
| 1005, V2X, Highway, [BS,RSU], 40MHz, RTT, Absolute | 0.325 | 0.484 | 0.741 | 1.091 |
| 1006, V2X, Highway, [BS,RSU], 100MHz, RTT, Absolute | 0.106 | 0.162 | 0.253 | 0.429 |
| 1007, V2X, Highway, [StaggerRSU], 20MHz, RTT, Absolute | 0.554 | 0.832 | 1.320 | 1.844 |
| 1008, V2X, Highway, [StaggerRSU], 40MHz, RTT, Absolute | 0.404 | 0.602 | 0.916 | 1.415 |
| 1009, V2X, Highway, [StaggerRSU], 100MHz, RTT, Absolute | 0.123 | 0.202 | 0.313 | 0.494 |

- Observation 1: For V2X Highway absolute positioning, the accuracy of less than 1.5m@90% (requirement Set A) can be achieved with at least 40MHz bandwidth, and the accuracy of less than 0.5m@90% (requirement Set B) can be achieved with at least 100MHz bandwidth.

### Relative positioning and ranging

The evaluated cases are for FR1, and the results of SL UE-UE only positioning and joint UE-UE and RSU-UE positioning are provided for Multi-RTT+AOA. The largest distance between the two UEs is X, where X=50m and 150m. The general description of all evaluated cases is shown in the Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 3 (for X=50m) and Table 4 (for X=150m). The ranging distance and angle error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 5 (for X=50m) and Table 6 (for X=150m).

Table 3  Positioning accuracy (meter) [X = 50m]

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 1101, V2X, Highway, [UE], 20MHz, RTT+AOA, Relative | 0.902 | 1.438 | 2.253 | 3.448 |
| 1102, V2X, Highway, [UE], 40MHz, RTT+AOA, Relative | 0.674 | 1.144 | 1.886 | 2.785 |
| 1103, V2X, Highway, [UE], 100MHz, RTT+AOA, Relative | 0.482 | 0.773 | 1.210 | 2.148 |
| 1104, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative | 0.869 | 1.350 | 2.071 | 2.954 |
| 1105, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative | 0.651 | 1.065 | 1.763 | 2.551 |
| 1106, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative | 0.273 | 0.402 | 0.599 | 0.955 |
| 1107, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative | 0.561 | 0.807 | 1.137 | 1.571 |
| 1108, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative | 0.373 | 0.573 | 0.819 | 1.182 |
| 1109, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative | 0.119 | 0.180 | 0.264 | 0.407 |

Table 4  Positioning accuracy (meter) [X = 150m]

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 1121, V2X, Highway, [UE], 20MHz, RTT+AOA, Relative | 3.286 | 5.992 | 10.899 | 24.749 |
| 1122, V2X, Highway, [UE], 40MHz, RTT+AOA, Relative | 2.544 | 4.691 | 8.469 | 22.251 |
| 1123, V2X, Highway, [UE], 100MHz, RTT+AOA, Relative | 1.665 | 3.319 | 6.495 | 24.901 |
| 1124, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative | 2.629 | 4.79 | 7.165 | 11.099 |
| 1125, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative | 2.017 | 3.326 | 5.318 | 8.364 |
| 1126, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative | 1.294 | 2.407 | 3.971 | 6.256 |
| 1127, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative | 0.980 | 1.547 | 2.280 | 3.636 |
| 1128, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative | 0.735 | 1.097 | 1.650 | 2.445 |
| 1129, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative | 0.252 | 0.384 | 0.609 | 1.046 |

Table 5  Ranging with distance accuracy (meter) and angle accuracy (degree) [X = 50m]

| Case number |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 1101, V2X, Highway, [UE], 20MHz, Relative | Distance | 0.234 | 0.369 | 0.524 | 0.749 |
| 1102, V2X, Highway, [UE], 40MHz, Relative | Distance | 0.160 | 0.226 | 0.332 | 0.451 |
| 1103, V2X, Highway, [UE], 100MHz, Relative | Distance | 0.045 | 0.070 | 0.102 | 0.148 |
| 1104, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative | Distance | 0.226 | 0.351 | 0.495 | 0.687 |
| 1105, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative | Distance | 0.155 | 0.221 | 0.309 | 0.430 |
| 1106, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative | Distance | 0.044 | 0.067 | 0.094 | 0.137 |
| 1107, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative | Distance | 0.193 | 0.323 | 0.481 | 0.732 |
| 1108, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative | Distance | 0.153 | 0.232 | 0.347 | 0.502 |
| 1109, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative | Distance | 0.049 | 0.077 | 0.115 | 0.176 |
| Case number |  | 50% | 67% | 80% | 90% |
| 1101, V2X, Highway, [UE], 20MHz, Relative | Angle | 1.800 | 2.973 | 4.412 | 6.549 |
| 1102, V2X, Highway, [UE], 40MHz, Relative | Angle | 1.449 | 2.463 | 3.624 | 5.600 |
| 1103, V2X, Highway, [UE], 100MHz, Relative | Angle | 1.086 | 1.673 | 2.704 | 4.348 |
| 1104, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative | Angle | 1.721 | 2.846 | 3.980 | 5.881 |
| 1105, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative | Angle | 1.357 | 2.316 | 3.362 | 4.756 |
| 1106, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative | Angle | 1.030 | 1.600 | 2.446 | 3.805 |
| 1107, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative | Angle | 0.874 | 1.553 | 2.353 | 4.423 |
| 1108, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative | Angle | 0.574 | 1.016 | 1.817 | 3.129 |
| 1109, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative | Angle | 0.192 | 0.349 | 0.633 | 1.029 |

Table 6  Ranging with distance accuracy (meter) and angle accuracy (degree) [X = 150m]

| Case number |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 1121, V2X, Highway, [UE], 20MHz, Relative | Distance | 0.246 | 0.396 | 0.656 | 1.343 |
| 1122, V2X, Highway, [UE], 40MHz, Relative | Distance | 0.172 | 0.267 | 0.419 | 0.747 |
| 1123, V2X, Highway, [UE], 100MHz, Relative | Distance | 0.056 | 0.092 | 0.148 | 0.248 |
| 1124, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative | Distance | 0.217 | 0.316 | 0.470 | 0.688 |
| 1125, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative | Distance | 0.152 | 0.219 | 0.305 | 0.451 |
| 1126, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative | Distance | 0.047 | 0.073 | 0.106 | 0.154 |
| 1127, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative | Distance | 0.226 | 0.335 | 0.513 | 0.943 |
| 1128, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative | Distance | 0.158 | 0.238 | 0.340 | 0.573 |
| 1129, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative | Distance | 0.053 | 0.081 | 0.120 | 0.202 |
| Case number |  | 50% | 67% | 80% | 90% |
| 1121, V2X, Highway, [UE], 20MHz, Relative | Angle | 2.949 | 4.8 | 7.531 | 15.959 |
| 1122, V2X, Highway, [UE], 40MHz, Relative | Angle | 2.119 | 3.79 | 5.775 | 16.8 |
| 1123, V2X, Highway, [UE], 100MHz, Relative | Angle | 1.495 | 2.562 | 4.358 | 15.948 |
| 1124, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative | Angle | 2.4 | 4.021 | 5.333 | 7.683 |
| 1125, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative | Angle | 1.819 | 2.834 | 4.12 | 5.744 |
| 1126, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative | Angle | 1.256 | 1.983 | 2.885 | 4.132 |
| 1127, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative | Angle | 0.762 | 1.242 | 1.929 | 3.490 |
| 1128, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative | Angle | 0.554 | 0.932 | 1.448 | 2.382 |
| 1129, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative | Angle | 0.195 | 0.336 | 0.532 | 1.205 |

- Observation 2: For V2X Highway relative positioning between two UEs within X=50m and 150m, 1.5m@90% (requirement Set A) cannot be achieved with 100MHz bandwidth for SL UE-UE only positioning.

- Observation 3: For V2X Highway relative positioning between two UEs within X=50m and 150m, RSU assisted positioning could help to increase the accuracy, the accuracy of less than 1.5m@90% (requirement Set A) can be achieved with at least 40MHz bandwidth for X=50m and with at least 100MHz bandwidth for X=150m for joint UE-UE and RSU-UE positioning.

- Observation 4: For V2X Highway relative positioning between two UEs within X=150m, 100MHz bandwidth cannot met the requirement Set B.

- Observation 5: For V2X Highway ranging, the distance accuracy of less than 1.5m@90% (requirement Set A) can be achieved with 20MHz bandwidth, and the distance accuracy of less than 0.5m@90% (requirement Set B) can be achieved with at least 40MHz bandwidth for X=50m and with at least 100MHz bandwidth for X=150m.

- Observation 6: For V2X Highway ranging, the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with 20MHz bandwidth for X=50m for all cases, and the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with 20MHz bandwidth for X=150m for joint UE-UE and RSU-UE positioning.

## Urban grid

### Absolute positioning

The evaluated cases are for FR1, and the results of SL only positioning and joint Uu/SL positioning are provided for Multi-RTT. The general description of all evaluated cases is shown in the Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 7.

Table 7  Positioning accuracy (meter)

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 2001, V2X, UrbanGrid, [RSU], 20MHz, RTT, Absolute | 4.749 | 6.996 | 9.938 | 16.222 |
| 2002, V2X, UrbanGrid, [RSU], 40MHz, RTT, Absolute | 3.911 | 7.001 | 10.070 | 12.726 |
| 2003, V2X, UrbanGrid, [RSU], 100MHz, RTT, Absolute | 3.509 | 5.782 | 7.88 | 10.489 |
| 2004, V2X, UrbanGrid, [BS,RSU], 20MHz, RTT, Absolute | 0.331 | 0.604 | 1.130 | 3.187 |
| 2005, V2X, UrbanGrid, [BS,RSU], 40MHz, RTT, Absolute | 0.258 | 0.548 | 1.100 | 2.777 |
| 2006, V2X, UrbanGrid, [BS,RSU], 100MHz, RTT, Absolute | 0.181 | 0.361 | 0.628 | 1.628 |

- Observation 7: For V2X Urban grid absolute positioning, the accuracy of less than 3m@90% can be achieved with 40MHz bandwidth for joint Uu/SL positioning, but both requirement Set A and requirement Set B cannot be met.

### Relative positioning and ranging

The evaluated cases are for FR1, and the results of SL only positioning and joint Uu/SL positioning are provided for Multi-RTT+AOA. The largest distance between the two UEs is X, where X=10m and 50m. The general description of all evaluated cases is shown in the Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 8 (for X=10m) and Table 9(for X=150m). The ranging distance and angle error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 10 (for X=10m) and Table 11 (for X=150m).

Table 8  Positioning accuracy (meter) [X = 10m]

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 2101, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative | 1.072 | 1.621 | 2.485 | 4.254 |
| 2102, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative | 0.878 | 1.203 | 1.723 | 2.614 |
| 2103, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative | 0.286 | 0.403 | 0.621 | 0.974 |
| 2104, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative | 1.014 | 1.507 | 2.290 | 3.423 |
| 2105, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative | 0.848 | 1.129 | 1.548 | 2.300 |
| 2106, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative | 0.270 | 0.379 | 0.554 | 0.808 |
| 2107, V2X, UrbanGrid, [BS,RSU,UE], 20MHz, RTT+AOA, Relative | 0.99 | 1.459 | 2.261 | 3.416 |
| 2108, V2X, UrbanGrid, [BS,RSU,UE], 40MHz, RTT+AOA, Relative | 0.814 | 1.087 | 1.523 | 2.269 |
| 2109, V2X, UrbanGrid, [BS,RSU,UE], 100MHz, RTT+AOA, Relative | 0.270 | 0.379 | 0.554 | 0.806 |

Table 9  Positioning accuracy (meter) [X = 50m]

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 2121, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative | 1.090 | 1.706 | 2.755 | 5.256 |
| 2122, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative | 0.796 | 1.163 | 1.732 | 3.010 |
| 2123, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative | 0.302 | 0.476 | 0.785 | 1.655 |
| 2124, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative | 1.025 | 1.543 | 2.324 | 4.235 |
| 2125, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative | 0.755 | 1.096 | 1.551 | 2.457 |
| 2126, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative | 0.280 | 0.439 | 0.671 | 1.071 |
| 2127, V2X, UrbanGrid, [BS,RSU,UE], 20MHz, RTT+AOA, Relative | 1.001 | 1.500 | 2.316 | 4.233 |
| 2128, V2X, UrbanGrid, [BS,RSU,UE], 40MHz, RTT+AOA, Relative | 0.732 | 1.077 | 1.535 | 2.466 |
| 2129, V2X, UrbanGrid, [BS,RSU,UE], 100MHz, RTT+AOA, Relative | 0.275 | 0.422 | 0.671 | 1.060 |

Table 10  Ranging with distance accuracy (meter) and angle accuracy (degree) [X = 10m]

| Case number |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 2101, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative | Distance | 0.736 | 1.261 | 2.066 | 3.387 |
| 2102, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative | Distance | 0.605 | 0.898 | 1.280 | 2.063 |
| 2103, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative | Distance | 0.090 | 0.141 | 0.208 | 0.286 |
| 2104, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative | Distance | 0.752 | 1.219 | 1.831 | 2.999 |
| 2105, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative | Distance | 0.606 | 0.888 | 1.258 | 1.997 |
| 2106, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative | Distance | 0.084 | 0.136 | 0.197 | 0.275 |
| 2107, V2X, UrbanGrid, [UE,RSU], 20MHz, RTT+AOA, Relative | Distance | 0.697 | 1.175 | 1.801 | 2.990 |
| 2108, V2X, UrbanGrid, [UE,RSU], 40MHz, RTT+AOA, Relative | Distance | 0.566 | 0.865 | 1.238 | 1.938 |
| 2109, V2X, UrbanGrid, [UE,RSU], 100MHz, RTT+AOA, Relative | Distance | 0.085 | 0.137 | 0.199 | 0.281 |
| Case number |  | 50% | 67% | 80% | 90% |
| 2101, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative | Angle | 3.309 | 5.358 | 8.345 | 14.954 |
| 2102, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative | Angle | 2.710 | 4.160 | 6.544 | 12.487 |
| 2103, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative | Angle | 1.988 | 3.220 | 4.914 | 8.660 |
| 2104, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative | Angle | 3.036 | 4.791 | 6.857 | 10.081 |
| 2105, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative | Angle | 2.457 | 3.648 | 5.489 | 8.059 |
| 2106, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative | Angle | 1.813 | 2.932 | 4.320 | 6.125 |
| 2107, V2X, UrbanGrid, [UE,RSU], 20MHz, RTT+AOA, Relative | Angle | 2.839 | 4.462 | 6.444 | 9.690 |
| 2108, V2X, UrbanGrid, [UE,RSU], 40MHz, RTT+AOA, Relative | Angle | 2.267 | 3.393 | 4.910 | 7.367 |
| 2109, V2X, UrbanGrid, [UE,RSU], 100MHz, RTT+AOA, Relative | Angle | 1.710 | 2.750 | 3.922 | 5.793 |

Table 11  Ranging with distance accuracy (meter) and angle accuracy (degree) [X = 50m]

| Case number |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 2121, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative | Distance | 0.748 | 1.190 | 1.902 | 3.487 |
| 2122, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative | Distance | 0.505 | 0.776 | 1.146 | 1.843 |
| 2123, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative | Distance | 0.091 | 0.148 | 0.208 | 0.281 |
| 2124, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative | Distance | 0.698 | 1.125 | 1.728 | 2.899 |
| 2125, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative | Distance | 0.493 | 0.745 | 1.052 | 1.571 |
| 2126, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative | Distance | 0.087 | 0.138 | 0.196 | 0.255 |
| 2127, V2X, UrbanGrid, [UE,RSU], 20MHz, RTT+AOA, Relative | Distance | 0.674 | 1.101 | 1.717 | 2.920 |
| 2128, V2X, UrbanGrid, [UE,RSU], 40MHz, RTT+AOA, Relative | Distance | 0.486 | 0.718 | 1.047 | 1.586 |
| 2129, V2X, UrbanGrid, [UE,RSU], 100MHz, RTT+AOA, Relative | Distance | 0.085 | 0.137 | 0.198 | 0.251 |
| Case number |  | 50% | 67% | 80% | 90% |
| 2121, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative | Angle | 2.933 | 4.805 | 7.909 | 15.600 |
| 2122, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative | Angle | 2.320 | 3.820 | 6.380 | 12.634 |
| 2123, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative | Angle | 1.847 | 3.083 | 5.070 | 10.245 |
| 2124, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative | Angle | 2.780 | 4.498 | 6.856 | 11.521 |
| 2125, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative | Angle | 2.220 | 3.491 | 5.400 | 9.473 |
| 2126, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative | Angle | 1.780 | 2.815 | 4.409 | 7.180 |
| 2127, V2X, UrbanGrid, [UE,RSU], 20MHz, RTT+AOA, Relative | Angle | 2.663 | 4.420 | 6.826 | 11.286 |
| 2128, V2X, UrbanGrid, [UE,RSU], 40MHz, RTT+AOA, Relative | Angle | 2.146 | 3.380 | 5.382 | 9.473 |
| 2129, V2X, UrbanGrid, [UE,RSU], 100MHz, RTT+AOA, Relative | Angle | 1.719 | 2.765 | 4.233 | 7.174 |

- Observation 8: For V2X Urban grid relative positioning, the accuracy of less than 1.5m@90% (requirement Set A) can be achieved with at least 100MHz bandwidth for two UEs within X=50m for SL UE-UE only positioning.

- Observation 9: For V2X Urban grid relative positioning between two UEs within X=10m and 50m, BS and RSU assisted positioning could help to increase the performance, the accuracy is almost the same as the LOS-only UE-UE positioning.

- Observation 10: For V2X Urban grid ranging between two UEs within X=10m and 50m, the distance accuracy of less than 0.5m@90% (requirement Set B) can be achieved with at least 100MHz bandwidth.

- Observation 11: For V2X Urban grid ranging between two UEs within X=10m and 50m, the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with at least 100MHz bandwidth for joint Uu and SL positioning.

# Evaluations for Public safety use cases

## General description

The RSUs with height 1.5m are distributed on a square lattice with spacing 200m (4×4, 16 in total). Each component of O2O channel model (UMi) in Section A.2.1.2 of [5] reuses what is specified in [3]. The UEs to be evaluated are randomly dropped.

## Absolute positioning

The evaluated cases are for FR1, and the results of SL only positioning and joint Uu/SL positioning are provided for Multi-RTT. The general description of all evaluated cases is shown in the Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 12.

Table 12  Positioning accuracy (meter)

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 3001, PS, [RSU], 10MHz, RTT, Absolute | 9.402 | 21.234 | 40.737 | 58.496 |
| 3002, PS, [RSU], 20MHz, RTT, Absolute | 4.258 | 12.351 | 26.327 | 40.256 |
| 3003, PS, [RSU], 40MHz, RTT, Absolute | 1.619 | 3.869 | 11.990 | 22.276 |
| 3004, PS, [RSU], 100MHz, RTT, Absolute | 0.341 | 0.582 | 1.326 | 7.742 |
| 3005, PS, [BS,RSU], 10MHz, RTT, Absolute | 10.748 | 14.816 | 17.752 | 23.886 |
| 3006, PS, [BS,RSU], 20MHz, RTT, Absolute | 10.832 | 14.823 | 17.791 | 23.986 |
| 3007, PS, [BS,RSU], 40MHz, RTT, Absolute | 1.795 | 5.477 | 12.564 | 23.298 |
| 3008, PS, [BS,RSU], 100MHz, RTT, Absolute | 0.334 | 0.558 | 1.163 | 7.555 |

Observation 12: For public safety absolute positioning, the accuracy requirement 1m@90% cannot be achieved.

## Relative positioning and ranging

The evaluated cases are for FR1, and the results of SL UE-UE only positioning and joint UE-UE and RSU-UE positioning are provided for Multi-RTT+AOA. The largest distance between the two UEs is 20m. The general description of all evaluated cases is shown in the Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 13. The ranging distance and angle error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 14.

Table 13  Positioning accuracy (meter)

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 3101, PS, [UE], 10MHz, RTT+AOA, Relative | 2.205 | 3.649 | 4.899 | 6.977 |
| 3102, PS, [UE], 20MHz, RTT+AOA, Relative | 0.699 | 1.098 | 1.628 | 2.351 |
| 3103, PS, [UE], 40MHz, RTT+AOA, Relative | 0.405 | 0.601 | 0.878 | 1.423 |
| 3104, PS, [UE], 100MHz, RTT+AOA, Relative | 0.176 | 0.295 | 0.497 | 0.956 |
| 3105, PS, [UE,RSU], 10MHz, RTT+AOA, Relative | 2.193 | 3.626 | 4.838 | 6.983 |
| 3106, PS, [UE,RSU], 20MHz, RTT+AOA, Relative | 0.703 | 1.106 | 1.622 | 2.341 |
| 3107, PS, [UE,RSU], 40MHz, RTT+AOA, Relative | 0.420 | 0.617 | 0.887 | 1.454 |
| 3108, PS, [UE,RSU], 100MHz, RTT+AOA, Relative | 0.208 | 0.332 | 0.544 | 0.971 |

Table 14  Ranging with distance accuracy (meter) and angle accuracy (degree)

| Case number |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 3101, PS, [UE], 10MHz, RTT+AOA, Relative | Distance | 1.878 | 3.326 | 4.384 | 5.567 |
| 3102, PS, [UE], 20MHz, RTT+AOA, Relative | Distance | 0.463 | 0.775 | 1.217 | 1.830 |
| 3103, PS, [UE], 40MHz, RTT+AOA, Relative | Distance | 0.224 | 0.375 | 0.528 | 0.805 |
| 3104, PS, [UE], 100MHz, RTT+AOA, Relative | Distance | 0.049 | 0.085 | 0.130 | 0.215 |
| 3105, PS, [UE,RSU], 10MHz, RTT+AOA, Relative | Distance | 1.870 | 3.288 | 4.343 | 5.522 |
| 3106, PS, [UE,RSU], 20MHz, RTT+AOA, Relative | Distance | 0.473 | 0.784 | 1.222 | 1.803 |
| 3107, PS, [UE,RSU], 40MHz, RTT+AOA, Relative | Distance | 0.232 | 0.396 | 0.561 | 0.869 |
| 3108, PS, [UE,RSU], 100MHz, RTT+AOA, Relative | Distance | 0.072 | 0.122 | 0.191 | 0.330 |
| Case number |  | 50% | 67% | 80% | 90% |
| 3101, PS, [UE], 10MHz, RTT+AOA, Relative | Angle | 1.521 | 2.672 | 4.589 | 8.356 |
| 3102, PS, [UE], 20MHz, RTT+AOA, Relative | Angle | 1.239 | 2.135 | 3.432 | 6.293 |
| 3103, PS, [UE], 40MHz, RTT+AOA, Relative | Angle | 1.047 | 1.810 | 2.791 | 4.942 |
| 3104, PS, [UE], 100MHz, RTT+AOA, Relative | Angle | 0.832 | 1.402 | 2.245 | 3.785 |
| 3105, PS, [UE,RSU], 10MHz, RTT+AOA, Relative | Angle | 1.530 | 2.633 | 4.410 | 7.983 |
| 3106, PS, [UE,RSU], 20MHz, RTT+AOA, Relative | Angle | 1.245 | 2.111 | 3.383 | 5.994 |
| 3107, PS, [UE,RSU], 40MHz, RTT+AOA, Relative | Angle | 1.048 | 1.779 | 2.764 | 4.771 |
| 3108, PS, [UE,RSU], 100MHz, RTT+AOA, Relative | Angle | 0.832 | 1.405 | 2.240 | 3.642 |

Observation 13: For public safety relative positioning and ranging, the accuracy requirement 1m@90% cannot be achieved with 10MHz bandwidth.

Observation 14: For public safety relative positioning, the accuracy requirement 1m@90% can be achieved with at least 100MHz bandwidth.

Observation 15: For public safety ranging, the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with at least 20MHz bandwidth for joint UE-UE and RSU-UE positioning.

# Evaluations for Commercial use cases

## General description

The layout parameters may reference Table 7.2-2 in [3]. RSUs with height 3m are distributed on a square lattice with spacing 20m. Each component of I2I channel model (InH) in Section A.2.1.2 of [5] reuses what is specified in TR [3].  The UEs to be evaluated are randomly dropped.

## Absolute positioning

The evaluated cases are for FR1, and the results of SL only positioning are provided for Multi-RTT. The general description of all evaluated cases is shown in the Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 15.

Table 15  Positioning accuracy (meter)

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 4001, Commercial, [RSU,UE], 20MHz, RTT, Absolute | 0.520 | 0.753 | 1.015 | 1.595 |
| 4002, Commercial, [RSU,UE], 40MHz, RTT, Absolute | 0.257 | 0.342 | 0.451 | 0.584 |
| 4003, Commercial, [RSU,UE], 100MHz, RTT, Absolute | 0.051 | 0.075 | 0.103 | 0.147 |

Observation 16: For commercial use case absolute positioning, the accuracy of less than 1m@90% can be achieved with at least 40MHz bandwidth.

## Relative positioning and ranging

The evaluated cases are for FR1, and the results of SL UE-UE only positioning and joint UE-UE and RSU-UE positioning are provided for Multi-RTT+AOA. The largest distance between the two UEs is 10m. The general description of all evaluated cases is shown in Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 16. The ranging distance and angle error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 17.

Table 16  Positioning accuracy (meter)

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 4101, Commercial, [UE], 20MHz, RTT+AOA, Relative | 0.592 | 0.847 | 1.384 | 2.885 |
| 4102, Commercial, [UE], 40MHz, RTT+AOA, Relative | 0.386 | 0.618 | 0.953 | 1.821 |
| 4103, Commercial, [UE], 100MHz, RTT+AOA, Relative | 0.104 | 0.171 | 0.278 | 0.534 |
| 4104, Commercial, [UE,RSU], 20MHz, RTT+AOA, Relative | 0.505 | 0.745 | 1.064 | 1.756 |
| 4105, Commercial, [UE,RSU], 40MHz, RTT+AOA, Relative | 0.331 | 0.448 | 0.585 | 0.775 |
| 4106, Commercial, [UE,RSU], 100MHz,RTT+AOA, Relative | 0.067 | 0.091 | 0.122 | 0.174 |

Table 17  Ranging with distance accuracy (meter) and angle accuracy (degree)

| Case number |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 4101, Commercial, [UE], 20MHz, RTT+AOA, Relative | Distance | 0.466 | 0.639 | 0.934 | 1.627 |
| 4102, Commercial, [UE], 40MHz, RTT+AOA, Relative | Distance | 0.248 | 0.444 | 0.640 | 1.046 |
| 4103, Commercial, [UE], 100MHz, RTT+AOA, Relative | Distance | 0.043 | 0.060 | 0.084 | 0.130 |
| 4104, Commercial, [UE,RSU], 20MHz, RTT+AOA, Relative | Distance | 0.406 | 0.606 | 0.878 | 1.448 |
| 4105, Commercial, [UE,RSU], 40MHz, RTT+AOA, Relative | Distance | 0.174 | 0.266 | 0.384 | 0.544 |
| 4106, Commercial, [UE,RSU], 100MHz, RTT+AOA, Relative | Distance | 0.033 | 0.050 | 0.069 | 0.097 |
| Case number |  | 50% | 67% | 80% | 90% |
| 4101, Commercial, [UE], 20MHz, RTT+AOA, Relative | Angle | 1.330 | 2.453 | 4.373 | 9.267 |
| 4102, Commercial, [UE], 40MHz, RTT+AOA, Relative | Angle | 1.056 | 1.898 | 3.353 | 6.771 |
| 4103, Commercial, [UE], 100MHz, RTT+AOA, Relative | Angle | 0.716 | 1.266 | 2.242 | 4.452 |
| 4104, Commercial, [UE,RSU], 20MHz, RTT+AOA, Relative | Angle | 1.316 | 2.414 | 4.223 | 8.281 |
| 4105, Commercial, [UE,RSU], 40MHz, RTT+AOA, Relative | Angle | 1.852 | 2.819 | 4.157 | 6.117 |
| 4106, Commercial, [UE,RSU], 100MHz, RTT+AOA, Relative | Angle | 0.426 | 0.653 | 1.004 | 1.485 |

Observation 17: For commercial use case relative positioning between two UEs within 10m, the accuracy of less than 1m@90% cannot be achieved with 40MHz bandwidth for SL UE-UE only positioning, the accuracy of less than 1m@90% can be achieved with at least 40MHz bandwidth for joint UE-UE and RSU-UE positioning.

- Observation 18: For commercial use case ranging, the distance accuracy of less than 1m@90% can be achieved with at least 40MHz bandwidth for joint UE-UE and RSU-UE positioning, the distance accuracy of less than 0.1m@90% can be achieved with at least 100MHz bandwidth for joint UE-UE and RSU-UE positioning.

Observation 19: For commercial use case ranging, the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with at least 40MHz bandwidth, the angle accuracy of less than 2°@90% can be achieved with at least 100MHz bandwidth for joint UE-UE and RSU-UE positioning.

# Evaluations for IIOT use cases

## General description

The scenarios to be presented in this contribution include

- InF-SH

- InF-DH

The layout parameters could reference Table 6.1-1 in [4]. The UEs to be evaluated are randomly dropped in the hall.

## Absolute positioning

The evaluated cases are for FR1, and the results of SL only positioning and joint Uu/SL positioning are provided for Multi-RTT. The general description of all evaluated cases is shown in the Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 18.

Table 18  Positioning accuracy (meter)

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 5001, IIOT, SH, [RSU], 20MHz, RTT, Absolute | 0.649 | 0.975 | 1.336 | 2.084 |
| 5002, IIOT, SH, [RSU], 40MHz, RTT, Absolute | 0.274 | 0.392 | 0.557 | 0.791 |
| 5003, IIOT, SH, [RSU], 100MHz, RTT, Absolute | 0.058 | 0.092 | 0.130 | 0.182 |
| 5004, IIOT, SH, [BS,RSU], 20MHz, RTT, Absolute | 0.046 | 0.073 | 0.115 | 0.215 |
| 5005, IIOT, SH, [BS,RSU], 40MHz, RTT, Absolute | 0.045 | 0.072 | 0.114 | 0.178 |
| 5006, IIOT, SH, [BS,RSU], 100MHz, RTT, Absolute | 0.030 | 0.040 | 0.057 | 0.085 |
| 5007, IIOT, DH, [RSU], 20MHz, RTT, Absolute | 0.914 | 1.363 | 1.835 | 2.710 |
| 5008, IIOT, DH, [RSU], 40MHz, RTT, Absolute | 0.362 | 0.540 | 0.735 | 0.972 |
| 5009, IIOT, DH, [RSU], 100MHz, RTT, Absolute | 0.065 | 0.098 | 0.135 | 0.196 |
| 5010, IIOT, DH, [BS,RSU], 20MHz, RTT, Absolute | 0.070 | 0.114 | 0.182 | 0.320 |
| 5011, IIOT, DH, [BS,RSU], 40MHz, RTT, Absolute | 0.065 | 0.103 | 0.166 | 0.287 |
| 5012, IIOT, DH, [BS,RSU], 100MHz, RTT, Absolute | 0.037 | 0.052 | 0.071 | 0.097 |

Observation 20: For IIOT InF-SH absolute positioning with joint Uu/SL, the accuracy of less than 0.2m@90% (requirement Set B) can be achieved with at least 40MHz bandwidth.

Observation 21: For IIOT InF-DH absolute positioning, the accuracy of less than 1m@90% (requirement Set A) can be achieved with at least 40MHz bandwidth, and the accuracy of less than 0.2m@90% (requirement Set B) can be achieved with at least 100MHz bandwidth.

## Relative positioning

The evaluated cases are for FR1, and the results of SL UE-UE only positioning and joint UE-UE and RSU-UE positioning are provided for Multi-RTT+AOA. The largest distance between the two UEs is 10m. The general description of all evaluated cases is shown in the Appendix.

The positioning error of 50%, 67%, 80%, and 90% for all cases are summarized in Table 19.

Table 19  Positioning accuracy (meter)

| Case number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| 5101, IIOT, SH, [UE], 20MHz, RTT+AOA, Relative | 0.813 | 1.351 | 1.902 | 2.555 |
| 5102, IIOT, SH, [UE], 40MHz, RTT+AOA, Relative | 0.469 | 0.713 | 1.018 | 1.489 |
| 5103, IIOT, SH, [UE], 100MHz, RTT+AOA, Relative | 0.136 | 0.215 | 0.358 | 0.899 |
| 5104, IIOT, SH, [RSU,UE], 20MHz, RTT+AOA, Relative | 0.812 | 1.224 | 1.753 | 2.429 |
| 5105, IIOT, SH, [RSU,UE], 40MHz, RTT+AOA, Relative | 0.352 | 0.532 | 0.722 | 1.022 |
| 5106, IIOT, SH, [RSU,UE], 100MHz, RTT+AOA, Relative | 0.077 | 0.123 | 0.185 | 0.304 |
| 5107, IIOT, DH, [UE], 20MHz, RTT+AOA, Relative | 0.856 | 1.381 | 1.923 | 2.596 |
| 5108, IIOT, DH, [UE], 40MHz, RTT+AOA, Relative | 0.456 | 0.672 | 0.995 | 1.620 |
| 5109, IIOT, DH, [UE], 100MHz, RTT+AOA, Relative | 0.121 | 0.196 | 0.334 | 0.844 |
| 5110, IIOT, DH, [RSU,UE], 20MHz, RTT+AOA, Relative | 0.907 | 1.425 | 1.972 | 2.573 |
| 5111, IIOT, DH, [RSU,UE], 40MHz, RTT+AOA, Relative | 0.396 | 0.590 | 0.787 | 1.095 |
| 5112, IIOT, DH, [RSU,UE], 100MHz, RTT+AOA, Relative | 0.075 | 0.116 | 0.172 | 0.271 |

Observation 22: For IIOT InF-SH or InF-DH relative positioning with joint UE-UE and RSU-UE positioning, the accuracy of less than 1m@90% (requirement Set A) can be approximately achieved with 40MHz bandwidth.

Observation 23: For IIOT InF-SH or InF-DH relative positioning with joint UE-UE and RSU-UE positioning, the accuracy requirement 0.2m@90% (requirement Set B) cannot be met with 100MHz bandwidth.

# Conclusion

In this contribution, we provided evaluations on SL positioning including joint Uu/SL positioning. With the outcome of the evaluation, we have the following observations.

- Observation 1: For V2X Highway absolute positioning, the accuracy of less than 1.5m@90% (requirement Set A) can be achieved with at least 40MHz bandwidth, and the accuracy of less than 0.5m@90% (requirement Set B) can be achieved with at least 100MHz bandwidth.

- Observation 2: For V2X Highway relative positioning between two UEs within X=50m and 150m, 1.5m@90% (requirement Set A) cannot be achieved with 100MHz bandwidth for SL UE-UE only positioning.

- Observation 3: For V2X Highway relative positioning between two UEs within X=50m and 150m, RSU assisted positioning could help to increase the accuracy, the accuracy of less than 1.5m@90% (requirement Set A) can be achieved with at least 40MHz bandwidth for X=50m and with at least 100MHz bandwidth for X=150m for joint UE-UE and RSU-UE positioning.

- Observation 4: For V2X Highway relative positioning between two UEs within X=150m, 100MHz bandwidth cannot met the requirement Set B.

- Observation 5: For V2X Highway ranging, the distance accuracy of less than 1.5m@90% (requirement Set A) can be achieved with 20MHz bandwidth, and the distance accuracy of less than 0.5m@90% (requirement Set B) can be achieved with at least 40MHz bandwidth for X=50m and with at least 100MHz bandwidth for X=150m.

- Observation 6: For V2X Highway ranging, the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with 20MHz bandwidth for X=50m for all cases, and the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with 20MHz bandwidth for X=150m for joint UE-UE and RSU-UE positioning .

- Observation 7: For V2X Urban grid absolute positioning, the accuracy of less than 3m@90% can be achieved with 40MHz bandwidth for joint Uu/SL positioning, but both requirement Set A and requirement Set B cannot be met.

- Observation 8: For V2X Urban grid relative positioning, the accuracy of less than 1.5m@90% (requirement Set A) can be achieved with at least 100MHz bandwidth for two UEs within X=50m for SL UE-UE only positioning.

- Observation 9: For V2X Urban grid relative positioning between two UEs within X=10m and 50m, BS and RSU assisted positioning could help to increase the performance, the accuracy is almost the same as the LOS-only UE-UE positioning.

- Observation 10: For V2X Urban grid ranging between two UEs within X=10m and 50m, the distance accuracy of less than 0.5m@90% (requirement Set B) can be achieved with at least 100MHz bandwidth.

- Observation 11: For V2X Urban grid ranging between two UEs within X=10m and 50m, the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with at least 100MHz bandwidth for joint Uu and SL positioning.

Observation 12: For public safety absolute positioning, the accuracy requirement 1m@90% cannot be achieved.

Observation 13: For public safety relative positioning and ranging, the accuracy requirement 1m@90% cannot be achieved with 10MHz bandwidth.

Observation 14: For public safety relative positioning, the accuracy requirement 1m@90% can be achieved with at least 100MHz bandwidth.

Observation 15: For public safety ranging, the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with at least 20MHz bandwidth for joint UE-UE and RSU-UE positioning.

Observation 16: For commercial use case absolute positioning, the accuracy of less than 1m@90% can be achieved with at least 40MHz bandwidth.

Observation 17: For commercial use case relative positioning between two UEs within 10m, the accuracy of less than 1m@90% cannot be achieved with 40MHz bandwidth for SL UE-UE only positioning, the accuracy of less than 1m@90% can be achieved with at least 40MHz bandwidth for joint UE-UE and RSU-UE positioning.

- Observation 18: For commercial use case ranging, the distance accuracy of less than 1m@90% can be achieved with at least 40MHz bandwidth for joint UE-UE and RSU-UE positioning, the distance accuracy of less than 0.1m@90% can be achieved with at least 100MHz bandwidth for joint UE-UE and RSU-UE positioning.

Observation 19: For commercial use case ranging, the angle accuracy of less than 8°@90% (requirement Set B) can be achieved with at least 40MHz bandwidth, the angle accuracy of less than 2°@90% can be achieved with at least 100MHz bandwidth for joint UE-UE and RSU-UE positioning.

Observation 20: For IIOT InF-SH absolute positioning with joint Uu/SL, the accuracy of less than 0.2m@90% (requirement Set B) can be achieved with at least 40MHz bandwidth.

Observation 21: For IIOT InF-DH absolute positioning, the accuracy of less than 1m@90% (requirement Set A) can be achieved with at least 40MHz bandwidth, and the accuracy of less than 0.2m@90% (requirement Set B) can be achieved with at least 100MHz bandwidth.

Observation 22: For IIOT InF-SH or InF-DH relative positioning with joint UE-UE and RSU-UE positioning, the accuracy of less than 1m@90% (requirement Set A) can be approximately achieved with 40MHz bandwidth.

Observation 23: For IIOT InF-SH or InF-DH relative positioning with joint UE-UE and RSU-UE positioning, the accuracy requirement 0.2m@90% (requirement Set B) cannot be met with 100MHz bandwidth.

# Reference

- RAN1, Chair’s Notes RAN1#109-e v19, RAN1#109-e, e-Meeting, 9th – 20th May, 2022.

- TR 37.885, Study on NR Vehicle-to-Everything (V2X).

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

- TR 38.857, Study on NR Positioning Enhancements (Release 17).

- TR 36.843, Study on LTE Device to Device Proximity Service.

# Appendix

- This appendix shows all evaluated cases in the contribution.

| Case number | Enable UE-type RSUs or not | Enable BSs or not | Bandwidth of SL (MHz) | UE-type RSUs distribution | Condition between two UEs |
| --- | --- | --- | --- | --- | --- |
| 1001, V2X, Highway, Absolute | Yes | No | 20 | 200m spacing on both sides of highway symmetrically | / |
| 1002 | Yes | No | 40 |  | / |
| 1003 | Yes | No | 100 |  | / |
| 1004 | Yes | Yes (UMa) | 20 |  | / |
| 1005 | Yes | Yes (UMa) | 40 |  | / |
| 1006 | Yes | Yes (UMa) | 100 |  | / |
| 1007 | Yes | No | 20 | Staggered on both sides of the highway with distance of 200m | / |
| 1008 | Yes | No | 40 |  | / |
| 1009 | Yes | No | 100 |  | / |
| 1101, V2X, Highway, Relative | No | No | 20 | / | LOS, NLOSv X = 50m |
| 1102 | No | No | 40 | / |  |
| 1103 | No | No | 100 | / |  |
| 1104 | No | No | 20 | / | LOS X = 50m |
| 1105 | No | No | 40 | / |  |
| 1106 | No | No | 100 | / |  |
| 1107 | Yes | No | 20 | 200m spacing on both sides of highway symmetrically | LOS, NLOSv X = 50m |
| 1108 | Yes | No | 40 |  |  |
| 1109 | Yes | No | 100 |  |  |
| 1121, V2X, Highway, Relative | No | No | 20 | / | LOS, NLOSv X = 150m |
| 1122 | No | No | 40 | / |  |
| 1123 | No | No | 100 | / |  |
| 1124 | No | No | 20 | / | LOS X = 150m |
| 1125 | No | No | 40 | / |  |
| 1126 | No | No | 100 | / |  |
| 1127 | Yes | No | 20 | 200m spacing on both sides of highway symmetrically | LOS, NLOSv X = 150m |
| 1128 | Yes | No | 40 |  |  |
| 1129 | Yes | No | 100 |  |  |
| 2001, V2X, UrbanGrid, Absolute | Yes | No | 20 | In the middle of the crossroads | / |
| 2002 | Yes | No | 40 |  | / |
| 2003 | Yes | No | 100 |  | / |
| 2004 | Yes | Yes (UMa) | 20 |  | / |
| 2005 | Yes | Yes (UMa) | 40 |  | / |
| 2006 | Yes | Yes (UMa) | 100 |  | / |
| 2101, V2X, UrbanGrid, Relative | No | No | 20 | / | LOS, NLOSv X = 10m |
| 2102 | No | No | 40 | / |  |
| 2103 | No | No | 100 | / |  |
| 2104 | No | No | 20 | / | LOS X = 10m |
| 2105 | No | No | 40 | / |  |
| 2106 | No | No | 100 | / |  |
| 2107 | Yes | Yes (UMa) | 20 | In the middle of the crossroads | LOS, NLOSv X = 10m |
| 2108 | Yes | Yes (UMa) | 40 |  |  |
| 2109 | Yes | Yes (UMa) | 100 |  |  |
| 2121, V2X, UrbanGrid, Relative | No | No | 20 | / | LOS, NLOSv X = 50m |
| 2122 | No | No | 40 | / |  |
| 2123 | No | No | 100 | / |  |
| 2124 | No | No | 20 | / | LOS X = 50m |
| 2125 | No | No | 40 | / |  |
| 2126 | No | No | 100 | / |  |
| 2127 | Yes | Yes (UMa) | 20 | In the middle of the crossroads | LOS, NLOSv X = 50m |
| 2128 | Yes | Yes (UMa) | 40 |  |  |
| 2129 | Yes | Yes (UMa) | 100 |  |  |
| 3001, PS, Absolute | Yes | No | 10 | Distributed on a square lattice with spacing 200m (4×4, 16 in total) | / |
| 3002 | Yes | No | 20 |  | / |
| 3003 | Yes | No | 40 |  | / |
| 3004 | Yes | No | 100 |  | / |
| 3005 | Yes | Yes (UMa) | 10 |  | / |
| 3006 | Yes | Yes (UMa) | 20 |  | / |
| 3007 | Yes | Yes (UMa) | 40 |  | / |
| 3008 | Yes | Yes (UMa) | 100 |  | / |
| 3101, PS, Relative | No | No | 10 | / | LOS X = 20m |
| 3102 | No | No | 20 | / |  |
| 3103 | No | No | 40 | / |  |
| 3104 | No | No | 100 | / |  |
| 3105 | Yes | No | 10 | Distributed on a square lattice with spacing 200m (4×4, 16 in total) |  |
| 3106 | Yes | No | 20 |  |  |
| 3107 | Yes | No | 40 |  |  |
| 3108 | Yes | No | 100 |  |  |
| 4001, Commercial, Absolute | Yes | No | 20 | Distributed on a square lattice with spacing 20m in InH. | / |
| 4002 | Yes | No | 40 |  | / |
| 4003 | Yes | No | 100 |  | / |
| 4101, Commercial Relative | No | No | 20 | / | LOS X = 10m |
| 4102 | No | No | 40 | / |  |
| 4103 | No | No | 100 | / |  |
| 4104 | Yes | No | 20 | Distributed on a square lattice with spacing 20m in InH. |  |
| 4105 | Yes | No | 40 |  |  |
| 4106 | Yes | No | 100 |  |  |
| 5001, IIoT, InF-SH, Absolute | Yes | No | 20 | Distributed on a square lattice with spacing D near the wall and at the center of each square of 4 BSs (4×7, 28 in total) InF-SH: D=50m InF-DH: D=20m | / |
| 5002 | Yes | No | 40 |  | / |
| 5003 | Yes | No | 100 |  | / |
| 5004 | Yes | Yes | 20 |  | / |
| 5005 | Yes | Yes | 40 |  | / |
| 5006 | Yes | Yes | 100 |  | / |
| 5007, IIoT, InF-DH, Absolute | Yes | No | 20 |  | / |
| 5008, | Yes | No | 40 |  | / |
| 5009 | Yes | No | 100 |  | / |
| 5010 | Yes | Yes | 20 |  | / |
| 5011 | Yes | Yes | 40 |  | / |
| 5012 | Yes | Yes | 100 |  | / |
| 5101, IIoT, InF-SH, Relative | No | No | 20 | Distributed on a square lattice with spacing D near the wall and at the center of each square of 4 BSs (4×7, 28 in total) InF-SH: D=50m InF-DH: D=20m | LOS, NLOS X = 10m |
| 5102 | No | No | 40 |  |  |
| 5103 | No | No | 100 |  |  |
| 5104 | Yes | No | 20 |  |  |
| 5105 | Yes | No | 40 |  |  |
| 5106 | Yes | No | 100 |  |  |
| 5107, IIoT, InF-DH, Relative | No | No | 20 |  |  |
| 5108 | No | No | 40 |  |  |
| 5109 | No | No | 100 |  |  |
| 5110 | Yes | No | 20 |  |  |
| 5111 | Yes | No | 40 |  |  |
| 5112 | Yes | No | 100 |  |  |