## B.1.X	Results from source [X]

### B.1.X.1	Description of evaluation scenarios

Common assumptions applicable to all evaluated scenarios that are different from or not provided in Tables A.1-1 through A.1-6 are provided in Table B.1.X.1-1.

Table B.1.X.1-1: Common assumptions for sidelink positioning evaluations that are different from or not provided in Annex A.1 from [X]

| Parameter |  |
| --- | --- |
| Subcarrier spacing | 30kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Existing pattern and sequence of DL-PRS (Comb-4, 4 symbol) |
| Reference signal including PRS, SRS and SL-PRS (type of sequence, number of ports, …) | Gold, single port |
| Number of symbols used per occasion | 4 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 6dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | PSO |
| Synchronization assumptions | Ideal |
| UE/gNB RX and TX timing error assumption | Ideal |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |

Evaluation cases and relevant additional assumptions for highway scenarios for V2X use cases are provided in Table B.1.X.1-2 - Table B.1.X.1-4.

Table B.1.X.1-2: Assumptions for sidelink positioning in highway scenarios for V2X use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 1001 | Case 1002 | Case 1003 | Case 1004 | Case 1005 | Case 1006 | Case 1007 | Case 1008 | Case 1009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | Uu: 4GHz SL: 6GHz |  |  |  |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |  |  |  |
| TRP antenna model | UPA 4x4x2 |  |  |  |  |  |  |  |  |
| BS deployment for absolute positioning | / |  |  | UMa |  |  | / |  |  |
| RSU deployment for absolute positioning | 200m spacing on both sides of highway symmetrically |  |  |  |  |  | Staggered on both sides of the highway with distance of 200m |  |  |
| Positioning method | RTT |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20MHz | 40MHz | 100MHz | 20MHz | 40MHz | 100MHz | 20MHz | 40MHz | 100MHz |

Table B.1.X.1-3: Assumptions for sidelink positioning in highway scenarios for V2X use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 1101 | Case 1102 | Case 1103 | Case 1104 | Case 1105 | Case 1106 | Case 1107 | Case 1108 | Case 1109 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | SL: 6GHz |  |  |  |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |  |  |  |
| BS deployment for relative positioning/ranging | / |  |  |  |  |  |  |  |  |
| RSU deployment for relative positioning/ranging | / |  |  |  |  |  | 200m spacing on both sides of highway symmetrically |  |  |
| Positioning method | RTT+AOA |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20MHz | 40MHz | 100MHz | 20MHz | 40MHz | 100MHz | 20MHz | 40MHz | 100MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | 50m |  |  |  |  |  |  |  |  |
| LOS condition between two UEs | LOS, NLOSv |  |  |  |  |  |  |  |  |

Table B.1.X.1-4: Assumptions for sidelink positioning in highway scenarios for V2X use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 1121 | Case 1122 | Case 1123 | Case 1124 | Case 1125 | Case 1126 | Case 1127 | Case 1128 | Case 1129 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | SL: 6GHz |  |  |  |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |  |  |  |
| BS deployment for relative positioning/ranging | / |  |  |  |  |  |  |  |  |
| RSU deployment for relative positioning/ranging | / |  |  |  |  |  | 200m spacing on both sides of highway symmetrically |  |  |
| Positioning method | RTT+AOA |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20MHz | 40MHz | 100MHz | 20MHz | 40MHz | 100MHz | 20MHz | 40MHz | 100MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | 150m |  |  |  |  |  |  |  |  |
| LOS condition between two UEs | LOS, NLOSv |  |  |  |  |  |  |  |  |

Evaluation cases and relevant additional assumptions for urban grid scenarios for V2X use cases are provided in Table B.1.X.1-5 - Table B.1.X.1-7.

Table B.1.X.1-5: Assumptions for sidelink positioning in urban grid scenarios for V2X use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 2001 | Case 2002 | Case 2003 | Case 2004 | Case 2005 | Case 2006 |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | Uu: 4GHz SL: 6GHz |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |
| TRP antenna model | UPA 4x4x2 |  |  |  |  |  |
| BS deployment for absolute positioning | / |  |  | UMa |  |  |
| RSU deployment for absolute positioning | In the middle of the crossroads |  |  |  |  |  |
| Positioning method | RTT |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20MHz | 40MHz | 100MHz | 20MHz | 40MHz | 100MHz |

Table B.1.X.1-6: Assumptions for sidelink positioning in urban grid scenarios for V2X use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 2101 | Case 2102 | Case 2103 | Case 2104 | Case 2105 | Case 2106 | Case 2107 | Case 2108 | Case 2109 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | SL: 6GHz |  |  |  |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |  |  |  |
| BS deployment for relative positioning/ranging | / |  |  |  |  |  | UMa |  |  |
| RSU deployment for relative positioning/ranging | / |  |  |  |  |  | In the middle of the crossroads |  |  |
| Positioning method | RTT+AOA |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | 10m |  |  |  |  |  |  |  |  |
| LOS condition between two UEs | LOS, NLOSv |  |  |  |  |  |  |  |  |

Table B.1.X.1-7: Assumptions for sidelink positioning in urban grid scenarios for V2X use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 2121 | Case 2122 | Case 2123 | Case 2124 | Case 2125 | Case 2126 | Case 2127 | Case 2128 | Case 2129 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | SL: 6GHz |  |  |  |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |  |  |  |
| BS deployment for relative positioning/ranging | / |  |  |  |  |  | UMa |  |  |
| RSU deployment for relative positioning/ranging | / |  |  |  |  |  | In the middle of the crossroads |  |  |
| Positioning method | RTT+AOA |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | 50m |  |  |  |  |  |  |  |  |
| LOS condition between two UEs | LOS, NLOSv |  |  |  |  |  |  |  |  |

Evaluation cases and relevant additional assumptions for IIoT use cases are provided in Table B.1.X.1-8 - Table B.1.X.1-9.

Table B.1.X.1-8: Assumptions for sidelink positioning for IIoT use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 5001 | Case 5002 | Case 5003 | Case 5004 | Case 5005 | Case 5006 | Case 5007 | Case 5008 | Case 5009 | Case 5010 | Case 5011 | Case 5012 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | Uu: 3.5GHz SL: 3.5GHz |  |  |  |  |  |  |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |  |  |  |  |  |  |
| TRP antenna model | UPA 4x4x2 |  |  |  |  |  |  |  |  |  |  |  |
| BS deployment for absolute positioning | / |  |  | 18 BSs on a square lattice with spacing 50m |  |  | / |  |  | 18 BSs on a square lattice with spacing 20m |  |  |
| RSU deployment for absolute positioning | Distributed on a square lattice with spacing 50m near the wall and at the center of each square of 4 BSs (4×7, 28 in total) |  |  |  |  |  | Distributed on a square lattice with spacing 20m near the wall and at the center of each square of 4 BSs (4×7, 28 in total) |  |  |  |  |  |
| Positioning method | RTT |  |  |  |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz |

Table B.1.X.1-9: Assumptions for sidelink positioning for IIoT use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 5101 | Case 5102 | Case 5103 | Case 5104 | Case 5105 | Case 5106 | Case 5107 | Case 5108 | Case 5109 | Case 5110 | Case 5111 | Case 5112 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | SL: 3.5GHz |  |  |  |  |  |  |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |  |  |  |  |  |  |
| RSU deployment for relative positioning/ranging | / |  |  | Distributed on a square lattice with spacing 50m near the wall and at the center of each square of 4 BSs (4×7, 28 in total) |  |  | / |  |  | Distributed on a square lattice with spacing 20m near the wall and at the center of each square of 4 BSs (4×7, 28 in total) |  |  |
| Positioning method | RTT+AOA |  |  |  |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz | 20 MHz | 40 MHz | 100 MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | 10m |  |  |  |  |  |  |  |  |  |  |  |
| LOS condition between two UEs | LOS, NLOS |  |  |  |  |  |  |  |  |  |  |  |

Evaluation cases and relevant additional assumptions for public safety use cases are provided in Table B.1.X.1-10 - Table B.1.X.1-11.

Table B.1.X.1-10: Assumptions for sidelink positioning for public safety use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 3001 | Case 3002 | Case 3003 | Case 3004 | Case 3005 | Case 3006 | Case 3007 | Case 3008 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | Uu: 3.5GHz SL: 0.7GHz |  |  |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |  |  |
| TRP antenna model | UPA 4x4x2 |  |  |  |  |  |  |  |
| BS deployment for absolute positioning | / |  |  |  | UMa |  |  |  |
| RSU deployment for absolute positioning | Distributed on a square lattice with spacing 200m (4×4, 16 in total) |  |  |  |  |  |  |  |
| Positioning method | RTT |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz |

Table B.1.X.1-11: Assumptions for sidelink positioning for public safety use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 3101 | Case 3102 | Case 3103 | Case 3104 | Case 3105 | Case 3106 | Case 3107 | Case 3108 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | SL: 0.7GHz |  |  |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |  |  |
| RSU deployment for relative positioning/ranging | / |  |  |  | Distributed on a square lattice with spacing 200m (4×4, 16 in total) |  |  |  |
| Positioning method | RTT+AOA |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | 20m |  |  |  |  |  |  |  |
| LOS condition between two UEs | LOS |  |  |  |  |  |  |  |

Evaluation cases and relevant additional assumptions for commercial use cases are provided in Table B.1.X.1-12 - Table B.1.X.1-13.

Table B.1.X.1-12: Assumptions for sidelink positioning for commercial use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 4001 | Case 4002 | Case 4003 |
| --- | --- | --- | --- |
| Carrier frequency | SL: 3.5GHz |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |
| TRP antenna model | UPA 4x4x2 |  |  |
| RSU deployment for absolute positioning | Distributed on a square lattice with spacing 20m (6×2, 12 in total) |  |  |
| Positioning method | RTT |  |  |
| Reference Signal Transmission Bandwidth | 20MHz | 40MHz | 100MHz |

Table B.1.X.1-13: Assumptions for sidelink positioning for commercial use cases that are different from or not provided in Annex A.1 from [X]

| Parameter | Case 4101 | Case 4102 | Case 4103 | Case 4104 | Case 4105 | Case 4106 |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | SL: 3.5GHz |  |  |  |  |  |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) dH=dV=0.5λ |  |  |  |  |  |
| RSU deployment for relative positioning | / |  |  | Distributed on a square lattice with spacing 20m (6×2, 12 in total) |  |  |
| Positioning method | RTT+AOA |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20MHz | 40MHz | 100MHz | 20MHz | 40MHz | 100MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | 10m |  |  |  |  |  |
| LOS condition between two UEs | LOS |  |  |  |  |  |

### B.1.X.2	Positioning accuracy evaluation results for Sidelink Positioning

#### B.1.X.2.1	Positioning accuracy evaluation results for Sidelink Positioning for Highway Scenarios for V2X

Table B.1.X.2.1-1 provides horizontal absolute positioning accuracy results using sidelink positioning for highway scenarios for V2X use cases.

Table B.1.X.2.1-1: Sidelink positioning - horizontal absolute accuracy for highway scenarios for V2X use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 1001, V2X, Highway, [RSU], 20MHz, RTT, Absolute | 0.570 | 0.828 | 1.165 | 1.631 | 87% | 44% |
| 1002, V2X, Highway, [RSU], 40MHz, RTT, Absolute | 0.383 | 0.586 | 0.861 | 1.231 | Yes | 61% |
| 1003, V2X, Highway, [RSU], 100MHz, RTT, Absolute | 0.120 | 0.183 | 0.275 | 0.432 | Yes | Yes |
| 1004, V2X, Highway, [BS,RSU], 20MHz, RTT, Absolute | 0.440 | 0.691 | 0.968 | 1.458 | Yes | 55% |
| 1005, V2X, Highway, [BS,RSU], 40MHz, RTT, Absolute | 0.325 | 0.484 | 0.741 | 1.091 | Yes | 68% |
| 1006, V2X, Highway, [BS,RSU], 100MHz, RTT, Absolute | 0.106 | 0.162 | 0.253 | 0.429 | Yes | Yes |
| 1007, V2X, Highway, [StaggerRSU], 20MHz, RTT, Absolute | 0.554 | 0.832 | 1.320 | 1.844 | 85% | 47% |
| 1008, V2X, Highway, [StaggerRSU], 40MHz, RTT, Absolute | 0.404 | 0.602 | 0.916 | 1.415 | Yes | 58% |
| 1009, V2X, Highway, [StaggerRSU], 100MHz, RTT, Absolute | 0.123 | 0.202 | 0.313 | 0.494 | Yes | Yes |

Table B.1.X.2.1-2 provides horizontal relative positioning accuracy results using sidelink positioning for highway scenarios for V2X use cases.

Table B.1.X.2.1-2: Sidelink positioning - horizontal relative accuracy for highway scenarios for V2X use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 1101, V2X, Highway, [UE], 20MHz, RTT+AOA, Relative, X=50m | 0.902 | 1.438 | 2.253 | 3.448 | 68% | 26% |
| 1102, V2X, Highway, [UE], 40MHz, RTT+AOA, Relative, X=50m | 0.674 | 1.144 | 1.886 | 2.785 | 74% | 37% |
| 1103, V2X, Highway, [UE], 100MHz, RTT+AOA, Relative, X=50m | 0.482 | 0.773 | 1.210 | 2.148 | 84% | 53% |
| 1104, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.869 | 1.350 | 2.071 | 2.954 | 70% | 28% |
| 1105, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.651 | 1.065 | 1.763 | 2.551 | 77% | 38% |
| 1106, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.273 | 0.402 | 0.599 | 0.955 | Yes | 72% |
| 1107, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative, X=50m | 0.561 | 0.807 | 1.137 | 1.571 | 45% | 89% |
| 1108, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative, X=50m | 0.373 | 0.573 | 0.819 | 1.182 | Yes | 62% |
| 1109, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative, X=50m | 0.119 | 0.180 | 0.264 | 0.407 | Yes | Yes |
| 1121, V2X, Highway, [UE], 20MHz, RTT+AOA, Relative, X=150m | 3.286 | 5.992 | 10.899 | 24.749 | 29% | 10% |
| 1122, V2X, Highway, [UE], 40MHz, RTT+AOA, Relative, X=150m | 2.544 | 4.691 | 8.469 | 22.251 | 36% | 15% |
| 1123, V2X, Highway, [UE], 100MHz, RTT+AOA, Relative, X=150m | 1.665 | 3.319 | 6.495 | 24.901 | 47% | 24% |
| 1124, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=150m | 2.629 | 4.79 | 7.165 | 11.099 | 33% | 12% |
| 1125, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=150m | 2.017 | 3.326 | 5.318 | 8.364 | 38% | 17% |
| 1126, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=150m | 1.294 | 2.407 | 3.971 | 6.256 | 57% | 27% |
| 1127, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative, X=150m | 0.980 | 1.547 | 2.280 | 3.636 | 65% | 25% |
| 1128, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative, X=150m | 0.735 | 1.097 | 1.650 | 2.445 | 77% | 37% |
| 1129, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative, X=150m | 0.252 | 0.384 | 0.609 | 1.046 | Yes | 75% |

Table B.1.X.2.1-3 provides ranging distance accuracy results using sidelink positioning for highway scenarios for V2X use cases.

Table B.1.X.2.1-3: Sidelink positioning - ranging distance accuracy for highway scenarios for V2X use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 1101, V2X, Highway, [UE], 20MHz, Relative, X=50m | 0.234 | 0.369 | 0.524 | 0.749 | Yes | 48% |
| 1102, V2X, Highway, [UE], 40MHz, Relative, X=50m | 0.160 | 0.226 | 0.332 | 0.451 | Yes | Yes |
| 1103, V2X, Highway, [UE], 100MHz, Relative, X=50m | 0.045 | 0.070 | 0.102 | 0.148 | Yes | Yes |
| 1104, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.226 | 0.351 | 0.495 | 0.687 | Yes | 47% |
| 1105, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.155 | 0.221 | 0.309 | 0.430 | Yes | Yes |
| 1106, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.044 | 0.067 | 0.094 | 0.137 | Yes | Yes |
| 1107, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative, X=50m | 0.193 | 0.323 | 0.481 | 0.732 | Yes | 51% |
| 1108, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative, X=50m | 0.153 | 0.232 | 0.347 | 0.502 | Yes | 65% |
| 1109, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative, X=50m | 0.049 | 0.077 | 0.115 | 0.176 | Yes | Yes |
| 1121, V2X, Highway, [UE], 20MHz, Relative, X=150m | 0.246 | 0.396 | 0.656 | 1.343 | Yes | 45% |
| 1122, V2X, Highway, [UE], 40MHz, Relative, X=150m | 0.172 | 0.267 | 0.419 | 0.747 | Yes | 62% |
| 1123, V2X, Highway, [UE], 100MHz, Relative, X=150m | 0.056 | 0.092 | 0.148 | 0.248 | Yes | Yes |
| 1124, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=150m | 0.217 | 0.316 | 0.470 | 0.688 | Yes | 49% |
| 1125, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=150m | 0.152 | 0.219 | 0.305 | 0.451 | Yes | Yes |
| 1126, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=150m | 0.047 | 0.073 | 0.106 | 0.154 | Yes | Yes |
| 1127, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative, X=150m | 0.226 | 0.335 | 0.513 | 0.943 | Yes | 50% |
| 1128, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative, X=150m | 0.158 | 0.238 | 0.340 | 0.573 | Yes | 61% |
| 1129, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative, X=150m | 0.053 | 0.081 | 0.120 | 0.200 | Yes | Yes |

Table B.1.X.2.1-4 provides ranging angle accuracy results using sidelink positioning for highway scenarios for V2X use cases.

Table B.1.X.2.1-4: Sidelink positioning - ranging angle accuracy for highway scenarios for V2X use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 1101, V2X, Highway, [UE], 20MHz, Relative, X=50m | 1.800 | 2.973 | 4.412 | 6.549 | Yes | Yes |
| 1102, V2X, Highway, [UE], 40MHz, Relative, X=50m | 1.449 | 2.463 | 3.624 | 5.600 | Yes | Yes |
| 1103, V2X, Highway, [UE], 100MHz, Relative, X=50m | 1.086 | 1.673 | 2.704 | 4.348 | Yes | Yes |
| 1104, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=50m | 1.721 | 2.846 | 3.980 | 5.881 | Yes | Yes |
| 1105, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=50m | 1.357 | 2.316 | 3.362 | 4.756 | Yes | Yes |
| 1106, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=50m | 1.030 | 1.600 | 2.446 | 3.805 | Yes | Yes |
| 1107, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative, X=50m | 0.874 | 1.553 | 2.353 | 4.423 | Yes | Yes |
| 1108, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative, X=50m | 0.574 | 1.016 | 1.817 | 3.129 | Yes | Yes |
| 1109, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative, X=50m | 0.192 | 0.349 | 0.633 | 1.029 | Yes | Yes |
| 1121, V2X, Highway, [UE], 20MHz, Relative, X=150m | 2.949 | 4.8 | 7.531 | 15.959 | 89% | 81% |
| 1122, V2X, Highway, [UE], 40MHz, Relative, X=150m | 2.119 | 3.79 | 5.775 | 16.8 | 89% | 85% |
| 1123, V2X, Highway, [UE], 100MHz, Relative, X=150m | 1.495 | 2.562 | 4.358 | 15.948 | 89% | 86% |
| 1124, V2X, Highway, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=150m | 2.4 | 4.021 | 5.333 | 7.683 | Yes | Yes |
| 1125, V2X, Highway, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=150m | 1.819 | 2.834 | 4.12 | 5.744 | Yes | Yes |
| 1126, V2X, Highway, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=150m | 1.256 | 1.983 | 2.885 | 4.132 | Yes | Yes |
| 1127, V2X, Highway, [UE,RSU], 20MHz, RTT+AOA, Relative, X=150m | 0.762 | 1.242 | 1.929 | 3.490 | Yes | Yes |
| 1128, V2X, Highway, [UE,RSU], 40MHz, RTT+AOA, Relative, X=150m | 0.554 | 0.932 | 1.448 | 2.382 | Yes | Yes |
| 1129, V2X, Highway, [UE,RSU], 100MHz, RTT+AOA, Relative, X=150m | 0.195 | 0.336 | 0.532 | 1.205 | Yes | Yes |

#### B.1.X.2.2	Positioning accuracy evaluation results for Sidelink Positioning for Urban Grid Scenarios for V2X

Table B.1.X.2.2-1 provides horizontal absolute positioning accuracy results using sidelink positioning for urban grid scenarios for V2X use cases.

Table B.1.X.2.2-1: Sidelink positioning - horizontal absolute accuracy for urban grid scenarios for V2X use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 2001, V2X, UrbanGrid, [RSU], 20MHz, RTT, Absolute | 4.749 | 6.996 | 9.938 | 16.222 | 15% | 4% |
| 2002, V2X, UrbanGrid, [RSU], 40MHz, RTT, Absolute | 3.911 | 7.001 | 10.070 | 12.726 | 20% | 8% |
| 2003, V2X, UrbanGrid, [RSU], 100MHz, RTT, Absolute | 3.509 | 5.782 | 7.88 | 10.489 | 20% | 17% |
| 2004, V2X, UrbanGrid, [BS,RSU], 20MHz, RTT, Absolute | 0.331 | 0.604 | 1.130 | 3.187 | 84% | 63% |
| 2005, V2X, UrbanGrid, [BS,RSU], 40MHz, RTT, Absolute | 0.258 | 0.548 | 1.100 | 2.777 | 84% | 65% |
| 2006, V2X, UrbanGrid, [BS,RSU], 100MHz, RTT, Absolute | 0.181 | 0.361 | 0.628 | 1.628 | 89% | 76% |

Table B.1.X.2.2-2 provides horizontal relative positioning accuracy results using sidelink positioning for urban grid scenarios for V2X use cases.

Table B.1.X.2.2-2: Sidelink positioning - horizontal relative accuracy for urban grid scenarios for V2X use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 2101, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative, X=10m | 1.072 | 1.621 | 2.485 | 4.254 | 64% | 20% |
| 2102, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative, X=10m | 0.878 | 1.203 | 1.723 | 2.614 | 75% | 24% |
| 2103, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative, X=10m | 0.286 | 0.403 | 0.621 | 0.974 | Yes | 73% |
| 2104, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=10m | 1.014 | 1.507 | 2.290 | 3.423 | 66% | 22% |
| 2105, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=10m | 0.848 | 1.129 | 1.548 | 2.300 | 78% | 26% |
| 2106, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=10m | 0.270 | 0.379 | 0.554 | 0.808 | Yes | 75% |
| 2107, V2X, UrbanGrid, [BS,RSU,UE], 20MHz, RTT+AOA, Relative, X=10m | 0.99 | 1.459 | 2.261 | 3.416 | 67% | 22% |
| 2108, V2X, UrbanGrid, [BS,RSU,UE], 40MHz, RTT+AOA, Relative, X=10m | 0.814 | 1.087 | 1.523 | 2.269 | 79% | 33% |
| 2109, V2X, UrbanGrid, [BS,RSU,UE], 100MHz, RTT+AOA, Relative, X=10m | 0.270 | 0.379 | 0.554 | 0.806 | Yes | 78% |
| 2121, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative, X=50m | 1.090 | 1.706 | 2.755 | 5.256 | 62% | 18% |
| 2122, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative, X=50m | 0.796 | 1.163 | 1.732 | 3.010 | 75% | 29% |
| 2123, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative, X=50m | 0.302 | 0.476 | 0.785 | 1.655 | 89% | 68% |
| 2124, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=50m | 1.025 | 1.543 | 2.324 | 4.235 | 66% | 19% |
| 2125, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.755 | 1.096 | 1.551 | 2.457 | 78% | 31% |
| 2126, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.280 | 0.439 | 0.671 | 1.071 | Yes | 72% |
| 2127, V2X, UrbanGrid, [BS,RSU,UE], 20MHz, RTT+AOA, Relative, X=50m | 1.001 | 1.500 | 2.316 | 4.233 | 67% | 25% |
| 2128, V2X, UrbanGrid, [BS,RSU,UE], 40MHz, RTT+AOA, Relative, X=50m | 0.732 | 1.077 | 1.535 | 2.466 | 79% | 32% |
| 2129, V2X, UrbanGrid, [BS,RSU,UE], 100MHz, RTT+AOA, Relative, X=50m | 0.275 | 0.422 | 0.671 | 1.060 | Yes | 72% |

Table B.1.X.2.2-3 provides ranging distance accuracy results using sidelink positioning for urban grid scenarios for V2X use cases.

Table B.1.X.2.2-3: Sidelink positioning - ranging distance accuracy for urban grid scenarios for V2X use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 2101, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative, X=10m | 0.736 | 1.261 | 2.066 | 3.387 | 70% | 38% |
| 2102, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative, X=10m | 0.605 | 0.898 | 1.280 | 2.063 | 83% | 42% |
| 2103, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative, X=10m | 0.090 | 0.141 | 0.208 | 0.286 | Yes | Yes |
| 2104, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=10m | 0.752 | 1.219 | 1.831 | 2.999 | 72% | 39% |
| 2105, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=10m | 0.606 | 0.888 | 1.258 | 1.997 | 84% | 42% |
| 2106, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=10m | 0.084 | 0.136 | 0.197 | 0.275 | Yes | Yes |
| 2107, V2X, UrbanGrid, [BS,RSU,UE], 20MHz, RTT+AOA, Relative, X=10m | 0.697 | 1.175 | 1.801 | 2.990 | 75% | 39% |
| 2108, V2X, UrbanGrid, [BS,RSU,UE], 40MHz, RTT+AOA, Relative, X=10m | 0.566 | 0.865 | 1.238 | 1.938 | 85% | 43% |
| 2109, V2X, UrbanGrid, [BS,RSU,UE], 100MHz, RTT+AOA, Relative, X=10m | 0.085 | 0.137 | 0.199 | 0.281 | Yes | Yes |
| 2121, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative, X=50m | 0.748 | 1.190 | 1.902 | 3.487 | 74% | 39% |
| 2122, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative, X=50m | 0.505 | 0.776 | 1.146 | 1.843 | 84% | 49% |
| 2123, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative, X=50m | 0.091 | 0.148 | 0.208 | 0.281 | Yes | Yes |
| 2124, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.698 | 1.125 | 1.728 | 2.899 | 76% | 41% |
| 2125, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.493 | 0.745 | 1.052 | 1.571 | 79% | 50% |
| 2126, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=50m | 0.087 | 0.138 | 0.196 | 0.255 | Yes | Yes |
| 2127, V2X, UrbanGrid, [BS,RSU,UE], 20MHz, RTT+AOA, Relative, X=50m | 0.674 | 1.101 | 1.717 | 2.920 | 76% | 39% |
| 2128, V2X, UrbanGrid, [BS,RSU,UE], 40MHz, RTT+AOA, Relative, X=50m | 0.486 | 0.718 | 1.047 | 1.586 | 89% | 51% |
| 2129, V2X, UrbanGrid, [BS,RSU,UE], 100MHz, RTT+AOA, Relative, X=50m | 0.085 | 0.137 | 0.198 | 0.251 | Yes | Yes |

Table B.1.X.2.2-4 provides ranging angle accuracy results using sidelink positioning for urban grid scenarios for V2X use cases.

Table B.1.X.2.2-4: Sidelink positioning - ranging angle accuracy for urban grid scenarios for V2X use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 2101, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative, X=10m | 3.309 | 5.358 | 8.345 | 14.954 | Yes | 79% |
| 2102, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative, X=10m | 2.710 | 4.160 | 6.544 | 12.487 | Yes | 85% |
| 2103, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative, X=10m | 1.988 | 3.220 | 4.914 | 8.660 | Yes | 89% |
| 2104, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=10m | 3.036 | 4.791 | 6.857 | 10.081 | Yes | 81% |
| 2105, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=10m | 2.457 | 3.648 | 5.489 | 8.059 | Yes | 89% |
| 2106, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=10m | 1.813 | 2.932 | 4.320 | 6.125 | Yes | Yes |
| 2107, V2X, UrbanGrid, [BS,RSU,UE], 20MHz, RTT+AOA, Relative, X=10m | 2.839 | 4.462 | 6.444 | 9.690 | Yes | 86% |
| 2108, V2X, UrbanGrid, [BS,RSU,UE], 40MHz, RTT+AOA, Relative, X=10m | 2.267 | 3.393 | 4.910 | 7.367 | Yes | Yes |
| 2109, V2X, UrbanGrid, [BS,RSU,UE], 100MHz, RTT+AOA, Relative, X=10m | 1.710 | 2.750 | 3.922 | 5.793 | Yes | Yes |
| 2121, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, Relative, X=50m | 2.933 | 4.805 | 7.909 | 15.600 | 89% | 80% |
| 2122, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, Relative, X=50m | 2.320 | 3.820 | 6.380 | 12.634 | Yes | 85% |
| 2123, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, Relative, X=50m | 1.847 | 3.083 | 5.070 | 10.245 | Yes | 87% |
| 2124, V2X, UrbanGrid, [UE], 20MHz, RTT+AOA, LOS-only, Relative, X=50m | 2.780 | 4.498 | 6.856 | 11.521 | Yes | 82% |
| 2125, V2X, UrbanGrid, [UE], 40MHz, RTT+AOA, LOS-only, Relative, X=50m | 2.220 | 3.491 | 5.400 | 9.473 | Yes | 87% |
| 2126, V2X, UrbanGrid, [UE], 100MHz, RTT+AOA, LOS-only, Relative, X=50m | 1.780 | 2.815 | 4.409 | 7.180 | Yes | Yes |
| 2127, V2X, UrbanGrid, [BS,RSU,UE], 20MHz, RTT+AOA, Relative, X=50m | 2.663 | 4.420 | 6.826 | 11.286 | Yes | 82% |
| 2128, V2X, UrbanGrid, [BS,RSU,UE], 40MHz, RTT+AOA, Relative, X=50m | 2.146 | 3.380 | 5.382 | 9.473 | Yes | 88% |
| 2129, V2X, UrbanGrid, [BS,RSU,UE], 100MHz, RTT+AOA, Relative, X=50m | 1.719 | 2.765 | 4.233 | 7.174 | Yes | Yes |

#### B.1.X.2.3	Positioning accuracy evaluation results for Sidelink Positioning for IIoT

Table B.1.X.2.3-1 provides horizontal absolute positioning accuracy results using sidelink positioning for IIoT use cases.

Table B.1.X.2.3-1: Sidelink positioning - horizontal absolute accuracy for IIoT use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 5001, IIOT, SH, [RSU], 20MHz, RTT, Absolute | 0.649 | 0.975 | 1.336 | 2.084 | 67% | 39% |
| 5002, IIOT, SH, [RSU], 40MHz, RTT, Absolute | 0.274 | 0.392 | 0.557 | 0.791 | Yes | 75% |
| 5003, IIOT, SH, [RSU], 100MHz, RTT, Absolute | 0.058 | 0.092 | 0.130 | 0.182 | Yes | Yes |
| 5004, IIOT, SH, [BS,RSU], 20MHz, RTT, Absolute | 0.046 | 0.073 | 0.115 | 0.215 | Yes | 89% |
| 5005, IIOT, SH, [BS,RSU], 40MHz, RTT, Absolute | 0.045 | 0.072 | 0.114 | 0.178 | Yes | Yes |
| 5006, IIOT, SH, [BS,RSU], 100MHz, RTT, Absolute | 0.030 | 0.040 | 0.057 | 0.085 | Yes | Yes |
| 5007, IIOT, DH, [RSU], 20MHz, RTT, Absolute | 0.914 | 1.363 | 1.835 | 2.710 | 54% | 6% |
| 5008, IIOT, DH, [RSU], 40MHz, RTT, Absolute | 0.362 | 0.540 | 0.735 | 0.972 | Yes | 25% |
| 5009, IIOT, DH, [RSU], 100MHz, RTT, Absolute | 0.065 | 0.098 | 0.135 | 0.196 | Yes | Yes |
| 5010, IIOT, DH, [BS,RSU], 20MHz, RTT, Absolute | 0.070 | 0.114 | 0.182 | 0.320 | Yes | 81% |
| 5011, IIOT, DH, [BS,RSU], 40MHz, RTT, Absolute | 0.065 | 0.103 | 0.166 | 0.287 | Yes | 84% |
| 5012, IIOT, DH, [BS,RSU], 100MHz, RTT, Absolute | 0.037 | 0.052 | 0.071 | 0.097 | Yes | Yes |

Table B.1.X.2.3-2 provides horizontal relative positioning accuracy results using sidelink positioning for IIoT use cases.

Table B.1.X.2.3-2: Sidelink positioning - horizontal relative accuracy for IIoT use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 5101, IIOT, SH, [UE], 20MHz, RTT+AOA, Relative | 0.813 | 1.351 | 1.902 | 2.555 | 56% | 8% |
| 5102, IIOT, SH, [UE], 40MHz, RTT+AOA, Relative | 0.469 | 0.713 | 1.018 | 1.489 | 79% | 23% |
| 5103, IIOT, SH, [UE], 100MHz, RTT+AOA, Relative | 0.136 | 0.215 | 0.358 | 0.899 | Yes | 64% |
| 5104, IIOT, SH, [RSU,UE], 20MHz, RTT+AOA, Relative | 0.812 | 1.224 | 1.753 | 2.429 | 59% | 9% |
| 5105, IIOT, SH, [RSU,UE], 40MHz, RTT+AOA, Relative | 0.352 | 0.532 | 0.722 | 1.022 | 89% | 26% |
| 5106, IIOT, SH, [RSU,UE], 100MHz, RTT+AOA, Relative | 0.077 | 0.123 | 0.185 | 0.304 | Yes | 81% |
| 5107, IIOT, DH, [UE], 20MHz, RTT+AOA, Relative | 0.856 | 1.381 | 1.923 | 2.596 | 57% | 7% |
| 5108, IIOT, DH, [UE], 40MHz, RTT+AOA, Relative | 0.456 | 0.672 | 0.995 | 1.620 | 80% | 25% |
| 5109, IIOT, DH, [UE], 100MHz, RTT+AOA, Relative | 0.121 | 0.196 | 0.334 | 0.844 | Yes | 67% |
| 5110, IIOT, DH, [RSU,UE], 20MHz, RTT+AOA, Relative | 0.907 | 1.425 | 1.972 | 2.573 | 53% | 6% |
| 5111, IIOT, DH, [RSU,UE], 40MHz, RTT+AOA, Relative | 0.396 | 0.590 | 0.787 | 1.095 | 89% | 25% |
| 5112, IIOT, DH, [RSU,UE], 100MHz, RTT+AOA, Relative | 0.075 | 0.116 | 0.172 | 0.271 | Yes | 83% |

#### B.1.X.2.4	Positioning accuracy evaluation results for Sidelink Positioning for Public Safety

Table B.1.X.2.4-1 provides horizontal absolute positioning accuracy results using sidelink positioning for public safety use cases.

Table B.1.X.2.4-1: Sidelink positioning - horizontal absolute accuracy for public safety use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the target requirement |
| --- | --- | --- | --- | --- | --- |
| 3001, PS, [RSU], 10MHz, RTT, Absolute | 9.402 | 21.234 | 40.737 | 58.496 | 3% |
| 3002, PS, [RSU], 20MHz, RTT, Absolute | 4.258 | 12.351 | 26.327 | 40.256 | 15% |
| 3003, PS, [RSU], 40MHz, RTT, Absolute | 1.619 | 3.869 | 11.990 | 22.276 | 35% |
| 3004, PS, [RSU], 100MHz, RTT, Absolute | 0.341 | 0.582 | 1.326 | 7.742 | 76% |
| 3005, PS, [BS,RSU], 10MHz, RTT, Absolute | 10.748 | 14.816 | 17.752 | 23.886 | 3% |
| 3006, PS, [BS,RSU], 20MHz, RTT, Absolute | 10.832 | 14.823 | 17.791 | 23.986 | 3% |
| 3007, PS, [BS,RSU], 40MHz, RTT, Absolute | 1.795 | 5.477 | 12.564 | 23.298 | 32% |
| 3008, PS, [BS,RSU], 100MHz, RTT, Absolute | 0.334 | 0.558 | 1.163 | 7.555 | 78% |

Table B.1.X.2.4-2 provides horizontal relative positioning accuracy results using sidelink positioning for public safety use cases.

Table B.1.X.2.4-2: Sidelink positioning - horizontal relative accuracy for public safety use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the target requirement |
| --- | --- | --- | --- | --- | --- |
| 3101, PS, [UE], 10MHz, RTT+AOA, Relative | 2.205 | 3.649 | 4.899 | 6.977 | 28% |
| 3102, PS, [UE], 20MHz, RTT+AOA, Relative | 0.699 | 1.098 | 1.628 | 2.351 | 63% |
| 3103, PS, [UE], 40MHz, RTT+AOA, Relative | 0.405 | 0.601 | 0.878 | 1.423 | 83% |
| 3104, PS, [UE], 100MHz, RTT+AOA, Relative | 0.176 | 0.295 | 0.497 | 0.956 | Yes |
| 3105, PS, [UE,RSU], 10MHz, RTT+AOA, Relative | 2.193 | 3.626 | 4.838 | 6.983 | 28% |
| 3106, PS, [UE,RSU], 20MHz, RTT+AOA, Relative | 0.703 | 1.106 | 1.622 | 2.341 | 63% |
| 3107, PS, [UE,RSU], 40MHz, RTT+AOA, Relative | 0.420 | 0.617 | 0.887 | 1.454 | 82% |
| 3108, PS, [UE,RSU], 100MHz, RTT+AOA, Relative | 0.208 | 0.332 | 0.544 | 0.971 | Yes |

Table B.1.X.2.4-3 provides ranging distance accuracy results using sidelink positioning for public safety use cases.

Table B.1.X.2.4-3: Sidelink positioning - ranging distance accuracy for public safety use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the target requirement |
| --- | --- | --- | --- | --- | --- |
| 3101, PS, [UE], 10MHz, RTT+AOA, Relative | 1.878 | 3.326 | 4.384 | 5.567 | 35% |
| 3102, PS, [UE], 20MHz, RTT+AOA, Relative | 0.463 | 0.775 | 1.217 | 1.830 | 74% |
| 3103, PS, [UE], 40MHz, RTT+AOA, Relative | 0.224 | 0.375 | 0.528 | 0.805 | Yes |
| 3104, PS, [UE], 100MHz, RTT+AOA, Relative | 0.049 | 0.085 | 0.130 | 0.215 | Yes |
| 3105, PS, [UE,RSU], 10MHz, RTT+AOA, Relative | 1.870 | 3.288 | 4.343 | 5.522 | 34% |
| 3106, PS, [UE,RSU], 20MHz, RTT+AOA, Relative | 0.473 | 0.784 | 1.222 | 1.803 | 73% |
| 3107, PS, [UE,RSU], 40MHz, RTT+AOA, Relative | 0.232 | 0.396 | 0.561 | 0.869 | Yes |
| 3108, PS, [UE,RSU], 100MHz, RTT+AOA, Relative | 0.072 | 0.122 | 0.191 | 0.330 | Yes |

Table B.1.X.2.4-4 provides ranging angle accuracy results using sidelink positioning for public safety use cases.

Table B.1.X.2.4-4: Sidelink positioning - ranging angle accuracy for public safety use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 3101, PS, [UE], 10MHz, RTT+AOA, Relative | 1.521 | 2.672 | 4.589 | 8.356 | Yes | 89% |
| 3102, PS, [UE], 20MHz, RTT+AOA, Relative | 1.239 | 2.135 | 3.432 | 6.293 | Yes | Yes |
| 3103, PS, [UE], 40MHz, RTT+AOA, Relative | 1.047 | 1.810 | 2.791 | 4.942 | Yes | Yes |
| 3104, PS, [UE], 100MHz, RTT+AOA, Relative | 0.832 | 1.402 | 2.245 | 3.785 | Yes | Yes |
| 3105, PS, [UE,RSU], 10MHz, RTT+AOA, Relative | 1.530 | 2.633 | 4.410 | 7.983 | Yes | Yes |
| 3106, PS, [UE,RSU], 20MHz, RTT+AOA, Relative | 1.245 | 2.111 | 3.383 | 5.994 | Yes | Yes |
| 3107, PS, [UE,RSU], 40MHz, RTT+AOA, Relative | 1.048 | 1.779 | 2.764 | 4.771 | Yes | Yes |
| 3108, PS, [UE,RSU], 100MHz, RTT+AOA, Relative | 0.832 | 1.405 | 2.240 | 3.642 | Yes | Yes |

#### B.1.X.2.5	Positioning accuracy evaluation results for Sidelink Positioning for Commercial use cases

Table B.1.X.2.5-1 provides horizontal absolute positioning accuracy results using sidelink positioning for commercial use cases.

Table B.1.X.2.5-1: Sidelink positioning - horizontal absolute accuracy for commercial use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the target requirement |
| --- | --- | --- | --- | --- | --- |
| 4001, Commercial, [RSU,UE], 20MHz, RTT, Absolute | 0.520 | 0.753 | 1.015 | 1.595 | 79% |
| 4002, Commercial, [RSU,UE], 40MHz, RTT, Absolute | 0.257 | 0.342 | 0.451 | 0.584 | Yes |
| 4003, Commercial, [RSU,UE], 100MHz, RTT, Absolute | 0.051 | 0.075 | 0.103 | 0.147 | Yes |

Table B.1.X.2.5-2 provides horizontal relative positioning accuracy results using sidelink positioning for commercial use cases.

Table B.1.X.2.5-2: Sidelink positioning - horizontal relative accuracy for commercial use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the target requirement |
| --- | --- | --- | --- | --- | --- |
| 4101, Commercial, [UE], 20MHz, RTT+AOA, Relative | 0.592 | 0.847 | 1.384 | 2.885 | 72% |
| 4102, Commercial, [UE], 40MHz, RTT+AOA, Relative | 0.386 | 0.618 | 0.953 | 1.821 | 81% |
| 4103, Commercial, [UE], 100MHz, RTT+AOA, Relative | 0.104 | 0.171 | 0.278 | 0.534 | Yes |
| 4104, Commercial, [UE,RSU], 20MHz, RTT+AOA, Relative | 0.505 | 0.745 | 1.064 | 1.756 | 77% |
| 4105, Commercial, [UE,RSU], 40MHz, RTT+AOA, Relative | 0.331 | 0.448 | 0.585 | 0.775 | Yes |
| 4106, Commercial, [UE,RSU], 100MHz,RTT+AOA, Relative | 0.067 | 0.091 | 0.122 | 0.174 | Yes |

Table B.1.X.2.5-3 provides ranging distance accuracy results using sidelink positioning for commercial use cases.

Table B.1.X.2.5-3: Sidelink positioning - ranging distance accuracy for commercial use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the target requirement |
| --- | --- | --- | --- | --- | --- |
| 4101, Commercial, [UE], 20MHz, RTT+AOA, Relative | 0.466 | 0.639 | 0.934 | 1.627 | 81% |
| 4102, Commercial, [UE], 40MHz, RTT+AOA, Relative | 0.248 | 0.444 | 0.640 | 1.046 | 88% |
| 4103, Commercial, [UE], 100MHz, RTT+AOA, Relative | 0.043 | 0.060 | 0.084 | 0.130 | Yes |
| 4104, Commercial, [UE,RSU], 20MHz, RTT+AOA, Relative | 0.406 | 0.606 | 0.878 | 1.448 | 83% |
| 4105, Commercial, [UE,RSU], 40MHz, RTT+AOA, Relative | 0.174 | 0.266 | 0.384 | 0.544 | Yes |
| 4106, Commercial, [UE,RSU], 100MHz, RTT+AOA, Relative | 0.033 | 0.050 | 0.069 | 0.097 | Yes |

Table B.1.X.2.5-4 provides ranging angle accuracy results using sidelink positioning for commercial use cases.

Table B.1.X.2.5-4: Sidelink positioning - ranging angle accuracy for commercial use cases from [X]

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| 4101, Commercial, [UE], 20MHz, RTT+AOA, Relative | 1.330 | 2.453 | 4.373 | 9.267 | Yes | 88% |
| 4102, Commercial, [UE], 40MHz, RTT+AOA, Relative | 1.056 | 1.898 | 3.353 | 6.771 | Yes | Yes |
| 4103, Commercial, [UE], 100MHz, RTT+AOA, Relative | 0.716 | 1.266 | 2.242 | 4.452 | Yes | Yes |
| 4104, Commercial, [UE,RSU], 20MHz, RTT+AOA, Relative | 1.316 | 2.414 | 4.223 | 8.281 | Yes | 89% |
| 4105, Commercial, [UE,RSU], 40MHz, RTT+AOA, Relative | 1.852 | 2.819 | 4.157 | 6.117 | Yes | Yes |
| 4106, Commercial, [UE,RSU], 100MHz, RTT+AOA, Relative | 0.426 | 0.653 | 1.004 | 1.485 | Yes | Yes |