3GPP TSG-RAN WG1 Meeting #110bis-e 	R1-2208612

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | x | rev | x | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on IUC related transmission based on latency bound |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022/9/30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A RRC parameter sl-LatencyBoundIUC-Report has been introduced to indicate the latency bound of SL Inter-UE coordination report from the associated SL Inter-UE coordination explicit request triggering. Similar to the CSI reporting, the one-to-one mapping relationship between the request signaling and IUC signaling should be clarified: - After sending a request signaling, UE-B shall not send a new request signaling within the latency bound of IUC signaling transmission corresponding to the prior request, until it receives the IUC signaling form UE-A. - After receiving a request signaling, UE-A shall not transmit an IUC signaling which is not corresponding to the request within the latency bound. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture the IUC report and IUC request procedure in a new Clause 8.X. - Clarification on IUC related transmission based on latency bound |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Although RRC parameters for IUC report and IUC request have been captured in TS 38.331, the related procedures are not completely captured in specification, i.e., neither in TS 38.321 nor in TS 38.214. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.X	UE procedures for reporting inter-UE coordination information

### 8.X.1 Inter-UE coordination information report

The sidelink inter-UE coordination information report can be triggered by an explicit request and/or a condition other than explicit request reception as indicated by the higher layer parameters sl-IUC-Explicit and sl-IUC-Condition.

The UE can be configured with one inter-UE coordination information reporting latency bound as indicated by the higher layer parameter sl-LatencyBoundIUC-Report. The sidelink inter-UE coordination information reporting procedure triggered by explicit request is described in [10, TS 38.321].

If the UE is triggered to report sidelink inter-UE coordination information by an explicit request, the UE is not allowed to report another sidelink inter-UE coordination information based on a condition other than the explicit request reception to the same UE within the latency bound as indicated by the higher layer parameter sl-LatencyBoundIUC-Report.

### 8.X.2 Inter-UE coordination information request

The sidelink inter-UE coordination request is transmitted when the condition as indicated by the higher layer parameter sl-TriggerConditionRequest is satisfied.

The UE that sent an explicit inter-UE coordination information request to another UE is not allowed to trigger another inter-UE coordination information report for the same UE before the last slot of the expected reception or completion of the ongoing inter-UE coordination information report.