3GPP TSG RAN WG1 #110bis-e	R1-2208631

e-Meeting, October 10th – 19th, 2022

Source: 	vivo

Title:	Discussion on UL precoding indication for multi-panel transmission

Agenda Item:	9.1.4.1

Document for:	Discussion and Decision

Introduction

The Rel-18 WID on MIMO evolution for downlink and uplink is approved [1], which includes the following objective:

Objective 7: Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/Industrial devices - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI-based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this contribution, we discuss the aspects related to simultaneous multi-panel transmission in mTRP operation.

STxMP in SDCI-based mTRP operation

In this section, we discuss transmission schemes related to STxMP PUCCH and PUSCH reliability in SDCI-based mTRP operation. In particular, we present our initial views on the following aspects:

Schemes for enabling STxMP PUSCH/PUCCH

Signaling design for STxMP in SDCI-based MTRP operation

Other relevant issues for the supported schemes of PUSCH

Transmission Schemes

Working assumption Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

In RAN1 #110, for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18 SDM scheme was agreed as working assumption. SDM scheme for PUSCH enables higher rank transmission in FR2, which improves UL throughput as well as higher reliability. Besides, UL transmission delay degrades by the benefit of two panels simultaneous transmission. SDM scheme shows the benefits of STxMP which we intend to specify. So, we propose to confirm the working assumption to support SDM as a basic scheme for STxMP.

Confirm the working assumption to specify SDM scheme for STxMP PUSCH transmission.

- The intention to introduce two CWs for SDM scheme derives from the fact that the channels of each panel vary significantly. So, independent MCS determination based on each channel quality seems more beneficial. However, with panel-specific power control which can compensate for the channel variation, the performance gain of supporting two CWs with separate MCS versus one CW with one MCS is unclear. Besides, since the maximum number of layers transmitted across all panels has been restricted to 4 in STxMP, the benefit of two-CW scheduling, mainly identified with transmission layers larger than 4, may hardly be obtained. What’s more, additional DCI overhead has to be introduced to indicate MCS/RV/NDI for the second CW. Since up to 11 bits have been introduced for facilitating SDCI-based mTRP operation in Rel-17, it is not convincing whether additional 8 bits added to the DCI are worthwhile for potentially marginal gain.

Do not support 2 CWs for SDM scheme.

SFN-based STxMP shall also be supported for the following reasons: firstly, when DFT-s-OFDM waveform is configured for PUSCH, SDM is not applicable since it supports only single-layer transmission. Under this circumstance, performing SFN-based transmission using the indicated TCI states is beneficial for higher reliability; secondly, for HST deployment, the SFN scheme was further enhanced in Rel-17, hence supporting SFN for STxMP seems natural; thirdly, for non-ideal backhaul mTRP deployment, soft bits cannot be combined at gNB side, the performance of SFN scheme will be better than SDM scheme in such case.

Support SFN scheme for STxMP.

Regarding PUCCH, SFN scheme is feasible. SFN with the same UCI bits multiplexed in the spatial domain can be considered for all PUCCH formats for simultaneous transmission with minor spec impact. We propose that SFN-based transmission is prioritized for all formats of PUCCH STxMP.

SFN-based transmission with STxMP should be supported for all PUCCH formats.

Signaling design

Design principles

Agreement Study the enhancement of SRS resource set configuration and SRI/TPMI indication for single-DCI based STxMP PUSCH scheme: - The configuration of two SRS resource sets, SRS resource set indicator field, two SRI fields and two TPMI fields of Rel-17 mTRP PUSCH TDM repetition is the starting point. - FFS: The configuration of one SRS resource set, one or two SRI fields and one or two TPMI fields - Note: This proposal does not mean that any possible SRI/TPMI enhancement on STxMP would be precluded. In RAN1#110, companies can suggest the detail SRI/TPMI enhancement with reasonable analysis and evaluation result. Agreement Study the layer combinations of {1+1, 1+2, 2+1, 2+2} for the SDM scheme (if supported) of single-DCI based STxMP PUSCH, - This is for 1 CW at least. - The layer combination for the SDM scheme can be further studied for 2 CW if 2 CW in SDM scheme is supported. - FFS: study the layer combinations of {1+3, 3+1} under the above conditions. - Companies are encouraged to provide SLS/LLS for their proposed layer combinations for the SDM scheme of single-DCI based STxMP PUSCH. Agreement (110) For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2}. - FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc. - FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4). - This applies to SDM with 1 CW at least.

Since the scenario of UL simultaneous transmission for multiple panels is restricted for mTRP operation, Rel-17 framework for SDCI-based PUSCH/PUCCH repetition can be reused as much as possible. With the indication of two SRI/TPMI/TPC/PTRS-DMRS association fields, TRP-specific transmission parameters can be indicated to the UE. Such design can be inherited to indicate transmission parameters for two panels towards two TRPs.

Rel-17 framework for SDCI-based PUSCH transmission should be reused as much as possible.

To guarantee scheduling flexibility, dynamic switching between sTRP and mTRP transmission is supported in Rel-17 by the indication of SRS resource set indication field, which can be directly reused to indicate dynamic switching between the transmission of two panels and a single panel. Since only multi-panel towards mTRP and single-panel towards sTRP are considered by selecting the SRS resource set used for PUSCH transmission, TRP selection is equivalent to panel selection.

Dynamic switching between the transmission of two panels and a single panel can be indicated by the SRS resource set indication field.

Precoding indication

Technically, two activated panels at UE may be capable of coherent transmission or non-coherent transmission depending on UE capability. For non-coherent transmission, two separate precoders based on Rel-15 codebook can be indicated for two panels using two TPMI fields similar to Rel-17 mTRP PUSCH repetition; and it should be noted that WID specifically mentions that no new codebook is introduced, hence, only non-coherent transmission across panels should be considered.

Only non-coherent STxMP transmission across panels is considered in this objective.

For SDM scheme, layer combinations of {1+1, 1+2, 2+1 and 2+2} are supported. We fail to see the necessity to support layers combination of {1+3} and {3+1}. Performance gain of layer combinations s of {1+3} and {3+1} compared with rank combination of {2+2} is unclear. What’s more, specification efforts are required that new DMRS ports entry should be introduced to support such combination. So, we do not support layers combination of {1+3} and {3+1}.

Do not support layers combination of {1+3} and {3+1}.

Two SRI/TPMI fields need to be further refined by the restricted rank combination with maximum of 4 layers across two panels. The rank combinations indicated by two SRI/TPMI fields should be one of {1+1, 1+2, 2+1, 2+2} because the number of DMRS ports derived from two CDM groups is {1+1, 1+2, 2+1, 2+2} based on current DMRS port indication table. Based on the above consideration, the field size of the first SRI/TPMI field is determined by max rank configuration considering the case of dynamically switching to single-panel/TRP transmission, e.g., max rank 4, while the second SRI/TPMI field can be further refined assuming the bit width corresponding to max rank 2.

For SFN scheme, two precoders with the same transmission rank should be indicated, which is the same as Rel-17 SDCI-based mTRP transmission. So, SRI/TPMI fields designed in Rel-17 can be reused directly.

Precoding indication designed in Rel-17 can be reused with necessary refinement for SDCI-based STxMP transmission.

DMRS port indication

Agreement To enhance the DMRS port indication for SDM scheme of STxMP PUSCH transmission in single-DCI based mTRP system, study the following aspects: - Whether the indicated DMRS ports can be in different or same CDM group? - How to determine the port partition among PUSCH layers. - How to map DMRS ports with PUSCH layers from different panels. - Whether to use one DCI field or two DCI fields for DMRS port indication - How to indicate layer combination that is used to partition DMRS port partition among PUSCH layers. - Other aspects are not precluded.

For SDM scheme, to mitigate interference between two panels and guarantee demodulation performance of PUSCH towards two TRPs, DMRS ports derived from two CDM groups are required for two panels to assure orthogonality in the SDM scheme. How to indicate two CDM groups of DMRS ports should be further discussed. Since the maximum transmission layer is limited to 4, the current one antenna port indication field which can indicates up to four DMRS ports groups is sufficient to indicate DMRS ports for SDM-based PUSCH. Current DMRS port interpretation table can be also reused. The target rank used to validate the DMRS port indication table should be determined by the sum of rank indicated by two TPMI or SRI fields. Besides, the association of mapping between DMRS ports and PUSCH layers in current spec can be reused. The association between PUSCH layers and panels or TCI states or SRS resource sets can be acquired by two TPMI or SRI fields which is associated to SRS resource sets.

Sum of rank indicated by two TPMI or SRI fields determines the DMRS ports indication table for current antenna port indication field for SDM scheme.

Transmission scheme indication

The transmission schemes discussed above target different scenarios or different requirements of data transmission that do not change often. Furthermore, DCI overhead which is not expected to change dynamically shall be provisioned to support different transmission schemes of PUSCH. So, it seems reasonable that all the possibly supported schemes should be configured by RRC.

Transmission schemes for STxMP is semi-statically configured by RRC.

With minimum spec change, the transmission scheme for STxMP dynamically switching to TDM-based transmission can be achieved by the indicated repetition number with the same SDCI-based mTRP framework. When UE is indicated with two TCI states that associate with two panels, SDM/SFN transmission scheme applies as configured by RRC if the repetition number equals one, otherwise TDM will be performed. The benefits of dynamically switching between STxMP scheme and TDM lie in that reliability can be guaranteed by switching to TDM scheme when there is heavy interference between two panels.

The transmission scheme for STxMP can dynamically switch to TDM depending on the indicated repetition number when two unified TCI states are applied.

Other issues

Issue 1: Whether to enhance PTRS transmission

In the current specification, up to 2 PTRS ports can be configured for PUSCH transmission based on non-coherent/partial-coherent codebook or non-codebook transmission to track phase noise from different groups of antennas or SRS ports. With two-panel simultaneous transmission, whether PTRS ports should be further increased to guarantee that maximum of two PTRS ports per panel should be further discussed. Increasing PTRS ports will undoubtedly lead to much work as well as overhead. Whether maximum of 2 PTRS ports is enough for two panels can be studied by properly designing the association between PTRS ports and DMRS ports from two panels as a start point.

Consider 2 PTRS ports for two panels as starting point.

In SDM scheme, antenna ports within one panel share one PTRS port. The PTRS-DMRS association should be further clarified. One of the DMRS ports from each CDM group is selected to be associated with PTRS port 0 and PTRS port 1 separately.

In SFN scheme, DMRS ports are transmitted in SFN mode. The strongest layer corresponding to different panels may be different. Coherence characteristics of the two scheduled TPMI may be also different, hence PTRS transmission will be separate. Whether to restrict PTRS transmission in SFN manner shall be further considered.

Further study PTRS-DMRS association and PTRS transmission for all possibly supported schemes.

Issue 2: CSI report on STxMP PUSCH

In Rel-17, to improve the reliability of the CSI report, SP-CSI and A-CSI reports on PUSCH are designed to be multiplexed on the first repetition towards each TRP separately. For STxMP towards mTRP in Rel-18, how to support multiplexed SP-CSI and AP-CSI reports on PUSCH with STxMP should be considered to improve the reliability of the CSI report.

Regarding SDM, the same time and frequency resource is used for 2 panels. Rate matching of CSI report performs per layer, so there is no other spec impact of supporting CSI report multiplexing on PUSCH with SDM scheme.

Similar to SDM, there is also no spec change expected for supporting CSI report multiplexing on PUSCH with SFN scheme.

Support CSI report on PUSCH using SDM/SFN scheme with minimum spec impact.

STxMP in MDCI-based MTRP operation

In Rel-16 MDCI-based mTRP transmission which targets improving throughput, both ideal backhaul and non-ideal backhaul between two TRPs are considered. To guarantee demodulation of overlapped PDSCH scheduled by two DCI from two TRPs, following restrictions for fully/partially overlapped PDSCHs are introduced at least for non-ideal backhaul scenarios.

Same DMRS configuration: the UE is not expected to be configured with a different DMRS configuration such as actual number of front-loaded DMRS symbol(s), the actual number of additional DMRS, and the actual DMRS symbol location and DMRS configuration type

No DMRS and PDSCH collision: the UE does not expect a PDSCH scheduling intended for that UE in a given slot if that PDSCH REs collide with DMRS REs associated with another PDSCH for the same UE

One TCI state per CDM group: the UE is not expected to have more than one TCI index with DMRS ports within the same CDM group

Same active BWP and SCS: The UE is expected to be scheduled with the same active BWP bandwidth and the same SCS

Full scheduling information per PDCCH: Full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH.

This section discusses overlapped PUSCHs and overlapped PUCCHs scheduled by two DCI from two TRPs transmissions based on the restriction of MDCI-based PDSCH transmission.

Overlapped PUSCHs

Agreement For multi-DCI based STxMP PUSCH+PUSCH transmission, study and evaluate the following aspects: - Two PUSCHs are associated with different TRPs and transmitted from different UE panels. The total number of layers of these two PUSCHs is up to 4. - Study STxMP of PUSCH+PUSCH transmission where it is some combination of DG-PUSCH, CG-PUSCH and msg3/msgA PUSCH. - The overlapping type(s) of fully/partially in time domain and fully/partially/non-overlapping in frequency domain are to be studied and justified for PUSCH+PUSCH. Note: The above study shall take into account the UE implementation and RF considerations. Note: Study the conditions required for STxMP PUSCH+PUSCH. Note: Other aspects are not precluded.

For MDCI-based PUSCH simultaneous transmission, the following aspects should be considered:

Relaxation on the scheduling constraints in the current spec

Conditions required for simultaneous transmission

Relaxation on uplink scheduling constraints

Subject to UE capability, UE does not expect to be scheduled with any two PUSCHs overlapping in time on a serving cell in Rel-15, and only one PUSCH with higher priority is transmitted when UE is scheduled with overlapped PUSCHs in Rel-16. With STxMP transmission, time-domain overlapped PUSCHs scheduled by two DCIs from two TRPs should be allowed to further improve the throughput of UL data transmission. As shown in Figure 1, for STxMP transmission, three scheduled PUSCH frequency-domain patterns can be considered with non-overlapped, partially-overlapped, and fully overlapped in frequency domain for two PUSCHs overlapping in the time domain.

Time-domain overlapped PUSCHs scheduled by two DCIs from two TRPs should be allowed.

Examples of frequency-domain patterns for overlapped PUSCHs scheduled by two DCIs from two TRPs

In the current spec, when multiple CGs are configured for a UE, only one CG can be selected for UL transmission. For a UE capable of STxMP transmission, two CGs associated with two panels can be selected for simultaneous transmission.  Besides, DG-PUSCH can be scheduled overlapping with a CG-PUSCH as shown in Figure 2, and the STxMP UE transmits both PUSCHs.

Such relaxation on the UL scheduling constraints will surely provide network with more scheduling flexibility and consequently higher performance for overall UL transmission.

Example of CG+DG PUSCH

Two overlapping CGs associated with two TRPs and one CG PUSCH overlapping with DG-PUSCH associated with two TRPs can be considered for STxMP transmission.

Conditions required for simultaneous transmission

To guarantee performance gain, for STxMP UEs, the following conditions should be further discussed:

Overlapping DG-PUSCHs/CG-PUSCHs/DG-PUSCH+CG-PUSCHs should be associated with different TRPs and different panels for simultaneous transmission are allowed only in such situations.

No DMRS and data collision: the UE does not expect a PUSCH scheduling intended for that UE in a given slot if that PUSCH REs collide with DMRS REs associated with another PUSCH for the same UE

One TCI state per CDM group: the UE is not expected to have more than one TCI index with DMRS ports within the same CDM group

The transmission layer across overlapped PUSCH should be limited to 4.

Whether the overlapped channels should be at the same or different priority levels.

Further study the conditions required for simultaneous transmission for the overlapped DG-PUSCHs/CG-PUSCHs/DG-PUSCH+CG-PUSCH.

Overlapped PUCCHs

For PUCCH transmission towards two TRPs, the same or different formats may be considered for simultaneous transmission. For flexible scheduling, it is preferred that all PUCCH formats should be considered for STxMP transmission with unified design. With the fact of distinct DMRS patterns for different PUCCH formats, considering the requirement of non-overlapped DMRS between two simultaneously transmitted overlapping PUCCHs for acceptable demodulation performance simultaneous, frequency-domain non-overlapped PUCCHs associated with two TRPs can be considered for STxMP transmission with first priority.

Frequency-domain non-overlapped PUCCHs associated with two TRPs can be considered for STxMP transmission with first priority, which simplifies design for simultaneous transmission of different formats considering requirement for non-overlapped DMRS.

Dynamic indication of simultaneous transmission

In practice, interference between two panels is time-varying due to UE movement, rotation and varying environment so that the dynamically indicated beams for two panels may not always be suitable for simultaneous transmission. Besides, different transmission power of two panels by independent power control will cause PUSCH transmitted from the panel with lower power to be overwhelmed by PUSCH transmitted from the panel with a higher power if both PUSCHs are received by the TRP. Unfortunately, current beam reporting cannot indicate the uplink interference between two panels measured by each TRP. For example, as shown in Figure 3 panel 1 transmits SRS1 and panel2 transmits SRS2, if the received power of SRS1 is comparable to that of SRS2, implying the interference from panel 1 is significant for the reception of UL transmission from panel 2, simultaneous transmission from two panels may not be beneficial in this circumstance.

An example of interference measurement

We show the SIR measured at TRPs between two back-to-back panels (Ω0,1=Ω0,0+180°) and orthogonal panels (Ω0,1=Ω0,0+90°) at the UE associating with different TRPs assuming same transmit power of the two panels when the UE rotates at the 50 rounds/minute, as shown in Figure 4 a) and b) respectively. The SIR represents the ratio between a desired signal transmitted from one panel with its optimal beam direction to the TRP of interest and the interference from the other panel with its optimal beam direction to its target TRP. The final SINR must be less than the SIR taking into account of other interference. The CDF of SIR shows that the inter-panel interference is an important factor to limit the SINR, sometimes severely impacts the SINR. The amount of inter-panel interference dynamically changes depending the relative positions between the UE and two TRPs, angles between two panels and two TRPs, which is unpredictable due to UE’s movement and rotation.

a) Two back-to-back panels (Ω0,1=Ω0,0+180°)                  b) Two orthogonal panels (Ω0,1=Ω0,0+90°)

CDF of SIR between two panels measured at TRP

We have following observations:

Inter-panel interference is one of the factors affecting the SINR for simultaneous UL channel reception transmitted from two panels.

SIR less than 0dB, i.e., the strength of interference from the interfering panel larger than the strength of signal of interest, is with probability of about 7%; SIR less than 5dB is with probability of 20% to 35% approximately; SIR less than 10dB is with probability of 50% to 70% approximately, etc.

The maximum SIR is below 25dB, meaning that the achievable SINR cannot exceed 25dB.

Inter-panel interference is severe between orthogonal panels than that of back-to-back panels.

The amount of inter-panel interference dynamically changes depending the relative positions between the UE and two TRPs, angles between two panels and two TRPs, which is unpredictable due to UE’s movement and rotation.

The unpredicted interference is easy to be managed in S-DCI based MTRP by dynamically indicting simultaneous two panel transmission or single panel transmission. For M-DCI-based MTRP, two PUSCHs can be scheduled non-overlapping by two TRPs for ideal-backhaul. However, for M-DCI-based MTRP with non-ideal backhaul, two TRPs do not have knowledge of whether the scheduled PUSCHs are overlapped unless it is pre-coordinated, which is not flexible. Therefore, dynamic indication of whether simultaneous transmission for the overlapped PUSCHs or PUCCHs should be supported. For a UE supporting STxMP transmission, when it receives an indictor disallowing simultaneous transmission and scheduled with overlapping PUSCHs or PUCCHs, UE will transmit one PUSCH or PUCCH depending on a predefined rule or up to its own decision.

Support to dynamically switch between single panel transmission and STxMP transmission for PUSCHs or PUCCHs for M-DCI based MTRP.

UCI multiplexing

In Rel-15 and Rel-16, the multiplexing rule based on the priority of UCI or physical channel is defined. In Rel-17, this rule is reused for UL SDCI-based and MDCI-based mTRP without any enhancement.  However, for STxMP transmission, two overlapped PUSCHs or two overlapped PUCCHs can be simultaneously transmitted towards two TRPs. How to multiplex the UCI on the overlapped PUSCHs or PUCCHs as shown in Figure 5 should be further discussed. One simple solution is to reuse legacy rules for each TRP. Since simultaneous transmission of a PUCCH and a PUSCH is not part of the WID, how to handle the overlapped PUCCH and PUSCH for TRP-specific UCI multiplexing should be further studied.

PUSCH scheduling cases

TRP/panel-specific UCI multiplexing can be considered.

Further study on how to handle the overlapped PUCCH and PUSCH for TRP-specific UCI multiplexing.

Beam indication for STxMP transmission

SRS resource set configuration

In Rel-17, two SRS resource sets are configured for two TRPs with usage set to codebook or non-codebook for mTRP PUSCH transmission. For SDCI-based STxMP PUSCH transmission, similarly, two SRS resource sets can be configured to achieve panel-specific channel measurement and precoder selection.

In Rel-16, MDCI-based mTRP PUSCH transmission, one SRS resource set is shared for PUSCH transmission scheduled by two TRPs, which can work but with low efficiency even if the SRS towards different TRPs are TDM transmitted. Spatial relation info or PL-RS for the SRS resources configured in the SRS resource set has to be updated by MAC CE or RRC re-configuration to adjust the transmission towards each TRP. It will get worse for MDCI-based STxMP transmission where overlapped PUSCH scheduled by two DCI are simultaneously transmitted, requiring exclusive SRS resource(s) associated with different panels respectively for codebook-based/non-codebook-based transmission. More specifically, each TRP should maintain UL channels state information persistently for STxMP scheduling at any time. Hence, two SRS resource sets should be configured, otherwise UE cannot trace the SRS resource(s) for each PUSCH.

Support to configure two SRS resource sets for STxMP transmission in both SDCI-based and MDCI-based mTRP operation.

Beam indication

Extension of Rel-17 unified TCI framework for indication of multiple UL TCI states focusing on mTRP use case will be discussed in Rel-18[1]. From our perspective, two unified UL TCI states indicated for SDCI-based UL mTRP transmission are applicable for STxMP given the UE supports the capability. Detailed consideration and design on beam report supporting STxMP and beam indication can be found in our companion contribution [2].

TCI states indication designed for SDCI-based mTRP is also applicable for 2 panels simultaneous transmission towards 2 TRPs.

Beam indication designed with the objective of unified TCI extension for mTRP transmission also can be directly used for STxMP in MDCI-based mTRP if the indicated TCI states are associated with two panels. To indicate the unified TCI state for each TRP/panel separately, CoresetPoolIndex can be used to achieve the association between the indicated TCI state and the scheduled channels. More details can be referred to [2].

CoresetPoolIndex can be used to achieve the association between the indicated TCI state and the scheduled channels.

If two SRS resource sets configured for codebook-based or non-codebook-based transmission are following unified TCI state, two unified TCI states indicated for SDCI-based or MDCI-based mTRP PUSCH transmission can be applied to two SRS resource sets respectively.

Two unified TCI states indicated for mTRP transmission are applied for two SRS resource sets respectively.

Power control for STxMP

In Rel-17 UL SDCI-based mTRP, to achieve TRP-specific open-loop power control, two sets of power control parameters including P0/alpha/PL-RS/closed-loop-index are indicated by two SRI fields for mTRP PUSCH repetition. And, two spatial relation Infos or power control parameter sets containing P0/PL-RS/closed-loop-index are activated for one PUCCH resource. TRP-specific closed-loop power control is also supported for both PUSCH and PUCCH by two TPC fields for two closed-loop indices. To report the power headroom for PUSCH transmission towards each TRP to assist TRP-specific power control, TRP-specific PHR reporting is supported. In the Rel-17 unified TCI framework, power control parameter setting including P0/alpha/closed-loop-index for UL channels or signals are associated with the common TCI state. Common PL-RS for UL transmission is also associated with or derived from the common TCI state.

Based on the TRP-specific power control design and the power control parameter determination in the unified TCI framework, panel-specific power control can be facilitated. From our perspective, the following aspects should be considered for both SDCI-based and MDCI-based STxMP:

Panel-specific open-loop and closed-loop power control

Power limit for multi-panel PUSCH transmission

Panel-specific PHR reporting

Panel/TRP-specific open-loop and closed-loop power control

Since we focus on multi-panel transmission towards mTRP, two sets of power control parameter settings and two PL-RSs can be acquired by two indicated TCI states as we illustrate in [2], and separate open-loop power control can be achieved. Regarding closed loop-power control, two TPC fields are designed for two closed-loop indices respectively in Rel-17 UL mTRP, which can be inherited to STxMP.

Panel-specific open-loop power control can be achieved by two PC settings associated with two indicated TCI states, while panel-specific closed-loop power control can inherit the design in Rel-17.

Panel-specific power limit

Power calculation should be further discussed to make it compatible with STxMP in SDCI-based mTRP operation. Currently power is calculated based on one transmission occasion corresponding to only one panel utilizing the following formula. For transmission occasion i, specific P0/alpha, Pathloss measured by PL-RS, and closed-loop power adjustment are involved to calculate transmission power. According to UE power class, the transmission power is limited by PCMAX, f,c(i)  which is the configured UE maximum output power for carrier f of a serving cell c.

[dBm]

- Regarding STxMP on one transmission occasion, panel-specific P0/alpha, Pathloss, and closed-loop power adjustment can be acquired. Then panel-specific transmission power can be calculated by using the above formula respectively to achieve panel-specific power control. To enable this, per-panel power limits should be defined. Considering panel-specific power control should be applicable for both STxMP and single panel transmission because dynamic switch between single panel transmission and STxMP is envisioned to be supported, current power limitation should be reused as panel-specific power limits.

Panel-specific power limits should be introduced for panel-specific power control.

Current power limitation PCMAX, f,c(i) is restricted by max TRP and max EIRP. There is regulatory requirements  for EIRP, which is per-UE based for a band, and TRP is preferred to be kept as per-UE to avoid work on related basic requirements in RAN4. Therefore, power limitation PCMAX, f,c(i)  is defined as per-UE limitation and shall be also mandatory fulfilled for STxMP. Based on this situation, power sharing rule should be further discussed in RAN1 together with panel-specific power control to meet the per-UE power limitation.

Further study power sharing rule to meet the per-UE power limitation.

PHR report for SDCI-based and MDCI-based STxMP

Panel-specific PHR should be reported to assist gNB in controlling transmission power for each panel in SDCI-based mTRP operation. To achieve this, the design in Rel-17 TRP-specific PHR reporting can be considered as a baseline. In Rel-17 SDCI-based mTRP, when PHR MAC-CE is reported in slot n, for a CC that is configured with mTRP PUSCH repetition, the second PHR value is determined as follows:

If the first PHR value is actual PHR (based on Rel. 15/16) corresponding to a repetition among mTRP PUSCH repetitions associated with a given TRP, the second PHR value

is actual only when a repetition associated with the other TRP is transmitted in slot n. Otherwise, it is virtual.

If the first PHR value is actual PHR (based on Rel. 15/16) but not corresponding to a repetition among mTRP PUSCH repetitions (corresponds to sTRP PUSCH), a second PHR value is reported as virtual PHR.

If the first PHR value is virtual, a second PHR value is reported as virtual PHR.

For panel-specific PHR reporting for STxMP, where transmission from panels occur in the same slot, two PHRs can be determined as follows:

If the first PHR value is the actual PHR corresponding to a panel for STxMP, the second PHR is actual PHR.

If the first PHR value is actual PHR corresponding to a panel for single panel transmission, a second PHR value is reported as virtual PHR.

If the first PHR value is virtual, a second PHR value is reported as virtual PHR.

TRP-specific PHR reporting designed in Rel-17 SDCI-based mTRP should be reused for panel-specific PHR reporting as much as possible.

For MDCI-based mTRP/STxMP, as shown in Figure 6, when PUSCHs scheduled by two DCIs corresponding to two different CoresetPoolIndex are non-overlapping with each other, the first PUSCH will be the transmission occasion for calculating and reporting PHR based on the description in current specification. However, for the overlapped PUSCHs simultaneously transmit from two panels, how UE reports and calculates PHR is unclear. Panel-specific PHR reporting for both non-overlapped and overlapped PUSCHs simplifies power control for each panel. For instance, gNB can make a better decision on whether to schedule two PUSCHs for STxMP based on the reported two PHRs. Therefore, we propose to further study panel-specific PHR reporting for the PUSCHs scheduled by two DCIs.

PHR reporting in mTRP operation

Further study panel-specific PHR reporting for the PUSCHs scheduled by two DCIs.

Conclusions

In this contribution, we have the following proposals:

Inter-panel interference is one of the factors affecting the SINR for simultaneous UL channel reception transmitted from two panels.

SIR less than 0dB, i.e., the strength of interference from the interfering panel larger than the strength of signal of interest, is with probability of about 7%; SIR less than 5dB is with probability of 20% to 35% approximately; SIR less than 10dB is with probability of 50% to 70% approximately, etc.

The maximum SIR is below 25dB, meaning that the achievable SINR cannot exceed 25dB.

Inter-panel interference is severe between orthogonal panels than that of back-to-back panels.

The amount of inter-panel interference dynamically changes depending the relative positions between the UE and two TRPs, angles between two panels and two TRPs, which is unpredictable due to UE’s movement and rotation.

- Confirm the working assumption to specify SDM scheme for STxMP PUSCH transmission.

Do not support 2 CWs for SDM scheme.

Support SFN scheme for STxMP.

SFN-based transmission with STxMP should be supported for all PUCCH formats.

Rel-17 framework for SDCI-based PUSCH transmission should be reused as much as possible.

Dynamic switching between the transmission of two panels and a single panel can be indicated by the SRS resource set indication field.

Only non-coherent STxMP transmission across panels is considered in this objective.

Do not support layers combination of {1+3} and {3+1}.

Precoding indication designed in Rel-17 can be reused with necessary refinement for SDCI-based STxMP transmission.

Sum of rank indicated by two TPMI or SRI fields determines the DMRS ports indication table for current antenna port indication field for SDM scheme.

Transmission schemes for STxMP is semi-statically configured by RRC.

The transmission scheme for STxMP can dynamically switch to TDM depending on the indicated repetition number when two unified TCI states are applied.

Consider 2 PTRS ports for two panels as starting point.

Further study PTRS-DMRS association and PTRS transmission for all possibly supported schemes.

Support CSI report on PUSCH using SDM/SFN scheme with minimum spec impact.

Time-domain overlapped PUSCHs scheduled by two DCIs from two TRPs should be allowed.

Two overlapping CGs associated with two TRPs and one CG PUSCH overlapping with DG-PUSCH associated with two TRPs can be considered for STxMP transmission.

Further study the conditions required for simultaneous transmission for the overlapped DG-PUSCHs/CG-PUSCHs/DG-PUSCH+CG-PUSCH.

Frequency-domain non-overlapped PUCCHs associated with two TRPs can be considered for STxMP transmission with first priority, which simplifies design for simultaneous transmission of different formats considering requirement for non-overlapped DMRS.

Support to dynamically switch between single panel transmission and STxMP transmission for PUSCHs or PUCCHs for M-DCI based MTRP.

TRP/panel-specific UCI multiplexing can be considered.

Further study on how to handle the overlapped PUCCH and PUSCH for TRP-specific UCI multiplexing.

Support to configure two SRS resource sets for STxMP in both SDCI-based and MDCI-based mTRP operation.

TCI states indication designed for SDCI-based mTRP is also applicable for 2 panels simultaneous transmission towards 2 TRPs.

CoresetPoolIndex can be used to achieve the association between the indicated TCI state and the scheduled channels.

Two unified TCI states indicated for mTRP transmission are applied for two SRS resource sets respectively.

Panel-specific open-loop power control can be achieved by two PC settings associated with two indicated TCI states, while panel-specific closed-loop power control can inherit the design in Rel-17.

Panel-specific power limits should be introduced for panel-specific power control.

Further study power sharing rule to meet the per-UE power limitation.

TRP-specific PHR reporting designed in Rel-17 SDCI-based mTRP should be reused for panel-specific PHR reporting as much as possible.

Further study panel-specific PHR reporting for the PUSCHs scheduled by two DCIs.

References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung

R1-2208626, Discussion on unified TCI framework extension for multi-TRP