3GPP TSG RAN WG1 #110bis-e	R1-2209011

e-Meeting, October 10th – October 19th, 2022

Agenda Item:	9.2.2.1

Source: 	Fujitsu

Title: 	    Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion/Decision

Introduction

AI/ML-based CSI feedback enhancement is one of the use cases in this study item. In the previous RAN WG1 110 meeting, a number of topics has been discussed, including baseline system-level simulation (SLS) EVM table, AI/ML model generalization, KPIs for evaluations, and various aspects for the (potential) sub-use cases of CSI compression and CSI prediction. In this contribution, we present our views on KPI selection, AI/ML model generalization, separate training for CSI compression. Relevant evaluation results are also presented.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

The CSI compression using two-sided AI/ML model is described in Fig. 1.

Fig. 1: The two-sided AI/ML model.

KPI Selection

- 2.1 Squared Generalized Cosine Similarity

It is agreed in the RAN WG1 110 meeting that the squared generalized cosine similarity (SGCS) is down selected between GCS and SGCS.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted.

The baseline vectors in the SGCS formula should be carefully chosen. One option is to use the vectors from the ideal noiseless channel matrix. The other is to use the noisy ones. Our view is that the ideal vectors should be used. The reason is that noise reduction is anticipated as one of the benefits of the AI/ML-based approach. As a result, we have the following proposal.

Proposal-1: The vectors from the ideal channel matrix are suggested to be used in the formula of the squared generalized cosine similarity (SGCS).

We preform SLS to evaluate the performance of the AI/ML-based CSI compression. The baseline is the legacy codebook approach using Rel-16 e-type II codebook. The simulation parameters are summarized in Table 6.

The gain of using the AI/ML approach over the legacy Rel-16 type II codebook method is presented in Fig. 2. It is observed that an up to 28.55% SGCS increment is achieved by the AI/ML-based approach.

Fig. 2. The gain of using the AI/ML approach over the legacy Rel-16 e-type II codebook method.

- 2.2 Intermediate KPI for Multi-Rank Scenario

In RAN WG1 109-e meeting, the following agreement was made regarding the GCS/SGCS in the multi-rank scenario [2]. In the RAN WG1 110 meeting, some other intermediate KPIs were also proposed, e.g., realized relative SNR, Chordal distance, numerical spectral efficiency gap, precoder error, normalized expected directional gain, and GCS in the log scale. A summary of these formulae can be found in [1].

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

- Method 1: Average over all layers

Note:  is the eigenvector of the target CSI at resource unit i and K is the rank. is the  output vector of the output CSI of resource unit i.  is the total number of resource units.  denotes the average operation over multiple samples.

- Method 2: Weighted average over all layers

Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

- Other methods are not precluded

- FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

In the scenario when the rank of a channel matrix is larger than 1, our view is that the normalized weighted SGCS

SGCS=E1Ni=1Nj=1Kλij2j=1Kλij2wijHwij2 wij2wij2,

should be chosen as the intermediate KPI. The SGCS should be weighted by the eigenvalues because the eigenvectors associated to larger eigenvalues contribute more to the channel matrix than those associated with the smaller ones. In addition, as a rigorous mathematical definition, the SGCS must strictly lie in the interval [0,1]. So the normalization needs to be performed for the reason that the channel may not be properly normalized because of the estimation error in realistic channel estimation. As a result, we have the following proposal.

Proposal-2: The KPI for the intermediate evaluation on AI/ML model performance for CSI feedback enhancement when the rank of the channel matrix is larger than 1 is the normalized weighted SGCS

SGCS=E1Ni=1Nj=1Kλij2j=1Kλij2wijHwij2 wij2wij2,

where λij is the j-th eigenvalue of the squared matrix GGH at resource unit i. The vector wij is the eigenvector associated with the eigenvalue λij, i=1,2,…,N, and j=1,2,…,K. The matrix G is the channel matrix, and the superscript H is the Hermitian operator. The vector wi is the reconstruction of the compressed wi.

We analyze our proposal with the other two options in the agreement [2]. It is shown in our previous contribution [3] the following observations obtained from link-level simulation results that

- The GCS decreases as the rank of the channel matrix increases.

- Instead of averaging over ranks evenly in the GCS formula, the stronger ranks whose GCS are larger are more heavily weighted than the weaker ranks in the normalized weighted GCS.

Compared to Method 1, in which the GCS/SGCS is averaged evenly over ranks, the GCS values obtained from our proposal for the multi-rank scenario looks consistent with those for the case when the rank of the channel matrix is one based on the observations. However, the GCS values show significantly low if the GCS is computed by Method 1. Considering the fact that the layer with a larger singular value is more important than that with a smaller one, the SGCS formula in Proposal 2 looks more reasonable in this sense.

In Method 3, the GCSs/SGCSs are computed separately for each layer. This enables a straightforward look at the performance of the AI/ML model for each layer. However, as a KPI, a single number is preferred for the purpose of performance comparison. In this sense, having multiple GCSs/SGCSs may be cumbersome.

AI/ML Model Generalization

The issue of AI/ML model generalization was discussed in the previous meeting [1]. The methodology of the model generalization, as well as the scenarios and configurations, were discussed. In this section, we present our views on the configuration-wise generalization. Relevant simulation results are also presented.

Agreement

For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point:

- Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband)

- Various sizes of CSI feedback payloads, FFS candidate payload number

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc.

- Companies to report the selected configurations for generalization verification

- Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

One approach for AI/ML model configuration-wise generalization/scalability can be achieved by pre-processing and/or post-processing. Several issues should be addressed for the configuration-wise generalization/scalability. In this paper, we take the generalization of payload as an example to present our views.

To evaluate the performance of configuration-wise generalization/scalability, the baseline to be compared with should be studied and agreed. An option of the baseline model for the configuration-wise generalization/scalability is a two-sided AI/ML model described in Fig. 1.

Proposal-3: The baseline for the evaluation of the generalization/scalability of an AI/ML model over various configurations should be further studied. The baseline is suggested to be the performance of the two-sided AI/ML model composed by an encoder and a decoder only, without pre-processing/post-processing. Other potential baseline schemes can be further studied.

To achieve the generalization of payload, various options of the CSI feedback bit numbers should be supported. Three potential schemes are described in Figs. 3—5. In Figs. 3 and 4, the generalization is achieved by truncating part of the CSI or by a fully connected layer (FCL), respectively. Specifically in Fig. 4, the size of the output of the AI/ML encoder is “shortened” by the FCL in some non-trivial way.

The training processes of the generalization of payloads described in Figs. 3—4 are similar. In the framework shown in Fig. 3, the encoder and three decoders are trained together. Specifically, the unique encoder and three decoders form three pairs of two-sided AI/ML models. The input of the three decoders are the truncations of the output of the encoder. The loss function of the entire training is the summation of the loss functions of the three pairs of two-sided AI/ML models, which is used for backpropagation. In the case of Fig. 4, the truncators are replaced by FCLs, and each of the FCL is trained together with its corresponding pair of two-sided AI/ML models. At the inference stage, a proper decoder is used based on the desired payload.

Observation-1: Generalization of payload can be achieved by a framework of a single encoder and multiple decoders with proper pre-processing/post-processing.

Fig. 3. Generalization of payload by truncators.

Fig. 4. Generalization of payload by FCLs.

We note that there are totally three decoders in each of the frameworks shown in Figs. 3 and 4. It turns out that a universal decoder is possible if three additional FCLs are added between the FCLs and the decoder, as shown in Fig. 5. The added FCLs perform the function similar to some non-trivial expansion. In this case, there is a single encoder, and a single decoder, which forms the backbone. Similar to the training process of the framework shown in Fig. 4. The entire training process is jointly operated for the three components, where each of them is formed by the encoder, the decoder, and a pair of FCLs. The loss function of the entire training is the summation of the loss functions of the three components.

Observation-2: Various payload configurations can be supported in a unified two-sided AI/ML model with proper pre-processing/post-processing for CSI compression.

Fig. 5. Generalization for payload by FCL, single-encoder-single-decoder case.

We perform link-level simulation to evaluate the performance of the proposed three generalization schemes and baseline. In particular, CDL-C channels are studied. We denote CDL-C-x-y as a CDL-C channel with delay spread x and Doppler shift y in this contribution. The simulation parameters are summarized in Table 7 in Appendix A. The performance is shown in Fig. 6 in terms of the SGCS. According to Fig. 6, it is observed that the performances of the three models are almost the same, which is also very similar to the baseline performance. We have the following observations based on the evaluation results.

Fig. 6. The performance for the generalization of payload.

Observation-3: The performances of the generalization of payload are very similar in terms of the SGCS, no matter whether the structure of the AI/ML model is composed by a single encoder and a single decoder with proper pre-processing/post-processing, or a single encoder and multiple decoders with proper pre-processing/post-processing.

Observation-4: The performance of the generalization of payload, in terms of the SGCS, are very similar to the baseline scheme, where the two-sided AI/ML model is composed by an encoder and a decoder only, without pre-processing/post-processing.

Noticing that a similar performance is achieved by various structures, we note that the issue of how an AI/ML model is defined should be studied in the configuration-wise generalization. Specifically, it should be clarified that whether the pre-processing and/or post-processing are counted as a part of the AI/ML model. In addition, whether the structure of one encoder vs. multiple decoders or vice versa can be counted as one model or not. As a result, we have the following proposal.

Proposal-4: For the generalization/scalability of an AI/ML model over various configurations, it should be clarified how an AI/ML model is defined.

Alt. 1. Pre-/post-processing are precluded.

Alt. 2. Pre-/post-processing are included, i.e., a unique AI/ML model in the sense that the backbone model is unique.

Alt. 3. One encoder vs. multiple decoders, or vice versa.

Proposal-5: The pre-/post-processing are suggested to be counted as part of an AI/ML model for the generalization/scalability of an AI/ML model over various configurations.

Since the performances of the configuration-wise generalization/scalability are similar in terms of the SGCS, other KPIs may be needed for the evaluation purpose. The complexity related KPIs may be relevant.

Proposal-6: The KPIs for the generalization/scalability of an AI/ML model should be further studied. In particular, both the performance-related KPI and the complexity-related KPI are suggested to be used.

Proposal-7: It is suggested that the comparison of the model complexity of baseline and generalization schemes can be taken as a KPI. As an example, the ratio of the model complexity of the baseline approach over that of the generalization scheme.

Training Collaboration: Separate Training

Joint training and separate training are two types of training collaboration of interest. Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation, while in separate training, the UE-side CSI generation part and the network-side CSI reconstruction part are trained separately by the UE side and network side, respectively [1]. In joint training, model transfer is often required, while dataset transfer is needed in separate training. In this paper, we focus on separate training.

Some dataset needs to be transferred from one side to the other over the air interface for the purpose of separate training. The ground truth channel vectors represented by floating point numbers are natural candidates to form the dataset. However, the overhead for dataset transfer is large. To reduce the overhead due to the dataset transfer, the dataset should be carefully designed in such a way that the space of the channel vectors is well represented by a set of small size.

The codebook used for CSI feedback in the legacy approach is naturally a good candidate for reducing the overhead. To begin with, Rel-16 e-type II codebook can be used for this purpose. The dataset can also be constructed by using the framework of codebook generation, but not restricted to the parameters given in the technical specification [4]. A larger dataset can be generated according to this idea, which may offer better training performance.

- 4.1 Dataset for Separate Training

We evaluate the performance of separate training using codebook-based dataset.

A dataset and a two-sided AI/ML model are paired in separate training. For example, an AI/ML model A is composed by an encoder (Encoder A) and a decoder (Decoder A), which is associated with a dataset A. Each element of a dataset is a pair composed by a label and its corresponding compressed version by its associated encoder. The labels in the codebook-based dataset are the quantization of the right singular vectors of the channel matrix. Specifically, the quantization is done by using the framework of Rel-16 e-type II codebook, but with non-3GPP specified parameters shown in Table 1. In particular, a number of 560 bits is used per codebook vector, which is equivalent to 70 bytes.

Table 1. The parameters of the Rel-16 e-type II like codebook that are used to construct the common dataset for separate training.

| L | pv | β | Mv | N3 | R | Reference Amplitude | Difference Amplitude | Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | 0.25 | 0.75 | 7 | 26 | 1 | 4 bits | 3 bits | 4 bits |

Two separate training schemes are studied in this paper. The general settings are described as follows. Suppose we have two pairs of two-sided AI/ML models that have been trained by some training set collected from two different locations, e.g., cells, well characterizing their communication environments, respectively. These two AI/ML models and their associated codebook-based datasets are indexed by A and B, respectively. Specifically, the AI/ML model A is composed by Encoder A and Decoder A, which is equipped with codebook-based dataset A. Similarly, AI/ML model B is composed by Encoder B and Decoder B, which is equipped with codebook-based dataset B.

The first separate training scheme is described as follows. In this scheme, the two two-sided AI/ML models are trained by their corresponding codebook-based datasets. Suppose a UE equipped with Encoder B moves to the area where Decoder A is operated at the gNB. Apparently, Encoder B and Decoder A do not form a pair, so Decoder A is very likely to fail the CSI recovery from Encoder B. The UE re-trains/finetunes its encoder using the codebook-based dataset A transferred, and obtain Encoder B’. The performance of the first scheme is measured by the SGCS of the input of the Encoder B’ and the output of the Decoder A, which are respectively the right singular vectors of the channel matrix and the recovered right singular vectors by Decoder A.

The second scheme is similar to the first one. The only difference is that, instead of being trained by the codebook-based datasets constructed by Table 1, the initial two AI/ML model pairs are trained by two datasets composed by unquantized right singular vectors of the channel matrices, respectively.

Link-level simulations are performed to evaluate the performance of the two schemes for separate training. The simulation parameters are summarized in Table 8 in Appendix A. The performance of the two schemes, together with the performance of the jointly trained AI/ML model and baseline Rel-16 codebook approach, are shown in Fig. 7. It is observed that the second scheme shows a similar performance of the joint training, and both the two proposed schemes outperform the legacy Rel-16 e-type II codebook approach. The results demonstrate that codebook-based dataset can be used for the purpose of separate training. In particular, the overhead is significantly smaller than using the right singular vectors represented by floating point numbers, but only with a minor performance degradation.

- CLD-C-30-10.

- CDL-C-300-10.

Fig. 7. The performance of the two schemes for separate training in LLS.

To see the overhead reduction, we do the following analysis. As discussed above, 560 bits are consumed for each codebook vector designed in Table 1, which is equivalent to 70 bytes. However, according to the antenna configuration and the number of subband used summarized in Table 8, a total number of 6656 bytes is needed to represent a right singular vector in floating-point numbers. By using codebook-based dataset, only 1.1% overhead is consumed compared to the dataset of right singular vectors, but only with a minor performance degradation in terms of the SGCS.

Observation-5: It is observed that there is a significant overhead reduction of transferring a codebook-based dataset than a dataset composed by channel vectors of floating-point numbers for separate training. So, it is worth to study codebook-based quantization method in order to achieve a low-overhead dataset transferring in separate training.

Proposal-8: In order to achieve a low-overhead dataset transferring in separate training, the codebook-based quantization approach should be further studied.

- 4.2 Parameters Design for the Codebook-based Dataset

In the framework of Rel-16 e-type II codebook, the size of the codebook vectors is determined by various parameters listed in Table 1. In this subsection, we present some initial study on the choice of parameters for the design of codebook-based dataset.

We study the effects of choosing various numbers of spatial-domain DFT beams (L), frequency-domain DFT beams (Mv), and the quantization of the difference amplitudes. The required bit numbers are highlighted in blue in Tables 2—5, and the SGCS of the quantized and ground-truth right singular vectors are highlighted in green.

By comparing Table 2 with Table 4, or Table 3 with Table 5, it is observed that the SGCS of the former is higher than that of the latter with a similar overhead. In addition, it is observed that the SGCS increases when the difference amplitude is quantized finer by comparing the 6th row of Table 2 and 6th row of Table 3, where the overhead for both cases are approximately 660 bits. Based on the results shown in Tables 2—5, we have the following observation and proposal.

Observation-6: The parameters of the codebook weight differently in the SGCS between the quantized codebook vectors and the ground-truth right singular vectors of the channel matrix.

Proposal-9: The choice of parameters for the codebook-based dataset should be studied, for the purpose of finding an optimal configuration where the performance and overhead tradeoff is achieved.

Table 2. The effect of various choices on the parameter L, when the difference amplitude is quantized to 3 bits.

| L | Mv | pv | β | Reference Amplitude | Difference Amplitude | phase | bit | SGCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 7 | 0.5 | 0.5 | 4 | 3 | 4 | 345 | 0.8631 |
| 6 | 7 | 0.5 | 0.5 | 4 | 3 | 4 | 408 | 0.883 |
| 7 | 7 | 0.5 | 0.5 | 4 | 3 | 4 | 472 | 0.8949 |
| 8 | 7 | 0.5 | 0.5 | 4 | 3 | 4 | 535 | 0.9036 |
| 9 | 7 | 0.5 | 0.5 | 4 | 3 | 4 | 598 | 0.9095 |
| 10 | 7 | 0.5 | 0.5 | 4 | 3 | 4 | 661 | 0.9142 |
| 11 | 7 | 0.5 | 0.5 | 4 | 3 | 4 | 724 | 0.9168 |
| 12 | 13 | 0.95 | 0.5 | 4 | 3 | 4 | 1424 | 0.9212 |

Table 3. The effect of various choices on the parameter L, when the difference amplitude is quantized to 4 bits.

| L | Mv | pv | β | Reference Amplitude | Difference Amplitude | phase | bit | SGCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 7 | 0.5 | 0.5 | 4 | 4 | 4 | 306 | 0.8375 |
| 5 | 7 | 0.5 | 0.5 | 4 | 4 | 4 | 379 | 0.8699 |
| 6 | 7 | 0.5 | 0.5 | 4 | 4 | 4 | 449 | 0.8917 |
| 7 | 7 | 0.5 | 0.5 | 4 | 4 | 4 | 520 | 0.9057 |
| 8 | 7 | 0.5 | 0.5 | 4 | 4 | 4 | 590 | 0.9163 |
| 9 | 7 | 0.5 | 0.5 | 4 | 4 | 4 | 660 | 0.9247 |
| 10 | 7 | 0.5 | 0.5 | 4 | 4 | 4 | 730 | 0.9319 |
| 12 | 13 | 0.95 | 0.5 | 4 | 4 | 4 | 1579 | 0.9762 |

Table 4. The effect of various choices on the parameter Mv, when the difference amplitude is quantized to 3 bits.

| L | Mv | pv | β | Reference Amplitude | Difference Amplitude | phase | bit | SGCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 8 | 0.6 | 0.5 | 4 | 3 | 4 | 315 | 0.8407 |
| 4 | 9 | 0.65 | 0.5 | 4 | 3 | 4 | 351 | 0.8464 |
| 4 | 10 | 0.7 | 0.5 | 4 | 3 | 4 | 386 | 0.8504 |
| 4 | 11 | 0.8 | 0.5 | 4 | 3 | 4 | 421 | 0.8531 |
| 4 | 12 | 0.9 | 0.5 | 4 | 3 | 4 | 454 | 0.8552 |
| 4 | 13 | 0.95 | 0.5 | 4 | 3 | 4 | 486 | 0.8568 |

Table 5. The effect of various choices on the parameter Mv, when the difference amplitude is quantized to 4 bits.

| L | Mv | pv | β | Reference Amplitude | Difference Amplitude | phase | bit | SGCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 8 | 0.6 | 0.5 | 4 | 4 | 4 | 346 | 0.847 |
| 4 | 9 | 0.65 | 0.5 | 4 | 4 | 4 | 386 | 0.8541 |
| 4 | 10 | 0.7 | 0.5 | 4 | 4 | 4 | 425 | 0.8595 |
| 4 | 11 | 0.8 | 0.5 | 4 | 4 | 4 | 464 | 0.8637 |
| 4 | 12 | 0.9 | 0.5 | 4 | 4 | 4 | 501 | 0.8672 |
| 4 | 13 | 0.95 | 0.5 | 4 | 4 | 4 | 537 | 0.87 |

- Evaluation Methodologies and Simulation Parameters

The evaluation is performed by link-level (LLS) and system-level simulations (SLS). The simulation parameters are summarized in this appendix.

- A.1 Performance of AI/ML-based CSI Compression

Table 6. Simulation parameters (system-level simulation).

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| BS Antenna Element Number (Nt) | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| UE Antenna Element Number (Nr) | 2: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Speed | 3 km/h (80%), 30 km/h (20%) |
| Channel Estimation | Ideal Channel Estimation |
| Rank per UE | 1 |
| Bandwidth | 20 MHz |
| Carrier Frequency | 4 GHz |
| Sub-Carrier Spacing | 30 KHz |
| RB Number | 51 |
| Sub-Band Number | 26 |

- A.2 Generalization of Payload

Table 7. Simulation parameters (link-level simulation for generalization).

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| BS Antenna Element Number (Nt) | 32: (16,1,2,1,1), (dH,dV) = (0.5, 0.8)λ |
| UE Antenna Element Number (Nr) | 2: (1,1,2,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Speed | 3 km/h |
| Channel Estimation | Realistic Channel Estimation |
| Rank per UE | 1 |
| Channel Model | CDL-C |
| Data size | 80000 for training, 4000 for testing. |
| Bandwidth | 10 MHz |
| RB Number | 24 |
| Sub-Band Number | 12 |
| Carrier Frequency | 4 GHz |
| Sub-Carrier Spacing | 30 kHz |
| Delay Spread | 30 ns |

- A.3 Separate Training

Table 8. Simulation parameters (link-level simulation for separate training).

| Parameter | Value |  |
| --- | --- | --- |
| Duplex, Waveform | FDD, OFDM |  |
| BS Antenna Element Number (Nt) | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |  |
| UE Antenna Element Number (Nr) | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |  |
| UE Speed | 3 km/h |  |
| Channel Estimation | Realistic Channel Estimation |  |
| Rank per UE | 1,2,3,4 |  |
| SNR | 10 dB |  |
| Channel Model | CDL-C |  |
| Data size | 80000 for training, 4000 for testing. |  |
| Bandwidth | 10 MHz |  |
| RB Number | 52 |  |
| Sub-Band Number | 26 |  |
| Carrier Frequency | 2 GHz |  |
| Sub-Carrier Spacing | 15 kHz |  |
| Delay Spread | 30 ns | 300 ns |

Conclusions

Observation-1: Generalization of payload can be achieved by a framework of a single encoder and multiple decoders with proper pre-processing/post-processing.

Observation-2: Various payload configurations can be supported in a unified two-sided AI/ML model with proper pre-processing/post-processing for CSI compression.

Observation-3: The performances of the generalization of payload are very similar in terms of the SGCS, no matter whether the structure of the AI/ML model is composed by a single encoder and a single decoder with proper pre-processing/post-processing, or a single encoder and multiple decoders with proper pre-processing/post-processing.

Observation-4: The performance of the generalization of payload, in terms of the SGCS, are very similar to the baseline scheme, where the two-sided AI/ML model is composed by an encoder and a decoder only, without pre-processing/post-processing.

Observation-5: It is observed that there is a significant overhead reduction of transferring a codebook-based dataset than a dataset composed by channel vectors of floating-point numbers for separate training. So, it is worth to study codebook-based quantization method in order to achieve a low-overhead dataset transferring in separate training.

Observation-6: The parameters of the codebook weights differently in the SGCS between the quantized codebook vectors and the ground-truth right singular vectors of the channel matrix.

Proposal-1: The vectors from the ideal channel matrix are suggested to be used in the formula of the squared generalized cosine similarity (SGCS).

Proposal-2: The KPI for the intermediate evaluation on AI/ML model performance for CSI feedback enhancement when the rank of the channel matrix is larger than 1 is the normalized weighted SGCS

SGCS=E1Ni=1Nj=1Kλij2j=1Kλij2wijHwij2 wij2wij2,

where λij is the j-th eigenvalue of the squared matrix GGH at resource unit i. The vector wij is the eigenvector associated with the eigenvalue λij, i=1,2,…,N, and j=1,2,…,K. The matrix G is the channel matrix, and the superscript H is the Hermitian operator. The vector wi is the reconstruction of the compressed wi.

Proposal-3: The baseline for the evaluation of the generalization/scalability of an AI/ML model over various configurations should be further studied. The baseline is suggested to be the performance of the two-sided AI/ML model composed by an encoder and a decoder only, without pre-processing/post-processing. Other potential baseline schemes can be further studied.

Proposal-4: For the generalization/scalability of an AI/ML model over various configurations, it should be clarified how an AI/ML model is defined.

Alt. 1. Pre-/post-processing are precluded.

Alt. 2. Pre-/post-processing are included, i.e., a unique AI/ML model in the sense that the backbone model is unique.

Alt. 3. One encoder vs. multiple decoders, or vice versa.

Proposal-5: The pre-/post-processing are suggested to be counted as part of an AI/ML model for the generalization/scalability of an AI/ML model over various configurations.

Proposal-6: The KPIs for the generalization/scalability of an AI/ML model should be further studied. In particular, both the performance-related KPI and the complexity-related KPI are suggested to be used.

Proposal-7: It is suggested that the comparison of the model complexity of baseline and generalization schemes can be taken as a KPI. As an example, the ratio of the model complexity of the baseline approach over that of the generalization scheme.

Proposal-8: In order to achieve a low-overhead dataset transferring in separate training, the codebook-based quantization approach should be further studied.

Proposal-9: The choice of parameters for the codebook-based dataset should be studied, for the purpose of finding an optimal configuration where the performance and overhead tradeoff is achieved.

- References

[1] 3GPP R1-2207840: “Summary#5 for CSI evaluation of [110-R18-AI/ML],” Jun. 2022.

[2] 3GPP SR RP-221347: “Status report for study on AI/ML for NR air interface”, Jun. 2022.

[3] 3GPP R1-2206164, “Evaluations on AI/ML for CSI feedback enhancement,” Fujitsu, Aug. 2022.

[4] 3GPP TS 38.214, “Physical layer procedures for data (Release 17),” V17.2.0, Jun. 2022.