3GPP TSG RAN WG1 #110bis-e			R1-2209266

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.11.3

Source:		Xiaomi

Title:	Discussion on the HARQ operation for IoT NTN

Document for:	Discussion

Introduction

In RAN1#110 meeting, the following agreements were made on the HARQ operation for IoT NTN [1]:

Agreement

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options:

- Option 1: per HARQ process via UE specific RRC signaling.

- Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field).

- Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS)

- Option 6: combinations of some options above.

Agreement

For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options:

- Option 1: per HARQ process via UE specific RRC signaling

- Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field)

- Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS)

- Option 6: combinations of some options above

Agreement

For a DL HARQ process with disabled HARQ feedback in NB-IoT, at least the following UE behavior(s) can be considered:

- Option 1: UE is not expected to receive another NPDCCH carrying a DCI scheduling a NPDSCH for a given HARQ process that starts until X(ms) after the end of the reception of the last NPDSCH for that HARQ process.

  - X =12

- Option 2: UE is not required to monitor NPDCCH in a period of Y(ms) from the end of reception of the last NPDSCH

  - Y=12

Note: it may be different UE behaviors for different UE categories (e.g., UE with single/multiple HARQ processes)

In this contribution, we present the discussion on the HARQ disabling operation in IoT NTN scenario.

Discussion

Scenario to support HARQ disabling

HARQ disabling for NR-NTN is supported in Rel-17, while for IoT NTN, this is discussed in Rel-17 however not introduced. It is argued that the HARQ disabling can bring the following advantages:

-	UE power saving

-	Throughput increase without increasing UE complexity

-	Improved resource utilization

However, support of HARQ disabling can have the negative impact on the reliability which in turn can possibly increase the UE’s power consumption due to enhanced transmission schemes such as increased repetition number. In our understanding, the main benefit to support HARQ disabling is to resolve the HARQ stalling issue due to the limited HARQ process available at the UE side. The UE can continue the transmission with the supported HARQ process without waiting for the HARQ feedback, and thus, the UE’s experienced data rate can be improved.

Observation 1: The main benefit to support HARQ disabling is to resolve the HARQ stalling issue.

The HARQ stalling issue is mainly due to the large transmission delay in NTN scenario as illustrated in the table below:

Table 1: RTT in different scenarios

| Constellation | RTT (ms) |
| --- | --- |
| GEO | 541.46 |
| LEO-600km | 25.77 |
| LEO-1200km | 41.77 |

Taken the NB-IoT as an example to analyze the HARQ stalling issue. The time used to deliver the data and corresponding HARQ information can be determined as:

- NPUSCH : NumberofslotNPUSCH=NTBNRepNRUNslotsRU, where NTB is the number of scheduled TB, NRep  is the repetition number, NRU is the RU number scheduled for transport block, NslotsRU is the number of slots in each RU;

- NPDSCH : NumberofsubframeNPDSCH=NTBNRepNSF, where NTB is the number of scheduled TB, NRep is the repetition number, NSF is the number of slots scheduled for transport block;

The maximum time used can be up to 8*10*128=10240 ms for a NPUSCH transmission while can be up to 2048*10=20480 ms for a NPDSCH transmission respectively.

For a NB-IoT device configured with a single HARQ process, the HARQ stalling issue happens as illustrated in the figure below. After the reception of a DL transmission, the UE needs to hold its DL reception until a new DL grant is received.

Figure 1: HARQ stalling issue for NB-IoT UE configured with one HARQ process

Observation 2: HARQ stalling issue happens when the IoT UEs are configured with only one HARQ process.

The performance gain for the NB-IoT UE configured with one HARQ process with HARQ disabling is illustrated in table 2 below:

Table 2: Performance gain for the NB-IoT UE configured with one HARQ process with HARQ disabling

| Constellation | Repetition number for NPDCCH | Repetition number for NPDSCH | Repetition number for NPUSCH | Performance gain with HARQ disabling |
| --- | --- | --- | --- | --- |
| GEO | 128 | 64 | 64 | 37.2% |
| LEO1200 | 32 | 16 | 16 | 14.9% |
| LEO600 | 16 | 8 | 8 | 17.1% |

For a NB-IoT device configured with more than one HARQ process, the HARQ stalling issue may not happen as illustrated in the figure below as the RTT can be filled by the NPDSCH with the other one HARQ process. The performance gain may be marginal which comes from the absence of the transmission of the HARQ information.

Figure 2: No HARQ stalling issue for NB-IoT configured with more than one HARQ process

Observation 3: No HARQ stalling issue happens when the IoT UEs are configured with more than one HARQ process.

Based on the analysis, the HARQ disabling can be supported for at least for the UE that is only configured/capable of single HARQ process.

Proposal 1: The HARQ disabling can be supported for at least for the IoT UE that is only configured/capable of single HARQ process.

Design aspects of supporting HARQ disabling

Indication of HARQ disabling

It was agreed that for NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options:

- Option 1: per HARQ process via UE specific RRC signaling

- Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field)

- Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS)

- Option 6: combinations of some options above

In Rel-17 NR NTN design, the HARQ disabling is configured by RRC signalling. For the transmission of the important information, the HARQ enabled process can be used. However, for the IoT device that is configured/capable of only one HARQ process, the semi-static configuration may not be flexible to guarantee the reception reliability of the important information. Thus dynamic HARQ disabling can be supported. Meanwhile, in order to avoid any limitation on the scheduling, the explicit indication in the DCI is preferred.

Proposal 2: Dynamic HARQ disabling can be supported at least for the IoT UE configured/capable of one HARQ process.

SPS PDSCH

In NR NTN design, the HARQ information is always reported by the UE for the first SPS PDSCH regardless of whether the associate HARQ process is enabled or not if HARQ feedback for SPS activation is additionally enabled. Then for other SPS PDSCH, whether the HARQ information is reported or not is dependent on HARQ disabling configuration. Meanwhile, for the DCI indicating SPS release, HARQ information is always reported either. In IoT NTN, the same design can be reused. We don’t see any further enhancement is needed.

Proposal 3: The HARQ design for SPS PDSCH in NR NTN can be reused in IoT NTN.

DL HARQ scheduling constraint

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12. The legacy scheduling constraint should be kept, and thus it should be supported that for a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH in a period of 12ms from the end of reception of the last NPDSCH.

Proposal 4: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH in a period of 12ms from the end of reception of the last NPDSCH.

HARQ feedback for scheduling multiple TB

For a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter npdsch-MultiTB-Config, there shall be a maximum of 2 downlink HARQ processes. If the UE is configured with higher layer parameter npdsch-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the HARQ process ID of 0 is for the first TB and HARQ process ID of 1 shall be assumed for the second TB [2]. It is possible that the HARQ process 0 is a HARQ enabled process, while the HARQ process 1 is a HARQ disabled process. Then how the UE should generate the HARQ information should be defined. A straightforward solution is that UE only report the HARQ information for the HARQ enabled process.

Proposal 5: UE only report the HARQ information for the HARQ enabled process in multi-TB scheduling scenario with different HARQ processes for NB-IoT.

Conclusions

In this contribution, we discuss the necessity and design aspect related to HARQ disabling. We have the following observations and proposals based on our analysis:

Observation 1: The main benefit to support HARQ disabling is to resolve the HARQ stalling issue.

Observation 2: HARQ stalling issue happens when the IoT UEs are configured with only one HARQ process.

Observation 3: No HARQ stalling issue when the IoT UEs are configured with more than one HARQ process.

Proposal 1: The HARQ disabling can be supported for at least for the IoT UE that is only configured/capable of single HARQ process.

Proposal 2: Dynamic HARQ disabling can be supported at least for the IoT UE configured/capable of one HARQ process.

Proposal 3: The HARQ design for SPS PDSCH in NR NTN can be reused in IoT NTN.

Proposal 4: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH in a period of 12ms from the end of reception of the last NPDSCH.

Proposal 5: UE only report the HARQ information for the HARQ enabled process in multi-TB scheduling scenario with different HARQ processes for NB-IoT.

References

- 3GPP RAN1 #110 meeting chairman-note, 3GPP TSG RAN WG1 #110, Toulouse, France, August 22nd – 26th, 2022.

- 3GPP TS 36.213, Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures (Release 17), v17.2.0 (2022-06)