3GPP TSG RAN WG1 #110							    R1-2209322

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.1.2

Title:	Discussion on CSI enhancement for high/medium UE velocities and CJT

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 WID for NR MIMO evolution for downlink and uplink was approved [1], in which two potential enhancements are for CSI enhancement for high/medium UE velocities and coherent JT (CJT) as shown below:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we will share our views for the CSI enhancements for high/medium UE velocities and coherent JT scheme respectively.

- Type-II codebook refinement for CJT

  - Codebook parameters enhancement for CJT

In RAN1#109-e meeting [2], we have the following agreement on the codebook configuration for CJT:

Agreement @RAN1#109-e On the Type-II codebook refinement for CJT mTRP, the resulting codebook(s) are associated with at least the following parameters: - Parameters for basis reporting, including - The number of basis vectors: gNB-configured via higher-layer signaling - FFS: Whether it is layer-common or layer-specific, whether it is per TRP/TRP-group or common for all TRPs - Basis selection indicator(s): a part of CSI report - FFS: Whether it is layer-common or layer-specific, whether it is per TRP/TRP-group or common for all TRPs - Quantized combining coefficients (W2): a part of CSI report - FFS: details of quantization scheme - Number of non-zero coefficients and bitmap to indicate non-zero coefficients, including whether it is per TRP/TRP-group (separate) or across all TRPs/TRP-groups (joint): a part of CSI report - Strongest coefficient indicator(s) (SCI(s)): a part of CSI report - FFS: One per TRP/TRP-group or common for all TRPs - FFS: Additional need for strongest TRP indicator

Based on the R18 WID description, the R18 CJT codebook is just a refinement of R16/17 Type-II codebook, hence the basic design should be reused as much as possible. In R16 and R17 codebook, the SD basis number and selection have already been layer-common and polarization common and the spatial domain characteristics in each layer and polarization seems not have big difference in CJT scenario, so the SD basis selection should be layer-common and polarization common for one TRP or one TRP group. And the definitions of parameters of L, N1, N2, O1, O2 in R16 eType-II codebook can also be reused for R18 CJT codebook.

Proposal 1: For the Rel-18 Type-II codebook for CJT, the SD basis selection should be layer-common and polarization common for a TRP or a TRP group.

Proposal 2: For the Rel-18 Type-II codebook for CJT, the definitions of parameters of L, N1, N2, O1, O2 in R16 eType-II codebook can be reused for R18 CJT codebook for a TRP or a TRP group.

As for the FD basis selection, the FD basis selection is layer specific in R16 regular eType-II codebook design, and in R17 PS FeType-II codebook design, it should be layer common. Therefore, for the refinement for R16 regular eType-II codebook, the FD basis selection should be layer specific and the definitions of parameters of Mv, pv, N3, and R in R16 eType-II codebook can be reused for a TRP or a TRP group; for the refinement for R17 PS FeType-II codebook, the FD basis selection should be layer common and the definitions of parameters of M, N3, and R in R17 PS FeType-II codebook can be reused for a TRP or a TRP group.

Proposal 3: On the Rel-18 Type-II codebook for CJT, for the refinement based on R16 regular eType-II codebook, the FD basis selection should be layer specific.

Proposal 4: On the Rel-18 Type-II codebook for CJT, for the refinement based on R16 regular eType-II codebook, the definitions of parameters of Mv, pv, N3, and R for a TRP or a TRP group can be reused.

Proposal 5: On the Rel-18 Type-II codebook for CJT, for the refinement based on R17 PS FeType-II codebook, the FD basis selection should be layer common.

Proposal 6: On the Rel-18 Type-II codebook for CJT, for the refinement based on R17 FeType-II codebook, the definitions of parameters of M, N3, and R for a TRP or a TRP group can be reused.

Regarding to the issue of the number of the SD and FD basis and the basis selection are per TRP/TRP-group or common for all TRPs, for FD basis selection, it should be more related to the codebook mode for CJT, i.e. FD basis selection should be per TRP/TRP-group for mode 1, and should be TRP/TRP-group common for all TRPs for mode 2. However, for SD basis selection, since the TRP/TRP-group might be have different locations (inter-cell CJT) or different boresights (intra-cell CJT), the spatial domain characteristics vary very large for different TRPs, it is natural to configure different parameter of L and different SD basis selection. Hence, for SD basis selection in R18 CJT codebook, the SD basis selection should be determined per TRP/TRP-group and the parameter of L also should be configured per TRP/TRP-group.

Proposal 7: For SD basis selection in R18 CJT codebook, the SD basis selection should be determined per TRP/TRP-group and the parameter of L also should be configured per TRP/TRP-group.

As for the number of non-zero coefficients and bitmap to indicate non-zero coefficients, considering that SD basis selection is TRP/TRP-group specific and the FD basis is TRP/TRP-group specific in codebook design mode 1, the number of non-zero coefficients and bitmap to indicate non-zero coefficients also should be TRP/TRP-group specific. Even though in some cases, the SD and FD basis selection is the same over different TRPs/TRP groups, the number of non-zero coefficients and bitmap also will be different, since the TRPs/TRP-groups have different locations or boresights. Hence, regarding the location of non-zero coefficients (NZCs) indicated by bitmap, separate bitmaps should be reported for all the TRPs/TRP groups.

Proposal 8: For the location of non-zero coefficients (NZCs) indicated by bitmap in R18 CJT codebook, separate bitmaps should be reported for all the TRPs/TRP groups.

Besides, since the locations or boresights of different TRPs/TRP groups might be different, the amplitude in W2 coefficients over different TRPs/TRP groups might have large difference. In addition, the coefficients of some strongest TRPs/TRP groups might be of much larger value in the description of CJT channel state, these coefficients might need more smaller granularity in quantization scheme. Hence, it is beneficial to report strongest coefficient indicator (SCI) per TRP/TRP group. And to establish the relationship between these strongest coefficient indicators on these TRPs/TRP groups, it is also better to indicate the strongest one SCI among these SCIs to locate the reference coefficient, i.e., strongest TRP/TRP group indicator.

Proposal 9: On the Rel-18 Type-II codebook for CJT, support to report strongest coefficient indicator (SCI) per TRP/TRP group.

Proposal 10: On the Rel-18 Type-II codebook for CJT, support to report strongest TRP/TRP group indicator.

  - TRP selection scheme for CJT

In RAN1#110 meeting [3], we have the following agreement on the TRP selection/determination scheme for CJT:

Agreement On the Type-II codebook refinement for CJT mTRP, down-select from the following TRP selection/determination schemes (where N is the number of cooperating TRPs assumed in PMI reporting) by RAN1#110bis-e: - Alt1. N is gNB-configured via higher-layer (RRC) signalling - The N configured TRPs are gNB-configured via higher-layer (RRC) signalling - Note: only one transmission hypothesis is reported - Alt2. N is UE-selected and reported as a part of CSI report where N∈{1,..., NTRP} - N is the number of cooperating TRPs, while NTRP is the maximum number of cooperating TRPs configured by gNB - In this case, the selection of N out of NTRP TRPs is also reported (FFS: exact reporting scheme) - FFS: Configuration of NTRP TRPs and the value of NTRP, whether explicit or implicit - Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses. FFS: Whether S-TRP transmission hypothesis is also reported

For the Alt2 in this agreement, UE need to report the number of N which is UE-selected as a part of CSI report. Also, the PMI(s) corresponding to selected N(s) are needed to be reported. There are two possibility about relationship between the reported N(s) and the corresponding PMI(s). The first relationship is that the reported N(s) and the corresponding PMI(s) are reported in two different CSI report or two different parts of one CSI report. It seems have great spec impact on current specifications. The second relationship is that the reported N(s) and the corresponding PMI(s) are reported in one CSI report. It means gNB need to allocate PUCCH/PUSCH resources assuming the PMI of NTRP TRPs are reported, then if UE reports the CSI of N < NTRP TRPs, some PUCCH/PUSCH resources will be wasted, especially in the case of the N is much smaller than NTRP.

Hence, our first preference for the TRP selection scheme for CJT is Alt1, since gNB could choose the best N for each UE considering the globally optimal scheduling scheme based on the overall channel condition and traffic load in the CJT area.

As for whether S-TRP transmission hypothesis can also be reported, now that the N configured TRPs are gNB-configured, if the gNB want to configure a CSI report for single TRP transmission, it can directly configure R16 eType-II codebook or R17 FeType-II PS codebook, there is no need to configure R18 CJT codebook. Therefor, it seems the S-TRP transmission hypothesis is not needed, at least for Alt1.

Proposal 11: For the TRP selection/determination scheme in Rel-16 Type-II codebook refinement for CJT mTRP, Alt1 should be considered:

- Alt1. N is gNB-configured via higher-layer (RRC) signaling

  - The N configured TRPs are gNB-configured via higher-layer (RRC) signaling

  - Note: only one transmission hypothesis is reported

- Type-II codebook refinement for high/medium UE velocities

  - Codebook structure for high/medium UE velocities

In RAN1#110 meeting [3], we have the following agreement on the codebook structure for high/medium UE velocities:

Agreement @RAN1#110 For the Rel-18 Type-II codebook refinement for high/medium velocities, down-select one from the following codebooks structures: - Alt2A: Doppler-domain basis commonly selected for all SD/FD bases, e.g. W1W2(Wf⨂Wd)H - Note that Wd may be the identity as a special case - Alt2B: Doppler-domain basis independently selected for different SD/FD bases - Note that Wd may be the identity as a special case - Alt3. Reuse Rel-16/17 (F)eType-II codebook with multiple W2 and a single W1 and Wf report.

Firstly, for Alt3, it seems more like an enhancement on PMI reporting, which decouple W2 and W1/ Wf, instead of codebook refinement based on Rel-16/17 (F)eType-II codebook. It does not involve any channel compression in time/Doppler domain currently. Hence, when UE need to report a large number of PMI, i.e., the length of the DD/TD basis vector N4 is quite large in Alt1 or Alt2, it will bring much PUCCH/PUSCH overhead due to multiple W2 need be reported. Besides, from the perspective of CSI report framework, Alt3 need change the PMI structure in current spec, which will bring much standardization efforts.

Secondly, for Alt 2A and Alt 2B, Alt 2B will definitely depict the channel more accurately, however, it will cost very much PMI overhead since the total number of TD/FD basis and the corresponding coefficients in W2 are dramatically increased. In addition, the mathematical formula of Alt2B is not easy to describe. Besides, Alt2B can be seen as a special case of Alt2A, if the number of Doppler-domain basis is really huge.

Therefore, considering the specification readability and the trade-off between throughput and PMI overhead, we prefer to adopt Alt2A as a starting point for the codebook structure for high/medium UE velocities.

Proposal 12: For the codebook structure of Type-II codebook refinement for high/medium velocities, support Alt2A:

- Alt2A: Doppler-domain basis commonly selected for all SD/FD bases, e.g. W1W2(Wf⨂Wd)H

  - Note that Wd may be the identity as a special case

  - CSI reporting and measurement for high/medium velocities

In RAN1#110 meeting [3], we have the following agreements on the CSI reporting and measurement for the Type-II codebook refinement for high/medium velocities:

Agreement @RAN1#110 On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the assumption of the UE-side prediction - On the definition of UE-side prediction, down-select one from the following alternatives: - Alt1. UE “predicting” channel/CSI after the slot with a reference resource - Alt2. UE “predicting” channel/CSI after slot n (where the CSI is reported) - Alt3. UE “predicting” channel/CSI after the slot where CSI-RS resides Agreement @RAN1#110 On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities assuming the UE-side prediction, on the definition of UE-side prediction, down-select one from the following alternatives by RAN1#110bis-e: - Alt1. UE “predicting” channel/CSI after the slot with a reference resource - Alt2. UE “predicting” channel/CSI after slot n (where the CSI is reported) Agreement @RAN1#110 On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, down-select one from the following alternatives by RAN1#110bis-e: - Alt1.B: l ≥ nref - nref (a CSI reference resource slot) as boundary - Alt2.B: l ≥ n - n (report slot) as boundary

In last meeting, we have already agreed the assumption of UE-side prediction for high/medium UE velocities, however, the definition of future CSI is needed, i.e., the location of CSI reporting window. From our perspective of view, the length of CSI reporting window is more related to the UE speed. When the speed of UE is not so high and UE implementation of CSI prediction is enough to predict CSI over a relatively long period, the reporting window can be relatively longer.

However, if the UE moves quite fast, then UE cannot guarantee the CSI prediction accuracy in much further future, so that the reporting window might be quite short and even the end of reporting window can be before the slot of CSI report. Alt2.B cannot include this case, since in Alt2.B, the beginning of CSI reporting window already should be later than report slot.

Alt2.B might have some advantages that the reported CSI is closer than the channel condition when the PDSCH is scheduled. However, there is still a gap between the scheduling slot and CSI reporting slot, this gap is much related to UE traffic state and scheduling strategy and may be even larger than the gap between CSI reference resource slot and report slot. Besides, if Alt2.B is adopted, the CQI calculation timeline will become a new issue and a new reference resource for CQI may be needed.

Therefore, we prefer adopt the CSI reference resource slot as the boundary of UE prediction channel/CSI.

Proposal 13: For the codebook structure of Type-II codebook refinement for high/medium velocities, on the definition of UE-side prediction, support Alt1.B:

- Alt1.B:  l ≥ nref

  - nref (a CSI reference resource slot) as boundary

- TRS-based reporting of time-domain channel properties (TDCP)

In RAN1#110 meeting [3], we have the following agreements on the TRS-based reporting of time-domain channel properties (TDCP):

Agreement @RAN1#110 For the Rel-18 TRS-based TDCP reporting, down select one of the following alternatives by RAN1#110bis-e: - AltA. Based on Doppler profile - E.g., Doppler spread derived from the 2nd moment of Doppler power spectrum, average Doppler shifts, Doppler shift per resource, maximum Doppler shift, relative Doppler shift, etc - AltB. Based on time-domain correlation profile - E.g. Correlation within one TRS resource, correlation across multiple TRS resources - Note: The correlation over one or more lags of TRS resource may be considered. The lags may be within one TRS burst or different TRS bursts - AltC: CSI-RS resource and/or CSI reporting setting configuration parameter(s) to assist network - E.g. gNB configures UE with multiple choices on what to assist (e.g. two or more CSI-RS/report periodicities, or precoding schemes depending mainly on UE velocity), then UE report according to configuration; parameters correspond to CSI reporting periodicity, codebook type, etc. Note: Different alternatives may or may not apply to different use cases

From our perspective of view, AltC is not practical in real 5G network, since if UE want to report CSI-RS resource or CSI report configuration parameters to assist gNB’s CSI configuration or precoding scheme, the real gNB implementation details are needed for UE, which is not realistic. Besides, gNB can make the globally optimal scheduling scheme based on the overall channel condition and traffic load, this information also cannot be acquired for UE.

Hence, we think AltA and AltB can be further studied and AltC can be excluded.

Proposal 14: For TRS-based TDCP reporting, AltA and AltB can be further studied as the TDCP report quantity:

- AltA. Based on Doppler profile

  - E.g., Doppler spread derived from the 2nd moment of Doppler power spectrum, average Doppler shifts, Doppler shift per resource, maximum Doppler shift, relative Doppler shift, etc

- AltB. Based on time-domain correlation profile

  - E.g. Correlation within one TRS resource, correlation across multiple TRS resources

  - Note: The correlation over one or more lags of TRS resource may be considered.  The lags may be within one TRS burst or different TRS bursts

- Conclusion

Proposal 1: For the Rel-18 Type-II codebook for CJT, the SD basis selection should be layer-common and polarization common for a TRP or a TRP group.

Proposal 2: For the Rel-18 Type-II codebook for CJT, the definitions of parameters of L, N1, N2, O1, O2 in R16 eType-II codebook can be reused for R18 CJT codebook for a TRP or a TRP group.

Proposal 3: On the Rel-18 Type-II codebook for CJT, for the refinement based on R16 regular eType-II codebook, the FD basis selection should be layer specific.

Proposal 4: On the Rel-18 Type-II codebook for CJT, for the refinement based on R16 regular eType-II codebook, the definitions of parameters of Mv, pv, N3, and R for a TRP or a TRP group can be reused.

Proposal 5: On the Rel-18 Type-II codebook for CJT, for the refinement based on R17 PS FeType-II codebook, the FD basis selection should be layer common.

Proposal 6: On the Rel-18 Type-II codebook for CJT, for the refinement based on R17 FeType-II codebook, the definitions of parameters of M, N3, and R for a TRP or a TRP group can be reused.

Proposal 7: For SD basis selection in R18 CJT codebook, the SD basis selection should be determined per TRP/TRP-group and the parameter of L also should be configured per TRP/TRP-group.

Proposal 8: For the location of non-zero coefficients (NZCs) indicated by bitmap in R18 CJT codebook, separate bitmaps should be reported for all the TRPs/TRP groups.

Proposal 9: On the Rel-18 Type-II codebook for CJT, support to report strongest coefficient indicator (SCI) per TRP/TRP group.

Proposal 10: On the Rel-18 Type-II codebook for CJT, support to report strongest TRP/TRP group indicator.

Proposal 11: For the TRP selection/determination scheme in Rel-16 Type-II codebook refinement for CJT mTRP, Alt1 should be considered:

- Alt1. N is gNB-configured via higher-layer (RRC) signaling

  - The N configured TRPs are gNB-configured via higher-layer (RRC) signaling

  - Note: only one transmission hypothesis is reported

Proposal 12: For the codebook structure of Type-II codebook refinement for high/medium velocities, support Alt2A:

- Alt2A: Doppler-domain basis commonly selected for all SD/FD bases, e.g. W1W2(Wf⨂Wd)H

  - Note that Wd may be the identity as a special case

Proposal 13: For the codebook structure of Type-II codebook refinement for high/medium velocities, on the definition of UE-side prediction, support Alt1.B:

- Alt1.B:  l ≥ nref

  - nref (a CSI reference resource slot) as boundary

Proposal 14: For TRS-based TDCP reporting, AltA and AltB can be further studied as the TDCP report quantity:

- AltA. Based on Doppler profile

  - E.g., Doppler spread derived from the 2nd moment of Doppler power spectrum, average Doppler shifts, Doppler shift per resource, maximum Doppler shift, relative Doppler shift, etc

- AltB. Based on time-domain correlation profile

  - E.g. Correlation within one TRS resource, correlation across multiple TRS resources

  - Note: The correlation over one or more lags of TRS resource may be considered.  The lags may be within one TRS burst or different TRS bursts

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94-e, e-Meeting, December 6 - 17, 2021.

- 3GPP RAN1#109-e, RAN1 Chairman’s Notes, May 2022.

- 3GPP RAN1#110, RAN1 Chairman’s Notes, August 2022.