3GPP TSG-RAN WG1 # 110bis-e	R1-2209476

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on determining NACK-only PUCCH in NACK-only mode1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | For multiplexing between NACK-only PUCCH and the other PUCCH or PUSCH, it is assumed that the UE would transmit the NACK-only PUCCH regardless the PDSCH decoding results according to the agreement. The reason is that the UE will not transmit NACK-only PUCCH when the HARQ-ACK information is ‘ACK’. This has been reflected in the specification when the UE is configured with NACK-only mode 2. However, the description for UE configured with NACK-only mode 1 is still missing since the HARQ-ACK feedback is still NACK-only mode when there is only one PDSCH for MBS. It should be added in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the UE behavior for multiplexing between NACK-only PUCCH and other PUCCH or PUSCH when NACK-only mode 1 is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior is unclear in the above case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR is based on RAN1’s common understanding. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

- Multicast Broadcast Services

<Unchanged parts are omitted>

If a UE would multiplex multicast HARQ-ACK information according to the second HARQ-ACK reporting mode with multicast HARQ-ACK information according to the first HARQ-ACK reporting mode, or unicast HARQ-ACK information, or CSI reports in a first PUCCH or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH when the UE is provided moreThanOneNackOnlyMode, the UE considers that the UE would transmit the second PUCCH using any PUCCH resource from the PUCCH resources associated with the second HARQ-ACK reporting mode when all values of the HARQ-ACK information are 'ACK'; otherwise, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK'.

<Unchanged parts are omitted>