3GPP TSG RAN WG1 #110bis-e			 R1-2209556

e-Meeting, October 10th – 19th, 2022

Title:	LS on beamManagementSSB-CSI-RS

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN2

Cc:	-

Contact person:

Name                          Haitong Sun

E-mail Address          Haitong_sun@apple.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

In RAN1#101-e meeting, to clarify the resource counting for L1-RSRP measurement, i.e., maxNumberSSB-CSI-RS-ResourceOneTx and maxNumberCSI-RS-ResourceTwoTx in feature group beamManagementSSB-CSI-RS when measurement is performed across multiple cells with different SCS, RAN1 agreed on the following conclusion

Conclusion For UE features maxNumberSSB-CSI-RS-ResourceOneTx and maxNumberCSI-RS-ResourceTwoTx in feature group beamManagementSSB-CSI-RS, - the total number of resources within a slot and across serving cells in FR1 is determined by x within 1 slot of subcarrier spacing of 15kHz - the total number of resources within a slot and across serving cells in FR2 is determined by y within 1 slot of the smallest subcarrier spacing configured for PDSCH in FR2 - the total number of resources within a slot and across FR1 and FR2 serving cells is determined by max (x, z*y) within 1 slot of subcarrier spacing of 15 kHz, - where x is the reported value in FR1 and y is the reported value in FR2 and z is the ratio of the smallest subcarrier spacing configured in FR2 and 15kHz.

# 2	Actions

To RAN2

ACTION: 	RAN1 respectfully requests RAN2 to take the above information into account.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#111			14th -18th Nov 2022	   			Toulouse, FR

RAN1#112			24th Feb - 3rd March 2023			Athens, GR