3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209562

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.214 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Q formula for the second most recent perioidic sensing occasion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For PBPS, UE monitors the most recent sensing occasion for a given reservation periodicity before the first slot of the set of Y or Y’ candidate slots by default. If (pre)configured, UE additionally monitors the second most recent sensing occasion for a given reservation periodicity before the first slot of the set of Y or Y’ candidate slots. For the latter case, the value Q in step 6) c) of TS 38.214 Section 8.1.4 should be adjusted accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When additionalPeriodicSensingOccasion is (pre)configured, Q=TscalPrsvp_RX+1 if Prsvp_RX'<n'-m≤2·Prsvp_RX', Prsvp_RX=Preserve and the sensing occasion tm'SL belongs to ty-k×Preserve''SL for a given periodicity Preserve. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The sensing result in the second most recent sensing occasion for a given reservation periodicity is not counted in the resource selection window. This implies the useless sensing on the second most recent sensing occasion for a given reservation periodicity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operatability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=================================== Start of Text Proposal =================================

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

< Unchanged parts are omitted >

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, when additionalPeriodicSensingOccasion is (pre)configured, Q=TscalPrsvp_RX+1 if Prsvp_RX'<n'-m≤2·Prsvp_RX', Prsvp_RX=Preserve and the sensing occasion tm'SL belongs to ty-k×Preserve''SL for a given periodicity Preserve. Otherwise, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer, t'n'SL=t'yiSL-Tproc,1SL if slot t'yiSL-Tproc,1SL belongs to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL), otherwise, slot t'n'SL is the first slot after slot t'yiSL-Tproc,1SL belonging to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL). Otherwise Q=1. If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=t'yLSL-(t'yiSL-Tproc,1SL) shall be converted to milliseconds, where slot t'yLSL is the last slot of the Y or Y' candidate slots. The slot t'yiSL is the first slot of the selected/remaining set of Y or Y' candidate slots.

< Unchanged parts are omitted >

=================================== End of Text Proposal ==================================