3GPP TSG- RAN WG1 Meeting #110bis-e                                               R1- 2209581

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.2.4.2

Source:	Apple Inc.

Title:	Other aspects on AI/ML for positioning accuracy enhancement

Document for:	Discussion/Decision

# Introduction

In the SID governing the AI/ML study, an initial set of use cases has been decided on for positioning accuracy enhancements in different scenarios including, e.g., those with heavy NLOS conditions with  the following objective [1]:

- finalize representative sub use cases for each use case for characterization and baseline performance evaluations.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

In this contribution, we provide our views on enhancement use cases and the potential specification impact on AI/ML for positioning accuracy enhancement.

# Use cases and potential spec impact

The following agreement was made in RAN1 #109-e [3],

Agreement For further study, at least the following aspects of AI/ML for positioning accuracy enhancement are considered. - Direct AI/ML positioning: the output of AI/ML model inference is UE location - E.g., fingerprinting based on channel observation as the input of AI/ML model - FFS the details of channel observation as the input of AI/ML model, e.g. CIR, RSRP and/or other types of channel observation - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement - E.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement - FFS the details of input and output for corresponding AI/ML model(s) - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - Companies are encouraged to clarify all details/aspects of their proposed AI/ML approaches/sub use case(s) of AI/ML for positioning accuracy enhancement

The following agreements were made in RAN1 #110 [4]:

Agreement For characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected. - Direct AI/ML positioning - AI/ML assisted positioning - Note 1: the selection does not intend to provide any indication of the prospects of any future normative project. - Note 2: further discussion (including selection of other sub use cases and/or down selection of selected sub use cases) are not precluded based on performance evaluation and potential specification impact study results

## Direct AI/ML based positioning

Sub-Use case 1: This can be used in heavy NLOS scenarios (such as  InF-DH) as shown in the simulation results in [2]. In this case, the input can be the Channel Impulse Response (CIR), the Power Delay Profile (PDP) and/or the Layer 1 Reference Signal Received Power (L1-RSRP) while the output is the UE position. Note that this may include a scenario where a NN may be used to select one of  multiple NNs based on a input such as Doppler.

Figure 1: Use Case 1 – CIR/PDP/L1-RSRP to UE position

Specification Impact: The following are possible specification impacts in this use case:

- Acquisition of the neural network inference input such as CIR, PDP or L1-RSRP for multiple gNBs from the candidate positioning UEs to the inference device.

  - Note that we may have a mix of inputs based on the difficulty of acquisition. For example, the L1-RSRP may be easier to acquire for more gNBs and as such we have a mix of CIR and L1-RSRP as input into the AI model.

- Possible acquisition of the neural network training input for multiple gNBs from the candidate positioning UEs or multiple gNBs to the inference device

- Calibration input and procedures to validate the AI model e.g. comparison with GNSS positions.

- Ground truth label assistance information to the inference device

- Applicable scenarios: this is suitable for high NLOS scenarios where there is enough network capacity to enable signaling/feedback of the channel observations to the LMF or TDD systems where there is channel reciprocity and the uplink channel may be used as the channel observation.

## AI/ML assisted positioning

Sub-Use case 2: AI-based LOS/NLOS identification can be used to estimate the LOS/NLOS probability of the different channel taps in the CIR. This can serve as input into a traditional Time-of-Arrival (ToA), Time-Difference of Arrival (TDoA), Angle of Arrival (AoA) or Angle of Departure (AoD) positioning scheme to enable identification of the LOS tap and as such, more accurate estimation of the UE position.

Figure 2: Use Case 2 - AI-based LOS/NLOS identification

Specification Impact: The following are possible specification impacts in this use case:

- Assistance information on the LOS/NLOS probability of the CIR taps.

- Channel measurement information for inference

- Channel measurement information for training

Sub-Use case 3: A direct TOA/AoA/AoD estimate may be made by the neural network from the CIR to enable direct estimation of the TOA/TDoA/AoA/AoD for input into traditional positioning schemes  such as UL/DL-DTOA, AoD or AoD positioning

Figure 3: Use Case 3 - AI-based TOA Estimation

Specification Impact: The following are possible specification impacts in this use case:

- Possible signaling of the TOA rather than the TDoA.

- Channel measurement information for inference

- Measurement information for training

Proposal 1: The following specification impacts can be seen in the use cases under consideration:

- Direct AI/ML based positioning model

  - Sub-Use case 1: CIR / PDP/L1-RSRP input to UE position output

    - Potential spec impact:

      - Channel measurement information for multiple gNBs for training

      - Channel measurement information for multiple gNBs for inference

      - Calibration input and procedures to validate the AI model

      - Ground truth label assistance information to the inference device

- AI-assisted positioning with output of AI model serving as input to traditional positioning

  - Sub-Use case 2: LOS/NLOS tap identification for input to traditional positioning

    - Potential spec impact:

      - indication of LOS/NLOS probability. This may already be supported in Rel-17

      - Channel measurement information for inference

      - Channel measurement information for training

  - Sub-Use case 3: TOA/AoA/AoD estimation for input into TDOA-based, AoA-based or AoD-based positioning

    - Potential spec impact:

      - Possible signaling of the TOA rather than the TDoA to LMF

      - Channel measurement information for inference

      - Channel measurement information for training

## Training and Inference Specification Impact

In RAN1 #109-e, the following agreement was made [3]:

From May 18th GTW session Agreement Study further on sub use cases and potential specification impact of AI/ML for positioning accuracy enhancement considering various identified collaboration levels. - Companies are encouraged to identify positioning specific aspects on collaboration levels if any in agenda 9.2.4.2. - Note1: terminology, notation and common framework of Network-UE collaboration levels are to be discussed in agenda 9.2.1 and expected to be applicable to AI/ML for positioning accuracy enhancement. - Note2: not every collaboration level may be applicable to an AI/ML approach for a sub use case

In RAN1 #110, the following agreement was made [4]:

Agreement Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least - UE-side or Network-side training - UE-side or Network-side inference - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches Note: companies are encouraged to clarify aspects of their proposed AI/ML approaches for positioning when AI/ML model training and inference are not performed at the same entity

A  one-sided AI-model with model inference at either the UE-side (UE-based positioning) or at the network side (LMF based/UE assisted positioning or LMF based/NG-RAN assisted positioning) is sufficient for AI/ML based positioning.  Note that the training can take place at either side with the resulting model used at the side at which it is trained or  transferred/deployed to/at the other side. Ideally, training and inference should occur at one side, (either the gNB/LMF or at the UE). This eliminates the need for model transfer/deployment between the UE and Network or vice-versa.

Training and inference at the UE (UE-side training + UE-side inference) allows the UE to keep its privacy while training and inference at the LMF (Network side training + Network side inference) allows for easy sourcing of the  training data from multiple UEs in the network. Ideally, RAN1 should support both options.

Proposal 2: For both direct AI/ML positioning and AI/ML assisted positioning, a one-sided AI model is sufficient with training and inference occurring at either the gNB/LMF or at the UE. This eliminates the need for model transfer between the UE and Network or vice-versa.

Proposal 3: RAN1 should support the following options:

- Inference at the UE

- Inference at the LMF

Training can occur at either side with model deployment/transfer enabled only if necessary

## Specification impact

The different use cases discussed require different types of input ranging from the CIR to the L1-RSRP. These inputs need to be acquired by sending or receiving signals (e.g. PRS, SRS, CSI-RS) from  or to the  different candidate positioning devices (e.g. gNBs) to or from the target UE depending on if the inference occurs at the gNB or UE.

In RAN1 #109-e, the following agreement was made [3]:

Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection - AI/ML model indication/configuration - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection) - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output - UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring) - Other aspects are not precluded - Note: not all aspects may apply to an AI/ML approach in a sub use case - Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

In RAN1 #110, the following agreement were made [4]:

Agreement Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded [KO: Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded

The following table discusses each of these elements in some detail:

| AI/ML model training | - training data type/size: Given the current sub-use cases selected, RAN1 should allow for flexibility in the data type needed e.g for direct AI/ML based positioning, the CIR, L1-RSRP and PDP can be used. - training data source determination (e.g. UE/PRU/TRP): For online training, this depends on if the training/inference is at the UE or at the LMF/gNB. It may also depend on beam correspondence i.e. if channel is reciprocal and model is at the LMF/gNB. In the case of beam correspondence, the channel estimates at the gNB based on DMRS/SRS may be sufficient otherwise some feedback may be needed. - assistance signalling and procedure for training data collection : This depends on if the training/inference is at the UE or at the LMF/gNB. It may need LPP/NRPPa based signaling to trigger feedback of training data to the training device. In addition, some assistance information may be needed for noisy ground truth labels. |
| --- | --- |
| AI/ML model indication/configuration | Model Indication: Signaling is needed to enable selection of AI/ML model to configure AI/ML model to be used e.g. to indicate training data needed from UE, or actual sub-use cases to be trained. Even for a specific sub-use case, multiple AI models may be trained for a a specific parameter value or value range(for example based on Doppler) and then the appropriate model is indicated based on the specifics of the UE whose position is to be estimated. Model Configuration: A discussion may be needed on how to configure the one or more AI models at the interference device and the associated configuration information needed in the network to provide training data. |
| AI/ML model monitoring and update | Monitoring : The traditional location services, PRUs or GPS-based location may be used to calibrate the AI-based location and vice versa. As an example, the calibration location [X,Y] can be based on (a) any of the RAT-independent techniques (e.g. GNSS) or (b) RAT-dependent techniques (e.g. TDOA) defined in 3GPP Rel-16 and Rel-17. Update: The calibration error may serve as input into AI model update rate decision and if the calibration error > calibration error threshold for time > calibration time duration, then an AI model update can be requested. The update signaling may have specification impact. |
| AI/ML model inference input | Inference Input: model input acquisition and pre-processing will depend on if the AI model is UE based, network based and on beam correspondence. In a simple example, for the direct method with training and inference at the gNB and beam correspondence, the LMF may indicate SRS configurations for the gNBs to transmit to the UE and request feedback of the channel measurements from the gNBs at the appropriate time. |
| UE capability for AI/ML model(s) | Capability Signaling: This is necessary for the UE to indicate its capability for AI-based positioning. It can include (a) the type of AI positioning it supports i.e. direct AI/ML and AI/ML assisted positioning (and the associated feedback) (b) whether it supports UE-based AI inferencing and training or not (c) the location scenario e.g. high Doppler, high NLOS. |
| UE-side or Network-side training UE-side or Network-side inference | Signaling and capability support: If both are supported by the specification, capability signaling may be needed to help the network/LMF identify the UE preference and to configure the UE for the appropriate location and associated procedure for the training and inference. |

# Conclusion

In this contribution, we provided our views on use cases and potential specification impacts on the  enhancement on AI/ML for positioning accuracy enhancement. Based on the discussion, we have the following proposals:

Proposal 1: The following specification impacts can be seen in the use cases under consideration:

- Direct AI/ML based positioning model

  - Sub-Use case 1: CIR / PDP/L1-RSRP input to UE position output

    - Potential spec impact:

      - Channel measurement information for multiple gNBs for training

      - Channel measurement information for multiple gNBs for inference

      - Calibration input and procedures to validate the AI model

      - Ground truth label assistance information to the inference device

- AI-assisted positioning with output of AI model serving as input to traditional positioning

  - Sub-Use case 2: LOS/NLOS tap identification for input to traditional positioning

    - Potential spec impact:

      - indication of LOS/NLOS probability. This may already be supported in Rel-17

      - Channel measurement information for inference

      - Channel measurement information for training

  - Sub-Use case 3: TOA/AoA/AoD estimation for input into TDOA-based, AoA-based or AoD-based positioning

    - Potential spec impact:

      - Possible signaling of the TOA rather than the TDoA to LMF

      - Channel measurement information for inference

      - Channel measurement information for training

Proposal 2: For both direct AI/ML positioning and AI/ML assisted positioning, a one-sided AI model is sufficient with training and inference occurring at either the gNB/LMF or at the UE. This eliminates the need for model transfer between the UE and Network or vice-versa.

Proposal 3: RAN1 should support the following options:

- Inference at the UE

- Inference at the LMF

Training can occur at either side with model deployment/transfer enabled only if necessary

Proposal 4: The following table discusses the specification impact of different elements of the AI-based positioning:

| AI/ML model training | - training data type/size: Given the current sub-use cases selected, RAN1 should allow for flexibility in the data type needed e.g for direct AI/ML based positioning, the CIR, L1-RSRP and PDP can be used. - training data source determination (e.g. UE/PRU/TRP): For online training, this depends on if the training/inference is at the UE or at the LMF/gNB. It may also depend on beam correspondence i.e. if channel is reciprocal and model is at the LMF/gNB. In the case of beam correspondence, the channel estimates at the gNB based on DMRS/SRS may be sufficient otherwise some feedback may be needed. - assistance signalling and procedure for training data collection : This depends on if the training/inference is at the UE or at the LMF/gNB. It may need LPP/NRPPa based signaling to trigger feedback of training data to the training device. In addition, some assistance information may be needed for noisy ground truth labels. |
| --- | --- |
| AI/ML model indication/configuration | Model Indication: Signaling is needed to enable selection of AI/ML model to configure AI/ML model to be used e.g. to indicate training data needed from UE, or actual sub-use cases to be trained. Even for a specific sub-use case, multiple AI models may be trained for a a specific parameter value or value range(for example based on Doppler) and then the appropriate model is indicated based on the specifics of the UE whose position is to be estimated. Model Configuration: A discussion may be needed on how to configure the one or more AI models at the interference device and the associated configuration information needed in the network to provide training data. |
| AI/ML model monitoring and update | Monitoring : The traditional location services, PRUs or GPS-based location may be used to calibrate the AI-based location and vice versa. As an example, the calibration location [X,Y] can be based on (a) any of the RAT-independent techniques (e.g. GNSS) or (b) RAT-dependent techniques (e.g. TDOA) defined in 3GPP Rel-16 and Rel-17. Update: The calibration error may serve as input into AI model update rate decision and if the calibration error > calibration error threshold for time > calibration time duration, then an AI model update can be requested. The update signaling may have specification impact. |
| AI/ML model inference input | Inference Input: model input acquisition and pre-processing will depend on if the AI model is UE based, network based and on beam correspondence. In a simple example, for the direct method with training and inference at the gNB and beam correspondence, the LMF may indicate SRS configurations for the gNBs to transmit to the UE and request feedback of the channel measurements from the gNBs at the appropriate time. |
| UE capability for AI/ML model(s) | Capability Signaling: This is necessary for the UE to indicate its capability for AI-based positioning. It can include (a) the type of AI positioning it supports i.e. direct AI/ML and AI/ML assisted positioning (and the associated feedback) (b) whether it supports UE-based AI inferencing and training or not (c) the location scenario e.g. high Doppler, high NLOS. |
| UE-side or Network-side training UE-side or Network-side inference | Signaling and capability support: If both are supported by the specification, capability signaling may be needed to help the network/LMF identify the UE preference and to configure the UE for the appropriate location and associated procedure for the training and inference. |

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R1-2209580, Evaluation on AI/ML for positioning accuracy enhancement, Apple, RAN1 #109-e, October 2022

- Chairman’s Notes RAN1 #109-e, May 2022

- Chairman’s Notes RAN1 #110, August 2022