3GPP TSG RAN WG1 #110bis-e		                                       R1-2209653
e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.7.1

Source:	InterDigital, Inc.

Title:	Performance evaluation for network energy saving

Document for:	Discussion and Decision

# Introduction

RAN approved a new study item on Network Energy Saving [1]. The aim and justification of the study item is to reduce operational cost and greenhouse emissions associated with operating 5G base stations, while balancing the impact on network and user performance and limiting impact to legacy UEs. KPIs such as spectral efficiency, capacity, user perceived throughput, latency, UE power consumption, handover performance, call drop rate, and initial access performance must be considered. Network energy savings techniques to be studied should thus avoid having a large impact to such KPIs.

In RAN1#109-e, RAN1 approved a TR skeleton for network energy savings [2] and made agreements with regards to energy consumption modeling for BS, deployment scenarios, #TXRUs, KPIs etc. for performance evaluation modeling. Relevant agreements are summarized in Annex B.

This contribution provides a discussion on the gNB energy consumption model and modes, a discussion on the simulation methodology, and evaluation results for the prioritized Urban macro scenario for FR1.

# gNB energy consumption model

A preliminary model for gNB power consumption can be based on the model developed for E-UTRA in [3] and customized for NR gNBs. Power consumption for a transceiver can be broken down to [4]:

- Power amplifier: 57%: an antenna interface, a power amplifier.

- RF: 6%: a radio frequency small-signal transceiver section.

- BB: 13%: a baseband interface including a receiver (uplink) and transmitter (downlink) section.

- DC-DC: 6%: a DC-DC power supply.

- Cooling: 10%: an active cooling system and an AC-DC unit for connection to the electrical power grid.

- Main Supply: 8%

When the PA turns off, cooling systems, main supply and DC-DC converters scale with the power consumption and the remaining do not. The gNB can be either fully on, in micro-sleep where the transceiver is awake only for a portion of slots in a radio frame, in deep sleep where the transceiver is assumed to be off for much longer periodicities, or the PA can be completely shut off for best energy savings results. A gNB energy consumption model should therefore take those power saving states into account.

The relation between relative RF output power Pout and BS power consumption Pin is nearly linear, as illustrated in Figure 1 below. The following linear approximation of the downlink power model can be assumed for a given network transceiver:

PinNES state= Relative PowerNES state . PActive DL;

PActive DL= P0+ΔpPout  (1)

Pout: the load dependent RF output power. 
This can be approximated as Pout=cell loadslot . Pmax  ;  0<Pout≤Pmax , where Pmax is the maximum RF output power at maximum load.

P0: The linear model parameter to represent power consumption at the zero RF output power.

∆p: The slope of the cell load dependent power consumption.

Relative PowerNES state: gNB power consumption per sleep mode relative to the ON state, which can be calculated based on the relative power (P) as part of the working assumptions from the last RAN meeting agreements. We utilize the gNB power consumption per sleep mode scaling based on the agreed Cat-2 parameters. We consider an additional “WUS-only reception” state with WUS reception to account for the wake-up scheme, with a power level that is larger than the “light sleep” state and less than “Active UL”, which involves full blind decoding. The relative power levels are provided in Table 1 below. Based on these parameters, we use a power scaling of 2.1/32 for “light sleep” and 2.5/32 for “WUS-only reception”. For our evaluations, we consider gNB sleep durations of up to 160ms, and treat this as light sleep.

Table 1: Relative power scaling, using agreed Cat-2 parameters

| NES state | Power level, relative units |
| --- | --- |
| Active transmission (100% PRB) | 32 |
| Active reception (100% PRB) | 6.5 |
| WUS-only reception | 2.5 |
| “Light”-sleep | 2.1 |
| “Deep”-sleep | 1 |

Figure 1: illustration of the gNB power model for 1 transceiver

The resultant energy consumed by the gNB in a given power savings state (on, off, or sleep) can therefore be:

EinPowSavingState=PinPowSavingState×TimePowSavingState 		(2)

Proposal 1: 	A cell-load dependent gNB energy consumption model is used for the study. gNB power consumption for sleep modes is scaled using power consumption relative to the ON state.

Proposal 2: 	A power state for low power uplink reception (e.g. for WUS) can be added in the power state relative powers table, where the relative power for such state is between "light sleep" and full "Active UL".

# Evaluation methodology

We consider an Urban macro deployment (which has been prioritized for FR1.) Details regarding the simulation assumptions can be found in the Appendix. We consider and evaluate downlink and uplink traffic scenarios separately.

For downlink traffic, we consider activation and deactivation of macro cells based on traffic observations and load of neighboring cells. Deactivation of cells is done based on the load of neighboring cells, while activation/and or offloading of cells is done based on the load of the current cell increasing above a critical threshold. Cell load is measured by resource utilization (RU) (ρ), defined as the number of available (frequency) resources utilized over total number of resources available. A high RU threshold (ρh) is used to indicate the critical value at which a cell is considered to be overloaded and needs to offload some traffic/activate a neighboring cell. A low RU threshold (ρl) is used to indicate the minimum RU below which a cell can be de-activated (as long as a neighboring cell can accommodate this cell’s load.) RU measured over a time window is used for making decisions regarding cell activation/deactivation and off loading. Utilizing a time average imparts hysteresis, preventing cells from switching states too frequently. For the purpose of these evaluations, we consider cells being in one of two states – “ON” or “PA OFF” as provided in Eqn. (1). We consider the following power model parameters as provided in [4] as a reference with the following parameters configured: Pmax = 79.43 W (based on agreed assumptions for Set 2 FR1), NTRX=1 (normalized), Δp= 4.7 [4]. The energy consumed for the “ON” or “PA OFF” state is calculated using Eqn. (2), where PinPowSavingsState corresponds to the Pin relating to the “ON” or “PA OFF” equations from Eqn. (1), where Pout is calculated based on cell load as measured by instantaneous RU for the cell: Pout=RUinst*Pc,max.

For uplink traffic, we consider a network energy savings scheme where the macro cell will enter a sleep state if cell load falls below a low threshold. Similar to the downlink case, cell load is measured by RU (ρ). A low RU (ρl) is used to indicate the minimum RU below which a cell may go to sleep for a certain sleep duration, before waking up. A UE that is served by a sleeping cell would have to wait for the gNB to wake up (i.e., ON occasion). We consider an enhancement to this scheme, where the UE may send a wake-up request, the timing of which may be based on or associated with traffic arrival occasions. For power consumption, we consider the same model as above, with the following additional aspects for uplink. Uplink power in the ON (active reception) state is estimated as   6.532*PinON, DL, based on relative power level scaling for active reception vs. active transmission from Table 1, with the uplink cell load (RU) is used instead of DL load in a given slot. Uplink power in the (light) sleep state is estimated as 2.132*PinON, DL, whereas uplink power consumption in the WUS-only reception state is estimated as as 2.532*PinON, DL.

# Evaluation results

Downlink

For the following downlink evaluations, we utilize a low load RU threshold (ρl) of 10%, and a higher load RU threshold (ρh) of 40%. A cell for which the RU falls below the low load threshold is considered for deactivation (based on the load of neighboring cells). On the other hand, a cell whose RU goes above the high load threshold can consider activating neighboring cells for offloading. The time window for RU averaging is 20ms. We simulate two different user loading scenarios: user traffic load 1 offers lower combined load, while user traffic load 2 offers a higher combined load. The evaluation results are provided in Table 2. In the table, UPT and energy saving gain (ESG) are compared for the proposed Network Energy Saving (NES) scheme and the baseline.

Table 2: Downlink Evaluation results

| User traffic load 1: {λ1, λ2} = {0.14, 0.33} |  |  |  | User traffic load 2: {λ1, λ2} = {0.33, 0.67} |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) Mean UPT(Mbps) | NES Mean UPT (Mbps) | UPT gain/loss (NES vs Baseline) (%) | ESG (NES vs Baseline) (%) | Baseline (Always ON) Mean UPT (Mbps) | NES Mean UPT (Mbps) | UPT gain/loss (NES vs Baseline) (%) | ESG (NES vs Baseline) (%) |
| 47.51 | 45.27 | -4.93% | 54.59% | 23.81 | 22.67 | -4.78% | 25.65% |

The results demonstrate that significant energy savings gain can be achieved while ensuring nearly comparable user level QoS (throughput). We can see that the achievable energy savings gain is higher when overall user traffic load on system is lower; the ESG is 54.59% with user traffic load 1, and with user traffic load 2 it is 25.65%. This is a result of the fact that lower user traffic loads allow for greater opportunities for cell de-activation, and hence greater energy savings gains.

Observation 1: Considerable energy savings gain can be achieved for low-medium cell loads while ensuring nearly comparable user level QoS (throughput).

Uplink

For the following uplink evaluations, we utilize a low load RU threshold (ρl) of 10% and 60% to evaluate the impact of different gNB sleep thresholds on system performance. A cell for which the RU falls below the low load threshold is a candidate for entering a sleep state. We consider sleep durations of 40ms and 160ms for the gNB. In the baseline scheme, a UE that is served by a sleeping gNB will have to wait for the gNB to transition out of the sleep state (to an ON state) before its uplink traffic needs can be served. We also consider an enhancement to this scheme, wherein the UE may send a wake-up request to the gNB instead of waiting for the gNB to wake up (at the end of its sleep cycle). This wake-up signal (WUS) may be sent periodically in certain uplink slots and can be based on traffic arrival patterns, allowing the UE with multiple opportunities to request the gNB to wake up. Our results compare the performance of these two schemes. As mentioned in Section 2, we consider sleep durations of up to 160ms, which we treat as light sleep.

Table 3: Uplink Evaluation results – 40 ms gNB sleep cycle

| User traffic load: λ = 0.24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (ρl)=0.10 |  |  |  | (ρl)=0.60 |  |  |  |  |  |
| NES (Sleep w/o WUS) Mean UPT (Mbps) | Enhanced NES (Sleep with WUS) Mean UPT (Mbps) | UPT gain/loss (Enhanced NES vs NES) (%) | ESG (Enhanced NES vs. NES) (%) | NES (Sleep w/o WUS) Mean UPT (Mbps) |  |  | Enhanced NES (Sleep with WUS) Mean UPT (Mbps) | UPT gain/loss (Enhanced NES vs. NES) (%) | ESG (Enhanced NES vs. NES) (%) |
| 21.35 | 23.77 | +11.34% | -3.00% |  |  | 21.92 | 22.92 | +4.56% | -3.12% |

| User traffic load: λ = 1.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (ρl)=0.10 |  |  |  | (ρl)=0.60 |  |  |  |  |  |
| NES (Sleep w/o WUS) Mean UPT (Mbps) | Enhanced NES (Sleep with WUS) Mean UPT (Mbps) | UPT gain/loss (Enhanced NES vs NES) (%) | ESG (Enhanced NES vs. NES) (%) | NES (Sleep w/o WUS) Mean UPT (Mbps) |  |  | Enhanced NES (Sleep with WUS) Mean UPT (Mbps) | UPT gain/loss (Enhanced NES vs. NES) (%) | ESG (Enhanced NES vs. NES) (%) |
| 23.03 | 24.11 | +4.70% | -1.67% |  |  | 22.20 | 22.90 | +3.15% | -1.82% |

Table 4: Uplink Evaluation results – 160 ms gNB sleep cycle

| User traffic load: λ = 0.24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (ρl)=0.10 |  |  |  | (ρl)=0.60 |  |  |  |  |  |
| NES (Sleep w/o WUS) Mean UPT (Mbps) | Enhanced NES (Sleep with WUS) Mean UPT (Mbps) | UPT gain/loss (Enhanced NES vs NES) (%) | ESG (Enhanced NES vs. NES) (%) | NES (Sleep w/o WUS) Mean UPT (Mbps) |  |  | Enhanced NES (Sleep with WUS) Mean UPT (Mbps) | UPT gain/loss (Enhanced NES vs. NES) (%) | ESG (Enhanced NES vs. NES) (%) |
| 17.51 | 23.84 | +36.14% | -5.95% |  |  | 17.37 | 23.07 | +32.78% | -6.20% |

| User traffic load: λ = 1.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (ρl)=0.10 |  |  |  | (ρl)=0.60 |  |  |  |  |  |
| NES (Sleep w/o WUS) Mean UPT (Mbps) | Enhanced NES (Sleep with WUS) Mean UPT (Mbps) | UPT gain/loss (Enhanced NES vs NES) (%) | ESG (Enhanced NES vs. NES) (%) | NES (Sleep w/o WUS) Mean UPT (Mbps) |  |  | Enhanced NES (Sleep with WUS) Mean UPT (Mbps) | UPT gain/loss (Enhanced NES vs. NES) (%) | ESG (Enhanced NES vs. NES) (%) |
| 19.76 | 24.27 | +22.83% | -3.03% |  |  | 18.83 | 23.56 | +25.14% | -3.08% |

Tables 3 and 4 present results for 40ms and 160 ms sleep durations, respectively. We utilize two different user traffic arrival rates and low load RU thresholds (ρl) for each sleep cycle duration, allowing us to investigate the impact of varying sleep cycles as well as varying cell load and gNB sleep thresholds on system performance. The results demonstrate that utilizing a WUS can improve User throughput and latency, even for the shorter sleep cycle of 40ms, where the improvement offered by WUS is greater at lower user traffic loads vs. higher user traffic loads (11.3% vs. 4.7%). This comes with a marginal increase in energy consumption (of no more than 3%). The benefit of WUS scheme is much more pronounced for the longer sleep (160ms) duration, where UPT gains of 36% and 25% are achieved with WUS with an increase in in energy consumption of 6% and 3% for low and high user traffic loads, respectively.

Observation 2: Utilizing a wake-up signal can significantly improve user level QoS (throughput). This benefit can be achieved while having marginal impact on gNB side energy consumption.

# Conclusion.

This contribution has provided initial evaluation results for network energy savings and has proposed the following:

Proposal 1: 	A cell-load dependent gNB energy consumption model is used for the study. gNB power consumption for sleep modes is scaled using power consumption relative to the ON state.

Proposal 2: 	A power state for low power uplink reception (e.g. for WUS) can be added in the power state relative powers table, where the relative power for such state is between "light sleep" and full "Active UL".

Observation 1: Considerable energy savings gain can be achieved for low-medium cell loads while ensuring nearly comparable user level QoS (throughput).

Observation 2: Utilizing a wake-up signal can significantly improve user level QoS (throughput). This benefit can be achieved while having marginal impact on gNB side energy consumption.

# Appendix A

Table 3: Urban Macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| System bandwidth | 20 MHz |
| Layout | Single layer: Hex. Grid 7 site, 3 sectors per site |
| ISD | 500m |
| Duplexing | FDD |
| Numerology | 15 KHz, 14 OFDM symbol slot |
| BS antenna config. | 32 Tx/Rx RUs - (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna config | 4 Tx/Rx RUs - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| BS Tx Power | 49dBm |
| UE Tx Power | 23 dBm |
| User distribution and UE speed | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h DL: 10 users per TRxP UL: 2 users per TRxP |
| Traffic model | Use 36.889 Table A.1.1. Downlink Two packet arrival rates considered. One for a low user traffic load (𝜆1), another for a medium/higher user traffic load (𝜆2). Half the UEs are assigned 𝜆1, the other half 𝜆2. The user traffic loads are re-assigned every 20% of the simulation duration, to randomize arrival rates across the network. For the PSG, we count ON and OFF state for the micro cell and utilize Eqn. (1) for energy consumption Uplink: We consider a uniform packet arrival rate across all UEs (and cells). We use two user traffic load values (low and medium), to investigate impact of different user traffic loads on the system. |

# Appendix B

RAN1-110 Agreements For evaluation and BS energy consumption modeling purpose, for single CC case, at least the following in table should be considered for reference configuration - Note: other TX-RX RU number and corresponding BS antenna configuration can be considered in SLS assumptions Set 1 FR1; Set 2 FR1; Set 3 FR2. Duplex; TDD; FDD; TDD. System BW; 100 MHz; 20 MHz; 100 MHz. SCS; 30 kHz; 15 kHz; 120 kHz. Number of TRP; 1; 1; 1. Total number of DL TX RUs; 64; 32; 2. Total DL power level; 55dBm; 49 dBm; 33 dBm EIRP limited to 63 dBm. Total number of UL Rx RUs; 64; 32; 2. Agreement For the purpose of evaluation, adopt the following as BS power consumption model. These entries for this table is per reference configuration set. - FFS: One or multiple values for relative power and transition time. Power state; Characteristic; Relative Power; Additional transition energy3; Total transition time. Deep sleep1; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state.; P1=1; E1; T1. Light sleep; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. (P2>P1); P2; E2; T2. Micro sleep; There is neither DL transmission nor UL reception. Immediate transition is assumed for network energy saving study purpose from or to a non-sleep state.; P3; 0; 0. Active DL; There is only DL transmission.; P4; NA; NA. Active UL; There is only UL reception. FFS: Whether multiple P5 values are needed to address low power UL mode; P5; NA; NA. Note 1: Depending on implementations, there could be a state that the power is lower than deep sleep and requires larger total transition time, e.g. hibernating sleep or Quasi-off, which is not explicitly modeled in this study for evaluation purpose. Note 3: Unit in relative power times duration. FFS: Details on how transition energy is defined.. - For simultaneous DL and UL transmission for FDD, the power for UL reception is neglected in this study. - FFS: Optionally, a state machine where BS may transit between sleep modes without entering non-sleep mode can be considered. Companies are to report the involved sleep modes and the assumptions for inter-sleep mode transition time used in their evaluations. - FFS: Details on how to use the above table for low power uplink reception (e.g. for WUS). Working Assumption For reference configuration set 1, the values are provided as below. FFS set2 and set 3. Power state; Relative Power P; Total transition time T. Deep sleep; 1; 1; Cat 1: 50ms; Cat 2: 10s. Light sleep; Cat 1: 25; Cat 2: 2.1; Cat 1: 6 ms; Cat 2: 640 ms. Micro sleep; Cat1: 55; Cat 2: 5.5; 0; 0. Active DL; Cat 1: 280; Cat 2: 32; N.A.; N.A.. Active UL; Cat 1: 110; Cat 2: 6.5; N.A.; N.A.. Agreement For evaluation purpose, - a load (L) of a cell is a percentage of resources used for UE specific PDSCH / PUSCH - The following load scenarios are considered Load scenario; Characteristics. Idle/empty load; - Include cell-specific signals and channels, and - L = 0. low load; - Include cell-specific signals and channels, and - 0 < L≤15. Light load; - Include cell-specific signals and channels, and - 0 < L≤ [30]. Medium load; - Include cell-specific signals and channels, and - [30] < L≤ [50]. For CA, the companies report whether the load is defined per CC or across all CCs.. RAN1-109 Agreements Agreement For evaluation purpose, the energy consumption modeling for a BS includes at least the following: - Reference configuration - FFS other details - Note FR1 and FR2 to be separately considered for detailed parameters - Multiple power state(s) including sleep/non-sleep mode(s) with relative power, and associated transition time/energy - Scaling method to be applied at least for non-sleep mode. - FFS other details including scaling for sleep mode For evaluation purpose, the BS energy consumption model should at least include the power consumption of BS on slot-level. - Note that symbol-level power consumption to reflect different BW (or RB utilization) / time-occupancy / tx-rx direction of different symbols in a slot is considered. - FFS details (e.g. explicit symbol-level power modelling, scaling slot-level power to symbol level power for various cases, etc.) - Note: system simulation evaluations can be per slot regardless of detailed approach for calculating symbol-level power consumption. For BS energy consumption evaluation, in addition to the energy saving gain, - At least UPT/UE power consumption/access delay/latency should be considered for performance impact evaluation - Note: this doesn’t necessarily mean that all the above are considered for all evaluation results. However, multiple KPIs are expected to be evaluated for a given technique. And this does not preclude to consider other KPIs when found appropriate for certain techniques/scenarios. At least urban macro is prioritized for FR1. FFS the baseline deployment assumption for FR2. As a starting point, - macro cell BS for FR1 is assumed for energy consumption model. - FFS: micro cell BS for FR2 is assumed for energy consumption model.

# References

RP-213554, “Study on network energy savings for NR,” Huawei.

- [2]   TR 38.864, “Study on network energy savings for NR”, v0.0.2

- [3]   TS 36.887 "Study on energy saving enhancement for E-UTRAN"

- [4]	https://cordis.europa.eu/docs/projects/cnect/3/247733/080/deliverables/001-EARTHWP2D23v2.pdf