3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209844

e-Meeting, October 10 - 19, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on LS on Per-FS L1 feature for NR sidelink discovery BC-list

Document for:	Discussion and Decision

- Introduction

A RAN2 LS was sent to RAN1 [1], with the following contents:

For R17 WID NR_SL_Relay-Core, RAN2 agreed to introduce UE capability of supported band combination list for NR sidelink relay-based discovery and NR sidelink non-relay-based discovery.

| supportedBandCombListPerBC-SL-RelayDiscovery-r17, supportedBandCombListPerBC-SL-NonRelayDiscovery-r17 Indicates, for a particular Uu band combination, the PC5 Relay discovery and non-Relay discovery band combination(s) on which the UE supports simultaneous transmission and reception. The leading / leftmost bit (bit 0) corresponds to the first band combination included in supportedBandCombinationListSL-RelayDiscovery-r17/supportedBandCombinationListSL-NonRelayDiscovery-r17, the next bit corresponds to the second band combination included in supportedBandCombinationListSL-RelayDiscovery-r17/supportedBandCombinationListSL-NonRelayDiscovery-r17 and so on. with value 1 indicating simultaneous transmission and reception is supported. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Considering for NR sidelink, discovery message is carried by PSSCH, in the same way as for communication, RAN2 assumed the following Rel-16 per-FS L1 features

| 15-25 | Transmitting NR sidelink mode 1 scheduled by NR Uu on a different carrier | 1) UE can monitor DCI format 3_0 on a different carrier from sidelink for NR sidelink dynamic scheduling and configured grant type 2 |
| --- | --- | --- |

and the following Rel-17 per-FS L1 features also apply to band combinations supporting discovery.

| 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu |
| --- | --- | --- |
| 32-5a-1 | Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set. |

RAN2 respectfully requests RAN1 to provide answer to the following questions:

Q1: Can RAN1 confirm the RAN2 assumption above, i.e., these per-FS L1 features above also apply to the band-combination supporting discovery?

Q2: If Yes to Q1, in case a band combination supporting both NR Sidelink communication and NR Sidelink discovery, whether the UE capability of these per-FS L1 feature would be different for NR Sidelink communication and NR sidelink discovery, or it would be the same?

RAN1 was respectfully asked to confirm the above RAN2’s understanding, and provide other feedback (if any concern). Thus we will discussion possible answer to RAN2 question.

- Discussions

For L2 and L3 relay discovery, the network may configure a resource pool dedicated for relay discovery. In such case, the resource pools for SL relay discovery and SL communication are separated, and thus the operation on those resource pools are physically isolated. From RAN1’s perspective, these per-FS L1 features above also apply to the band-combination supporting discovery and the UE capability of these per-FS L1 feature would be the same for SL communication and SL relay discovery. Thus, following RAN1 answer is proposed:

Proposal 1:  Reply to RAN2 as follows:

- RAN1 confirms that these per-FS L1 features also apply to the band-combination supporting discovery.

- The UE capability of these per-FS L1 feature would be the same for NR Sidelink communication and NR sidelink discovery

- Conclusions

It is confirmed that these per-FS L1 features above also apply to the band-combination supporting discovery by configuring a dedicated resource pool for relay discovery, and the UE capability of these per-FS L1 feature would be the same for NR Sidelink communication and NR sidelink discovery.

Proposal 1:  Reply to RAN2 as follows:

- RAN1 confirms that these per-FS L1 features also apply to the band-combination supporting discovery.

- The UE capability of these per-FS L1 feature would be the same for NR Sidelink communication and NR sidelink discovery

# References

- R2-2208867, “LS on Per-FS L1 feature for NR sidelink discovery BC-list”, RAN2, RAN2#119-e, August, 2022.