3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209885

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | Draft | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on multiplexing NACK-only based feedback with SR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Behavior when a PUCCH with NACK-only based HARQ-ACK for multicast and a PUCCH with SR overlap is not defined. When a PUCCH format 0 with unicast HARQ-ACK overlaps with a PUCCH with positive SR, the SR and the HARQ-ACK are multiplexed and transmitted in the PUCCH format 0 using a different cyclic shift value. If the rule is directly applied to a PUCCH resource for NACK-only based feedback, gNB will not be able to identify which UE sent the SR since PUCCH resources for NACK-only based feedback will be shared among UEs in a group. This makes no sense to send SR, so in that case it is appropriate to drop SR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When PUCCH format 0 for NACK-only based feedback and SR overlap, the UE drops the SR. Otherwise, the same rules as for ACK/NACK based feedback are applied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Behavior when a PUCCH with NACK-only based HARQ-ACK and a PUCCH with SR overlap is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

18	Multicast Broadcast Services

*** Unchanged text is omitted ***

If a UE would multiplex multicast HARQ-ACK information according to the second HARQ-ACK reporting mode with multicast HARQ-ACK information according to the first HARQ-ACK reporting mode, or unicast HARQ-ACK information, SR, or CSI reports in a first PUCCH or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH when the UE is provided moreThanOneNackOnlyMode, the UE considers that the UE would transmit the second PUCCH using any PUCCH resource from the PUCCH resources associated with the second HARQ-ACK reporting mode when all values of the HARQ-ACK information are 'ACK'. If a UE would transmit a PUCCH with SR and a HARQ-ACK information bit according to the second HARQ-ACK reporting mode in a resource using PUCCH format 0 and they overlap in time, the UE transmits only the HARQ-ACK information. Otherwise, if a UE would transmit a PUCCH with SR and a HARQ-ACK information bit according to the second HARQ-ACK reporting mode and they overlap in time, the UE determines a PUCCH resource as described in clause 9.2.5.1.

*** Unchanged text is omitted ***