3GPP TSG RAN WG1 Meeting # 110bis-e	 R1-2210149

# E-meeting, October 10th - 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] Handling of PUCCH with high-priority HARQ-ACK and HP SR overlapping with a PUCCH with low-priority HARQ-ACK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture, based on the RAN1#110 agreement copied below, the handling for the scenario where a PUCCH with high-priority HARQ-ACK and HP SR overlaps with a PUCCH with low-priority HARQ-ACK and where the PUCCH carrying HP HARQ-ACK and HP SR is with format 0/1. Agreement When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK, for HP SR with F1 and HP HARQ-ACK with F0/F1, and HP SR with F0 and HP HARQ-ACK with F0 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs , K>=1 - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, LP HARQ-ACK is dropped when colliding with a PUCCH with HP HARQ-ACK and positive SR. - No specification change is needed.. In addition, the above agreement doesn’t seem to be providing complete handling for the scenario where a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK, for HP SR with F1 and HP HARQ-ACK with F0/F1, and HP SR with F0 and HP HARQ-ACK with F0 overlaps with a PUCCH carrying LP HARQ-ACK. Specifically, in case of HP negative SR, it’s not clear how the UE should handle the overlap. |
| Summary of change: |  | Although the agreement states that ‘no specification change is needed’, capture the behaviour of dropping of the LP HARQ-ACK in the above scenario as this is missing in the current specifications. In addition, capture the following handling in the specifications: When a PUCCH carrying HP negative SR and HP HARQ-ACK, for HP HARQ-ACK with F0, overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs, where K>=1 - HP negative SR is dropped. Note that the case where HP HARQ-ACK is with F1 is not covered above given that HP negative SR is not really multiplexed with HP HARQ-ACK in this case, since (i) if SR is with F0, SR is dropped, and (ii) if SR is with F1, HARQ-ACK is transmitted on the HARQ-ACK resource when SR is negative. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specs as the behaviour of dropping of the LP HARQ-ACK is not captured in the current specifications, for the scenario where a PUCCH with high-priority HARQ-ACK and HP SR overlaps with a PUCCH with low-priority HARQ-ACK and where the PUCCH carrying HP HARQ-ACK and HP SR is with format 0/1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

#### 9.2.5.3	UE procedure for reporting UCI of different priorities

If a UE

-	is provided PUCCH-ConfigurationList for PUCCH transmissions with priority 0 and 1,

-	is provided uci-MuxWithDiffPrio, and

-	would transmit overlapping PUCCHs that include a first PUCCH with OACK,0 HARQ-ACK information bits of priority 0 and a second PUCCH with OACK,1 HARQ-ACK information bits of priority 1

-	if the PUCCH resource for the second PUCCH includes PUCCH format 2, 3, or 4 and additionally includes OSR,1 SR bits of priority 1, OACK,1 is replaced by OACK,1+OSR,1 where OSR,1 is determined according to clause 9.2.5.1

-	if the PUCCH resource for the second PUCCH includes PUCCH format 0 and additionally includes a negative SR, the negative SR is dropped

the UE

-	drops the first PUCCH with the OACK,0 HARQ-ACK information bits if PUCCH resource for the second PUCCH includes PUCCH format 0 or 1 and additionally includes a positive SR, or otherwise

-	determines

-	a PUCCH resource set from the second PUCCH-Config using OUCI=OACK,0+OACK,1 as described in clause 9.2.1, and a PUCCH resource from the PUCCH resource set as described in clause 9.2.3 where a DCI format, if any, triggers PUCCH transmission of priority 1, or

-	a PUCCH resource from the second sps-PUCCH-AN-List using OUCI=OACK,0+OACK,1 as described in clause 9.2.1, and

-	multiplexes the OACK,0 and OACK,1 HARQ-ACK information bits in a same PUCCH using the PUCCH resource.

If the PUCCH resource includes PUCCH format 2 or PUCCH format 3 and MRBPUCCH PRBs, the UE determines a number of MRB,minPUCCH≤MRBPUCCH PRBs for the PUCCH transmission to be the minimum number of PRBs that starts from the first PRB from the MRBPUCCH PRBs and results to

OACK,0+OCRC,0⋅r1+OACK,1+OCRC,1⋅r0≤MRB,minPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r0⋅r1

where OCRC,0 or OCRC,1 is a number of CRC bits, if any, for encoding the OACK,0 or the OACK,1 HARQ-ACK information bits, respectively, r0 is provided by maxCodeRateLP, and the remaining parameters are as defined in clause 9.2.5.2 with r1=r. For PUCCH format 3, if MRB,minPUCCH is not equal to 2α2⋅3α3⋅5α5 [4, TS 38.211], MRB,minPUCCH is increased to a nearest value that is equal to 2α2⋅3α3⋅5α5 and does not exceed nrofPRBs.

< Unchanged parts are omitted >