3GPP TSG RAN WG1 #110bis-e			R1-2210331

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.4.3

Source:	Moderator (Apple)

Title:	FL summary #2 for AI 9.4.3 Enhanced sidelink operation on FR2 licensed spectrum

Document for:	Discussion/Decision

# Introduction

The latest Rel-18 WID on sidelink evolution (RP-221938) includes the following objective regarding enhanced sidelink operation on FR2 licensed spectrum (SL-FR2):

- Study and specify enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] (Determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18) - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. - Work is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

This contribution provides discussions related to the evaluation methodology for commercial deployment scenario for SL-FR2, including summary of contributions, email discussions, outcome of this meeting, etc. The related email thread is as below:

[110bis-e-R18-SL-04] Email discussion on SL operation for FR2 by October 19 – Chunxuan (Apple)

- Check points: October 14, October 19

# Contact information

Since this meeting is conducted electronically, it has been requested to collect contact information of the delegate(s) who handle this topic from each company during this meeting. You are kindly requested to provide the following contact information.

| Company | Delegate name(s) | Email address(es) |
| --- | --- | --- |
| OPPO | Kevin Lin | kevin.lin@oppo.com |
| Intel | Kilian Roth Salvatore Talarico | kilian.roth@intel.com salvatore.talarico@intel.com |
| Nokia, NSB | Daniel Medina Torsten Wildschek | daniel.medina@nokia.com torsten.wildschek@nokia.com |
| JHU APL | Arnab Das | arnab.das@jhuapl.edu |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| Transsion | Xingya Shen | Xingya.shen@transsion.com |
| vivo | Huan Wang | wanghuan@vivo.com |
| CEWiT | Abhijeet A. Masal Sayyed Shafivulla | abhijeetmasal@cewit.org.in shafivulla@cewit.org.in |
| Qualcomm | Wooseok Nam | wnam@qti.qualcomm.com |
| Ericsson | Jose Leon Calvo | jose.leon.calvo@ericsson.com |
| MediaTek | Xuanbo Shao | Xuanbo.Shao@mediatek.com |
| Lenovo | Xin GUO Zhennian SUN | guoxin9@lenovo.com sunzn1@lenovo.com |

# Topics for email discussion

## Topic #1: Evaluation scenario

### Background

Scenarios

For the evaluation methodology of sidelink operation on FR2 licensed spectrum, multiple deployment scenarios are proposed by companies. They are listed below:

- Urban grid scenario as in TR37.885: vivo, CATT, CEWiT, Intel

- Highway scenario as in TR37.885: vivo, CATT, CEWiT, Intel

- Indoor-A scenario as in TR38.808: Nokia, vivo

- Indoor-B scenario as in TR38.808: Nokia

- Rel-18 SL-U indoor scenario (Option 1 without WiFi nodes) (similar to Indoor-E scenario as in TR38.808): vivo, OPPO, Lenovo, MediaTek, Interdigital, LG, Spreadtrum, Apple, Ericsson, Huawei

- Layout options 1, 3, 5 as in TR36.843: OPPO, Xiaomi, Samsung, LG, Spreadtrum, Apple

- Dense urban with 200m ISD as in TR38.802: Huawei

According to WID, the evaluation methodology for sidelink operation on FR2 licensed spectrum (SL-FR2) is to be updated for commercial deployment scenario. The urban grid and highway scenarios as in TR37.885 are defined for V2X deployment scenario. Since V2X deployment scenario is not in the scope of this objective, and hence it is the FL’s recommendation that these two deployment scenarios are not considered.

Indoor scenarios for SL-FR2 were proposed by many companies, where three different indoor scenarios were mentioned: indoor-A scenario in TR38.808, indoor-B scenario in TR38.808, and Rel-18 SL-U indoor scenario (Option 1 without WiFi nodes). Note that the last scenario is similar to Indoor-E scenario in TR38.808.

In Rel-18 NR sidelink, the evaluation methodology for SL-U was recently agreed for commercial uses cases. The related agreements are provided in Appendix 6.3. Among the three proposed scenarios, Rel-18 SL-U indoor scenario (Option 1 without WiFi nodes) got most supports. Hence, the Rel-18 SL-U indoor scenario (Option 1 without WiFi nodes) is suggested by FL in Proposal 1-1-a (next section). Without the WiFi nodes, the total number of SL UEs deployed in the layout will be increased, e.g., from 12 as for unicast traffic in SL-U. The actual number can be further discussed, as it may be related to the simulation bandwidth.

Additionally, FL would like to collect companies’ views on whether to reuse indoor-A or indoor-B scenario in TR38.808 in the evaluation methodology, as in Question 1-1.

Outdoor scenarios for SL-FR2 were also proposed by several companies, where several different outdoor scenarios were mentioned: layout options 1, 3 and 5 as in TR36.843 and Dense urban with 200m ISD as in TR38.802.

In Rel-17 NR sidelink, the sidelink evaluation methodology study includes the update of reference system deployments, channel models, UE antenna models, traffic models, and performance evaluation metrics for commercial use cases. The related agreements are collected in Appendix 6.1 and 6.2. Among the layout options 1, 3 and 5 as in TR36.843, since layout 3 was considered as mandatory in Rel-17 NR sidelink for commercial use cases, while the other two layouts were considered as optional in Rel-17 NR sidelink, FL suggests focusing on layout option 3 as in TR36.843. Also, the dense urban with 200m ISD in TR38.802 was not agreed as sidelink layout, hence, it is suggested to treat it as lower priority than the layout option 3 as in TR36.843. Hence, layout option 3 in TR36.843 is suggested by FL in Proposal 1-2-a (next section).

Parameters:

Carrier frequency

Three carrier frequencies of SL-FR2 were proposed by companies:

- 30 GHz: vivo, ZTE, OPPO, Lenovo, LG, Huawei, Apple, Nokia, Ericsson

- 60 GHz: vivo, Nokia

- 63 GHz: ZTE, Lenovo, LG

The carrier frequency for SL-FR2 of 30 GHz and 63 GHz are used in TR37.885 and the carrier frequency for SL-FR2 of 30 GHz is used in TR38.885. Note that 63 GHz frequency band was initially introduced for ITS operation as in ETSI EN 302 686, which may not fit commercial use cases. Hence, It is FL’s suggestion to prioritize 30 GHz carrier frequency.

The carrier frequency for SL-FR2 of 60 GHz are used in TR38.808. Some company observes there is currently no licensed band around 60 GHz. Hence, it is suggested by FL that the carrier frequency of 30 GHz is used.

Sub-carrier spacing (SCS)

Two SCS of SL-FR2 were proposed by companies:

- 60 kHz: vivo, ZTE, LG, Lenovo

- 120 kHz: vivo, ZTE, LG, Huawei, Nokia, Lenovo

Considering 60 kHz and 120 kHz are supported SCS for sidelink operation on FR2, FL suggests supporting both of them in the evaluation methodology. However, for the purpose of reduced workload, it is suggested that 120 kHz as mandatory, while 60 kHz as optional.

Simulation bandwidth

Three sidelink simulation bandwidth values of SL-FR2 were proposed by companies:

- 100 MHz: vivo, Apple, ZTE, OPPO, Samsung, LG, Ericsson, Qualcomm

- 200 MHz: LG, Huawei

- 400 MHz: vivo, Samsung, Nokia, Qualcomm

The sidelink simulation bandwidth of 200 MHz are used in TR37.885 based on carrier frequency of 30 GHz or 63 GHz. The sidelink simulation bandwidth of 100 MHz are used in TR38.885 based on carrier frequency of 30 GHz. The simulation bandwidth of 400 MHz is based on TR38.808 based on carrier frequency of 60 GHz.

If the carrier frequency of 30 GHz is used, then it is preferred to use 100 MHz sidelink simulation bandwidth.

UE receiver noise figure

Two UE receiver noise figure values of SL-FR2 were proposed by companies:

- 13 dB: ZTE, Apple, Nokia

- 10 dB: ZTE, Apple, Nokia

Both of these numbers are used in TR37.885, where 13 dB is mandatory, and 10 dB is optional. FL thinks the same assumption can be used for SL-FR2.

Overall, the above suggested parameters are summarized in Proposal 1-3-a.

The following table provides a summary of company proposals on this issue:

| Company | Company proposal related to this issue |
| --- | --- |
| Nokia | Proposal 1: Reuse evaluation assumptions defined for NR FR2 studies focusing on indoor deployment scenario and follow commercial traffic models and metrics as agreed for SL-U evaluations as a starting point. |
| Huawei | Proposal 1: The evaluation methodologies of FR2 for NR Uu defined in TR 38.802, NR V2X defined in TR 37.885 and agreements for Rel-17 SL commercial use cases, should be reused as much as possible, with adjustment for sidelink over FR2, where both indoor and outdoor can be considered. Proposal 2: Reuse simulation assumptions on operating frequency and simulation bandwidth as defined in TR 37.885 for sidelink over FR2, where the carrier frequency for FR2 can be 30 GHz and the simulation bandwidth is 200 MHz. Proposal 3: It is recommended to use 120 kHz subcarrier spacing for sidelink over FR2. Proposal 4: Reuse the value of TX power, i.e, 23 dBm for sidelink over FR2 as defined in TR 37.885. Proposal 5: Reuse pairs topology in SL-U for indoor scenarios in the assessment/evaluation for sidelink operation over FR2 licensed spectrum. - For indoor layout with the size of 120m*80m, the value of maximum distance between communicating peer is 10m for indoor scenarios. - The number of UEs is 80 for 200MHz bandwidth. Proposal 8: Reuse dense urban defined in TR 38.802 for sidelink over FR2 licensed band. - For the dense urban with 200m ISD, the value of maximum distance between communicating peer for outdoor scenarios is 10m or 30m. - Only one site is considered. - The number of UEs is 60 for 200MHz bandwidth. |
| Spreadtrum | Proposal 1: The commercial use cases supported in SL-U could also be considered for Sidelink FR2. Proposal 2: Urban macro scenario could also be supported for Sidelink FR2. |
| Vivo | Proposal 1: The evaluation methodology defined in TR37.885 can be the start point for the evaluation of FR2-1 in lincensed spectrum. Proposal 2: Confirm the evaluation model defined in TR38.808 for B52.6G as the start point for the evaluation of sidelink operating on FR2-2. Proposal 3: Select the indoor scenario as the baseline for the FR2-2 evaluation of sidelink. Proposal 4: The network topology shown in Figure 1 should be introduced to the methodology as baseline and that in Figure 2 can be considered as optional. Proposal 5: The subcarrier space should be introduced to the methodology to align the simulation paramters. |
| ZTE | For the deployment scenario of SL FR2 At least support the NR InH Mixed Office layout, where the simulation assumptions in Table 2 can be used； FFS: the support of general scenarios defined in TR 36.843 Proposal 3: The following fundamental assumptions are considered in SL-FR2 evaluations： • Carrier frequency：30 or 63 GHz • UE Tx power：23 dBm for 30 GHz, 21 dB baseline for 63 GHz, 27 dBm optional for 63 GHz. For both 30 and 63 GHz, EIRP should not exceed 43 dBm • UE receiver noise figure：13 dB (baseline), 10 dB (optional) • SL simulation bandwidth (MHz)：100 MHz for 30 or 63 GHz • SCS: 60 or 120 kHz |
| OPPO | Proposal 1: The evaluation methodology updated for commercial use cases from Rel-17 eSL should be used as the baseline for SL operation in FR2. Proposal 2: For carrier frequency and simulation bandwidth, 30GHz and 100MHz should be supported respectively for SL operation in FR2. Proposal 3: For commercial scenario, the following layout should be supported: - Indoor scenario (baseline): Scenario 1-Option 1 from SL-U (remove interference nodes) - a = 80m, b = 120m - UE dropping: 60 SL UE pairs for 100MHz - Outdoor scenario (optional): Option 3 in TR 36.843: Urban macro (500m ISD) (all UEs outdoor) - UE dropping as in Table A.2.2.1-1 |
| CATT | Proposal 1: Regarding the scenarios for enhanced sidelink operation on FR2 licensed spectrum: - V2X scenarios can be divided into the following categories: - Vehicles Platooning - Advanced Driving - Extended Sensors - Remote Driving - The impact of FR2 licensed spectrum on each of the preceding scenarios should be studied. Proposal 6: The UE dropping and mobility modeling Option A defined in TR 37.885 for both highway scenario and urban grid scenario can be used for the study of sidelink enhancement for FR2 licensed spectrum. |
| Intel | - Define evaluation methodology only for FR2-1 - Use FR2-1 SLS evaluation methodology defined in 37.885 as a starting point |
| JHU | Proposal 1: Consider the FR2 sidelink in the Urban scenario. Use representative values for the number of devices, device density and device mobility in this scenario. Proposal 2: Consider the FR2 sidelink in the Indoor scenario. Use representative values for the number of devices, device density and device mobility in this scenario. Proposal 3: Consider the FR2 sidelink in the PAN scenario. Use representative values for the number of devices, device density and device mobility in this scenario. Proposal 4: Consider the FR2 sidelink in the Device-to-Device Backhaul scenario. Use representative values for the device mobility, device heights, and device mobility, where applicable. Proposal 5: Consider the FR2 sidelink in the Device-to-Device Disconnected Network scenario. Use representative values for the number of devices and device mobility in this scenario. |
| Xiaomi | Proposal 1: Use the existing agreements in Rel-17 SL WI on deployment scenario for commercial use cases as the baseline, with necessary update on carrier frequency, bandwidth, antenna pattern, etc. - FFS whether additional deployment scenario is needed. |
| Lenovo | Proposal 1: Reuse the agreed indoor layout in SL-U without the deployment of Wi-Fi node for commercial use cases for SL-FR2. Proposal 2: On the evaluation assumptions for SL-FR2, we suggest following aspects: - Carrier frequency: 30GHz and 63 GHz - Reuse the antenna modeling defined in 37.885 - Reuse NR InH Mixed office model with carrier frequency of 30GHz and 63GHz defined in 38.901 - Reusing the agreed traffic model in SL-U for SL-FR2 evaluation including - R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10) - FTP model 3 with arrival rate satisfying one of the followings: - BO Low load: 10%~25% - BO Mid load: 35%~50% - BO High load: above 55% - XR cloud gaming model in TR38.838 - The performance metric including UPT, latency, PRR and RSRP between TX UE and RX UE. |
| LG | Proposal 1: The evaluation methodology defined in Rel-17 [2] with the clarifications below is the baseline for the commercial deployment scenario for SL operation on FR2. - Carrier frequency - 30 GHz, [63 GHz] - Simulation bandwidth - 100 MHz, [200 MHz] - Sub-carrier spacing - 120 KHz, [60 KHz] - In-band emission model - Vehicle UE - Reuse Table 6.4.2.3.3-1 in Section 6.4.2.3.3 of TS 38.101-2 - Handheld UE (or Pedestrian UE) - Reuse Table 6.4.2.3.4-1 in Section 6.4.2.3.4 of TS 38.101-2 - UE pairing for Unicast - Reuse what is defined in Section 6.1.5 of TR 37.885 Proposal 2: The evaluation methodology defined for SL-U in Rel-18 [3] can be optionally considered, but further discussion is needed at least on how many SL UE pairs of Unicast are dropped. |
| MediaTek | Proposal 1: The evaluation methodology for SL FR2 shall focus on the commercial use case. Proposal 3: The evaluation methodology in SL-U and TR38.808 can be as a baseline for the SL FR2 evaluation discussion. Proposal 4: For SL FR2 deployment scenario, the agreed SL-U deployment scenario or the scenario Indoor-E as defined in TR38.808 can be as a baseline. Proposal 5: For SL FR2, the indoor layout scenario with pair-based topology is used for evaluation deployment scenario. - The same Indoor layout size, e.g., 80m*100m, is reused for SL FR2 evaluation. - Only one operator or RAT is considered for the SL FR2 evaluation - The number of total UE pairs for SL FR2 evaluation is suggest twice larger than SL-U pair, e.g., 6 pairs shall be configured for SL FR2 - The UE’s position in the indoor is random configured. Proposal 6: Further discuss whether it is feasible to evaluate the cluster-based topology for SL FR2 deployment scenario. If it is feasible, the SL-U cluster-based scenario can be as a baseline for SL FR2. |
| Apple | Proposal 1: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse layout option 3 in Section A.2.1.1 of TR 36.843 with 7 macro sites with 3 cells per site. Proposal 2: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse indoor layout defined for sidelink operation on unlicensed spectrum with option 1 of sidelink UE dropping and association. - A total number of 24 UEs is to be deployed in the layout. Proposal 3: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, consider the following parameters modification: - Carrier frequency: 30 GHz - Simulation bandwidth: 100 MHz - UE receiver noise figure: 13 dB (baseline), 10 dB (optional) |
| Interdigital | Proposal 1: Adopt the indoor deployment used in SL-U as a baseline for SL FR2: - Consider updating the number of SL pairs of UEs based on FR2 carrier BW - SL UEs are uniformly distributed in the deployment area - Reuse the Tx-Rx association model defined in TR 37.885 with updated association distance for SL FR2 |
| Samsung | Proposal 1: For evaluation of commercial deployments in SL FR2, the carrier frequency is 30 GHz. Proposal 2: For evaluation of commercial deployments in SL FR2, evaluate the following system bandwidth options 100 MHz at 60 kHz SCS and 400 MHz at 120 kHz SCS. Proposal 3: For evaluation of commercial deployments in SL FR2, use a hexagonal grid with 7 macro sites and 3 sectors per macro site with wrap around. The ISD is 500m with all UEs outdoor. Proposal 4: For evaluation of commercial deployments in SL FR2, there are a total of 150 UEs per cell. The UEs are drop randomly and uniformly throughout the simulated geographical area. Proposal 5: For evaluation of commercial deployments in SL FR2, UEs have a speed of 3 km/hr. Proposal 6: For evaluation of commercial deployments in SL FR2, UEs have a maximum transmit power of 23 dBm. |
| Qualcomm | Proposal 5: For the link-level evaluation of FR2 SL commercial use cases, the simulation assumptions in the following table can be considered as baseline. |
| CEWiT | Proposal 3: In Rel-18 SL beam enhancement in FR2, Highway and Urban grid scenarios can be used for evaluation purpose. Proposal 4: In Rel-18 SL beam enhancement in FR2, the evaluation parameters for V2X scenario can be reused from 37.885 as shown in Table 2. - Additional parameters can be provided by company. |
| Ericsson | - Proposal 1: The evaluation methodology for SL in FR2 uses the evaluation methodology for SL unlicensed and for commercial use cases in Rel-17 and as a starting point. - Proposal 2: Indoor layout used in SL-U is used for evaluating SL in FR2. - Proposal 3: RAN1 to focus on pairs topology for evaluating SL in FR2. Details are FFS. - Proposal 4: 30 GHz is used as the carrier frequency for SL in FR2. - Proposal 5: The baseline for system bandwidth in SL FR2 is 100 MHz. FFS other optional values. |

### [Closed] First round discussions

#### Proposal 1-1-a

Proposal 1-1-a: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse indoor layout defined for SL-U with pairs topology and without WiFi nodes

- FFS: total number of UEs deployed in the layout

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Ericsson | Yes |  |
| Nokia, NSB | Yes | Resource allocation assumption for indoor scenario needs to be clarified (e.g., random resource selection). |
| JHU APL | No | SL-U power levels are very low and therefore range is limited; coverage could be better with FR2; example indoor scenarios could be airports, train stations, and indoor stadiums – rectangular area of 100m x 150m |
| InterDigital | Yes |  |
| CATT/GOHIGH | Yes |  |
| Samsung | Yes | To avoid any misunderstanding, it would be good to refer to a document |
| Transsion | Yes |  |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes | Agree to reuse pairs topology in SL-U for indoor scenarios in the evaluation for SL FR2 licensed spectrum. For indoor layout with the size of 120m*80m, we suggest 80 UEs derived from the total number of UEs of 200MHz bandwidth deployed in SL-U with scaling factor of 1/3. |
| Vivo | Yes | Reusing the indoor layout for SL-U can reduce the work load for the methodology definition |
| Xiaomi | Yes |  |
| Lenovo | Yes |  |
| Qualcomm | Yes |  |
| CEWiT | Yes |  |
| MTK | Yes with suggestion | Regarding the scenario topology, the cluster-based topology can be used in some cases, e.g., considering the head TX UE and RX UEs have different UE capability and RX UEs can use a coarse beam to receive a TX UE’s finer beam. We are open to discuss it and suggest adding an FFS as following: Proposal 1-1-a: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse indoor layout defined for SL-U with pairs topology and without WiFi nodes - FFS: total number of UEs deployed in the layout - FFS: whether to consider the cluster-based topology if TX UE and RX UEs have different UE capability. |

#### Proposal 1-2-a

Proposal 1-2-a: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse layout option 3 in Section A.2.1.1 of TR 36.843 with 7 macro sites with 3 cells per site.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Ericsson | No | In our view, for commercial cases and SL FR2, we should focus on indoor scenarios. |
| Nokia, NSB | Yes | Resource allocation assumption for outdoor scenario needs to be clarified (e.g., mode 1). Also, it needs to be clarified whether Uu is included in the simulation (e.g., in case of SL relay use case) and if so, whether Uu and PC5 use the same carrier and therefore may be subject to cross-interference. |
| JHU APL | Yes | Useful for outdoor festivals, sports stadiums, etc. |
| InterDigital | No | Same view with Ericsson. Our focus should be indoor scenario. |
| CATT/GOHIGH | Yes |  |
| Samsung | Yes |  |
| Transsion | Yes |  |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Ok to define outdoor but suggest using different layout | It might not appropriate to reuse layout option 3 directly for FR2, although it is accepted in Rel-17 which is specified for FR1. - The pathloss in FR2 is much more severe than FR1, directly reusing the Urban macro with 500m ISD for outdoor commercial case for SL-FR2 is too large to consider SL interference in FR2. Additionally, sidelink system is target for short distance transceiver pairs, the issue on coverage is not as demanding as Urban macro. - It is not necessary to consider as many as 7 macro sites. The intention to have more sites in Uu is to simulate the inter-cell interference. However, evaluation in SL focuses on UEs other than BS and beam interference is limited in such high loss case. - Too large simulation layout would result in too large number of simulated UEs, which consumes unnecessary simulation effort. NR Uu FR2 has defined dense urban scenario with 200m ISD and we propose to reuse such layout for SL-FR2, with considering only one site given there is no need to consider inter cell interference for SL-FR2. |
| vivo | Yes | Alternatively, the urban scenario defined for V2X use case can also be reused for the commercial scenario to align the layout for the two use cases. |
| Xiaomi | OK | We are also fine with HW suggestion to reduce the simulation layout. |
| Lenovo | No | We prefer to focus on indoor scenarios. Outdoor deployment is only supported if it is specifically defined for FR2. |
| Qualcomm | No | As Huawei commented, urban macro with 500m ISD may not be a typical deployment for FR2. We think dense urban is more suitable for FR2 evaluation. Note that dense urban has mainly been assumed for FR2 evaluation in the XR study (TR 38.838) and Rel-18 NR duplex evolution. |
| CEWiT | Yes |  |
| MTK | Comments | Actually, we have the similar view with Ericsson, Interdigital and Lenovo that the SL FR2 evaluation should focus on indoor scenario for commercial use case as agreed in SL-U. If majority views support the outdoor scenario, we can live with it and slightly prefer indoor case as a baseline scenario and outdoor case as an optional scenario. |

#### Proposal 1-3-a

Proposal 1-3-a: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, consider at least the following parameters:

- Carrier frequency: 30 GHz

- Sub-carrier spacing: 120 kHz (baseline), 60 kHz (optional)

- Simulation bandwidth: 100 MHz

- UE receiver noise figure: 13 dB (baseline), 10 dB (optional)

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes |  |
| OPPO | Yes |  |
| Intel | Comments | Given that there is currently no licensed spectrum available in FR2-2, we do not see any need to simulate or consider 120 kHz SCS. Therefore, our preference is to consider 60 kHz as baseline. |
| Ericsson | Yes |  |
| Nokia, NSB | Yes | Add TX power: 23 dBm (EIRP should not exceed 43 dBm) Add UE speed: 3 km/h |
| JHU APL | Yes |  |
| InterDigital | Yes |  |
| CATT/GOHIGH | yes |  |
| Samsung | Comment | Add 400 MHz as an optional simulation BW. |
| Transsion | Yes |  |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes for the most but suggest to have 200MHz simulation bandwidth | - Agree to 30GHz carrier frequency, 120KHz SCS, and 13 dB (baseline), 10 dB (optional) UE receiver NF - Support 200MHz simulation bandwidth: - XR service, a particularly demanding application for NR, could be considered a typical use case operating in FR2 spectrum, of which a key requirement is to increase sidelink data rate and expand the applicability of sidelink to commercial use cases. On the basis, it requires larger bandwidth to satisfy the requirement. - In FR1, it already supports the maximum bandwidth of 100MHz. If 100MHz is reused in FR2, the benefits brought by larger band cannot be simulated. Note, in FR2, bandwidth is no longer a limitation. Therefore, it should enlarge the simulation bandwidth in FR2 and we suggest to reuse 200MHz bandwidth for 30G in 37.885. |
| Vivo | Yes, with comment | 60GHz can be considered as an optional carrier frequency for the evaluation. |
| Xiaomi | Yes |  |
| Lenovo | Yes |  |
| Qualcomm | Yes | Other bandwidths, such as 400MHz, can be optionally considered. |
| CEWiT | Yes |  |
| MTK | Yes |  |

#### Questions

Question 1-1: Do you agree to (additionally) reuse indoor-A or indoor-B scenario in TR38.808 in the evaluation methodology?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Ericsson | No | In our view, we can focus on the indoor scenario defined in SL-U for commercial deployments. |
| Nokia, NSB | Yes | In addition to indoor scenario from Proposal 1-1-a, indoor-B scenario can be included as an optional scenario for evaluation of beam management without interference. |
| JHU APL | No | The scenarios covered in the previous proposals are sufficient. |
| InterDigital | No | Same view with Ericsson. We should focus on indoor scenario. |
| Samsung | No | Prefer not to include |
| Huawei, HiSilicon | No | Both SL-U and FR2 are considered for commercial use cases, so many defined simulation parameter for SL-U can be borrowed in FR2. As for indoor scenario, the Rel-18 SL-U indoor scenario (Option 1 without WiFi nodes) have been well discussed and agreed. Therefore, we support to reuse it to reduce discussion effort and save simulation workload. |
| vivo | Yes | We can accept either the layout of indoor-A defined in TR38.808 and/or the indoor layout defined for SL-U. |
| Lenovo | No | Prefer not to include. |
| Qualcomm | No | We are okay with the proposal, as long as we keep it optional. |
| MTK | No | Share the similar view with Ericsson and InterDigital. |

### [Closed] Second round discussions

#### [Closed] Proposal 1-1-b

In the first round discussion, 17 out of 18 companies support the proposal.

JHU APL thinks SL-U power levels are very low and therefore range is limited. Coverage could be better with FR2 and a larger rectangular area of 100m x 150m is proposed. According to FL, the pathloss of FR2 is generally larger than the pathloss of FR1. Hence is fine to reuse the smaller rectangular area (i.e., 120m x 80m).

@ Nokia: In general, the resource allocation schemes are not assumed in the evaluation methodology. This also address your comments in Section 3.6.

@Samsung: We could cite RAN1 #110 meeting for the agreed SL-U layout. See the modification.

@MTK: It was mentioned by some company that the UEs in cluster topology may be considered as wearable and smart-home devices, which in practice does not equip for supporting beam-based FR2 operation due to hardware complexity and cost. Hence, cluster topology is not considered.

@ZTE: for the comments in Section 3.6, the main consideration is the pair of UEs. Like in SL-U, we could let companies report how UEs are paired. See the modification.

Proposal 1-1-b: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse indoor layout defined for SL-U with pairs topology and without WiFi nodes as in RAN1 #110 meeting

- FFS: total number of UEs deployed in the layout

- Companies should report how UEs are paired

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes | Ok with FL's explanation to our question in section 3.6 |
| Nokia, NSB | Yes |  |

#### [Closed] Proposal 1-2-b

In the first round discussion, 13 out of 18 companies support the proposal in principle.

4 (Ericsson, Interdigital, Lenovo, MTK) out of 18 companies think that SL-FR2 should focus on indoor scenarios, and hence outdoor scenarios are not supported. However, majority of the companies support outdoor scenario, while some mentioned the existence of applications where SL-FR2 is used in outdoor scenario (e.g. Pokemon GO, interactive advisements). Considering that, it is suggested keeping the proposal for further discussion online.

2 (Huawei, Qualcomm) out of 18 companies mention the ISD of 500 meters is too large to consider SL interference in FR2. If ISD of 500 meters with denser UE dropping and with the existing UE pairing scheme as in TR36.843 Section A.2.1.1.1, then it may be effectively similar to ISD of 200 meters. Here, we could discuss the number of UEs deployed in the layout. Please see modification.

Huawei additionally mentions that it is unnecessary to consider 7 macro sites since inter-cell interference is limited. Instead, a single site is used for the ease of simulations. We could consider this as an option. Please see the modification.

Proposal 1-2-b: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse layout option 3 in Section A.2.1.1 of TR 36.843 with

- Option 1: 7 macro sites with 3 cells per site

- Option 2: a single site

- FFS: total number of UEs deployed in the layout

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes | Echo the benefit with option 2 to reduce simulation efforts. Current formulation of both options is fine. |
| Nokia, NSB | See comments | We prefer to focus on Indoor scenario only. It needs to be clarified whether Uu is included in the simulation (e.g., in case of SL relay use case) and if so, whether Uu and PC5 use the same carrier and therefore may be subject to cross-interference. Proposal 1-2-b: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse layout option 3 in Section A.2.1.1 of TR 36.843 with - Option 1: 7 macro sites with 3 cells per site - Option 2: a single site - FFS: total number of UEs deployed in the layout - FFS: whether Uu and PC5 use same carrier |

#### Proposal 1-3-b

In the first round discussion, 16 out of 18 companies support the proposal in principle.

@ Intel: It should be mentioned that even in FR2-1, 120 kHz SCS is supported. In Rel-16 NR SL, the basic UE SL feature (e.g., FG15-1) mentioned to support 120 kHz SCS for FR2. For the sake of progress, we could consider both 120 kHz and 60 kHz SCS are supported. See the modification.

@ Nokia: added UE Tx power and UE speed. See the modification.

@Samsung, Huawei, Qualcomm: Since 400 MHz simulation bandwidth is proposed in TR38.808 based on carrier frequency of 60 GHz. It may be too much for the carrier frequency of 30 GHz. As a compromise, we could add 200 MHz simulation bandwidth as an option, as it is agreed in TR37.885. See the modification.

@vivo: Some company observes there is currently no licensed band around 60 GHz. Hence, it is suggested by FL that the carrier frequency of 30 GHz is used.

Hence, Proposal 1-3-b is modified as follows:

Proposal 1-3-b: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, consider at least the following parameters:

- Carrier frequency: 30 GHz

- Sub-carrier spacing: 120 kHz (baseline), 60 kHz (optional)

- Simulation bandwidth: 100 MHz (baseline), 200 MHz (optional)

- UE receiver noise figure: 13 dB (baseline), 10 dB (optional)

- UE Tx power: 23 dBm (EIRP should not exceed 43 dBm)

- UE speed: 3 km/h

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes |  |
| Nokia, NSB | Yes |  |
| JHU APL | Yes |  |
| OPPO | Yes |  |
| Qualcomm | Yes |  |
| Lenovo | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes |  |
| Vivo | Yes |  |
| MTK | Yes |  |
| CEWiT | Yes |  |
| Huawei, HiSilicon | Yes for the most but suggest to 120kHz SCS as basline | For multiple subcarrier spacing 120kHz and 60kHz, a baseline should be set as original proposal (proposal 1-3-a). Otherwise, companies may be not clear which one should be selected. Considering one of the target traffic type for NR SL FR2 is XR traffic, which has strict requirements on service latency, it’s recommended to use 120 kHz subcarrier spacing in the evaluation to reduce workload. In addition, sidelink system is target for short distance transceiver pairs, the issue on coverage is not as demanding as NR Uu. We believe that 120 kHz SCS can meet the coverage requirements for NR SL FR2. As the question from Intel, it is not true 120kHz is only available in FR2-2. It can be also used in FR2 at such as 30GHz carrier frequency, this is already defined in Table A.1.6-4 of TR38.802. |
| InterDigital | Yes |  |

#### Response to Question 1-1

In the first round discussion, 8 out of 10 companies are fine without additional indoor scenarios beyond that for SL-U.

@ vivo: since indoor layout for SL-U is already captured in Proposal 1-1-b, it is fine to avoid the indoor-A scenario.

@Nokia: majority of the companies prefer not to introduce additional scenario. Hope it is fine for you to focus on the indoor scenario for SL-U.

#### [New] Questions for indoor scenario

The following agreement was made in Tuesday (10/11) GTW session:

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2

- Reuse indoor layout defined for SL-U with pairs topology and without WiFi nodes

  - FFS: total number of UEs deployed in the layout

  - Companies should report how UEs are paired

- FFS: whether to consider the cluster-based topology defined for SL-U

Regarding the first FFS in the above agreement, FL would like to collect companies views the total number of UEs deployed in the layout. Note that in SL-U for unicast traffic, 6 SL-U pairs and 4 NR-U UEs/WiFi nodes per gNB/AP per 20 MHz are deployed. Since there is no WiFi node in SL-FR2 evaluation methodology, the NR-U UEs could be replaced by SL UEs in the deployment. Two additional considerations are

            - FR2 has larger pathloss than FR1, hence smaller distance between UEs are expected in FR2, which implies more UEs deployed in the layout.

            - FR2 has much larger bandwidth than FR1, whether the UE number scaling for SL-U (per 20 MHz) is applied for FR2

Question 1-2: What is the total number of UEs to be deployed in the indoor layout (defined for SL-U) for SL-FR2 evaluation methodology?

| Company | Number of UEs | Comments |
| --- | --- | --- |
| OPPO | 60 SL pairs for 100MHz | In SL-U for unicast traffic, 6 SL-U pairs and 4 NR-U UEs/WiFi nodes per gNB/AP per 20 MHz are deployed. Since NR-U UEs/WiFi nodes are replaced by SL UEs in SL FR2, there will be 12 SL pairs deployed in the layout per 20MHz. In the above proposal 1-3-b, 100MHz is considered as the baseline bandwidth for SL FR2, which is much larger than FR1. We think the UE number scaling should be applied for FR2 and then 60 SL pairs are deployed in the indoor layout for 100MHz. |
| Ericsson | 6 SL pairs per 20 MHz | In our view, we should reuse the same scenario as for SL-U considering only the SL-U pairs. For the case of SL-U, 6 SL-U pairs per 20 MHz were defined. The other pair of UEs (NR-U and WiFi) were intended to evaluate the performance with other RATs and for the case of SL FR2 are not necessary. |
| Intel | 12 (6 UE pairs) | We feel that 6 pairs of UEs may be sufficient, and there is no need to consider a further densified network, which would require a much longer computational time to be simulated. |
| vivo | See comment | The UE dropping in NRU can be baseline, and the UE number should follow the principle of that in unicast traffic, 12 UE pairs per 20 MHz are deployed and in groupcast and broadcast, 24 UE pairs per 20 MHz are deployed. The UE number scaling can also be applied for FR2 evaluation, e.g., can be defined with factors such as1.2 or 1.5. |
| MTK | See comment | For the pair-based topology, the total number of UEs in the SL-U unicast traffic can be as baseline, i.e., 6 SL-U pairs and 4 NR-U UEs/WiFi nodes per gNB/AP per 20 MHz are deployed. The total 12 UE pairs per 20MHz. Considering larger bandwidth will be used for SL FR2 and larger number of UEs will make the evaluation more complexity, the number scaling is preferred, e.g., 8 or 10 pairs per 20MHz is suggested. |
| Huawei, HiSilicon | 80 UEs for 200MHz bandwidth. | For indoor layout with the size of 120m*80m as defined in SL-U, we support to scale down the UE number deployed in SL-U (per 20 MHz) for FR2. - Considering that not all devices in the given area are to operate in FR2 spectrum, it needs to scale down possible number of UEs in the evaluation. In addition, the number of UEs in the area will affect interference level to other UEs. We propose to consider the scaling factor of 1/3 and recommend 80 UEs (24*10*1/3) for 200MHz bandwidth for SL-FR2. |

Regarding the second FFS in the above agreement, FL would like to learn companies’ views on whether the cluster-based topology as for SL-U is supported for SL-FR2.

Question 1-3: Do you support to consider the cluster-based topology (defined for SL-U) for SL-FR2 evaluation methodology?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia, NSB | No |  |
| OPPO | No |  |
| Ericsson | No |  |
| Intel | No | Given that the WID targets unicast systems, we do not see any need for a cluster-based topology which is meant to model groupcast or broadcast systems. Our understanding is that the target of this evaluation is to evaluate directional beam management which is clearly in contradiction with the direction of using a model typically used for groupcast system for such evaluations. |
| Vivo | Yes | The cluster based topology defined for SLU can be reused as optional scenario |
| MTK | Yes | Cluster-based topology is also a typical scenario for SL-FR2, e.g., in the smart home scenario, the smartphone as a head-UE can serve the surrounding wearable UEs. To be honest, the Tx UE and Rx UEs in this scenario has different capability, that is also the reason why we propose to consider UE’s capability to discuss the issue, e.g., as we mentioned before, the Rx UEs can use a coarse beam to receive the head-UE’s finer beam with TDMed manner. We are also ok to consider the pair-based topology as a baseline scenario and cluster-based topology with different UE capability as an optional scenario. |
| Huawei, HiSilicon | No | For cluster topology, since some of the UEs are considered as wearable and smart-home devices, which in practice does not equipped for supporting beam-based FR2 operation due to hardware complexity and cost. Therefore, we propose to reuse the pairs-topology only which should be enough for SL-FR2. |

Also in the GTW session, there is a proposal to consider the resource allocation schemes used in the evaluation methodology.

Question 1-4: Do you agree the resource allocation schemes (e.g., random resource selection, IUC, etc) should be reported by companies in the SL-FR2 evaluation methodology?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia, NSB | Yes | If resource allocation scheme is not reported by Company1 and Company2, there is a risk that Company1 obtains, e.g., significantly better UPT than Company2 as a result of better interference avoidance (e.g., Company2 uses random resource selection while Company1 uses something smarter like IUC) even though the actual beam alignment solution of Company2 is better (i.e., misalignment occurs less frequently than with Company1’s solution). This is especially true for high intensity traffic, as the collision probability increases. |
| JHU APL | Yes | Specific approach used should be stated by companies. |
| OPPO | Yes |  |
| Ericsson | Yes | The used resource allocation scheme can be reported when performing simulations. |
| Intel | Yes | In order to get better insights from the results and be able to compare them, the resource allocation scheme used should be definitely reported. |
| Vivo | See comment | The simulated scheme is usually described by companies in their simulations, but it seems not necessary to discuss the topic in evaluation methodology. |
| Huawei, HiSilicon | Yes | Specific resource allocation scheme used by SL UE can be reported by companies for SL-FR2, mode 1 or mode 2（sensing-based or random resource selection）. |

#### [New] Questions for outdoor scenario

The following agreement was made in Tuesday (10/11) GTW session:

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse layout option 3 in Section A.2.1.1 of TR 36.843 with

- Option 1: 7 macro sites with 3 cells per site

- Option 2: a single site

- Companies should report how UEs are paired

- FFS: total number of UEs deployed in the layout

- FFS: whether Uu and PC5 use same carrier

- FFS: ISD for this layout option 3

Regarding the last FFS in the above agreement, the two options are ISD of 200 meters and 500 meters. FL would like to learn companies’ views on this topic. It is preferred to down-select one option in order to facilitate the related discussions like “total number of UEs deployed in the layout”, “channel model”, etc.

Question 1-5: What is the ISD for the layout option 3 in TR36.843?

Option 1: 200 meters

Option 2: 500 meters

| Company | Preferred Option | Can accept the other (non-preferred) option? | Comments |
| --- | --- | --- | --- |
| JHU APL | Option 2 | Yes |  |
| Qualcomm | Option 1 | Yes | In both XR study (TR 38.838) and in Rel-18 NR duplexing evolution study, urban macro with 500m ISD was considered for FR1, while dense urban with 200m ISD was considered for FR2 evaluation. We think we can reuse the same assumption. |
| Intel | Option 1 | Yes |  |
| vivo | Option 1 | Baseline can be option 1 | Larger ISD implies the UE density may increase to guarantee the transmission quality between the SL UE. The increasing UE density will introduce extra evaluation overhead, which is not desirable. |
| Huawei, HiSilicon | Option 1 | Comment | We have already explained why 200m ISD should be used for FR2 in first round. Reasons are also summarized as below. - The pathloss in FR2 is much more severe than FR1, directly reusing the Urban macro with 500m ISD for outdoor commercial case for SL-FR2 is too large to consider SL interference in FR2. Additionally, sidelink system is target for short distance transceiver pairs, the issue on coverage is not as demanding as Urban macro. - Too large simulation layout would result in too large number of simulated UEs, which consumes unnecessary simulation effort. NR Uu FR2 has defined dense urban scenario with 200m ISD and we propose to reuse such layout for SL-FR2 with considering only one site. |

In layout option 3 in TR36.843, the number of UEs dropped per cell is 150 where the ISD is 500 meters. However, the number could be different for SL-FR2. FL would like to learn companies’ views on this topic.

Question 1-6: What is the total number of UEs to be deployed in the outdoor layout, where separate numbers for ISD of 200 meters or 500 meters could be proposed?

| Company | Number of UEs dropping in ISD 200 meters | Number of UEs dropping in ISD 500 meters | Comments |
| --- | --- | --- | --- |
| Qualcomm | 60 UEs per macro geographical area (at least for FTP traffic) |  | The same assumption in TR 38.802 (Table A.2.1-1) |
| Intel | 10 pairs of UEs should be sufficient |  |  |
| vivo | 10UEs per cell can be considered as the baseline. The total UE number scaling can be applied, e.g., 1.2 or 1.5 |  |  |
| Huawei, HiSilicon | 60 UEs within dense macro geographical area for single site 200ISD |  | The value of UE numbers dropped at a given area is 60 for dense urban scenario with 200ISD as defined in TR38.802 and we can reuse it for SL-FR2. |

There is an FFS in the above agreement. FL would like to learn companies’ views on this topic.

Question 1-7: In the outdoor layout, whether Uu link and PC5 use same carrier?

| Company | Yes or No | Any agreement needed for this? | Comments |
| --- | --- | --- | --- |
| Nokia, NSB | Yes | At least a common working assumption is needed to allow for a fair comparison of performance results (e.g., UPT) among companies. | It needs to be clarified whether Uu is included in the simulation (e.g., in case of SL relay use case) and if so, whether Uu and PC5 use the same carrier and therefore may be subject to mutual interference: - A beamformed PSSCH transmission may cause severe interference to PUSCH reception if the SL Tx beam happens to point in the direction of the gNB. - A PUSCH transmission may cause severe interference to PSSCH reception if the SL Rx beam happens to point in the direction of the PUSCH transmitter. |
| Qualcomm | Yes |  |  |
| Intel | Yes |  | As mentioned by other companies, one important aspect to be discussed is on whether the Uu should be simulated and gNb-to-UE CLI should be taken into account, since this aspect may influence the simulations results. |
| Vivo | Yes |  |  |
| Huawei, HiSilicon | Comment |  | It is a possible scenario that Uu and SL can share the same carrier, and thus interference to both SL and UL can be considered. We are open to have this scenario and it can be optional. If further clarification is needed, RAN1 can conclude following scenarios can be considered for FR2 - Uu link and PC5 use different carrier (baseline) - Uu link and PC5 use same carrier (optional) |

### [Closed] Third round discussions

#### Proposal 1-3-c

In the second round discussion, 12 out of 13 companies support the proposal. Huawei supports most of the proposal, except that they think 120 kHz should be the baseline SCS, while 60 kHz should be optional. The reason is that to meet latency requirement, XR recommended to use 120 kHz SCS.

FL would like to check if the following proposal could be acceptable to companies.

Proposal 1-3-c: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, consider at least the following parameters:

- Carrier frequency: 30 GHz

- Sub-carrier spacing: 120 kHz (baseline), 60 kHz (optional)

- Simulation bandwidth: 100 MHz (baseline), 200 MHz (optional)

- UE receiver noise figure: 13 dB (baseline), 10 dB (optional)

- UE Tx power: 23 dBm (EIRP should not exceed 43 dBm)

- UE speed: 3 km/h

| Company | Yes or No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE,Sanechips | Yes |  |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| Ericsson | Yes |  |
| Nokia, NSB | Yes |  |
| JHUAPL | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes |  |

#### Proposal 1-4-c

Regarding Question 1-2, the proposals from the companies can be summarized as

      - OPPO: 60 pairs for 100 MHz

      - Ericsson: 6 pairs/20 MHz

      - Intel: 6 pairs/20 MHz

      - Vivo, MTK: 12 pairs/20 MHz with possible scaling

      - Huawei: 40 pairs for 200 MHz

The proposal of 12 pairs/20 MHz is equivalent of 60 pairs/100 MHz. Hence, we could merge Option 1 and 4.

Note that the simulation bandwidth is discussed in Proposal 1-3-b, which is 100 MHz (baseline) or 200 MHz (optional). Hence, it is preferred to align the number of UEs in per 20 MHz bandwidth.

Given the situation, FL has the following proposal.

Proposal 1-4-c: For the indoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the total number of UEs is to be down-selected from the following options:

- Option 1: 12 pairs/20 MHz

- Option 2: 6 pairs/20 MHz

- Option 3: 6 pairs

- Option 4: 20 pairs/20 MHz

- FFS: whether scaling factors is applicable

| Company | Y or N | Preferred option | Unacceptable option | Support scaling factor? (Y or N) | Comments (including other options or scaling factor values if supported) |
| --- | --- | --- | --- | --- | --- |
| Intel | Y | Option 2 |  | Yes | Considering 100 MHz of channel BW, we are OK with option 2 (which was our initial intention – corrected our view above), which is preferred compared to other options considering simulation time. |
| ZTE,Sanechps | Y | Option 2 |  | Yes | Seems to be comparable UE number c.f. SL-U simulation |
| OPPO | Y | Option 1 |  | Yes | For the pair-based topology in SL-U unicast traffic, total 12 UE pairs are dropped in the layout per 20MHz, which can be reused as the baseline for SL FR2. Considering simulation complexity with larger number of UE pairs, we are fine with possible scaling factors as FFS. |
| Lenovo | Y | Option 2 |  | Y |  |
| Ericsson | Y | Option 2 |  |  | Keep the FFS for the scaling factor for the moment |
| JHUAPL | Y | Option 2 |  | Y | Option 2 results in a reasonably large UE network at 100 MHz |
| vivo | Y | Option 1 |  | Y | If companies can report the scaling factor, it should be fine to keep 12 UE per 20M |
| Huawei, HiSilicon | Y | Comments |  | Yes | It seems same UE number can be derived based on different combinations of option and scaling factor. For example, Option 1 with scaling factor of 1/2 is equal to Option 2. - In SL-U for unicast traffic, 6 SL-U pairs and 4 NR-U UEs/WiFi nodes per gNB/AP per 20 MHz are deployed. Since there is no WiFi node in SL-FR2 evaluation methodology, NR-U UEs/WiFi nodes can be replaced by SL UEs in the deployment, that is option 1, 12 pairs/20 MHz. - Considering that not all devices in the given area are to operate in FR2 spectrum, it needs to scale down possible number of UEs in the evaluation. 1/3 we think is a reasonable scaling factor for option 1. |

#### Response to Question 1-3

Regarding Question 1-3, 5 out of 7 companies object the cluster-based topology for SL-FR2 evaluation methodology. It seems no consensus can be achieved in this meeting. Proponent companies are encouraged to have offline discussion with other companies. We could treat this issue in the next RAN1 meeting if possible.

#### Proposal 1-5-c

Regarding Question 1-4, all companies support that companies report resource allocation schemes in the simulations. However, one company mentions that there is no agreement is needed in evaluation methodology. FL would like to collect companies’ views whether the following proposed conclusion is necessary.

Proposed conclusion 1-5-c: When reporting the simulation results for sidelink operation on FR2, companies is encouraged to report the used resource allocation scheme.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE,Sanechips | Yes |  |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| Ericsson | Yes |  |
| Nokia, NSB | Yes |  |
| JHUAPL | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes |  |

#### Proposal 1-6-c

Regarding Question 1-5, 4 out of 5 companies prefer ISD=200 meters while one company prefer ISD=500 meters, although majority of the companies can accept the other option. Based on the majority view, FL makes the following proposal:

Proposal 1-6-c: For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, ISD is 200 meters.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE,Sanechips | Yes |  |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| JHUAPL | Yes |  |
| Vivo | Yes |  |
| Huawei, HiSilicon | Yes |  |

#### Proposal 1-7-c

Regarding Question 1-6, the proposals from the companies can be summarized as

            - Qualcomm, Huawei: 60 UEs/cell (ISD=200 m)

            - Intel: 20 UEs/cell (ISD=200 m)

            - Vivo: 10 UEs/cell (ISD=200 m)

Considering that 60 UEs/ cell is supported by more companies and also defined in TR38.802, FL makes the following proposal.

Proposal 1-7-c: For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the number of UEs per cell is 60.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Intel | Yes | We are OK with the proposal. |
| ZTE,Sanechips | Yes | OK. One thing to mention is companies may choose to model 7 sites is to use single site and wrap around the others without necessarily implement the actual channel modelling. This can also reduce the simulation load given 60 UEs per cell for 21 cells seem quite computationally consuming. |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| vivo | See comment | 60 UEs may be not acceptable for some companies’ simulator due to simulation workload. We can also introduce a scaling factor, which can be reported by companies or list multiple candidate value, e.g., 10, 20 ,60. |
| Huawei, HiSilicon | Yes | Ok. And we prefer to choose single site rather than 7 sites in the actual channel modelling. The simulation load given 60 UEs per cell for 21 cells with 7 sites seem quite computationally consuming. |

#### Question

Regarding Question 1-7, 5 out of 5 companies think Uu link and PC5 can use same carrier. Then the question is whether Uu link is included in the simulation. Note that if Uu link is included in the simulation, then there is mutual interference between Uu and PC5.

Question 1-8: In the outdoor layout, do you agree that Uu link with same carrier as PC5 is considered in the simulation?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE,Sanechips | Yes |  |
| Ericsson | No | In our view, the modelling of the interference between Uu and PC5 introduces unnecessary complexity in the simulations. It is not clear the motivation of this scenario. |
| Nokia, NSB | Yes |  |
| JHUAPL | Yes |  |
| Vivo | Yes |  |
| Huawei, HiSilicon | Comment | We are open to have this scenario as optional. We prefer that RAN1 can conclude using different carrier between Uu link and PC5 as the baseline for FR2. |

### [Active] Fourth round discussions

#### [Email] Proposal 1-3-c

In the third-round discussion, 9 out of 9 companies support the proposal.

FL thinks the proposal is stable and will further check in email thread directly before the first check point.

#### Proposal 1-4-d

In the third-round discussion, 8 out of 8 companies support the proposal.

2 companies support Option 1, and 5 companies support Option 2. Additionally, Huawei pointed out Option 2 is scaled down from Option 1 by ½. They also propose the scaling factor of 1/3.

7 companies support scaling factor.

Considering no company supports Option 3 and Option 4, let us remove these two options. Furthermore, we could support both Option 1 and Option 2 via the scaling factor. Hence, the following proposal is made.

Proposal 1-4-d: For the indoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the total number of UEs is 12 pairs/20 MHz with scaling factors of 1, ½ or 1/3.

| Company | Yes or No | Comments |
| --- | --- | --- |

#### [Email] Proposal 1-5-c

In the third-round discussion, 9 out of 9 companies support the proposal.

FL thinks the proposal is stable and will further check in email thread directly before the first check point.

#### [Email] Proposal 1-6-c

In the third-round discussion, 7 out of 7 companies support the proposal.

FL thinks the proposal is stable and will further check in email thread directly before the first check point.

#### Proposal 1-7-d

In the third-round discussion, 5 out of 6 companies support the proposal. Vivo comment that 60 UEs per cell may lead to large simulation workload, and suggest to consider multiple options.

However, as companies mentioned in second round that 60 UEs per cell dropping for dense urban scenario is already defined in TR38.802. This is just re-using the assumption. With this consideration, we may keep the proposal unchanged from Proposal 1-7-c.

Proposal 1-7-d: For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the number of UEs per cell is 60.

| Company | Yes or No | Comments |
| --- | --- | --- |

#### Proposal 1-8-d

Regarding Question 1-7, 5 out of 7 companies agree to support Uu link with same carrier as PC5 is considered in the simulation. Ericsson think this will introduce unnecessary complexity in the simulations and the motivation is unclear. Huawei think this can be considered as optional scenario, while keeping the scenario where different carrier between Uu link and PC5 as the baseline for FR2.

FL think the suggestion from Huawei may the compromised solution.

Proposal 1-8-d: For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, Uu link has different carrier as PC5 in the simulation is the baseline; Uu link has same carrier as PC5 in the simulation is optional.

| Company | Yes or No | Comments |
| --- | --- | --- |

## Topic #2: Channel model

### Background

For the channel model of sidelink operation on FR2 licensed spectrum, the following are proposed by companies.

- Indoor mixed office as in TR38.901: vivo, Apple, ZTE, OPPO, Lenovo, MediaTek, Interdigital, Qualcomm, Nokia, Ericsson, Huawei, HiSilicon

- Channel model as in TR36.843 (Section A.2.1.2) by reusing each component of channel model in TR38.901: LG, OPPO, Xiaomi

- Uma as in TR38.901: CATT, Qualcomm, Apple, Samsung

- Umi as in TR38.901: Huawei, Qualcomm

- Rural as in TR38.901: CATT

- Channel models as in TR37.885: vivo, CATT, CEWiT, Intel

Some channel models are associated with the scenarios. Since the urban grid and highway scenarios from TR37.885 are for V2X use cases, which are not considered in the scope, the corresponding channel models as in TR37.885 are not considered for commercial use cases.

In TR38.901, channel models are defined for multiple scenarios, including Uma, rural, indoor mixed office, etc. Since the channel mode specified in TR38.901 is applicable for 0.5-100 GHz, FL thinks the corresponding procedures and parameters of pathloss, LOS probability, O2I penetration loss, shadowing, as well as fast fading specified in TR38.901 could be reused for SL-FR2.

Associated with Proposal 1-1-a, for indoor scenario, the channel model for InH mixed office as in TR38.901 could be reused.

Associated with Proposal 1-2-a, for outdoor scenario, we could reuse the agreement in Rel-17 sidelink (cf Appendix 6.1). Specifically, reuse the parameters of “Channel models” specified in Section A.2.1.2 of TR36.843 with the following modification: each component of channel model reuses what is specified in TR38.901.

The following table provides a summary of company proposals on this issue:

| Company | Company proposal related to this issue |
| --- | --- |
| Nokia | Proposal 1: Reuse evaluation assumptions defined for NR FR2 studies focusing on indoor deployment scenario and follow commercial traffic models and metrics as agreed for SL-U evaluations as a starting point. |
| Huawei | Proposal 6: Reuse InH mixed office pathloss model as defined in TR 38.901 for indoor scenario as SL-U. Proposal 7: Reuse LOS probability for indoor Mixed-office defined in TR 38.901 as SL-U. Proposal 9: Reuse channel model for Umi – Street Canyon as defined in TR 38.901 for outdoor scenario. |
| Vivo | Proposal 6: The channel model generation procedure and parameters for the indoor scenario defined in TR38.901 can be reused for the link between SL UE and other UE or control node from different RAT. |
| ZTE | Proposal 2: In the evaluation of SL FR2, the NR indoor mixed office channel model defined in TR 38.901 is re-used for the NR inH Mixed Office scenario. |
| OPPO | Proposal 4: Channel model from R17 eSL can be reused for SL operation in FR2 |
| CATT | Proposal 3: Regarding the channel model for enhanced sidelink operation on FR2 licensed spectrum: Channel model defined in TR37.885 and TR38.901 can be reused as much as possible. A suitable channel model for the highway scenario needs to be proposed. Proposal 4: Define vehicle dimensions as provided in Table 8 for V2X evaluations. |
| JHU | Proposal 6: Study modification of scenario parameters to update FR2 sidelink evaluation methodology as it pertains to channel model calibration. The parameter updates include BS height modified to UT height, minimum BS-UT distance values modified to minimum UT-UT distance values, and BS power values modified to UT power values. |
| Xiaomi | Proposal 2: Use the channel models specified in TR 36.843 with each component updated as specified in TR 38.901. |
| Lenovo | Proposal 2: On the evaluation assumptions for SL-FR2, we suggest following aspects: - Carrier frequency: 30GHz and 63 GHz - Reuse the antenna modeling defined in 37.885 - Reuse NR InH Mixed office model with carrier frequency of 30GHz and 63GHz defined in 38.901 - Reusing the agreed traffic model in SL-U for SL-FR2 evaluation including - R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10) - FTP model 3 with arrival rate satisfying one of the followings: - BO Low load: 10%~25% - BO Mid load: 35%~50% - BO High load: above 55% - XR cloud gaming model in TR38.838 - The performance metric including UPT, latency, PRR and RSRP between TX UE and RX UE. |
| MediaTek | Proposal 8: The InH open office channel model defined in TR38.808 is recommended for SL FR2 evaluation. |
| Apple | Proposal 4: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, - For the outdoor layout, the channel model reuses the parameters for Uma specified in TR 38.901. - For the indoor layout, the channel model reuses the parameters for InH mixed office specified in TR 38.901. |
| Interdigital | Proposal 4: Reuse the indoor channel model used in SL-U included in TR 38.901. |
| Samsung | Proposal 8: For evaluation of commercial deployments in SL FR2, use the channel model parameters of urban macro (Uma) as described in TR 38.901 with hUT=hBS=1.5 m. |
| Qualcomm | Proposal 1: For the system-level evaluation of FR2 SL commercial use cases, the dense urban (Uma/Umi-street canyon) and indoor hotspot (InH) scenarios in Section 7.2 of TR 38.901 are used as baseline. For other detailed parameters, the values in the following table can be considered as baseline. |
| Ericsson | - Proposal 7: NR InH Mixed Office model specified in TR 38.901 is used for evaluating SL in FR2. |

### [Closed] First round discussions

#### Proposal 2-1-a

Proposal 2-1-a: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2,

- For the outdoor layout, the channel model reuses the parameters of “Channel models” specified in Section A.2.1.2 of TR 36.843 with following modification

  - Each component of channel model reuses what is specified in TR 38.901.

- For the indoor layout, the channel model reuses the procedures and parameters for InH mixed office specified in TR 38.901.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE, Sanechips | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Ericsson | Yes |  |
| Nokia, NSB | Yes |  |
| JHU APL | Yes |  |
| InterDigital | Yes |  |
| CATT/GOHIGH | Yes |  |
| Samsung | Yes |  |
| Transsion | Yes |  |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes for indoor layout, Comments on outdoor layout | - For the indoor layout, we agree to InH mixed office specified in TR 38.901. - For outdoor scenario, the logic should be clarified. If each component is going to reuse that specified in 901, why we do not simply say reusing the channel model(s) in 901, and there are many channel models specified in 901, like Umi or Uma etc., so which one is used? - we support dense urban with 200ISD dropping model rather than the layout option 3 in Section A.2.1.1 of TR 36.843 with 7 macro sites with 3 cells per site, the reasons are as provided in proposal 1-2-a. Accordingly, we propose to reuse channel model for Umi – Street Canyon as defined in TR 38.901 for outdoor scenario. - I also add our company name to support the indoor channel model, which is missing in the background section before. |
| Vivo | Yes |  |
| Xiaomi | Yes |  |
| Lenovo | Yes |  |
| Qualcomm | Yes |  |
| CEWiT | Yes |  |
| MTK | Yes |  |

### [Closed] Second round discussions

#### Proposal 2-1-b

In the first round discussion, 17 out of 18 companies support the proposal.

@Huawei: Section A.2.1.2 of TR36.843 specifies the framework of channel model, which does not fully follow TR38.901. Since this channel model has already been agreed in Rel-17 NR SL for commercial use cases, it could be reused.

Hence, Proposal 2-1-b remains the same as Proposal 2-1-a.

Proposal 2-1-b: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2,

- For the outdoor layout, the channel model reuses the parameters of “Channel models” specified in Section A.2.1.2 of TR 36.843 with following modification

  - Each component of channel model reuses what is specified in TR 38.901.

- For the indoor layout, the channel model reuses the procedures and parameters for InH mixed office specified in TR 38.901.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes |  |
| Nokia, NSB | Yes |  |
| JHU APL | Yes |  |
| OPPO | Yes |  |
| Qualcomm | Yes |  |
| Lenovo | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes |  |
| Vivo | Yes |  |
| Samsung | Yes |  |
| MTK | Yes |  |
| CEWiT | Yes |  |
| Xiaomi | Yes |  |
| Huawei, HiSilicon | Yes for indoor layout, Comments on outdoor layout | - For the indoor layout, we agree to InH mixed office specified in TR 38.901. - For outdoor scenario, we propose to reuse channel model of UMi - Street Canyon as defined in TR 38.901 for dense urban scenario with 200ISD dropping model. It should be noted the channel models specified in Section A.2.1.2 of TR 36.843 is designed for FR1 spectrum, although it is applied for commercial use case in Rel-17, it is not suitable for FR2 due to following reasons. - FR2 has more severe pathloss - FR2 has smaller coverage requirement |
| InterDigital | Yes |  |

### [Closed] Third round discussions

#### [Email] Proposal 2-2-c

In the second round discussion, 14 out of 15 companies support the proposal. Huawei supports the channel model for indoor layout. However, for outdoor layout, they prefer to use UMi channel model as in TR38.901.

Given this situation, FL proposes splitting the proposal 2-1-b to two: one for outdoor layout and one for indoor layout. FL thinks the following proposal for indoor layout is stable and will further check in email thread directly.

Proposal 2-2-c: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for the indoor layout, the channel model reuses the procedures and parameters for InH mixed office specified in TR 38.901.

#### Proposal 2-3-c

For the channel model for outdoor layout, FL lists two options for companies to further down select.

Proposal 2-3-c: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for the outdoor layout, down-select from the following options

- Option 1: the channel model reuses the parameters of “Channel models” specified in Section A.2.1.2 of TR 36.843 with following modification

  - Each component of channel model reuses what is specified in TR 38.901.

- Option 2: the channel model reuses the procedures and parameters for UMi - Street Canyon specified in TR 38.901.

| Company | Preferred option | Can accept the other option (Y or N) | Comments |
| --- | --- | --- | --- |
| Intel | Option 2 |  | As companies prefer InH mixed office specified in TR 38.901, it may be preferrable to align all channel models with 38.901. |
| Ericsson | Option 1 |  | We have already used Option 1 during Rel-17 for commercial use cases. There is no strong motivation to change the channel model in this release. |
| JHUAPL | Option 2 |  |  |
| vivo | Option 2 |  |  |
| Huawei, HiSilicon | Option 2 |  | We support Option 2. Although Option 1 is applied for commercial use case in Rel-17, it is designed for FR1 spectrum and not suitable for FR2, reason can be concluded as following: - FR2 has more severe pathloss - FR2 has smaller coverage requirement Therefore, Option 2 is preferred. |

### [Active] Third round discussions

#### Proposal 2-3-d

In the third round discussion, 4 out of 5 companies support Option 2. One company support Option 1. One company mentioned that FR2 has severe pathloss and smaller coverage requirement than FR1, and hence, the agreed channel model in Option 1 is not suitable for FR2.

With this, FL suggests taking Option 2.

Proposal 2-3-d: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for the outdoor layout, down-select from the following options

- Option 1: the channel model reuses the parameters of “Channel models” specified in Section A.2.1.2 of TR 36.843 with following modification

  - Each component of channel model reuses what is specified in TR 38.901.

- Option 2: the channel model reuses the procedures and parameters for UMi - Street Canyon specified in TR 38.901.

| Company | Yes or No | Comments |
| --- | --- | --- |

## Topic #3: Antenna model

### Background

For UE antenna model of sidelink operation on FR2 licensed spectrum, the following are proposed by companies:

- Vehicle UE (Option 1) as in TR37.885: vivo, CATT

- Pedestrian UE and cellular UE as in TR37.885: ZTE, OPPO, Interdigital, Samsung, Huawei, Apple, Ericsson

- Table A.2.1-4 as in TR38.802: Nokia

Since the vehicle UE is not considered in commercial use cases, FL suggests not considering vehicle UE antenna model as in TR37.885. The UE antenna model defined in TR38.802 is mainly considered for the communication between UE and gNB, which implies not every configuration fits sidelink operation.

Actually, the antenna array configuration for pedestrian UE and cellular UE in Table 6.1.4-7 of TR37.885 is based on Config (1) plus Config (a) of Table A.2.1-4 as in TR38.802. The antenna element pattern for pedestrian UE and cellular UE as in Table 6.1.4-6 of TR37.885 is equal to Table A.2.1-8 of TR38.802.

Hence, it is recommended by FL to reuse the antenna model for pedestrian UE and cellular UE as in Tables 6.1.4-6 and 6.1.4-7 as in TR37.885 is reused. This is in Proposal 3-1-a.

For the given UE antenna model, OPPO additionally propose to define UE analog beam steering. FL would like to collect companies’ views on the UE analog beam steering, as in Question 3-1.

The following table provides a summary of company proposals on this issue:

| Company | Company proposal related to this issue |
| --- | --- |
| Nokia | Proposal 1: Reuse evaluation assumptions defined for NR FR2 studies focusing on indoor deployment scenario and follow commercial traffic models and metrics as agreed for SL-U evaluations as a starting point. |
| Huawei | Proposal 12: Reuse antenna model as defined in TR 37.885. Baseline Tx/Rx: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2), dH = dV = 0.5 λ,Θmg,ng=90°, Ω0,1=Ω0,0+180°, (dg,H, dg,V)=(0,0). |
| Vivo | Proposal 7: The antenna model option 1 defined in TR37.885 should be taken as baseline. Proposal 8: Capture the antenna model in table 1 to the methodology in the FR2 evaluation. |
| ZTE | Proposal 5: For the antenna configuration of SL-FR2 evaluations, the P-UE and cellular UE antenna characteristics in table 5 and 6 can be re-used. • FFS:TXRU mapping |
| OPPO | Proposal 5: The antenna model defined for pedestrian and cellular UE in TR 37.885 should be reused for SL operation in FR2. Proposal 7: For direction of UE analog beam steering, the following parameters are supported: - Azimuth angle: [-3*pi/8, -pi/8, pi/8, 3*pi/8] - Zenith angle: [pi/4, 3*pi/4] |
| JHU | Proposal 8: Multi-panel antennas that can be used for digital beamforming should be considered in the evaluation model and the simulation results. |
| Apple | Proposal 5: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for UE antenna parameters, reuse - Antenna height in Table 6.1.4-1 in TR 37.885 - Antenna element pattern for pedestrian UE and cellular UE in Table 6.1.4-6 in TR 37.885 - Antenna array configuration for pedestrian UE and cellular UE in Table 6.1.4-7 in TR 37.885. |
| Interdigital | Proposal 3: Reuse pedestrian antenna configuration defined for FR2 included in TR. 37.885. |
| Samsung | Proposal 7: For evaluation of commercial deployments in SL FR2, reuse the antenna element pattern and antenna array configuration of pedestrian and cellular UEs as described in Table 6.1.4-6 and Table 6.1.4-7 of TR 37.885. |
| Ericsson | - Proposal 8: For SL FR2, the antenna array configuration included in TR 37.885 for pedestrian UE is used as a starting point. |

### [Closed] First round discussions

#### Proposal 3-1-a

Proposal 3-1-a: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for UE antenna parameters, reuse the antenna element pattern and antenna array configuration for pedestrian UE and cellular UE as in Table 6.1.4-6 and Table 6.1.4-7 of TR37.885.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanchips | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Ericsson | Yes, with comment | In our view, it is only necessary to re-use the antenna configuration for the pedestrian UEs. |
| Nokia, NSB | Yes |  |
| JHU APL | No | Antenna panels in this table are 2x4 elements on each side, which is 1cm x 2cm. This is a very small antenna size. To enhance performance, next-generation UEs can be designed with more panels and more antenna elements per panel. Propose 2 panels, 2x4 elements each, on each side of the UE (2,4,2,2,2). |
| InterDigital | Yes |  |
| CATT/GOHIGH | Yes |  |
| Samsung | Yes |  |
| Transsion | Yes |  |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes | During Rel-16 V2X SI, simulation assumptions on antenna model for sidelink over FR2 have been defined in TR 37.885, and they can be directly reused in SL-FR2 to avoid repeated definition. |
| vivo | No | In our opinion, the antenna model defined in TR38.808 can be regarded as the start point for the evaluation. |
| Xiaomi | Yes |  |
| Lenovo | Yes |  |
| Qualcomm | Yes |  |
| CEWiT | Yes |  |
| MTK | Yes |  |

#### Question

Question 3-1: Do you think the direction of UE analog beam steering needs to be defined for evaluation methodology?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | No | Prefer to leave to companies’ report if needed |
| OPPO | Yes | Companies should make a common configuration to compare simulation results. |
| Intel | No | As mentioned by other companies we would prefer to leave this up to companies to report. |
| Ericsson | No | This can be left up to the decision of the companies. |
| JHU APL | No | This should be up to the discretion of the companies. |
| InterDigital | No |  |
| Samsung |  | We can leave for companies to report |
| Transsion |  | We can leave it to companies’ report |
| Huawei, HiSilicon | No | Have not justified why such parameter should be considered in sidelink simulation. |
| vivo | No | Each company provides the UE analog beam steering configuration in the simulations |
| Xiaomi |  | It can be up to companies report |
| Lenovo | No | Leaving it up to companies is preferred. |
| Qualcomm | No | Up to each company’s report |
| CEWiT | No | Prefer to leave to companies to have their own algorithms. |
| MTK | No | Considering the different TX UE and RX UEs capabilities, we share the similar view with majority views that it can leave for companies to report, especially c |

### [Closed] Second round discussions

#### Proposal 3-1-b

In the first round discussion, 16 out of 18 companies support the proposal.

@ Ericsson: the antenna configuration is same for pedestrian UE and cellular UE according to Table 6.1.4-7 of TR37885.

@JHU APL: Considering that the antenna array configuration for pedestrian UE/cellular UE has already agreed in TR37.885, it is suggested to reuse the agreed configuration.

@vivo: The antenna model in TR38.808 is more suitable for FR2-2. Since only 30 GHz is assumed, we could just reuse the antenna model in TR37.885.

Hence, Proposal 3-1-b remains the same as Proposal 3-1-a.

Proposal 3-1-b: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for UE antenna parameters, reuse the antenna element pattern and antenna array configuration for pedestrian UE and cellular UE as in Table 6.1.4-6 and Table 6.1.4-7 of TR37.885.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanehcips | Yes |  |
| Nokia, NSB | Yes |  |
| JHU APL | Yes | We would still be interested in enhanced antenna systems for next-generation UEs as the latest version of TR37.885 is from 2019. |
| OPPO | Yes |  |
| Qualcomm | Yes |  |
| Lenovo | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes |  |
| vivo | Yes | . |
| Samsung | Yes |  |
| CEWiT | Yes |  |
| Xiaomi | Yes |  |
| Huawei, HiSilicon | Yes |  |
| InterDigital | Yes |  |

#### Response to Question

In the first round discussion, 14 out of 15 companies do not think the direction of UE analog beam steering needs to be defined. Instead, it is left for companies’ report.

As majority companies mentioned this can be the decision of companies. It is suggested not to define the direction of UE analog beam steering in the evaluation methodology for SL-FR2.

### [Active] Third round discussions

#### [Email] Proposal 3-1-b

In the second round discussion, 14 out of 14 companies support the proposal.

FL thinks the proposal is stable and will further check in email thread directly.

## Topic #4: Traffic model

### Background

For traffic model of sidelink operation on FR2 licensed spectrum, the following are proposed by companies:

- Periodic traffic model 3 as in TR37.885: vivo, Apple, ZTE, OPPO, CATT, Xiaomi, Lenovo, MediaTek, Interdigital, Samsung, Qualcomm, Nokia, Ericsson

- Aperiodic traffic model 2 as in TR37.885: vivo, CATT, Interdigital

- FTP model 3 as in TR36.872: vivo, Apple, ZTE, OPPO, Xiaomi, Lenovo, MediaTek, Qualcomm, Nokia, Huawei

- XR cloud gaming model as in TR38.838: ZTE, Xiaomi, Lenovo, MediaTek, Qualcomm, Huawei, Apple, JHU

- XR VR and AR model as in TR38.838: Huawei

In SL-U for commercial use cases, the periodic model 3, FTP model 3 and XR cloud gaming model, and their mixture are supported. It is recommended by FL that these traffic models are also reused for SL-FR2. This is in Proposal 4-1-a.

As mentioned by proponent companies, the aperiodic traffic model 2 is mainly for V2X use cases. Considering that V2X use cases are out of the scope, FL suggest deprioritizing aperiodic traffic model 2.

Regarding the periodic traffic model 3, it is unclear if packet size should be reduced by a factor like in SL-U. Note that in Rel-17 SL for commercial use case, the periodic traffic model 3 does not consider packet size scaling, while in Rel-18 SL-U, the periodic traffic model 3 has packet size scaling. FL would like to collect companies’ views on the packet size scaling of periodic traffic model 3 for SL-FR2, as in Question 4-1.

Regarding FTP traffic model 3, SL-U supports arrival rate with low/mid/high load by buffer occupancy (BO) of 10-25%/35-50%/above 55%, respectively. The similar arrival rate could be applied to SL-FR2. The packet size of 0.5 Mbytes is considered for FTP model 3. On the other hand, vivo mentions that in B52.6G, FTP traffic model 3 has the packet size of 27 Mbytes, and hence the packet size can be reduced to 2 Mbytes for SL-FR2 due to the resource reservation procedure in sidelink. FL would like to collect companies’ views on the possible packet size of FTP traffic model 3 for SL-FR2, as in Question 4-2.

FL would like to collect companies’ views on supporting AR traffic model and VR traffic model as in TR38.838 for SL-FR2, as in Question 4-3.

The following table provides a summary of company proposals on this issue:

| Company | Company proposal related to this issue |
| --- | --- |
| Nokia | Proposal 1: Reuse evaluation assumptions defined for NR FR2 studies focusing on indoor deployment scenario and follow commercial traffic models and metrics as agreed for SL-U evaluations as a starting point. |
| Huawei | Proposal 10: Reuse FTP3 as defined in TR 36.889 for SL-FR2 as SL-U. Proposal 11: Cloud gaming defined in TR 38.838 is suggested as a starting point for introducing XR traffic models into the assessment/evaluation for sidelink operation on FR2 licensed spectrum, while Virtual Reality and Augmented Reality can also be considered. |
| Vivo | Proposal 9: The data packet size with high intensity should be update by a scale factor, e,g., 0.1 or 0.2 in V2X scenario. Proposal 10: FTP 3 is used for the evaluation of FR2 with the packet size of 0.5M or 2M bytes. |
| ZTE | Proposal 6: For the evaluation of SL-FR2, companies report traffic model from the following options: • R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10) • FTP model 3 with RU satisfying one of the followings: -RU Low load: 10%~25% -RU Mid load: 35%~50% -RU High load: above 55% • XR cloud gaming model in TR38.838 Note : Subchannel instead of RB is used in the RU calculation. The occupied subchannels shall take into account the actual number of UEs mutliplexed on the same resources. |
| OPPO | Proposal 6: For commercial scenario, both FTP model 3 and periodic model 3 should be supported for traffic model. |
| CATT | Proposal 2: Regarding the traffic model for enhanced sidelink operation on FR2 licensed spectrum: - For periodic traffic, the Model 3 (high traffic intensity) in Table 1 should be used. - For aperiodic traffic, the Model 2 (high traffic intensity) in Table 2 should be used. |
| JHU | Proposal 7: Consider XR traffic models for the FR2 sidelink by modifying the models in [5] for FR2 sidelink parameters. |
| Xiaomi | Proposal 3: Traffic model with high throughput and more flexible packet inter-arrival time shall be considered |
| Lenovo | Proposal 2: On the evaluation assumptions for SL-FR2, we suggest following aspects: - Carrier frequency: 30GHz and 63 GHz - Reuse the antenna modeling defined in 37.885 - Reuse NR InH Mixed office model with carrier frequency of 30GHz and 63GHz defined in 38.901 - Reusing the agreed traffic model in SL-U for SL-FR2 evaluation including - R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10) - FTP model 3 with arrival rate satisfying one of the followings: - BO Low load: 10%~25% - BO Mid load: 35%~50% - BO High load: above 55% - XR cloud gaming model in TR38.838 - The performance metric including UPT, latency, PRR and RSRP between TX UE and RX UE. |
| MediaTek | Proposal 7: The existing SL-U traffic model can be reused for the SL FR2 traffic model, e.g., R17 SL commercial traffic with periodic mode 3, FTP mode 3 and XR cloud gaming model in TR38.838. |
| Apple | Proposal 6: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, periodic traffic mode 3, FTP model 3, XR cloud gaming model, and their mixture are supported. For FTP model 3 with arrival rate satisfying (BO low load: 10-25%; BO mid load: 35-50%; BO high load: above 55%) |
| Interdigital | Proposal 2: Reuse periodic and aperiodic traffic models for SL defined in TR 37.885 : - Periodic traffic: Use periodic model 3 - Aperiodic traffic: Use Medium aperiodic traffic |
| Samsung | Proposal 9: For evaluation of commercial deployments in SL FR2, use periodic traffic Model 3 as specified in TR 37.855. |
| Qualcomm | Proposal 2: For the system-level evaluation of FR2 SL commercial use cases, periodic, aperiodic, and/or XR traffic models are considered. |
| Ericsson | - Proposal 6: Support periodic traffic model 3 as specified in TR 37.885. |

### [Closed] First round discussions

#### Proposal 4-1-a

Proposal 4-1-a: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, support at least the following traffic model:

- Option 1: periodic traffic mode 3

- Option 2: FTP model 3 with arrival rate satisfying one of the followings:

  - BO low load: 10%-25%

  - BO mid load: 35%-50%

  - BO high load: above 55%

- Option 3: XR cloud gaming model

- It is up to each company to use either Option 1 or 2 or 3 or mixed of them.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Comment | RU instead of BO should be used |
| OPPO | Yes |  |
| Intel | Yes |  |
| Ericsson | Comment | We are supportive of only having Option 1. |
| Nokia, NSB | Yes |  |
| JHU APL | Yes | These three options cover expected traffic model characteristics. |
| InterDigital | Yes |  |
| CATT/GOHIGH | Yes |  |
| Samsung | Comment | To easy comparison between companies, it better to have one of the options as baseline and the others as optional. We suggestion to make periodic traffic mode 3 as baseline |
| Transsion | Yes |  |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes with comments | We are fine with the traffic models in general, but for the option 3, we think VR and AR traffic models should be also considered. The reason to define cloud gaming only in SL-U is because of the uncertainty to access unlicensed spectrum and limited bandwidth in FR1.However, in licensed bands of FR2, the bandwidth is wide enough to satisfy the requirement of AR and VR and no interference from other RATs (VR/AR have higher requirements on data rate and PDB than cloud gaming). Therefore, AR and VR traffic models can be supported in FR2 and we suggest some modifications on option 3. Option 3: XR cloud gaming traffic models including cloud gaming, virtual reality and augmented reality. |
| vivo | Yes |  |
| Xiaomi | Yes | For cloud gaming traffic model, we would like to clarify whether only DL traffic is considered in SL traffic, or both DL and UL traffic will be considered as SL traffic. |
| Lenovo | Yes |  |
| Qualcomm | Yes |  |
| CEWiT | Yes |  |
| MTK | Yes |  |

#### Questions

Question 4-1: For periodic traffic model 3, do you think packet size scaling is supported for SL-FR2?

| Company | Support packet scaling? | Comments (including possible scaling factors) |
| --- | --- | --- |
| ZTE,Sanechips | Comment | Can be left to companies' report |
| OPPO | Support | The scaling factor of SL-U can be reused, i.e., 1, 5, 10 for low, mid, high traffic load respectively. |
| Intel | Yes |  |
| Ericsson | Comment | Up to the companies to report |
| Nokia, NSB | Yes | In order to have different traffic intensities if Option 1 is used, different scaling can be enabled, as agreed in SL-U AI. |
| JHU APL | Yes | This should be up to the discretion of the companies. |
| InterDigital | Yes | To support low/medium/high traffic loads |
| CATT/GOHIGH | Comment | Should not be mandatory |
| Samsung | Support |  |
| Transsion | Support |  |
| Huawei, HiSilicon | Comment. | Companies can report when scaling is used. |
| vivo | Yes | High volume of traffic load would require an extremely long execution time to complete the simulation according to the previous verification experience, which is very inefficient for evaluation and investigation. Therefore, it is desirable to introduce multiple scale factors to reduce the packet size, e.g., 0.1, 0.2, 0.5, 0.8. |
| Lenovo | Yes |  |
| Qualcomm | Support |  |
| CEWiT | Support | R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10) can be reused. |
| MTK | Comment | Up to the companies to report |

Question 4-2: For FTP traffic model 3, which do you think of the following packet sizes for SL-FR2?

| Company | 0.5 Mbytes | 2 Mbytes | Other packet sizes? | Comments |
| --- | --- | --- | --- | --- |
| ZTE,Sanechips |  |  |  | Can be left to companies' report which option is used |
| OPPO | Support |  |  |  |
| Intel |  |  |  | To reduce simulation time, smaller packet size should be used, but we are OK to leave this up to companies to report. |
| Ericsson |  |  |  | Up to the companies to report |
| Nokia, NSB | Yes |  |  | Keep at least 0.5 Mbytes. Other sizes can be discussed. |
| InterDigital | Yes |  |  | To reduce packet quantization |
| Samsung |  |  |  | Companies can report |
| Transsion | Yes |  |  |  |
| Huawei, HiSilicon | Yes |  |  | Reuse the packet sizes of 0.5Mbytes specified in TR36.889, which is also agreed in SL-U. |
| vivo | Yes |  |  |  |
| Lenovo | Yes |  |  |  |
| Qualcomm | Yes |  |  |  |
| CEWiT |  |  |  | Left to each company and could be reported with the model. |
| MTK |  |  |  | Up to the companies to report |

Question 4-3: For XR traffic, do you think AR traffic model and VR traffic model as in TR38.838 are supported for SL-FR2?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips |  | OK to have the traffic models. |
| Intel |  | We believe that other traffic models could be optionally considered and left up to companies to report. |
| Ericsson | No | No need to include these traffic models. |
| JHU APL | Yes |  |
| InterDigital | Yes | Up to company to mix up different traffic models from options listed above |
| CATT/GOHIGH | No |  |
| Samsung | Comment | Fine if the XR traffic model is optional |
| Transsion | Yes |  |
| Huawei, HiSilicon | Yes | - Cloud Gaming (CG) is expected as dominated application to be widely used, it is suggested as a starting point of introducing XR traffic model into SL-FR2. - Additionally, AR and VR should be also supported for FR2. The PDB is 15ms for single stream CG traffic model, while it is 10ms for single stream AR/VR traffic model as defined in TR36.838. Comparing with unlicensed bands in FR1, licensed spectrum in FR2 could satisfy tighter requirements on service latency, and could support higher traffic model thanks to the larger simulation bandwidth (e.g., 200MHz). Therefore, such VR and AR traffic models can be supported in FR2. |
| Vivo | Yes |  |
| Xiaomi | Yes | We are fine to optionally consider the traffic models. |
| Lenovo | Yes |  |
| Qualcomm |  | We are okay to have them as optional traffic models. |

### [Closed] Second round discussions

#### Proposal 4-1-b

In the first round discussion, 15 out of 18 companies support the proposal in principle.

@ZTE: The replacing BO by RU seems not necessary. Considering that BO is already agreed in SL-U evaluation methodology, hope you could accept BO for the justification of load.

@ Ericsson: Majority companies think that SL-FR2 should not only support periodic traffic, but also support aperiodic traffic. Hope you could accept all three options.

@Samsung: Since all three types of traffic are supported in SL-U without the mark of baseline/option, hope you could accept all three options in the same way.

@Huawei: Please see the discussions of Question 4-3.

Regarding Question 4-1, majority companies support packet size scaling and left it to companies’ reporting. Hence, option 1 in Proposal 4-1-b is modified.

Regarding Question 4-2, 6 out of 14 companies support the packet size for FTP model 3 is up to the companies to report, while 8 out of 14 companies support 0.5 Mbytes packet size. For flexibility, it is suggested that the packet size for FTP model 3 is up to companies’ reporting. Hence, option 2 in Proposal 4-1-b is modified.

Regarding Question 4-3, as majority (11 out of 13) companies are open to AR and VR traffic models in TR38.838, option 3 in Proposal 4-1-b is modified.

Proposal 4-1-b: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, support at least the following traffic model:

- Option 1: periodic traffic mode 3

  - Packet size is up to companies’ reporting

- Option 2: FTP model 3 with arrival rate satisfying one of the followings:

  - BO low load: 10%-25%

  - BO mid load: 35%-50%

  - BO high load: above 55%

  - Packet size is up to companies’ reporting

- Option 3: XR traffic models including cloud gaming model, virtual reality and augmented reality.

- It is up to each company to use either Option 1 or 2 or 3 or mixed of them.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | No | The concept of BO appears only for unlicensed scenario in order to reflect the LBT caused additional transmission latency. RU is used to model different load in various specifications as well as the current ongoing full duplex topic discussion. This clarification is needed. |
| Nokia, NSB | Yes | It may become complicated to compare simulation results with this variability of traffic models, so we would prefer to down-select if possible. One example is latency which is heavily impacted by the packet size and if this is too large, the latency needed for transmission might be significantly higher than the latency of the beam alignment procedure (and hence hides the truly important performance metric for this objective). |
| JHU APL | Yes | We are ok with the additions. |
| OPPO | Yes |  |
| Qualcomm | Yes |  |
| Lenovo | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes | While we are OK with the proposal in principle, we may prefer to limit the number of options or at least to identify one of them to use as a baseline, and others as optional. |
| Vivo | Yes | For the packet size, size scaling should be sufficient - Packet size scaling factor is up to companies’ reporting |
| Samsung | Comments | Leaving it up to UE to choose between the 3 options could make comparison between results difficult. It might be better to have a baseline configuration and additional optional configurations. |
| MTK | Yes |  |
| CEWiT | Yes |  |
| Xiaomi | Yes |  |
| Huawei, HiSilicon | Yes | - For Option 1: Companies can report when scaling is used. - For Option 2: We prefer to reuse the packet sizes of 0.5Mbytes specified in TR36.889 is sufficient, which is also agreed in SL-U. |
| InterDigital | Yes |  |

### [Closed] Third round discussions

#### Proposal 4-1-c

In the second round discussion, 13 out of 15 companies support the proposal. Additionally, after offline checking, ZTE can accept the proposal.

@ Samsung (as well Intel, Nokia): Based on contribution review (cf. Section 3.4.1), 13 companies support Option 1; 10 companies support Option 2; 8 companies support Option 3. Considering the number of supporting companies are similar among these 3 options, it is not easy to select one traffic model as baseline. Note that SL-U also supports all these options equally.

@vivo: Added the scaling factor for Option 1. For Option 2, we do not use scaling factor according to TR38.808.

@Huawei: In second round, almost half of the companies propose to keep the packet size up to companies’ reporting. It seems hard to converge to a single value and let us keep it flexible.

With the above considerations, FL has the minor modification of the proposal.

Proposal 4-1-c: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, support at least the following traffic model:

- Option 1: periodic traffic mode 3

  - Packet size scaling factor is up to companies’ reporting

- Option 2: FTP model 3 with arrival rate satisfying one of the followings:

  - BO low load: 10%-25%

  - BO mid load: 35%-50%

  - BO high load: above 55%

  - Packet size is up to companies’ reporting

- Option 3: XR traffic models including cloud gaming model, virtual reality, and augmented reality.

- It is up to each company to use either Option 1 or 2 or 3 or mixed of them.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Intel | Comments | Our preference is still to down-select some of them or at the very least identify one of the as baseline and leave others as optional. |
| ZTE,Sanechips | Yes |  |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| Ericsson | Yes |  |
| Nokia, NSB | Yes |  |
| JHUAPL | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes |  |

### [Active] Fourth round discussions

#### [Email] Proposal 4-1-c

In the third-round discussion, 8 out of 9 companies support the proposal, and there is no objection to the proposal.

FL thinks the proposal is stable and will further check in email thread directly before the first check point.

## Topic #5: Performance metric

### Background

For performance metric of sidelink operation on FR2 licensed spectrum, the following are proposed by companies:

- UPT: vivo, Apple, ZTE, OPPO, Xiaomi, Lenovo, MediaTek, Interdigital, Samsung, Qualcomm, Ericsson, Huawei, HiSilicon

- Latency: ZTE, Lenovo, Apple, MediaTek, Huawei, HiSilicon

- PRR: vivo, Apple, ZTE, CATT, Xiaomi, Lenovo, MediaTek, Interdigital, Samsung, Qualcomm, Huawei, HiSilicon

- PIR: CATT, Xiaomi, Interdigital, Samsung

- Max. number of UE pairs with satisfying XR services: Xiaomi

- CDF of wideband SINR with beamforming: Intel

- Time for beam acquisition: Intel

In SL-U for commercial use cases, UPT, latency and PRR are supported as performance metric. It is recommended by FL that these performance metrics are also reused for SL-FR2. This is in Proposal 5-1-a.

Some companies propose to also consider PIR for SL-FR2. FL would like to collect companies’ views on the PIR for SL-FR2, as in Question 5-1.

Since the other performance metrics are proposed by single company, they are currently not put in the proposal or question unless concerns are raised.

The following table provides a summary of company proposals on this issue:

| Company | Company proposal related to this issue |
| --- | --- |
| Huawei | Proposal 13: Reuse performance metrics on PRR as defined in TR 37.885, and UPT and latency as defined in SL-U for SL-FR2 evaluations. |
| Vivo | Proposal 11: UPT and PRR are considered as the performance metric. |
| ZTE | Proposal 7: The following can be considered as performance metrics for FR2 evaluation: • PRR • Latency • Mean, 5% and CDF of UPT |
| OPPO | Proposal 8: For performance metric, perceived user throughput (UPT) should be supported for evaluation of SL operation in FR2. |
| CATT | Proposal 5: Performance metric defined in TR 37.885 can be reused, especially PRR type 2 and PIR type 2. Whether to use PRR type 1 and PIR type 1 needs further study. |
| Intel | Define the following performance metrics for the SL beam management evaluations: CDF of wideband SINR with beamforming Time for beam acquisition |
| Lenovo | Proposal 2: On the evaluation assumptions for SL-FR2, we suggest following aspects: - Carrier frequency: 30GHz and 63 GHz - Reuse the antenna modeling defined in 37.885 - Reuse NR InH Mixed office model with carrier frequency of 30GHz and 63GHz defined in 38.901 - Reusing the agreed traffic model in SL-U for SL-FR2 evaluation including - R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10) - FTP model 3 with arrival rate satisfying one of the followings: - BO Low load: 10%~25% - BO Mid load: 35%~50% - BO High load: above 55% - XR cloud gaming model in TR38.838 - The performance metric including UPT, latency, PRR and RSRP between TX UE and RX UE. |
| MediaTek | Proposal 9: The PRR, latency and UPT are suggested as performance metric for SL FR2 evaluation. If the XR traffic model is evaluated, the UE satisfaction and system capacity as defined in TR38.838 is used as performance metric. |
| Apple | Proposal 7: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, performance metric includes UPT, latency and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure. |
| Interdigital | Proposal 5: Consider at least the following performance metrics defined in TR37.885 - Packet reception ratio (PRR) for SL devices - Packet inter-reception ration (PIR) for SL devices - User Perceived Throughput (UPT) is optional. |
| Samsung | Proposal 10: For evaluation of commercial deployments in SL FR2, consider the following evaluation metrics: - 5%, and CDF of user throughput. - Packet reception ratio (PRR). - Packet inter-reception (PIR). |
| Qualcomm | Proposal 3: FR2 For the system-level evaluation of FR2 SL commercial use cases, application-specific performance metrics are selected among the following candidates: User throughput or UPT/latency Per-cell (e.g., for SL relaying use case) Per-UE (or UE-group) PRR (if applicable) |
| Ericsson | - Proposal 9: User perceived throughput (UPT) is used as a performance metric for SL in FR2. FFS other metrics. |

### [Closed] First round discussions

#### Proposal 5-1-a

Proposal 5-1-a: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, performance metric includes UPT, latency and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Ericsson | No | In our view, it is enough with UPT. We are not convinced about the usefulness of latency and PRR in the case of SL FR2 where we are modeling beam management procedures. |
| Nokia, NSB | Yes |  |
| JHU APL | Yes |  |
| InterDigital | Yes |  |
| CATT/GOHIGH | Yes |  |
| Samsung | Yes |  |
| Transsion | Yes |  |
| Huawei, HiSilicon | Yes | We are fine with the metrics, PRR, UPT, and latency in current proposal which is reused from SL-U. BTW, I also add our company name to support the metrics, which is missing in the background. |
| vivo | Yes |  |
| Xiaomi | Yes | For XR traffic, we would like to clarify whether PDB defined in XR study will be reused here. |
| Lenovo | Yes |  |
| Qualcomm | Yes | We think UPT and latency are enough. PRR can be optional. |
| CEWiT | Yes |  |
| MTK | Yes with suggestion | As we indicated in our contribution, the performance metric is not only relevant with deployment scenario, but also have relationships with traffic models. For the traffic model discussion, almost all companies support XR cloud gaming traffic model as discussed in Proposal 4-1-a, Thus, we suggest adding a new performance traffic for XR traffic evaluation, e.g., the UE satisfaction/system capacity as section 7.2 in TR 38.838, which also is aligned with NR-U evaluation metric discussion as copied following. Agreement for SL -U: - Performance metric: UPT, latency, and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure. - FFS: UE satisfaction/system capacity as section 7.2 in TR 38.838 for XR traffic evaluation So, we suggest the proposal 5-1-a can be modified as following: Proposal 5-1-a: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, performance metric includes UPT, latency, and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure and UE satisfaction/system capacity as section 7.2 in TR 38.838 for XR traffic evaluation. |

#### Question

Question 5-1: For performance metric, do you think if PIR should be supported for SL-FR2?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | No | No need to consider PIR additionally. |
| OPPO | No | SL operation on FR2 spectrum is expected to obtain high data rate/throughput for commercial cases, so no need to consider PIR. |
| Ericsson | No | It is enough with UPT. |
| Nokia, NSB | No | Not essential for commercial use cases. |
| InterDigital | Yes | To show communication reliability in FR2 |
| CATT/GOHIGH | Yes | Similar view as interdigital |
| Samsung | Yes |  |
| Transsion | Yes |  |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | No | PRR, UPT and latency are enough to measure the performance from multiple perspective such latency and reliability. So no need to consider PIR additionally. |
| vivo | Yes |  |
| Xiaomi | Yes |  |
| Lenovo | No |  |
| Qualcomm | No |  |
| MTK | No |  |

### [Closed] Second round discussions

#### Proposal 5-1-b

In the first round discussion, 17 out of 18 companies support the proposal in principle.

@Ericsson: Majority of companies think all the 3 performance metrics are needed, where both latency and reliability (PRR) are important metrics for SL-FR2. Hope you could accept all these performance metrics.

@Xiaomi: Yes, it is FL’s understanding that PDB defined in XR is used for XR traffic.

@Qualcomm: Considering all 3 performance metrics are not optional in SL-U, it is preferred to keep all these performance metrics as in SL-U.

@MTK: UE satisfaction/system capacity is still FFS in SL-U. We may not consider them unless it is agreed in SL-U.

Proposal 5-1-b: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, performance metric includes UPT, latency and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE,Sanechips | Yes |  |
| Nokia, NSB | Yes |  |
| JHU APL | Yes |  |
| OPPO | Yes |  |
| Qualcomm | Yes |  |
| Lenovo | Yes |  |
| Ericsson |  | We are not convinced about the usefulness of PRR and latency metrics but we can compromise based on majority views. |
| Intel | Yes |  |
| Vivo | Yes |  |
| Samsung | Yes |  |
| MTK | Yes with suggestion | Thanks for the FL’s reply. Based on the FL’s comments, it is better to add an FFS to reflect the XR service’s performance metric, which is similar with SL’s agreement. - FFS: UE satisfaction/system capacity as section 7.2 in TR 38.838 for XR traffic evaluation |
| CEWiT | Yes |  |
| Xiaomi | Yes |  |
| Huawei, HiSilicon | Yes |  |
| InterDigital | Yes |  |

#### Response to Question

In the first round discussion, 8 out of 15 companies do not consider PIR as additional performance metric, while 7 out of 15 companies think PIR should be used as performance metric for SL-FR2. Some companies mention PIR can show communication reliability in FR2, while other companies think PRR, UPT and latency are enough to show communication reliability in FR2.

It seems companies’ view are split. It is suggested to discuss PIR after some progresses have been made for Proposal 5-1-b.

### [Closed] Third round discussions

#### Proposal 5-1-c

In the second round discussion, all companies can accept the proposal. Additionally, MTK proposes to add the FFS for UE satisfaction/system capacity as Section 7.2 in TR38.838 for XR traffic evaluation. Since the similar FFS is used for SL-U, we could add this FFS for SL-FR2.

Proposal 5-1-c: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, performance metric includes UPT, latency and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure.

  - FFS: UE satisfaction/system capacity as section 7.2 in TR 38.838 for XR traffic evaluation

| Company | Yes or No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE,Sanechips | Yes | For the FFS point, the system capacity shall take into account only the paired UEs and the Tx/Rx links shall be both considered. Thus the capacity may be link perspective instead of UE perspective. |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| Ericsson | No. Delete FFS | In our view, there is no need to include the FFS. Based on the XR definition of these parameters in TR 38.838 (UE satisfaction and system capacity), the PRR fulfils a really similar role, and therefore, no additional metric is needed. |
| Nokia, NSB | Yes |  |
| JHUAPL | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes with removing FFS | For the FFS points, we think they are not needed. The XR service is per PC5 link in SL scenario, and there is no cell concept in term of capacity, so it is difficult to have a capacity metric in XR. On the UE satisfaction metric, it measures the quality of a link, however, PRR already reflects the quality of PC5 link. Therefore, no additional performance metric is needed and the FFS should be deleted. |

### [Active] Fourth round discussions

#### [Email] Proposal 5-1-b

In the second round discussion, all companies can accept the proposal. Only MTK proposes to add the FFS for UE satisfaction/system capacity as Section 7.2 in TR38.838 for XR traffic evaluation.

However, this FFS is opposed by 2 companies in the third round. One reason is that XR service is per PC5 link and there is no system capacity concept. Also, UE satisfaction metric is reflected by PRR.

Based on this, FL would like to switch back to the original proposal 5-1-b. Note that this proposal already got no objection in the second round. Hence, FL thinks the proposal is stable and will further check in email thread directly before the first check point.

Proposal 5-1-b: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, performance metric includes UPT, latency and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure.

## Other topics

The following table provides a summary of company proposals on other topics:

| Company | Company proposal related to this issue |
| --- | --- |
| Qualcomm | Proposal 4: For the system-level evaluation of FR2 SL commercial use cases, path/angle-specific blockage models in Section 7.6.4 of TR 38.901 are used as baseline. Proposal 5: For the link-level evaluation of FR2 SL commercial use cases, the simulation assumptions in the following table can be considered as baseline. Proposal 6: For the link-level evaluation of FR2 SL commercial use cases, the performance metrics in the following table can be used for alignment and calibration across companies. |
| CEWiT | Proposal 1: Evaluation of S-SSB and SL -CSI-RS for SL beam enhancement on FR2 including the reporting mechanism should performed in Rel 18 study. Proposal 2: Link level and system level simulations should be performing for evaluating the beam enhancement on FR2. |
| LG | Proposal 3: For SL operation on FR2, further discussion is needed on how to model the inter-carrier interference (ICI) due to the channels/signals received outside of the CP region. |
| Nokia | Proposal 2: For resource allocation mode 2, random resource selection is assumed for the NR SL FR2 BM evaluations. |
| Intel | No SL FR2 LLS evaluation methodology is defined |
| JHU | Proposal 9: Number of simultaneous beams together with the respective beam isolations should be used in the evaluation methodology and simulation results. |
| ETRI | Proposal 1: When updating evaluation methodology for enhanced sidelink operation on FR2, the following considerations should be discussed: - New bandwidth-demanding use cases, e.g., exchanging raw sensor data for extended sensor use case, delivering multiple streams of 4K/8K video for in-vehicle entertainment, etc. - New UE connection topology such as sidelink relaying - Coexistence of Uu and sidelink on FR2 licensed band |

Besides the proposals and questions raised in the first round in previous sections, if you think other topics need to be discussed, please raise them in the following table?

| Company | Comments |
| --- | --- |
| ZTE,Sanechips | Prefer to discuss whether to take into account the beamforming gain with the issue of RSRP threshold for unicast pairing. In sidelink evaluation, the minimum association RSRP for a unicast pair should still be guaranteed above a threshold. In case the threshold is not met, the received signal may be buried in noise or interference so that no efficient communication could be performed. For SL-FR2, there exists three approaches to model the minimum association RSRP, out of which companies shall report the one adopted in evaluations. - The calculation of minimum association RSRP takes into account both the Tx and Rx beamforming gain - The calculation of minimum association RSRP takes into account only the Tx beamforming gain - The calculation of minimum association RSRP takes into account neither the Tx beamforming nor Rx beamforming gain In terms of the modeling for beamforming gain, there exists the following two options, - Option 1 is to apply an offset to the association RSRP which shall be determined based on the TXRU mapping and number of antenna elements - Option 2 is to apply the realistic beamforming gain of the actual best matched beam after beam training procedure Proposal In the evaluations for SL FR2, the minimum association RSRP shall be guaranteed above a threshold for unicast pairing wherein the calculation of minimum association RSRP shall be one of the following - The calculation of minimum association RSRP takes into account both the Tx and Rx beamforming gain - The calculation of minimum association RSRP takes into account only the Tx beamforming gain - The calculation of minimum association RSRP takes into account neither the Tx beamforming nor Rx beamforming gain |
| Nokia, NSB | RAN1 needs to discuss what are the simulation assumptions on resource allocation (Mode 1, Mode 2) for PSCCH/PSSCH transmission. For Mode 1, the network allocates resources for PSCCH/PSSCH transmission in a centralized fashion. But it is unclear how to do so if the network does not have knowledge of the actual interference potential among the different unicast links (including SL and UL) in the system, which is a function of the beams used for transmission and/or reception. For Mode 2, we propose to use random resource selection, as the sensing-based resource exclusion behavior currently specified in clause 8.1.4 of TS 38.214 does not cause meaningful UE behavior when PSCCH/PSSCH transmissions are not omnidirectional. |
| Huawei, HiSilicon | We recommend to discuss the number of UEs in outdoor scenario for FR2 SL, which should satisfy that the UE density is lower than that in indoor scenarios. |
| Huawei, HiSilicon2 | For the number of UEs in outdoor scenario for SL-FR2, we recommend that the UE density is lower than that in indoor scenarios. |

# Outcomes of RAN1 #110bis-e meeting

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2

- Reuse indoor layout defined for SL-U with pairs topology and without WiFi nodes

  - FFS: total number of UEs deployed in the layout

  - Companies should report how UEs are paired

- FFS: whether to consider the cluster-based topology defined for SL-U

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse layout option 3 in Section A.2.1.1 of TR 36.843 with

- Option 1: 7 macro sites with 3 cells per site

- Option 2: a single site

- Companies should report how UEs are paired

- FFS: total number of UEs deployed in the layout

- FFS: whether Uu and PC5 use same carrier

- FFS: ISD for this layout option 3

# References

- RP-221938, “WID revision: NR sidelink evolution,” Sept. 2022.

- R1-2208359, On Evaluation Methodology for Sidelink in FR2	Nokia, Nokia Shanghai Bell

- R1-2208451, Evaluation methodology for sidelink operation on FR2 licensed spectrum	Huawei, HiSilicon

- R1-2208557, Discussion on enhanced sidelink operation on FR2 licensed spectrum	 Spreadtrum Communications

- R1-2208646, Enhanced sidelink operation on FR2 licensed spectrum	vivo

- R1-2208725, Discussion on the evaluation methodology for SL on FR2	ZTE, Sanechips

- R1-2208825, On updating evaluation methodology for SL operation in FR2	OPPO

- R1-2208979, Discussion on the evaluation methodology for enhanced sidelink operation on FR2 licensed spectrum	CATT, GOHIGH

- R1-2209057, Evaluation Methodologies for SL Operating in FR-2 licensed band	Intel Corporation

- R1-2209082, Discussion on evaluation methodology for commercial deployment scenario	Johns Hopkins University APL

- R1-2209289, Discussion on updating the evaluation methodology for commercial deployment scenario	xiaomi

- R1-2209304, Discussion on evaluation methodology for sidelink on FR2	Lenovo

- R1-2209407, Discussion on evaluation methodology for enhanced SL operation on FR2	ETRI

- R1-2209481, Discussion on evaluation methodology for sidelink on FR2 licensed spectrum	LG Electronics

- R1-2209531, Discussion on evaluation methodology for SL FR2 licensed spectrum	MediaTek Inc.

- R1-2209588, Evaluation Methodology for Sidelink Operation on FR2 Licensed Spectrum	Apple

- R1-2209638, On enhanced SL FR2 operation	InterDigital, Inc.

- R1-2209734, Evaluation Methodology for Enhanced SL Operation in FR2	Samsung

- R1-2209988, Enhanced sidelink operation on FR2 licensed spectrum	Qualcomm Incorporated

- R1-2210110, Discussion on evaluation methodology for beam enhancement on sidelink FR2 operation	CEWiT

- R1-2210134, Evaluation methodology for sidelink in FR2	Ericsson

# Appendix (agreements related to evaluation methodology for commercial deployment scenario in past RAN1 meetings)

## RAN1#103-e meeting

Agreements:

For the public safety and commercial use cases, reuse the parameters of “Reference system deployments” specified in Section A.2.1.1 of TR 36.843 with following modification:

- Carrier frequency:

  - Include 3.5 GHz for commercial use case (optional)

- System bandwidth:

  - Include 40 MHz for commercial use case (optional) and 20 MHz dedicated spectrum for out-of-coverage scenarios (optional)

- “eNB” is replaced by “gNB”

- FFS any refinement/variation is necessary, e.g., 19 vs. 7 sites, etc.

Agreements:

For the public safety and commercial use cases, reuse the parameters of “Channel models” specified in Section A.2.1.2 of TR 36.843 with following modification:

- Each component of channel model reuses what is specified in TR 38.901.

Agreements:

      - For the layout for public safety and commercial use cases, support “7 macro sites with 3 cells per site in the layout”

Agreements:

- For public safety and commercial use cases, at least following option is supported for UE RF parameters:

  - Reuse the number of TX AP, the number of RX AP, antenna gain for P-UE specified in TR 37.885.

Agreements:

- For public safety and commercial use cases, one OFDM symbol of NR SL slot is used for AGC

Agreements:

For public safety and commercial use cases, at least performance metrics for communication specified in A2.1.4.2 of TR 36.843 are reused with following modification:

- “FTP2 traffic model” is replaced with “FTP traffic model or periodic traffic model”

- Power consumption model agreed in R-17 NR sidelink enhancement WI is used

- the metrics for latency and WAN are not needed

Agreements:

- For public safety and commercial use cases, reuse in-band emission model used for NR V2X specified in section 6.4E.2.4 in TS 38.101

Agreements:

- For commercial use case, at least following option is supported for traffic model:

- Option 7: Periodic traffic model 3 specified in TR 37.885

## RAN1#104-e meeting

Agreements:

- For commercial use case, at least following layout options are supported:

  - Option 3 of TR 36.843: Urban macro (500m ISD) (all UEs outdoor)

    - UE dropping as in Table A.2.1.1-1

      - All UEs are outdoors UEs

  - Option 1: Urban macro (500m ISD) + 1 RRH/Indoor Hotzone per cell for optional

    - UE dropping as in Table A.2.1.1-1

      - Mix of outdoor and indoor UEs

  - Option 5 of TR 36.843: Urban macro (1732m ISD) for optional

    - UE dropping as in Table A.2.1.1-1

      - All UEs are outdoors UEs

      - Mix of outdoor and indoor UEs

## RAN1#110 meeting

Agreement

The following evaluation scenario can be used for evaluating performance of SL-U designs, resource allocation schemes, and coexistence study with another RAT in a shared channel.

- Scenario 1 (commercial use cases) – recommended:

  - Evaluation methodology baseline is NR-U from TR 38.889 with the following updates.

  - Indoor layout

    - Option 1: a pairs topology for SL-U from R1-2205033 – recommended

      - a = 20m, b = 60m, c = 20m, d = 80 m

      - There are two operators to model two RATs at a time. The red one is SL-U UE, the blue one is Wi-Fi or NR-U.

      - For NR-U / Wi-Fi, the same number of UEs / Wi-Fi STA as the total number of SL-U devices are dropped in the area. The NR-U UE / Wi-Fi nodes are dropped uniformly per gNB/AP per 20 MHz.

        - Companies should report if they used a different number of UEs / Wi-Fi STA as the total number of SL-U devices, as an additional evaluation scenario.

      - For evaluation of unicast traffic, the topology of SL-U is pair topology and the SL-U UEs are dropped uniformly at random in the area.

        - Companies should report how SL-U UEs are paired

        - 6 SL-U pairs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz

      - For evaluation of groupcast traffic, SL-U UEs are dropped uniformly at random in the area, SL-UEs form groupcast UE group based on TX-RX UE distancing, the distance is provided by each company.

        - Companies should report how SL-U UEs form a group

        - 12 SL-U UEs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz

      - For evaluation of broadcast traffic, SL-U UEs are dropped uniformly at random in the area.

        - 12 SL-U UEs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz

    - Option 2: SL UE clusters (R1-2203146)

      - Indoor layout and UE dropping model with N = 3 or 6 clusters and each with M=5 UEs

      - Each cluster is a circle, with a central point and radius Rmax = 15 or 10m and Rmin = 5 or 1m

      - No overlapping among the N clusters

      - For coexistence, there are two operators to model two RATs at a time, where the red one is Wi-Fi AP or NR-U gNB. NR-U UE / Wi-Fi STA are dropped uniformly per gNB/AP.

      - Simulation bandwidth can be larger than 20MHz (e.g., 80MHz)

  - Channel model follows NR InH Mixed Office model used in NR-U (TR38.889)

  - Traffic model

    - Option 1: R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10)

      - FFS whether/how the PDB requirement can be captured

    - Option 2: FTP model 3 with arrival rate satisfying one of the followings:

      - BO Low load: 10%~25%

      - BO Mid load: 35%~50%

      - BO High load: above 55%

    - Option 3: XR cloud gaming model in TR38.838

      - FFS whether/how the PDB requirement can be captured

    - It is up to each company to use either Option 1 or 2 or Option 3 or mixed of them

  - Interference model:

    - Layout option 1: Explicit modelling of NR-U / WiFi transmissions (as per TR38.889)

    - Note, for the interference traffic model:

      - The same or equivalent traffic model setting as SL-U should be used as much as possible to achieve equal load (e.g., SL-U RAT offered load equal the interfering RAT’s offered load).

      - The same number of traffic flows should be used between SL-U and the interfering RAT (e.g., 10 UEs with 10 flows, and 5 STAs with 2 flows each, one for DL and one for UL)

        - Companies should report if they used a different assumption, as an additional evaluation scenario.

  - Performance metric: UPT, latency, and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure.

    - FFS: UE satisfaction/system capacity as section 7.2 in TR 38.838 for XR traffic evaluation

    - FFS for groupcast and broadcast

  - Fair coexistence criterion between SL-U and the interfering RAT (e.g., according to NR-U TR38.889)