3GPP TSG RAN WG1 Meeting #110bis-e	R1-2210383

eMeeting, October 10-19, 2022

Agenda Item:	9.1.3.2

Source:	Moderator (Futurewei)

Title:	FL Summary #2 on SRS enhancements

Document for:	Discussion and decision

# Introduction

In RAN#94-e, a new Work Item for Rel-18 on “MIMO Evolution for Downlink and Uplink” was approved, and the motivations, scopes, and objectives were agreed in [1]. Among the objectives, the underlined in the following are related to SRS enhancements, mainly in the aspects of SRS for TDD Coherent Joint Transmission (CJT or C-JT) and 8 Tx operation:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

25 contributions [2-26] have been submitted to Agenda Item 9.1.3.2 of RAN1#110bis-e on SRS Enhancements targeting TDD CJT and 8 Tx operations. Main views and further discussion points based on these contributions are collected in this document. Any additional inputs from any company can also be provided in this document.

# SRS enhancements to manage inter-TRP cross-SRS interference targeting TDD CJT

In RAN1#109-e, 11 categories of potential SRS enhancements to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization were agreed for further study. Some of the categories also include sub-categories of different schemes. Prioritization or down-selection of the categories, if any, should be based on good technical understanding of the potential enhancements. In RAN1#110, the group was focused on technical aspects of the enhancements, especially on clarifying the key ideas of the enhancements, analyzing pros (e.g., performance benefit, necessity, the problems that they can solve) and cons (e.g., limitations, issues to addressed), etc. Based on these discussions, the group should have achieved common understanding of the technical aspects.

Please provide your inputs on the following questions for the enhancements in the subsections:

Q1: Description/clarification of the enhancement

- Most of the enhancements should be well understood by most companies by now. If you still feel the key idea of the enhancement is not clear enough, please ask for clarification from the proponents.

- The proponents can answer the questions and can provide any additional descriptions to make the enhancement clearer to the group.

Q2: Relation to other enhancement(s)

- For some enhancements, it may be related to some other enhancements. For example, comb offset hopping enhancement (as in Sec. 2.1) is also related to resource mapping based on network-provided parameters or system parameters such as time indexes (as in Sec. 2.11). If, say, Sec. 2.11 is not agreed but comb offset hopping is agreed, then confusion may arise on how to support comb offset hopping.  For another example, both Sec. 2.4 and Sec. 2.8 may be relevant to the enhancement of dynamic indication of power control parameters proposed by some companies.

- Please clearly indicate the relation of one enhancement to other enhancements.

Q3: Pros and cons of the enhancement

- Please analyze pros (e.g., performance benefit, necessity, the problems that they can solve, use cases, potential source of gains, etc.) and cons (e.g., limitations, issues to addressed, etc.). Possible alternative solutions can also be suggested.

Q4: The next study point / decision point for the enhancement

- Please suggest what the next study point and/or decision point for the enhancement would be. For example, additional evaluation results comparing certain schemes or with certain assumptions, the next level of details to be agreed if the group plans to move forward with the enhancement, etc.

- FL proposal may be provided by summarizing the inputs on the study point / decision point of the enhancement.

A list of proposed schemes under each category is provided, together with a list of proponents. Please feel free to suggest if something is missing or inaccurate.

## Randomized frequency-domain resource mapping for SRS transmission

At least the following potential enhancements have been discussed:

A1: Further enhancements to frequency hopping (e.g., Futurewei, Samsung, ZTE; 3 proponents)

A1.1: Different bandwidths in FH (e.g., Futurewei, Samsung, ZTE)

Pros: Interference randomization, SRS capacity enhancement

Cons: Non-uniform SRS sample pattern in time/frequency domain

A1.2: More SRS bandwidth configurations for FH (e.g., Futurewei)

Pros: Interference randomization, SRS capacity enhancement

A2: Comb hopping

A2.1: Comb offset hopping (e.g., CMCC, Ericsson, Futurewei, Huawei, HiSilicon, Lenovo, Qualcomm, Spreadtrum, Xiaomi, Samsung, Intel; 11 proponents)

Pros: Interference randomization; already specified in existing standards for positioning SRS

These enhancements may be related to those in Sec. 2.11.

Based on the large number of supports, the FL suggests to focus on further discussion on comb offset hopping and provide views on the next study point / decision point. In addition, Q1~Q4, e.g., clarification/pros/cons, can be discussed for all above enhancements.

| Company | View |
| --- | --- |
| QC | Support A2.1, which is easier compared to other enhancements in this category. |
| Google | Open to A2.1. |
| OPPO | For comb hopping, we still have concern on the compatibility with legacy UE and the loss of scheduling flexibility due to some reserved comb for enhanced UEs. Also, with ideal backhaul for CJT, we think the comb collision can be controlled by gNB. |
| Huawei, HiSilicon | Open to A2.1. |
| MediaTek | Fine with A2.1 |
| Xiaomi | Open to A2.1. A2.1 and K1 can be jointly discussed. |
| ZTE | Same view as OPPO. Especially when both comb offset hopping and CS hopping are supported, the co-scheduling between legacy UE and new UE should be concerned. |
| Spreadtrum | Support A2.1 |
| Lenovo | Fine with A2.1. We think the compatibility with legacy UE can be handled by gNb’s realization. |
| CMCC | Open to A2.1. |
| Samsung | Support A1.1 and A2.1. In our view, A1.1 can randomize interference depending on how to determine FH patterns. Different BWs in FH can differentiate/randomize cross-SRS interference. |
| Intel | Support A2.1 |
| LGE | Similar view as OPPO. |
| Nokia/NSB | We are open to A.2.1, however, the issue raised by OPPO should be clarified. |
| FL | @OPPO, @ZTE, @LGE, @Nokia/NSB: Agree with Lenovo that the multiplexing with legacy UE can be handled by gNB implementation. Please note that positioning SRS already supports comb offset hopping, so the multiplexing with legacy UE when comb offset hopping is utilized can already be handled by existing gNB using essentially the existing implementation. So far there are 11 proponents for comb offset hopping, and a number of companies are open to it. The FL thus suggests the following potential proposal: Potential Proposal: Extend comb offset hopping defined for SRS-PosResource to SRS-Resource. |
| vivo | As majority companies support comb offset, we are ok to study it with high priority. However, whether/how to support it in Rel-18 need more discussion an evaluation, since there are other potential solutions, such as CS hopping. Therefore, we prefer to modify the proposal as follows. Potential Proposal: Study comb offset hopping with high priority, e.g., extend comb offset hopping defined for SRS-PosResource to SRS-Resource. |
| QC | “Extend comb offset hopping defined for SRS-PosResource” is not needed. For positioning SRS, there is no randomness. However, the purpose here is interference randomization. We suggest: Potential Proposal: Support comb offset hopping for SRS. - The comb offset is determined pseudo-randomly as a function of time (e.g., slot number, symbol number) with a certain UE-specific initialization. - FFS: Other details |
| Ericsson | We are open to continued/prioritized study of A2.1. However, we think it is too early to decide on a specific hopping pattern. Indeed, the existing hopping pattern for SRS for positioning is valid only for a single-port SRS resource and may not be suitable for a mTRP scenario. |

### FL update

@vivo: Thank you for the suggestion. Since there is a large number of proponents, I think we can see if “Support comb offset hopping” is agreeable.

@QC: Thank you for the suggestion. Positioning SRS defines different koffsetl' values for different symbol indexes (see TS 38.211, Table 6.4.1.4.3-2: The offset koffsetl' for SRS as a function of KTC and l'.), so I think it may be viewed as a type of hopping/randomization. On the other hand, what you suggested can also be discussed.

@Ericsson: Thanks for the good point. This can be part of the FFS of other details.

Proposal 2.1: Support at least one of the following options for comb offset hopping for SRS.

- Option 1: Extend comb offset hopping as a function of symbol index defined for SRS-PosResource to SRS-Resource.

- Option 2: The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) with a certain UE-specific initialization.

- FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

| Company | View |
| --- | --- |
| QC | We do not think listing option 1 and option 2 is helpful. As commented before, koffsetl' in SRS for positioning is not for randomization (it is for SRS staggering because all REs need to be sounded for the purpose of enhanced positioning). In other words, if 2 UEs transmit SRS in the same set of symbols using SRS-PosResource, there is no randomization between them (if the initial comb offset is the same they collide in all symbols, and if it is different they are FDMed in all symbols). Can FL clarify how option 1 achieves randomization? |
| ZTE | For SRS positioning SRS, only comb offset hopping is supported and CS hopping is not supported, then legacy UE and new UE can use different cyclic shifts. Now if both comb offset hopping and CS hopping are supported, then it is difficulty to co-schedule legacy UE and new UE in one OFDM symbol, so we suggest to down select one from CS hopping and comb hopping. In addition, as mentioned by QC the comb hopping for position SRS is not for randomization. So it can not be reason to support comb offset hopping |
| OPPO | Not support. Comb offset hopping and cyclic shift hopping are different ways to achieve the same functionality (SRS collision and interference randomization). We need to further study whether they are beneficial or not as proposed by vivo, or whether to down select one with better interference randomization as suggested by ZTE. When one scheme is supported, the gain of another scheme would be discounted, and maybe no gain at all. It is not an easy issue of 1+1=2. |
| Xiaomi | We are open to study A2.1 with high priority. For Option 2, we prefer the comb offset is determined pseudo-randomly as a function of time or network/cell ID. |
| Lenovo | We are fine with updated Proposal 2.1. For FFS part, we think candidate values of comb offset for hopping also can be further discussed. |
| LGE | As commented by vivo, it seems further study is needed to improve our understanding on the schemes. |
| Ericsson | We are open to further study comb offset hopping with high priority. However, we agree with, e.g., ZTE and OPPO that it is too early to support both cyclic-shift hopping and comb-offset hopping. Indeed, both these schemes achieves a similar functionality (collision mitigation and interference randomization). Perhaps an acceptable and reasonable compromise, at this stage, is to support, at least, one of comb-offset hopping and cyclic-shift hopping. In other words, merging FL proposal 2.1 and 2.2. Finally, we agree with QC that listing Option 1 and 2 is not helpful at this stage. The exact hopping pattern needs careful consideration. |

### Round 2

Tuesday GTW agreement:

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Thank you all for the discussions. We will continue to discuss comb offset hopping for SRS based on the above agreement. The plan is to identify the next level of details. Enhancement A1 can also be discussed, and proponents can feel free to suggest potential proposals.

Potential Proposal: For comb offset hopping for SRS, the next decision points at least include:

- The hopping pattern (e.g., the pseudo-random sequence)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

| Company | View |
| --- | --- |
| Apple | We are fine with those study areas. |
| Samsung | As some companies mentioned in the last GTW, there may be an issue multiplexing with legacy UEs when CS hopping and comb offset hopping are considered together. Hence, we prefer to down-select one of two schemes first. |
| ZTE | Same view as Samsung. |
| QC | Support to further study these aspects. Please also see our comment for the next proposal as the study points between these proposals should be aligned. |
| LGE | Same view as Samsung. |
| Sharp | Same view as Samsung. |
| vivo | Prefer to further compare the performance of comb offset hopping with other potential solutions, at least with CS hopping. Then we can decide whether to support it. Proposal: To evaluation the performance of different potential enhancements, periodic SRS with longer period can be as the baseline. |
| Lenovo | Support to further study these aspects. For Comb hopping pattern, the time granularity for hopping can be further discussed, which may be hopping per SRS symbol or per SRS resource since we may consider multiple symbol SRS transmission with repetition, SRS for antenna switching with different TxRx number, etc. So we suggest the following updating: - The hopping pattern (e.g., the pseudo-random sequence, time granularity for hopping) For “UE-specific initialization” in the third bullet, we think it may be a general concept. More detail schemes can be further discussed. Also, we want to clarify whether 8 port SRS is in the scope for the discussion. |
| Intel | One question for clarification. Is this scheme only for periodic/semi-persistent SRS, correct? It should be clarified. |
| Xiaomi | We are fine with studying these aspects. |
| OPPO | Same view as Samsung. |
| Nokia/NSB | We are open to study. but, share view with Samsung and other companies. |

### FL update 2

Thank you all for the constructive inputs.

@Samsung @ZTE @LGE @Sharp @OPPO @Nokia/NSB: My suggestion is that we add have a study aspect on the multiplexing with legacy UEs when both CS hopping and comb offset hopping are enabled. If there is indeed an issue, we can down select or maybe leave this for gNB implementation (i.e., not to enable both hopping when multiplexing with legacy UEs) or forbid both are enabled on the same time/frequency resources in standards. In addition, these two hopping schemes are the most supported schemes and the other schemes have less support, so we could wait a little longer to see if there is a need to down select one from the two most supported schemes based on the decisions on the other schemes.

@vivo: We can add more EVM assumptions if needed. Could you please clarify “SRS with longer period”?

@Lenovo @Intel: Those are good points/question and now included in the updated proposal. I believe the enhancement should be applicable to 8 Tx SRS, but companies can provide their views.

Companies are encouraged to provide evaluation results, to compare with baseline without comb offset hopping or with other enhancements.

Potential Proposal: For comb offset hopping for SRS, the next discussion points / decision points at least include:

- The hopping pattern (e.g., the pseudo-random sequence, time granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if both CS hopping and comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

| Company | View |
| --- | --- |
| Samsung | We still think that down-selecting is the first step since the potential proposal above seems the second level details. Is the intention of separate proposal for comb-offset hopping and cyclic shift hopping that companies analyze/discuss about detailed levels of each scheme first, and down-select as a next step? |
| Huawei, HiSilicon | Considering these two hopping schemes are the most supported schemes, support to list the further discussion points / decision points. |
| DOCOMO | Besides the issue of multiplexing, we would like to see the reason why “support both (comb hopping and cyclic shift hopping” needs to be considered here. If the benefit of each option is similar, and if no benefit by supporting both is observed, we think down-selection should be considered as early as possible so that we can spend more time for other critical issues. In short, we share Samsung’s point. |
| Ericsson | We are supportive of studying these aspects. However, since the discussion/decision point are more or less common for both cyclic-shift and comb-offset hopping, the potential proposals could be merged. |
| vivo | Sorry for the typo, “SRS with longer periodicity” means periodic/semi-persistent SRS transmitted with longer periodicity in time-domain. For example, assume there is no enough symbols to map two different SRS resources (e.g., with periodicity as 10ms) in TDM manner, then they collide. Based on this pre-requisite, CS hopping/comb offset hopping is proposed to randomize the across-SRS interference. However, another simple way to void the interference is directly to configure SRS resources with longer periodicity (e.g., 20ms), then different SRS resources can be configured on different symbols without across-SRS interference. Therefore, to compare performance of different potential schemes and do down-selection, the performance of periodic SRS with longer periodicity could be considered as a baseline. Besides, we share the same view with Samsung and DOCOMO, prefer to do down-selection first. |
| Xiaomi | We are fine to study these aspects. As commented in last round or online, one of comb hopping and CS hopping is sufficient. |
| Nokia/NSB | We are fine to study, however, we prefer not to support multiple schemes for a single functionality. |

### FL update 3

Thank you for the comments. Most of the inputs are about how to handle comb offset hopping and CS hopping. We can merge the two proposals into one, perform the study / analysis / evaluation, and then we can come back to decide if we want to support only one or both. Such a decision may be made in the next meeting, but companies can feel free to suggest. Since companies mentioned “study these aspects”, the proposal is also updated to “study”.

@vivo: We can discuss additional EVM in the next round.

Any technical discussion for any of the schemes in this category can still be provided in the next round after the Friday GTW.

Proposal 2.1-1: For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following:

- The hopping pattern (e.g., the pseudo-random sequence, time granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if both CS hopping and comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

| Company | View |
| --- | --- |

## Randomized code-domain resource mapping for SRS transmission

At least the following potential enhancements have been discussed:

B1: Cyclic shift hopping / randomization (e.g., CMCC, Ericsson, Futurewei, Huawei, HiSilicon, Intel, Lenovo, Nokia, Nokia Shanghai Bell, Samsung, Sharp, Spreadtrum, InterDigital, NTT DOCOMO; 14 proponents)

Pros: Interference randomization; two sources show performance benefit

Cons: potential multiplexing issue with legacy UE; restriction on scheduling flexibility

B2: Sequence hopping / randomization (e.g., Qualcomm, Xiaomi, ZTE, InterDigital, NTT DOCOMO;  5 proponents)

Pros: Interference randomization

B3: Per-hop sequence from a long SRS sequence (e.g., Huawei, HiSilicon, Futurewei; 3 proponents)

Pros: SRS multiplexing; one source shows performance benefit

Cons: PAPR increased

B4: FD OCC (e.g., Nokia, Nokia Shanghai Bell; 2 proponents)

These enhancements may be related to those in Sec. 2.11.

Based on the large number of supports, the FL suggests to focus on further discussion on cyclic shift hopping / randomization and provide views on the next study point / decision point. In addition, Q1~Q4, e.g., clarification/pros/cons, can be discussed for all above enhancements, especially for the newly proposed enhancement B4.

| Company | View |
| --- | --- |
| QC | Support B2. Regarding B1, it seems that at least a good part of gain can be achieved by B2 (to decrease the probability of collision), and B2 is much simpler (just combining two existing hopping schemes). We do not support B3 (increases PAPR and requires joint processing) or B4 (motivation is not clear given that we have cyclic shift in freq domain already; also, instead of introducing new schemes, we should really down-select at this point). |
| InterDigital | Support B1 and B2. |
| Google | Open to B1 |
| OPPO | We have concern on B1, similar to the concern on comb offset hopping. |
| Huawei, HiSilicon | Support B1 and B3. For B1, the benefit over B2 has been shown in [3]. The hopping granularity can be further studied. In terms of the Cons, “potential multiplexing issue with legacy UE” and “restriction on scheduling flexibility” are essentially the same issue, which is the SRS multiplexing issue, and can be simply addressed by FDM (one of the most common methods to deal with multiplexing issue used since Rel-15) or other potential methods. For B3, the performance taking the transmission power fallback caused by PAPR increase into consideration has been shown in [3] and obvious benefit can be obtained. |
| MediaTek | We are open to B1 and B2 |
| Xiaomi | Open to B2 due to its simplicity. |
| ZTE | Support B2. |
| Spreadtrum | Support B1 |
| Lenovo | Support B1 |
| CMCC | Open to B1. |
| Sharp | Support B1 |
| Samsung | Support B1. |
| Intel | Fine with B1. |
| Nokia/NSB | Support B1 and B4. For B4, FD-OCC can improve the capacity. |
| FL | There are 13 proponents for B1 and a few companies are also open to it. Thus, the FL suggests the following potential proposal: Potential Proposal: Introduce cyclic shift hopping / randomization to SRS resource. FFS: cyclic shift allocation pattern(s) and parameter(s) used to determine the cyclic shift for a SRS port in a SRS transmission occasion. |
| Vivo | Ok to study cyclic shift hopping / randomization with high priority. However, we believe whether/how to support it in Rel-18 need more discussion an evaluation, since there are other potential solutions, such as comb offset hopping. We prefer to modify the proposal as follows. Potential Proposal: Study cyclic shift hopping / randomization to SRS resource with high priority. Considering cyclic shift allocation pattern(s) and parameter(s) used to determine the cyclic shift for a SRS port in a SRS transmission occasion. |
| QC | If we are going ahead with B1, we should also include B2 as an additional but simpler enhancement. This is because probability of SRS sequence collision can be reduced directly with B2 by just combining two existing hopping’s. For code domain randomization enhancement, we would like to at least allow the simpler solution which can be enabled with smaller spec impact / implementation efforts. |
| Ericsson | We are open to continued/prioritized study of B1 as we believe this has the potential to reduce interference due to delay differences in mTRP scenarios. We agree with Vivo that it is important to compare B1 with other interference-randomization solutions (e.g., A2.1) before deciding on support in NR Rel-18. |

### FL update

@vivo: Thank you for the suggestion. Since there is a large number of proponents, I think we can see if “Support cyclic shift hopping” is agreeable.

@QC: Thank you for the suggestion. I now suggest a broader main bullet below so that we can also discuss B2~B4 if there is enough interest.

Proposal 2.2: Support randomized code-domain resource mapping for SRS transmission.

Introduce at least cyclic shift hopping / randomization to SRS resource

FFS Cyclic shift allocation pattern(s) and parameter(s) used to determine the cyclic shift for each SRS port of a SRS resource in a SRS transmission occasion.

FFS other schemes.

| Company | View |
| --- | --- |
| QC | We still have a concern with this proposal. If anything, it makes sense to first focus on schemes with smaller spec impact that can already achieve a good portion of the benefit. Then, we suggest to remove both bullets, and further discuss and evaluate B1~B4. |
| ZTE | If both CS hopping and comb offset hopping are supported, then legacy UE and new UE can not be scheduled on same OFDM symbol. So we suggest to down select one from comb hopping and CS hopping. |
| Sharp | Support |
| OPPO | Comb offset hopping and cyclic shift hopping are different ways to achieve the same functionality (SRS collision and interference randomization). We need to further study whether they are beneficial or not as proposed by vivo, or whether to down select one with better interference randomization as suggested by ZTE. When one scheme is supported, the gain of another scheme would be discounted, and maybe no gain at all. It is not an easy issue of 1+1=2. |
| Xiaomi | We have similar view with vivo and Ericsson. Since there are many other candidate schemes, it is too early to determine to support randomized code-domain resource mapping at this stage. |
| LGE | Similar view as Xiaomi. |
| Ericsson | We are open to further study cyclic shift hopping with high priority. However, we agree with, e.g., ZTE, OPPO, Xiaomi and LGE that it is too early to support both cyclic-shift hopping and comb-offset hopping. Indeed, both these schemes achieves a similar functionality (collision mitigation and interference randomization). Perhaps an acceptable and reasonable compromise, at this stage, is to support, at least, one of comb-offset hopping and cyclic-shift hopping. In other words, merging FL proposal 2.1 and 2.2. |

### Round 2

Tuesday GTW agreement:

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Thank you all for the discussions. We will continue to discuss randomized code-domain resource mapping for SRS based on the above agreement. The plan is to identify the next level of details. Enhancement B2~B4 can also be discussed, and proponents can feel free to suggest potential proposals.

Potential Proposal: For randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, the next decision points at least include:

- The hopping / randomization pattern (e.g., the pseudo-random sequence)

- The time-domain parameter and/or behaviour (e.g., slot index, symbol index, re-initialization behaviour)

- Network-configured ID for UE-specific initialization

- How the cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

| Company | View |
| --- | --- |
| Apple | We are fine with those study areas. However, we prefer to downselect one from cyclic shift and comb offset hopping |
| Samsung | As some companies mentioned in the last GTW, there may be an issue multiplexing with legacy UEs when CS hopping and comb offset hopping are considered together. Hence, we prefer to down-select one of two schemes first. |
| ZTE | Same view as Samsung |
| QC | It would be good to align the text with the proposal on comb hopping. For example, here it says “The hopping / randomization pattern” but for comb hopping it says “The hopping pattern”. |
| LGE | Same view as Samsung. |
| Sharp | Same view as Samsung. |
| Vivo | Prefer to further compare the performance of CS hopping with other potential solutions, at least with comb offset hopping. Then we can decide whether to support it. Proposal: To evaluation the performance of different potential enhancements, periodic SRS with longer period can be as the baseline. |
| Lenovo | We agree with QC that it is better to align the proposal with comb hopping. Similar comments as those in section 2.1. |
| Intel | Same question as in Section 2.1. It should be clarified whether this scheme is only for periodic/semi-persistent SRS. |
| Xiaomi | For cyclic shift and comb hopping, we think one of them is sufficient. Considering its simplicity, comb hopping is preferred. |
| OPPO | Same view as Samsung. |
| Nokia/NSB | We are fine to study. Also, share view with Samsung and other companies. |

### FL update 2

Thank you all for the constructive inputs. Please see Sec. 2.1 for some of the replies and below updated proposal based on the inputs.

Potential Proposal: For randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, the next discussion points / decision points at least include:

- The hopping / randomization pattern (e.g., the pseudo-random sequence, time granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if both CS hopping and comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

| Company | View |
| --- | --- |
| Samsung | We still think that down-selecting is the first step since the potential proposal above seems the second level details. Is the intention of separate proposal for comb-offset hopping and cyclic shift hopping that companies analyze/discuss about detailed levels of each scheme first, and down-select as a next step? |
| Huawei, HiSilicon | Considering these two hopping schemes are the most supported schemes, support to list the further discussion points / decision points. Prefer cyclic shift hopping. Regarding the performance, significant performance gain of CS hopping has been observed in [3] especially under inter-cell interference randomization scenario, which is one of the most common CJT scenarios considering the scarcity of SRS resource and the feasibility as well as complexity of joint resource allocation. Regarding the complexity, considering that CS hopping doesn’t require collecting channel estimation result on different subcarriers and the CSs can simply be added and removed in the way already supported by the current UE/gNB, we think CS hopping has similar or even lower complexity compared with comb offset hopping. Regarding the next level of details, besides the aspects mentioned in the potential proposal, code domain hopping granularity should also be investigated. Regarding the coexistence of CS hopping and Comb offset hopping, we agree with FL that this can be left for gNB implementation or guaranteed by Spec. We can list the further discussion points / decision points first and potential (if really necessary) down-selection can be decided in the next stage. |
| DOCOMO | Besides the issue of multiplexing, we would like to see the reason why “support both (comb hopping and cyclic shift hopping” needs to be considered here. If the benefit of each option is similar, and if no benefit by supporting both is observed, we think down-selection should be considered as early as possible so that we can spend more time for other critical issues. In short, we share Samsung’s point. |
| Ericsson | We are supportive of studying these aspects. However, since the discussion/decision point are more or less common for both cyclic-shift and comb-offset hopping, the potential proposals could be merged. |
| vivo | Share the same view with Samsung and DOCOMO, prefer to do down-selection first. |
| Xiaomi | Similar view with other companies, the two proposal on comb hopping and CS hopping should be down selected. |
| Nokia/NSB | We are fine to study. |

### FL update 3

The potential proposal is merged into Proposal 2.1-1 in Sec. 2.1. Any technical discussion for any of the schemes in this category can still be provided in the next round after the Friday GTW.

## Randomized transmission of SRS

C: Randomized transmission of SRS, e.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS (e.g., vivo, Qualcomm, Futurewei, NTT DOCOMO; 4 proponents)

Pros: Interference randomization; two sources show performance benefit

These enhancements may be related to those in Sec. 2.11.

Q1~Q4, e.g., clarification/pros/cons, can be discussed for the above enhancement.

| Company | View |
| --- | --- |
| QC | Support. At least 2 companies provided simulations result showing the gains. Also, we clarified some of the misunderstanding (based on the discussions in the previous meeting) in our contribution as follows: - Misunderstanding 1: “Periodic SRS with larger periodicity can achieve the same goal”. Obviously, periodic SRS cannot achieve interference randomization if in every period the same set of UEs create interference. In the simulation results above, baseline (“static allocation”) is exactly the periodic SRS with larger periodicity, and the benefit of the enhancement can be seen from the Figures above. The whole point of this scheme is the fact that SRS transmission is randomized in time domain, and for fair comparison, the baseline should also have the same average number of transmission occasions. - Misunderstanding 2: “This scheme decreases the SRS channel estimation accuracy and is unpredictable”. First, pseudo-random muting avoids persistent interference, and hence, it improves the tail UE performance as shown above. Second, the degree of predictability can be controlled by the network as discussed in detail in Approach 2 above. Third, any interference randomization scheme aims to create difference interference patterns, and we do not understand why randomized transmission in time domain in particular is subject to channel estimation inaccuracy compared to frequency-domain or code-domain randomizations. - Also, it was mentioned that similar functionality can be achieved by AP-SRS. However, AP-SRS comes with the cost of DCI overhead as one DCI is required for each transmission occasion. We do not think it is reasonable to use so many DCIs just for the purpose of interference randomization when alternative solutions exist that do not require any DCI overhead. |
| Google | Do not support. Benefit is minor with regard to aperiodic SRS. |
| OPPO | If the total number of SRS transmission is the same for periodic SRS with longer period and random muting with shorter period, where does the gain come from? If the gain comes from persistent interference, gNB should avoid this type of persistent collision considering CJT is based on ideal backhaul. |
| Huawei, HiSilicon | Not support. The basic principle of randomization in any domain should be balancing the performance and avoiding the “worst” case. However, although time-domain randomization can avoid the “worst” interference case, it will simultaneously enlarge the SRS delay (the gap between SRS transmission and data) range experienced by SRS transmissions and consequently deteriorate the “worst” delay case, which to some degree violates the basic principle of randomization and should be carefully treated and is different with frequency-domain or code-domain randomizations. |
| MediaTek | Not Support |
| Xiaomi | Not support. Uplink channel can be jointly estimated in TD even though SRS are transmitted in FD and CD with randomization. However, Channel estimation cannot jointly implemented if pseudo-random muting of SRS transmission are operated in TD, which may have impact on channel estimation accuracy. |
| ZTE | Do not support because the gNB can not determine preference PRB when frequency hopping is enable and the gNB can not get timely CSI, then the UE have to transmit SRS with short periodicity. |
| Spreadtrum | Not support, We have concern on the performance. Since muting scheme can instead reduce SRS capacity, it is not fair to only see the benefit on interference randomization. |
| Lenovo | Not support. There may be channel estimation performance loss for the burst transmission. Also, this function can be carried by aperiodic/semi-persistent SRS to some extent. |
| Sharp | Not support |
| Samsung | Not support. As many companies mentioned, the corresponding performance loss rather than the merit of scheme would be large, and this can be achieved by SRS with other time domain behaviors. |
| Intel | Not support. Similar view as other companies. |
| LGE | Not support. |
| Nokia/NSB | Do not support. This may increase the overhead in time. Without increase of time instance, the randomization gain cannot be obtained. |
| Vivo | The gain of random muting is based on randomizing the persistent collision. We can regard as random muting as an automatic interference mechanism. Compared with frequent RRC reconfiguration for SRS, it doesn’t need additional signaling overhead and can save the effort of the network to pre-judge the level of varying interferences from different UEs. Regarding comparing with periodic SRS with longer period, we believe that it should be the baseline for all of 11 categories of potential SRS enhancements. (Suggest this as an agreement or conclusion when comparing the performance of different potential enhancements) As shown in our simulation results, it can be observed that random muting can achieve a statistical benefit based on randomizing the interference compared with periodic SRS with longer period. Regarding CSI delay and channel estimation accuracy, if the SRS occasion is the same as periodic SRS with longer period, to be honest, CSI would be measured more timely in some adjacent SRS occasions, while CSI would be measured with more latency in non-adjacent some SRS occasions. However, in general, it would not affect too much. Proposal: To evaluation the performance of different potential enhancements, periodic SRS with longer period should be as the baseline. |
| QC | From evaluations, we see 2 companies have already shown the benefit. For some other schemes such as cyclic shift hopping, we only observed one company evaluating the scheme (please correct us if we missed some evaluations). The decision should be based on evaluation results, and not just “support / not support”. |
| Ericsson | In our view, since there are too many candidate schemes for SRS interference randomization, we prefer to prioritize schemes that are applicable to all time-domain types of SRS (i.e., periodic, semi-persistent, and aperiodic). Note that aperiodic SRS is important as it can minimize uplink overhead by triggering SRS only when it is needed. Since Enhancement C is not applicable to aperiodic SRS, we prefer to down-prioritize further study of this scheme. |

### Round 2

Thank you all for the discussions. I personally think some of issues pointed out by opponents can already be addressed by QC/vivo’s replies, especially QC’s clarifications on the ‘misunderstanding’. There is also performance benefits shown by companies. The proposed muting pattern is not based on a pseudo-random sequence such as c(i) but a different design to ensure both (pseudo) randomness and some desirable minimum gap / average properties.  Please further discuss the scheme, and if this scheme may be included/ combined with any other schemes, it can also be considered.

Potential Proposal: Study randomized transmission of SRS, e.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS.

| Company | View |
| --- | --- |
| Apple | We are fine to study |
| Samsung | Not support as mentioned in previous round. |
| ZTE | Not support |
| QC | As FL mentioned, most of the concerns are already answered. We are open to address other specific concerns if companies can elaborate the details. |
| LGE | Low priority. |
| Sharp | Not support. We think other schemes has higher performance in terms of channel estimation than pseudo-random muting of SRS transmission. |
| Vivo | Support the proposal. If companies have concerns on the performance of pseudo-random muting, appreciate to provide the simulation results. At least, in QC and vivo’s tdoc, we have shown the performance gain, compared with periodic SRS with longer period. Proposal: To evaluation the performance of different potential enhancements, periodic SRS with longer period can be as the baseline. |
| Lenovo | Not support as mentioned in previous round. |
| Intel | Low priority. |
| Xiaomi | Not supported. As pointed by Ericsson in the last round, this scheme is not applicable to AP SRS. In addition, if higher-layer parameter FreqScalingFactor and higher-layer parameter StartRBIndex is configured, the whole band cannot be sounding for randomly muting SRS transmission. Hence, this solution is not suitable for partial frequency sounding. We can see that there are too many restriction conditions of application for this solution. |
| OPPO | Not support. |
| Nokia/NSB | Now support, we have already provided concern and we don’t think this issue has been correctly answered. More time opportunity shall spend more UL symbols to be reserved for SRS transmission. So, we don’t agree with any time-domain schemes. We prefer to focus the already agreed schemes. |
| FL update 2 | @QC @vivo: Can you please try to address Sharp/Xiaomi/Nokia/NSB’s concerns? Companies are encouraged to elaborate their concerns. |
| Ericsson | Do not support since this scheme not applicable to aperiodic SRS. We prefer to prioritize schemes that are applicable to all time-domain types of SRS (i.e., periodic, semi-persistent, and aperiodic). |
| vivo | We try to address the concerns from companies. To sharp: we need further evaluation to compare the performance of different enhancements, we don’t see any simulation result to show that they have better performance gain than random muting, appreciate companies to provide further simulation results. To Xiaomi/E///: AP SRS would not cause persistent interference, however random muting also can be used on AP SRS if needed, thought we think it is unnecessary. Regarding your concern on partial sounding, we can just mute some of FHs, anyway it depends on the muting granularity. To Nokia/NSB: If I’m not mistaken, you mean the gain of random muting is from more transmission occasion for SRS or shorter periodicity, right? If yes, as vivo and QC mentioned, random muting is to improve the performance of tail UE (i.e., suffering persistent large across-SRS interference). In other words, the unbalance of UE performance can be handled. However, this gain doesn’t need more transmission occasion for SRS. Based on the same number of transmission occasions, we observe the gain in our simulation results showing that random muting can improve the performance of UE suffering persistent interference. |
| QC | @FL: As you may have noticed, it is not easy to address some of the comments such as “We think other schemes has higher performance in terms of channel estimation than pseudo-random muting of SRS transmission” w/o explaining the reason for such conclusions even though 2 companies provided evaluation results for this scheme. Anyway, we can at least try to address some of the concerns that are clearer / elaborated more: - Regarding applicability to AP-SRS, we agree with vivo. The situation is the same also for cyclic shift hopping or comb offset hopping. It is about the unit of hopping. If it is per SRS resource, none of the schemes are applicable to AP-SRS (since network can trigger the SRS resource with the intended comb offset / CS). If the hopping unit is per symbol, it is applicable to AP-SRS for all schemes, even though randomization only within a few OFDM symbols may not be the most useful case here. - Regarding “more time opportunity shall spend more UL symbols to be reserved for SRS transmission”: The overall reserved symbols across UEs should be the same for fair comparison against legacy, which is the case in our evaluation. In legacy with longer periodicity, the SRS symbols are spread across two groups of UEs (static orthogonalization), which results in the same SRS symbols in the system. |
| FL update 3 | Thank you for the good technical discussions. Opponents please let the group know if any of the concerns have been addressed. Regardless of whether this scheme will be supported or not, we may try to at least reach a technical conclusion on it. |

## Per-TRP power control and/or power control of one SRS towards to multiple TRPs

At least the following potential enhancements have been discussed:

D: Per-TRP power control and/or power control of one SRS towards to multiple TRPs (e.g., ZTE, Google, Intel, KDDI, Samsung, Apple, Ericsson, LG, Nokia, Nokia Shanghai Bell, Sony; 11 proponents)

D1: Per-TRP power control (Ericsson, KDDI, Intel)

Pros: No gNB receive power imbalance issue due to near-far problem

Cons: Higher SRS overhead

D2: Power control based on more than 1 DL pathloss RS and/or more than 1 UL power control parameter set (e.g., ZTE, Nokia, Nokia Shanghai Bell, Apple, Google)

Pros: Low SRS overhead; one source shows performance benefit

Cons: gNB receive power imbalance issue due to near-far problem

Some of these enhancements may be related to those in Sec. 2.8.

Based on the large number of supports, the FL suggests to focus on further discussion on SRS power control enhancement and provide views on the next study point / decision point. However, the support for either D1 or D2 is small. Proponents should further clarify their positions. Proponents for D1 may also clarify whether additional standard support is needed. The next study point / decision point can also be discussed.

| Company | View |
| --- | --- |
| QC | Regarding D1, it would be good to list the spec impact / exact scheme. Regarding D2, we would like to check if any company evaluated the benefit comparing to existing OLPC and CLPC mechanisms. We are open to this, if the benefit is shown. |
| InterDigital | We support further studying D1 and D2. |
| Google | Support, for D2, we think it can be revised as “power control based on more than 1 uplink power control parameters sets” |
| OPPO | We are fine to further study D2. For D1, we don’t think it is necessary. |
| Huawei, HiSilicon | Regarding D1, the spec impact is unclear. Regarding D2, the benefit needs to be justified. |
| Xiaomi | For D1, we do not support TRP-specific power control. From our understanding, the channel is estimated based on one SRS resource, then how could it possible to apply different transmission power to one SRS resource. About D2, we cannot see that power control is able to randomize SRS interference and increase SRS capacity. The motivation, from our understanding, is to ensure that each CJT TRP can accurately receive the SRS, which can be discussed latter. |
| ZTE | Support D2. TRP common SRS is better than TRP specific SRS with respective to UE power saving and inter-cell reduction. In addition, if TRP specific SRS is used, the gNB can not determine accurate downlink CJT precoding because the gNB does not know the actually transmitting power ratio of the multiple TRP specific SRS resources. Current OLPC and CLPC mechanisms is for single TRP. Power enhancement for SRS toward multiple TRPs can not be supported by current OLPC and CLPC mechanisms. @xiaomi, if the TRP common SRS can not be accurately received by each CJT TRP, then the UE has to transmit TRP specific SRS which leads unnecessary more UE power consumption and more SRS resources. In a word, if TRP common SRS is well supported, each UE transmits less SRS resources.The needed SRS capacity is reduced. So it is a way for SRS capacity enhancement. |
| Spreadtrum | Need further study on D1 and D2. |
| Lenovo | We are fine to have more study for D2 on account of less SRS overhead. |
| CMCC | For D1, the spec impact is not clear for us. Does that mean multiple SRS resources for multiple TRPs with TRP specific power control? Or just one SRS resource with multiple transmission occasions for multiple TRPs? For D2, we are OK to further study. |
| Samsung | Support further study on D1 and D2. |
| Intel | Support D1. As discussed in previous meeting, there is near-far issue if the same SRS is received by multiple TRPs, which impacts the channel measurement quality. There could be different options for D1. For example, SRS could be configured for each TRP with corresponding power control setting. In another example, the same SRS could be configured, but the TRP specific power control setting could be applied for different SRS transmission. |
| LGE | We propose to consider TRP switching across different SRS transmission occasion for periodic/semi-persistent SRS or for multi-symbol SRS. We are not sure whether this method belongs to D1 or D2. |
| KDDI | We support D1. Considering the near-far problem, it is necessary to control the transmit power appropriate for each TRP. However, methods to reduce SRS overhead need to be considered. |
| Nokia/NSB | Support D2, but we are open to D1 as well. This is at least required for mTRP operation not only limited to antenna switching but also for codebook/non-codebook. |
| FL | Companies have clarified their positions. The FL took Google’s suggestion and added to D2. Now D2 seems to also cover Intel’s suggestion on “TRP specific power control setting could be applied for different SRS transmission”, LGE’s suggestion on “TRP switching”, KDDI’s suggestion on “control the transmit power appropriate for each TRP” if I understand correctly; @Intel @LGE Please check. The FL suggests to focus on the updated description of D2. |
| Ericsson | We prefer to continue study of D1 and D2 to keep both TRP-specific and TRP-common SRS for CJT on the table but are, for the sake of progress , fine with focusing on D2 for solving the near/far issue for SRS for CJT. |
| Xiaomi | @ZTE, Thanks for your explanation. We got your intention and we are OK to discuss it to ensure that SRS can be accurately received. While, about D2, what we means is that the total number of SRS resources is still the same, does not increase. D2 is a way to make sure one SRS resource is enough for UE to estimate the joint channel. Actually, we do believe that D2 is needed, just we do not think that it is a method to increase the SRS capacity. To make progress, if most the companies are fine with this, then we can live with D2. |
| Intel2 | Thanks FL for the explanation. Although D1 is our preference, we could be fine with the updated D2. |
| LGE | @FL, we also think multiple power control parameter set for CJT TRPs can be configured for a SRS resource set. In that perspective, D2 covers our view. However, we think that the power control parameters can be switched across different SRS transmission occasion. This can achieve SRS resource overhead reduction and resolve near-far problem, too. |

### Round 2

Thank you all for the discussions and being flexible. We can focus on the updated D2 for now. D2 still have two different designs and we can discuss the next level of details.

Potential Proposal: For per-TRP power control and/or power control of one SRS towards to multiple TRPs, select at least one of the options for a SRS resource set:

Option 1: Power control of the SRS resource set is based on one UL power control parameter set and more than one DL pathloss RS.

Option 2: Power control of the SRS resource set is based on more than one UL power control parameter set each associated with a DL pathloss RS.

| Company | View |
| --- | --- |
| Samsung | Fine with down-select. |
| ZTE | Support. I want to confirm that the transmission power of different resources in the SRS resource set is same as legacy , right? So can we add a note that the transmission power of different resources in the SRS resource set is same as legacy? So we support following updated proposal: Updated Proposal: For per-TRP power control and/or power control of one SRS towards to multiple TRPs, select at least one of the options for one SRS resource set: - Option 1: Power control of the SRS resource set is based on one UL power control parameter set and more than one DL pathloss RS. - Option 2: Power control of the SRS resource set is based on more than one UL power control parameter set each associated with a DL pathloss RS. Note: the transmission power of different SRS resources in the SRS set are same. |
| QC | We are okay to further study these, but the benefit needs to be shown (compared to legacy power control mechanisms) before agreeing to support. Also, for Option 1, configuring one alpha and two PL-RS may be limiting at this stage. Hence, suggest: Option 1: Power control of the SRS resource set is based on one UL power control parameter set and more than one DL pathloss RS and/or more than one alpha. |
| LGE | In our view, it is better to support per-TRP SRS transmission in different time instances, e.g. in different period for P/SP SRS, rather than single SRS toward multiple TRPs which could degrade SRS reception quality of both TRPs. Thus, we propose the following revision: Potential Proposal: For per-TRP power control and/or power control of one or multiple SRS transmission instance(s) towards to multiple TRPs, select at least one of the options for a SRS resource set: Option 1: Power control of the SRS resource set is based on one UL power control parameter set and more than one DL pathloss RS. Option 2: Power control of the SRS resource set is based on more than one UL power control parameter set each associated with a DL pathloss RS. FFS: whether one SRS transmission instance is for one TRP or for multiple TRPs |
| Sharp | We are OK with the proposal. We prefer Option 1 because we think it is enough that open loop parameter is considered to mitigate interference power for each TRP. |
| KDDI | We think that it is better to support per-TRP SRS transmission in different transmission power to handle near-far problem. So, the revision of LGE seems good. |
| Vivo | Low priority. |
| Lenovo | We are fine with further study on these two alternatives. |
| Intel | Generally fine with the proposal. Option 2 is preferred. |
| OPPO | We are fine to further study the two alternatives, if the proposal doesn’t imply that per TRP power control is supported. For the note added by ZTE, we don’t think it is needed since it can only be applied for SRS for BM/AS. For other usages, different TPC commands can be applied to different SRS resources in one set, which will lead to different power for different resources. |
| Nokia/NSB | Support the proposal. Fine with the study. |

### FL update 2

Thank you all for the constructive inputs.

@ZTE: The transmission power of each SRS resource in the resource set follows the power control formula / TPC command and may or may not be the same. Please further clarify.

An updated proposal is provided based on the inputs.

Potential Proposal: For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, select at least one of the options for a SRS resource set:

Option 1: Power control of the SRS resource set is based on one UL power control parameter set and more than one DL pathloss RS and/or more than one alpha.

Option 2: Power control of the SRS resource set is based on more than one UL power control parameter set each associated with a DL pathloss RS.

| Company | View |
| --- | --- |
| Samsung | For the updated potential proposal, we are confused the difference between option 1 and option 2. Is the main difference whether one or multiple number of power control parameter sets, although both options can have multiple PLRS or alpha? |
| DOCOMO | A clarification question: Would the proposal mean at least one of the options will be specified? |
| ZTE | Support |
| LGE | Support. |
| Ericsson | Support. |
| vivo | I’m a bit confused about this proposal. Does it mean just to further study or at least to support one of two options in Rel-18. We can accept to further study it, since the benefit of power control enhancements needs to be further evaluated at this stage. |
| QC | Do not support to “select at least one of …”. If the intention is further study, it is ok. Also, the description of Options are not fully clear. We understand option 1 as one Tx power is calculated based on f(α1∙PL1, α2∙PL2), where the function f(.) returns one value (can be min/max/average/etc.). Is this correct understanding? If so, how option 2 is different? Is Option 2 for switching between different sets of parameters at different occasions? If so, this needs to be clarified in the description. |
| Nokia/NSB | Support the proposal. |

### FL update 3

Thank you all for the constructive inputs.

I believe QC has the correct understanding of the options:

Option 1: 1 PC parameter set, 1 PC formula based on multiple alphas and PLs, and each SRS is intended for multiple TRPs to receive. For example, fα1∙PL1, α2∙PL2=(α1∙PL1+ α2∙PL2)/2, i.e., the SRS is targeting both TRPs but the power is set as the mean of respective TRP-specific SRS transmission powers.

Option 2: 2 PC parameter sets, 2 PC formulas each based on its own alpha and PL, PC set 1 is for TRP 1 and PC set 2 is for TRP 2. Switching among the TRPs are possible. This is like 2 different SRS resource sets but the only difference is in the PC, and hence they may be combined into 1 SRS resource set with 2 PC parameter sets.

An updated proposal is provided based on the inputs.

Proposal 2.4: For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for a SRS resource set:

Option 1: Power control of the SRS resource set is based on one UL power control parameter set and more than one DL pathloss RS and/or more than one alpha. Each transmission occasion of the SRS resource is towards multiple TRPs.

Option 2: Power control of the SRS resource set is based on more than one UL power control parameter set each associated with a DL pathloss RS. Different transmission occasions of the SRS resource can be towards different TRPs.

| Company | View |
| --- | --- |

## SRS TD OCC

At least the following potential enhancements have been discussed:

E: TD OCC (e.g., ZTE, Spreadtrum, Lenovo, NEC, Intel, CMCC, Ericsson, OPPO; 8 proponents)

Pros: Capacity enhancement (for SRS with repetition); two sources show performance benefit

Cons: High overhead (compared to SRS without repetition); may not be highly relevant to TDD CJT; FFS when SRS is dropped on one of the OFDM symbols

Since TD OCC is also proposed in 8 Tx SRS enhancements, to avoid duplicated discussions, the FL suggests to focus on the SRS TD OCC discussion in 8 Tx SRS part.

Companies’ views on the above are collected as follows.

| Company | View |
| --- | --- |
| QC | As discussed before, the use case is limited because multiple symbols are required for TD-OCC to make sense. For SRS for CJT, given that the purpose is DL CSI acquisition (antenna switching), for a more practical antenna assumption such as 1T4R, each of the four SRS resources require at least a repetition factor of 2 (which is a lot of overhead). Also, spec impact is huge. Most legacy SRS dropping rules need to be revisited with TD-OCC. |
| Google | Do not support. No clear benefit. |
| OPPO | We are open to study TD-OCC when SRS repetition is configured. |
| Huawei, HiSilicon | Agree with FL that TD-OCC can be discussed in 8 Tx SRS part. |
| MediaTek | Not Support |
| Xiaomi | Not support. The application of this scheme has too much constraint, which restricts the flexibility of SRS transmission. |
| ZTE | The repetition case is enhanced from Rel-15 to Rel-17. So it is very common case especially for cell-edged CJT UE. It has no backward compatibility issue. I believe that not every enhancement is suitable for every case. TD-OCC is suitable for common repetition case and does not introduce additional overhead and flexibility restriction. We don’t see the spec effort is huge compared with other enhancement. We don’t agree to discuss it in 8 Tx part because it is suitable for any number SRS ports and it is very good way to increase SRS capacity. |
| Spreadtrum | Open to study TD-OCC. |
| Lenovo | Support TD-OCC for multiplexing UEs with multiple symbol repetition transmission. |
| CMCC | We are open to study TD-OCC for SRS with repetition, no matter for CJT SRS or 8Tx SRS. |
| Sharp | Fine with the FL’s suggestion |
| Samsung | Not support. For capacity enhancement, since TD-OCC was already discussed in Rel-17, but no consensus. In order to down-select lots of candidate schemes, we prefer to remove TD-OCC. |
| Intel | Support TD-OCC for capacity enhancement in CJT scenario. |
| LGE | Same view with Samsung. |
| Nokia/NSB | Do not support. TD-OCC increase the SRS overhead in time. This is only applicable when SRS repetition is required, and this is not the proper scenario of CJT. |
| FL | If there is enough support for TD OCC in CJT SRS and/or 8Tx SRS, we could add a new section “SRS enhancements targeting TD OCC for CJT SRS (incl. at least 1/2/4 ports) and/or 8Tx SRS”. Proponents please try to address others’ concerns as much as possible so that we can move forward. |
| Vivo | Open to further discuss the benefit of TD-OCC and clarify the potential issue on it, such as collision, coverage performance, frequency hopping bundling and so on. From the perspective of the whole cell, it seems the capacity is not improved by TD-OCC. For example, TD-OCC can double the number of SRS ports based on two symbols, however it can be replaced by configuring two SRS resources to two Ues separately based on TDM manner, where each SRS resource is on one symbol. In other words, TD-OCC can’t help reduce the cross-SRS interference in CJT scenario. |
| Ericsson | As discussed in our contribution, one of the main challenges with SRS for CJT are delay differences between TRPs, which could cause SRS ports assigned to different cyclic shifts to collide at the receiver and would, in turn, limit the number of usable cyclic shifts. TD-OCC has the potential to increase SRS capacity for such systems as a same number of SRS ports would occupy a smaller number of cyclic shifts compared to legacy NR. Therefore, we are open to further study Enhancement E. Note that Rel-17 decisions did not target CJT and, hence, are not so relevant when discussing SRS enhancements targeting CJT. It is important to point out that TD-OCC for CJT and TD-OCC for 8 Tx are somewhat different and, hence, the discussion should not be merged. Indeed, for SRS for CJT, one could configure a TD-OCC for all ports in SRS resource whereas for SRS for 8 Tx, one configures separate TD-OCCs for ports within an SRS resource. |
| ZTE | Same view as Ericsson. The target scenario of Rel-17 enhancement is not CJT. CS hopping is also discussed in Rel-17, but now it is further discussed because of the new target scenario. In addition, it is due to time restriction to discuss more candidates in Rel-17. So we don’t agree TD-OCC is not discussed because it is discussed in Rel-17. |

### Round 2

Thank you all for the discussions. I think we can further discuss TD OCC for TDD CJT SRS. Companies please provide your suggestions on what needs to be studied for TD OCC.

Potential Proposal: For SRS TD OCC, study:

Comparison against SRS on 1 OFDM symbol

Comparison against SRS repeated on multiple OFDM symbols

Decision factors include evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, etc.

| Company | View |
| --- | --- |
| Apple | Fine to study |
| Samsung | Not support as mentioned in previous round. |
| ZTE | Comparison against SRS on 1 OFDM symbol, the TD-OCC can allow different Ues with different SRS bandwidths occupy same comb and CS. The power can be boosted especially for SRS resources with multiple SRS ports. It is suitable for large delay spread. Comparison against repeated on multiple OFDM symbols, two legacy Ues have to be allocated with different comb or CS, but now they can be allocate same comb and CS. Comparison against SRS on 1 OFDM symbol is discussed in 8 TX part in which case TD-OCC brings obvious gain, here we can just focus on comparison against repeated on multiple OFDM symbols in which case TD-OCC bring obvious capacity enhancement gain. So we suggest following updated proposal: Updated Proposal: For SRS TD OCC, study: - Comparison against SRS repeated on multiple OFDM symbols - Decision factors include evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, etc. |
| QC | Any study should also consider the amount of spec impact including SRS dropping rules. |
| LGE | Low priority. |
| Sharp | Not support |
| vivo | We think from the perspective of the whole cell, the capacity may not be improved by TD-OCC, since TD-OCC should occupy multiple symbols. Anyway, we are open to further discuss it. Two points are added in the proposal. Potential Proposal: For SRS TD OCC, study: Comparison against SRS on 1 OFDM symbol Comparison against SRS repeated on multiple OFDM symbols Comparison against two SRS resources on two different OFDM symbols Decision factors include evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc. |
| Lenovo | We are fine with focusing study SRS TD-OCC on SRS with multiple symbol repetition. And we have similar view as ZTE’s updating with deleting the first bullet. |
| Intel | Fine with the update from ZTE. |
| Xiaomi | We still concern that application condition of TD-OCC is too restricted. E.g., TD-OCC at least requests multiple SRS symbols configuration and SRS transmission of multiple UE, and once SRS dropping is occurred, TD-OCC is not able to applied. |
| OPPO | We are fine to study it as updated by ZTE. |
| Nokia/NSB | We don’t support the proposal. For CJT, we don’t think coverage is the key issue to solve. Single symbol SRS resource should be the baseline. |

### FL update 2

Thank you all for the constructive inputs.

@ZTE: Nokia/NSB suggests 1-symbol SRS should be the baseline for CJT. Could you please further clarify?

@vivo: The suggestion “Comparison against two SRS resources on two different OFDM symbols” may need more clarification. The two SRS resources are different SRS resources and each may have its separate performance metrics, e.g., two different MSEs/BLERs/throughputs. Should we consider the average MSE/etc.? Maybe the TD OCC should also configure two different SRS resources multiplexed via TD OCC?

An updated proposal is provided based on the inputs. Please try to have more Q&A.

Potential Proposal: For SRS TD OCC, study:

Comparison against SRS on 1 OFDM symbol

Comparison against SRS repeated on multiple OFDM symbols

Decision factors include evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

| Company | View |
| --- | --- |
| Samsung | Not support as mentioned in previous round. |
| ZTE | Thanks for FL’s update. @Nokia, First most CJT UE is cell-edged UE, then repetition will be used. Second, if there is no repetition case, then comparison again SRS on 1 OFDM symbol can be baseline. But repetition has been enhanced in each of Rel-15 to Rel-17, so we believe repetition is a common case. Even you assume the CJT UE is not with repetition, but we believe there is UE with repetition. If the SRS capacity of UE with the repetition is increased by TD-OCC, then it allows gNB to allocate orthogonal SRS ports to CJT UE. In addition, comparison against SRS on 1 OFDM symbol, TD-OCC has repetition gain. So now we can focus on the case where two SRS resources from two UEs with repetition, now they can be allocated with same CS and comb because they can be distinguished by TD-OCC. |
| Ericsson | Fine to study the above points. |
| vivo | We are not sure whether two different SRS resources multiplexed via TD-OCC is considered in the issue. If yes, we think it should be compared with two different SRS resources multiplexed via TDM. |
| QC | Study is ok, but we think this should be low priority due to large spec impact as well as the usefulness being limited, e.g., for 1T4R, each of the four SRS resources require at least a repetition factor of 2, resulting in 8 symbols SRS. |
| Xiaomi | We are open to study these aspects for TD-OCC. |
| Nokia/NSB | We don’t support any time domain scheme which requires higher overhead. @ZTE, CJT requires higher timing alignment, and we may not assume very large cell. |

### FL update 3

We can try to see if the group is fine with the following proposal for study. Please note that this proposal is for TDD CJT SRS; TD OCC for 8 Tx SRS can be separately discussed.

Proposal 2.5: For SRS TD OCC for SRS enhancements for TDD CJT, study:

Comparison against SRS on 1 OFDM symbol

Comparison against SRS repeated on multiple OFDM symbols

Decision factors include evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

| Company | View |
| --- | --- |

## Increasing the maximum number of cyclic shifts

At least the following potential enhancements have been discussed:

F: Increasing the maximum number of cyclic shifts (e.g., Huawei, HiSilicon, Spreadtrum, Xiaomi, Futurewei, MediaTek; 6 proponents)

Pros: Capacity enhancement; two sources show performance benefit

Cons: Applicable only for cases with short enough delay spreads

F1: Multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts (e.g., Huawei, HiSilicon)

Q1~Q4, e.g., clarification/pros/cons, can be discussed for the above enhancement.

| Company | View |
| --- | --- |
| QC | Do not support. Given the increased delay spread (due to CJT and the fact that further way TRPs need to also receive the SRS), increasing the max number of cyclic shifts does not make sense. |
| Apple | We do not prefer to increase the maximum number of cyclic shifts |
| Google | It can be discussed for 8 ports SRS. |
| OPPO | As commented before, we don’t think it is helpful to increase the max number of CSs. |
| Huawei, HiSilicon | Support F1. The feature of directly increasing the maximum number of cyclic shifts is to maintain the ZCZ property but with a reduced ZCZ length. However, the reduced ZCZ length can no longer maintain the mutual orthogonality among the SRSs, i.e., the intra-TRP cross-SRS interference will be introduced. Considering that the introduction of intra-TRP cross-SRS interference is inevitable when the SRS capacity is enhanced, how to efficiently handle the introduced interference becomes a key issue. One candidate solution is to break the ZCZ property to pursue interference equalization through multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts, which can avoid the Cons of F. Its benefit is show in [3]. |
| MediaTek | Support |
| ZTE | Same view as QC |
| Lenovo | Do not support. Same view as QC that the motivation for increasing the maximum number of cyclic shifts is not clear in case of CJT. |
| CMCC | Not support. It may not work well for CJT scenario which the delay spread is much larger than single-TRP transmission. |
| Samsung | Not support. For capacity enhancement, since this was already discussed in Rel-17, but no consensus. In order to down-select lots of candidate schemes, we prefer to remove “increasing the maximum number of cyclic shifts”. |
| Intel | Not support. It’s not necessary for CJT case. |
| Nokia/NSB | Do not support. We don’t need to waste the time with already studied items in Rel-17. |
| FL | Google made a good point that this may also be discussed in 8 Tx SRS. |
| Vivo | Share the same view with QC. The TRP delay would be large in CJT scenario, thus it is not useful to increase the maximum number of cyclic shifts. |
| Ericsson | Do not support, as pointed out by, e.g., QC, increasing the number of CSs increases the sensitivity to delay spread/shifts, which is not desirable for CJT. |

### Round 2

Thank you all for the discussions. I think we can further discuss the feasibility of increasing the maximum number of cyclic shifts, especially given the positive evaluation results. I agree that if the delay spread is long, then the number of cyclic shifts should not be increased, but not all CJT scenarios (such as Dense Urban) have long delay spread; note that a long delay is not the same as a long delay spread. Therefore, increasing the maximum number of cyclic shifts may still be discussed. Also as Google suggested, the maximum number of cyclic shifts can also be discussed in 8 Tx SRS (as there may still be a need to increase the maximum number of cyclic shifts to support 8 ports).

@Huawei HiSilicon: Can you please clarify if F1 allows for increased maximum number of cyclic shifts even if the delay spreads are long?

Potential Proposal: Study the feasibility of increasing the maximum number of cyclic shifts for SRS at least for scenarios with short delay spreads.

| Company | View |
| --- | --- |
| Apple | We prefer not to increase the maximum number of cyclic shifts |
| Samsung | Not support as mentioned in the previous round, especially share a view of QC that it is unclear increasing the number of CSs can be used in CJT scenario. |
| ZTE | Not support |
| QC | Do not support. The question is what is specific to CJT that motivates this? Just because “in some deployment models delay spread is not large” does not motivate this. Otherwise, for those deployment models, such enhancements could have been motivated also for sTRP. The point is that for the same deployment model, DS of CJT is not going to be smaller than sTRP. |
| Sharp | We agree with Samsung’s view. |
| Vivo | Not support |
| Lenovo | We also have the same view as QC, Samsung, etc and not support to increase the maximum number of cyclic shifts. |
| Intel | Low priority. Same view as Samsung and QC. |
| OPPO | Not support. |
| Nokia/NSB | Not support. |
| FL update 2 | So far it seems that there are more opponents than proponents. Proponents please try to address the concerns from other companies. If there is not enough support, we may close this discussion with a conclusion (but it may still be discussed in 8 Tx SRS as Google suggested). @QC: I agree with you that from the delay spread perspective, CJT SRS is not that different from other scenarios. However, I think this enhancement for CJT SRS can be motivated for SRS capacity. If this can increase SRS multiplexing capacity without losing orthogonality, it will be useful to CJT. Understood that this has to rely on proper gNB implementation, though. |
| Huawei, HiSilicon | @FL, Yes. F1 does allow for increased maximum number of cyclic shifts even if the delay spreads are long. And thus should not be precluded at this stage given that companies concern only focus on increasing the number of cyclic shift directly. Option F leads to a reduced zero correlation zone (ZCZ) length and becomes more sensitive to long delay spreads. While in option F1, the generated SRSs can keep a low correlation zone (LCZ) among their periodic auto-/cross-correlation functions, and this LCZ length is the same as the zero correlation zone (ZCZ) length of the legacy SRSs, i.e., not reduced. Therefore, F1 allows for an effectively increased number of cyclic shifts, and in the meanwhile is robust against long delay spreads. |
| Ericsson | Do not support. We agree with QC, Samsung, and numerous other companies that this enhancement s not targeting CJT. Even for channels with very small delay spread, increasing number of CSs may not increase SRS capacity in CJT scenarios due to delay-shift differences between TRPs. |

## Precoded SRS for DL CSI acquisition

At least the following potential enhancements have been discussed:

G: Precoded SRS for DL CSI acquisition (e.g., CMCC, Google, Huawei, HiSilicon, ZTE, Nokia, Nokia Shanghai Bell, Sony, Futurewei; 9 proponents)

Pros: Capacity enhancement; two sources show performance benefit

Cons: NW does not have full MIMO channel information; UE Tx/Rx calibration complexity

Q1~Q4, e.g., clarification/pros/cons, can be discussed for the above enhancement.

| Company | View |
| --- | --- |
| QC | Do not support. There are various issues/concerns with this scheme as we outlined in our contribution such as limiting DL max rank, the fact that actual rank also depends on other UEs’s channel condition in MU-MIMO, impact to precoders with interference nulling in MU-MIMO (which requires accurate channel info), inapplicability to practical antenna switching configurations such as 1T4R, Tx/Rx calibration at the UE, CSI-RS overhead, the fact that precoder cannot be PRG/subband-based in UL while WB precoding is not very effective for the purpose of DL CSI acquisition, etc. |
| Apple | To certain extended, precoded SRS is already supported based on proprietary solution, for example noise whitening at the UE receiver side. |
| Google | Support |
| OPPO | As commented in the previous meetings, there are many issues on this proposal. We don’t think it is practically helpful at all. |
| Huawei, HiSilicon | Support. Precoded SRS is an effective solution to reduce the number of required SRS ports to the number of PDSCH layers. Meanwhile, the gNB can still obtain the strongest rank eigenvectors of the DL channel based on the effective channel measured by SRS channel estimation. Regarding the “rank limitation”, considering that the number of PDSCH layers of majority Ues are usually less than the number of Rx with limited varying range, semi-persistently configures the number of SRS ports to a certain value smaller than the number of Rx can obtain benefit. More flexible configuration can be treated as next-step detail. Regarding the MU-precoder calculation, eigenvector-based zero-forcing can achieve good performance and is widely used. Reporting SRS precoding matrix to obtain complete DL channel for more advanced signal processing can be further studied. Regarding the Tx/Rx calibration, as long as SRS port virtualization based on CSI-RS is a candidate option for NCB, arguing NCB and precoded SRS need different calibration capability seems meaningless. Regarding the “CSI-RS overhead”, considering that CSI-RS resources are often configured for DL channel measurement to obtain more accurate CQI information in practical TDD systems due to UE’s better knowledge of DL interference and noise, precoded SRS will not incur additional CSI-RS overhead. Regarding the precoder granularity, it should be emphasized that even if the SRS uses WB precoding, the gNB can still obtain an accurate estimation result of the eigenvectors of DL channel as shown in [3], where WB precoding is used for precoded SRS and subband precoding (4 RBs) is used for DL PDSCH. Significant performance gain can be observed under DS=300ns. |
| MediaTek | Not Support |
| Xiaomi | We prefer to studying the scheme with low priority. |
| ZTE | Support |
| Lenovo | We prefer to study this kind of scheme with lower priority on account of realization complexity. If supported, more discussion may be needed for possible performance loss from channel information loss from precoded SRS, CSI-RS overhead, the impact on realization such as calibration. |
| CMCC | Support. |
| Samsung | Not support. |
| Intel | Not support. Similar view as QC and OPPO. |
| Nokia/NSB | Support. This is useful when SRS transmission is coverage limited and network would like enhanced interference awareness at UE-side. UE can determine its RX filter based on the DL RS reception and applying RX filter to SRS transmission is also providing proper UE RX information to gNB to derive the best precoder with the considering transmission channel as well as UE RX filter. |
| Vivo | Don’t support. When gNB to obtain the DL CSI, if beamformer should be reported by UE, then additional uplink resources would be occupied. Why we not use these uplink resources for SRS transmission? Then more SRS can be transmitted in different time position. Therefore, this scheme is against the chance of SRS allocation. |
| Ericsson | Do not support. Results in incomplete channel state information (i.e., not full channel knowledge) at the gNB, which is desired (especially in CJT scenarios where knowledge of the full channel is needed for accurate nullforming). Also, not applicable to practical 1TyR antenna switching configurations. |

### Round 2

Thank you all for the discussions. Given that there are 9 proponents and positive performance results, I suggest further study this scheme. Companies please feel free to suggest study aspects on top of the below potential proposal.

Potential Proposal: For precoded SRS for DL CSI acquisition, study aspects at least include:

Impact on the maximum DL rank

Impact due to partial channel information at the gNB

UE Tx/Rx calibration

CSI-RS overhead

Applicability to UE antenna configuration with fewer Tx ports than Rx ports

| Company | View |
| --- | --- |
| Samsung | Given the situation of so many candidate schemes and trying to minimize the number, we prefer not to discuss precoded SRS as opponents already showed lots of technical concerns on this scheme. |
| ZTE | Fine to discuss |
| QC | Do not support. Same view as Samsung. Also please refer to our Tdoc on the list of 9 issues / concerns. |
| LGE | Same view as Samsung. |
| Vivo | Same view as Samsung. |
| Lenovo | We prefer to lower the priority for this kind of schemes on account of potential issues as listed for study in the proposal. |
| Intel | Low priority. Same view as Samsung. |
| Xiaomi | We share same view with Samsung and QC. |
| OPPO | Same view as Samsung. |
| Nokia/NSB | We are fine to study. But, CSI-RS overhead has no relation with the proposal. |
| FL update 2 | QC’s list of issues/concerns are pasted below. Proponent please try to address them as much as possible. - Such precoding means that a max rank smaller than UE capability is RRC-configured (as the number of SRS ports should be fixed and cannot be autonomously decided by the UE). - With this, gNB has no longer access to the actual channel, and hence scheduling flexibility is seriously impacted. - It is not clear how MU-MIMO operation can be performed anymore as precoding should be a function of channel from all co-scheduled UEs. Zero-forcing precoding (or any precoder with interference nulling toward co-scheduled UEs) can no longer be used with precoded SRS. In order to do interference nulling, the full channel matrix to all UEs are needed, and also the rank can no longer be decided by the UE as it depends on other UEs’ channel. - PAPR may be impacted depending on the choice of precoding. - The type of precoder that the UE uses for SRS may require standardization as this impacts the DL scheduling decisions (unlike SRS with usage set to non-codebook). - It is not clear how such precoding can be applied for antenna switching especially in the case of xTyR with x<y as SRS transmissions will be on different OFDM symbols. As it was discussed in RAN1 #110, there is no benefit (capacity or otherwise) in case of e.g., 1T4R. Some companies argued that power is boosted due to repeating one layer four times. However, this cannot be the case as precoding for a 1-port transmission is meaningless, and obviously, UE cannot transmit from 4 Tx antennas simultaneously to form a virtual port (otherwise, this UE would have been 4T4R). - The actual benefit is not clear in terms of capacity enhancement / overhead reduction considering the additional overhead associated with two CSI-RS resources each with potentially large number of CSI-RS ports. - Tx/Rx calibrations are needed at the UE for this scheme. For NCB-based UL with associated CSI-RS, we do not think UE is mandated to calibrate Tx/Rx chains. UE may choose to do so, but the point is that it is not required. For example, UE may only do antennas selection or only do SRS port virtualization based on CSI-RS. Calibrating Tx/Rx chains is very difficult for UE. This is not an issue for NCB-based PUSCH as the SRS is measured by gNB based on which one or more SRS resources are indicated to the UE for PUSCH transmission. However, the calibration seems to be necessary for this scheme to work since the estimated channel is directly used for PDSCH precoding later (unlike NCB-based PUSCH, where gNB anyway gets to measure multiple SRS resources and indicate one or more of them for PUSCH transmission). - Such precoding based on CSI-RS for the purpose of DL CSI acquisition is not supported even for the single-TRP case (e.g., based on a single CSI-RS resource). It needs to be first justified and supported for single-TRP (with considering the issues mentioned above) before extension to mTRP/CJT can be considered. |
| Huawei, HiSilicon | Although we do not want to repeat ourselves again, now that the questions are repeatedly proposed (although some of them are indeed valuable, and is consequently captured in the potential proposal), following are our previous response: Regarding the “rank limitation”, considering that the number of PDSCH layers of majority Ues are usually less than the number of Rx with limited varying range, semi-persistently configures the number of SRS ports to a certain value smaller than the number of Rx can obtain benefit. More flexible configuration can be treated as next-step detail. Regarding the MU-precoder calculation, eigenvector-based zero-forcing can achieve good performance and is widely used. Reporting SRS precoding matrix to obtain complete DL channel for more advanced signal processing can be further studied. Regarding the Tx/Rx calibration, as long as SRS port virtualization based on CSI-RS is a candidate option for NCB, arguing NCB and precoded SRS need different calibration capability seems meaningless. Regarding the “CSI-RS overhead”, considering that CSI-RS resources are often configured for DL channel measurement to obtain more accurate CQI information in practical TDD systems due to UE’s better knowledge of DL interference and noise, precoded SRS will not incur additional CSI-RS overhead. Regarding the precoder granularity, it should be emphasized that even if the SRS uses WB precoding, the gNB can still obtain an accurate estimation result of the eigenvectors of DL channel as shown in [3], where WB precoding is used for precoded SRS and subband precoding (4 RBs) is used for DL PDSCH. Significant performance gain can be observed under DS=300ns. Considering that significant performance gain has been shown in [3], we think precoded SRS is a promising solution for SRS capacity enhancement that deserves further discussion. |
| Ericsson | Given that a number of technical concerns with this scheme have been raised and since it can’t be used for common 1TyR antenna switching configurations, we prefer to not study this scheme further. |
| QC | Thanks Huawei for the follow-up. As we discussed in our Tdoc (and FL copied above), Tx/Rx calibration is not mandated for NCB. However, for this scheme for the purpose of DL CSI acquisition, not having Tx-Rx calibration is not an option. Also, I assume we all agree that this scheme is not applicable to more practical 1TyR antenna switching configurations. Furthermore, as you suggested, some (but not all) of the concerns such as maxrank can be addressed by additional enhancements such as flexible signaling of SRS port number, but this requires a lot of discussions / spec change. Overall, we think we should focus on other schemes instead at this point given these concerns. |

## Enhanced signaling for flexible SRS transmission

At least the following potential enhancements have been discussed:

H: Enhanced signaling for flexible SRS transmission, e.g., dynamic update of SRS parameters (e.g., Interdigital, Intel, Xiaomi, Samsung, CMCC, Qualcomm, NTT DOCOMO, Sharp, Ericsson, Futurewei, vivo, Lenovo, LG; 13 proponents)

E.g, frequency-domain parameter (e.g., BW change, comb change and/or hopping location change), power control parameter, activation by DCI for SP SRS, RPFS parameters, number of antennas in antenna switching, cancellation indication in DCI format 2_4, etc.

Pros: Flexible SRS for interference randomization

Cons: Higher signaling overhead

These enhancements may be related to those in Sec. 2.1, 2.3, 2.4, 2.9, and 2.11.

Based on the large number of supports, the FL suggests to focus on further discussion on enhanced signaling for flexible SRS transmission (e.g., dynamic update of SRS parameters) and provide views on the next study point / decision point. In addition, companies can further clarify which parameters are to be updated in the enhanced signaling. Q1~Q4, e.g., clarification/pros/cons, can be discussed for the above enhancement.

| Company | View |
| --- | --- |
| QC | Support, but suggest to narrow-down to P/SP-SRS. For AP-SRS, existing mechanisms allow to define multiple SRS resource sets and map them to one or more “SRS request” codepoints. |
| Apple | Low priority |
| InterDigital | Support, and we think this enhancement applies to any of P/SP/AP-SRS. |
| Google | Do not support. Benefit is minor and the proposal contains too many study points. |
| OPPO | We don’t think the proponents have common understanding on the SRS parameters that need dynamic update. It would leave significant standardization effort for down selection if we only agree on a high-level solution. |
| Huawei, HiSilicon | Agree with OPPO that further down-selection maybe needed. |
| MediaTek | We are open to discussing this further. Agree with OPPO. |
| Xiaomi | We are open to study or further down select the scheme. |
| ZTE | Agree with OPPO that down-selection is needed. |
| Spreadtrum | Low priority and benefit is still unclear. |
| Lenovo | We are open for discussion. Agree with OPPO and other companies that down-selection is needed for SRS parameters with dynamic change. |
| CMCC | We are open to further study and down-selection can be helpful to reduce workload. |
| Samsung | We are open to discuss and down-select is needed. |
| Intel | The proposal is too wide. |
| LGE | Support to discuss. |
| Nokia/NSB | We don’t see the need. AP-SRS can be used for this purpose. SP-SRS can be switched. P-SRS should not be dynamically changing its configuration. |
| FL | Based on the inputs from the contributions and the summary document so far, we can consider down-selection within this category. The FL suggest a potential proposal below to encourage more discussion on the next study point / decision point; any other suggestions (e.g., code/sequence domain parameters, TRP index, etc.) are still welcome. Potential Proposal: For enhanced signaling for flexible SRS transmission, e.g., dynamic update of SRS parameters, down select one or more SRS parameters to be updated by enhanced signaling from the following list: Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change) Power control parameter Activation by DCI for SP SRS RPFS parameters Number of antennas in antenna switching Cancellation indication in DCI format 2_4 |
| vivo | The detail and benefit are not clear, prefer to treat is with low priority. |
| QC | From the potential proposals above, we support “activation by DCI for SP SRS”. This can be used to activate SRS dynamically (activation DCI), change SRS params dynamically (reactivation DCI), or for dynamically deactivate SRS (release DCI). This functionality is supported for CSI (SP CSI on PUSCH) as well as CG/SPS, but is not supported for SRS yet. |

### FL update

Based on the large number of proponents, the FL suggests the following proposal:

Proposal 2.8: Support enhanced signaling for flexible SRS transmission, e.g., dynamic update of SRS parameters, and down select one or more SRS parameters to be updated by enhanced signaling from the following list:

Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change)

Power control parameter

Activation by DCI for SP SRS

RPFS parameters

Number of antennas in antenna switching

Cancellation indication in DCI format 2_4

| Company | View |
| --- | --- |
| QC | We suggest the following given that some bullets are not only related to SRS parameters. Also, code domain parameters are included as they play an important role in orthogonalization: Proposal 2.8: Support enhanced signaling for flexible SRS transmission, e.g., dynamic update of SRS parameters, and down select one or more SRS parameters to be updated by enhanced signaling from the following list: For dynamic update of SRS parameters, study one or more of the following parameters Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change) Power control parameter RPFS parameters Number of antennas in antenna switching Code-domain parameters (cyclic shift / SRS sequence) Activation by DCI for SP SRS Enhanced cancellation indication in DCI format 2_4 |
| ZTE | When TRP is switched, the can also be updated by DCI. In addition, the time domain parameter, such as OFDM symbol index, can be updated by DCI considering the co-scheduled UE may changes. So e suggest the and time domain parameter are listed in above list. |
| Sharp | In our view, dynamic update of code-domain parameter regarding cyclic shift is beneficial, but it is included in Proposal 2.2. For this reason, Proposal 2.8 is unnecessary for us. |
| OPPO | We agree with Nokia that AP SRS can achieve the same flexibility. We still cannot see the necessity to introduce this enhancement. Furthermore, it needs significant standardization effort for further down selection, considering the proponents of different parameters. |
| Xiaomi | We are fine with FL’s proposal and QC’s rewording. |
| LGE | OK with QC’s revision. We support dynamic update of frequency domain parameters. This can achieve instance interference avoidance when gNB triggers aperiodic SRS, where the current specification only support RRC-level update of SRS bandwidth/location. |

### Round 2

Tuesday GTW proposal:

Proposal 2.8-A: Support enhanced signaling for flexible SRS transmission, and down select one or more enhanced signaling from the following list:

For dynamic update of SRS parameters, study one or more of the following parameters

Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change)

Power control parameter

RPFS parameters

Number of antennas in antenna switching

Code-domain parameters (cyclic shift / SRS sequence)

Time-domain parameters

Activation by DCI for SP SRS

Enhanced cancellation indication in DCI format 2_4

Thank you all for the discussions. The above proposal was based on QC/ZTE’s suggestions and was briefly discussed in the GTW session. I suggest the following proposal for further discussion.

Proposal 2.8-B: Support enhanced signaling for flexible SRS transmission, and down select one or more enhanced signaling from the following list:

Adopt one or more of the following parameters for dynamic update of SRS parameters

Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change)

Power control parameter

RPFS parameters

Number of antennas in antenna switching

Code-domain parameters (cyclic shift / SRS sequence)

Time-domain parameters

Activation by DCI for SP SRS

Enhanced cancellation indication in DCI format 2_4

| Company | View |
| --- | --- |
| Apple | We prefer to minimize the usage of “downselect one”. For this one, we do not support to downselect. |
| Samsung | Given the situation of so many candidate schemes and trying to minimize the number, we prefer not to have proposal 2.8-B, which each view of proponent is still diverged and was already discussed in the last GTW. |
| ZTE | Same view as Samsung |
| QC | Ok, but seems SRS sequence is listed two times (is the same as SRS sequence) |
| DOCOMO | We support. On the comments raised in GTW, we think it would be difficult to say legacy A-SRS can be an alternative instead of this enhancement. A-SRS requires a DCI triggering per SRS occasion, which causes an overhead. If A-SRS can just be reused, we actually do not see of any other SRS enhancements in this summary. |
| LGE | Support the proposal. Similar view as DOCOMO. |
| vivo | Support to study, but we think it’s too early to say we need down-select one in proposal at this stage. Proposal 2.8-B: Study enhanced signaling for flexible SRS transmission from the following list: Adopt one or more of the following parameters for dynamic update of SRS parameters Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change) Power control parameter RPFS parameters Number of antennas in antenna switching Code-domain parameters (cyclic shift / SRS sequence) Time-domain parameters Activation by DCI for SP SRS Enhanced cancellation indication in DCI format 2_4, e.g., Association with SRS resource(s) Periodic/persistent cancellation |
| Lenovo | We have similar view with Samsung, ZTE and more discussion is needed on aligning proponent view and how to make down-selection for these schemes in the long list. |
| Intel | Given a lot of candidates in this proposal, it will take a lot of efforts to proceed the discussion. Considering we have 11 alternatives for CJT, suggest to put this proposal low priority. In addition, we think some items could be discussed in other sections. For example, the frequency domain parameters could be discussed in Section 2.1, the code domain parameters could be discussed in Section 2.2, the power control parameters could be discussed in Section 2.4. |
| Xiaomi | We are fine with the proposal. |
| ZTE | Same view as Samsung, ZTE and Lenovo. |
| Nokia/NSB | We don’t support. We still think that AP-SRS can already support the functionality. No need for study. |

### FL update 2

Thank you all for the discussions.

@Apple: Can you please clarify “do not support to downselect”?

@QC: The sequence depends on the sequence ID / sequence hopping / group hopping. So I think 
SRS sequence is a bit more general.

@vivo: We can try both versions and see what companies prefer.

@Intel: Agree that some of these can be discussed in other sections.

It is true that there are a lot of candidates in this proposal. Since this is the 3rd meeting of the WI, I suggest that we keep discussing them for now, and down selection can be made later this meeting or in the next meeting for progress.

Proponents please feel free to provide proposals in the other sections as Intel suggested.

Proposal 2.8-C: Study enhanced signaling for flexible SRS transmission from the following list:

Adopt one or more of the following parameters for dynamic update of SRS parameters

Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change)

Power control parameter

RPFS parameters

Number of antennas in antenna switching

Code-domain parameters (cyclic shift / SRS sequence)

Time-domain parameters

Activation by DCI for SP SRS

Enhanced cancellation indication in DCI format 2_4, e.g.,

Association with SRS resource(s)

Periodic/persistent cancellation

Proposal 2.8-D: Support enhanced signaling for flexible SRS transmission, and down select one or more enhanced signaling from the following list:

Adopt one or more of the following parameters for dynamic update of SRS parameters

Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change)

Power control parameter

RPFS parameters

Number of antennas in antenna switching

Code-domain parameters (cyclic shift / SRS sequence)

Time-domain parameters

Activation by DCI for SP SRS

Enhanced cancellation indication in DCI format 2_4, e.g.,

Association with SRS resource(s)

Periodic/persistent cancellation

| Company | View |
| --- | --- |
| Samsung | We prefer not to discuss on this area and suggest to focus on other possible schemes. |
| DOCOMO | Support |
| LGE | Support. Also, Proposal 2.8-C is also fine for us, if it is too early to support specific schemes. |
| vivo | Prefer Proposal 2.8-C with further study at this stage. |
| QC | Study is more appropriate in our view given the wide variety of schemes under this category. |
| Xiaomi | We prefer to study these aspects firstly. |
| Nokia/NSB | We still don’t know the main benefit over AP-SRS. NW has enough flexibility for SRS configuration. |

### FL update 3

Thank you all for the discussions. We can focus on the following proposal for study.

Proposal 2.8-C: Study enhanced signaling for flexible SRS transmission from the following list:

Adopt one or more of the following parameters for dynamic update of SRS parameters

Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change)

Power control parameter

RPFS parameters

Number of antennas in antenna switching

Code-domain parameters (cyclic shift / SRS sequence)

Time-domain parameters

Activation by DCI for SP SRS

Enhanced cancellation indication in DCI format 2_4, e.g.,

Association with SRS resource(s)

Periodic/persistent cancellation

| Company | View |
| --- | --- |

## Partial frequency sounding extensions

At least the following potential enhancements have been discussed:

I: Partial frequency sounding extensions (e.g., ZTE, Xiaomi, NTT DOCOMO, Futurewei, Huawei, HiSilicon; 6 proponents)

I1: Larger partial frequency sounding factor (e.g., ZTE, Xiaomi, NTT DOCOMO, Futurewei, Spreadtrum)

Pros: SRS capacity enhancement

Cons: Need more transmissions to cover the desired bandwidth

I2: Starting RB location hopping enhancements (e.g., Futurewei, Xiaomi, Huawei, HiSilicon, Spreadtrum)

Pros: Interference randomization

Cons: Non-uniform SRS sample pattern in time/frequency domain

I3: Partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS (e.g., ZTE)

Pros: Interference randomization

I4: Dynamic changing partial sounding parameters (e.g., Intel, ZTE, LGE)

Pros: Flexible SRS for interference randomization

Cons: Higher signaling overhead

Some of these enhancements may be related to those in Sec. 2.8 and 2.11.

Q1~Q4, e.g., clarification/pros/cons, can be discussed for all above enhancements.

| Company | View |
| --- | --- |
| QC | Do not support I1-I4. These have been discussed in Rel-17, and so far, we have not seen enough justification (e.g., evaluation results) that can justify re-opening all these aspects again. |
| Google | Do not support. It has already been studied in R17. |
| OPPO | Do not support. |
| Huawei, HiSilicon | Open for I2. |
| MediaTek | Not Support |
| Xiaomi | Support and prefer to I1 and I2. |
| ZTE | Open for I2 and I3 |
| Spreadtrum | Support I1 and I2. |
| Lenovo | Do not support. Similar view with Google. |
| CMCC | Not support. RPFS design has already been studied in R17. |
| Sharp | Not support. |
| Samsung | Not support. Already extensively discussed in Rel-17. |
| Intel | Low priority. |
| Nokia/NSB | Do not support. Same view as QC, Google, CMCC and Samsung. |
| Vivo | Don’t support. |
| Ericsson | Do not support. |

### Round 2

So far there does not seem to be enough interest in this category. Interested companies can still discuss and suggest potential proposals.

| Company | View |
| --- | --- |
| Xiaomi | For I2, it can be regarded as one of resource mapping with randomization. Since different starting RB location implies that SRS are transmitted at different resource elements. Hence, we suggest I2 is merged into K., i.e., Resource mapping for SRS transmission based on network-provided parameters or system parameters. |
| FL update 2 | @Xiaomi: Can you please clarify if this is for RRC or MAC/DCI? Please note that companies are proposing to merge K into respective enhancements. There seems to be low interest in this category. The group can decide whether to close this discussion for Rel-18 by the end of this meeting. |
| Xiaomi | @FL, our intention is that the starting RB location can be dynamically changed based on the network-provided ID (different from cell ID) which can be configured by RRC. We are also fine to merge K into this solution if other companies are interested it. |

## Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

At least the following potential enhancements have been discussed:

J: Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment (e.g., Samsung, Qualcomm, Futurewei; 3 proponents)

J1: Configuration of v (sequence index within a group) per SRS resource (e.g., Qualcomm)

Pros: Improved reused factor

J2: Configuration of cyclic shift per SRS port per SRS resource (e.g., Samsung, Qualcomm, Futurewei)

Pros: Optimized CS allocation; two sources show performance benefit

Q1~Q4, e.g., clarification/pros/cons, can be discussed for all above enhancements.

| Company | View |
| --- | --- |
| QC | Support both J1 and J2. These two enhancements are very simple yet effective in terms of increasing SRS efficiency. Regarding J1, there were some misunderstanding last time. Please note that this enhancement is not related to interference randomization. Instead, the benefit is enhancing the reuse factor of SRS sequence, i.e., more SRS sequences can be configured by the network to ensure that 2 UEs with the same SRS sequence are far away and do not create inter-cell / inter-cluster interference. This is for the case that network carefully assigns SRS sequences (not when network enables randomization). Regarding J2, we would like to address the following concerns mentioned by companies in the previous meeting for this scheme (please also see the evaluation results in our contribution): - “Backward compatibility”: There is no backward compatibility issue as the proposal is to configure cyclic shift per SRS port. When legacy Ues are multiplexed, the occupied cyclic shifts are simply not configured for the Rel-18 UE. - “DCI overhead”: The proposal is not related to dynamic indication of cyclic shift. Instead, it is related to enhanced RRC configuration. The additional RRC overhead is negligible as only configuration of one cyclic shift per port is needed (e.g., for SRS resource with 4 ports, three cyclic shifts in addition to the existing initial cyclic shift for the first port can be RRC-configured). - “Increase the channel estimation complexity”: This is not correct as different ports are on different cyclic shifts whether evenly / uniformly distributed or not. For channel estimation at the gNB, the only knowledge needed is cyclic shift per port, and the rest is same as legacy. - “The scheme cannot enhance SU performance”: This is true only if SU is just referring to SRS transmission since conditioned on a single UE transmitting SRS on a set of Res, the best cyclic shift assignment across ports is obviously uniform, which achieves the highest cyclic shift spacing among the ports. However, if SU refers to DL scheduling, this may not be true since multiple Ues can still use the same comb offset and symbols (and be separated by different cyclic shifts) for the purpose of SRS overhead reduction. The point is that, as long as there are more than one UE transmitting SRS on the same set of Res using different cyclic shifts, the scheme allows for more optimized configuration of cyclic shift per SRS port for each UE. |
| Google | Do not support. Benefit is minor. |
| OPPO | We still think the benefit is not worthy of the standardization effort. |
| Huawei, HiSilicon | Regarding J1, the benefit should be justified. Regarding J2, the benefit scenario seems limited (i.e., large delay/delay spread difference is needed). |
| MediaTek | Do not support |
| Xiaomi | We prefer to studying the scheme with low priority. |
| ZTE | Same view as Google |
| Lenovo | We think the priority is lower for this kind of schemes. |
| CMCC | We prefer put the scheme with low priority. |
| Samsung | Support at least J2 and same view with Qualcomm. The proposal is just configuring cyclic shift per SRS port in order to avoid/mitigate interference. Performance gain is achieved by very simple approach. |
| Intel | Low priority. |
| LGE | Low priority. |
| Nokia/NSB | Do not support. |
| Vivo | Low priority. |
| QC | Given that two companies evaluated the performance and showed the benefit, and also given the small spec impact for these two enhancements, we think this should be prioritized. Also, we did not see any concern / issue being raised by companies above other than stating their preference. |
| Ericsson | We are open to continued study of J2 as this could increase the tolerance towards delay shifts, which is important for CJT (legacy equidistant configuration may not be optimal in such scenarios). However, updating the SRS port-to-cyclic-shift mapping formula may be preferred over configuring a cyclic shift per SRS port. Indeed, it is unclear how a per-port allocation would be compatible together with, e.g., a cyclic-shift hopping scheme. Hence, we propose to widen the scope to: “J2: Configuration of non-equidistant cyclic shifts for a multi-port SRS resource”. |

### Round 2

Thank you for the discussions. Given the positive evaluation results so far, I suggest further study the enhancements in this category. A potential proposal is provided, and interested companies can provide revisions / alternative potential proposals for discussion.

Potential Proposal: For enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment, at least study the configuration of cyclic shift per SRS port per SRS resource with possibly non-equidistant cyclic shifts for the multi-port SRS resource.

| Company | View |
| --- | --- |
| Apple | Fine to study |
| Samsung | Support potential proposal to study. |
| ZTE | Not support. The gain is mirror and have some issue to increase the number of SRS resources considering MU schedule and SU schedule needs different SRS resource compared with legacy method. |
| QC | Seems the only new part is the last part suggested by Ericsson. Suggest the following: Potential Proposal: Study whether / how to allow non-equidistant cyclic shifts for the multi-port SRS resource, e.g., by configuration of cyclic shift per SRS port per SRS resource. @ZTE: I did not understand your concern above. Can you please elaborate a bit more (maybe with some examples)? |
| LGE | Low priority. |
| ZTE | @QC. For example, there is a first SRS resource with two SRS ports in one comb. If there is no a second SRS scheduled on the same comb, then the optimal CS allocation of the two ports of the first SRS resource is equidistant cyclic shifts as done by legacy allocation, otherwise it may be non-equidistant cyclic shifts as you mentioned. Then the first SRS resource can not be used in the first SU case and the second MU case. |
| Lenovo | We prefer to lower the priority for this kind of schemes on account of many available candidates and limited supporters. |
| Intel | Low priority. |
| Xiaomi | We are fine to study. |
| OPPO | Low priority. |
| Nokia/NSB | We don’t see benefit. |
| QC | @ZTE: Thank you for the response. I assume “MU / SU” is not referring to PDSCH scheduling but it is referring to SRS multiplexing across multiple UEs in cyclic shift domain. If so, let’s focus on the mixed case that you mentioned, i.e., for one SRS resource, in one occasion is MU and in another occasion is SU. Then: - W/o this enhancement, the MU occasion suffers - With non-equidistant cyclic shifts, the SU occasion suffers Hence, we cannot say “SRS resource cannot be used” (otherwise, with the same argument, such SRS resource cannot be also in legacy due to MU occasion suffering). The whole point of this enhancement is when SRS of multiple UEs are multiplexed in cyclic shift domain, which is a typical case in CJT. If the same SRS resource is used for both SU and MU, then network can still configure the per-port cyclic shift to achieve the right tradeoff. Two extreme cases of this configuration are “optimized for SU” and “optimizes for MU”, there are also other possibilities to achieve a trade-off in between. The point is that even in this mixed case, the overall performance cannot be worse than legacy. |

### FL update 2

There are some good technical discussions above. Please continue. As QC suggested, the proposal is updated below.

Potential Proposal: Study whether / how to allow non-equidistant cyclic shifts for the multi-port SRS resource, e.g., by configuration of cyclic shift per SRS port per SRS resource.

| Company | View |
| --- | --- |
| Samsung | Support, and same view with QC’s analysis for mixed case of MU and SU. |
| DOCOMO | Ok |
| ZTE | @ QC, Thanks for your further explanation. We still think it is difficulty for gNB to configure one SRS resource of a first UE to be suitable for different co-scheduling UE of the first UE. In addition, we wonder whether the gNB knows the delay difference between different UEs and delay spread of each UE. |
| LGE | We are not sure there is clear benefit compared to specification efforts. Since there are too many schemes to be discussed, prefer to have priority on schemes with majority support. |
| Ericsson | Support, but propose to update/generalize the proposal as follows: Potential Proposal: Study whether / how to allow non-equidistant cyclic shifts for the multi-port SRS resource, e.g., by new mapping of SRS ports to cyclic shifts or by configuration of cyclic shift per SRS port per SRS resource. As mentioned above, we have some concern whether/how per-port configuration would be compatible, e.g., with cyclic-shift hopping. |
| QC | Support. @ZTE: Regarding whether the gNB knows the delay difference between different UEs: Yes, we think gNB knows this based on TA or other implementation methods such as estimate based on positioning of UEs. For delay spread of each UE, this can be obviously estimated from any UL channels / RS. @Ericsson: Per-port configuration is compatible with cyclic-shift hopping. The hopping formula is the same for all ports (obviously, we do not want to rely on randomization between different ports of the UE). Then, for each port, the configured value is added to the hopping formula modulo nSRScs,max. In other words, the cyclic shift spacing between ports is not impacted by hopping. I hope this clarifies and eliminates the need for “new mapping of SRS ports to cyclic shifts” because such new mappings probably requires a lot of discussions. |
| Nokia/NSB | Unless the scheme has a fundamental difference from CS hopping or comb-offset hopping, we don’t prefer to study another schemes. |
| FL update 3 | Ericsson’s proposal on new CS mapping seems to be more general than QC’s proposal. It is fine to discuss either the broader one or the narrower one. Ericsson can further clarify what the new CS mappings may be, e.g., Option 1: per port configuration of CS, Option 2 / Option 3, etc. |

## Resource mapping for SRS transmission based on network-provided parameters or system parameters

At least the following potential enhancements have been discussed:

K: Resource mapping for SRS transmission based on network-provided parameters or system parameters (e.g., ZTE, Spreadtrum, Lenovo, NTT DOCOMO, Futurewei, Xiaomi, NEC; 7 proponents)

K1: Based on network-provided parameters, e.g., network-provided ID (e.g., Spreadtrum, Lenovo, NTT DOCOMO, Futurewei, ZTE, Xiaomi, NEC)

E.g., based on network-provided ID (different from cell ID) at least for nIDSRS, which may also be related to TRP IDs of the CJT network

Pros: Interference randomization

K2: Based on system parameters, e.g., time indexes (e.g., ZTE, Lenovo, NTT DOCOMO, Futurewei, NEC)

E.g., based on OFDM symbol index, slot index, radio frame index, etc.

Pros: Interference randomization

These enhancements may be related (or even essential) to those in Sec. 2.1, 2.2, 2.3, 2.4, 2.8, 2.9, and 2.11.

Q1~Q4, e.g., clarification/pros/cons, can be discussed for all above enhancements.

| Company | View |
| --- | --- |
| QC | Randomization is already based on slot/symbol index. The enhancement that some companies have in mind is related to “not resetting c-init every radio frame”, but this is not clear from the description. Also, it is not clear if this is for the purpose of legacy hopping (group/sequence hopping) or the new hopping’s discussed in Sections 2.1/2.2/2.3. |
| Google | Do not support. We do not see clear benefit. |
| OPPO | We think current interference randomization based on UE specific ID and slot/symbol index is sufficient. |
| Huawei, HiSilicon | Regarding K1, it seems already be supported in current Spec. Regarding K2, it can be discussed in B2. |
| MediaTek | Not Support |
| Xiaomi | Support K1 and prefer to jointly discussing K1 and A2. |
| ZTE | Open for K2 |
| Spreadtrum | Both K1 and K2 could be further studied. |
| Lenovo | We think this topic may be discussed together with A2/B1 if needed. The network-provided parameters can be related with coordination on Comb hopping set or cyclic shift hopping set. |
| CMCC | Not clear if it is related with legacy hopping or the new enhancement in above section. If it is more related to those in above sections, we think it’s better to first discuss the new enhancement in the above sections. |
| Samsung | Not support |
| Intel | Low priority. |
| LGE | Low priority. |
| Nokia/NSB | Not clear the proposal. Is this like a signaling/implementing of different schemes? |
| FL | @QC @ CMCC: I believe these are for the new enhancements in above sections. @Nokia/NSB: The scheme K is to allow the SRS transmission parameters to be dependent on, e.g., TRP ID, other configured ID, and/or slot/symbol index. It may be necessary for hopping / randomization. For example, existing group hopping depends on symbol index and SRS sequence identity nIDSRS, and new hopping / randomization enhancements may require similar dependency to be supported. |
| Vivo | Low priority. |
| Ericsson | Do not support. |
| DOCOMO | Yes, randomization is already based on slot/symbol index as well as SRS ID which is configurable. We agree with Lenovo that it is a kind of sequence-domain randomization. Even though it is called as “randomization”, we believe the exact “randomized” sequence should be visible to NW/gNB. We believe changing SRS ID should be considered very carefully; otherwise there is quite a lot of inter-cell interference. We do not thins CJT operation means we can ignore inter-cell interference. The concept of this “randomization based on TRP ID” is quite similar to the existing sequence / sequence group hopping randomization based on ID/slot id/symbol id. If SRS for a TRP doesn’t interfere the one for another TRP, there should be no problem in this WI. Meanwhile, companies admit that there is an issue on that in our view. With above, we are ok with discussing K1 and/or K2 together with B. |

### Round 2

Thank you for the discussions. As clarified by some proponents, the schemes can be discussed together with schemes A2 / B and so on. Please let the group know if this is ok, and interested companies can still provide potential proposals if needed.

| Company | View |
| --- | --- |
| Samsung | We don’t support on studying this category as standalone scheme, but this category seems more appropriate on being mixed in other schemes, which is already mixed with comb-offset hopping. |
| ZTE | Fine to combined with A2/B |
| Lenovo | We are fine to discuss together with A2/B |
| Xiaomi | As commented for I2, i.e., starting RB location hopping enhancements, I2 can be merged into this solution. Hence, we suggest K1 can be rewording as follows. K1: Based on network-provided parameters, e.g., network-provided ID and/or starting RB location. |
| FL update 2 | @Xiaomi: Please note that companies are proposing to merge this category into other enhancements. We may close this discussion based on the discussions so far. Any views can still be shared in this round. |
| Xiaomi | @FL, as comment given for solution I (partial frequency sounding extensions) by us. we are also fine to discuss this solution together with I. |

### FL update 3

Companies are fine to discuss the enhancements in this category in other categories. If needed we can draw the following conclusion.

Proposal for conclusion: The discussion of resource mapping for SRS transmission based on network-provided parameters or system parameters is merged into the discussions of other SRS enhancements for TDD CJT.

## Others

Any other views, issues, potential enhancements, and clarifications, if any, can be provided in below table.

| Company | View |
| --- | --- |

# SRS enhancements targeting 8 Tx operation

It is well known that increasing UE Tx antenna ports can significantly improve various performance metrics for UL/DL transmissions. 8 Tx transmissions can be feasible for at least CPE/FWA/vehicle/industrial devices and hence can be beneficial.

Some remaining issues on the number of SRS resources for 8 Tx SRS and the number of SRS resource sets for 8 Tx SRS will be discussed in agenda item 9.1.4.2 covering “SRI/TPMI enhancement for enabling 8 TX UL transmission; To support up to 4 or more layers per UE in UL targeting CPE/FWA/vehicle/industrial devices”.

## Whether and how to support 8 ports in one or multiple OFDM symbols for 8 Tx SRS

Regarding whether and how to support 8 ports in one or multiple OFDM symbols for 8 Tx SRS, the following has been agreed in previous RAN1 meetings:

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Therefore, in this meeting, the group may decide on the following issues:

For 8-port SRS resource(s) or SRS resource set(s) with usage ‘codebook’, whether to support the 8 ports in one and/or multiple OFDM symbols

Options include one OFDM symbol only, multiple OFDM symbols only, and one or multiple OFDM symbols

Pros and cons of the options can be analyzed, e.g., SRS overhead, per-symbol per-port transmission power, etc.

For an 8-port SRS resource in an SRS resource set with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), whether to support the 8 ports in multiple OFDM symbols

Pros and cons can be analyzed, e.g., SRS overhead, per-symbol per-port transmission power, etc.

When 8-port SRS resource(s) or SRS resource set(s) in multiple OFDM symbols is/are to be supported in a non-repetition way, high-level designs on how to support the 8 ports in the multiple OFDM symbols

E.g., different ports are mapped to different OFDM symbols, i.e., TDMed 8 ports

E.g., the 8 ports are mapped to different OFDM symbols according to TD OCC, i.e., 8 ports with TD-OCC

The discussions may be different for different usages (for CB, it depends on the outcome of the first bullet point)

When 8-port SRS resource(s) or SRS resource set(s) in one or multiple OFDM symbols is/are to be supported, detailed designs on how to support the 8 ports in the multiple OFDM symbols

E.g., comb and comb offset allocation, cyclic shift allocation, etc.

Detailed discussions are provided in below subsections.

### Whether to support 8 ports in one or multiple OFDM symbols for 8 Tx SRS for CB

Please provide inputs to the following discussion point:

For 8-port SRS resource(s) or SRS resource set(s) with usage ‘codebook’, whether to support the 8 ports in one and/or multiple OFDM symbols

Options include:

Option 1: Only one OFDM symbol

Supported by NTT DOCOMO, Ericsson, Intel, Sharp, Google, Nokia (open to Option 3), Nokia Shang Bell (open to Option 3); 7 proponents

Option 3: One or multiple OFDM symbols

Supported by CATT, CEWiT, CMCC, Futurewei, InterDigital, KDDI, Lenovo, LG, OPPO, Qualcomm, Samsung, Spreadtrum, ZTE,Xiaomi, Huawei, HiSilicon, MediaTek, vivo (wait for 9.1.4.2) , Ericsson (wait for 9.1.4.2); 19 proponents

Pros and cons of the options can be analyzed, e.g., SRS overhead, per-symbol per-port transmission power, etc.

| Company | View |
| --- | --- |
| Apple | - 8 ports SRS should be supported for CB - The baseline is 8 port SRS in one SRS-Resource and in minimum 1 symbol |
| Google | Support option 1 only. We failed to see the necessity for >1 symbol SRS. |
| OPPO | We think two symbols can be beneficial for power-limited Ues, while one symbol can be applied to other Ues. |
| Huawei, HiSilicon | Please move our position to Option 3. |
| MediaTek | We support Option 3. |
| Xiaomi | We support Option 3 as TDM multiplexing of 8 ports or repetitions can all be applicable. |
| ZTE | We support Option 3. |
| Spreadtrum | Support option.3 |
| Lenovo | Sounding 8 antennas over two symbols can improve the sounding coverage and also be beneficial for power-limited UE. We support option 3. |
| CMCC | Pros of multiple OFDM symbols: it is beneficial for cell edge UE or power-limited UE, which can sound the 8 ports in multiple OFDM symbols, with guaranteed sounding quality. |
| Sharp | We support Option 1 (an 8-port SRS resource of an SRS resource set on one OFDM symbol). For 8-ports in multiple OFDM symbols, partial dropping of SRS ports occurs. In our view, even for cell edge, Option 1 is beneficial by using repetition. |
| Samsung | Support Option 3. |
| Intel | Support Option 1. Using multiple symbols for 8-ports occupies more time domain resource. Regarding the power benefit, with the same amount of time resource, Option 1 can configure SRS with repetition which is also beneficial for coverage. |
| KDDI | We support Option 3. Considering the orthogonality of CS multiplexing, we believe that transmitting 8-port SRS with multiple OFDM symbols will provide better coverage than repetition with a single OFDM symbol. Transmission with 1 OFDM symbol may be effective in cases where transmit power is not an issue, such as in the center of a cell. We should be able to use both single and multiple OFDM symbol(s). |
| Nokia/NSB | Support Option 1. Also open to Option 3 if clear benefit is justified. Do not support Option 2. |
| QC | We don’t have very strong view here. But we slightly prefer allow mapping a 8-ports SRS resource for usage “codebook” to >1 OFDM symbols in TDM manner for flexibility and better coverage. Yes, repetition can achieve the same coverage. But repetition would double the number of CS/comb resources to achieve the same coverage, unless TD-OCC is introduced. However, we don’t support introduce TD-OCC across >1 OFDM symbols, because the orthogonality of TD-OCC breaks with freq hopping, UL transmission prioritization, and high Doppler. |
| FL | For this discussion point and a few below, the group can further discuss the pros and cons for: 8 ports on 1 OFDM symbol 8 ports repeated on each of multiple OFDM symbols 8 ports on multiple OFDM symbols, different ports are mapped to different OFDM symbols, i.e., TDMed 8 ports 8 ports on multiple OFDM symbols according to TD OCC - Decision factors include evaluation performance, SRS overhead, per-symbol per-port transmission power, etc. |
| vivo | Support option 3. We think it’s up to how to design the codebook. For full-coherent codebook, we support 8 port one SRS resource with 8 ports in one OFDM symbol For non-coherent and partial-coherent codebook, we support to aggregate 8 ports from two SRS resources from two different SRS resource sets. We suggest waiting the process of AI 9.1.4.2 |
| Ericsson | We think it is not clear what is being discussing here since it has not yet been agreed in AI 9.1.4.2 how many SRS resource sets, SRS resources, and ports per SRS resource that will be supported for an 8 Tx UE. We would also like to clarify our position: We do not think that 8 ports should be supported in only one OFDM symbol. Note that a legacy SRS resource can be configured with a number of OFDM symbols ranging from 1 to 14 by using the following schemes: repetition, frequency hopping, and/or partial sounding. We see no good reason for not supporting these schemes also for an 8-port SRS resource. Hence, we are not a proponent of Option 1. If an 8-port SRS resource in an SRS resource set with usage ‘codebook’ is supported, to ensure a consistent design for codebook and antenna switching, the 8 ports can be transmitted in at least one OFDM symbol. In short, we suggest postponing the discussion regarding SRS for usage ‘codebook’ until an agreement has been found in AI 9.1.4.2. |
| CATT | Support Option 3. We support 8 SRS ports in multiple OFDM symbols where different OFDM symbols with different ports, since it is benefit for per-port power, which is important for UL coverage. |

#### FL update

@vivo @Ericsson: Thanks for clarifying your positions and suggest wait for 9.1.4.2. I updated the proponent list accordingly. In my personal opinion, the number of OFDM symbols and the number of SRS resource(s) / resource set(s) can be related but do not have to be tied. We can see if we can make any progress here, and we can always take into account the outcomes of 9.1.4.2.

The FL suggests the following proposal, in which Option 1 has 7 proponents, and Option 3 has 19 proponents:

Proposal 3.1.1: For 8-port SRS resource(s) or SRS resource set(s) with usage ‘codebook’, down select one from the following options:

Option 1: Only one OFDM symbol

Option 3: One or multiple OFDM symbols

| Company | View |
| --- | --- |
| QC | The formulation of the options seems not very clear. I guess the intention is to say mapping the 8 SRS ports to “Only one OFDM symbol” or “One or multiple OFDM symbols”. Also, we have a question for clarification: does option 1 exclude the repetitions of 8 SRS ports across multiple OFDM symbols? |
| ZTE | Same view as QC |
| Sharp | We prefer Option 1 which has less spec impacts than Option 3. Additionally, “or” in the main bullet is unclear for us. We suggest the following proposal: Proposal 3.1.1: For 8-port SRS resource(s) in SRS resource set(s) with usage ‘codebook’, down select one from the following options: Option 1: Only one OFDM symbol Option 3: One or multiple OFDM symbols |
| OPPO | Agree with QC. The 8 ports are multiplexed in one symbol or multiple symbols. |
| Xiaomi | Same view as QC , option.1 needs further clarification. |
| Intel2 | We think the discussion including both Option1 and Option 3 is for SRS without repetition. And we should focus on the 8-port operation over SRS resource instead of SRS resource set. Suggest the following change: Proposal 3.1.1: For 8-port SRS resource(s) or SRS resource set(s) with usage ‘codebook’, which is not configured with repetition, down select one from the following options: Option 1: Only one OFDM symbol Option 3: One or multiple OFDM symbols |
| Lenovo | We understand that 8 ports may be sounded by multiple SRS resources, e.g., by two 4-port SRS resource, while the main bullet seems preclude this type of configuration. We suggest the following update: Proposal 3.1.1: For 8-port SRS resource(s) or SRS resource set(s) with usage ‘codebook’ for 8Tx PUSCH, down select one from the following options: Option 1: Only one OFDM symbol Option 3: One or multiple OFDM symbols |
| LGE | At least repetition of 8 ports in one OFDM symbol should be supported. |
| DOCOMO | Intel2’s clarification looks good. At least we can focus on no-repetition case. Here we support option 1 for Intel2’s provided update. Then, of course, SRS with 8 ports can be repeated. |
| CATT | Prefer to clarify that the proposal is at least for non-repetition case. Similar view as Lenovo that 8 SRS ports in multiple SRS resources should not be precluded. |

#### Round 2

Tuesday GTW proposal:

Proposal 3.1.1-B:

For SRS with usage ‘codebook’ for 8Tx PUSCH and not configured with repetition, down select one from the following options:

Option 1: Mapping 8 SRS ports to only one OFDM symbol

Option 3: Mapping 8 SRS ports to one or multiple OFDM symbols

Thank you all for the discussions. Please note that the proposal does not preclude anything related to the number of SRS resource(s) or resource set(s), as 2 resources may be mapped to 1 OFDM symbol or multiple OFDM symbols, and 1 resource may be mapped to 1 OFDM symbol or multiple OFDM symbols. We can further discuss based on the outcome of the GTW session.

Proposal 3.1.1-B:

For SRS with usage ‘codebook’ for 8Tx PUSCH and not configured with repetition, down select one from the following options:

Option 1: Mapping 8 SRS ports to only one OFDM symbol

Option 3: Mapping 8 SRS ports to one or multiple OFDM symbols

| Company | View |
| --- | --- |
| Apple | Suggest to clarify For a single SRS resource with usage ‘codebook’ for 8Tx PUSCH and not configured with repetition |
| Samsung | We are fine with Proposal 3.1.1-B since whether 8 ports in one or multiple SRS resources for ‘codebook’ is still FFS. |
| ZTE | Support Option 3 considering power issue and co-scheduling flexibility |
| DOCOMO | Agree with Apple’s clarification. |
| QC | We are fine with the proposal 3.1.1-B. |
| CATT | We are fine with the proposal 3.1.1-B. |
| LGE | Apple’s clarification is valid. 8 port in a single resource could be the baseline. So, we suggest to revise as: Proposal 3.1.1-B: For a single SRS resource in SRS resource set with usage ‘codebook’ for 8Tx PUSCH and not configured with repetition, down select one from the following options: Option 1: Mapping 8 SRS ports to only one OFDM symbol Option 3: Mapping 8 SRS ports to one or multiple OFDM symbols |
| Sharp | We are fine with Apple’s proposal. |
| KDDI | We have similar views of Samsung. We are fine with the proposal 3.1.1-B, and we think whether 8-port SRS in one or/and multiple SRS resource is need for further study. |
| Vivo | To make the main bullet more clearly, we can refer to the agreement for antenna switching which is achieved in the last meeting. Agreement For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol. FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols. Therefore, we suggest updating the proposal as follows. Proposal 3.1.1-B: For an 8-port SRS resource in an SRS resource set with usage ‘codebook’ for 8Tx PUSCH and not configured with repetition, down select one from the following options: Option 1: Mapping 8 SRS ports to only one OFDM symbol Option 3: Mapping 8 SRS ports to one or multiple OFDM symbols |
| Lenovo | Support option 3. |
| Intel | The version from vivo looks better. |
| Xiaomi | Fine with FL’s proposal |
| OPPO | We prefer vivo’s version. |
| Nokia/NSB | Fine with either FL’s or vivo update. |
| FL update 2 | @Apple @vivo and others: The decision on one or more SRS resources for CB has not been made yet and is left for 9.1.4.2, so I suggest we do not make the decision here. Other than that, there seems to be no concern on this proposal so far. |
| LGE | vivo’s version is also looks fine for us. |
| Ericsson | It is still not clear if are we discussing here about one or multiple SRS resource set(s), and/or one or multiple SRS resource(s)? The proposal will have different implication depending on what will be agreed on CB-based operation for 8 Tx UE. Therefore, we prefer to postpone this discussion until an agreement has been reached in AI 9.1.4.2 on how many SRS resource sets, SRS resources, and ports per SRS resource that will be supported for this usage. This way, it will be clearer what this proposal is targeting. Note also that for an 8-port SRS resource, there are schemes other than repetition (frequency hopping and partial sounding) for mapping a multi-port SRS resource to multiple OFDM symbols Is it correct that what is meant here is if TDM /TD-OCC should be supported for an 8-port SRS resource in an SRS resource set with usage ‘codebook’? |

### Whether to support 8 ports in multiple OFDM symbols for 8 Tx SRS for AS

Please provide inputs to the following discussion point:

For an 8-port SRS resource in an SRS resource set with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), whether to support the 8 ports in multiple OFDM symbols in a non-repetition way

Multiple OFDM symbols are supported by CATT, CEWiT, CMCC, Futurewei, InterDigital, Lenovo, OPPO, Qualcomm, Samsung, Spreadtrum, ZTE, Xiaomi, Huawei, HiSilicon

Pros and cons can be analyzed, e.g., SRS overhead, per-symbol per-port transmission power, etc.

| Company | View |
| --- | --- |
| Apple | Do not see strong need |
| Google | Do not support |
| OPPO | Consistent design for codebook and antenna switching is preferred since the SRS resource for CB can be reused for antenna switching. |
| Huawei, HiSilicon | Support. |
| MediaTek | Not Support |
| Xiaomi | Support the proposal |
| ZTE | Support |
| Spreadtrum | Support |
| Lenovo | Support. |
| CMCC | Support consistent design for codebook and antenna switching as OPPO mentioned. Pros of multiple OFDM symbols: it is beneficial for cell edge UE or power-limited UE, which can sound the 8 ports in multiple OFDM symbols, with guaranteed sounding quality. |
| Sharp | Not support |
| Samsung | Support |
| Intel | Not support. 8-port SRS with single symbol is sufficient for antenna switching. |
| LGE | Not support. It is unclear whether the proposed scheme is really beneficial. Regarding per-port Tx power, 8 ports in one symbol can be repeated in multiple symbols for fair comparison. |
| Nokia/NSB | Do not support. |
| QC | We don’t have very strong view here. But we slightly prefer allow mapping a 8-ports SRS resource for usage “antenna switching” to >1 OFDM symbols in TDM manner for flexibility and better coverage. Yes, repetition can achieve the same coverage. But repetition would double the number of CS/comb resources to achieve the same coverage, unless TD-OCC is introduced. However, we don’t support introduce TD-OCC across >1 OFDM symbols, because the orthogonality of TD-OCC breaks with freq hopping, UL transmission prioritization, and high Doppler. |
| Vivo | Don’t support. |
| Ericsson | Do not support, SRS power for coverage-limited Ues can be increased over multiple OFDM symbols with legacy schemes including repetition, frequency hopping, and/or partial sounding. We don’t see a strong need for specifying a fourth (and a fifth) scheme for achieving the same power per port. |
| CATT | Support. |

#### FL update

Note that 8-port SRS for AS on one OFDM symbol has already been agreed and only multiple OFDM symbols needs to be decided. There are 14 proponents for multiple symbols but there are also a number of opponents. The FL suggests to consider if the following proposal for multiple OFDM symbols is agreeable by the group.

Proposal 3.1.2: For 8-port SRS resource(s) or SRS resource set(s) with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), support the 8 ports mapped onto multiple OFDM symbols with different mappings on different OFDM symbols.

| Company | View |
| --- | --- |
| QC | We are not sure “with different mappings” means in the FL proposal. Suggest removing this phrase to avoid confusion or adding details in a sub-bullet to clarify. |
| ZTE | The different mappings on different OFDM symbols is not clear. Does it mean different port groups mapping on different OFDM symbols. Then it means TDM. We can not support unless the restriction of different mappings on different OFDM symbols are deleted. |
| Sharp | We think Proposal 3.1.2 is unnecessary due to increasing spec impact. Furthermore, we’d like to clarify that “with different mapping” means TDM, not TD-OCC. |
| OPPO | We are fine to delete different mappings. |
| Xiaomi | We support TDM multiplexing of 8 ports on multiple symbols |
| Lenovo | “different mappings on different OFDM symbols” is not clear to us. If it means TDMed ports multiplexing scheme, all the 8 ports may be sounded by multiple SRS resources, e.g., by two 4-port SRS resource. We suggest the following update: Proposal 3.1.2: For 8-port SRS resource(s) or SRS resource set(s) with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), support the 8 ports mapped onto multiple OFDM symbols with different mappings on different OFDM symbols. |
| LGE | Not support. As we commented before, different ports across different OFDM symbols for 8T8R don’t have clear benefit. |
| Ericsson | It is not clear what is meant by “different mappings on different OFDM symbols”. Indeed, legacy frequency hopping scheme matches this descriptions as different frequency-domain mappings are used on the different OFDM symbols. Clearly, it should be possible for an 8-port SRS resource to span one or multiple OFDM symbols (configured by legacy higher-layer parameter ‘resourceMapping’). Then the question is whether different time-domain mappings (i.e., TDM) should be supported for an 8-port SRS resource. SRS power for coverage-limited Ues can be increased over multiple OFDM symbols with legacy schemes including repetition, frequency hopping, and/or partial sounding. We do not see a need for supporting additional schemes for increasing SRS coverage. Proponents of SRS TDM should clarify what is the advantage over, e.g., SRS frequency hopping. Besides, it is likely that an 8T8R UE will indicate support for both 4T8R and 8T8R antenna switching in which case two SRS resources that are TDM:ed can be configured. Hence, it seems that TDM between SRS ports within an SRS resource results in duplicate configurations. |
| CATT | We also think “different mappings on different OFDM symbols” is not clear. One alternative is changing it to “different OFDM symbols are mapped with different SRS ports”. |

#### Round 2

Tuesday GTW proposal:

Proposal 3.1.2: For 8-port SRS resource(s) or SRS resource set(s) with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), support the 8 ports mapped onto multiple OFDM symbols with different mappings on different OFDM symbols.

Thank you all for the discussions. It seems the main confusion is related to “different mappings”. CATT’s suggestion is good but it does not allow TD OCC (we cannot preclude it at this stage). The proposal is updated and companies can further discuss.

Proposal 3.1.2-A: For an 8-port SRS resource in a SRS resource set(s) with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), support the 8 ports mapped onto multiple OFDM symbols, where different SRS ports are mapped onto different OFDM symbols or the SRS ports are mapped onto the multiple OFDM symbols according to TD OCC configuration.

| Company | View |
| --- | --- |
| Apple | Do not support. Also we should make the design unified between codebook and antenna switching usage. |
| Samsung | Support the proposal in principle, and prefer 8 ports mapping >1 OFDM symbols in TDM manner (for different ports) for better flexibility. We can understand that coverage would be similar although repetition requires additional combining from gNB side. If 8 ports are allocated in one OFDM symbol, how to allocate Comb/Cyclic shift is another issue, but anyway, it may need more resources rather than legacy 2 or 4 ports SRS from perspective of both comb and cyclic shift. Then, if this is repeated in multiple OFDM symbols, it may occupy lots of resources, then capacity within a single OFDM symbol may be degraded. However, when 8 ports are mapped in multiple OFDM symbols in TDM manner (for different ports), we can achieve similar capacity to multiplex other Ues. As QC mentioned in previous round, we don’t support introducing TD-OCC based approach as the orthogonality of TD-OCC would be affected by many aspects. |
| ZTE | Support. @Samsung, compared with TDM, TD-OCC can improve SINR and needs no time gap to switch between different SRS port groups. For the partial dropping issue, it is same as legacy method. We don’t see any new orthogonal issue brought by TD-OCC. |
| DOCOMO | Do not support. We share apple’s point. |
| QC | We don’t support the proposal. The proposal seems saying support both TDM based mapping and TD-OCC based mapping. We don’t support two schemes for same functionality. We only support TDM based mapping. |
| CATT | Fine with the proposal. |
| LGE | Not support as commented before. |
| Sharp | Not support. 8-port on multi OFDM symbols has high spec impacts such as dropping of at least one of SRS symbols, mapping SRS ports to symbols, and revision of legacy frequency hopping/sequence hopping for TD-OCC. |
| Vivo | Don’t support. In the last meeting, we have agreed that 8-ports are mapped on one OFDM symbol for antenna switching, we think it’s enough. If companies wants to improve the coverage performance of SRS, repetition can be configured. |
| Lenovo | Support. We prefer to have unified design for SRS for codebook and antennaSwitching. |
| Intel | Not support. Same view as Apple. |
| Xiaomi | Same view as Samsung. We don’t support TD-OCC which has been discussed in Rel-17 SRS enh., but we are fine to make it FFS. |
| OPPO | We think the same solution should be applied to SRS resource for codebook and antenna switching. We also support QC that we don’t need to support two schemes. |
| Nokia/NSB | Antenna Switching with distributed symbols are equivalent to 4T8R. What is the difference. When NW configure such transmission for the UE supporting 8T8R. We still don’t understand the usecase. |

#### FL update 2

Thank you all for the discussions.

@QC: Sorry I did not make it clear. I meant to say TDM and/or TD OCC may be supported for 8 Tx on multiple symbols, but which one or both will be supported is still to be discussed.

Proponents please also answer Nokia/NSB’s question.

Proposal 3.1.2-B: For an 8-port SRS resource in a SRS resource set(s) with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), support the 8 ports mapped onto multiple OFDM symbols, where different SRS ports are mapped onto different OFDM symbols (i.e., TDM) or the SRS ports are mapped onto the multiple OFDM symbols according to TD OCC configuration.

FFS: Support one or both of TDM and TD OCC

| Company | View |
| --- | --- |
| QC | @FL, thanks FL for updating the proposal. But the formulation is still not clear. Can we formulate the proposal following the principle of “down selection to one scheme…”, as I think no one want to support both. A suggested reformulation is listed below. QC modified Proposal 3.1.2-B: For an 8-port SRS resource in a SRS resource set(s) with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), support the 8 ports mapped onto multiple OFDM symbols, with a mapping scheme down selected from the following two options. - Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM) - Option 2: The SRS ports are mapped onto the multiple OFDM symbols according to TD OCC configuration. , where different SRS ports are mapped onto different OFDM symbols (i.e., TDM) or the SRS ports are mapped onto the multiple OFDM symbols according to TD OCC configuration. FFS: Support one or both of TDM and TD OCC |
| Samsung | Support QC modified Proposal 3.1.2-B and support Option 1 only. |
| CATT | Fine with QC’s modified Proposal 3.1.2-B and support Option 1. @Nokia/NSB We don’t think 8T8R with distributed symbols and 4T8R is equivalent. For 4T8R, gap is needed between the two 4-port SRS resources. For 8T8R, no gap is needed between the symbols. |
| ZTE | Fine with FL’s proposal @CATT, for 8T8R with distributed symbols, the gap also needs as 4T8R for antenna switching, because the different port groups needs to change the power level which is required by Figure 6.3.3.2-1 in 38.101-1-h50. Figure 6.3.3.2-1: General on/off time gap for NR UL transmission in FR1 |
| LGE | We still think that already agreed 8 ports in one symbol is enough. There is no clear benefit compared to repetition of 8 ports in one symbol. In addition, parallel discussion is there in SRS for ‘codebook’, so we should wait for it if we chase unified solution across different usages. |
| Ericson | Do not support. SRS power for coverage-limited UEs can be increased over multiple OFDM symbols with legacy schemes including repetition, frequency hopping, and/or partial sounding. We do not see a need for supporting additional schemes for increasing SRS coverage. Proponents of SRS TDM and/or TD-OCC should clarify what is the advantage over, e.g., SRS frequency hopping. We therefore propose to modify the proposal according to the following: Proposal 3.1.2-B: For an 8-port SRS resource in an SRS resource set(s) with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), support one of the following options: - Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM) - Option 2: The SRS ports are mapped onto the multiple OFDM symbols according to TD OCC configuration. - Option 3: The SRS ports are mapped onto multiple OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof) We support Option 3. Note that it has already been agreed in the previous meeting that 8 ports can be supported on, at least, single OFDM symbol. In our view, it is more important to decide at this stage how to map these ports onto cyclic shifts and comb offsets. |
| vivo | Don’t support at this stage, and share the similar view with LG and Ericson. We should make a unified design between the usages of ‘codebook’ and ‘antenna switching’. Thus, we should wait for the process of AI 9.1.4.2. It’s better to continue to discuss the SRS design for ‘antenna switching’, after we determine the SRS design for codebook. In other words, if we agree the SRS design for ‘antenna switching’ first, then it may affect the SRS design for ‘codebook’ and the conclusion in AI 9.1.4.2. |
| Nokia/NSB | We don’t support. |

### High-level design to support 8 ports in multiple OFDM symbols for 8 Tx SRS

Please provide inputs to the following discussion point:

When 8-port SRS resource(s) or SRS resource set(s) in multiple OFDM symbols is/are to be supported in a non-repetition way, high-level designs on how to support the 8 ports in the multiple OFDM symbols

E.g., different ports are mapped to different OFDM symbols, i.e., TDMed 8 ports

Supported by CATT, CEWiT, CMCC, Futurewei, Huawei, HiSilicon, InterDigital, KDDI, Lenovo, OPPO, Qualcomm, Samsung, Spreadtrum, Xiaomi, vivo

E.g., the 8 ports are mapped to different OFDM symbols according to TD OCC, i.e., 8 ports with TD-OCC

Supported by CMCC, Futurewei, Lenovo, Spreadtrum, ZTE

If supported, RAN1 needs to further discuss TD OCC for SRS with less than 8 ports

The discussions may be different for different usages and may depend on the outcome of subsections 3.1.1 and 3.1.2.

| Company | View |
| --- | --- |
| Apple | It is TD-OCC There can be different TD-OCC, identity, Hadamard, DFT, etc. |
| Google | Do not support |
| OPPO | We think TDM would outperform TD-OCC for power limited UEs. |
| Xiaomi | Support in general, more discussion are needed. |
| ZTE | Support TD-OCC. @OPPO, TD-OCC has same power boosting effect as TDM. In addition, compared with TDM, TD-OCC can further improve SINR and does not need time gap for switching between different SRS port groups. |
| Spreadtrum | We think it can be jointly discussed with Section 2.5. |
| Lenovo | We prefer TDM scheme compared with TD-OCC. |
| CMCC | We are fine with either one of them. |
| Sharp | Not support |
| Samsung | Support TDM. |
| Intel | Not support. There is no agreement to support 8-port with multiple symbols |
| LGE | Prefer to discuss after the discussion on section 3.1.1. |
| KDDI | We prefer TDM. Further study is need for TD-OCC. |
| Nokia/NSB | This depends on the proposal 3.1.1 and 3.1.2. We don’t support without clear justification. Between two schemes, we slightly prefer TD-OCC, and need more study for the main motivation of multi-symbol transmission. |
| QC | Like we commented above, we don’t support the TD-OCC approach on this topic. |
| vivo | Prefer two 4-port SRS resources based on TDM manner |
| Ericsson | As discussed above, we are not supportive of either scheme as there are already several standardized coverage-enhancement schemes for an SRS resource (that are supported irrespectively of the number of ports). |
| CATT | Support TDM. Further study is need for TD-OCC. |
| DOCOMO | We think at first we need to conclude whether to support multiple OFDM symbols different ports. |

### Detailed designs to support 8 ports in one or multiple OFDM symbols for 8 Tx SRS

The following discussion point will be discussed based on the outcomes of previous subsections:

When 8-port SRS resource(s) or SRS resource set(s) in one or multiple OFDM symbols is/are to be supported, detailed designs on how to support the 8 ports in the multiple OFDM symbols

E.g., comb and comb offset allocation, cyclic shift allocation, etc.

## Other designs / design parameters for 8 Tx SRS

Some other designs / design parameters for 8 Tx SRS were also discussed:

Precoded SRS for AS

Full power mode

Antenna switching downgrade

PAPR issue

- These can be further discussed. Companies’ views on the above are collected as follows. Any other views, issues, potential enhancements, and clarifications can also be provided.

| Company | View |
| --- | --- |

# Conclusions

## For Tuesday GTW

Proposal 2.2: Support randomized code-domain resource mapping for SRS transmission.

Introduce at least cyclic shift hopping / randomization to SRS resource

FFS Cyclic shift allocation pattern(s) and parameter(s) used to determine the cyclic shift for each SRS port of a SRS resource in a SRS transmission occasion.

FFS other schemes.

Proposal 2.1: Support at least one of the following options for comb offset hopping for SRS.

- Option 1: Extend comb offset hopping as a function of symbol index defined for SRS-PosResource to SRS-Resource.

- Option 2: The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) with a certain UE-specific initialization.

- FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Proposal 2.8-A: Support enhanced signaling for flexible SRS transmission, and down select one or more enhanced signaling from the following list:

For dynamic update of SRS parameters, study one or more of the following parameters

Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change)

Power control parameter

RPFS parameters

Number of antennas in antenna switching

Code-domain parameters (cyclic shift / SRS sequence)

Time-domain parameters

Activation by DCI for SP SRS

Enhanced cancellation indication in DCI format 2_4

Proposal 3.1.1-A: For SRS resource(s) or SRS resource set(s) with usage ‘codebook’ for 8Tx PUSCH and not configured with repetition, down select one from the following options:

Option 1: Only one OFDM symbol

Option 3: One or multiple OFDM symbols

Proposal 3.1.2: For 8-port SRS resource(s) or SRS resource set(s) with usage ‘antennaSwitching’ (i.e., for 8T8R antenna switching), support the 8 ports mapped onto multiple OFDM symbols with different mappings on different OFDM symbols.

## Tuesday GTW agreement:

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

## For Friday GTW:

Proposal 2.1-1: For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following:

- The hopping pattern (e.g., the pseudo-random sequence, time granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if both CS hopping and comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

Proposal 2.4: For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for a SRS resource set:

Option 1: Power control of the SRS resource set is based on one UL power control parameter set and more than one DL pathloss RS and/or more than one alpha. Each transmission occasion of the SRS resource is towards multiple TRPs.

Option 2: Power control of the SRS resource set is based on more than one UL power control parameter set each associated with a DL pathloss RS. Different transmission occasions of the SRS resource can be towards different TRPs.

Proposal 2.5: For SRS TD OCC for SRS enhancements for TDD CJT, study:

Comparison against SRS on 1 OFDM symbol

Comparison against SRS repeated on multiple OFDM symbols

Decision factors include evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

Proposal 2.8-C: Study enhanced signaling for flexible SRS transmission from the following list:

Adopt one or more of the following parameters for dynamic update of SRS parameters

Frequency-domain parameter (e.g., BW change, comb change and/or hopping location change)

Power control parameter

RPFS parameters

Number of antennas in antenna switching

Code-domain parameters (cyclic shift / SRS sequence)

Time-domain parameters

Activation by DCI for SP SRS

Enhanced cancellation indication in DCI format 2_4, e.g.,

Association with SRS resource(s)

Periodic/persistent cancellation

Proposal for conclusion: The discussion of resource mapping for SRS transmission based on network-provided parameters or system parameters is merged into the discussions of other SRS enhancements for TDD CJT.

# References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), RAN#94-e.

R1-2208376, SRS enhancements for TDD CJT and 8TX operation, FUTUREWEI.

R1-2208443, SRS enhancement for TDD CJT and UL 8Tx operation in Rel-18, Huawei, HiSilicon.

R1-2208497, Discussion on SRS Enhancements, InterDigital, Inc.

R1-2208506, SRS enhancement targeting TDD CJT and 8 TX operation, ZTE.

R1-2208543, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, Spreadtrum Communications.

R1-2208630, Discussion on SRS enhancement, vivo.

R1-2208744, Discussion of SRS enhancement, Lenovo.

R1-2208796, SRS enhancement targeting TDD CJT and 8 TX operation, OPPO.

R1-2208874, On SRS Enhancement, Google.

R1-2208895, SRS enhancement targeting TDD CJT and 8 TX operation, LG Electronics.

R1-2208949, Discussion on SRS enhancement, CATT.

R1-2209043, Discussion on SRS enhancement in Rel-18, Intel Corporation.

R1-2209091, Considerations on SRS enhancement targeting TDD CJT and 8 TX operation, Sony.

R1-2209142, Discussion on SRS enhancement, NEC.

R1-2209260, Discussion on SRS enhancements, xiaomi.

R1-2209324, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, CMCC.

R1-2209383, SRS enhancement targeting TDD CJT and 8 TX operation, Sharp.

R1-2209434, Views on enhanced SRS configuration for 8TX and TDD CJT, KDDI Corporation.

R1-2209572, Views on Rel-18 MIMO SRS enhancement, Apple.

R1-2209623, On SRS enhancements targeting TDD CJT and 8 TX operation, Ericsson.

R1-2209718, Views on SRS enhancements, Samsung.

R1-2209892, Discussion on SRS enhancement, NTT DOCOMO, INC.

R1-2209971, SRS enhancement for TDD CJT and 8 Tx operation, Qualcomm Incorporated.

R1-2210065, SRS enhancement for TDD CJT and 8Tx operation, Nokia, Nokia Shanghai Bell.

R1-2210106, Discussion on SRS Enhancements for 8Tx Operation, CEWiT.

# Appendix

## Appendix 1: Agreements from RAN1#109-e

Agreement

For SRS EVM, adopt combined relevant parts from Rel-17 SRS EVM and Rel-18 FDD CJT EVM as starting point

- Details are provided in Appendix 3 of R1-2205330 for system-level simulations

- Details are provided in Appendix 4 of R1-2205330 for link-level simulations.

Agreement

For 8 Tx SRS, a starting point of UE antenna configurations can be:

- (M, N, P; Mg,Ng; Mp, Np) = (2,2,2; 1,1; 2,2), (dH, dV) = (0.5, 0.5)λ, or

- (M, N, P; Mg,Ng; Mp, Np) = (1,4,2; 1,1; 1,4), (dH, dV) = (0.5, 0.5)λ.

- FFS other 8 Tx UE antenna configuration and alignment with outcomes from other agenda items.

Agreement

For SRS EVM, consider additional EVM as follows

- Realistic channel estimation based on sequence generation for SRS modelling, at least for TDD CJT SRS LLS and 8 Tx SRS LLS as baseline

- Evaluation metrics for 8 Tx SRS LLS can be MSE , BLER or throughput

- TDL-C for TDD CJT SRS LLS can be included as optional.

Agreement

Consider the scenario where there exists SRSs sent by a UE and utilized by multiple TRPs for channel estimation, and the pathlosses between the UE and the TRPs differ by at least x dB in Rel-18 SRS study

- x can be {3,6,10}, and other values can be used.

Agreement

Study the following for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS interference randomization and/or capacity enhancement

- Randomized frequency-domain resource mapping for SRS transmission

  - E.g., further enhancements to frequency hopping, comb hopping

- Randomized code-domain resource mapping for SRS transmission

  - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence

- Randomized transmission of SRS

    - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

- Per-TRP power control and/or power control of one SRS towards to multiple TRPs

- SRS TD OCC

- Increasing the maximum number of cyclic shifts

  - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts

- Precoded SRS for DL CSI acquisition

- Enhanced signaling for flexible SRS transmission

  - E.g., dynamic update of SRS parameters

- Partial frequency sounding extensions

  - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS

- Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

  - E.g., configuration of v (sequence index within a group) per SRS resource

  - E.g., configuration of cyclic shift per SRS port per SRS resource.

- Resource mapping for SRS transmission based on network-provided parameters or system parameters

  - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index)

Note: PAPR performance and maintaining DFT waveform property should be considered when deciding the enhancement for Rel-18.

Agreement

Study the potential enhancements for SRS of 8T8R with usage antennaSwitching.

Agreement

Study the potential enhancements for SRS for 8 Tx operation

- SRS resource(s) with 8 ports are configured for codebook-based PUSCH

- Up to 8 single-port SRS resources are configured for non-codebook-based PUSCH

Agreement

For SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices, study aspects include, for SRS for CB/NCB/AS,

- Design parameters, including the maximum number of SRS resource sets, number of SRS resource sets, number of SRS resources, number of ports per resource, number of OFDM symbols, the allowed configurations for comb / comb shifts / cyclic shifts, number of simultaneous ports / resources / resource sets per OFDM symbol

- For the next decision point, study

  - Whether to support 8 ports in one or multiple resources

  - Whether to support 8 ports in one or multiple OFDM symbols

  - The maximum number of SRS resource sets.

- Note: For SRS for NCB, number of ports per SRS resource is still 1 (same as R15)

| Rel-18 SLS Assumptions for TDD CJT SRS |  |  |  |
| --- | --- | --- | --- |
| Parameter |  |  | Value |
| Duplex, Waveform |  |  | TDD, OFDM |
| Multiple access |  |  | OFDMA |
| Scenario |  |  | Outdoor OptA Companies can simulate from the following 2 layouts. 1) Outdoor (typical 57-sector, or 21-sector, SLS): OptionA: 1 TRP per sector, 3 sectors per site. N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). The N_TRP TRPs can be selected either only from the same site (intra-site - limited to 3 TRPs), or also from other sites (inter-site) - company should describe what is assumed OptionB: N_TRP co-located (at BS) panels per sector - companies describe how the panels are (azimuthally) oriented - Dense Urban (macro only) 200m ISD or Urban Macro 500m ISD 2) Indoor Hotspot: model in TS 38.802 - N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). |
| Frequency Range |  |  | FR1 only, 3.5GHz |
| Inter-BS (site) distance |  |  | Outdoor: 200m or 500m Indoor Hotspot: per TS 38.802 |
| Channel generation model |  |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| Antenna setup and port layouts at gNB |  |  | - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32,64} |
| Antenna setup and port layouts at UE |  |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 |
| BS Tx power |  |  | Dense Urban or Urban Macro: - Per TRP: 44 dBm for 20MHz, 47dBm for 40MHz, 51dBm for 100MHz Indoor: per TRP 24dBm |
| BS antenna height |  |  | Depending on scenarios (cf. table A.2.1-1 of TS 38.802): DU (25m), UMa (25m), Indoor Hotspot (3m) |
| UE antenna height & gain |  |  | Follow TR36.873 |
| UE receiver noise figure |  |  | 9dB |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |  |
|  | SCS | 30kHz |  |
| Number of RBs |  | 52RB for 20MHz, 104RB for 40MHz, 272RB for 100MHz |  |
| Frame structure |  | DSUDD, or companies to state the used frame structure |  |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |  |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers |  |
| Overhead |  | Companies shall provide the downlink overhead assumption |  |
| Traffic model |  | FTP 1 or FTP 3 with 20%, 50% or 70% traffic load |  |
| UE distribution |  | According to TS 38.802 - DU and UMa: 80% indoor (3km/h), 20% outdoor (30km/h) - Indoor Hotspot: 100% indoor (3km/h) |  |
| UE receiver |  | MMSE-IRC as the baseline receiver |  |
| DL Channel estimation |  | Realistic |  |
| Evaluation Metric |  | DL throughput |  |
| Baseline for performance evaluation |  | R17 SRS design |  |
| SRS modeling for UL channel estimation |  | Companies to state the used SRS periodicity. Companies to state the SRS channel estimation modeling Number of ports = 2 or 4 Tx power = 23 dBm |  |

| Rel-18 LLS Assumptions for TDD CJT SRS |  |
| --- | --- |
| Parameter | Value |
| Scenario | N_TRP (#TRPs): 2, 3, 4 |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 40MHz, 100MHz |
| Channel model | CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| UE velocity | 3km/h |
| Antennas at UE | 1T4R, 2T4R, 4T4R |
| Antennas at gNB | 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Rank and MCS | Rank/MCS can be adaptive or fixed. |
| Evaluation metrics | MSE, BLER or throughput |
| Baseline | R17 SRS design |
| Precoding granularity | Fixed: 2, 4 or wideband for DL, wideband for UL. |
| SRS configurations | Companies to state the used SRS periodicity. Frequency hopping：Companies to state whether SRS frequency hopping is enabled and the hopping pattern if so. |
| DL SNR | Companies to state the used difference between DL SNR and UL SNR |

## Appendix 2: Agreements from RAN1#110

Agreement

For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

- SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

RAN1 should strive to minimize the number of schemes supported in Rel-18

- SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

Target usage includes antenna switching, codebook/non-codebook based SRS

Agreement

For 8 Tx SRS, at least support

- 8 ports in 1 SRS resource for ‘antennaSwitching’;

  - FFS 8 ports in one or multiple SRS resources for ‘codebook’

Above does not imply support for 8 ports in one or multiple OFDM symbols

Agreement

For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR)

Agreement

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Agreement

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.