3GPP TSG RAN WG1 #110bis-e																           R1-2210401

e-Meeting, October 10th – 19th, 2022

Source:	Moderator (Intel Corporation)

Title:	Moderator Summary for Rel. 17 NR FeMIMO –

Maintenance on HST-SFN (Round 0)

Agenda item:	8.1

Document for:	Discussion and Decision

# Introduction

A moderator summary of maintenance issues related to Rel-17 FeMIMO HST-SFN based on contributions submitted to RAN1#110bis-e is provided below. A total of 4 issues have been identified. Based on the first Round of discussions, Issues 2, 3 and 4 are recommended for further discussion in RAN1#110bis-e.

# Maintenance Issues

Issue 1: CORESET#0 associated with SS#0 in Type 0/0A/2 CSS

Two companies LGE [1], Lenovo [2] have provided draft CRs to capture the following agreement from RAN1-110 in TS 38.213.

Agreement UE does not expect CORESET#0 to be activated with two TCI states when it is associated with SS#0 for Type 0/0A/2 CSS

Table 1: Summary of Issue 1

| Issue (summary of CR proposal) | FL assessment | Company inputs (if any) |
| --- | --- | --- |
| In RAN1#110, potential UE behavior for monitoring Type0/0A/2-PDCCH CSS when two TCI states are activated for CORESET 0 was discussed and it was agreed that UE does not expect CORESET#0 to be activated with two TCI states when it is associated with SS#0 for Type 0/0A/2 CSS. Summary of change: The agreement should be captured in Section 10.1 of TS38.213 | Do not Discuss in RAN1#110bis-e | - Discuss (10): Samsung, Nokia/NSB, Spreadtrum, LG, Lenovo, OPPO, ZTE. QC, Apple - Not Discuss (5): vivo, Ericsson, Google, DOCOMO, CATT |

| Company Name | Company inputs (if any) |
| --- | --- |
| Mod | This is a valid issue, and the CR is needed to capture the agreement from RAN1-110. Both CRs provide almost identical corrections and either one can be selected. Therefore, initial FL assessment is to treat and conclude this issue in RAN1-110bis-e. Companies can provide further comments. |
| Samsung | We are fine to discuss and support to capture the above agreement. |
| Nokia/NSB | We are fine to discuss. |
| Spreadtrum | We are fine to discuss. |
| vivo | In the last meeting, we have sent a LS R1-2208203 to RAN2 for updating the corresponding specification for this agreement if needed. Now this issue is also discussed in RAN2’s CR of TS 38.321. In principle, we are fine to capture this agreement in either RAN1’s spec or RAN2’s spec. However, at this stage, we think it’s better to wait RAN’2 progress. If RAN2 captures it in TS 38.321, then RAN1 doesn’t need to capture it in TS 38.213. |
| LG | Discussion is needed. |
| Lenovo | We are fine to capture this agreement in TS 38.213 or TS 38.321. We prefer to capture this agreement in TS 38.213 since this agreement is related with UE monitoring behavior and it is agreed based on hot discussion in RAN1 meetings. It will be helpful for TS38.213 reader to understand UE monitoring behavior Type0/0A/2-PDCCH CSS if this agreement is captured in TS38.213. Also, there is many similar description for configuration as “UE does not expect …” in TS 38.213. |
| OPPO | We are fine to discuss it and capture it in 38.213. |
| ZTE | Support to capture the above agreement in 38.213. |
| Ericsson | We share same view as vivo. It is not clear to us if we need to capture this in 38.213. We’ve sent LS to RAN2 on last meeting, and RAN2 is discussing the LS and eventually will update 38.321 accordingly. After RAN2 updates we can discuss further if we still need the CR in RAN1. |
| Google | We share similar views as vivo and Ericsson. RAN2 also discussed how to capture this in 38.321. We should wait for their result to avoid duplicated discussion. |
| QC | Fine to discuss to reflect RAN1 agreement. |
| Apple | We are fine to discuss |
| NTT Docomo | We agree with vivo, Ericsson. If it will be captured in TS38.321, we don’t need to capture it in TS38.213. |
| CATT | No need to capture it in TS38.213 |
| Mod | Based on comments from companies, we can defer this to the next meeting to check back if any RAN1 agreement is necessary after conclusion of RAN2 discussion. |

Issue 2: Default QCL Assumption

One company, ZTE, has provided a draft CR on default QCL assumptions for prioritizing PDCCH reception when associated CORESET overlaps with SFN-PDSCH [3] regardless of whether one or two TCI states are configured for CORESET. The summary of proposed changes is provided below.

Table 2: Summary of Issue 2

| Issue (summary of CR proposal) | FL assessment | Company inputs (if any) |
| --- | --- | --- |
| Draft CR for TS 38.214 Section 5.1.5 provided in [3]: Summary of change: In the case of SFN based transmission is configured for PDCCH and not configured for PDSCH, the reception of PDCCH should have higher priority, regardless of ‘with single active TCI state’ or not. Then, one editorial typo is corrected. | Discuss in RAN1#110bis-e | - Discuss (14): Samsung, Nokia/NSB, Spreadtrum, vivo, LG, Lenovo, OPPO, ZTE, Ericsson, Google, QC, Apple, CATT - Not Discuss (1): DOCOMO |

| Company Name | Company inputs (if any) |
| --- | --- |
| Mod | A CR on this issue was endorsed in RAN1-110 in R1-2208092. The current proposal from ZTE is based on this CR and proposes to change the wording “CORESET with single active TCI state” to “CORESET”. Initial FL assessment is to discuss this issue in RAN1-110bis-e. Companies can provide further comments to clarify if they think this is an essential correction. |
| Samsung | We are fine to discuss and support in principle since there can be overlapping case between CORESET with two TCI states and PDSCH with default beam. |
| Nokia/NSB | We are fine to discuss. We need to double check why “with single active TCI state” included since no other part with similar text doesn’t have such clarification. |
| Spreadtrum | We are fine to discuss and support it in principle. |
| Vivo | Support |
| LG | We are fine to discuss |
| Lenovo | We support it on account of overlapping case for CORESET with two activate TCI states. |
| OPPO | We are fine to discuss it. |
| ZTE | Support to discuss. |
| Ericsson | OK to discuss. |
| Google | OK to discuss |
| QC | Fine to discuss |
| Apple | We are fine to discuss |
| Docomo | Not essential. We made an agreement in RAN1#110, and “with single active TCI state” was a compromised solution from Qualcomm. To our understanding, the reason of “with single active TCI state” is that in case of two active TCI state on the overlapped CORESET, “prioritize the reception of PDCCH with two active TCI states” is unclear on which condition the overlapped PDSCH with one active TCI state can be received. The proposed CR does not solve this issue. |
| CATT | Fine to discuss |
| Mod | Based on comments from companies, this issue should be further discussed in RAN1#110bis-e. |

Issue 3: SFN Dynamic Switching Terminology in 38.214

A joint draft CR from Ericsson and Qualcomm has been submitted to align terminology of TS 38.214 with UE capability parameters in TS 38.306 [4]. The summary of proposed changes is provided below.

Table 3: Summary of Issue 3

| Issue (summary of CR proposal) | FL assessment | Company inputs (if any) |
| --- | --- | --- |
| Draft CR for TS 38.214 Section 5.1.5 provided in [4]: Summary of change: Change 1: Replace undefined terminology and acronym “SFN PDSCH and non-SFN PDSCH” with correct UE capability parameters from 38.306. Change 2: Replace undefined UE capability “[dynamicSFN]” with correct UE capability parameters from 38.306. Change 3: Add UE capability parameter sfn-DefaultDL-BeamSetup-r17 before “DCI scheduling without TCI field” to clarify the related UE capability. | Editorial – Discuss in RAN1#110bis-e | - Discuss (15): Samsung, Nokia/NSB, Spreadtrum, vivo, LG, Lenovo, OPPO, ZTE, Ericsson, Google, QC, Apple, DOCOMO, CATT - Not Discuss: |

| Company Name | Company inputs (if any) |
| --- | --- |
| Mod | This is a valid issue and should be editorial in nature. Initial FL assessment is to discuss this issue in RAN1#110bis-e |
| Samsung | We are fine to discuss. |
| Nokia/NSB | We are fine with this as editorial. |
| Spreadtrum | We are fine with this as editorial. |
| Vivo | We are fine with this as editorial. |
| LG | We are fine with this as editorial. |
| Lenovo | We are fine with this as editorial. |
| OPPO | Fine with this CR as editorial |
| ZTE | We are fine with this as editorial. |
| Ericsson | We support this CR. |
| Google | Agree with FL’s assessment. |
| QC | Support the CR. |
| Apple | Fine with the CR |
| NTT Docomo | OK. |
| CATT | Fine with the CR |
| Mod | Based on comments, this issue should be discussed as editorial in RAN1#110bis-e |

Issue 4: Default UL beam setup for SFN PDCCH

One company, Ericsson, has submitted a draft CR on default UL beam setup for SFN-PDCCH in [5]. The summary of changes is provided in Table 4.

Table 4: Summary of Issue 4

| Issue (summary of CR proposal) | FL assessment | Company inputs (if any) |
| --- | --- | --- |
| Draft CR for TS 38.214 Sections 6.1 and 6.2.1 provided in [5]: Summary of change: Change 1: Replace undefined UE capability “[DefaultBeamPL-ForPUSCH-SfnPdcch]” with related UE capability parameter from 38.306. Change 2: Replace undefined UE capability “[DefaultBeamPL-ForSRS-SfnPdcch]” with correct UE capability parameter from 38.306. | Editorial – Discuss in RAN1#110bis-e | - Discuss (15): Samsung, Nokia/NSB, Spreadtrum, vivo, LG, Lenovo, OPPO, ZTE, Ericsson, Google, QC, Apple, DOCOMO, CATT - Not Discuss:: |

| Company Name | Company inputs (if any) |
| --- | --- |
| Mod | This is a valid issue and should be editorial in nature. Initial FL assessment is to discuss this issue in RAN1#110bis-e |
| Samsung | We are fine to discuss and can be treated as editorial. |
| Nokia/NSB | We are fine with this as editorial. |
| Spreadtrum | We are fine with this as editorial. |
| Vivo | We are fine with this as editorial. |
| LG | We are fine with this as editorial. |
| Lenovo | We are fine with this as editorial. |
| OPPO | Fine with this CR as editorial |
| ZTE | We are fine with this as editorial. |
| Ericsson | We support this CR. Fine to treat it as editorial CR. |
| Google | OK with it. |
| QC | Fine with the CR. |
| Apple | Fine with CR |
| NTT Docomo | Fine. |
| CATT | Fine with this as editorial. |
| Mod | Based on comments, this issue should be discussed as editorial in RAN1#110bis-e |

# References

|  | R1-2208890 | Draft CR on Type0/0A/2 PDCCH CSS for HST-SFN | LG Electronics |
| --- | --- | --- | --- |
|  | R1-2208755 | Draft CR on not activating two TCI states for CORESET#0 associated with SS#0 for Type 0/0A/2 CSS to TS38.213 | Lenovo |
|  | R1-2208760 | Draft CR on default QCL assumption in HST-SFN in TS 38.214 | ZTE |
|  | R1-2210076 | Draft CR on SFN dynamic switching | Ericsson, Qualcomm |
|  | R1-2210077 | Draft CR on default UL beam setup for SFN PDCCH | Ericsson |