3GPP TSG RAN WG1 #110bis Meeting	        		   R1-2210446

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.202 | CR |  | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on the MBS reception type combinations to TS 38.202 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (CMCC), MediaTek, Huawei, HiSilicon, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In previous RAN1 meeting, the following agreements were achieved, Agreement For RRC_IDLE/INACTIVE UEs, a UE is required to support reception of FDMed MCCH PDSCH and PBCH in PCell. Agreement For RRC_IDLE/INACTIVE UEs, a UE is not required to support reception of FDMed MTCH PDSCH and PBCH in PCell. Agreement: For RRC_CONNECTED UEs, - a UE is not required to support reception of FDMed MCCH/MTCH/multicast PDSCH and SIB PDSCH in Pcell. - a UE is required to support reception of FDMed MCCH PDSCH and PBCH in Pcell. - a UE is not required to support reception of FDMed MTCH PDSCH and PBCH in Pcell. - a UE is not required to support reception of FDMed multicast PDSCH and PBCH in Pcell. Agreement For RRC_IDLE/INACTIVE UEs, a UE is not required to support reception of FDMed MCCH/MTCH PDSCH and SIB PDSCH in PCell.. However, it didn’t been capatured in the latest spec. In Table 6.2-2, for RRC_CONNECTED state and for PCell, the combination of (B and/or (D0 or (m1*D1+m2*D2+((m3*D3+m4*D4) or m5*(D5 or D6)) lacks explanation whether UE is required to support FDMed MCCH/MTCH/Multicast and SIB or FDMed MTCH/multicast with PBCH, which is against the agreements above. |
| Summary of change: |  | Add the folloiwng notes in the table: - Note 13: For a UE supporting MBS multicast or broadcast reception in RRC_CONNECTED state, it is not required to support reception of FDMed MCCH PDSCH/broadcast MTCH PDSCH/multicast MTCH PDSCH and SIB PDSCH in PCell. - Note 14: For a UE supporting MBS multicast or broadcast reception in RRC_CONNECTED state, it is not required to support reception of FDMed broadcast MTCH PDSCH/multicast MTCH PDSCH and PBCH in PCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The misaligned issue between gNB configuration and UE reception capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.213 (R1-2210208), TS38.214 (R1-2210209) |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2	Downlink

*** Unchanged text is omitted ***

Table 6.2-2: Downlink "Reception Type" combinations

| Supported Combinations |  |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCell | PSCell |  | SCell |  |  |  |
| 1. RRC_IDLE |  |  |  |  |  |  |
| 1.1 All UEs |  |  |  |  |  |  |
| A + (B and/or (C1 or Q) and/or D0) + F0 |  |  |  |  | Note 1 |  |
| 1.2 UEs supporting MBS broadcast reception |  |  |  |  |  |  |
| A+D5 |  |  |  |  |  |  |
| 2. RRC_INACTIVE |  |  |  |  |  |  |
| 2.1 All UEs |  |  |  |  |  |  |
| A + (B and/or (C1 or Q) and/or D0) + F0 |  |  |  |  | Note 1 |  |
| 2.2 UEs supporting MBS broadcast reception |  |  |  |  |  |  |
| A+D5 |  |  |  |  |  |  |
| 3. RRC_CONNECTED |  |  |  |  |  |  |
| (A + ((C0 + (B and/or (D0 or (m1*D1+m2*D2+((m3*D3+m4*D4) or m5*(D5 or D6))))) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + L0 + L1 + M + N + P) or D5)) | (A + (D0 or (m1*D1+m2*D2)) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + N + P) |  | m1*D1 + m2*D2 + ((m3*D3+m4*D4) or m5*(D5 or D6)) + E + n*F1 + G + H + J0 + J1 + J2 + K + O + L0 + L1 + M + P |  | Note 2, Note 3, Note 4, Note 5, Note 6, Note 7, Note 8, Note 9, Note 10, Note 11, Note 12, Note13, Note 14 |  |
| Note 1: UE is not required to decode more than two PDSCH simultaneously, and decoding prioritization when more than two are received is up to UE implementation. Note 2: For PCell, UE is not required to decode SI-RNTI PDSCH simultaneously with C-RNTI PDSCH, unless in FR1. Note 3: Supported combinations are subject to UE capabilities for dual connectivity, carrier aggregation, receiving of group TPC commands, pre-emption indication and dynamic SFI monitoring. Note 4: The values of m2 ≥ 0 and n≥ 0 in the supported combinations are subject to the UE capability. Note 5: Support of monitoring PDCCH with SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI are subject to UE capability. Note 6: The values of m1 ≥ 1 in the supported combinations are subject to the UE capability. Note 7: In Active time, a UE is not expected to monitor the DCI format for the PDCCH scrambled by PS-RNTI. Note 8: The PDCCH scrambled by PS-RNTI can only be configured on the PCell and PSCell. Note 9: For a UE supporting MBS multicast reception, the values of 1 ≥ m3 ≥ 0 and m4 ≥ 0 are subject to UE capability and applicable to RRC connected UEs. If m3 = 1, then m1 ≤ 1. Note 10: For a UE supporting MBS multicast or broadcast reception, the UE is not expected to be configured simultaneously with more than one component carrier for multicast or broadcast reception. Note 11: For a UE supporting MBS broadcast reception, the values of 1≥m5 ≥ 0 are subject to UE capability and applicable to RRC connected UEs. If m5=1, then m1≤1. Note 12: For a UE supporting MBS broadcast reception in RRC_CONNECTED state, it is required to support reception of FDMed MCCH PDSCH and PBCH in Pcell. Note 13: For a UE supporting MBS multicast or broadcast reception in RRC_CONNECTED state, it is not required to support reception of FDMed MCCH PDSCH/broadcast MTCH PDSCH/multicast MTCH PDSCH and SIB PDSCH in PCell. Note 14: For a UE supporting MBS multicast or broadcast reception in RRC_CONNECTED state, it is not required to support reception of FDMed broadcast MTCH PDSCH/multicast MTCH PDSCH and PBCH in PCell. |  |  |  |  |  |  |

*** Unchanged text is omitted ***