3GPP TSG-RAN WG1 Meeting #110bis-e	R1-22xxxxx

e-Meeting, 10th – 19th October, 2022

Agenda Item:	8.2

Source:	Moderator (Ericsson)

Title:	FL Summary #2 for AI 8.2 – Enhancements for PUCCH Formats 0/1/4

Document for:	Discussion, Decision

# 1	Introduction

This document summarizes contributions under AI 8.2 related to Enhancements for PUCCH formats 0/1/4 in the Rel-17 work item "Extending current NR operation to 71 GHz."

| Issue# | Issue | References | FL initial assessment |
| --- | --- | --- | --- |
| PUCCH-1 | RRC parameter name alignment | [1] | Handle in editor alignment CR |

The outcome of the preparation phase was that a the issue can be handled as an editor alignment CR for 38.213, hence the open issue is what CR to recommend to the editor. For discussing what CR to recommend to the editor, the vice chair has assigned the following email thread:

[110bis-e-R17-FR2-2-05] Email discussion for maintenance for PUCCH formats 0/1/4 for FR2-2 for issue PUCCH-1 in R1-2210392 – Steve (Ericsson)

- Until October 14

# 2	RRC parameter name alignment for cell-specific PUCCH

A draft CR is provided in [1] to correct an RRC parameter misalignment between 38.213 and 38.331. The draft CR is copied into Appendix A of this summary for convenience. The rationale provided in [1] is as follows:

| Reason for change: | The RAN1 specifies multi-PRB PUCCH for format 0/1/4, the RRC parameter nrofPRBs provided in PUCCH-ConfigCommon in TS 38.331 indicates the number of PRBs for common PUCCH, and the RRC parameter pucch-ResourceCommon indicates an index to a row of Table 9.2.1-1 in TS 38.213. However, in the current spec 38.213, the parameter pucch-ResourceCommon is used to indicate the PRB number of common PUCCH which is the incorrect. |
| --- | --- |
| Summary of change: | Change the parameter used to indicate the number of PRBs of common PUCCH, from “pucch-ResourceCommon” to “nrofPRBs provided in PUCCH-ConfigCommon”. |
| Consequences if not approved: | Incorrect parameter to indicate the number of PRBs of common PUCCH, UE cannot acquire the correct common PUCCH resource. |

The outcome of the preparation phase was that a the issue can be handled as an editor alignment CR for 38.213, hence the open issue is what CR to recommend to the editor. To decide this, please provide an answer on the following questions from the moderator:

## Question 1: For the editor alignment CR, should RAN1 recommend the CR in [1] to the editor of 38.213?

## Question 2:	If not, then what CR do you propose to recommend instead?

| Company | View/Position |
| --- | --- |
| Moderator Recommendation | Q1: Yes |
| Intel | Q1: Agree with FL’s recommendation. |
| vivo | Yes to Q1. |
| Qualcomm | Yes to Q1 |
| OPPO | Q1: Yes |
| Nokia, NSB | Q1: Yes |
| Huawei, HiSilicon | fine with the intention. Not sure whether the field name “pucch-ConfigCommon” or IE name “PUCCH-ConfigCommon” should be used in RAN1 spec. The former according to my memory. |
| LG Electronics | Q1: Yes |
| ZTE, Sanechips | Q1: Yes |
| Moderator | Thank-you for providing your inputs. There is consensus that the CR in [1] should be included in the editor alignment CR for 38.213. Hence, I will recommend this for email approval. |

# References

R1-2208596	Correction on the parameter indicating  the number of PRBs for common PUCCH	vivo

# Appendix A – Draft CR from [1]

*** Unchanged text omitted ***

### 9.2.1	PUCCH Resource Sets

If a UE does not have dedicated PUCCH resource configuration, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial UL BWP of NBWPsize PRBs. For operation in FR2-2, nrofPRBs provided in PUCCH-ConfigCommon can also provide a number of NRB RBs for the PUCCH resource set; otherwise NRB=1.

*** Unchanged text omitted ***