3GPP TSG-RAN WG1 #110bis			R1-2210703

e-Meeting, October 10 – 19, 2022

Title:	LS on the RRC parameter for multicast HARQ-ACK feedback

Response to:

Release:	Rel-17

Work Item:	NR_MBS-Core

Source:	TSG-RAN WG1

To:	TSG-RAN WG2

Cc:

Contact Person:

#### Name:	Jinhuan XIA

Tel. Number:

E-mail Address:	jinhuan.xia@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R1-2210701

- Overall Description:

Background:

The RRC parameter harq-FeedbackEnablerMulticast in TS38.331 V17.2.0 is described as follows:

harq-FeedbackEnablerMulticast Indicates whether the UE shall provide HARQ feedback for MBS multicast. Value dci-enabler means that whether the UE shall provide HARQ feedback for MBS multicast is indicated by DCI. Value enabled means the UE shall always provide HARQ feedback for MBS multicast. When the field is absent, the value "disabled" is used as defined in TS 38.213 [3].

RAN1 discussed the applicability of this RRC parameter if configured as ‘dci-enabler’ in RAN1#110bis-e meeting and discussed UE behaviour for HARQ-ACK feedback for PDSCHs scheduled by DCI format 4_1 that does not include the ‘Enabling/disabling HARQ-ACK feedback indication’ field, and the following agreement is achieved:

Agreement

If a G-RNTI/G-CS-RNTI is configured with ‘dci-enabler’, the UE transmits the multicast feedback when scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI.

RAN1 would like to attach the corresponding RAN1 CR endorsed in R1-2210701 for RAN2 information and also would like RAN2 to check whether the RAN1 agreement impacts RAN2 specifications and request RAN2 to update the RAN2 specifications if needed.

2. Actions:

To TSG-RAN WG2

ACTION: 	RAN1 respectfully requests RAN2 to check whether the RAN1 agreement impacts RAN2 specifications and request RAN2 to update the RAN2 specifications if needed.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #111                             14 – 18 November 2022			Toulouse, France

TSG-RAN WG1 Meeting #112                              27 February – 3 March 2023	  	 Athens, Greece