3GPP TSG RAN WG1 #112		R1-2300693

Athens, Greece, February 27th – March 3rd, 2023

Source:	CATT

Title:	DTX/DRX for Network Energy Saving

Agenda Item:	9.7.2

Document for:	Discussion and Decision

Introduction

During the study in the network energy saving SI phase, the techniques in time domain mainly aim to reduce the power consumption for dynamic part by trying to shut down more symbols on one or more carriers to achieve BS micro sleep, and the static power part by enlarging the interval between the contiguous active transmission/reception occasions to achieve BS light/deep sleep. From the study report of network energy saving in TR 38.864 [1], the potential enhancement technology in time domain, e.g. cell DTX/DRX transmission, can reduce power consumption, in which the cell may have no transmission/reception or only keep limited transmission/reception.

The scope of Work Item “Network energy savings for NR” [2] includes the enhancement technology in time domain, i.e. cell DTX/DRX transmission in the following items:

Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In this contribution, we mainly discuss the alignment of cell DTX/DRX transmission and UE C-DRX in Section 2. The gNB and UE behaviors in cell DTX/DRX transmission are provided in Section 3. The detailed configuration of cell DTX/DRX transmission is proposed in Section 4. Finally, the cell DTX/DRX transmission for CA is discussed in Section 5.

The alignment of cell DTX/DRX transmission and UE C-DRX

The cell DTX/DRX technique can keep the cell in sleeping mode for a relatively long time without frequent waking-up, which is beneficial to network energy saving. When system load is low and less number of UEs access the system, the staggering C-DRX configuration for system load balancing becomes unnecessary. Instead, the cell could reduce the energy consumption with the DTX transmission in low system load state by allocating same set of C-DRX configuration for all UEs, which including DTX-ON and DTX-OFF. During DTX-OFF, the cell could go to sleep to reduce energy consumption.

For Rel-18 RRC CONNECTED mode UEs, DTX parameters, such as the DTX cycle, the DTX-ON offset and the duration, etc. can be configured by gNB through high layer parameters. The cell DTX cycle could be aligned with that of UE C-DRX. In order not to affect the reception of the DCI format 2_6 outside the active time of UE C-DRX, the cell DTX-ON should start earlier than ps-offset the time where the UE monitoring DCI format 2_6 [3], even earlier than its preparation time for the detection of DCI format 2_6. The cell DTX-ON would completely cover the DRX-ON of UEs as shown in Figure 1.

Figure 1: The cell DTX/DRX structure

Proposal 1: The cell DTX/DRX parameters could be configured to ensure the alignment between the cell DTX/DRX and UE C-DRX, i.e. the cell DTX-ON duration should completely capture the DRX-ON of UEs and at least cover the reception window of DCI format 2_6 in order not to affect the legacy UEs.

For the legacy UE, if C-DRX parameters are configured, UE stays in the active time during the drx-onDurationTimer running, which is configured by high layer signaling. The cell DTX-ON should at least capture the UE DRX-ON duration configured by drx-onDurationTimer. However, if a UE is scheduled a new UL/DL data or expects a retransmission data, the UE still stays in the active time when drx-InactivityTimer, drx-RetransmissionTimerDL or drx-RetransmissionTimerUL is running, the start point of these timers is related to dynamic scheduling. If the UE’s active time is extended by any of drx-InactivityTimer, drx-RetransmissionTimerDL or drx-RetransmissionTimerUL, the cell may enter DTX-OFF before these timers expired. Therefore, if UE DRX-ON is extended beyond the end of cell DTX-ON, UE behaviors should be specified.

Proposal 2: If the cell DTX/DRX is applied, the UE behaviors should be specified when the cell DTX-ON ends earlier than the UE DRX-ON extended by any of drx-InactivityTimer, drx-RetransmissionTimerDL or drx-RetransmissionTimerUL.

The gNB/UE behaviors in cell DTX/DRX transmission

The cell DTX/DRX mechanism should not affect the IDLE/INACTIVE UEs. The gNB could transmit SSB for UE to perform synchronization and RRM measurement in order that the IDLE/INACTIVE UEs can camp on the cell. The IDLE/INACTIVE UEs may perform paging and RACH procedure to access the cell, and thus the paging and RACH procedure should not be affected by the cell DTX/DRX. One possible solution is to have the full subset of the cell DTX-ON/DRX-ON overlapped with the Paging occasion/PRACH resource based on gNB implementation, e.g. the cell DTX-ON/DRX-ON captures the Paging occasion/PRACH resource. Another possible solution is that gNB continue performing paging and RACH procedure within cell DTX-OFF or DRX-OFF period, i.e. the paging and RACH procedure are not restricted by the cell DTX/DRX mechanism.

Proposal 3: If the cell DTX/DRX is applied, the following options can be considered in order not to affect the IDLE/INACTIVE UEs:

- Option 1: At least some cell DTX-ON/DRX-ON and the Paging occasion/PRACH resource are overlapped based on gNB implementation.

- Option 2: Specify that paging and RACH procedure are not affected by cell DTX/DRX mechanism.

For legacy RRC_CONNECTED mode UE, the UE performs the PDCCH monitoring with the CRC scrambled by the specific RNTI (e.g., P-RNTI, SI-RNTI, etc.), reception of the configured PDSCH, transmission and reception without dynamic scheduling and performing channel tracking operation outside the C-DRX active time. The cell DTX/DRX behavior definitions may affect the RRC_CONNECTED UE behavior and transmission performance within cell DTX/DRX OFF duration. The cell DTX/DRX behavior definition has been discussed in RAN2 [4] and four examples for defining the cell DTX/DRX were agreed in RAN2#119bis-e. One example 1 is that gNB is expected to turn off all transmissions and receptions for data traffic and reference signals during Cell DTX/DRX non-active periods. In this example, the reference signal contains SSB, which conflicts with the WID and should be excluded. The remaining three examples can be further discussed. For the convenience of discussion in this contribution, we use option 1/2/3 to represent the remaining three examples and the cell DTX/DRX behavior definition are as following:

- Cell DTX/DRX ON duration: gNB performs the same transmission/reception behaviors as Rel-17.

- Cell DTX/DRX OFF duration:

  - Option 1: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals).

  - Option 2: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

  - Option 3: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The main difference between option 1 and option 2 is whether to support transmission/reception in periodic resources, the option 2 only turns off dynamic scheduling, so the network energy saving gain of option 2 is lower than that of option 1. For legacy RRC_CONNECTED UE, the UE can perform transmission/reception in periodic resources outside the C-DRX active time. If the option 1 of cell DTX/DRX behavior is supported, Rel-18 RRC_CONNECTED UE will not transmit/receive data in periodic resources outside the C-DRX active time when the cell DTX/DRX parameters are configured. The delay of data transmission/reception in periodic resources could increase depending on the implementation. If the option 1 of cell DTX/DRX behavior is supported, the legacy RRC_CONNECTED UE cannot recognize the cell DTX/DRX parameters and the UE will continue to transmit/receive data in periodic resources. Thus, the UE’s power consumption and transmission performance are affected. However, option 2 of cell DTX/DRX behavior can ensure the data transmission/reception in periodic resources as normal. Compared to option 1, the network energy saving gain of option 3 is larger, and the UE only can perform synchronization and measurement by discontinuously detecting SSB and CSI-RS. The gNB/UE behaviors in cell DTX/DRX OFF are summarized in Table 1.

Table 1: gNB/ RRC_CONNECTED UE behaviours during cell DTX/DRX OFF

| Options | gNB behaviors | RRC_CONNECTED UE behaviors |
| --- | --- | --- |
| Option 1 | gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals). | UE transmits/receives reference signals for synchronization, measurement and so on. Legacy/Rel-18 UE cannot transmit/receive data in periodic resources. |
| Option 2 | gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS). | UE transmits/receives reference signals for synchronization, measurement and so on. Legacy/Rel-18 UE transmits/receives data in periodic resources. |
| Option 3 | gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement). | UE performs synchronization and measurement by detecting SSB and CSI-RS. |

Observation 1: If gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods, legacy/Rel-18 UE cannot transmit/receive data in periodic resources as specified in Rel-17, but the network energy saving gain is significant.

Observation 2: If gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods, legacy/Rel-18 UE can transmit/receive data in periodic resources and the UE transmission performance is not affected, but the network energy saving gain is degraded.

Proposal 4: For the cell DTX/DRX technique, the following three options for defining cell DTX/DRX behaviors can be discussed as the baseline and the corresponding UE behaviors should be specified:

- Option 1: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals).

- Option 2: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Option 3: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The configuration of cell DTX/DRX transmission

Regarding the configuration of cell DTX/DRX transmission, cell DTX and cell DRX could be configured jointly or separately and should be discussed firstly. In current NR system, the UL and DL transmission of UE may be related or paired, in particular with HARQ operation. If cell DTX and cell DRX are jointly configured, the cell DTX-ON and cell DRX-ON can be aligned and gNB services both UL data reception and DL data transmission and the respective control signaling during the aligned gNB active time. The reception of UL PUSCH would need to have the DL PDCCH for scheduling the UL transmission. And DL data transmission would require the UL control signaling, such as CSI for link adaptation and HARQ-ACK for HARQ operation. In addition, the RRC_CONNECTED UE can perform transmission/reception in periodic resources outside the C-DRX active time, e.g. UL CG and DL SPS. The cell DTX and cell DRX can be configured separately that the cell DTX-ON and cell DRX-ON flexibly capture CG/SPS periodic resources to avoid affecting UE transmission performance. Since the cell DTX/DRX is applied in low system load, the signaling overhead of separate configuration of cell DTX and cell DRX is acceptable. Considering that the aligned cell DTX-ON and cell DRX-ON can also be implemented by separate cell DTX and DTX configurations, separate cell DTX and DTX configurations are flexible and preferred.

Proposal 5: The cell DTX and cell DRX can be configured and operated separately by higher layer signaling to adapt to the individual characteristics of DL and UL operations.

For the cell DTX/DRX configuration, periodic cell DTX/DRX can be considered. The potential configuration methods related to the cell DTX/DRX can be considered as following:

- Option 1: The DTX/DRX parameters including the DTX/DRX cycle, the DTX-ON/DRX-ON offset and the DTX-ON/DRX-ON duration, are semi-static configured.

- Option 2: The cell DTX/DRX configuration only including the DTX/DRX cycle and the DTX-ON/DRX-ON offset, the end point of cell DTX-ON/DRX-ON may be related the UE’s C-DRX active time.

For option 1, the DTX/DRX parameters may include the DTX/DRX cycle, the DTX-ON/DRX-ON offset and the DTX-ON/DRX-ON duration, and then the periodic cell DTX/DRX transmission is determined. For option 2 that cell DTX/DRX configuration only includes the DTX/DRX cycle and the DTX-ON/DRX-ON offset, the end point of cell DTX-ON/DRX-ON may be related the UE’s C-DRX active time, i.e. UE expects that the end point of cell DTX-ON/DRX-ON is later than its C-DRX active time. The offset between the end of cell DTX-ON/DRX-ON and the end of C-DRX active time could be configured via the high layer signaling.

Proposal 6: Periodic cell DTX/DRX transmission is preferred.

Proposal 7: The potential configuration methods related to the cell DTX/DRX can be considered as following:

- Option 1: The DTX/DRX parameters including the DTX/DRX cycle, the DTX-ON/DRX-ON offset and the DTX-ON/DRX-ON duration, are semi-static configured.

- Option 2: The cell DTX/DRX configuration only including the DTX/DRX cycle and the DTX-ON/DRX-ON offset, the end point of cell DTX-ON/DRX-ON may be related the UE’s C-DRX active time.

The cell DTX/DRX can be configured to Rel-18 CONNECTED UEs via broadcast on system information, e.g. SIB-1. UEs obtain cell DTX/DRX as one of the common system information from SIB-1. In addition, the cell DTX/DRX can be configured by dedicated RRC signaling, e.g. UE receives the cell DTX/DRX configuration along with the C-DRX configuration. Another possible configuration is that the cell DTX/DRX is configured by both broadcast and UE-specific RRC signaling, e.g. SIB carries some common parameters such as cell DTX-ON/DRX-ON duration, and the UE-specific parameters such as cell DTX/DRX cycle are configured by RRC signaling.

Proposal 8: The following signaling design for cell DTX/DRX configuration can be considered:

- Alt 1: SIB broadcast.

- Alt 2: UE specific RRC signaling.

- Alt 3: SIB broadcast and UE specific RRC signaling.

As for whether to support multiple cell DTX/DRX configurations, the traffic arrival may vary and robust over time. It may not be appropriate to use single cell DTX/DRX configuration for a long time. The cell DTX/DRX can be reconfigured by the system information update or RRC reconfiguration. However, one semi-static configuration may be not sufficient to cover all different traffic characteristics at a time. Therefore, multiple cell DTX/DRX can be configured and L1/L2 signaling can dynamically activate/deactivate/switch to at least one cell DTX/DRX configuration. Moreover, multiple DTX/DRX configurations can be activated simultaneously, and the multiple DTX/DRX configurations can be used to capture different semi-static periodic resources by gNB implementation, such as PRACH, CG, SPS, etc.

Proposal 9: Multiple cell DTX/DRX configurations would be supported.

Proposal 10: The cell DTX/DRX is configured to Rel-18 CONNECTED UEs via SIB broadcast or RRC signaling, and L1/L2 signaling is used to activate at least one cell DTX/DRX configuration.

The cell DTX/DRX transmission for CA

For operation with carrier aggregation (CA), the number of active carriers used for data transmission should be as few as possible in case of low system load. From UE power saving perspective, the unused SCells can be deactivated or indicated switching to the dormant BWP for reducing PDCCH monitoring. When the SCell is deactivated or is in the dormant BWP, UE would not receive/transmit the data traffic and reference signals except the sparse CSI-RS/normal SSB or periodic CSI report. In this case, the cell would still be activated for transmitting the CSI-RS/normal SSB or receiving the periodic CSI report. When the system is in the low load, the cell DTX/DRX transmission would be considered to increase gNB light/deep sleep for power saving purpose.

For the cell DTX/DRX transmission for CA, there are several potential DTX/DRX schemes to be considered as following:

- Option 1: The cell DTX/DRX transmission would only be applied in PCell/PSCell.

- Option 2: The cell DTX/DRX transmission could be applied in CA scenario, in which the cell DTX/DRX transmissions for the SCells in the two DRX groups are separately applied.

- Option 3: The cell DTX/DRX transmission could be applied in CA scenario, in which the cell DTX/DRX transmissions for the SCells or SCell groups are separately applied.

For option 1, when the system is in the low load, multiple carrier aggregation might not be applied. Otherwise, for the case that the SCells are activated and have finished the data transmission, gNB might get more network energy saving gain by deactivated SCell or switching to dormant BWP before PCell switching to the DTX/DRX OFF, which belongs to the gNB implementation.

For option 2, according to the description of the cell DTX/DRX transmission in above sections, the cell DTX/DRX would align with UE C-DRX. From the UE C-DRX configuration in TS 38.321 [5], UE can be configured with two DRX groups, in which the DRX parameters drx-onDurationTimer and drx-InactivityTimer are separately configured for each DRX group to achieve the UE power saving, and the other parameters are common to the DRX groups. That’s means the active time of the SCells in the different DRX group could be configured differently according to the data transmission. If the cell DTX/DRX transmission aligned with the UE C-DRX is supported in CA, the cell DTX/DRX transmission for the SCells in the two DRX groups could be separately configured.

For option 3, it is the most flexible and genie method for the cell DTX/DRX transmission in CA scenario. When multiple carriers are used for the data transmission, the active times of the SCells are different for the separate activation/deactivation operations for the SCells. Thus, the cell DTX/DRX transmissions for the different SCells would not necessary to be the same, thus the cell DTX/DRX transmissions for the SCells could be separately applied based on their active time.

If the cell DTX/DRX transmission is applied in CA scenario, the configuration of cell DTX/DRX transmission of SCells could be configured via the SIB broadcast signaling in PCell, or via UE-specific RRC signaling on the SCells.

Proposal 11: For the cell DTX/DRX transmission for CA, there are several potential DTX/DRX schemes to be considered as following:

- Option 1: The cell DTX/DRX transmission would only be applied in PCell/PSCell.

- Option 2: The cell DTX/DRX transmission could be applied in CA scenario, in which the cell DTX/DRX transmissions for the SCells in the two DRX groups are separately applied.

- Option 3: The cell DTX/DRX transmission could be applied in CA scenario, in which the cell DTX/DRX transmissions for the SCells or SCell groups are separately applied.

Conclusion

In this contribution, we discuss the DTX/DRX for network energy saving and have the following observations and proposals.

Observation 1: If gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods, legacy/Rel-18 UE cannot transmit/receive data in periodic resources as specified in Rel-17, but the network energy saving gain is significant.

Observation 2: If gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods, legacy/Rel-18 UE can transmit/receive data in periodic resources and the UE transmission performance is not affected, but the network energy saving gain is degraded.

Proposal 1: The cell DTX/DRX parameters could be configured to ensure the alignment between the cell DTX/DRX and UE C-DRX, i.e. the cell DTX-ON duration should completely capture the DRX-ON of UEs and at least cover the reception window of DCI format 2_6 in order not to affect the legacy UEs.

Proposal 2: If the cell DTX/DRX is applied, the UE behaviors should be specified when the cell DTX-ON ends earlier than the UE DRX-ON extended by any of drx-InactivityTimer, drx-RetransmissionTimerDL or drx-RetransmissionTimerUL.

Proposal 3: If the cell DTX/DRX is applied, the following options can be considered in order not to affect the IDLE/INACTIVE UEs:

- Option 1: At least some cell DTX-ON/DRX-ON and the Paging occasion/PRACH resource are overlapped based on gNB implementation.

- Option 2: Specify that paging and RACH procedure are not affected by cell DTX/DRX mechanism.

Proposal 4: For the cell DTX/DRX technique, the following three options for defining cell DTX/DRX behaviors can be discussed as the baseline and the corresponding UE behaviors should be specified:

- Option 1: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals).

- Option 2: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Option 3: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

Proposal 5: The cell DTX and cell DRX can be configured and operated separately by higher layer signaling to adapt to the individual characteristics of DL and UL operations.

Proposal 6: Periodic cell DTX/DRX transmission is preferred.

Proposal 7: The potential configuration methods related to the cell DTX/DRX can be considered as following:

- Option 1: The DTX/DRX parameters including the DTX/DRX cycle, the DTX-ON/DRX-ON offset and the DTX-ON/DRX-ON duration, are semi-static configured.

- Option 2: The cell DTX/DRX configuration only including the DTX/DRX cycle and the DTX-ON/DRX-ON offset, the end point of cell DTX-ON/DRX-ON may be related the UE’s C-DRX active time.

Proposal 8: The following signaling design for cell DTX/DRX configuration can be considered:

- Alt 1: SIB broadcast.

- Alt 2: UE specific RRC signaling.

- Alt 3: SIB broadcast and UE specific RRC signaling.

Proposal 9: Multiple cell DTX/DRX configurations would be supported.

Proposal 10: The cell DTX/DRX is configured to Rel-18 CONNECTED UEs via SIB broadcast or RRC signaling, and L1/L2 signaling is used to activate at least one cell DTX/DRX configuration.

Proposal 11: For the cell DTX/DRX transmission for CA, there are several potential DTX/DRX schemes to be considered as following:

- Option 1: The cell DTX/DRX transmission would only be applied in PCell/PSCell.

- Option 2: The cell DTX/DRX transmission could be applied in CA scenario, in which the cell DTX/DRX transmissions for the SCells in the two DRX groups are separately applied.

- Option 3: The cell DTX/DRX transmission could be applied in CA scenario, in which the cell DTX/DRX transmissions for the SCells or SCell groups are separately applied.

References

  - 3GPP TR 38.864, Study on network energy savings for NR, v1.0.0.

  - RP-223540,	 New WID: Network energy savings for NR, Huawei, RAN#98-e, December 12nd – 16th, 2022.

  - TS38.213, Physical layer procedures for control, v16.5.0.

  - RAN2 chairman notes, 3GPP TSG RAN WG2 #119bis-e.

  - TS38.321, Medium Access Control (MAC) protocol specification, v17.3.0