3GPP TSG RAN WG1 #112 								     R1-2301428

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.8.2

Source: 	Qualcomm Incorporated

Title: 	On other NCR aspects

Document for:	Discussion/Decision

# 1 Introduction

This contribution provides our views on the remaining aspects of the following objective from the NCR WID:

- Specify control plane signalling and procedures [RAN2, RAN1]

  - The configuration of signalling for side control information indication

    - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed

# 2 Feedback to Side Control Information

RAN1#110-bis-e agreed to the following.

Agreement: HARQ-ACK feedback for PDSCH carrying the side control information from higher layer (e.g., MAC-CE, RRC) is supported. The legacy HARQ-ACK feedback mechanism is reused. - FFS: Whether HARQ-ACK feedback for PDCCH carrying side control information is supported - Note: This does not mean all legacy HARQ-ACK feedback mechanism will be supported.

Regarding the HARQ-ACK feedback for PDCCH carrying the side control info, there is a trade-off between reliability (provided by the feedback), and extra signalling overhead and possibly longer end-to-end latency. In our view, it is beneficial to support HARQ-ACK for PDCCH. However, the network should be given the flexibility to enable/disable this feature as needed.

Observation 1

HARQ-ACK feedback for PDCCH carrying side control can improve the reliability at the expense of more overhead and possibly more end-to-end latency.

Observation 2

Network should have the flexibility to enable/disable HARQ-ACK feedback for PDCCH carrying side control.

Proposal 1

Support optional HARQ-ACK feedback for PDCCH carrying side control information.

# 3 Capability Report

A repeater, just like a UE, should share information about it capability and configurations with the network. A repeater has two components: (a) NCR-MT, and (2) NCR-Fwd. NCR-MT supports features/capabilities, where some of them may need to be reported to the network. Legacy procedures, based on RRC, can be used for NCR-MT’s capability reporting.

Proposal 2

NCR-MT’s capability report follows the same framework as legacy UEs (i.e., via RRC).

The repeater also has some features/configurations associated with its NCR-Fwd, that should be reported to the network. The following was agreed in the last meeting.

Agreement The following aspects should be NCR capability: - Simultaneous UL transmission of C-link and backhaul link - Adaptive beam for C-link/backhaul-link - Note-1: Fixed beam for C-link/backhaul link is default capability - Note-2: TDMed UL transmission of C-link and backhaul link is default capability. - FFS: How to define the capability for adaptive beam for C-link/backhaul-link

RAN1 also discussed the information that should be shared with the network to characterize the NCR-Fwd’s access-link beams (e.g., in terms of number of beams and spatial relationship). RAN1#111 agreed this information should be provided by the OAM.

Agreement: The following is supported to deliver the information to characterize the supported physical beam of NCR-Fwd for access link: Option-2: The information is informed to gNB and NCR via OAM - Note-1: In this option, how to characterize the beam information is based on implementation (e.g., declaration from NCR vendor). - Note-2: In this option, the beam(s) used by NCR-Fwd for access link is configured for gNB and NCR by OAM based on implementation. - The beam index in SCI corresponds to the configured beam(s) sequentially.

In addition to the beam-related information, we believe NCR needs to also share information about its frequency and power configurations.

A repeater may support operating in multiple passbands (consecutive or non-consecutive). The repeater may also have the capability to selectively forward signals in different passbands. If so, such a capability can be utilized by the network to more efficiently (e.g., in terms of interference or power consumption) configure NCR operation – e.g., by instructing the repeater not to forward the signal in a passband for a duration of time.

It is also beneficial for the network to have information about NCR’s power setting (such as NCR-Fwd’s [max] TX power, [max] amplification gain).

As discussed in [1], when multiple RF chains are employed for FR1 NCR beamforming, there could be uncontrollable phase errors on the RF chains. If the relative phase errors across UL chains are different from those across DL chains, the UL composite channel of UE-NCR-gNB link cannot be the same as DL composite channel of gNB-NCR-UE link, which doesn’t guarantee the TDD reciprocity. Therefore, it would be beneficial if the network has information about NCR-FWD’s phase control/calibration aspects, including whether NCR-FWD can support UL-DL reciprocity or not.

Observation 3

A repeater has features/configurations associated with its NCR-Fwd – such as power-related information (e.g., NCR-Fwd’s [max] power, [max] amplification gain), frequency-related information (e.g., supported passbands, and whether NCR supports selective forwarding across the passbands), and information about NCR-FWD’s phase control/calibration aspects including UL-DL reciprocity information.

Proposal 3

The following NCR-Fwd’s information is provided to the network,

- Power-related information (e.g., NCR-Fwd’s [max] power, [max] amplification gain)

- Frequency-related information (e.g., supported passbands, and whether NCR supports selective forwarding across the passbands)

- Information about NCR-FWD’s phase control/calibration, including UL-DL reciprocity info (whether the reciprocity can be supported or not)

FFS: signaling method (e.g., OAM-based signaling or indication by NCR-MT)

# 4 Behaviour of NCR-FWD

There is an FFS item from RAN1#111:

- FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link.

We note that RAN2 already agreed to the following:

- After RLF is declared by NCR-MT, NCR-MT performs cell selection and trigger RRC re-establishment;

- If NCR-MT enters RRC_IDLE due to no suitable cell is find, NCR-Fwd is OFF;

- During RRC re-establishment procedure, NCR-Fwd is OFF.

Considering RAN2’s discussions and agreements, we propose NCR-FWD to be OFF during the BFR procedure. However, it should be further discussed whether NCR-FWD can follow its last configuration (e.g., periodic access-link beam indication via RRC) or should wait for a new RRC reconfiguration after a successful beam failure recovery.

Proposal 4

NCR-Fwd is OFF during NCR-MT’s BFR.

- FFS: NCR-FWD’s behavior after NCR-MT’s BFR (e.g., whether it can follow its last configuration or not)

# 5 Conclusion

In this contribution, we provided our views on NCR’s procedures, and capabilities and made the following observations and proposals.

Observation 1

HARQ-ACK feedback for PDCCH carrying side control can improve the reliability at the expense of more overhead and possibly more end-to-end latency.

Observation 2

Network should have the flexibility to enable/disable HARQ-ACK feedback for PDCCH carrying side control.

Proposal 1

Support optional HARQ-ACK feedback for PDCCH carrying side control information.

Proposal 2

NCR-MT’s capability report follows the same framework as legacy UEs (i.e., via RRC).

Observation 3

A repeater has features/configurations associated with its NCR-Fwd – such as power-related information (e.g., NCR-Fwd’s [max] power, [max] amplification gain), frequency-related information (e.g., supported passbands, and whether NCR supports selective forwarding across the passbands), and information about NCR-FWD’s phase control/calibration aspects including UL-DL reciprocity information.

Proposal 3

The following NCR-Fwd’s information is provided to the network,

- Power-related information (e.g., NCR-Fwd’s [max] power, [max] amplification gain)

- Frequency-related information (e.g., supported passbands, and whether NCR supports selective forwarding across the passbands)

- Information about NCR-FWD’s phase control/calibration, including UL-DL reciprocity info (whether the reciprocity can be supported or not)

FFS: signaling method (e.g., OAM-based signaling or indication by NCR-MT)

Proposal 4

NCR-Fwd is OFF during NCR-MT’s BFR.

- FFS: NCR-FWD’s behavior after NCR-MT’s BFR (e.g., whether it can follow its last configuration or not)

# 6 References

[1] R1-2212130, “On side control information and NCR behavior”, Qualcomm Incorporated.