3GPP TSG RAN WG1 #112	R1-2301477

Athens, Greece, February 27th – March 3rd, 2023

Source:	NTT DOCOMO, INC

Title:	Discussion on unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are three objectives for extension of unified TCI framework, as shown below.

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. […] - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In this contribution, we discuss unified TCI framework related enhancements.

- Unified TCI framework extension for multi-TRP (M-TRP)

  - Single-DCI based M-TRP

- Mapping of indicated TCI state to PDSCH

In RAN1#111, the following agreement was made.

Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI.

Regarding to the 1st FFS point, since existing TCI state field in DCI format 1_1/1_2 is already used for indication of the TCI state, new DCI field should be added to indicate index(es) of indicated TCI state(s). Figure 2-1 illustrates an example of scheduling DCI which indicates one or two index(es) of indicated TCI state(s). In this example, 2 indicated TCI states are indicated by beam indication DCI and/or MAC CE. 2-bit new DCI (e.g. “index(es)”) field in the scheduling DCI of PDSCH indicates the one or two index(es) of indicated TCI states. The DCI codepoint of the new DCI field can be specified or higher layer configured. For example, if the scheduling DCI indicates “00” = “1st”, 1st indicated TCI is applied to the scheduled PDSCH.

Regarding to the 2nd FFS point, UE may receive PDSCH before finishing DCI decoding, and in advance UE should receive and buffer received signal in a certain QCL assumption(s). This behavior is called default QCL assumption in Rel.15-17. In Rel.18, new DCI field is added to indicate index(es) of the indicated TCI state(s). If one or two indicated TCI state(s) are indicated, UE can buffer received signal with all the indicated TCI state(s), and after finishing DCI decoding, UE can understand the index(es) of the indicated TCI state(s) and decode PDSCH with the indicated index(es) of indicated TCI state(s).

Regarding to the last FFS point, since UE cannot distinguish DCI format before finishing DCI decoding, UE should assume the same QCL assumption to buffer received signal for all DCI formats (including whether the new DCI field for index(es) of indicated TCI state(s) exists or not). As discussed above, UE can buffer both of indicated TCI state(s), and after UE identifies the DCI format, UE can receive PDSCH with one or two of indicated TCI state(s). In RAN1#111, it was agreed that the DCI field for the dynamic switching is RRC configured, and if the DCI field is not configured, we need to discuss which of the indicated TCI state(s) is applied to PDSCH. One simple solution is to select one (e.g. 1st) indicated joint/DL TCI state. However, it is not proper to always select one of the two joint/DL TCI states, because of the following reason.

In Rel.17, two TCI states are applied to single DCI based M-TRP PDSCH scheduled by DCI format 1_0 or DCI format 1_1/1_2 without TCI state field in the following cases.

- If UE is configured with both sfnSchemePdcch and sfnSchemePdsch, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, both of two TCI state(s) of scheduling CORESET is applied to PDSCH scheduled by DCI format 1_0 or by DCI format 1_1/1_2.

- For single DCI based M-TRP, if a UE is configured with enableTwoDefaultTCI-States, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one TCI codepoint indicates two TCI states, both of the two TCI states are applied to PDSCH scheduled by DCI format 1_1/1_2 without TCI state field.

Hence, in Rel.18, we believe DCI format 1_0 or DCI format 1_1/1_2 without the new DCI field for dynamic switching should be also able to schedule SFN-PDSCH or M-TRP PDSCH with the two joint/DL TCI states. For example, if sfnSchemePdsch is configured, both of two indicated TCI states are applied to PDSCH scheduled by DCI format 1_0 or by DCI format 1_1/1_2 without the new DCI field. Also, for Single-DCI based M-TRP PDSCH NCJT/repetition, both of two indicated TCI states are applied to PDSCH scheduled by DCI format 1_1/1_2 without the new DCI field.

Proposal 2-1:

  - On unified TCI framework extension for S-DCI based MTRP, support 2-bit new DCI field in DCI format 1_1/1_2 to indicate which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception.

    - Note: if the offset between the reception of the DL DCI and the corresponding PDSCH is less than timeDurationForQCL, UE buffers received signal with both of the indicated joint/DL TCI state(s).

    - For PDSCH scheduled by DCI format 1_0, if sfnSchemePdsch is configured, both of the indicated joint/DL TCI state(s) are applied to PDSCH. Else, only 1st indicated joint/DL TCI state is applied to PDSCH.

    - For PDSCH scheduled by DCI format 1_1/1_2 without the new 2-bit DCI field, both of the indicated joint/DL TCI state(s) are applied to PDSCH.

Figure 2-1. Example of scheduling DCI which indicates one or two index(es) of indicated TCI states.

- Mapping of indicated TCI state to PUSCH

For PUSCH, we have the following agreement in RAN1#111 meeting.

Agreement On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

For single-DCI based M-TRP PUSCH repetition in Rel.17, one or two SRS resource sets with usage = CB/NCB can be configured, and 2-bit “SRS resource set indicator” field in DCI format 0_1/0_2 indicates dynamic switching between S-TRP PUSCH repetition and M-TRP PUSCH repetition. Table 2-2 shows “SRS resource set indicator” field in DCI format 0_1/0_2 in Rel.17.

In RAN1#111 meeting, it was discussed whether to add new DCI field or reuse the SRS resource set indicator in DCI format 0_1/0_2 to inform which indicated joint/UL TCI state(s) UE shall apply to PUSCH transmission. In our view, typical operation is that gNB configures followUnifiedTCIstateSRS to both SRS resource sets with usage = CB/NCB (i.e. 1st indicated joint/UL TCI state is applied to the 1st SRS resource set, and 2nd indicated joint/UL TCI state is applied to the 2nd SRS resource set.) In this case, the two options (i.e. add new DCI field or reuse the SRS resource set indicator) have no difference. If we reuse the SRS resource set indicator field, we can reuse Rel.17 spec mostly.

Proposal 2-2:

  - On unified TCI framework extension for S-DCI based MTRP,

    - for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, reuse 2-bit “SRS resource set indicator” field to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2.

    - for PUSCH transmission scheduled/activated by a DCI format 0_0, only 1st indicated joint/UL TCI state is applied to PUSCH.

Table 2-2. “SRS resource set indicator” field in DCI format 0_1/0_2 in Rel.17.

| Codepoint | S-TRP/M-TRP PUSCH repetition |
| --- | --- |
| 0 | S-TRP (1st SRS resource set only) |
| 1 | S-TRP (2nd SRS resource set only) |
| 2 | M-TRP (1st SRS resource set 2nd SRS resource set) |
| 3 | M-TRP (2nd SRS resource set 1st SRS resource set) |

  - Multi-DCI based M-TRP

- Mapping of indicated TCI state to PDCCH/PDSCH/CSI-RS

In RAN1#111, the following agreement was made for PDCCH/PDSCH.

Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH.

For A-CSI-RS, considering a DCI may trigger multiple CSI-RS resources and the CSI-RS resources in a triggered CSI-RS resource set may be transmitted from different TRPs, it is beneficial that coresetPoolIndex can be configured per CSI-RS resource for A-CSI-RS to indicate which indicated joint/DL TCI state is applied to the CSI-RS resource. Similarly, for P/SP CSI-RS, coresetPoolIndex can be configured per CSI-RS resource.

Proposal 2-3:

  - For unified TCI extension for multi-DCI based M-TRP, support the following explicit association between coresetPoolIndex value and each CSI-RS resource to determine the applied indicated joint/DL TCI state.

    - For A-CSI-RS, coresetPoolIndex is RRC-configured per CSI-RS resource.

    - For P/SP-CSI-RS, coresetPoolIndex is RRC-configured per CSI-RS resource.

- Mapping of indicated TCI state to PUSCH/SRS/PUCCH

For A-SRS, considering a DCI may trigger multiple SRS resource sets and the SRS resources in the multiple SRS resource sets may be transmitted to different TRPs, it is beneficial that CORESETPoolIndex can be configured per SRS resource for AP SRS. For dynamic HARQ-ACK, considering joint ACK/NACK feedback mode in M-DCI M-TRP, a DCI may indicate a PUCCH resource to either of the TRPs (i.e. TRP1 may send DCI to indicate PUCCH resource to TRP2). Thus, for dynamic HARQ-ACK, it is beneficial that CORESETPoolIndex can be configured per PUCCH resource.

Proposal 2-4:

  - For unified TCI extension for multi-DCI based M-TRP, support the following implicit/explicit association between coresetPoolIndex value and each applied channel/RS to determine the applied indicated joint/UL TCI state.

    - For dynamically scheduled/activated PUSCH, CORESETPoolIndex associated with the scheduling/activating PDCCH is utilized.

    - For Type 1 CG, P/SP-SRS, and P/SP-PUCCH, CORESETPoolIndex is RRC-configured.

    - For AP-SRS, CORESETPoolIndex is RRC-configured per SRS resource.

    - For dynamic HARQ-ACK and other UCIs, CORESETPoolIndex is RRC-configured per PUCCH resource.

  - TRP specific BFR for unified TCI

In RAN1#110bis-e meeting, the following agreement was made.

Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request

We believe it is beneficial to support TRP specific BFR for unified TCI state. For example, implicit BFD-RS determination rule should be updated so that the two indicated joint/DL TCI states for Single-DCI based M-TRP can be used for BFD-RS for the two TRPs. For implicit BFD-RS for multi-DCI based M-TRP, no spec. enhancement would be needed to determine two indicated joint/DL TCI states for two TRPs, while we are open to discuss if any spec. enhancement is needed.

After the completion of the BFR for both Single-DCI based and Multi-DCI based M-TRP, indicated joint/DL/UL TCI state(s) and TPC assumptions should be updated.

Proposal 2-5:

  - For unified TCI extension for single/multi-DCI based M-TRP, support the following enhancement for TRP specific BFR.

    - For implicit BFD RS determination of single-DCI based M-TRP, use two joint/DL TCI states for BFD RS for two TRPs.

      - Note: For implicit BFD RS determination of multi-DCI based M-TRP, use each joint/DL TCI state of each CoresetPoolIndex for each BFD RS set.

    - After NW response of TRP-specific BFR request for both Single-DCI based and Multi-DCI based M-TRP, joint/DL/UL TCI state(s) and TPC assumptions should be updated.

- Power control for M-TRP operation

Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH , and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case - FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 (approved on May 24th, see below). R1-2205639 LS on UE power limitation for STxMP in FR2 RAN1, MediaTek Decision: As per email decision posted on May 24th, the LS is approved.

For S-DCI based TDM PUSCH/PUCCH repetition, it is agreed that if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH , and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. And how to extend such association to STxMP is FFS. In RAN1#110, a working assumption was made to support STxMP PUSCH SDM scheme. In RAN1#110b-e and RAN#111, agreements were made to support STxMP PUSCH SFN scheme and PUCCH SFN scheme. Association between UL PC parameter setting and PUSCH/PUCCH in STxMP needs to be defined. To determine UL PC parameter setting for STxMP PUSCH/PUCCH schemes, similar solution as TDM M-TRP repetition can be used, i.e., for STxMP PUSCH SDM/PUSCH SFN/PUCCH SFN scheme, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission.

Proposal 3-1:

  - For STxMP PUSCH SDM/PUSCH SFN/PUCCH SFN scheme, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission.

In addition to PC parameter setting, PCMAX in STxMP needs to be studied. For UL power control, UE configured maximum output power PCMAX is defined. UE UL Tx power at a transmission occasion cannot exceed PCMAX. For example, PUSCH transmission power is determined in the following equation.

[dBm]

For STxMP, how to determine PCMAX of each panel needs to be studied. Following two assumptions were discussed in the previous meeting, and an LS was sent to RAN4 regarding the feasibility of the two assumptions. Details of UL power control of STxMP can be discussed after receiving reply from RAN4.

- Assumption1: power limitation per panel is assumed. UE UL Tx power of each panel cannot exceed the per panel limitation.

- Assumption2: power limitation per UE is assumed. Sum of UE UL Tx power of two panels cannot exceed the per UE limitation.

Proposal 3-2:

  - For STxMP, how to determine PCMAX for transmission from each panel can be further studied after receiving RAN4 reply on the feasibility of assumptions of power limitation.

For M-TRP PUSCH/PUCCH, the applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH was discussed. Default power control parameter setting was supported in Rel-17 unified TCI. In Rel-17, a default power control parameter setting can be configured in BWP-UplinkDedicated and the configured default power control parameter setting can be used when joint or UL TCI is not associated with an UL PC parameter setting. Furthermore, per TRP default power control parameter was supported in Rel-17 S-DCI M-TRP PUSCH repetition. In Rel-17 M-TRP PUSCH, per TRP default power control parameter can be determined and used when SRI field is not included in DCI or association between PC parameter and SRI is not provided. Thus, from our perspective, per TRP default power control parameter setting should also be supported with Rel-18 unified TCI framework.

A straightforward enhancement is to configure two default power control parameter settings in BWP-UplinkDedicated for two TRPs. The default power control parameter settings can be used when joint or UL TCI is not associated with an UL PC parameter setting. For M-TRP PUSCH/PUCCH, the first/second default power control parameter can correspond to first/second “indicated TCI” respectively. If the first/second “indicated TCI” applies to a PUSCH/PUCCH transmission and is not associated with a PC parameter setting, the corresponding default power control parameter can be applied to the PUSCH/PUCCH transmission.

Proposal 3-3:

  - Configure two default power control parameter settings in BWP-UplinkDedicated for two TRPs. The default power control parameter settings can be used when joint or UL TCI is not associated with an UL PC parameter setting.

  - For M-TRP PUSCH/PUCCH, the first/second default power control parameter can correspond to first/second “indicated TCI” respectively. If the first/second “indicated TCI” applies to a PUSCH/PUCCH transmission and is not associated with a PC parameter setting, the corresponding default power control can be applied to the PUSCH/PUCCH transmission.

Beam reporting for STxMP

To support STxMP, gNB needs to be aware of whether two beams can be transmitted simultaneously by UE. One option is to report correspondence between beams and UE panels. The beams corresponding to different UE panels can be transmitted simultaneously by UE. In Rel-17, UE capability value index can be reported for each beam in beam reporting. However, in Rel-17, each UE capability value refers to unique UE capability. For STxMP, to report correspondence between beams and panels and considering two panels may have same capability, enhancement based on Rel-17 UE capability value reporting framework can be considered. Based on Rel-17 UE capability value reporting framework, UE capability values with same capability can be supported.

Another option is to directly report pair(s) of beams that can be transmitted simultaneously by UE. In Rel-15/17, group-based beam reporting is supported for DL Rx, i.e., UE can report pair(s) of beams that can be received simultaneously by UE. For STxMP, to support UE reporting pair(s) of beams that can be transmitted simultaneously by UE, enhancement based on Rel-15/17 group-based beam reporting framework can be considered. Based on Rel-15/17 group-based beam reporting framework, if STxMP is supported, UE shall report pair(s) of beams that can be received simultaneously by UE and can also be transmitted simultaneously by UE. Alternatively, new UL group-based beam reporting which is independent from Rel-15/17 group-based beam reporting can be considered. In the new UL group-based beam reporting, UE shall report pair(s) of beams that can be transmitted simultaneously by UE.

Proposal 4-1:

  - To support STxMP, following options of enhancement on beam reporting can be considered.

    - Option1: reuse Rel-17 UE capability value reporting framework with enhancement that when STxMP is supported UE can report UE capability values with same capability. Two beams with different UE capability value set index correspond to different panel and can be transmitted simultaneously by UE.

    - Option2: reuse Rel-15/17 group-based beam reporting framework with enhancement that when STxMP is supported UE shall report pair(s) of beams that can be received simultaneously by UE and can also be transmitted simultaneously by UE.

    - Option3: introduce new UL group-based beam reporting. In UL group-based beam reporting, UE shall report pair(s) of beams that can be transmitted simultaneously by UE.

- Conclusion

In this contribution, we discussed unified TCI framework related enhancements. Based on the discussion, we made the following proposals.

Unified TCI framework extension for M-TRP

Proposal 2-1:

  - On unified TCI framework extension for S-DCI based MTRP, support 2-bit new DCI field in DCI format 1_1/1_2 to indicate which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception.

    - Note: if the offset between the reception of the DL DCI and the corresponding PDSCH is less than timeDurationForQCL, UE buffers received signal with both of the indicated joint/DL TCI state(s).

    - For PDSCH scheduled by DCI format 1_0, if sfnSchemePdsch is configured, both of the indicated joint/DL TCI state(s) are applied to PDSCH. Else, only 1st indicated joint/DL TCI state is applied to PDSCH.

    - For PDSCH scheduled by DCI format 1_1/1_2 without the new 2-bit DCI field, both of the indicated joint/DL TCI state(s) are applied to PDSCH.

Proposal 2-2:

  - On unified TCI framework extension for S-DCI based MTRP,

    - for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, reuse 2-bit “SRS resource set indicator” field to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2.

    - for PUSCH transmission scheduled/activated by a DCI format 0_0, only 1st indicated joint/UL TCI state is applied to PUSCH.

Proposal 2-3:

  - For unified TCI extension for multi-DCI based M-TRP, support the following explicit association between coresetPoolIndex value and each CSI-RS resource to determine the applied indicated joint/DL TCI state.

    - For A-CSI-RS, coresetPoolIndex is RRC-configured per CSI-RS resource.

    - For P/SP-CSI-RS, coresetPoolIndex is RRC-configured per CSI-RS resource.

Proposal 2-4:

  - For unified TCI extension for multi-DCI based M-TRP, support the following implicit/explicit association between coresetPoolIndex value and each applied channel/RS to determine the applied indicated joint/UL TCI state.

    - For dynamically scheduled/activated PUSCH, CORESETPoolIndex associated with the scheduling/activating PDCCH is utilized.

    - For Type 1 CG, P/SP-SRS, and P/SP-PUCCH, CORESETPoolIndex is RRC-configured.

    - For AP-SRS, CORESETPoolIndex is RRC-configured per SRS resource.

    - For dynamic HARQ-ACK and other UCIs, CORESETPoolIndex is RRC-configured per PUCCH resource.

Proposal 2-5:

  - For unified TCI extension for single/multi-DCI based M-TRP, support the following enhancement for TRP specific BFR.

    - For implicit BFD RS determination of single-DCI based M-TRP, use two joint/DL TCI states for BFD RS for two TRPs.

      - Note: For implicit BFD RS determination of multi-DCI based M-TRP, use each joint/DL TCI state of each CoresetPoolIndex for each BFD RS set.

    - After NW response of TRP-specific BFR request for both Single-DCI based and Multi-DCI based M-TRP, joint/DL/UL TCI state(s) and TPC assumptions should be updated.

Power control for M-TRP operation

Proposal 3-1:

  - For STxMP PUSCH SDM/PUSCH SFN/PUCCH SFN scheme, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission.

Proposal 3-2:

  - For STxMP, how to determine PCMAX for transmission from each panel can be further studied after receiving RAN4 reply on the feasibility of assumptions of power limitation.

Proposal 3-3:

  - Configure two default power control parameter settings in BWP-UplinkDedicated for two TRPs. The default power control parameter settings can be used when joint or UL TCI is not associated with an UL PC parameter setting.

  - For M-TRP PUSCH/PUCCH, the first/second default power control parameter can correspond to first/second “indicated TCI” respectively. If the first/second “indicated TCI” applies to a PUSCH/PUCCH transmission and is not associated with a PC parameter setting, the corresponding default power control can be applied to the PUSCH/PUCCH transmission.

Beam reporting for STxMP

Proposal 4-1:

  - To support STxMP, following options of enhancement on beam reporting can be considered.

    - Option1: reuse Rel-17 UE capability value reporting framework with enhancement that when STxMP is supported UE can report UE capability values with same capability. Two beams with different UE capability value set index correspond to different panel and can be transmitted simultaneously by UE.

    - Option2: reuse Rel-15/17 group-based beam reporting framework with enhancement that when STxMP is supported UE shall report pair(s) of beams that can be received simultaneously by UE and can also be transmitted simultaneously by UE.

    - Option3: introduce new UL group-based beam reporting. In UL group-based beam reporting, UE shall report pair(s) of beams that can be transmitted simultaneously by UE.

- Reference

[1] 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.