3GPP TSG-RAN WG1 Meeting #112	R1-2302139

Athens, Greece, February 27 – March 03, 2023

Agenda Item:	7.1

Source:	Moderator (HiSilicon)

Title:	Summary #2 of discussion on the multiplexing between SR/SPS A/N and CSI

Document for:	Discussion and Decision

# Introduction

This document is created to collect company views on the proposals in [1].

# Problem description

## Case A: a UE is not provided multi-CSI-PUCCH-ResourceList

In the Subclause 9.2.5 of TS38.213, if the UE is not provided multi-CSI-PUCCH-ResourceList, the UE would determine two non-overlapped CSI PUCCHs according to the CSI priority according to the spec (highlighted in yellow in Appendix A).

Then if these two PUCCHs overlaps with SPS HARQ-ACK, the UE would multiplex the SPS HARQ-ACK in the CSI PUCCH with the higher priority and drop the CSI PUCCH with the lower priority. The corresponding spec is marked in pink in Appendix A. An example is shown in figure 1 below.

Figure 1 if a UE is not provided multi-CSI-PUCCH-ResourceList, the SPS A/N and the higher priority CSI would be multiplexed, the lower priority CSI PUCCH would be dropped

## Case B: a UE is provided multi-CSI-PUCCH-ResourceList

If a UE is provided multi-CSI-PUCCH-ResourceList, the UE would multiplex all CSI reports in a resource if any of the multiple CSI PUCCH resources overlapped. However, if in some slots the CSI PUCCH resources do not overlap in time, then according to the spec (marked in green in Appendix A), the UE would still determine two non-overlapped CSI PUCCHs according to the CSI priority (similar as Case A). An example is shown in figure 2 below assuming the period of CSI report 1 and 2 is 4 slots but the period of CSI report 3 is 8 slots. Then in slot 0 and slot 8, the UE would multiplex all CSI reports in a resource. But in slot 4, the UE would not perform CSI multiplexing.

Figure 2 if a UE is provided multi-CSI-PUCCH-ResourceList, in slot 0 and slot 8, the UE would multiplex all CSI reports in a resource. But in slot 4, the UE would not perform CSI multiplexing.

However, in slot 4 if these two PUCCHs still overlap with SPS HARQ-ACK shown in figure 3 below, then it is not clear about the UE behavior since the condition in the spec (marked in pink in Appendix A) for the multiplexing rule between SPS A/N and two CSI PUCCHs is not satisfied.

Figure 3 if a UE is provided multi-CSI-PUCCH-ResourceList, UE behavior is not clear if the SPS HARQ-ACK overlaps with two CSI PUCCHs

To solve this problem, a TP is proposed below. That is, if a UE is provided multi-CSI-PUCCH-ResourceList but the CSI PUCCHs do not overlap in the slot, the UE still multiplex the SPS A/N and the higher priority CSI, the lower priority CSI is dropped. It is the same behavior as figure 1.

--------------------------------------------------Start of the TP--------------------------------------------------

If

-	a UE is not provided multi-CSI-PUCCH-ResourceList or if PUCCH resources for transmissions of CSI reports do not overlap in the slot, and

-	a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and

-	there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and

-	the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports

the UE

-	multiplexes the HARQ-ACK information and/or the SR in the resource for the PUCCH transmission with the CSI report having the higher priority, and

-	does not transmit the PUCCH with the CSI report having the lower priority

--------------------------------------------------End of the TP--------------------------------------------------

# Discussion

## 1st Round Discussion

### Companies’ view

Q1: for case B i.e. the UE is provided multi-CSI-PUCCH-ResourceList,  the UE may still  determine two non-overlapped CSI PUCCHs according to the CSI priority in some slots (e.g. slot 4 in figure 2) where the CSI PUCCH resources do not overlap in time.  If these two non-overlapped CSI PUCCHs overlaps with the SPS A/N (e.g. in figure 3), do you agree that UE behavior is not clear according to the current spec? If not, why?

| Company | Yes/No | Comment |
| --- | --- | --- |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes | The spec does not cover the case that multi-CSI-PUCCH-ResourceList is provided and the determined two non-overlapped PUCCH resources with CSI reports overlap with another SPS PUCCH transmission, such as SPS HARQ feedback or SR. |
| Ericsson | No | For clause 9.2.5, first the following is executed to resolve overlapping between any CSI PUCCH resource, if any. Therefore, the outcome of this part would be non-overlapping CSI resources. If UE is considered with multi-CSI-PUCCH-ResourceList, we could have only one CSI PUCCH resource in a slot. If the UE is not provided with multi-CSI-PUCCH-ResourceList, we could have multiple non-overlapping CSI PUCCH. Then, the paragraph pointed in the CR, determines in case there are multiple CSI PUCCH resources (non-overlapping), and overlap with SR and Dl SPS HARQ-ACK, which CSI PUCCH to use (among those ones), for multiplexing. Therefore, we think spec is clear and also covers the case, raised in the CR. This clause is applicable to the case that a UE has resources for PUCCH transmissions or for PUCCH and PUSCH transmissions that overlap in time and each PUCCH transmission is over a single slot without repetitions. Any case that a PUCCH transmission is with repetitions over multiple slots is described in clause 9.2.6. If a UE is configured with multiple PUCCH resources in a slot to transmit CSI reports - if the UE is not provided multi-CSI-PUCCH-ResourceList or if PUCCH resources for transmissions of CSI reports do not overlap in the slot, the UE determines a first resource corresponding to a CSI report with the highest priority [6, TS 38.214] - if the first resource includes PUCCH format 2, and if there are remaining resources in the slot that do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting - if the first resource includes PUCCH format 3 or PUCCH format 4, and if there are remaining resources in the slot that include PUCCH format 2 and do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting - if the UE is provided multi-CSI-PUCCH-ResourceList and if any of the multiple PUCCH resources overlap, the UE multiplexes all CSI reports in a resource from the resources provided by multi-CSI-PUCCH-ResourceList, as described in clause 9.2.5.2.. |
| QC | Yes | To Ericsson: our understanding is the case Huawei identified is: UE is provided multi-CSI-PUCCH-ResourceList, but none of the multiple PUCCH resources overlap overlap. So, to us, it seems that the above spec Ericsson cited does not cover the case Huawei mentioned. |
| Samsung | No | We think that current specification is still working if the SPS HARQ-ACK PUCCH starts later than CSI PUCCH. Since PUCCH with CSI report 1 is reference PUCCH for resolving overlapping multiple PUCCHs, HARQ A/N will be multiplexed into PUCCH with CSI report 1. Then, UE would transmit both PUCCHs in a slot. Basically, the pseudo-code in 9.2.5 applies. With the CR, the UE behavior would be incorrect as the UE will drop the second CSI report even when, after resolving overlapping, the corresponding PUCCH does not overlap with another PUCCH. |
| ZTE | No | Samsung raised a good point that the UE behavior is clear for some of the overlapping cases (when PUCCH with CSI report 1 is the reference PUCCH). The CR would change legacy UE behavior. For other cases, i.e., the reference PUCCH is the PUCCH for HARQ-ACK, it can be considered as error cases. |
| Spreadtrum | Yes | The issue is valid. UE’s behavior is not clear in this case, it should be same as multi-CSI-PUCCH-ResourceList is configured. |

Q2: if yes for Q1, then do you agree the above TP to solve this issue? If not, please indicate the suggested changes?

| Company | Yes/No | Comment |
| --- | --- | --- |
| vivo |  | The issue is valid. But such overlapping case can be avoided by gNB’s configuration, for example, the two CSI PUCCHs are overlapped. Or the PUCCH for HARQ-ACK only overlaps with one them. It is too late to adopt the CR for Rel-15. |
| Huawei, HiSilicon | Yes | We are fine with the moderator’s suggestion and we are also open to other changes, such as delete the first condition as following: If - a UE is not provided multi-CSI-PUCCH-ResourceList, and - a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and - there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and - the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports |
| QC |  | The issue is valid. But the wording of the CR can be improved such as the following. “a UE is not provided multi-CSI-PUCCH-ResourceList or if a UE is provided multi-CSI-PUCCH-ResourceList and PUCCH resources for transmissions of CSI reports do not overlap in the slot, and” |
| Samsung | No | If TP is adopted, it makes different UE behavior to what current specification provides as we explained the result in Q1. |
| Spreadtrum |  | We are fine with the intention. |

Q3: if changes in Q2 are needed, since which release of spec do you prefer to capture the changes?

| Company | Yes/No | Comment |
| --- | --- | --- |
| vivo |  | Rel-16 can be acceptable to us. |
| Huawei, HiSilicon |  | Our prioritization is to capture the missing case since R15, however, considering R15 devices already deployed, R16 can be also acceptable for us |
| QC |  | We prefer to take the change in Rel-16 spec. |
| Spreadtrum |  | It is too late for Rel-15. Rel-16 or 17 is our preference. |

### Summary of Companies’ view

Based on the inputs of first round discussion, 4 companies think current spec is not clear and support to have a change on the spec. 3 companies think the changes are not needed. From moderator perspective, I think the companies’ concern can be explained as following.

@Ericsson: The cited paragraph is used to determine the non-overlapped PUCCH resources for CSI reports, and the CR intends resolve the case that these two non-overlapped resources overlapping with another SPS transmission. Thus, it is different.

@Samsung, ZTE: Following the pseudo code, UE can determine a PUCCH overlapping group and requests to generate a single PUCCH resource to resolve the overlapping (highlighted in yellow below). However, it does not define a processing order, for example, it does not require UE to multiplex HARQ on the first PUCCH shown in the figure 3. Since all three PUCCH resources belong to the same PUCCH overlapping group (set Q), only one PUCCH resources should be determined, but such rule is still missing when multi-CSI resource is provided and PUCCH resources for CSI report are non-overlapped.

if o>0 determine a single resource for multiplexing UCI associated with resources Qj-o,Qj-o+1,…,Q(j) as described in clauses 9, 9.2.5.0, 9.2.5.1, 9.2.5.2, and 9.2.5.3 set the index of the single resource to j Q=Q\Qj-o,Qj-o+1,…,Q(j-1) j=0 % start from the beginning after reordering unmerged resources at next step o=0; order(Q) % function that re-orders resources in current set Q Set CQ to the cardinality of Q

### Proposal for Wednesday online

Since the majority can accept to have a change on Rel-16 spec, and take the advice from QC, following proposal is provided for online discussion.

Proposal: To endorse following changes on Rel-16 spec of TS 38.213 Section 9.2.5

--------------------------------------------------Start of the TP--------------------------------------------------

If

-	a UE is not provided multi-CSI-PUCCH-ResourceList or if a UE is provided multi-CSI-PUCCH-ResourceList and PUCCH resources for transmissions of CSI reports do not overlap in the slot, and

-	a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and

-	there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and

-	the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports

the UE

-	multiplexes the HARQ-ACK information and/or the SR in the resource for the PUCCH transmission with the CSI report having the higher priority, and

-	does not transmit the PUCCH with the CSI report having the lower priority

--------------------------------------------------End of the TP--------------------------------------------------

## Discussion in next meeting

### Justification on the overlapping cases

Based on current spec, pseudo code in Section 9.2.5 of TS 38.213 is used to determine an overlapping PUCCH group (Set Q) and a single PUCCH resource will be determined to resolve the overlapping. In addition, the Set Q could be different according to which reference PUCCH is selected. The reference PUCCH is selected based on starting symbol, duration, etc.

After further discussing with companies, it seems the case described in Figure 3 can be further divided in two sub-cases as per different reference PUCCH resources are selected. These two cases can be further discussed in future meeting.

Case 1: PUCCH with SPS HARQ-ACK overlapping with two non-overlapped PUCCH with CSI reports, and CSI report starting earlier is the reference PUCCH determined by the pseudo code in TS 38.213 Section 9.2.5

Case 2: PUCCH with SPS HARQ-ACK overlapping with two non-overlapped PUCCH with CSI reports, and SPS HARQ is the reference PUCCH determined by the pseudo code in TS 38.213 Section 9.2.5

### Proposal for Friday online

To leave a hint for discussion in next meeting, moderator suggests to have following proposal.

Proposed conclusion:

To clarify how to handle the overlapping between SPS PUCCH transmission and two non-overlapped PUCCH with CSI reports, the two cases identified in Section 3.2.1 of R1-2302139 can be further discussed.

# Conclusions

Will update after discussion.

# References

R1-2301716	Discussion on the multiplexing between SR/SPS A/N and CSI	Huawei, HiSilicon

# Appendix A. TS 38.213, Clause 9.2.5

### 9.2.5 UE procedure for reporting multiple UCI types This Clause is applicable to the case that a UE has overlapping resources for PUCCH transmissions or for PUCCH and PUSCH transmissions and each PUCCH transmission is over a single slot without repetition. Any case that a PUCCH transmission is with repetitions over multiple slots is described in Clause 9.2.6. If a UE is configured with multiple PUCCH resources in a slot to transmit CSI reports - if the UE is not provided multi-CSI-PUCCH-ResourceList or if PUCCH resources for transmissions of CSI reports do not overlap in the slot, the UE determines a first resource corresponding to a CSI report with the highest priority [6, TS 38.214] - if the first resource includes PUCCH format 2, and if there are remaining resources in the slot that do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting - if the first resource includes PUCCH format 3 or PUCCH format 4, and if there are remaining resources in the slot that include PUCCH format 2 and do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting - if the UE is provided multi-CSI-PUCCH-ResourceList and if any of the multiple PUCCH resources overlap, the UE multiplexes all CSI reports in a resource from the resources provided by multi-CSI-PUCCH-ResourceList, as described in Clause 9.2.5.2. …… If - a UE is not provided multi-CSI-PUCCH-ResourceList, and - a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and - there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and - the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports the UE - multiplexes the HARQ-ACK information and/or the SR in the resource for the PUCCH transmission with the CSI report having the higher priority, and - - does not transmit the PUCCH with the CSI report having the lower priority