3GPP TSG-RAN WG1 Meeting #112	R1- 2302184

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0138 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to ChanneAccess-CPext field in DCI formats x_2 in TS38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia(Moderator), Huawei, HiSilicon, vivo, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Although DCI formats 0_1/1_1 support indicating the channel access type in FR 2-2 via the field ChannelAccess-CPext(-CAPC), currently the DCI formats 0_2/1_2 do not support such indication in FR 2-2. This precludes the use of the compact DCI formats 0_2/1_2 for scheduling UL with shared spectrum access in FR 2-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ChannelAccess-CPext-CAPC field in DCI format x_2 is present for operation in a cell with shared spectrum channel access in FR2-2 and if ChannelAccessMode2-r17 is enabled. Higher layer parameter for DCI x_1 is reused for DCI x_2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Compact DCI formats x_2 cannot be used for UL scheduling with shared spectrum access in FR 2-2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.2, 7.3.1.1.3, 7.3.1.2.2, 7.3.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.1.2	Format 0_1

*** Unchanged text omitted ***

Table 7.3.1.1.2-35: Allowed entries for DCI format 0_1 and DCI format 0_2, configured by higher layer parameter ul-AccessConfigListDCI-0-1 and ul-AccessConfigListDCI-0-2, respectively, in frequency range 1

| Entry index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, 38.211] | CAPC |
| --- | --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 1 |
| 1 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 2 |
| 2 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 3 |
| 3 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 4 |
| 4 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 1 |
| 5 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 2 |
| 6 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 3 |
| 7 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 4 |
| 8 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 0 | 1 |
| 9 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 0 | 2 |
| 10 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 0 | 3 |
| 11 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 0 | 4 |
| 12 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 2 | 1 |
| 13 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 2 | 2 |
| 14 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 2 | 3 |
| 15 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 2 | 4 |
| 16 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 1 |
| 17 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 2 |
| 18 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 3 |
| 19 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 4 |
| 20 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 1 |
| 21 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 2 |
| 22 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 3 |
| 23 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 4 |
| 24 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 1 |
| 25 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 2 |
| 26 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 3 |
| 27 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 4 |
| 28 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 1 |
| 29 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 2 |
| 30 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 3 |
| 31 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 4 |
| 32 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 1 |
| 33 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 2 |
| 34 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 3 |
| 35 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 4 |
| 36 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 1 |
| 37 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 2 |
| 38 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 3 |
| 39 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 4 |
| 40 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 1 |
| 41 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 2 |
| 42 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 3 |
| 43 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 4 |

Table 7.3.1.1.2-35A: Allowed entries for DCI format 0_1 and DCI format 0_2, configured by higher layer parameter ul-AccessConfigListDCI-0-1 in frequency range 2-2

| Entry index | Channel Access Type |
| --- | --- |
| 0 | Type 1 channel access defined in clause 4.4.1 of 37.213 |
| 1 | Type 2 channel access defined in clause 4.4.2 of 37.213 |
| 2 | Type 3 channel access defined in clause 4.4.3 of 37.213 |

*** Unchanged text omitted ***

##### 7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

*** Unchanged text omitted ***

-	ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-2 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or the number of entries in the high layer parameter ul-AccessConfigListDCI-0-1 for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 are configured by the higher layer parameter ul-AccessConfigListDCI-0-2 in frequency range 1. One or more entries from Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1 in frequency range 2-2.

*** Unchanged text omitted ***

##### 7.3.1.2.2	Format 1_1

*** Unchanged text omitted ***

Table 7.3.1.2.2-6: Allowed entries for DCI format 1_1 and DCI format 1_2, configured by higher layer parameter ul-AccessConfigListDCI-1-1 and ul-AccessConfigListDCI-1-2, respectively, in frequency range 1

| Entry index | Channel Access Type | The CP extension Text index defined in Clause 5.3.1 of [4, TS 38.211] |
| --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 |
| 1 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 |
| 2 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 0 |
| 3 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 2 |
| 4 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 |
| 5 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 |
| 6 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 |
| 7 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 |
| 8 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 |
| 9 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 |
| 10 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 |

Table 7.3.1.2.2-6A: Allowed entries for DCI format 1_1 and DCI format 1_2, configured by higher layer parameter ul-AccessConfigListDCI-1-1 in frequency range 2-2

| Entry index | Channel Access Type |
| --- | --- |
| 0 | Type 1 channel access defined in clause 4.4.1 of 37.213 |
| 1 | Type 2 channel access defined in clause 4.4.2 of 37.213 |
| 2 | Type 3 channel access defined in clause 4.4.3 of 37.213 |

##### 7.3.1.2.3	Format 1_2

DCI format 1_2 is used for the scheduling of PDSCH in one cell.

*** Unchanged text omitted ***

-	ChannelAccess-CPext – 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-2 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or the number of entries in the high layer parameter ul-AccessConfigListDCI-1-1 for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 are configured by the higher layer parameter ul-AccessConfigListDCI-1-2 in frequency range 1. One or more entries from Table 7.3.1.1.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1 in frequency range 2-2

*** Unchanged text omitted ***