3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302341

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.11.3

Source:	Huawei, HiSilicon

Title:	Signal design and procedure for LP-WUS

Document for:	Discussion and Decision

Introduction

In this contribution, signal designs for LP-WUS are discussed in details and corresponding link level evaluations are provided. Functionality and procedures for IDLE and INACTIVE mode are studied as well as further considerations for CONNECTED mode.

Signal design

Modulation

Waveform generation

OOK modulation

4 options of OOK generation methods are agree for further study as shown below.

Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. - Option OOK-3: Multi-tone single-bit OOK - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view) - OOK=0 means all SCs in all segments are zero power (from base-band point of view) - FFS architecture - Option OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-1 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K - FFS modulated SCs are e.g. QAM symbols, sequences or other signals - Companies to report their assumptions - potential guard-band SCs are zero power (from base-band point of view) - [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] - Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol)

As further details based on this OOK agreement, there is one FFS bullet regarding the modulated SCs, e.g. QAM symbols, sequences, or other signals. The waveforms of the OOK modulations would impact the detected energy accuracy especially when there is timing/frequency error, as demodulation of OOK is non-coherent. Therefore, the detailed choice of OOK waveform is important.

For OOK-1 option, a sequence y  in frequency domain can be obtained based on the time domain signal x by K-point FFT, where K equals to the IFFT size of CP-OFDM. A sequence y'  is truncated from K-points y to satisfy the bandwidth restriction of LP-WUS, which is mapped to the N REs of LP-WUS. The sequence y’ could be pre-stored by gNB to produce the target waveform.

If OOK-4 option is used, the “signal generation and modification” module is used to determine the final shape of waveform of OOK symbols. As shown by the figure of Option-OOK-4 in the agreement, the DFT size is N’ and therefore the time domain sequence for each OOK ON symbol is a N’/M length sequence (assuming OFF symbol corresponds to all zero-value sequence).

The generation of OOK-2 is a simple frequency extension of OOK-1. Therefore, we shall not provide too much further discussion on OOK-2 herein.

Rectangular OOK waveform

The OOK receiver estimates the energy within an OOK symbol to demodulate the symbol, therefore a straight forward waveform design is to target a close-to-rectangular waveform, x’, as shown in Figure 1.

Figure 1:  Rectangular-based OOK waveform design

For OOK-1, the K-point FFT of an ideal rectangular time-domain sequence, obtained by an all-ones sequence, can be prestored by gNB.

For OOK-4, [2] provides a method to obtain the time domain signal x’ minimizing the least square error from x, given the WUS bandwidth constraint. A similar rectangular shape waveform x’ can also be obtained with DFT-s-OFDM, where the corresponding bit-spreading sequence before DFT precoding can be a sequence of alternating of +1 and -1, i.e. “+1, -1, ……, +1, -1” to produce the time domain signal x’ as shown in Figure 1. According to the DFT-s-OFDM generation defined in current NR specification, the zero-frequency component (after DFT precoding) is mapped to the first WUS subcarrier (before IFFT operation). In other words, a cyclic shifting of the signal spectrum is performed. By time-frequency reciprocity of the DFT, a compensating cyclic shifting of signal spectrum can be implemented in the time domain (before DFT precoding) by using an alternating sequence of +1 and -1 instead of an all-one sequence.

ZC-sequence based OOK waveform

The close-to-rectangular waveform looks good in the time-domain for energy detection, however the corresponding frequency domain power distribution has most of its power concentrated in a very narrow bandwidth. As a result, the performance of the close-to-rectangular OOK waveform degrades a lot under fading channel due to frequency-selective fading.

ZC sequence can be another choice, which also has constant-envelope in the time domain, and a more evenly distributed power over the occupied bandwidth for better performance under fading channel.  An example of ZC-based OOK waveform is shown in Figure 2.

Figure 2 ZC-based OOK waveform design

For OOK-1, if the target ZC sequence is with length X, the K-points FFT of the ZC sequence can be obtained and truncated to N samples, which is prestored and used as the inputs for OOK-1, where X can be the number of samples per OOK symbol assuming the receiver sampling rate and K is the number of the IFFT of the transmitter.

If OOK-4 is used, the length of the ZC sequence should be N’/M to match the OOK ON symbol length.

Concentrated OOK waveform

As shown in simulation results (see Section 2.1.2.1), the performance of both OOK-1 and OOK-4 is relatively sensitive to the timing error. Improving the robustness of OOK modulation against timing error is desirable for better performance. For example, if the OOK waveform has ON symbols with slightly more concentrated energy around their centers, the impact of timing error would be greatly reduced. We refer to this type of waveform design as “concentrated OOK waveform” in this contribution, and will provide corresponding evaluations. An example is illustrated as following.

Figure 3 Illustration of concentrated OOK waveform providing robustness against ISI and timing error among OOK symbols

Meanwhile, the multi-path delay may cause some inter-symbol interference, especially for OOK-4 considering there is no explicit CP between neighboring OOK pulses/segments/symbols within the same CP-OFDM symbol. The concentrated waveform of OOK symbol could minimize the ISI issue considering less energy of the delayed signal is likely to fall into the detection window of the next OOK symbol.

As a by-product of timing-error robustness is that the detection window on receiver could be adjusted shorter accordingly considering the collected energy in the detection window is still the same which results in an increase of received SNR which helps to improve the link performance. Thus, during the concentrated duration the equivalent SNR can be increased, which can help to improve the link performance.

One example of concentrated waveform could be produced by using a shorter sequence with some guard time with zero values as the sequence of OOK-4.

FSK modulation

Agreement

For M-bit MC-FSK generation study further the following options

- Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around.

  - segment comprises one sub-carrier or multiple contiguous SCs

  - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view)

- Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around.

  - segment comprises one sub-carrier or multiple contiguous SCs

  - one segment from 2^M segments is modulated, other segments of SCs are zero power (from base-band point of view)

- M >0

- N >1

- Study how to generate segment in time domain, e.g. OOK-1 or OOK-4

- Other options are not precluded.

In RAN1#112, the following agreements were reached regarding FSK modulation.

The maximal supported data rate and number of frequency segements needed for each of the two options are shown in Table 1. The exact values of M that need to be supported depends on the required data rate for a UE, which can be studied further.

Table 1 Frequency segment number required

| M | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Maximal supported data rate when SCS=30kHz | 28 kbps | 56 kbps | 84 kbps | 112 kbps |
| Number of segements for Option FSK-1 | 2 | 4 | 6 | 8 |
| Number of segements for Option FSK-2 | 2 | 4 | 8 | 16 |

For the case of M=1, the two options are exactly the same, both require 2 segments to transmit 1 bit/symbol.

For the case of M=2, both options uses 4 segments to transmit 2 bit/symbol. As shown in Figure 4, for option FSK-1, the 4 segments are divided into M=2 segment pairs. And to transmit 2 bit, each bit is conveyed using one of the pairs, therefore two segments are activated. Meanwhile for option FSK-2, only one segment of the four is activated.

Figure 4 Modulation of 2-bit MC-FSK using Option FSK-1 and Option FSK-2

The demodulation performance can be found in Figure 15 in section 2.1.2.2. Option FSK-2 shows 2.8 dB performance gain compare to option FSK-1 at 1% BLER. This can be explained by the fact that for option FSK-2, only one segment among the four is activated in a symbol. So that with the same total transmisison power, the activated segment can be boosted by 6 dB. While for option FSK-1, two segments of four are activated, so each activated segment can be boosted by 3 dB. Thus for M=2, Option FSK-2 benefits more from power boosting than Option FSK-1.

For the cases of M>2, segment number required is larger than four. Note that large number of segments may incease the receiver complexity accordingly. For FSK receiver architecture with parallel envelope detector, the number of segments equals to the number of parallel branches. It is therefore reasonable to keep M from being too high for FSK. This can be decided after the supported data rate is agreed upon.

If it turns out that M>2 needs to be supported, then direct application of Option FSK-2 results in drastic increment in receiver complexity, as the number of segments grows exponentially with M. Therefore it is suggested that Option FSK-2 can be decompose into parallel channels, each channel uses FSK with M=1 or 2. For example, M=3 can be decomposed into a 2-bit MC-FSK and a 1-bit MC-FSK, which requires 6 segments in total. M=4 can be decomposed into two 2-bit MC-FSK, which requires 8 segments in total. By this approach, the required segments number is the same as Option FSK-1. Indeed, Option FSK-1 can be considered as decomposition into parallel channels with 1-bit MC-FSK.

- For the two options for FSK signal generation, the following observations are made

- For M=1, option FSK-1 and option FSK-2 are the same

- For M=2, option FSK-2 outperforms FSK-1 due to higher power boosting

- For M>2,

  - The segments required for option FSK-1 grows linearly with M.

  - For option FSK-2, the segments required grows exponentially with M. In this case, it is suggested to decompose the transmission into parallel channels with M=1 or M=2. With this method, segments required are the same as option FSK-1.

To generate the signal for the activated segments, a sequence can be mapped to the set of contiguous subcarrier of the segment. Ideally, the generated time domain signal should have flat envelope or low PAPR. The existing sequence design such as Zaddoff-Chu may be reused. Such sequence can also be exhaustively computer searched.

Further consideration on OOK and FSK

In RAN1#112, it was agreed to further study how to generate segment in time domain for FSK [1]. If the time domain waveform for FSK is generated by the method of OOK-4, is can be regarded as a joint modulation of OOK and FSK. One example is shown in Figure 5, where the mapping from information bits to time domain waveform and frequency location is summarized in Table 2, where Manchester coding in time domain is assumed. In this example, 2 bits can be carried by one OFDM symbol. By using this joint modulation, gNB can flexibly configure the time domain waveform and the number of frequency locations, which enable a flexible modulation in both time and frequency domain. If gNB configure a single segment in time domain and multiple segments in frequency domain, then it fallbacks to normal FSK-2; if gNB configures a single segment frequency domain and multiple segments in time domain, then it fallbacks to normal OOK-4.

Figure 5 Example of joint modulation of OOK and FSK.

Table 2 Mapping from information bits to time domain waveform and frequency location for Figure 5

| Information bit | Time domain waveform | Frequency domain position |
| --- | --- | --- |
| 00 | ON-OFF | f1 |
| 01 | OFF-ON | f1 |
| 10 | ON-OFF | f0 |
| 11 | OFF-ON | f0 |

Compared with OOK-4, it has longer symbol duration which makes it more robust to timing error. And compared with FSK-2, its frequency segment is wider which provides more frequency diversity gain.

- A joint modulation of OOK-4 and FSK-2 can be more robust to timing error than OOK-4 due to its longer symbol duration.

- A joint modulation of OOK-4 and FSK-2 can have better frequency diversity than FSK-2.

As introduced in our companion paper [4], to support OOK-2 there should be multiple branches in LP-WUR, which is almost the same as the agreed parallel receiver architecture for FSK. This parallel receiver architecture can also be used by the joint modulation to enable both OOK and FSK.

- The parallel receiver architecture can be used by the joint modulation to enable both OOK and FSK.

- Study joint modulation of OOK and FSK by e.g. multiple segments in time domain for FSK.

Sequence detection of LP-WUS

As discussed in our companion paper [4], a correlation detection can be achieved with similar receiver architectures as envelope detection, e.g. IF or zero-IF based receiver architecture. When the sliding window size is small, the power consumption is still much smaller than that of deep sleep power consumption.

By utilizing the zero/low-IF architecture-based correlation detection receiver, it enables the detection of sequences with good correlation property, e.g. the ZC sequences which have been used for legacy channel in NR. The detection of the corresponding sequence can be realized by a correlator, where a time domain correlation between the local sequence and the received samples is performed within a sliding time window. The sliding window could be controlled with a small size to keep low power considering timing error/frequency error can be corrected periodically with the help of synchronization signal.

OOK and FSK may suffer from noise and inter-cell interference due to envelope-based detection, considering the energy from the serving cell, neighbor cells, and noise cannot be distinguished by envelope detection. When utilizing sequences with a good correlation property, e.g. Zadoff-Chu (ZC) sequences with different cyclic shifts or ZC sequence with different roots, the interference, including the inter-cell interference and the interference from the in-band neighboring NR legacy signal, can be reduced by correlation detection in the low-power wake-up receiver.

Figure 6 An example of transmitter block diagram of sequence-shift-keying modulation

- OOK and FSK detection can be made more robust with respect to interference and noise by modulating with sequences having good correlation properties. Sequences suitable for existing gNB transmitter, such as ZC sequences, have such properties.

Simulations are provided in section 2.1.2.3, and it shows the significant performance gain compared with normal envelope detection. As observed in Observation 5, using sequence for LP-WUS signal can significantly improve the detection performance of LP-WUS. Furthermore, if multiple sequences are modulated with LP-WUS signal, more information bits can be carried. An example is illustrated in Figure 6, where four sequences are used to modulate 2-bits within each OFDM symbol.

- Study carrying information via different sequences or cyclic shifts for the OFDM based signal/channel for LP-WUS.

As illustrated in Figure 6, difference sequences or the same sequence with difference cyclic shifts in time domain can be used to carry information bits. DFT-s-OFDM signal process could be reused to transmit difference sequences on different OFDM symbols. As another alternative, gNB could pre-store the corresponding frequency response for each candidate sequences, and in this case the on-the-fly DFT processing is not needed. It can be observed that existing CP-OFDM modulation hardware can be reused.

Link level simulations

Agreement - When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least - raw information bit-size - [time/frequency resources (including any guard bands), if applicable] - [total energy of LP-WUS across the time/frequency resources] - FFS: false alarm probability/rate - FFS: misdetection probability/rate are kept [comparable or fixed]. Study at least - impact of timing error - impact of frequency error - impact of phase noise and I/Q imbalance, if applicable - impact of ADC resolution and sampling rate - impact of interference - impact of delay spread - impact of doppler spread - Companies to report - how they modelled SINR - time/frequency resources (including any guard bands) for the scheme - false alarm probability/rate and misdetection probability/rate - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms - When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additional hardware for WUS.

In RAN1#112, the above agreement was made [1].

First, to make fair comparison it is reasonable to use equal time/frequency resources. Regarding the comment raised by some companies during RAN1#112 that it is difficult to align the time/frequency resources for different schemes, in the following we give some examples for the alignment, where it is assumed that the reference BW of a LP-WUS is α a, reference transmission time for a LP-WUS is β.

- For OOK-1 with Manchester coding, the BW is α4, and transmission time is 4β,

- For OOK-2 with Manchester coding, the BW of each segment in frequency domain is α4 (thus the total BW is α), and the transmission time is β.

- For OOK-4 with Manchester coding, the BW is α, and the transmission time is β

- For FSK-1, there are two pairs of segments in frequency domain, and the BW of each segment in frequency domain is α4 (thus the total BW is α), and the transmission time is β.For FSK-2, there are four segments in frequency domain, and the BW of each segment in frequency domain is α4 (thus the total BW is α), and the transmission time is β.

- When including sequences, the BW is α and the transmission time is β, where in each OFDM symbol there are four candidate cyclic shift values to carry 2 bits.

Second, it is also reasonable to make equal the total energy of LP-WUS. In our simulation, the SNR is modelled as the ratio of the signal power to the noise power within the BW of a LP-WUS, where the BW of a LP-WUS can refer to the above examples. To align the total energy of LP-WUS, when there are resources with zero power, the resources with non-zero power should be power-boosted. For example, if the average signal energy in one OFDM symbol within the BW of a LP-WUS is E0, then the following are the examples for the energy alignment.

- For OOK-1 with Manchester coding, the average signal energy in one OFDM symbol within BW = α4 is E04, but due to the half ON/OFF symbol distribution, the signal energy of ON symbol within BW = α4 is E02.

- For OOK-2 with Manchester coding, the signal energy in one OFDM symbol for each frequency segment is E04, due to the half ON/OFF symbol distribution, the signal energy of ON symbol for each frequency segment is E02.

- For OOK-4 with Manchester coding, the signal energy in one OFDM symbol for the BW is E0, and due to the half ON/OFF symbol distribution, the signal energy of ON symbol for the BW is E02.

- For FSK-1, the signal energy in one OFDM symbol for the frequency segment with non-zero power is E02, since in each OFDM symbol, there is only two frequency segments (one segment in each pair) with non-zero power.

- For FSK-2, the signal energy in one OFDM symbol for the frequency segment with non-zero power is E0, since in each OFDM symbol, there is only one frequency segment with non-zero power.

- For code-shift-keying modulation, the signal energy in one OFDM symbol for the BW is E0.

OOK modulation

In this subsection we investigate the performance of Option OOK-1, Option OOK-2 and Option OOK-4 and the impacts from ADC sampling rate, timing error and frequency error.

Companies are requested to report the FAR applicable to their evaluations. In this case, the FAR is related to the length of the paging ID and demodulation method. Based on our calculation, with a paging ID of 48 bits, as simulated here, the false alarm rate is lower than 10-15.

- Simulation set up

To align the time-frequency resource and energy used, we choose the following set up as default for comparison. We assume a wake-up message of 48 bits. The bandwidth allocation for the three options are shown in the figure below.

In the evaluations we assume SCS of 30kHz. LP-WUS transmission is assumed to be FDM with legacy NR signals. For legacy NR signals, we modelled random data transmissions using 16QAM.

Between legacy NR signals and LP-WUS, two guard bands, each with 12 SC are allocated, one on each side of LP-WUS channel.

For Option OOK-1, we assume 1RB is used for LP-WUS transmission. Since we also assume Manchester code is used, total 96 OFDM symbols are needed to transmit 48bit message.

For Option OOK-2, we assume 4RB, i.e. 48 subcarriers are used for LP-WUS transmission. M=4 segments are assumed. Each segment uses 9 SC for OOK modulation, and there are 4 SC zero power SC allocated between adjacent segments for protection. As Manchester code is used, each OFDM symbol can transmit 2 information bits so that 24 OFDM symbols are needed to transmit 48bit message.

For Option OOK-4, we evaluate M=4bit OOK transmission. 4RB, i.e. 48 subcarriers are used for LP-WUS transmission. As Manchester code is used, each OFDM symbol can transmit 2 information bits so that 24 OFDM symbols are needed to transmit 48bit message.

If the two RB guard band is not considered, then the three options use same amount of time-frequency resources and the total energy used for transmission is also aligned. However, it is not feasible to align the data rate, i.e. Option OOK-1 provides data rate of 14kbps while it is 56kbps for Option OOK 2/4.

Other simulation assumptions can be found in Appendix.

Figure 7 Bandwidth configurations for evaluation (Option OOK-1/2/4)

Firstly, baseline performance comparison of different OOK options is shown in Figure 8, temporarily neglecting non-ideal factors such as timing/frequency error. ADC sampling rate is assumed to be 3.84 MHz with digitize resolution of 8 bit/sample.

Figure 8 Baseline performance comparison of different OOK options

From the left figure, it can be observed that Option OOK-4 provides best performance among the three, it outperforms Option OOK1/2 by 3.8dB and 4.8 dB respectively. The gap can be explained by higher frequency diversity gain harvest by wider transmission bandwidth. Although correspondingly, Option OOK1/2 should have higher time diversity gain due to longer symbol duration, still for low mobility, the time diversity gain is marginal compare to frequency diversity.

Option OOK-1 and OOK-2 have same configuration of bandwidth and symbol duration. From Figure 8 it can be seen that Option OOK-1 outperforms Option OOK-2 by 1 dB. The difference is due to the fact that Option OOK-2 need to support parallel LP-WUS channels with narrow protection subcarriers, and inter-channel interference cause performance degradation.

In addition, we evaluate the three options with wider bandwidth allocations, as shown in the right figure. For all three options, two guard bands of 1RB are allocated on both sides. For OOK-1, 3 RBs are used for signal transmission. For OOK-2, totally 12 RBs are used for transmission which are divided into 4 segments, each segment uses 33 subcarriers, and four zeros power subcarriers are placed between neighbor segments for protection. For OOK-4, 12 RBs are used for transmission. It can be seen that all three options show improved performance. This is because wider bandwidth brings higher frequency diversity gain. Still, OOK-4 shows best performance, and OOK-1 outperforms OOK-2 by 0.9 dB at@1% BLER level.

- Impacts from ADC sampling rate

To achieve low power consumption, an LP-WUS receiver can use ADC with low sampling rate and low quantization resolution. In our previous contribution, we show that quantization resolution of 4 bit/sample provides comparable performance to the ideal cases. In this section, we focus on the impacts from ADC sampling rate.

From Figure 9, we evaluate three sampling frequency, i.e. 3.84MHz / 1.92 MHz / 960 kHz. It can be seen that Options OOK-1 and OOK-2 are insensitive to sampling rate reduction. For Option OOK-4, while 3.84MHz and 1.92MHz ADC have the same performance, further reduction to 960kHz cause 2dB performance loss.

Such performance loss is caused by the shortened symbol length. Since M=4-bit OOK is assumed for Option OOK-4, the symbol duration of TS=TOS4≈8.3 μs, the symbol rate is 112 kHz. For the case of 960 kHz sample, only eight samples are taken per OOK symbol. When OOK signal is generated using multiple subcarriers there can be some valleys within the ON symbol envelope. And if multiple samples are taken at signal valleys it prone to error. A lower sampling rate results in more such errors.

- Options OOK-1 and OOK-2 experience marginal performance loss when ADC sampling rate is reduced to 960 kHz. Option OOK-4 sees 2 dB performance loss when sampling is reduced from 1.92MHz to 960kHz.

Figure 9 Impacts of ADC to OOK demodulation

- Impacts from timing error

Another important non-ideal factor to consider is timing error. This is mainly due to accumulated timing error caused by an inaccurate LO. Since the agreed simulation assumptions assumes frequency error of 50 or 200 ppm, the receiver accumulates large timing error in a short time span. For example, with a frequency error of 50 ppm, the receiver accumulates timing error of 1 μs every 20 ms. Therefore, the resilience against timing error is crucial for the modulation scheme.

| Option OOK-1 | Option OOK-2 |
| --- | --- |
| Option OOK-4 (TDL-C 300ns) | Option OOK-4 (TDL-C 1000ns) |

Figure 10 Impacts from timing error to different OOK options

The impacts from timing error on different OOK options are provided in Figure 10. Timing error of 1, 2, 4 μs are imposed in the simulations. And we also evaluate delay spread of both 300ns and 1000ns. The following observations can be made from the evaluations.

- For Option OOK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.5, 1.6 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 1.4 dB respectively.

- For Option OOK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.2, 0.4, 0.5 dB respectively.

- For Option OOK-4 with channel model of TDL-C 300ns, the performance loss caused by 1, 2 μs timing error are 2.5, 4.8 dB respectively. Timing error of 4 μs causes error floor. For channel model of TDL-C 1000ns, the performance loss caused by 1 μs timing error is 2.7 dB. Timing error of 2 μs causes error floor.

Compare to OOK-1 and OOK-2, demodulation performance of OOK-4 is much more sensitive to timing error. Especially when the delay spread is large. Similar to the impacts from ADC, this can be explained by the short symbol duration of Option OOK-4. In the evaluations, with symbol duration of TS=TOS4≈8.3 μs, timing errors of 1 or 2 μs is already significant and thus brings performance degradation.

- Impacts from frequency error

We evaluate the case of frequency offsets of 10 and 20ppm on Option OOK-1, OOK-2 and OOK-4, and are shown in Figure 11 below.

It can be observed that the performance loss caused by frequency offset is marginal, this is because the assumed OOK receiver performs envelope detection, and signal envelope is not affected by frequency offset. Also, there is enough guard band to cover the frequency offset as shown in Figure 7, so that adjacent channel interference doesn’t cause significant performance loss.

Figure 11 Impacts from frequency offsets to OOK demodulation

- If the guard bands can cover frequency offset, Options OOK-1, OOK-2 and OOK-4 are insensitive to frequency offset.

- Impact of OOK generation sequences/waveforms

Simulation are performed to compare the performance with respect to the difference OOK waveforms. Figure 12 shows the performance to compare the performance of close-to-rectangular waveform and ZC sequence based waveform. It can be observed that the performance of ZC sequence-based waveform outperforms much than that of close-to-rectangular waveform under fading channel. This is because close-to-rectangular waveform has a frequency domain power distribution with most of its power concentrated in a very narrow bandwidth. Therefore, it shall be impacted a lot by frequency-selective fading.

Figure 12  Performance to compare the performance of close-to-rectangular waveform and ZC sequence-based waveform

- ZC sequence-based generation of FSK/OOK waveform outperforms significantly the OOK modulation with rectangular waveform in fading channel.

Figure 13 normal ZC based waveform vs. concentrated waveform

Figure 13 compares the performance with normal sequence based OOK waveform where the sequence length matches the OOK symbol duration, and the concentrated OOK waveform. The concentrated OOK uses a shorter sequence, here a shorter ZC sequence appended with two zero values on each side, all together matching an OOK symbol duration. It can be observed firstly that the concentrated waveform provides a 0.5 dB gain for ideal timing case. This is because that the ISI can be better resolved by using the concentrated OOK.

- For OOK-4, concentrated OOK provides0.5dB gain for ideal timing case compared with the normal sequence-based waveform because ISI between OOK symbols are mitigated

Secondly, it can be observed that the concentrated waveform has very robust performance with respect to the timing error, while here the normal ZC sequence-based waveform degrades by 4-5 dB under 2us timing error. This is due to the fact, as shown in Figure 3 that concentrated energy enables more accurate estimation of energy detection with respect to timing error.

- Concentrated OOK significantly improves robust against timing error, e.g. at least 2dB improvement under 2us timing error.

Furthermore, it worth remarking that while the concentrated waveform was applied here to OOK-4 with ZC sequence, it could be applied in general to other OOK and FSK modulations to improve their respective performance.

- Concentrated waveform is applicable to OOK and FSK to improve their respective performance.

FSK modulation

In this subsection we investigate the performance of Option FSK-1 and FSK-2 and the impacts from ADC sampling rate, timing error and frequency error.

- Simulation set up

Similar to OOK evaluations, we assume a wake-up message of 48 bit. The default bandwidth allocation for the two options are shown in the figure below. Legacy NR signal and LP-WUS are FDMed, with 12 subcarriers in between as guard band.

For both Option FSK-1 and FSK-2, 4RB is used for signal transmission, i.e. 48 subcarriers are used for LP-WUS transmission. 4 segments are used and each segment uses 9 SC for signal modulation. 4 SC zero power SC are inserted between adjacent segments for protection. Both options can transmit 2 information bits per OFDM symbol so that 24 OS are needed to transmit 48bit message.

Such configurations align the time-frequency resources and the total energy used for transmission with Option OOK-1, OOK-2 and OOK-4. The data rate is also aligned with OOK-2 and OOK-4 for a fair comparison.

Other simulation assumptions can be found in Table 5 of Appendix.

Figure 14 Bandwidth configurations for evaluation (Option FSK-1/2)

Firstly, baseline performance comparison of different FSK options is shown in Figure 15, temporarily neglecting non-ideal factors such as timing/frequency error. ADC sampling rate is assumed to be 3.84 MHz with digitize resolution of 8 bit/sample.

Figure 15 Baseline performance comparison of different FSK options

For bandwidth of 1.44MHz, it can be observed that Option FSK-2 outperforms FSK-1 by 2.8 dB, this is due to higher power boosting gain from FSK-2, as already discussed in 2.1.1.2.

In addition to the default bandwidth configuration, we also evaluate the two options with wider bandwidth, i.e. 4.32MHz (12 PRBs). It can be seen that both options show improved performance with wider bandwidth, due to higher frequency diversity gain. Similar to the 1.44MHz case, FSK-2 outperforms FSK-1 by 2.8 dB.

The false alarm rate is less than 10-15, following the same analysis from section 2.1.2.1.

- Impacts from ADC sampling rate

Three sampling frequencies, i.e. 3.84MHz / 1.92 MHz / 960 kHz are evaluated and the results are shown in Figure 16. It can be seen that both Options FSK-1 and FSK-2 are not affected even when sampling rate is reduced to 960kHz. This is because the FSK symbol rate is only 28 kbps, 960kHz sampling rate provides x34 over-sampling, which is more than enough.

- Both FSK-1 and FSK-2 sees no performance loss when ADC sampling rate is reduced to 960 kHz.

Figure 16 Impacts of ADC to FSK demodulation

- Impacts from timing error

The impacts from timing error on different FSK options are provided in Figure 17. Timing error of 1, 2, 4 μs are imposed in the simulations. And we also evaluate delay spread of both 300ns and 1000ns. The following observations can be made from the evaluations.

- For Option FSK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 2 dB respectively.

- For Option FSK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.5, 1.7 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.6, 2.4 dB respectively.

Figure 17 Impacts from timing error to different FSK options

- Impacts from frequency error

We evaluate the case of frequency offsets of 10 and 20ppm on Option FSK-1, FSK-2, and the results are shown in Figure 18.

It can be observed that the performance loss caused by frequency offset is marginal, this is because the assumed FSK receiver performs envelope detection and not is affected by frequency offset. The allocated guard bands both in between and around provide enough protection so that frequency error of 20 ppm has only marginal performance impacts.

Figure 18 Impacts from frequency error to FSK demodulation

- If the guard bands can cover frequency offset, Options FSK-1, FSK-2 are insensitive to frequency offset.

Sequence detection of LP-WUS signal

The performance of modulating LP-WUS signal, e.g. OOK/FSK with sequences is evaluated by link level simulation with the following simulation assumption in Table 6 and Table 7 in the Appendix. In this section, for each 48-bits UE-ID transmitted in 24 symbols, the transmitter transmits the sequence corresponding to the 2-bits on each symbol. The receiver detects the target sequence corresponding to the 2bits of its UE-ID on each symbol.

In Figure 19, simulation results under ideal cases without phase noise and I/Q imbalance are provided under both FAR targets of 0.1% for single attempt and 1500 attempts of continuous monitoring within 1.28s DRX cycle. The simulation evaluated different effective signal bandwidth, i.e. 1.44 MHz and 3.6 MHz with 4 cyclic shifts detection.

Figure 19 Link level performance of modulating LP-WUS signal, e.g. OOK/FSK, with sequences under ideal cases

As shown in Figure 19, the performance can be improved by 4 dB with effective signal bandwidth increased by from 4RB to 10RBs. And the 4RB sequence-based detection performance is much better than that of energy detection based OOK.

Some comments were raised with respect to the performance impact due to non-ideal factors, e.g. phase noises and I/Q imbalance, when the discussed LP-WUS architectures are used. The modeling of phase noises can be found in our companion paper [3] and in [5].

In this evaluation, we choose power consumption of LO as 120uW and find c=0.24fs to make phase noise equals to minimum phase noise through simulation results. 2RB LP-WUS seems have too large gap compared with the NR legacy channel coverage, therefore, we will focus on the analysis on 4RB and 10RB LP-WUS. As seen in Figure 20(a), when BLER=1%, the performance losses are marginal for 10RB and 4RB, respectively. Therefore, although sequence-based detection performance can be impacted by phase noise, the performance loss is within 1dB since the cyclic shift space between different candidate sequences is relatively large.

In Figure 20(b), the result shows that the impact of I/Q impact is so marginal that it can be neglected. The reason is that factory calibration has been applied to most hardware, which can also be applied to LP-WUR.

| (a) | (b) |
| --- | --- |

Figure 20 Link level performance of sequence-based detection considering (a) phase noise and (b) I/Q imbalance

- For sequence-based detection, it can be observed that the performance would be degraded by 0.5dB due to the phase noise and I/Q imbalance when 120 uW oscillator is used.

The performance impact due to the frequency error and timing error are also investigated. In the simulation, the ZC sequence with root u value of 1 is simulated. Considering there is a small sliding window for the correlation-based receiver, the timing error in certain range can be handled when the detection window slides.

The frequency error may also have impact on the detection performance of sequence detection of LP-WUS signal. Therefore, the performance impact due to frequency error can be further investigated. It can be observed within 10ppm frequency error, the performance degradation is about 1dB.

Figure 21 Link level performance of sequence-based detection considering (a) frequency error and (b) timing error

- Sequence-based OOK/FSK modulation has better coverage performance than energy detection, even if phase noise and I/Q imbalance, frequency error within 10ppm and 1us timing error are considered.

Summary of modulations and further consideration

- Summary and further consideration for OOK

As can be observed in section 2.1.2.1, OOK modulation has some advantages: 1) it is insensitive to frequency offset and phase noise, 2) it can minimize the frequency resource cost from network side to support LP-WUS transmission, 3) for OOK-1 and OOK-4, the single-channel envelope detection mode can be naturally suitable for LP-WUR power saving.

The time error impacts should also be further considered in the OOK signal generation since time error will degrade the detection performance of OOK and the robustness of anti-time error capability is essential to OOK waveform.

- Summary and further consideration for FSK

Three major benefits from FSK are 1) the resilience against inter symbol interference (ISI) caused by timing error and channel delay spread, 2) its capability to correct LO frequency/timing error and 3) relaxed requirement on ADC sampling rate.

FSK is robust against timing error due to its relatively long symbol duration. As FSK achieve higher data rate by exploiting frequency domain resources, the time domain symbol duration is kept the same as OFDM symbol. As can be observed from Figure 17, timing error up to 2 μs has negligible impacts on FSK demodulation performance, when the FSK symbol duration is around 36 μs (i.e. OFDM symbol duration for 30kHz SCS). This advantage is more evident for situations that experience large delay spread. This can be observed by comparing Figure 10 and Figure 17. The robustness against timing error allows FSK receiver to perform timing alignment less frequently and therefore reduces power consumption.

Another benefit from FSK is its capability to correct frequency/timing error. Frequency offset correction using FSK receiver is explained extensively in [4]. Once the frequency error is corrected, the timing error is also accumulated at a much lower rate. Therefore, not only are smaller guard bands sufficient, but also the timing error is greatly reduced.

FSK is also more resilient against ADC sampling rate relaxation. This is demonstrated by Figure 16. Again, this is due to its long symbol duration, and correspondingly, lower symbol rate. Even ADC sampling rate as low as hundreds of kHz provides enough oversampling margin.

- Summary and further consideration for sequence-based modulation/detection

As can be observed in section 2.1.1.3, this has some advantages: 1) Good coverage performance, considering sequence with good correlation property can have better rejection effect with respect to noises and interference within certain frequency/timing error; 2) By using a complex-valued sequence, it is natural that it is feasible to estimate the frequency error and timing error.

From the evaluation results, it can be observed that for root u=1 with 10ppm frequency error the performance degradation is small (around 1dB). Considering there is a small sliding window for the correlation-based receiver, the timing error in certain range can be handled when the detection window slides. This is proven by our simulation that 1us timing error shall only cause 0.7dB performance loss.

Further consideration on the design of LP synchronization signal may be studied for frequency/timing error estimation/correction.

Coverage

The coverage performance of LP-WUS can be limited due to a large receiver noise figure, and non-coherent detection. To improve the coverage performance of LP-WUS there are several ways:

- Power boosting. The transmitted power of LP-WUS can be power boosted to compensate the coverage gap between LP-WUS and target NR channel. But depending on gNB implementation, it is not always possible for LP-WUS to get power boosting gain.

- Channel coding. Channel coding is a more efficient way to improve the detection performance compared with simple repetition. More analysis can be found in Section 2.2.1.

- Time domain diversity. Time-domain repetition or interleaving can be considered for LP-WUS to utilize time diversity. There are also different methods to exploit time diversity. For example, a packet can be transmitted on non-contiguous timing slots, that span over a time duration longer than the coherent time. Providing channel coding (can be as simple as repetition code) is used, more timing diversity gain can be obtained.

- Frequency domain diversity. Frequency-domain repetition or frequency hopping can be considered to utilize frequency diversity. Frequency diversity can be exploited in different ways, the simplest approach is to expand the signal bandwidth to be larger than the radio channel’s coherence bandwidth. Such method is also receiver friendly, as it doesn’t require extra efforts at the receiver side. However, from system perspective, it is not an economic way to exploit frequency diversity. A more resource efficient solution is to introduce mechanisms such as frequency hopping. For example, a encoded of LP-WUS message can be divided into sections are each sent at a different frequency, each occupying a narrow bandwidth. It is expected such mechanism can obtain frequency diversity with less resource consumption compared to bandwidth expansion. Meanwhile, it does rely on channel coding, which may lead to additional power consumptions.

- Space domain diversity. For example, if multiple receive antennas can be used for LP-WUS, spatial diversity can be utilized by e.g. antenna selection.

- Study how to improve the coverage performance of LP-WUS.

Considerations on channel coding

Due to the stringent requirements on receiver power consumption, it is unrealistic to adopt advanced FEC codes as they typically have high decoding complexity. Nonetheless, LP-WUS transmissions have a much lower data rate compared to the transmissions for the main radio. It means that decoding is performed much less frequently compared to the main radio. Also, the required decoding delay can be relaxed, since it has a marginal contribution to the LP-WUS delay budget. Therefore, it is reasonable to expect that the power consumption of a LP-WUS decoder can be lower than its main radio counterpart by several orders of magnitude.

FEC methods with low decoding complexity can be considered for LP-WUS. For example, codes with minimized decoding iterations, polar codes, or block codes including identification codes based on [6], for which simple hard decoding algorithms are available. It is well known that it is necessary to study the tradeoff between the code-rate, i.e. amount of redundancy to transmit, and the coverage extension an FEC offers.

- Study methods to extend coverage and reduce misdetection probability by block coding.

LP-WUS multiplexing

Since the gNB may need to wake-up multiple UEs, or multiple groups of UEs, LP-WUS multiplexing needs to be supported, and the following methods can be considered, separately or jointly:

Time Division Multiplexing (TDM): The LP-WUSs are transmitted on the same PRBs but on orthogonal time resources. The benefits include simple demultiplexing of LP-WUSs at the UEs, the applicability to OOK/FSK modulation and to sequence modulation and no need for guard bands among LP-WUSs.

Frequency Division Multiplexing (FDM): The LP-WUSs are transmitted on the same time resources but on orthogonal PRBs. The benefits are similar as for TDM, although guard bands and multiple RX filters may be needed.

Code Division Multiplexing (CDM): The LP-WUSs are transmitted on the same time resources and on the same PRBs. CDM is also applicable to sequence modulation and to OOK/FSK modulation, and may be an advantageous channel access method if there are many infrequent low-rate transmissions, e.g., as is expected for LP-WUS.

For sequence modulation, CDM of LP-WUSs can be performed by letting orthogonal sequences carry modulation symbols, and this approach was used for the WUS in LTE, which is based on OFDM. For OOK/FSK modulation, CDM of LP-WUSs can be performed by operations directly on the bits before the modulator. Suppose C is an N×M binary matrix N≥M and the M×1 binary vector b contains bits of LP-WUSs of different UEs. For example, the vector b can either contain the bits of the UE IDs (in case of no FEC), or contain FEC encoded bits of the UE IDs. The N×1 binary vector y=Cb mod 2 contains the multiplexed bits, which are transmitted by the OOK/FSK modulator. Demultiplexing is performed on the received bits that are output from the demodulator. For orthogonal multiplexing, C should have rank equal to M, because then a UE can perfectly demultiplex the bits of its LP-WUS.

As an example, Figure 22 shows a comparison between CDM and TDM with repetition coding for 2 users, each transmitting a UE ID of 48 bits, i.e., M=96 and in total N=384, 768, 1536 bits are transmitted. It can be observed that TDM is favorable for the larger N and CDM is favorable for the smaller N.

Figure 22. Mis-detection rate after demultiplexing for a UE ID of 48 bits as function of the BER at the demodulator output, for 2 multiplexed users using CDM or TDM, where in total N bits are transmitted.

- For multiplexing of LP-WUSs for different UEs, study at least TDM, FDM, CDM.

Functionality and procedures for IDLE/INACTIVE mode

Information carried in LP-WUS

Paging information

For IDLE/INACTIVE UEs, the most important information to be received is paging information. Hence it is natural that paging information can be carried by LP-WUS. For example, when a LP-WUS carrying paging information related to UE1 is received by UE1, inside UE1 the low-power wake-up receiver can trigger the MR to perform normal NR procedures.

When such information is carried by LP-WUS, it can replace legacy paging by indicating per-UE information, then after MR wakes up UE does not receive legacy paging by MR, but transmits PRACH directly after obtaining synchronization. Or, LP-WUS can be jointly used with legacy paging, e.g. LP-WUS may indicate per-group information like Rel-17 PEI, then after MR wakes up UE still receives legacy paging by MR to see whether it is really paged.

- At least paging information is carried in LP-WUS, where LP-WUS can indicate per-UE information or per-group information.

The target data rate of LP-WUS can be set to support the specific functionality. For indicating the paging information, data rate required by legacy NR paging procedures can be taken as the reference. In legacy NR paging, with the densest PO configuration there can be at most 4 POs in a PF, and at most 1 PF per radio frame. Within a PO, there can be up to 32 UE_IDs carried by paging PDSCH, where a UE_ID is 48-bit 5G-S-TMSI. The maximum required data rate for paging is 32*48*4 bit/10ms = 614.4kbps. However, the paging capacity configured by gNB in real deployment is usually lower than the upper bound. According to our evaluation, to meet the real paging requirement, the target data rate can be x101~x102 kbps level.

- The target data rate for LP-WUS design can be x101~x102 kbps level.

Some evaluation results on power saving gain regarding LP-WUS carrying per-UE or per-group information are provided in our companion paper [3]. It can be observed that there is trade-off between power saving gain/latency and required data rate. Per-UE indication can provide larger power saving gain and smaller latency than per-group indication, but requires higher data rate which may degrade the coverage performance of LP-WUS.

- To indicate paging information by LP-WUS, there is trade-off between power saving gain/latency and required data rate/coverage performance.

According to current spec, in IDLE/INACTIVE mode paging is transmitted by network within a tracking area/RAN area, where 5G-S-TMSI, the length of which is 48-bit, is used as the UE ID. UE will perform tracking area/RAN area update procedure when the tracking area/RAN area changes, thus as long as an ID carried by LP-WUS is unique among the UEs within a tracking area/RAN area it can be per-UE indication. A 48-bit UE ID provides a 248≈3*1014 address space, which is highly redundant for paging. Therefore, some enhancements to reduce the number of indicated bits can be considered. For example, by some method to eliminate the redundant to reduce the bit length while strive to minimize the false wakeup rate.

- Further study how to indicate paging information to get a good balance between low data rate requirement and low false wakeup rate.

Other information

Since paging is transmitted within a tracking area/RAN area, when UE moves to new tracking area/RAN area, network may not be able to reach the UE. On the other hand, configuration of LP-WUS can be different for different cell. Therefore, it is necessary to let the UE know the tracking area/RAN area/cell change when the UE monitors LP-WUS. One method to solve this issue is to indicate tracking area/RAN area/cell information by LP-WUS.

Besides, to reduce the latency, SI change and ETWS/CMAS information can also be indicated by LP-WUS.

- The following information can be indicated by LP-WUS in addition to paging information:

  - Tracking area/RAN area information

  - Cell information

  - SI change and ETWS/CMAS information

To reduce the signaling overhead, such kind of information is not necessary to be carried in every LP-WUS for paging. Instead, it can be transmitted with large periodicity.

Monitoring behavior of LP-WUS

In RAN1#112, the following agreement was made.

Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

First, it is important to clarify the understanding of the two options. When LP-WUR monitors LP-WUS, it first receives all the sampling points in a duration of one LP-WUS, then performs demodulation. Therefore, per our understanding, continuous monitoring does not necessarily mean to attempt demodulating LP-WUS at every sampling point. On the other hand, in order to reduce the FAR, attempting demodulating LP-WUS at every sampling point should be avoided. One possible way is to define the slot format for LP-WUS. As shown in Figure 23, if no slot format is defined UE may attempt to demodulate LP-WUS at each sampling point. However, if slot format is defined and the starting position of LP-WUS is restricted to be the starting sampling point of a ‘WUR slot’, the number of attempts can be reduced, as well as the total FAR. In this case, LP-WUR still need to continuously receive every sample, so it is also continuous monitoring since the LP-WUR is ON all the time. The slot format can also be used for the case of duty-cycle based monitoring.

Figure 23 Example of defining slot format for LP-WUS

- Defining slot format for LP-WUS is helpful to reduce the FAR, and thus increase the power saving gain.

- Study the slot format of LP-WUS.

Based on the simulation results provided in our companion paper [3], duty-cycle monitoring can introduce larger latency than legacy baseline scheme, which is not beneficial for latency sensitive traffics. Therefore, in our view, at least continuous monitoring of LP-WUS should be supported. After that, duty-cycle monitoring can be further studied on e.g. how to reduce the latency impact, and configured if necessary.

- At least continuous monitoring of LP-WUS should be supported.

Synchronization

Synchronization of LP-WUR

Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2

According to the above agreement made in RAN1 #112 [1], low power synchronization signal (LP-SS) should be further studied. In our view, periodic signal transmitted separately from LP-WUS can provide reliable resource for time/frequency tracking, as well as for RRM measurement. Therefore, at least option 2 should be supported. After that, we can further discuss whether aperiodic signal transmitted as part of LP-WUS (i.e. option 1) is needed in addition.

- At least support periodically transmitted LP-SS for LP-WUS/WUR.

While LP-SS is transmitted, it should also be discussed how the LP-WUR to perform time/frequency error estimation and correction. For time error, the LP-WUR can perform correlation operation in time domain, then determines the correct timing based on the peak of the correlation. It is feasible for any modulation type and the corresponding receiver.

However, for frequency error, specific receiver architecture may be required. For example, parallel OOK/FSK receiver (for OOK-2 or FSK) has parallel branches in frequency domain, which enables LP-WUR to compare the power difference for different frequency location and thus enables frequency error estimation. The basic idea is shown in Figure 24, where there are two branches in LP-WUR and the reference signal for frequency error estimation can be transmitted at the middle of the two frequency locations. If the center frequency is not aligned between LP-WUR and gNB, the energy received by each branch will be imbalanced. Then LP-WUR can estimate the frequency error based on the energy difference of the two branches. Receiver for sequence-shift-keying modulation can acquire the phase information, which can be used for frequency error estimation. But for the (non-parallel) OOK receiver, it is not clear how to do frequency error estimation and correction.

Figure 24. An example of frequency error estivation by parallel OOK/FSK receiver

- Parallel OOK/FSK receiver architecture is capable to do frequency error estimation and correction.

Based on the method in Figure 24, the simulation results are shown in Figure 25, where the initial frequency error = 10ppm (30kHz @3GHz center frequency). At different SNR level, the residual CFO varies. If the working point of LP-WUS (e.g. 1% MDR) is within the range of 5dB~15dB and if the initial CFO is 10ppm, after the synchronization signal reception with 95 probability the residual frequency error can be within ±2.5 ppm.

Figure 25. Residual frequency error after synchronization for parallel OOK/FSK receiver

For periodic LP-SS (i.e. option 2), the periodicity can be determined by the time error and frequency error requirement. If we assume that after a synchronization signal the residual time error is 0 us and the residual frequency error is 5ppm (with some margin compared with 2.5ppm shown in Figure 25), and the target time error is 2us and the target frequency error is 10ppm, based on the agreed Model 1 for time/frequency error, the required periodicity is analyzed in Table 3. The result shows that to satisfy the time error requirement the periodicity of LP-SS can be ~400ms, and to satisfy the frequency error requirement the periodicity of LP-SS can be 50s. if a single LP-SS is designed for both time and frequency estimation/correction, the smaller value (i.e. ~400ms) should be used. Therefore, it is observed that the requirement of periodicity is dominated by the time error, since according to the model time error grows linear to frequency error even with fixed frequency error. And for the time error, residual frequency error contributes much more than the frequency drift since the drift value is small.

Table 3 Required periodicity of LP-SS

| Assumed residual time and frequency error | Target time or frequency error | Required periodicity of LP-SS |
| --- | --- | --- |
| Time error = 0us Frequency error = 5ppm | Time error = 2us | 398.4ms |
|  | Frequency error = 10ppm | 50s |

- The requirement of periodicity is usually dominated by the time error, where the residual frequency error contributes most of the time error.

- Further study the periodicity of LP-SS based on the requirement of max time error and/or frequency error.

One concern to support option 2 is that periodic LP-SS seems to be an ‘always-on’ signal, which may lead to resource overhead. However, on the one hand, according to the above analysis the periodicity can be hundreds of ms level. If a LP-SS is with a length of 1ms and a BW of 5MHz, the overhead of LP-SS is only ~0.01%, which is quite small. On the other hand, gNB can also configure the resources used for LP-WUS (including LP-SS), thus the resource overhead can be fully controlled to be even less than 0.01%.

- The resource overhead of periodic LP-SS is marginal for the following reasons: 1) based on the requirement of time/frequency estimation, the overhead can be low to ~0.01%, and 2) the overhead can be reduced by gNB configuration.

Re-sync of MR

When UE monitors LP-WUS by LP-WUR, the MR stays in ultra-deep sleep to save power. When MR wakes up from ultra-deep sleep, the time and frequency synchronization is completely lost. So, MR has to spend a long time to perform re-sync procedure, which increases both power consumption and latency. To reduce the effort for re-sync, LP-WUR can assist the re-sync procedure of MR. Specifically, if the symbol/slot/frame structure of LP-WUR has some relationship with the symbol/slot/frame structure of MR, the LP-WUR can get timing information based on LP-SS, and send the information to MR for initial timing calibration. Then the number of SSB used by MR can be reduced since the coarse timing is already maintained by MR.

Similarly, LP-WUR can also estimate the frequency offset by LP-SS. If the frequency location of LP-WUS and that of SSB have special relationship, the estimated frequency offset by LP-WUR can be indicated to MR for initial frequency calibration. Then the residual frequency error can be small enough to be adjusted within a short time.

Some simulation results are provided in our companion paper [3].

- If there is time and frequency relationship between LP-WUR and MR, then LP-WUR can assist the re-sync procedure of MR, which can reduce the power consumption and latency.

Measurements and mobility

The RRM measurement includes serving cell measurement and neighbor cell measurement. Since low-power wake-up receiver will anyway receive signals from the serving cell, serving cell measurement can be performed together with normal LP-WUS/LP-SS reception. For example, a dedicated reference signal for measurement, e.g. the LP-SS, may be used.

- At least serving cell measurement is supported by LP-WUR using LP-WUS/LP-SS.

If RRM measurement can be performed by LP-WUR, it should be discussed how to define the measurement quantity (e.g. LP-RSRP, LP-RSSI, or LP-BLER). For example, if OOK/FSK is used for LP-WUS, due to the non-coherent detection of LP-WUR, UE cannot extract the LP-WUS along from the received signal (i.e. the noise and/or interference cannot be decoupled from LP-WUS). Therefore, the LP-RSRP or LP-RSSI can be defined as the received power within the BW of a LP-WUS and within a pre-defined time duration (e.g. the time duration for reference signal transmission).

Also, how to use the measurement quantities should also be discussed. For example, in current specification cell re-selection and S-criteria are supported based on SS-RSRP and SS-RSRQ. If new quantities are defined, there may be relationship between the new quantities and legacy quantities, so that UE can perform cell re-selection and S-measure based on the new quantities.

- Further discuss how to define and use the measurement quantities by LP-WUR.

For neighbor cell measurement, there can be two ways. One way is to perform neighbor cell measurement by LP-WUR. In this way, power saving gain can be maximized due to minimized switching to MR. But due to the non-coherent detection of LP-WUR (if OOK/FSK is used), advanced interference handling is not feasible. Thus, it should be carefully designed how to transmit the reference signal for measurement. For example, to avoid interference the reference signals from different cells can be TDM-ed. An example is shown in Figure 26, where the blocks in white are the ‘interference-free’ measurement occasions for different cell. Another way is that UE use MR to perform measurements, which may reduce the power saving gain due to frequent switching between low-power wake-up receiver and MR. To further reduce the power consumption, the measurement by MR can be controlled by the result of serving cell measurement by low-power wake-up receiver, e.g., only when the channel state of serving cell is worse than a threshold, is neighbor cell measurement performed.

Figure 26 An example for neighbor cell measurement in TDM manner

- Further study how to support neighbor cell measurement.

Some evaluation results on power saving gain regarding different RRM measurement assumptions are provided in our companion paper[3].

Activation / deactivation of LP-WUS monitoring

When LP-WUS is used in IDLE/INACTIVE state, it should be configured via cell specific signaling, e.g. SIB. After receiving the configuration, UE can start to monitor LP-WUS based on certain conditions. For example, depending on the coverage of LP-WUS, UE may start to monitor LP-WUS when the channel condition is good enough. Similarly, UE can stop monitor LP-WUS when the channel condition is bad enough, where UE can estimate the channel condition by LP-WUR via e.g. LP-SS. UE can also stop monitor LP-WUS when MR is ON, where MR can be turned on due to paging arrival or RRM measurement.

- Further study how to define activation/deactivation conditions for monitoring LP-WUS.

Consideration for CONNECTED mode

The signal design is highly related to receiver architecture. A unified LP-WUS signal design should be considered for CONNECTED mode and IDLE/INACTIVE mode to avoid supporting two kinds of LP-WUS receiver architecture for different RRC states. This will reduce hardware design complexity. However, the UE behavior in CONNECTED mode is different from that in IDLE/INACTIVE mode. So, the functionality and procedure of LP-WUS for CONNECTED mode can be designed separately. For example, in IDLE/INACTIVE mode, the power consumption is mainly caused by unnecessary paging monitoring. But in CONNECTED mode, the power consumption mainly comes from unnecessary PDCCH monitoring.

- A unified LP-WUS signal design for CONNECTED mode and IDLE/INACTIVE mode can avoid supporting two kinds of LP-WUS receiver architecture for different RRC states.

- A unified LP-WUS signal design should be considered for CONNECTED mode and IDLE/INACTIVE mode.

Conclusions

In this contribution, we discuss the signal design, functionality and procedure for LP-WUS. The following observations and proposals are made:

- For the two options for FSK signal generation, the following observations are made

- For M=1, option FSK-1 and option FSK-2 are the same

- For M=2, option FSK-2 outperforms FSK-1 due to higher power boosting

- For M>2,

  - The segments required for option FSK-1 grows linearly with M.

  - For option FSK-2, the segments required grows exponentially with M. In this case, it is suggested to decompose the transmission into parallel channels with M=1 or M=2. With this method, segments required are the same as option FSK-1.

- A joint modulation of OOK-4 and FSK-2 can be more robust to timing error than OOK-4 due to its longer symbol duration.

- A joint modulation of OOK-4 and FSK-2 can have better frequency diversity than FSK-2.

- The parallel receiver architecture can be used by the joint modulation to enable both OOK and FSK.

- OOK and FSK detection can be made more robust with respect to interference and noise by modulating with sequences having good correlation properties. Sequences suitable for existing gNB transmitter, such as ZC sequences, have such properties.

- Options OOK-1 and OOK-2 experience marginal performance loss when ADC sampling rate is reduced to 960 kHz. Option OOK-4 sees 2 dB performance loss when sampling is reduced from 1.92MHz to 960kHz.

- For Option OOK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.5, 1.6 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 1.4 dB respectively.

- For Option OOK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0.3, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.2, 0.4, 0.5 dB respectively.

- For Option OOK-4 with channel model of TDL-C 300ns, the performance loss caused by 1, 2 μs timing error are 2.5, 4.8 dB respectively. Timing error of 4 μs causes error floor. For channel model of TDL-C 1000ns, the performance loss caused by 1 μs timing error is 2.7 dB. Timing error of 2 μs causes error floor.

- If the guard bands can cover frequency offset, Options OOK-1, OOK-2 and OOK-4 are insensitive to frequency offset.

- ZC sequence-based generation of FSK/OOK waveform outperforms significantly the OOK modulation with rectangular waveform in fading channel.

- For OOK-4, concentrated OOK provides 0.5dB gain for ideal timing case compared with the normal sequence-based waveform because ISI between OOK symbols are mitigated.

- Concentrated OOK significantly improves robustness against timing error, e.g. at least 2dB improvement under 2us timing error.

- Concentrated waveform is applicable to OOK and FSK to improve their respective performance.

- Both FSK-1 and FSK-2 sees no performance loss when ADC sampling rate is reduced to 960 kHz.

- For Option FSK-1 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.8, 1.2 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0.1, 0.8, 2 dB respectively.

- For Option FSK-2 with channel model of TDL-C 300ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.5, 1.7 dB respectively. For channel model of TDL-C 1000ns, the performance loss caused by 1, 2, 4 μs timing error are 0, 0.6, 2.4 dB respectively.

- If the guard bands can cover frequency offset, Options FSK-1, FSK-2 are insensitive to frequency offset.

- For sequence-based detection, it can be observed that the performance would be degraded by 0.5dB due to the phase noise and I/Q imbalance when 120 uW oscillator is used.

- Sequence-based OOK/FSK modulation has better coverage performance than energy detection, even if phase noise and I/Q imbalance, frequency error within 10ppm and 1us timing error are considered.

- The target data rate for LP-WUS design can be x101~x102 kbps level.

- To indicate paging information by LP-WUS, there is trade-off between power saving gain/latency and required data rate/coverage performance.

- Defining slot format for LP-WUS is helpful to reduce the FAR, and thus increase the power saving gain.

- Parallel OOK/FSK receiver architecture is capable to do frequency error estimation and correction.

- The requirement of periodicity is usually dominated by the time error, where the residual frequency error contributes most of the time error.

- The resource overhead of periodic LP-SS is marginal for the following reasons: 1) based on the requirement of time/frequency estimation, the overhead can be low to ~0.01%, and 2) the overhead can be reduced by gNB configuration.

- A unified LP-WUS signal design for CONNECTED mode and IDLE/INACTIVE mode can avoid supporting two kinds of LP-WUS receiver architecture for different RRC states.

- Study joint modulation of OOK and FSK by e.g. multiple segments in time domain for FSK.

- Study carrying information via different sequences or cyclic shifts for the OFDM based signal/channel for LP-WUS.

- Study how to improve the coverage performance of LP-WUS.

- Study methods to extend coverage and reduce misdetection probability by block coding.

- For multiplexing of LP-WUSs for different UEs, study at least TDM, FDM, CDM.

- At least paging information is carried in LP-WUS, where LP-WUS can indicate per-UE information or per-group information.

- Further study how to indicate paging information to get a good balance between low data rate requirement and low false wakeup rate.

- The following information can be indicated by LP-WUS in addition to paging information:

  - Tracking area/RAN area information

  - Cell information

  - SI change and ETWS/CMAS information

- Study the slot format of LP-WUS.

- At least continuous monitoring of LP-WUS should be supported.

- At least support periodically transmitted LP-SS for LP-WUS/WUR.

- Further study the periodicity of LP-SS based on the requirement of max time error and/or frequency error.

- If there is time and frequency relationship between LP-WUR and MR, then LP-WUR can assist the re-sync procedure of MR, which can reduce the power consumption and latency.

- At least serving cell measurement is supported by LP-WUR using LP-WUS/LP-SS.

- Further discuss how to define and use the measurement quantities by LP-WUR.

- Further study how to support neighbor cell measurement.

- Further study how to define activation/deactivation conditions for monitoring LP-WUS.

- A unified LP-WUS signal design should be considered for CONNECTED mode and IDLE/INACTIVE mode.

- References

Chairman’s notes of RAN1 #112.

N. Mazloum and O. Edfors, "Interference-Free OFDM Embedding of Wake-Up Signals for Low-Power Wake-Up Receivers," in IEEE Transactions on Green Communications and Networking, vol. 4, no. 3, pp. 669-677, Sept. 2020, doi: 10.1109/TGCN.2020.2970312.

R1-2302339, Evaluations for LP-WUS, Huawei, HiSilicon.

R1-2302340, On architectures of LP-WUS receiver, Huawei, HiSilicon.

M. Park, et.al., “WUR phase noise model follow-up,” doc.: IEEE 802.11-17/0326r0.

P. Moulin and R. Koetter, "A framework for the design of good watermark identification codes," Proc. SPIE 6072, Security, Steganography, and Watermarking of Multimedia Contents VIII, 60721H (17 February 2006).

- Appendix

Table 4 Simulation assumptions for OOK options

| Parameter | Option OOK-1 | Option OOK-2 | Option OOK-4 |
| --- | --- | --- | --- |
| Data rate | 14 kbps | 56 kbps | 56 kbps |
| Scenario | 2.6GHz dense urban |  |  |
| Packet size | 48 bits |  |  |
| Bandwidth configuration | See Figure 7 |  |  |
| Subcarrier spacing | 30kHz |  |  |
| Channel model | TDL-C 300ns (for Figure 10, TDL-C 1000ns is also used) |  |  |
| UE speed | 3 km/h |  |  |
| Number of RX antennas | 1 |  |  |
| Channel coding | Non |  |  |
| Timing error | - 0 μs for Figure 8, Figure 9 and Figure 11 - 0, 1, 2, 4μs for Figure 10 |  |  |
| Frequency error | - 0ppm for Figure 8, Figure 9, Figure 10 - 0/10/20 ppm for Figure 11 |  |  |
| ADC | - Digitization resolution: 8bit/Sample - Sampling frequency: 3.84 MHz for Figure 8, Figure 10, Figure 11 - Sampling frequency: 3.84/1.92/0.96 MHz for Figure 9 |  |  |
| Manchester code | ½-rate Manchester |  |  |
| Simulated sequence to generate the OOK waveform | For OOK-4: - Normal ZC-sequence based waveform: ZC sequence with length 11, u=1, cyclically extended to 12 points for each OOK segment before DFT; Concentrated Waveform: ZC sequence with length 7, cyclically extended to 8-points sequence, and two zero values added before and after the sequence to form a 12-points sequence for each OOK segment before DFT. |  |  |

Table 5 Simulation assumptions for FSK options

| Parameter | Assumptions for both Option FSK-1 and FSK-2 |
| --- | --- |
| Data rate | 56 kbps |
| Scenario | 2.6GHz dense urban |
| Packet size | 48 bits |
| Bandwidth configuration | See Figure 14 |
| Subcarrier spacing | 30kHz |
| Channel model | TDL-C 300nsTDL-C 300ns (for Figure 17, TDL-C 1000ns is also used) |
| UE speed | 3 km/h |
| Number of RX antennas | 1 |
| Channel coding | Non |
| Timing error | - 0 μs for Figure 15, Figure 16 and Figure 18 - 0, 1, 2, 4μs for Figure 17 |
| Frequency error | - 0ppm for Figure 15, Figure 16 and Figure 17 - 0/10/20 ppm for Figure 18 |
| ADC | - Digitize resolution: 8bit/Sample - Sampling frequency: 3.84 MHz for Figure 15, Figure 17, Figure 18 - Sampling frequency: 3.84/1.92/0.96 MHz for Figure 16 |

Table 6  Simulation assumptions for Figure 19

| Parameter | Assumptions |
| --- | --- |
| Scenario | 2.6GHz dense urban |
| Packet size | 48 bits |
| Modulation | Sequence base modulation |
| Simulated sequence | ZC sequence with different cyclic shifts are simulated root u=1, the length of the root sequence is first maximum prime which is smaller or equal than the DFT size of LP-WUS: - 47 for 4RB LP-WUS - 11 for 2RB LP-WUS 119 for 12RB LP-WUS root u=1 maximum number of cyclic shifts nCS,max=4 |
| Signal Bandwidth | 2RB (720kHz)/ 4RB (1.44MHz)/ 10RB (3.6MHz) |
| Subcarrier spacing | 30kHz |
| Channel model | TDL-C 300ns |
| UE speed | 3 km/h |
| Number of RX antennas | 1 |
| Channel coding | Non |
| Time/frequency synchronization | Ideal |
| Phase noise | Non |
| ADC | Ideal, no down sampling or quantization |
| Block FAR | 0.1% |

Table 7   Simulation assumptions for Figure 20

| Parameter | Assumptions |
| --- | --- |
| Scenario | 2.6GHz dense urban |
| Packet size | 48 bits |
| Modulation | Sequence base modulation |
| Simulated sequence | ZC sequence root u=1 maximum number of cyclic shifts nCS,max=4 |
| Signal Bandwidth | 2RB (720kHz)/ 4RB (1.44MHz)/ 10RB (3.6MHz) |
| Subcarrier spacing | 30kHz |
| Channel model | TDL-C 300ns |
| UE speed | 3 km/h |
| Number of RX antennas | 1 |
| Channel coding | Non |
| Time/frequency synchronization | Ideal |
| Phase noise | Modelled as discussed in [3] white frequency noise coefficient c=0.24 fs |
| ADC | Ideal, no down sampling or quantization |
| Block FAR | 0.1% |