3GPP TSG RAN WG1 #112bis-e		R1-2302506

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Evaluation methodologies for R18 LP-WUS/WUR

Agenda Item:	9.11.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #112 meeting, the agreements as shown in Appendix D for R18 low-power Wake-up Signal and Receiver for NR was approved [1]. In this contribution, the target use cases, design targets are firstly discussed. Then we give our views on the power evaluation methodologies for both RRC idle/inactive and connected modes, followed by the initial simulation results. Finally, LLS and coverage evaluation methodology for LP-WUS is discussed as well.

- Design targets

- Relative power of LP-WUR “on” state

The overall target for the power consumption of LP-WUR is to strive for a longer battery life or standby time of device. For legacy UEs in RRC idle/inactive mode, the average UE power consumption is typically several or tens of mW. To achieve substantial power saving gain, a reasonable target power consumption of LP-WUR “on” state needs to be reduced by a factor of at least 100 to 1000, which is tens or hundreds of μW. Based on the evaluation results provided in section 3.2 and 3.4, to achieve substantial power saving gain e.g., 80%, less than 1unit relative power of LP-WUR “on” state is desirable.

For a general usage, LP-WUR should address all traffic arrival cases for IoT/Wearable/eMBB. Especially, when the traffic arrival is relatively dense, battery life is critical. Thus, keeping the “on” state power consumption as low as possible is important as the results shown in the section 3.2.

Observation 1: In the case that the relative power of LP-WUR “on” state is 0.03~0.5 unit, substantial power saving gain e.g., up to 80% can be achieved.

Observation 2: LP-WUR should address all traffic arrival cases for IoT/Wearable/eMBB, thus keeping the “ON” state power as low as possible is important for increasing the battery life of the device.

Proposal 1: The target relative power of LP-WUR “on” state should be less than 1 unit.

- Latency

The latency caused by adopting LP-WUS/WUR is the time interval between the data arrival time and the time that the MR is ready to receive paging, which consists of two parts as follows:

- Wake-up delay:

- It depends on LP-WUS monitoring cycle

- MR transition time:

- The ramp-up transition time of main radio

- The sync/resync time of main radio (only for RRC idle/inactive mode)

Observation 3: The latency brought by using LP-WUS/WUR consists of two parts: wake-up delay and MR transition time.

For RRC idle/inactive mode, since there are variety of use cases with different latency requirements, it will be difficult to set certain latency targets. For IoT cases, latency in the order of seconds is required. Specially, for latency-sensitive IoT cases, target latency may need to be hundreds of milliseconds if necessary. In addition, latency within several seconds is expected for wearable cases.

For RRC connected mode, considering the stringent latency requirement of XR traffic, the total latency by adopting LP-WUS/WUR should be no more than several milliseconds. For example, to be able to recover data transmission quickly, MR is expected to enter micro or light sleep state during continuously LP-WUS monitoring, so as to control latency within 3ms (i.e., half of light sleep transition time). Apart from XR use case, the target latency of other eMBB use cases can be tens of milliseconds which is mainly contributed by the LP-WUS monitoring cycle.

Proposal 2: Consider the following as the latency target by applying LP-WUS/WUR:

- Within 1 or 2 seconds for latency-sensitive IoT cases and within several or tens of seconds for other IoT cases;

- Within several seconds for wearable cases;

- Within several milliseconds for XR use case and within tens of milliseconds for other eMBB use cases;

- Data rate

For idle UEs, the maximum data rate for paging can be calculated as R = 48 bits (UE ID length) * 32 (UEs per PO) * 4(POs per radio frame)/10ms = 614kbps. While for LP-WUS, highest data rate may not necessary, e.g., full UE ID delivery may be not required. Hence, around 100kbps data rate design target can be considered.

Proposal 3: Around 100kbps data rate can be considered as design target for LP-WUS.

- Evaluation methodologies and results

  - Assumptions for RRC idle/inactive mode

  - Sync/re-sync time and energy assumption for MR

It has been agreed that the number of SSBs for sync/re-sync of MR is up to 10. For evaluation, two different numbers of SSB corresponding to different SINR cases is reasonable, e.g., 5 SSBs for low SINR case and 3 SSBs for medium or high SINR case. Note that these SSBs can be used for both synchronization and RRM measurement. Besides, as shown in Figure 1, the detailed processing timeline of sync/re-sync is assumed as: UE needs to do continuously detection to determine the slot and symbol boundary of SSB from the beginning of sync/re-sync process; After acquiring the timing information, the rest SSBs can be measured discontinuously i.e., UE can enter sleep state between the time interval of two SSB bursts. According to these analyses, we provide the sync/re-sync time and energy as the following Table 1.

Figure 1. Sync/re-sync timeline including RRM measurement procedure by MR

Table 1. The assumptions of Sync/re-sync time and energy

|  | Sync/re-sync time [ms] | Sync/re-sync energy [ms*units] |
| --- | --- | --- |
| Number of SSBs for sync/re-sync =3[1] | 60 | 2180 |
| Number of SSBs for sync/re-sync =5[1] | 100 | 3340 |
| Note 1: 10ms continuous monitoring from the beginning of sync-re-sync time is assumed. |  |  |

Proposal 4: For sync/re-sync time and energy, Table 1 of R1-2302506 can be assumed.

  - Transition time between LP-WUR ‘on’ and ‘off’ states for LP-WUR

For the transition from LP-WUR ‘off’ to ‘on’ state, it mainly includes the starting and settling process of MCU or DSM. The MCUs boot up for a fixed number of cycles, or a fixed time period, to enable an external oscillator to become stabilized. After that, a DSM will initialize the runtime environment of the device etc. Accordingly, when assuming the stabilization of the digital circuit needs 1024 cycles and the clock is on the order of Msps, the transition time is approximately tens of milliseconds e.g., 10ms.

Proposal 5: Transition time between LP-WUR ‘on’ and ‘off’ states can be assumed as 10ms.

  - Power model for OFDMA-based signals/channels detection

In the last meeting, LP-WUR for OFDMA-based signals/channels detection has been agreed for study in 9.13.2 agenda item. However, the current agreed power model for LP-WUR is typically considered for OOK or FSK detection. Thus, the power model for OFDMA-based signals/channels detection should be discussed. In our companion contribution [2], the reported power consumption of OFDM receiver with relaxed hardware performance requirements in the literatures are summarized in Table 3 of [2], it is observed that the absolute power consumption of such OFDM receivers mainly fall into level of tens of milliwatts. Furthermore, for sequence-based OFDM signals/channels detection, the power consumption of SSB/CSI-RS processing can be used as a reference, i.e., 50 units defined for NR RedCap UE in TR 38.875. Considering the hardware performance requirements is relaxed and only sequence detection is performed without PBCH decoding for OFDMA-based signals/channels detection, the relative power of LP-WUR “on” state can be assumed as 20 or 40 units. And for the relative power of its “off” state, less relative power than that of the ultra-deep sleep state of main radio can be assumed, i.e., 0.01unit.

Given the analysis above, the power model of LP-WUR for OFDMA-based signals/channels detection can be assumed as follows:

- Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: 0.01unit

- Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: 20 or 40 units

Proposal 6: Adopt the following power model for LP-WUR for OFDMA-based signals/channels detection :

- Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: 0.01unit

- Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: 20 or 40units

  - RRM measurement assumption

In RRC idle/inactive mode, to meet RRM measurement requirements defined by RAN4, a legacy UE is supposed to perform measurement by MR every several I-DRX cycles. In the case of adopting LP-WUS monitoring for UE power saving, to avoid frequent transitions from/to ultra-deep sleep of MR which will result in plenty of power consumption, RRM measurement performed by MR is expected to be relaxed by means of partially or totally offloading it to LP-WUR. Following three RRM measurement options can be considered.

- Option 1 (legacy behavior): RRM measurement is only performed by MR.

- Option 2: MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20. And LP-WUR performs assistant RRM measurement based on periodic lower power signal e.g., beacon or LP-SS.

- Option 3: RRM measurement is totally performed by LP-WUR.

Additionally, the feasibility and detailed design of RRM measurement performed by LP-WUR need to be further evaluated and discussed. Note that we assume RRM measurement performed by MR is included in the process of sync-/re-sync as shown in Figure 1. And the relative power of RRM measurement by LP-WUR is the same as that of LP-WUS detection.

Proposal 7: For RRM measurement assumptions in RRC idle/inactive mode, the following options can be considered.

- Option 1: RRM measurement is only performed by MR.

- Option 2: LP-WUR performs RRM measurement based on periodic lower power signal e.g., LP-SS. MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20.

- Option 3: RRM measurement is only performed by LP-WUR.

  - Power evaluation results for RRC idle/inactive mode

Based on the agreed evaluation methodologies for RRC idle/inactive mode and the aforementioned additional assumptions, we provide the power evaluation results considering the following aspects which will have impact on power consumption and latency performance.

- Baselines: PO monitoring with i-DRX, e-DRX, with or without PEI

- Assumption on MR sleep state during LP-WUS monitoring: deep sleep or ultra-deep sleep

- The relative power of LP-WUR “ON” state: [0.01/0.05/0.1/0.5/1/2/4/20/40] units

- MR Ramp-up time: [400ms] and Ramp-up and down transition energy: [15000]

- Sync/re-sync time X: X= [60, 100ms] and sync/re-sync energy: [2180, 3340]

- RRM:

- For baseline with option 1: RRM measurement is only performed by MR.

- For LP-WUS scheme:

Option 2: MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20. And LP-WUR performs assistant RRM measurement based on periodic lower power signal e.g., LP-SS.

Option 3: RRM measurement is totally performed by LP-WUR.

- The false-alarm rate (FAR) of LP-WUS: [0, 0.1%, 1%]

Note: Adopt Alt-2 of FAR target which is determined across a reference time duration (e.g., 1.28s) of one or multiple WUS attempts/trials.

- Per UE paging rate i.e., RE=RE, REF= [1%, 0.1%, 0.01% or 0.001%], Y=YREF = 1.28s

- Per group paging rate i.e., RG=1 – (1 – RE)N , N is the number of UEs in the group: [10, 20]

- LP-WUS monitoring:

  - Option 1: continuously monitoring;

  - Option 2: duty cycle based monitoring

- [200ms, 600ms, 1000ms] as the period for an on-and-off cycle, and for each cycle the duty cycle ratio is [0.1%, 2%, 10%, 40%].

- LP-WUR on-off transition time: 10ms.

The following is the detail of evaluation results and observations considering the mentioned aspects.

- Power consumption comparison for i-DRX, e-DRX (with or without PEI) with LP-WUS scheme

Figure 2 shows the simulation results with different baseline schemes under the configurations in Table 5.

Table 5. Configurations for evaluation with different baselines.

| Parameters | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate | Assumed MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Values | 400ms; 15000 | 100ms; 3340 | 0.1% | continuous | 0.001% | Ultra-deep sleep | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |

Figure 2.  Initial simulation results of power consumption and latency for different schemes

According to the results, the following observations are given:

Observation 4: Compared with I-DRX paging, LP-WUR/WUS scheme with continuously monitoring configuration can achieve around 50%~98% power saving gain when the relative power of LP-WUR “ON” state is no more than 1 unit, with marginal latency increase.

Observation 5: Compared with eDRX, LP-WUR/WUS scheme with continuously monitoring configuration can largely reduce the paging latency (23x), with comparable UE power consumption.

Observation 6: LP-WUR/WUS scheme provides a much better trade-off between latency and power consumption when relative power of LP-WUR “ON” state is no more than 1unit, compared with I-DRX paging and eDRX scheme.

Observation 7: Battery life is inversely proportional to relative power of LP-WUR “ON” especially when the traffic is extremely sparse e.g., paging rate is 0.001%.

- The relative power of LP-WUR “On” state: [0.01/0.05/0.1/0.5/1/2/4/20/40] units

Figure 3 shows the simulation results with different relative power of LP-WUR under configurations in Table 3.

Table 3. Configurations for evaluation with different relative power of LP-WUR “ON” state.

| Parameters | Baseline: | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate | Assumed MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 400ms; 15000 | 100ms; 3340 | 0%, 0.1% | continuous; duty cycle | 0.001% | Ultra-deep sleep, Deep sleep | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |
| Notes: Considering latency sensitive use cases, the configuration for duty cycle of LP-WUS is: [200ms] as the duty cycle, and [4ms] as the active time for monitoring LP-WUS every cycle. |  |  |  |  |  |  |  |  |

(Notes: When the WUR ON power=20/40, it is assumed that WUR OFF power = 0.01; The transition time between LP-WUR “on” and “off” is assumed as 10ms)

Figure 3.  The performance of LP-WUS/WUR scheme with different relative powers of LP-WUR.

According to the results, the following observations are given:

Observation 8:  Continuously LP-WUS monitoring is not a feasible configuration when the relative power of LP-WUR “ON” is higher than 1unit.

Observation 9: Duty cycled LP-WUS monitoring can significantly reduce UE power consumption of LP-WUS scheme except for the case with the relative power of LP-WUS “ON” to be 20 or 40units, but latency will increase accordingly.

Observation 10: Up to 45% power saving gain will be lost if UE main radio goes into deep sleep rather than ultra-deep sleep during LP-WUS monitoring.

- LP-WUS monitoring configuration: Duty-cycled LP-WUS with various duty cycle length and ratio

Figure 4 shows the simulation results with different duty-cycled LP-WUS with various duty cycle length and ratio under configurations in Table 4.

- [200ms, 600ms, 1000ms] as the period for an on-and-off cycle, and for each cycle the duty cycle ratio is [0.1%, 2%, 10%, 40%].

Table 4. Configurations for evaluation with different LP-WUS monitoring configurations.

| Parameters | Baseline: | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | Relative power of LP-WUR ON | Per UE paging rate | Assumed MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 400ms; 15000 | 100ms; 3340 | 0.1% | 0.1, 1 units | 0.001% | Ultra-deep sleep | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |

Figure 4. The power and latency performances of LP-WUS/WUR scheme with different DRX cycle lengths and ratio.

According to the results, the following observations are given:

Observation 11: With fixed duty cycle ratio and relative power of LP-WUR “ON” state, different LP-WUR duty cycle lengths have no or less impact on UE power consumption of LP-WUS scheme, and the difference is due to the number of LP-WUR ON-OFF transition.

Observation 12错误!未指定顺序。: For LP-WUS scheme, latency will be reduced with the increase of duty cycle ratio of LP-WUS monitoring, while UE power consumption will increase accordingly.

Observation 13: The power consumption of LP-WUS scheme with 20 or 40units LP-WUR “ON” power is ten or hundred times larger than that with less than 1unit LP-WUR “ON” power. So does the corresponding battery life.

Observation 14错误!未指定顺序。: Only when duty cycle ratio is extremely low e.g., 0.1%, substantial power saving gain (e.g., up to 50%) can be achieved in the case of high relative power of LP-WUS “ON” i.e., 20 or 40units.

- RRM

Figure 5 shows the simulation results with different RRM schemes under configurations in Table 5.

Table 5. Configurations for evaluation with different RRM assumptions.

| Parameters | Baseline | Relative power of LP-WUR | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Assumed MR sleep state | Per UE paging rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 0.1/1 unit | 400ms; 15000 | 100ms; 3340 | 0.1% | continuous; duty cycle | Ultra-deep sleep | 0.001% |
| Note: Considering latency sensitive use cases, the configuration for duty cycle of LP-WUS is: [200ms] as the duty cycle, and [4ms] as the active time for monitoring LP-WUS every cycle. |  |  |  |  |  |  |  |  |

No power saving gain can be obtained even in MR RRM relaxed mode with 2% duty cycle ratio

Figure 5.  The power and latency performances of LP-WUS/WUR scheme with different RRM measurement.

According to the results, the following observations are given:

- Power: UE power consumption of LP-WUS scheme will significantly increase, once RRM performed by MR since plenty of MR transition times will be resulted in. But, as the RRM measurement by MR is gradually relaxed, UE power consumption decreases somewhat. Until RRM measurement completely offload to WUR i.e., RRM measurement is only performed by WUR, there will be the most power saving gain by adopting LP-WUS.

- Latency: Latency does not change much as RRM measurement period decreases since it is assumed that MR and LP-WUR can work at the same time and the network is supposed to page UE in the PO after a fixed time offset (e.g., ramp up time + sync/re-sync time) from sending LP-WUS.

Observation 15: Even if RRM measurement is performed by main radio at a relaxed level, the total UE power consumption of LP-WUS scheme increases distinctly.

Observation 16: No power saving gain can be obtained by duty cycled LP-WUS monitoring (with 2% duty cycle ratio) scheme, even for the case MR RRM periodicity is relaxed, when the relative power of LP-WUR “ON” is 20 or 40units.

Observation 17: RRM measurement completely performed by WUR will greatly help reduce UE power consumption of LP-WUS scheme.

- The false-alarm rate (FAR) of LP-WUS: [0, 0.1%, 1%]

Figure 6 shows the simulation results with different FAR under configurations in Table 6

Table 6. Configurations for evaluation with different FAR.

| Parameters | Baseline: | Ramp-up time and transition energy | Per UE paging rate | Sync/re-sync time and energy | LP-WUS monitoring configuration | Relative power of LP-WUR ON | Assumed MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 400ms; 15000 | 0.001% | 100ms; 3340 | continuous | 0.1 or 1units | Ultra-deep sleep | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |

Figure 6.  The power and latency performances of LP-WUS/WUR scheme with different FAR.

(Note: FAR here refers to the FAR of channel which not contain the FAR caused by group paging. If we both consider FAR and group wake-up, UE power consumption of LP-WUS scheme will sharply increase.)

According to the results, the following observations are given:

Observation 18: As FAR for LP-WUS increases, UE power consumption goes up.

- Per UE paging rate: [1%, 0.1%, 0.01%, 0.001%] and per group paging rate: N= [1, 10, 20]

Figure 7 shows the simulation results with different paging rate under configurations in Table 7.

Table 7. Configurations for evaluation with different paging rate.

| Parameters | Baseline | Relative power of LP-WUR ON | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Assumed MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 0.1 unit | 400ms; 15000 | 100ms; 3340 | 0.1% | continuous | Ultra-deep sleep | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |
| Note: RE = 0.001% or 1%, N = [1 10 20] when calculate per group paging rate. |  |  |  |  |  |  |  |  |

Figure 7. The power and latency performances of LP-WUS/WUR scheme with different paging rate.

According to the results, the following observations are given:

Observation 19: As the paging rate and the number of UE in group increase, power consumption of LP-WUS scheme will increase.

  - Assumptions for RRC connected mode

XR is one of the target use cases for applying LP-WUS. Per the discussion in R17/18 XR, one of the identified pain point issues is XR data packets have a wide range of unpredictable jitter. That is, UE has to perform intensive PDCCH monitoring in the entire jitter range in order to receive the scheduling of XR traffic timely and avoid packet loss caused by exceeding the PDB (e.g., 10ms for XR video traffic). However, the existing R15/16/17 power saving schemes cannot completely avoid the unnecessary PDCCH monitoring actually. In this regard, the motivation to study LP-WUS/WUR in RRC connected mode is to seek a better solution to reduce the unnecessary power consumption caused by uncertain jitter.

Observation 20: The motivation to study LP-WUS/WUR in RRC connected mode for XR use case is to reduce the excessive PDCCH monitoring due to unpredictable jitter.

The power model of LP-WUR agreed in the last meeting can also be adopted for power evaluation in RRC connected mode. To guarantee the latency requirement of XR traffic, main radio should enter micro or light sleep state rather than the ultra-deep sleep state during LP-WUS monitoring. Accordingly, the latency caused by LP-WUS detection will be 0 or 3ms. And the sum of relative power of LP-WUR “ON” state and MR in micro/light sleep state will be close to 45 or 20 units, which is less than half of the relative power of MR PDCCH-only state i.e., 100 units.

As shown in Figure 8(b), enhanced C-DRX, i.e. aligned C-DRX cycle to XR traffic periodicity is adopted. To further reduce the unnecessary PDCCH monitoring during the jitter range (covered by DRX on duration), a UE can firstly monitor LP-WUS instead of PDCCH at the beginning of DRX onduration. And after LP-WUS is detected, the UE just start to monitor PDCCH. As such, by replacing the unnecessary PDCCH monitoring with LP-WUS detection, UE will have more opportunity to achieve more power saving gain, compared to the existing power saving schemes as depicted in Figure 8 (a).

Figure 8.  Potential working procedure for LP-WUS/WUR based jitter handling

Apart from XR traffic, FTP model 3 is another one of typical eMBB traffics for applying LP-WUS in RRC connected mode. However, ultra-deep sleep state of MR is still inapplicable for UE to enter in RRC connected mode since hundreds of milliseconds latency which is unacceptable for data transmission will be caused. By means of LP-WUS monitoring, MR of UE can enter deep/light/micro sleep state and it will be woken up right after data arrives at the gNB side so that the long latency caused by DRX OFF can be largely reduced. In this sense, the transmission latency can be reduced deeply by applying LP-WUS, which is conducive to improving the performance of UE perceived throughput (UPT) accordingly.

Observation 21: The motivation to study LP-WUS/WUR in RRC connected mode for eMBB use case e.g., FTP3 is to reduce latency and improve UPT performance.

Proposal 8: In RRC connected mode, Ultra-deep sleep state of main radio should not be applied. And UE main radio can enter micro, light or deep sleep state during LP-WUS monitoring.

  - Power evaluation results for RRC connected mode

In the following evaluation, performance metrics including power consumption, latency, capacity, UPT are considered. Besides, both XR traffic and eMBB traffic are evaluated, the detail evaluation methodologies and assumptions can be found in TR 38.838 and 38.840, respectively.

- Evaluation for XR traffic model (refer to TR38.838 and the agreed jitter model in R18 XR SI)

Based on the R17 and R18 XR power evaluation methodologies and assumptions, we further evaluate power consumption and system capacity of different cases. And the detailed configurations can be founded in Appendix B. Note that for capacity results of LP-WUS/WUR scheme (combined with main receiver light sleep) corresponding to the UE satisfaction metric with both 95% and 99% packet successful rate are shown. Evaluations for both low load and high loads are given, corresponding to 5 and 10 UEs per cell, respectively.

Figure 9.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

Figure 10.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

According to the results shown in Figure 9 and Figure 10, compared to always-On scheme (no DRX configuration), R17 PDCCH monitoring adaptation scheme can obtain 10~19% power saving gain, while LP-WUS monitoring schemes can achieve about 25% or 30% power saving gain, meaning up to 15% additional power saving gain.

For both low load ang high load cases, comparing to R17 PDCCH monitoring adaptation scheme, LP-WUS/WUR schemes with main radio enters micro sleep during LP-WUS monitoring can bring {6%~15%} additional UE power saving gain with no capacity loss.

In addition, for low load case, comparing to R17 PDCCH monitoring adaptation scheme, LP-WUS/WUR schemes with main radio enters light sleep during LP-WUS monitoring can bring {10%~22%} additional UE power saving gain with 10% (UE satisfaction metric as 99% packet successful rate) or no (UE satisfaction metric as 95% packet successful rate) capacity loss. But, for high load case, LP-WUS/WUR schemes with main radio enters light sleep during LP-WUS monitoring can achieve similar additional UE power saving gain with as that in low load, but the capacity loss is significant.

In this sense, the LP-WUS monitoring combined with main receiver micro sleep can be applied to all the cases, while the combination with main receiver light sleep are mainly useful for low load scenarios. Moreover, it can be observed that the larger jitter range, the more additional power saving gain can be obtained by LP-WUS/WUR.

Observation 22: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {6%~15%} additional UE power saving gain with no capacity loss in both low load and high load cases.

Observation 23: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {10%~22%} additional UE power saving gain, with acceptable capacity loss at least in low load case.

- Evaluation for general eMBB traffic model (refer to TR 38.840)

Based on FTP model 3 traffic model (200ms mean arrival interval) and its corresponding CDRX configuration provided in TR 38.840, we further evaluate UE perceived throughput (UPT) and power consumption with the following schemes:

- Always-On (baseline): Without adopting any power saving mechanism.

- C-DRX:

| DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- |
| 160 | 8 | 100 |

- C-DRX +DCI format 2_6: the time gap between DCI format 2_6 and DRX onduration is 3ms, and the DCI format 2_6 monitoring duration is 2ms.

- R17 PDCCH monitoring adaptation + C-DRX +DCI format 2_6: PDCCH skipping duration is 100ms, SSSG with PDCCH monitoring every slot.

- LP-WUS monitoring+ PDCCH skipping: During PDCCH skipping duration, the UE performs continuous LP-WUS monitoring. Once detecting LP-WUS, UE will start PDCCH monitoring after a wake-up delay e.g., 0ms, 3ms, or 10ms (which corresponds to the ramp-up delay of main receiver from micro/light/deep sleep states).  The assumptions on the relative power of LP-WUR “ON” state is 0.01/1/20/30/40 units.

Figure 11. UPT and power consumption of evaluation schemes

Table 8. PSG and UPT loss compared with always-On scheme

| Schemes | relative power of LP-WUR “ON” | PSG (compared to “AlwaysON”) | UPT loss(compared to “AlwaysON”) |
| --- | --- | --- | --- |
| C-DRX | - | 52.19% | 60.93% |
| C-DRX + DCI 2_6 | - | 53.78% | 63.74% |
| R17 PDCCH monitoring adaptation + C-DRX | - | 58.42% | 66.92% |
| R17 PDCCH monitoring adaptation + C-DRX + DCI 2_6 | - | 77.35% | 86.73% |
| LP-WUS scheme (wake-up delay 0ms) | 0.01 | 44.00% | 0.00% |
|  | 1 | 42.95% |  |
|  | 20 | 22.83% |  |
|  | 30 | 12.24% |  |
|  | 40 | 1.65% |  |
| LP-WUS scheme (wake-up delay 3ms) | 0.01 | 69.00% | 25.98% |
|  | 1 | 67.95% |  |
|  | 20 | 47.83% |  |
|  | 30 | 37.24% |  |
|  | 40 | 26.65% |  |
| LP-WUS scheme (wake-up delay 10ms) | 0.01 | 86.73% | 57.20% |
|  | 1 | 85.68% |  |
|  | 20 | 65.56% |  |
|  | 30 | 54.98% |  |
|  | 40 | 44.39% |  |

As shown in Figure 11 and Table 8, the following observations are given in terms of UPT :

- Compared to existing power saving schemes, LP-WUS can achieve higher UPT gain, especially for cases with low wake-up delay under LP-WUR scheme.

- It can also be seen that with LP-WUR scheme (wake-up latency 0ms), the same UPT as always-on schemes can be achieved.

As shown in Figure 11 and Table 8, the following observations are given in terms of power consumption when relative power of WUR “ON” is 0.01 unit:

- By applying advanced UE power saving schemes such as DCI format 2_6, the power saving gain will be further increased, with the cost of increased latency, or decreased UPT.

- Comparing with C-DRX scheme, LP-WUR schemes with main radio enters light/deep sleep can bring {16.81% /34.54%} additional UE power saving gain.

- Comparing with C-DRX + DCI format 2_6 scheme, LP-WUR schemes with main radio enters light/deep sleep can bring {15.22% /32.95%} additional UE power saving gain.

- Compared with R17 PDCCH monitoring adaptation + C-DRX + DCI 2_6 scheme, LP-WUR schemes with main radio enters deep sleep during LP-WUS monitoring can bring 9.38% additional UE power saving gain.

Observation 24: UPT of LP-WUR scheme (wake-up latency 0ms case) is the same as always-on scheme which means it can effectively reduce power consumption without affecting network scheduling.

Observation 25: When the relative power of WUR “ON” state is no more than 1unit, LP-WUS monitoring with PDCCH skipping scheme can achieve the best trade-off performance in both UPT and power saving, compared with the existing UE power saving schemes.

Observation 26: When the relative power of WUR “ON” state is larger than 1unit i.e., 20 or 40 units, LP- WUS monitoring with PDCCH skipping scheme has no power saving gain advantage, compared with the existing power saving schemes.

  - System performance (Overhead and gNB Energy consumption)

- System resource overhead

In order to study the system resource overhead by LP-WUS, we perform the following resource overhead calculation by means of the following assumptions:

- The bandwidth of LP-WUS denoted as BWLP-WUS is 4.32MHz +2RB (30kHz) = 5.04MHz.

- The bandwidth of a BWP in a serving cell denoted as BWCell is 20MHz or 100MHz.

- The duration of LP-WUS represented by DLP-WUS is 4 or 14 symbols.

Hence, the resource overhead ratios of LP-WUS to the overall system resource can be calculated as below. And Table 9 and Table 10 provide some reference values based on the above assumptions.

- RRC IDLE/INACTIVE mode:

- For RRC idle/inactive mode, paging configuration:

  - Number of pages for a UE per cell per second (NPage_per_cell_second) is calculated in table X

  - Number of UEs carried in a LP-WUS is NUE_per_WUS

  - Number of beams is Nbeam

R=BWLP_WUS*DLPWUS*NPagepercellsecond*NUEperWUS*Nbeam BWCell

Assuming the resource of one LP-WUS is:

- Alt 1: 5MHz * 4 OFDM symbol (30kHz)

- Alt 2: 5MHz * 14 OFDM symbol (30kHz)

Table 9.  Resource overhead ratio to the overall system resource for RRC idle/inactive mode.

| Scenarios | Paging density |  | Alt | LP-WUS overhead(ratio of the resource compared to the total resources) |  |
| --- | --- | --- | --- | --- | --- |
|  | Scenarios description | Number of pages/cell/second |  | Per UE LP-WUS | Per group LP-WUS (NUE_per_WUS=4) |
| For IoT cases | Assuming 1,000,000 user/km2 connection density [ITU M.2412], with packet arrival every 1 message/2 hours/device [ITU M.2412], assuming 500m ISD with 3 sectors | 10 | Alt 1 | 0.06% (100MHz sysBW) | 0.01% (100MHz sysBW) |
|  |  |  |  | 0.29% (20MHz) | 0.07% (20MHz) |
|  |  |  | Alt 2 | 0.2% (100MHz sysBW) | 0.05% (100MHz sysBW) |
|  |  |  |  | 1.01% (20MHz) | 0.25% (20MHz) |
| For eMBB cases | Assuming very high population density, e.g., sports event with 5,000 users/cell population density, and per UE paging arrival rate = 1% when paging i-DRX cycle is Tpagingcycle = 1.28s | 39 | Alt 1 | 0.22% (100MHz sysBW) | 0.06% (100MHz sysBW) |
|  |  |  |  | 1.12% (20MHz) | 0.28% (20MHz) |
|  |  |  | Alt 2 | 0.79% (100MHz sysBW) | 0.2% (100MHz sysBW) |
|  |  |  |  | 3.93% (20MHz) | 0.98% (20MHz) |
| Notes: - If the tracking area is larger than one cell and the base station needs to blindly page all the UEs that wants to reach in all cells. Then the above value may be increased. - number of beams for LP-WUS is assumed to be 8, which means the LP-WUS is transmitted 8 times. |  |  |  |  |  |

- RRC Connected mode:

- For RRC connected mode, UE number per cell NUE_connected is assumed to be 10. Different UEs are using orthogonal LP-WUS resources which are not overlapped with each other.

- The traffic interval TTrafficInterval of XR video with 16.67ms (30FPS) and eMBB FTP3 with 200ms are assumed for RRC connected mode.

R=BWLP_WUS*DLP_WUS*NUE_connected BWCell*TTrafficInterval

Assuming the resource of one LP-WUS is: 5MHz * 4 OFDM symbol (30kHz) (Considering the coverage is compared to PUSCH)

Table 10.  Resource overhead ratio to the overall system resource for RRC connected mode.

| Resource overhead ratio, R | RRC Connected mode |  |
| --- | --- | --- |
|  | XR traffic | eMBB traffic |
| 100MHz, 30KHz SCS | 0.43% | 0.036% |

Observation 27: For IDLE/INACTIVE mode, the upper bound of resource overhead used for LP-WUS is less than 1% for 100MHz system BW even in worst UE Connection density for both IoT and eMBB cases. For CONNECTED mode, the resource overhead of LP-WUS is less than 0.5% even in high load cases such as 10 XR UEs per cell.

| Resource overhead ratio, R | RRC Idle/inactive mode | RRC Connected mode |  |
| --- | --- | --- | --- |
|  |  | XR traffic | eMBB traffic |
| 20MHz, 30KHz SCS | 0.07% ~ 3.93% | - | - |
| 100MHz, 30KHz SCS | 0.01% ~ 0.79 % | 0.43% | 0.036% |

- Network energy consumption

Network energy consumption for transmitting LP-WUS depends on the LP-WUS occasion/periodicity configuration and gNB implementation and cell load etc. For example, if the LP-WUS is to be transmitted in the slot with some common signalling transmission such as SSB/SIB transmission, e.g. LP-WUS is frequency domain multiplexed with legacy NR signals/channels that to be transmitted, it will not cause serve additional network energy.

Observation 28: Additional network energy consumption caused by LP-WUS/WUR operation can be minimized, e.g., gNB can transmit LP-WUS in the slot with existing NR signal that to be transmitted.

- Link performance and coverage evaluation

  - Definition of FAR

In last meeting, how to define FAR in evaluation is discussed, and following conclusions are made:

- Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behavior, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials

For monitoring of LP-WUS, multiple attempts are required for LP-WUS detection due to the following reasons.

- Reason 1: Time drift due to non-ideal oscillator for periodical LP-WUS monitoring

- Reason 2: WUR may performs always-on monitoring, in which case NW may starting transmission of LP-WUS in any slot in each DRX cycle. UE monitors LP-WUS on each WUS occasion.

Figure 12. Multiple attempts due to time drift and always on monitoring

For Main radio, the oscillator is calibrated, UE maintains the timing well, and the DL channels can be detected without multiple attempts. Similarly, for WUR with high accuracy LO or with assistance from Main radio (e.g., for RRC connected state), and the accurate timing can also be assumed. Thus, for detection of each WUS Tx occasion, UE only need one attempt for detection of the LP-WUS, regardless of LP-WUS channel format.

Observation 29: If accurate timing can be assumed, UE only need one attempt for detection of the LP-WUS, regardless of LP-WUS channel format.

If accurate timing cannot be assumed, for LP-WUS with sequence, either sequence only LP-WUS or LP-WUS with both preamble sync sequence and payload, UE has to sliding the correlation window to search the sequence, considering the time uncertainty in idle state. Hence, UE needs to do multiple correlations for detection of a single WUS transmission. To ensure the overall FAR alarm rate less than 1%/0.1%, the threshold should be higher than that with only single attempt correlation under ideal synchronization, and leading to degraded MDR performance.

For LP-WUS format with sync + payload + CRC bits, only when correlation for preamble part is higher than certain threshold, UE start to decode the payload part. Due to the CRC part, the FAR can be controlled lower than sequence-only LP-WUS.

Observation 30: If accurate timing cannot be assumed, multiple attempts are needed for detection of a single LP-WUS transmission occasion.

In addition to duty cycled monitoring, always on monitoring can also be considered to reduce latency. The difference between duty cycled monitoring and always on monitoring, is the number of occasions transmitted within certain duration. Similarly, monitoring more transmission occasions leading to high FAR. MDR performance is sacrificed to ensure the overall FAR target on multiple transmission occasions is fulfilled.

Observation 31: For always on monitoring, monitoring more transmission occasions leading to high FAR. MDR performance is degraded to make sure the overall FAR target on multiple transmission occasions is fulfilled.

The reason FAR is important is to avoid excessively waking up main radio, which leads to high power consumption. Hence, evaluate FAR across a reference time duration can be considered. Since paging is transmitted in DRX manner, prefer to use DRX cycle as the reference time duration for FAR evaluation. The number of attempts and the threshold used for sequence appearance determination can be up to implementation. Considering different assumptions for sequence length, ADC sampling rate and ADC bit-width, these differences lead to varied threshold setting and per attempt FAR. Hence, reporting reference time duration and corresponding FAR is sufficient.

Per attempt FAR is less intuitive for UE power consumption compared with per reference duration FAR. Besides, for WUS detection with multiple attempts, the detection starting from neighboring sample may leading to same detection output. Since the detection results from multiple attempts may be highly correlated, per attempt FAR cannot be easily derived from the per reference duration FAR.

Proposal 9: Reporting per attempt FAR is not necessary

- UE power consumption mainly depends on reference duration FAR.

- The detection results from multiple attempts may be highly correlated, per attempt FAR cannot be easily derived from the per reference duration FAR.

Proposal 10: The reference time duration can be T, e.g., one DRX cycle, during which UE detect N WUS occasions,

- For always on monitoring, N>1, is the number of occasions within T.

- For duty cycled monitoring, value N depends on WUS periodicity. N=1, if reference time duration is WUS periodicity.

For duty cycled monitoring, the impact to MDR is limited if time drift can be periodically calibrated.

For RRC idle UE, always on monitoring LP-WUS, assume I-DRX=1.28 sec and 1 slot for each LP-WUS transmission, UE detect LP-WUS about 2560 times and 2560*N attempts in the duration. For sequence only LP-WUS, the sequence length should be long enough to achieve both MDR and FAR target. For LP-WUS structure with both preamble, payload and CRC. The FAR should be overall FAR considering both sequence and payload part, rather than FAR for preamble only or payload part only, UE only start detection of the payload part and check CRC, when the correlation of preamble part is higher than certain threshold. The actual number of CRC check per I-DRX cycle is greatly reduced. For example, the FAR rate of preamble part is 1%, UE only need to monitor ~26 times per 1.28 sec. To ensure the overall FAR rate lower than 1%, the CRC bits should be 12bits.

For RRC connected, Main radio is on, and the accurate timing can be assumed for WUR with assist from MR. For detection of each WUS occasion, UE only need one attempt for detection of the LP-WUS. In this case, number of attempts is equivalent to number of occasions. For a typical XR traffic with for 16.67ms C-DRX cycle, UE need to detect 32 LP-WUS occasions, assuming 1 slot LP-WUS duration and 30kHz SCS, per DRX cycle for always on monitoring, and even less times for duty cycled monitoring. In this case, both sequence-only and sequence +payload +CRC formats for LP-WUS are feasible.

Observation 32: For duty cycled monitoring, the impact to MDR is limited if time drift can be periodically calibrated.

Observation 33: For always on monitoring across long reference time duration in RRC idle state

- For sequence-only LP-WUS, the sequence should be properly designed to achieve both MDR and FAR target.

- For LP-WUS with payload and CRC attachment, the CRC bits should be sufficient to achieve the FAR target.

Observation 34: For RRC connected state, the number of attempts is reduced if accurate timing can be assumed. Hence, both LP-WUS with sequence-only and LP-WUS with payload and CRC attachment are feasible to achieve target FAR and MDR.

  - Assumptions in coverage Evaluation

In this section, we provide the evaluation assumptions and coverage evaluation results for LP-WUS.

- Frequency error/frequency drift

In last meeting, four clock assumptions are agreed as follows, while for applicability for the high accuracy opt-3 and opt-4 clock is FFS.

Working assumption: - For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. LO XO Oscillator max frequency error [ppm], LO XO Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm]; 20. - Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option 3/4, cannot be used - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state, - e.g., option 3/4 is not applicable - when MR is in ’ultra-deep sleep state’ with [0.015] power units and LR is in off state or, - when LR monitoring power less than 1 [TBD] power unit, - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc.

5 ppm and 10 ppm oscillator are usually assumed for main radio, higher power consumption should be assumed when 5ppm for 10ppm clock is assumed. When main radio is off, and WUR is on, 5/10 ppm accuracy are not achievable. Hence, we suggest to confirm the FFS bullet in previous working assumption that, Option 3/4 cannot be used when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state.

Proposal 11:  Option 3/4 cannot be used when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state.

In our evaluation,

oscillator opt-1 is assumed, and we assume the oscillator always work in saturate region. For time drift, assuming beacon periodicity 1.28 sec, and calculate the time drift ΔT in 1.28 sec, according to the relationship between the maximum frequency error (Fe) and time drift (ΔT) over a time (T), ΔT(sec)= Fe (ppm)* T(sec) =200 ppm*1.28s=256us. For each WUS transmission, UE start searching for preamble ΔT time before the actual starting time of the WUS transmission. Once UE successfully detects a LP-WUS or beacon, the timing drift can be assumed to be zero. For carrier frequency offset(ΔF), max frequency error is assumed, and the frequency displacement in Hz is ΔF(Hz) = ±Fe (ppm) * Fcarrier(Hz)= ±200 ppm*2.6GHz =520kHz.

Figure 13.  Starting detection in advance due to time drifting

- Noise Figure

In previous meetings, it was agreed that noise figure can be selected from [9, 12, 15, 18, 21, 24] db. One of the above values should be reported together with MIL.

According to [3] [4], 8.7dB and 11dB noise figure can be achieved for WUR, 12dB noise figure is assumed for MIL calculation in this section.

  - Preliminary Evaluation results

In the section, we provide our preliminary evaluation results. Detailed simulation assumptions are provided in Appendix C and several configurations definition are listed as followed:

- Config-1: 4.32MHz/8.64MHz BW, preamble length-16 chips, payload-12bits, CRC-8bits

- Config-2: 4.32MHz/8.64MHz  BW, sequence length 28 chips

- Config-3: 4.32MHz/8.64MHz BW, sequence length 8 chips

The modulation of the above three configurations is OOK-4 and the chip rate is 56kbps, which means there are two segments for on/off states within one OFDM symbol. For better coverage performance, we show both 4.32MHz/8.32MHz BW, and more simulation parameters can be found in Appendix C.

In the following figures, we provide the coverage comparison between LP-WUS and legacy NR signals for both normal UE and Redcap UE. As shown in following figures, the LP-WUS configs can achieve higher MIL than PUSCH. In the three configurations above, LP-WUS Config-2 with length 28, can achieve the best performance. The MIL of LP-WUS configs cannot achieve comparable MIL as PDCCH AL16-2Rx for normal UE. And the MIL of LP-WUS Config-2  can achieve comparable MIL as PDCCH  AL8. For R18 Redcap UE with 1Rx, the MIL of LP-WUS can be comparable with PDCCH AL16, and the MIL of LP-WUS Config-2 is 1dB higher than the MIL of PDCCH AL16 in Rural scenario.

Figure 14. MIL comparison between PUSCH/PDCCH and LP-WUS for Urban 2.6GHz for normal UE

Figure 15. MIL comparison between PUSCH/PDCCH and LP-WUS for Rural 700MHz for normal UE

Figure 16. MIL comparison between PUSCH/PDCCH and LP-WUS for Urban 2.6GHz for Redcap UE

Figure 17. MIL comparison between PUSCH/PDCCH and LP-WUS for Rural 700MHz for Redcap UE

- Conclusion

In this contribution, design targets and evaluation methodologies for R18 LP-WUS/WUR study are discussed with the following observations and proposals:

Observation 1: In the case that the relative power of LP-WUR “ON” state is 0.03~0.5 unit, substantial power saving gain e.g., up to 80% can be achieved.

Observation 2: LP-WUR should address all traffic arrival cases for IoT/Wearable/eMBB, thus keeping the “ON” state power as low as possible is important for increasing the battery life of the device.

Observation 3: The latency brought by using LP-WUS/WUR consists of two parts: wake-up delay and MR transition time.

Observation 4: Compared with I-DRX paging, LP-WUR/WUS scheme with continuously monitoring configuration can achieve around 50%~98% power saving gain when the relative power of LP-WUR “ON” state is no more than 1 unit, with marginal latency increase.

Observation 5: Compared with eDRX, LP-WUR/WUS scheme with continuously monitoring configuration can largely reduce the paging latency (23x), with comparable UE power consumption.

Observation 6: LP-WUR/WUS scheme provides a much better trade-off between latency and power consumption when relative power of LP-WUR “ON” state is no more than 1unit, compared with I-DRX paging and eDRX scheme.

Observation 7: Battery life is inversely proportional to relative power of LP-WUR “ON” especially when the traffic is extremely sparse e.g., paging rate is 0.001%.

Observation 8:  Continuously LP-WUS monitoring is not a feasible configuration when the relative power of LP-WUR “ON” is higher than 1unit.

Observation 9: Duty cycled LP-WUS monitoring can significantly reduce UE power consumption of LP-WUS scheme except for the case with the relative power of LP-WUS “ON” to be 20 or 40units, but latency will increase accordingly.

Observation 10: Up to 45% power saving gain will be lost if UE main radio goes into deep sleep rather than ultra-deep sleep during LP-WUS monitoring.

Observation 11: With fixed duty cycle ratio and relative power of LP-WUR “ON” state, different LP-WUR duty cycle lengths have no or less impact on UE power consumption of LP-WUS scheme, and the difference is due to the number of LP-WUR ON-OFF transition.

Observation 12: For LP-WUS scheme, latency will be reduced with the increase of duty cycle ratio of LP-WUS monitoring, while UE power consumption will increase accordingly.

Observation 13: The power consumption of LP-WUS scheme with 20 or 40units LP-WUR “ON” power is ten or hundred times larger than that with less than 1unit LP-WUR “ON” power. So does the corresponding battery life.

Observation 14: Only when duty cycle ratio is extremely low e.g., 0.1%, substantial power saving gain (e.g., up to 50%) can be achieved in the case of high relative power of LP-WUS “ON” i.e., 20 or 40units.

Observation 15: Even if RRM measurement is performed by main radio at a relaxed level, the total UE power consumption of LP-WUS scheme increases distinctly.

Observation 16: No power saving gain can be obtained by duty cycled LP-WUS monitoring (with 2% duty cycle ratio) scheme, even for the case MR RRM periodicity is relaxed, when the relative power of LP-WUR “ON” is 20 or 40units.

Observation 17: RRM measurement completely performed by WUR will greatly help reduce UE power consumption of LP-WUS scheme.

Observation 18: As FAR for LP-WUS increases, UE power consumption goes up.

Observation 19: As the paging rate and the number of UE in group increase, power consumption of LP-WUS scheme will increase.

Observation 20: The motivation to study LP-WUS/WUR in RRC connected mode for XR use case is to reduce the excessive PDCCH monitoring due to unpredictable jitter.

Observation 21: The motivation to study LP-WUS/WUR in RRC connected mode for eMBB use case e.g., FTP3 is to reduce latency and improve UPT performance.

Observation 22: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {6%~15%} additional UE power saving gain with no capacity loss in both low load and high load cases.

Observation 23: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {10%~22%} additional UE power saving gain, with acceptable capacity loss at least in low load case.

Observation 24: UPT of LP-WUR scheme (wake-up latency 0ms case) is the same as always-on scheme which means it can effectively reduce power consumption without affecting network scheduling.

Observation 25: When the relative power of WUR “ON” state is no more than 1unit, LP-WUS monitoring with PDCCH skipping scheme can achieve the best trade-off performance in both UPT and power saving, compared with the existing UE power saving schemes.

Observation 26: When the relative power of WUR “ON” state is larger than 1unit i.e., 20 or 40 units, LP- WUS monitoring with PDCCH skipping scheme has no power saving gain advantage, compared with the existing power saving schemes.

Observation 27: For IDLE/INACTIVE mode, the upper bound of resource overhead used for LP-WUS is less than 1% for 100MHz system BW even in worst UE Connection density for both IoT and eMBB cases. For CONNECTED mode, the resource overhead of LP-WUS is less than 0.5% even in high load cases such as 10 XR UEs per cell.

| Resource overhead ratio, R | RRC Idle/inactive mode | RRC Connected mode |  |
| --- | --- | --- | --- |
|  |  | XR traffic | eMBB traffic |
| 20MHz, 30KHz SCS | 0.07% ~ 3.93% | - | - |
| 100MHz, 30KHz SCS | 0.01% ~ 0.79 % | 0.43% | 0.036% |

Observation 28: Additional network energy consumption caused by LP-WUS/WUR operation can be minimized, e.g., gNB can transmit LP-WUS in the slot with existing NR signal that to be transmitted.

Observation 29: If accurate timing can be assumed, UE only need one attempt for detection of the LP-WUS, regardless of LP-WUS channel format.

Observation 30: If accurate timing cannot be assumed, multiple attempts are needed for detection of a single LP-WUS transmission occasion.

Observation 31: For always on monitoring, monitoring more transmission occasions leading to high FAR. MDR performance is degraded to make sure the overall FAR target on multiple transmission occasions is fulfilled.

Observation 32: For duty cycled monitoring, the impact to MDR is limited if time drift can be periodically calibrated.

Observation 33: For always on monitoring across long reference time duration in RRC idle state

- For sequence-only LP-WUS, the sequence should be properly designed to achieve both MDR and FAR target.

- For LP-WUS with payload and CRC attachment, the CRC bits should be sufficient to achieve the FAR target.

Observation 34: For RRC connected state, the number of attempts is reduced if accurate timing can be assumed. Hence, both LP-WUS with sequence-only and LP-WUS with payload and CRC attachment are feasible to achieve target FAR and MDR.

Proposal 1: The target relative power of LP-WUR “on” state should be less than 1 unit.

Proposal 2: Consider the following as the latency target by applying LP-WUS/WUR:

- Within 1 or 2 seconds for latency-sensitive IoT cases and within several or tens of seconds for other IoT cases;

- Within several seconds for wearable cases;

- Within several milliseconds for XR use case and within tens of milliseconds for other eMBB use cases;

Proposal 3: Around 100kbps data rate can be considered as design target for LP-WUS.

Proposal 4: For sync/re-sync time and energy, Table 1 of R1-2302506 can be assumed.

Proposal 5: Transition time between LP-WUR ‘on’ and ‘off’ states can be assumed as 10ms.

Proposal 6: Adopt the following power model for LP-WUR for OFDMA-based signals/channels detection :

- Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: 0.01unit

- Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: 20 or 40units

Proposal 7: For RRM measurement assumptions in RRC idle/inactive mode, the following options can be considered.

- Option 1: RRM measurement is only performed by MR.

- Option 2: LP-WUR performs RRM measurement based on periodic lower power signal e.g., LP-SS. MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20.

- Option 3: RRM measurement is only performed by LP-WUR.

Proposal 8: In RRC connected mode, Ultra-deep sleep state of main radio should not be applied. And UE main radio can enter micro, light or deep sleep state during LP-WUS monitoring.

Proposal 9: Reporting per attempt FAR is not necessary

- UE power consumption mainly depends on reference duration FAR.

- The detection results from multiple attempts may be highly correlated, per attempt FAR cannot be easily derived from the per reference duration FAR.

Proposal 10: The reference time duration can be T, e.g., one DRX cycle, during which UE detect N WUS occasions,

- For always on monitoring, N>1, is the number of occasions within T.

- For duty cycled monitoring, value N depends on WUS periodicity. N=1, if reference time duration is WUS periodicity.

Proposal 11:  Option 3/4 cannot be used when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state.

# References

- Chairman’s Notes of RAN1 #112 (Athens, Greece, February 27th – March 3rd, 2023).

- R1-2302508, “Discussion on physical signal and procedure for low power WUS”, vivo.

- Jaeho Im, Jacob Breiholz, Shuo Li, Benton Calhoun, David D. Wenzloff , “A Fully Integrated 0.2V 802.11ba Wake-Up Receiver with -91.5dBm_Sensitivity”, 2020, IEEE Radio Frequency Integrated Circuits Symposium.

- Renzhi Liu, Asma Beevi K. T., Richard Dorranc, Deepak Dasalukunte, etc. “An 802.11ba 495W -92.6dBm-Sensitivity Blocker-Tolerant Wake-up Radio Receiver Fully Integrated with Wi-Fi Transceiver”, 2019, IEEE Radio Frequency Integrated Circuits Symposium.

# Appendix A – Processing timeline for the schemes in RRC idle/inactive mode

The assumed processing timelines of I-DRX paging, eDRX and LP-WUS/WUR schemes.

# Appendix B – Evaluation assumption for RRC connected mode

The detail configurations for R17 PDCCH monitoring adaptation and LP-WUS/WUR schemes are illustrated as below.

R17 PDCCH monitoring adaptation scheme includes both PDCCH skipping indication and SSSG switching indication, and its key assumptions are listed below:

- Default/sparse SSSG: do PDCCH monitoring in every two slots;

- Dense SSSG: do PDCCH monitoring in every slot.

- Two candidate PDCCH skipping durations: 4ms and 6ms.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

For LP-WUS/WUR scheme, the key assumptions are introduced below:

- The LP-WUS monitoring is performed within DRX onduration.

- The main receiver enters into the micro sleep when doing LP-WUS monitoring.

- The total relative power for LP-WUS monitoring is assumed as 45 or 20 power units (i.e., the main radio enters micro or light sleep).

- And the latency from receiving LP-WUS to be ready for PDCCH monitoring is set to 1 slot.

- R17 PDCCH skipping indication is adopted. And three candidate PDCCH skipping durations are 2ms, 4ms and 6ms respectively.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

- System capacity is assumed as the maximum number of users per cell with at least 90% of UEs being satisfied. And a UE can be regarded as a satisfied UE if more than 99% or 95% of packets are successfully transmitted within a given air interface PDB.

Table 14. Simulation assumption for FR1 Indoor Hotspot scenario

| Parameter | value |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

Table 15. The DL video traffic models with 60 FPS adopted in R17 XR SI

| Traffic model | VR/AR | VR/AR |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |
| Mean packet size (Bytes) | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 | 16.67 |
| PDB (ms) | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |
| Jitter Mean (ms) | 0 |  |
| Jitter STD (ms) | 2 or 5 |  |
| Jitter Range (ms) | [-4, +4], [-6, +6] or [-8, +8] |  |

# Appendix C – Evaluation Results on Coverage for LP-WUS

In the evaluation results shown in Appendix D, some common parameters are assumed, provided as follows

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Carrier Frequency | 2.6GHz |  |  |
| Case name | LP-WUS Config1 | LP-WUS Config2 | LP-WUS Config3 |
| Channel structure | sync: 16 chips data: 12bits(24 chips) CRC: 8 bits (16 chips) | Sequence only: 28 chips | Sequence only: 8 chips |
| Chip rate | 56kbps | 56kbps | 56kbps |
| WUS duration | 2 slots | 1 slot | 4 symbols |
| Performance metric | FAR <0.1% MDR 1% | FAR < 1% MDR 1% | FAR < 1% MDR 1% |
| Waveform | OOK -4 |  |  |
| Coding | 1/2 rate Manchester coding (For information bits and CRC bits) |  |  |
| Impairment modelling | {200 ppm,0.1ppm/s} , in saturated region |  |  |
| Beacon periodicity | 1.28 sec Note: beacon periodicity is used to calculate the time drift for WUS monitoring |  |  |
| SCS | 30kHz |  |  |
| gNB Channel BW | 20MHz (50 RB) |  |  |
| WUS BW | 4.32MHz /8.64M |  |  |
| Guard band | 1RB on each side of LP-WUS bandwidth |  |  |
| Filter | 5th Order Butterworth with 4.32MHz bandwidth |  |  |
| ASCI | PDSCH mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH = 1:1. |  |  |
| Sampling Rate | 3.84 MHz |  |  |
| ADC bitwidth | 4 bit ADC |  |  |
| Channel Model | TDL-C 300 |  |  |

Detailed MIL calculation for LP-WUS for above configurations, are provided in the following attachment which is calculated according to the template.

# Appendix D – Agreements of RAN1#112 meeting

Conclusion: The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group Agreement The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - FFS: latency - primary for small form devices - power-sensitive - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - FFS: latency - primary for small form devices, - power-sensitive - low/medium speed, FFS: high speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - FFS: latency - devices form is various and not restricted - power-sensitive - low/medium speed, FFS: high speed Note: other use cases/characteristics are not precluded if any. Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases. Agreement For coverage evaluation, the following is used, Number of RX chains at the UE’s MR antenna elements for UE; Case 1: 1 Rx for Redcap Case 2: 2 Rx Case 3: 4 Rx Company to report which case is being used. Further decision on antenna assumption for coverage is FFS.. Number of RX chains antenna elements for LP-WUR; 1 Rx Note: agreed in RAN1#110bis. Scenario and frequency; Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD). Reference data rates for MR eMBB; Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional). Reference PDCCH configuration; SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,.. Pathloss model (select from LoS or NLoS); Urban: NloS Rural: NloS and LoS. Bandwidth; 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD). Channel model for link-level simulation; TDL-C for NLOS, TDL-D for LOS.. Delay spread; Urban: 300ns, optional: 1000ns and companies to provide descriptions for such scenarios Rural: 300ns Rural with long distance: 30ns. UE velocity; Urban: 3km/h Rural: 3km/h, FFS: 120km/h (optional 30km/h) for outdoor. Number of antenna elements for BS; - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1). Number of TxRUs for BS; gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled.. Note: The descriptions above does not change the agreements for coverage in the RAN1#110-bis. Agreement For link-level simulation of LP-WUS, the following table is used as starting point, - FFS for other assumptions if any - Note: The assumptions are not intended to limit the scope of the study or the design. Table XX. Simulation assumptions for LP-WUS Attributes; Assumptions. Carrier Frequency; 2.6GHz/4GHz/700MHz. Waveform; OOK , FSK , OFDM Company to report which option for OOK /FSK /OFDM is used. Channel structure; - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence).. SCS of OFDM generator for NR signal; 30kHz/15KHz. Configuration for LP-WUS signal; For OOK/FSK waveform, - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report. WUS duration; Number of OFDM symbols: e.g., 1,2,4, 8, 16,24 symbols. MDR/FAR assumption; - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials. Code scheme; Companies to report, if any, the coding scheme (e.g., Manchester code or any other schemes) and the code rate (e.g., 1/2, 1/4, ….). gNB Channel BW; 20MHz, FFS other values. LP-WUS BW; Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS. Filter; X-th Order filter (e.g. Butterworth, Chebyshev, …) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y Companies to report any other assumptions if needed. Adjacent subcarrier interference; - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline, ρ= {3, 6} dB as optional. Sampling Rate; - Companies to report.. ADC bit width; 1-bit, 4-bit, 8-bit, ideal and other options are not precluded. Channel Model; See link coverage assumption table (will copy and paste here). Impairment modelling; - FFS: Frequency and time error model - Phase noise up to company report, e.g. the modelling used for 802.11ba - Other cell interference is up to company to report. Working Assumption - For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm]; 20. - Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option 3/4, - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state, - e.g., option 3/4 is not applicable - when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or, - when LR monitoring power less than [TBD] power unit, - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc. - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR. - assumptions important for achieving performance by using MR clock for LR should be declared - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option - Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1: - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1 - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach. - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm). - Model 2: random frequency drifting, FFS details - Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1 [R1-2301438] [R1-2301558][R1-1714993]: - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region) - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region) - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)] - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr - Model 2: random time drifting, FFS details - FFS: Phase noise model