3GPP TSG RAN WG1 #112bis-e                                                   R1-2302622

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.11.3

Source:	Spreadtrum Communications

Title:	Discussion on L1 signal design and procedure for low power WUS

Document for:	Discussion and decision

# Introduction

L1 design for signal/channel and procedures for low power wakeup signal (LP-WUS) is important to achieve the benefits of the LP-WUS.

The SID of the LP-WUS can be found in [1].

The study item includes the following objectives: - Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

The contribution mainly focuses on the design of procedures and LP-WUS.

# KPIs

The KPIs and requirements can be a guidance of design of the LP-WUS. In RAN1#110bis-e [2], the following KPIs can be considered.

- the power saving gain,

- the latency,

- the resource overhead to meet the coverage requirement, and

- the mobility in terms of the measurement relaxation at the main radio.

We should keep in mind in every details of designs.

# Sync and RRM measurement at the LP-WUR

In RAN1#111 [3], feasibility study of RRM measurement performed at the LP-WUR, at least for serving/camping cell, was agreed.

Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

## Neighboring-cell measurement

If the LP-WUR can support neighboring-cell measurement, the main radio can stay in ultra-deep sleep for long time. On the contrary, if the LP-WUR cannot support neighboring-cell measurement, the main radio may be timely wake up due to ping-pong effect at cell edge.

However, the corresponding reference signal for neighboring-cell measurement should be widely deployed in network and configured by gNB as measurement object. In other words, it is like SSB due to always-on characteristics. Furthermore, if the LP-WUR supports neighboring-cell measurement, it should support cell re-selection also, and the corresponding S criterion should be defined. The predicted spec impact seems large. Moreover, if the reference signal for the LP-WUR is out of band for the main radio, the measurement results may not be easily taken as reference for cell camping for the main radio.

On the other hand, it is not excluded that legacy SSS/PBCH-DMRS can be used as reference signal for neighboring-cell measurement for the LP-WUR. So, neighboring-cell measurement by the LP-WUR may be feasible.

Anyway, whether the LP-WUR can support neighboring cell measurement can be discussed further.

Proposal 1: Whether the LP-WUR can support neighboring-cell measurement can be discussed further.

## Sync and serving-cell measurement

In some companies’ contributions [4], the large time error (due to the large frequency drift) will degrade the detection performance of the LP-WUS significantly, especially when the LP-WUR monitors the LP-WUS periodically and the LP-WUS is not transmitted by gNB for long time. Usually sync and serving-cell measurement can be performed together for the same reference signal. Therefore, both sync and RRM measurement at the LP-WUR can  be discussed.

Proposal 2: At least, sync and serving-cell measurement is supported by the LP-WUR.

## Periodic signal or aperiodic signal

In RAN1#112 [5], there were three options for sync signal agreed, if sync signal is to be introduced.

Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2

It should be noted that as a DTX signal the LP-WUS is not suitable for sync or RRM measurement. As a part of the LP-WUS, preamble can be used for fine sync to mitigate the residual time/frequency error. Therefore, we prefer Option 2 and 3.

Proposal 3: For sync signal, at least periodic signal is studied.

## Alternatives of periodic signal

In our view, there could be two alternatives for sync and RRM measurement at the LP-WUR.

- Alt-1: Legacy SSS/PBCH-DMRS.

- Alt-2: Low power synchronization signal (LP-SS) with MC-ASK/FSK waveform.

In RAN1#112 [5], it was agreed that some measurement metrics can be studied.

Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements

In our view, RSRP is the baseline as measurement metric. The long sequence of legacy SSS/PBCH-DMRS has gain for interference mitigation, and cell ID in the sequence make the inter-cell interference randomized. It is noted that the total sequence length of legacy SSS/PBCH-DMRS is about 270 in total. For MC-ASK/FSK waveform, the sequence may not be long, since the number of MC-ASK/FSK symbols per OFDM symbol is very limited (i.e. value of M could be no greater than 8 in general view), and thus the interference mitigation and randomization gain are not good enough.

Moreover, it is better that the MR can use the periodic signal, which can reduce SSB overhead and then reduce the overall system overhead .

Proposal 4: OFDMA-based signals/channels (e.g. SSS/PBCH-DMRS) is prioritized for periodic sync signal.

To guarantee the measurement accuracy, the periodicity of the periodic signal can be similar to the severing-cell measurement periodicity implied by RRM measurement requirement, e.g. once per I-DRX cycle. In some companies’ contributions [6], the long periodicity of time error correction, e.g. once for several periodicities of the LP-WUS, may be enough, and the preamble at the beginning of the LP-WUS can be also used for residual time error correction. Therefore, the periodicity of the LP-SS may be long but no shorter than I-DRX cycle.

# Design of LP-WUS

## Waveform

In RAN1#111 [3], multi-carrier (MC)-ASK/FSK and OFDMA-based signals/channels were determined to be studied.

Agreement - Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform - Study techniques to generate waveform by modulating sub-carriers of CP-OFDM [FFS : drop CP at transmitter)] symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Note that above does not preclude DFT-S-OFDMA - Study generation and link performance of multi-carrier (MC)-FSK waveforms - Study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol [FFS : drop CP at transmitter)] symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS) - Other signal/channel structures are not precluded - For next meeting, companies to provide input on aspects to consider that might impact link performance

For OFDMA waveform, it may be sensitive to CFO and phase noise and the performance is limited by ICI. In general, it needs higher frequency synchronization requirement. Hence, we do not prefer OFDMA waveform for the LP-WUS.

In RAN1#112 [5], there were four options for MC-ASK waveform.

Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. - Option OOK-3: Multi-tone single-bit OOK - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view) - OOK=0 means all SCs in all segments are zero power (from base-band point of view) - FFS architecture - Option OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-1 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K - FFS modulated SCs are e.g. QAM symbols, sequences or other signals - Companies to report their assumptions - potential guard-band SCs are zero power (from base-band point of view) - [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] - Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol)

In RAN1#112 [5], there were two options for MC-FSK waveform.

Agreement For M-bit MC-FSK generation study further the following options - Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view) - Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - one segment from 2^M segments is modulated, other segments of SCs are zero power (from base-band point of view) - M >0 - N >1 - Study how to generate segment in time domain, e.g. OOK-1 or OOK-4 - Other options are not precluded.

It can be observed that MC-ASK and MC-FSK have no big difference. Especially for Option OOK-2/3, they are alike MC-FSK, since they are also segments FDMed. For segments FDMed, some companies [7] [8] reported the possible larger guard bands are needed, since the residual frequency error is still very large for the low-complexity architecture, e.g. low-precise oscillator. Therefore, MC-ASK is our preference and Option OOK-1/4 is preferred in MC-ACK waveform.

Proposal 5: MC-ASK waveform is preferred, and Option OOK-1/4 is preferred.

## Channel coding

The 1/2 code rate Manchester code is standardized in 802.11ba LP-WUS, so the 1/2 code rate Manchester code can be considered in NR LP-WUS too. To improve the sensitivity to extend coverage in WAN, the code rate may be lower than 1/2.

Proposal 6: The code rate can be equal to or lower than 1/2 (e.g. Manchester code).

## The channel bandwidth

In RAN1#111 [3], the channel bandwidth of the LP-WUS was discussed and only provided with the candidate upper bounds.

Agreement For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further - whether BW of LP-WUS is configurable (implicitly or explicitly) - size of guard band [FFS: within or outside of BW X], if any - whether there is different X for Idle, Connected, Inactive modes FFS: Whether FR2 is included in the scope of LP-WUS SI

Resource of LP-WUS can be scalable for different coverage. On the other hand, the LP-WUR can be further extended for more functionalities, and we should consider the forward compatibility of the LP-WUS. The BW of one LP-WUS can be scalable to enable the forward compatibility.

Proposal 7: The BW of one LP-WUS can be scalable.

## Structure

The structure of the LP-WUS can be like that of 802.11ba [9]. It includes two parts, such as delimiter/preamble and data payload. The structure is shown in the following figure.

Figure 1: Illustration of the structure of the LP-WUS

For the structure, it looks like a new slot format should be defined like a new frame structure in 802.11ba. However, even if we accept this structure in the LP-WUS in NR, we do not see the need to change slot format. It is common understanding that the slot format in NR only means the direction of transmission but does not mean anything on any definition of signal/channel. In other words, a physical signal/channel for the delimiter/preamble and the data payload is sufficient. Furthermore, the new slot format will cause the coexistence issue for the legacy UEs.

Furthermore, the new slot format will cause the critical coexistence issue between new UE and legacy UE.

Hence, there is no need to introduce a new slot format.

Proposal 8: The structure of the LP-WUS does not mean a new slot format.

As mentioned above, structure includes the delimiter/preamble and the data payload.

Delimiter/preamble

In some literatures [9], it can provide some functions, e.g. starting of LP-WUS for continuous monitoring case and fine synchronization to mitigate the residual time/frequency error. The delimiter/preamble is an aperiodic signal for sync as mentioned before. Furthermore, it can be regarded as a demodulation reference signal (DMRS) also.

Data payload

In common understanding, it can provide information like paging in which potentially tens of bits should be contained. Since it contains the large number of bits, it could be a channel more than a signal to achieve the better balance between link level performance and resource overhead.

This structure is running in 802.11ba, and is proved efficient in power consumption in unlicensed band.

Whether it need to be introduced directly in NR should be studies. Firstly, this structure seems natural for unlicensed band and the Delimiter/preamble can provide the preamble detection based LBT. Nevertheless, it is not so useful in licensed band. Secondly, KPIs in 802.11ba and in NR may be different, e.g. KPIs in 802.11ba does not have resource overhead to meet the coverage/mobility requirement. The coexistence of multiple types of signals/channels are more important for NR system, so whether a signal/sequence and a channel have to be adjacent in time domain is questionable. Therefore, the structure of the LP-WUS containing delimiter/preamble and data payload can be studied.

Proposal 9: The LP-WUS can contain two part, i.e. an aperiodic signal and a channel.

# Conclusion

We have the following proposals.

Sync and RRM measurement at the LP-WUR

Proposal 1: Whether the LP-WUR can support neighboring-cell measurement can be discussed further.

Proposal 2: At least, sync and serving-cell measurement is supported by the LP-WUR.

Proposal 3: For sync signal, at least periodic signal is studied.

Proposal 4: OFDMA-based signals/channels (e.g. SSS/PBCH-DMRS) is prioritized for periodic sync signal.

Design of LP-WUS

Proposal 5: MC-ASK waveform is preferred, and Option OOK-1/4 is preferred.

Proposal 6: The code rate can be equal to or lower than 1/2 (e.g. Manchester code).

Proposal 7: The BW of one LP-WUS can be scalable.

Proposal 8: The structure of the LP-WUS does not mean a new slot format.

Proposal 9: The LP-WUS can contain two part, i.e. an aperiodic signal and a channel.

# Reference

- RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#97e, Sep.  12th - 16th, 2022.

- 3GPP RAN1#110bis-e Chairman’s Notes, Oct. 10th - 19th, 2022.

- 3GPP RAN1#111 Chairman’s Notes, Nov. 14th - 18th, 2022.

- R1-2210012, “L1 signal design and procedures for LP-WUR”, Qualcomm, RAN1#110bis-e, Oct. 10th - 19th, 2022.

- 3GPP RAN1#112 Chairman’s Notes, Feb. 27th – Mar. 3rd, 2022.

- R1-2212144, “L1 signal design and procedures for LP-WUR”, Qualcomm, RAN1#111, Nov. 14th - 18th, 2022.

- R1-2211908, “LP-WUS receiver architectures”, ZTE, RAN1#111, Nov. 14th - 18th, 2022.

- R1-2212071, “Receiver architecture for LP-WUS”, Samsung, RAN1#111, Nov. 14th - 18th, 2022.

- “Study Item: Low-power Wake-up Signal and Receiver for NR”, workshop on R18 low power WUS, vivo, Sep. 20th, 2022.